3GPP TSG-RAN WG2 Meeting #110-e	R2-19xxxxx

Electronic, 1st Jun. – 12th Jun. 2020

Agenda Item:	6.11.4

Source:	vivo (rapporteur)

Title:	Report of [Post109bis-e][940][PowSav] RRM open issues

Document for:	Discussion and Decision

# Introduction

This document is a summary of the following email discussion [Post109bis-e][940][PowSav] RRM open issues.

- [Post109bis-e][940][PowSav] RRM open issues (vivo)

- Address stage-3 remaining open issues. Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

- Intended outcome: CR for 38.304 addressing open issues (including editorials received offline)

- Deadline: Next Meeting

This email discussion captures open issues of RRM measurements from the offline/online/email discussion in RAN2#109bis-e and from CR implementation phase, and aims to result sets of agreeable proposals for updating CRs.

The deadline for tdoc submission is 2020-05-21. So I suggest to have 2 phase for this email discussion as below:

Phase 1: RRM open issues discussion

Companies can provide their views on the listed discussion points (as well as new identified open issues, if any) to address stage-3 remaining open issues.

Deadline: 2020-05-18 00:00

Phase 2: draft CR for 38.304 to reflect the discussion summary

Companies to provide their views on the draft CR for 38.304 based on the summary of phase 1.

We can also further discuss the new identified open issues (if not everyone replied the question in phase 1) or some diverse issues which has no consensus in phase 1.

Deadline: 2020-05-20 00:00 (if possible)

# Discussion

In the last email discussion [POST109bis-e][505][PowSav] CR on 38.304 for power saving, companies have provided inputs regarding the TS 38.304 CR [1]. Meanwhile, some progress was made in RAN4 and an LS [2] to RAN2 has been agreed in RAN4 on RRM relaxation in power saving. Based on the above discussion, we would like to provide the following questions on open issues for RRM measurement relaxation.

Companies are welcome to add any NEW identified issues in the end of this document. Please note the Chair Guidance: Issues that have already been discussed and not pursued should not be brought up again.

## Terminology and Description

- How to define/describe the parameter for “And/Or” configuration?

According to the CR on 38.304/38.331 for power saving, the parameter combineRelaxedMeasCondition to describe “And/Or” configuration is captured in RRC and 38.304 as below:

combineRelaxedMeasCondition This indicates whether the UE combines the configured two conditions when determining whether to relax measurements.

In the email discussion [POST109bis-e][505][PowSav] CR on 38.304 for power saving, it was raised by some companies that the current description of parameter combineRelaxedMeasCondition is confusion, since the word “combine” seems to imply “AND” in the above description, but the “combine” can also be interpreted as “OR” in some way. The rapporteur thinks the confusion can be solved by updating the description, such as “this indicates whether the UE is allowed to perform measurement relaxation when either of the configured two criteria is fulfilled”.

Q1: Companies are invited to provide opinions on whether the current description for parameter combineRelaxedMeasCondition needs to be updated? If yes, please provide the preferred description of the parameter combineRelaxedMeasCondition.

- Option 1: Yes. Please specify. Note: this will be updated in both RRC specification and TS 38.304.

- Option 2: No.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic |  | combineRelaxedMeasCondition can mean either “AND” or “OR”, it is not a big issue as long as companies have the same understanding. However, based on RAN4’s conclusion, this parameter seems to be useless. In their thinking if only one criterion is fulfilled, UE can relax the measurement; however if both criteria are fulfilled, UE can stop the measurement completely. It doesn’t matter what value this parameter indicates. We will have the contribution on this in coming meeting. |

## Measurement relaxation approach

How to perform RRM measurement relaxation was discussed in RAN2 before. And RAN2 agreed in #108:

- Whether higher priority frequencies can be relaxed is up to network configuration.  FFS on how the configuration is done.

Further conclusion was made in RAN2#109-e meeting as below:

- Ask RAN4 (In the same LS to RAN4 listing the RAN2 agreements) about the behaviour of relaxation of higher priority carriers:

a)   For the case where Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, does RAN4 envision to relax higher priority carriers measurements further than Thigher_priority_search if RAN2-defined relaxation criterion(s) is/are met?

b)  For the case where Srxlev < SnonIntraSearchP or Squal < SnonIntraSearchQ, does it make sense / is there a performance benefit to only relax equal/lower priority carriers but not higher priority carriers measurements if RAN2-defined relaxation criterion(s) is/are met?

In RAN2#109-e, the above conclusion and question were sent to RAN4.

In RAN4#93 in Nov. 2019, it was concluded that:

|  | Applicable RRM relaxation methods |
| --- | --- |
| Scenarios #1 Low mobility scenario | Option 1: Allow RRM measurements with longer intervals |
| Scenarios #2 Not in cell-edge scenario | Option 1: Allow RRM measurements with longer intervals |
| Scenarios #3: Low-mobility + Not in cell-edge scenario | Option 2: UE is not required to meet the intra-frequency and inter-frequency neighbor cell measurement requirements |

After receiving RAN2 LS, RAN4 had more discussion and concluded the following behavior of measurement relaxation for higher/equal/lower priority carriers, and sent them to RAN2 in the reply LS [2]. I understand the discussion logic in RAN4#95bis-e was based on the conclusion in RAN4#93 in Nov.2019 above.

RAN4 also discussed the behaviour of relaxation of higher priority carriers, and had the following conclusion: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - No relaxation measurements on higher priority carriers further than Thigher_priority_search is expected, when the criteria of low mobility is not configured or not fulfilled. - RAN4’s assumption is that criteria of not in cell edge must be fulfilled in this scenario; - UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements, when criteria of low mobility and not in cell edge are both fulfilled. - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: - The relaxed requirement for the frequency layer of higher priority uses the same relaxed measurement requirement as those for the frequency layer of equal/lower priority.

According to the RAN4 conclusion, we can summarize the RRM measurement relaxation approach for different scenarios as below:

| Channel condition | Frequency | Criteria | Measurement approach (or with relaxation) | Use case ID |
| --- | --- | --- | --- | --- |
| For inter-frequency / inter-RAT frequency measurement, this part is deduced from RAN4 LS |  |  |  |  |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Higher priority carriers | Low mobility | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. | A |
|  |  | Not at cell edge | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. | B |
|  |  | Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements. | C |
|  | Equal/Lower priority carriers | All scenario | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) | D |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, Equal/Lower priority carriers | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | E |
|  |  | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | F |
|  |  | Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements. (Note: for higher/equal/lower carriers) | G |
| For intra-frequency measurement, this part is deduced from RAN2/RAN4 conclusion |  |  |  |  |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) | H |
| Srxlev ≤ SIntraSearchP and Squal ≤ SIntraSearchQ | N/A | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | I |
|  | N/A | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | J |
|  | N/A | Low mobility and Not at cell edge | No description in the RAN4 reply LS [2]. No conclusion in RAN2. But we can deduce from the conclusion in RAN4#93: UE can stop intra-frequency measurement. | K |

Note 1: All scenarios above include: Low mobility (Scenario #1), Not at cell edge (Scenario #2), Low mobility and Not at cell edge (Scenario #3).

Note 2: I put the index for all use cases in the above table in order to facilitate the following discussion.

Note 3: Some companies may have different interpretation for the word “May/Can” for the above RRM measurement relaxation approach. We can discuss this part in CR draft stage.

Rapporteur suggest to have three steps to discuss the measurement relaxation mechanisms for all scenarios above:

  - Step 1: Confirm and align the interpretation for the RAN4 reply LS [2].

  - Step 2: Whether to agree the conclusion from RAN4.

  - Step 3: For higher priority frequency, how to use the highPriorityMeasRelax indication.

Step 1: Confirm and align the interpretation for the RAN4 reply LS [2].

Q2: Whether the above interpretation for the measurement relaxation is aligned with the RAN4 reply LS?

- Option 1: Yes.

- Option 2: No. Please indicate which use case (e.g. A, B, C, …) and the corresponding reasons.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Mostly yes | In RAN4’s LS, case A (only low mobility is fulfilled) doesn’t exist as the “not at cell edge” must be fulfilled when Srxlev > SnonIntraSearchP. It seems they assume SSearchThresholdP ≤ SnonIntraSearchP already. |

Step 2: Whether to agree the conclusion from RAN4.

In the last several RAN2 meetings, we also had some discussion on the measurement relaxation mechanism for different scenarios. The above conclusion or interpretation from RAN4 may not align with some companies’ understanding. For example, some companies think that UE should not stop measurement for use case C and G. They think UE can perform measurements with the requirement of Thigher_priority_search according to TS 38.133, or UE can perform relaxed measurement but the relaxed requirement should not exceed the requirement of Thigher_priority_search, etc.

Q3: Companies are invited to provide opinions on whether to agree the conclusion from RAN4.

- Option 1: Yes, we should follow RAN4 conclusion.

- Option 2: No. Please indicate which use case (e.g. A, B, C, …) and the corresponding reasons.

- Option 3: We should further check with RAN4 for some use cases, on which we have concern. Please indicate which use case should be check with RAN4.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |

Step 3: For higher priority frequency, how to use the highPriorityMeasRelax indication.

In RAN2#108 and #109-e meeting, it was agreed to introduce an indication highPriorityMeasRelax to configure the measurement relaxation for higher priority frequencies. We had some discussion on the UE behaviors on how to apply this parameter for higher priority frequency, but no conclusion was made in RAN2. Thus, there is an Editor’s Note in current TS 38.304: FFS how to configure whether higher priority frequencies can be relaxed, and behaviour of relaxation of higher priority carriers pending RAN4 decisions.

As far as I know, there is no discussion and conclusion in RAN4 about this part. Here, the rapporteur suggests to discuss this issue based on the latest RAN4 progress. According to the above summary, only the use case C, E, F, and G have measurement relaxation. For use case A and B, there is no measurement relaxation. Thus, we should discuss how to use the highPriorityMeasRelax indication for use case C, E, F, and G.

During previous RAN2 discussion, there are several directions of understanding on how to use the indication highPriorityMeasRelax on the relaxation of higher priority carriers.

Q4: Companies are invited to provide views on which option is the preferred UE behavior of measurement relaxation for higher priority carriers:

Option 1: The behaviour for indication highPriorityMeasRelax is also controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True,

  - For use case C, E, F, G, when the criterion is fulfilled, the UE can perform relaxed measurement for higher priority frequency. How to relax measurement for higher priority frequency is up to the conclusion of above Q2 and Q3.

- Otherwise (i.e. highPriorityMeasRelax is not configured, or highPriorityMeasRelax is configured and set to True but the criterion is not fulfilled),

  - The UE will not perform relaxed measurement for higher priority frequency. The measurement requirement for higher priority frequency should follow the legacy defined in 38.133.

Option 2: The behaviour for indication highPriorityMeasRelax is not controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True,

  - For use case C, RAN4 already define the the UE can perform relaxed measurement for higher priority frequency when criteria of low mobility and not in cell edge are both fulfilled.

  - For use case E, F, G, UE can perform relaxed measurement for higher priority frequency regardless whether the trigger criterion is fulfilled.

- Otherwise,

  - The UE will not perform relaxed measurement for higher priority frequency. The measurement requirement for higher priority frequency should follow the legacy defined in 38.133.

Option 3: The measurement relaxation of higher priority carriers is not configured by the indication highPriorityMeasRelax, i.e. the behavior of relaxation for higher priority frequency is just based on the conclusion of Q2 and Q3. Note: this option is contradict with previous RAN2 agreement made in RAN2#108.

Option 4: Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 3 | If we follow RAN4’s decision and also the table summarized in Q2, everything is clear and there is no need to use highPriorityMeasRelax. |

## Configuration of threshold for relaxation criteria

During the discussion on the CR on 38.304, some companies commented that the threshold for not-at-cell-edge criterion should be smaller than SIntraSearchP/SIntraSearchQ. We will discuss this issue.

In Rel-15, UE is not required to perform intra-frequency measurements if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ. It is reasonable to perform measurement relaxation only when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ. Thus, the measurement relaxation is applicable only when SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ. Based on this understanding, network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ.

On the other hand, we should not restrict the network configuration. When network configures SSearchThresholdP > SIntraSearchP, or SSearchThresholdQ > SIntraSearchQ, there will be no measurement when Srxlev > SIntraSearchP and Squal > SIntraSearchQ according to current mechanism in TS 38.304. So that there is also no measurement relaxation even the criteria are accordingly fulfilled (i.e. Srxlev > SSearchThresholdP and Squal > SSearchThresholdQ).

Rapporteur think the above two understanding can work well without any problem. For inter-frequency measurement, the situation is same as above.

Q5: Companies are invited to provide views on which option is the preferred understanding for the configuration of threshold for not-at-cell-edge:

- Option 1: Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ,

- Option 2: There should be no restriction to network configuration.

- Option 3: others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | Option 2 would need to specify that UE shall prioritize the SIntraSearchP/SIntraSearchQ checking over the SSearchThresholdP/SSearchThresholdQ checking. |

Q6: If the answer for the above question (Q5) is Option 1, whether need/where to capture this restriction?

- Option 1: Nothing need to capture in the specification. Only need to align the understanding in the chair’s minutes.

- Option 2: Add a note or something in the field description in TS 38.331.

- Option 3: Add a note or something in the field description in TS 38.304.

- Option 4: others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option2 /Option 3 |  |

## Coordination between RAN2 and RAN4

- Capture the RAN4 conclusion on relaxation rule.

In legacy LTE, the relaxed monitoring measurement rule is captured in TS 36.304 as below:

5.2.4.12.0 Relaxed monitoring measurement rules When the UE is required to perform intra-frequency or inter-frequency measurement according to the measurement rules in sub-clause 5.2.4.2 or 5.2.4.2a, the UE may choose not to perform intra-frequency or inter-frequency measurements when: - The relaxed monitoring criterion in sub-clause 5.2.4.12.1 is fulfilled for a period of TSearchDeltaP, and - Less than 24 hours have passed since measurements for cell reselection were last performed, and - The UE has performed intra-frequency or inter-frequency measurements for at least TSearchDeltaP after selecting or reselecting a new cell.

Similar mechanism was agreed in RAN4, but with different values of Time Period. The detailed conclusion, which is highlighted, can be found in the LS [2] from RAN4 as below:

- UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements when: - both low mobility and not-at-cell-edge criteria are fulfilled, and - Less than 1 hour have passed since measurements for cell reselection were last performed.

Rapporteur would like to confirm with companies whether we agree to capture this in RAN2 specification (TS 38.304) as legacy.

Q7: Companies are invited to provide opinions on whether the RAN4 conclusion “Less than 1 hour have passed since measurements for cell reselection were last performed” needs to be captured in TS 38.304.

- Option 1: Yes, it needs to be captured in TS 38.304.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | However, we found it a bit difficult to implement the above RAN4 conclusion in section 5.2.4.9.0, as this section is describing the conditions when UE can “relax” the measurement but not “stop” the measurement. Maybe a new section is required to implement the above RAN4 conclusion. If a new section is created, then section 5.2.4.9.0 needs to remove the case where both “low mobility” and “not at cell edge” criteria are met. |

- Capture the RAN4 conclusion on detailed methods for relaxed measurements

There is an Enditor’s Note in TS 38.304, “FFS whether detailed methods for relaxed measurements is captured in TS 38.133”. According to the RAN4 reply LS [2], RAN4 has concluded on the detailed methods for measurement relaxation. In legacy LTE, this part has been captured in RAN4 specification.

- For low mobility scenario, RRM measurement relaxation with longer measurement intervals is applied. - The scaling factor of measurement interval is fixed. - For not in cell-edge scenario, RRM measurement relaxation with longer measurement intervals is applied. - The scaling factor of measurement interval is fixed. - UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements when: - both low mobility and not-at-cell-edge criteria are fulfilled, and - Less than 1 hour have passed since measurements for cell reselection were last performed.

Rapporteur would like to confirm with companies whether we agree to capture this part in RAN4 specification (TS 38.133) as legacy.

Q8: Companies are invited to provide opinions on whether the RAN4 conclusion on the detailed methods for measurement relaxation need to be captured in RAN4 specification.

- Option 1: Yes, it should to be captured in RAN4 specification.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |

- Measurement relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured

In RAN4 agreed WF [3], it was agreed that:

When network configures the parameters of both low mobility and not-at-cell-edge criteria, - If network indicates option a, (i.e. combineRelaxedMeasCondition is configured and set to TRUE) - the relaxation method corresponding to scenario #3 when both relaxation criteria have been fulfilled - If network indicates option b, (i.e. combineRelaxedMeasCondition is not configured) - the relaxation method corresponding to scenario #1 when only low mobility criteria is fulfilled - the relaxation method corresponding to scenario #2 when only not-at-cell-edge criteria is met. - FFS the relaxation method corresponding to scenario #3 when both relaxation criteria have been fulfilled

It is FFS in RAN4 that the relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured. This issue was also discussed in RAN2 email discussion, but had no conclusion. As far as I know, RAN4 will further discuss this issue to solve this FFS in the coming RAN4 meeting(s). Rapporteur would like to confirm with companies whether we agree to wait for RAN4 conclusion.

Q9: Companies are invited to provide opinions on whether we should to wait for RAN4 further conclusion on measurement relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured.

- Option 1: Yes, it will be further discussed and concluded in RAN4.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |

- Coordination with RAN4

Once we have the conclusion on the above questions, we should inform RAN4 to instruct them how to make futher discussion, and draft the specification, if we agreed something needs to confirmed/captured in RAN4. Thus, an LS should be sent to RAN4.

Q10. Whether an LS should be sent to RAN4 on the RRM measurement relaxation?

- Option 1: Yes, why;

- Option 2: No, why;

- Option 3: Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | At lease we shall update RAN4 regarding the RAN2 specification change (e.g., removal of highPriorityMeasRelax if RAN2 agree to remove it). We can also inform RAN4 how their agreements/conclusions are implemented in RAN2 specifications. |

## Other issues

- To be added

Q11. Any other issues that should be discussed in this email discussion? Please kindly specify, if any.

| Company | Comments |
| --- | --- |

# Summary and Proposals

This contribution summarizes the email discussion [Post109bis-e][940][PowSav] RRM open issues, and achieves the following proposals:

TO BE ADDED.

# References

- R2-2003959 CR for UE Power Saving in NR, vivo

- R4-2005331 Reply LS on RRM relaxation in power saving, RAN4.

- R4-2005330 WF on RRM measurement relaxation, RAN4.