3GPP TSG-RAN WG2 Meeting #110-e	R2-2004444

Electronic, 1st Jun. – 12th Jun. 2020

Agenda Item:	6.11.6

Source:	vivo (rapporteur)

Title:	Report of [Post109bis-e][940][PowSav] RRM open issues

Document for:	Discussion and Decision

# Introduction

This document is a summary of the following email discussion [Post109bis-e][940][PowSav] RRM open issues.

- [Post109bis-e][940][PowSav] RRM open issues (vivo)

- Address stage-3 remaining open issues. Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

- Intended outcome: CR for 38.304 addressing open issues (including editorials received offline)

- Deadline: Next Meeting

This email discussion captures open issues of RRM measurements from the offline/online/email discussion in RAN2#109bis-e and from CR implementation phase, and aims to result sets of agreeable proposals for updating CRs.

The deadline for tdoc submission is 2020-05-21. So I suggest to have 2 phase for this email discussion as below:

Phase 1: RRM open issues discussion

Companies can provide their views on the listed discussion points (as well as new identified open issues, if any) to address stage-3 remaining open issues.

Deadline: 2020-05-18 00:00

Phase 2: draft CR for 38.304 to reflect the discussion summary

Companies to provide their views on the draft CR for 38.304 based on the summary of phase 1.

We can also further discuss the new identified open issues (if not everyone replied the question in phase 1) or some diverse issues which has no consensus in phase 1.

Deadline: 2020-05-22 00:00 (if possible)

# Discussion

In the last email discussion [POST109bis-e][505][PowSav] CR on 38.304 for power saving, companies have provided inputs regarding the TS 38.304 CR [1]. Meanwhile, some progress was made in RAN4 and an LS [2] to RAN2 has been agreed in RAN4 on RRM relaxation in power saving. Based on the above discussion, we would like to provide the following questions on open issues for RRM measurement relaxation.

Companies are welcome to add any NEW identified issues in the end of this document. Please note the Chair Guidance: Issues that have already been discussed and not pursued should not be brought up again.

## Terminology and Description

- How to define/describe the parameter for “And/Or” configuration?

According to the CR on 38.304/38.331 for power saving, the parameter combineRelaxedMeasCondition to describe “And/Or” configuration is captured in RRC and 38.304 as below:

combineRelaxedMeasCondition This indicates whether the UE combines the configured two conditions when determining whether to relax measurements.

In the email discussion [POST109bis-e][505][PowSav] CR on 38.304 for power saving, it was raised by some companies that the current description of parameter combineRelaxedMeasCondition is confusion, since the word “combine” seems to imply “AND” in the above description, but the “combine” can also be interpreted as “OR” in some way. The rapporteur thinks the confusion can be solved by updating the description, such as “this indicates whether the UE is allowed to perform measurement relaxation when either of the configured two criteria is fulfilled”.

Q1: Companies are invited to provide opinions on whether the current description for parameter combineRelaxedMeasCondition needs to be updated? If yes, please provide the preferred description of the parameter combineRelaxedMeasCondition.

- Option 1: Yes. Please specify. Note: this will be updated in both RRC specification and TS 38.304.

- Option 2: No.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic |  | combineRelaxedMeasCondition can mean either “AND” or “OR”, it is not a big issue as long as companies have the same understanding. However, based on RAN4’s conclusion, this parameter seems to be useless. In their thinking if only one criterion is fulfilled, UE can relax the measurement; however if both criteria are fulfilled, UE can stop the measurement completely. It doesn’t matter what value this parameter indicates. We will have the contribution on this in coming meeting. |
| OPPO | Needs update | We do think “combine” confused and does not reflect the true meaning directly. To us, we would like to suggest another terminology, e.g., “fulfillEitherRelaxedMeasCondition”, and the interpretation from the rapporteur is ok from us. |
| Ericsson | Option 1 | In our understanding RAN4 will further discuss the “AND” / “OR” configuration in next RAN4 meeting, and RAN4 intends to introduce the requirements for relaxed RRM measurements when “OR” is configured, and both criteria are fulfilled. What about the following wording? This indicates how the UE combines lowMobilityEvalutation and cellEdgeEvalutation criterion when both are configured to determine the allowed measurement relaxation. The description of the parameter should remain on a high, i.e. the details are captured in the procedure text. RAN2 can also not complete those details because RAN4 is still discussing them. The word “must” should not be used. RAN4 is considering a transition period when UE transits between scenarios#1, #2 and #3. Furthermore RAN4 has an FFF if relaxation method for scenario#3 applies when option b (“OR”) is configured and both low mobility and not-at-cell-edge are fulfilled. Based on the final RAN4 agreements, there may or may not be a difference anymore when option a (“AND”) or option b (“OR”) is configured. It is our understanding that the RAN2 agreement to make “AND” / “OR” configurable was intended to allow a different UE behavior in the two cases. Perhaps we should remind RAN4 about the RAN2 intention? When “AND” is configured the UE can only apply the REL-16 relaxation when both criteria are met, and when “OR” is configured when one of them is met, otherwise the legacy requirements apply. In our understanding the RAN4 transition periods are not needed, and only add complexity, because TSearchDeltaP already provides a hysteresis for frequent transitions in and out of low mobility state. Furthermore for legacy measurement thresholds there also do not use transition periods. |
| vivo | Option 1 | We can keep the terminology combineRelaxedMeasCondition, but the description can be further modified. We think the wording suggested by Ericsson is OK, or further updated as: This indicates how the UE combines lowMobilityEvalutation and cellEdgeEvalutation criterion when both are configured to determine the allowed measurement relaxation. |
| ZTE | Option 1 | combineRelaxedMeasCondition This indicates whether the UE needs to fulfill both low mobility criterion and not at cell edge criterion simultaneously when determining whether to relax measurements. |
| CATT | Option 1 | The latest RRC CR defines this parameter as: configured = AND, not configured = OR. So we only need to clarify the field description in RRC as follows: “When both lowMobilityEvalutation and cellEdgeEvalutation are present in SIB2, this parameter, when present, configures whether the UE must meet both combines the two conditions when determining whether to relax measurements (see TS 38.304 [20], clause 5.2.4.9.0). If the field is absent, the UE is allowed shall not combine the two conditions when determining whether to relax measurements (see TS 38.304 [20], clause 5.2.4.9.0) when either of the two conditions is met.” |
| LG | Option 1 | combineRelaxedMeasCondition This indicates whether the UE is allowed to perform relaxed measurements only if both conditions are met, when both conditions are configured. FFS if this parameter is not configured but the UE satisfies both conditions. |
| Intel | Option 1 | We agree that “combine” can be misleading, however we do not agree with current suggestion as “when either of the configured two criteria is fulfilled” describes the operation of an OR when 38.304 CR explains an AND, as shown below: if combineRelaxedMeasCondition is configured and set to True, - the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and, - the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled; We are open on the field description of combineRelaxedMeasCondition as soon as it is not misleading. Considering that 38.304 describes UE’s operation, the description could be a little more general. For example: This indicates when the UE needs to fulfill both configured criteria (for low mobility and for UE not at cell edge) as specified in TS 38.304 [x] in order to perform measurement relaxation. In addition, we would also be open to update the terminology used (e.g. ApplyBothRelaxedMeasCriteria or BothRelaxedMeasCriteria or AllRelaxedMeasCriteria) |
| MediaTek | Option 1 | We suggest explicitly stating that when the field is configured, UE needs to fulfill both low mobility and not at cell edge criteria for RRM measurement relaxation. Thus ZTE or CATT’s text can be considered. |
| Huawei |  | The detailed behavior is specified in 38.304, therefore there is no strong need to update the field description because this is already clear - but we are open to improving the wording e.g. as suggested by ZTE or Ericsson. |
| Samsung | Option 1 | We agree with Intel’s view. The term “Combine” is confusing, and rapporteur’s proposal which describes OR operation is not in accordance with 38.304 CR which describes AND operation. Intel’s description for combineRelaxedMeasCondition looks good for us. |
| Apple | Option 1 | Explicitly clarifying the RRC description as suggested by CATT or ZTE is preferred. |

Summary: 11 companies provided views.

9 companies prefer Option 1, i.e. the current description for parameter combineRelaxedMeasCondition needs to be updated.

- 1 company of them thinks the description in RAN2 should remain on a high level. 1 company points out RAN4 has an FFS if relaxation method for scenario#3 applies when option b (“OR”) is configured and both low mobility and not-at-cell-edge are fulfilled. Based on the final RAN4 agreements, there may or may not be a difference anymore when option a (“AND”) or option b (“OR”) is configured. we should remind RAN4 about the RAN2 intention that the RAN2 agreement to make “AND” / “OR” configurable was intended to allow a different UE behavior in the two cases. Rapporteur suggests this issue can be further discussed in phase 2 in Q9-2.

1 company thinks the current description is clear but is open to improve the wording.

1 company thinks this parameter is useless based on the latest RAN4 conclusion.

Based on the inputs from companies, rapporteur suggests to go for the clear majority to update the description of parameter combineRelaxedMeasCondition. According to the companies’ suggestions and preference, rapporteur suggests not to use the terminology “combine” which can be misleading. Jointly considering the suggestions from Ericsson, ZTE, CATT, LG, and Intel, rapporteur suggests the following proposal:

Proposal 1: The description for parameter combineRelaxedMeasCondition in TS 38.304 can be:

This indicates whether the UE needs to fulfil both low mobility criterion and not-at-cell-edge criterion when determining whether to relax measurements.

The field description for parameter combineRelaxedMeasCondition in TS 38.331 can be:

When both lowMobilityEvalutation and cellEdgeEvalutation are present in SIB2, this parameter, when present, configures whether the UE needs to fulfil both configured criteria when determining whether to relax measurements (see TS 38.304 [20], clause 5.2.4.9.0.

Whether to change the terminology to applyBothRelaxedMeasCriteria or bothRelaxedMeasCriteria or allRelaxedMeasCriteria is up to RRC rapporteur.

Phase-2 discussion: whether agree the above proposal.

Q1-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal.

| Company | Comments, if any |
| --- | --- |
| OPPO | For 304 description of the parameter, we suggest the following: This indicates whether the UE must fulfil both low mobility criterion and not-at-cell-edge criterion (if both of them are configured) in order to relax measurements. We prefer “applyBothRelaxedMeasCriteria” |
| Ericsson | In our view, we either have a high level description which say “how to combine” or we specify both AND and OR conditions in details. If companies think that the OR condition is specified further in the procedure text, i.e. this comment applies to both AND and OR. In 38.331 we also have a good tradition to specify the UE behavior when an IE is present and absent, i.e. set or not set. |

  - Measurement relaxation approach

How to perform RRM measurement relaxation was discussed in RAN2 before. And RAN2 agreed in #108:

- Whether higher priority frequencies can be relaxed is up to network configuration.  FFS on how the configuration is done.

Further conclusion was made in RAN2#109-e meeting as below:

- Ask RAN4 (In the same LS to RAN4 listing the RAN2 agreements) about the behaviour of relaxation of higher priority carriers:

a)   For the case where Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, does RAN4 envision to relax higher priority carriers measurements further than Thigher_priority_search if RAN2-defined relaxation criterion(s) is/are met?

b)  For the case where Srxlev < SnonIntraSearchP or Squal < SnonIntraSearchQ, does it make sense / is there a performance benefit to only relax equal/lower priority carriers but not higher priority carriers measurements if RAN2-defined relaxation criterion(s) is/are met?

In RAN2#109-e, the above conclusion and question were sent to RAN4.

In RAN4#93 in Nov. 2019, it was concluded that:

|  | Applicable RRM relaxation methods |
| --- | --- |
| Scenarios #1 Low mobility scenario | Option 1: Allow RRM measurements with longer intervals |
| Scenarios #2 Not in cell-edge scenario | Option 1: Allow RRM measurements with longer intervals |
| Scenarios #3: Low-mobility + Not in cell-edge scenario | Option 2: UE is not required to meet the intra-frequency and inter-frequency neighbor cell measurement requirements |

After receiving RAN2 LS, RAN4 had more discussion and concluded the following behavior of measurement relaxation for higher/equal/lower priority carriers, and sent them to RAN2 in the reply LS [2]. I understand the discussion logic in RAN4#95bis-e was based on the conclusion in RAN4#93 in Nov.2019 above.

RAN4 also discussed the behaviour of relaxation of higher priority carriers, and had the following conclusion: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - No relaxation measurements on higher priority carriers further than Thigher_priority_search is expected, when the criteria of low mobility is not configured or not fulfilled. - RAN4’s assumption is that criteria of not in cell edge must be fulfilled in this scenario; - UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements, when criteria of low mobility and not in cell edge are both fulfilled. - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: - The relaxed requirement for the frequency layer of higher priority uses the same relaxed measurement requirement as those for the frequency layer of equal/lower priority.

According to the RAN4 conclusion, we can summarize the RRM measurement relaxation approach for different scenarios as below:

| Channel condition | Frequency | Criteria | Measurement approach (or with relaxation) | Use case ID |
| --- | --- | --- | --- | --- |
| For inter-frequency / inter-RAT frequency measurement, this part is deduced from RAN4 LS |  |  |  |  |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Higher priority carriers | Low mobility | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133.N/A | A |
|  |  | Not at cell edge | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. | B |
|  |  | Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements. | C |
|  | Equal/Lower priority carriers | All scenario | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) | D |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, Equal/Lower priority carriers | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | E |
|  |  | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | F |
|  |  | Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. (Note: for higher/equal/lower carriers) | G |
| For intra-frequency measurement, this part is deduced from RAN2/RAN4 conclusion |  |  |  |  |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) | H |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | I |
|  | N/A | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | J |
|  | N/A | Low mobility and Not at cell edge | No description in the RAN4 reply LS [2]. No conclusion in RAN2. But we can deduce from the conclusion in RAN4#93: UE can stop intra-frequency measurement with a minimum measurement time interval of 1 hour. | K |

Note 1: All scenarios above include: Low mobility (Scenario #1), Not at cell edge (Scenario #2), Low mobility and Not at cell edge (Scenario #3).

Note 2: I put the index for all use cases in the above table in order to facilitate the following discussion.

Note 3: Some companies may have different interpretation for the word “May/Can” for the above RRM measurement relaxation approach. We can discuss this part in CR draft stage.

Rapporteur suggest to have three steps to discuss the measurement relaxation mechanisms for all scenarios above:

  - Step 1: Confirm and align the interpretation for the RAN4 reply LS [2]. In this step, we only discuss how to understand the RAN4 conclusion and reply LS. It is encouraged for companies to check with their RAN4 colleagues. Companies’ preference should be provided and discussed in next step 2.

  - Step 2: Whether to agree the conclusion from RAN4. Companies can express their views on their preference in this step.

  - Step 3: For higher priority frequency, how to use the highPriorityMeasRelax indication.

Step 1: Confirm and align the interpretation for the RAN4 reply LS [2].

Q2: Whether the above interpretation for the measurement relaxation is aligned with the RAN4 reply LS?

- Option 1: Yes.

- Option 2: No. Please indicate which use case (e.g. A, B, C, …) and the corresponding reasons.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Mostly yes | In RAN4’s LS, case A (only low mobility is fulfilled) doesn’t exist as the “not at cell edge” must be fulfilled when Srxlev > SnonIntraSearchP. It seems they assume SSearchThresholdP ≤ SnonIntraSearchP already. |
| OPPO | No | For A, B, C, we have different understandings, i.e., corresponding to the following part in the RAN4 LS: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - No relaxation measurements on higher priority carriers further than Thigher_priority_search is expected, when the criteria of low mobility is not configured or not fulfilled. - RAN4’s assumption is that criteria of not in cell edge must be fulfilled in this scenario; - UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements, when criteria of low mobility and not in cell edge are both fulfilled. In our understanding, for higher priority frequency, if Srxlev>SnonIntraSearchP and Squal>SnonIntraSearchQ, UE does not need to further relax the measurement of higher priority measurement. The assumption from RAN4 could be correct, i.e., the “not in cell edge” is fulfilled in this case. However, why higher priority measurement should be further relaxed if low mobility criteria is fulfilled? Then for E,F,G, which are corresponding to the following part in the RAN4 LS: - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: The relaxed requirement for the frequency layer of higher priority uses the same relaxed measurement requirement as those for the frequency layer of equal/lower priority. Actually, in RAN4 LS, they only say that the relaxed requirement for higher priority frequency use the SAME for equal/lower, it does not actually say the criteria should be used also for higher priority frequency. Maybe we should clarify this to RAN4. Then, I guess if “Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ”, UE should relax higher priority frequency measurement irrespective of criteria, and which relaxed measurement requirement is used is up to RAN4. |
| Ericsson | No | We think the RAN4 LS is confusing and we cannot confirm the correct interpretation. We also understand that RAN4 will continue the discussion in next RAN4 meeting. We want the UE to measure at least every minute higher priority frequencies all the time. In REL-15 the UE is required to search at least every minute on higher priority frequencies, both in good and bad coverage conditions. In bad conditions the UE has to measure higher priority frequencies more often than one minute. RAN2 agreed that higher priority frequency relaxation is under NW control, i.e. only allowed when highPriorityMeasRelax is set. We think that higher priority measurements are critical for load balancing and should not be further relaxed. We also note that higher priority measurements only happen every minute, while the lower/equal priority measurements happen every DRX cycle, i.e. from a power consumption perspective they are not comparable. Furthermore the higher priority measurements are scaled already among the higher priority frequencies the UE is required to measure, see 38.133 section 4.2.2.7: The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information. This means that if the UE is camped on a low priority frequency, and several higher priority frequencies are indicated in system information (e.g. 4), then the UE measures each higher priority frequency only once every 4 minutes. We do not think that further scaling should be applied on top of that. Having said all that, in case further relaxation of higher priority measurements every minute is considered, we think this should be coupled to the low mobility criterion, i.e. if the UE is stationary, and the UE did not find higher priority frequencies while being mobile, then perhaps the UE should not continue to those measurements when stationary, because apparently the UE is at a spot where there are no higher priority frequency coverage: From Ericsson perspective we wanted a similar and simple approach as in NB-IoT/MTC i.e. based on low mobility detection allowing the UE not to measure. We did not see the need for another threshold (not-at-cell-edge), because we have the existing measurement thresholds already. But now the NW has to configure not-at-cell-edge threshold to allow the UE not to measure. Also note that when not-at-cell-edge is configured only, there is only a relaxation factor 2 in some/small region of the cell, i.e. we still don’t believe in the usefulness of another not-at-cell-edge threshold. Furthermore when configured together with low mobility, the new threshold just seems to move the legacy measurement threshold. But when stationary at the cell border, the UE is not allowed not to measure. After further internal discussion, the RAN4 LS can be read saying: - In good coverage: - only relax higher priority frequency measurement when low mobility is fulfilled: - Not-at-cell-edge is fulfilled in case A - Case A and 1 are re-confirmed here and it is clarified that “relax” means “not required to measure”, i.e. UE is not required to measure higher (and equal/low) priority frequencies when both low mobility and not-at-cell-edge are fulfilled. - In bad coverage: - The same relaxation as for equal/low priority frequency layers is applicable for higher priority frequency layers, i.e. scaling is applied when one criterion is fulfilled, and when both criterion are fulfilled UE is not required to measure. |
| vivo | Yes | The above summary is aligned with RAN4 conclusion and LS, even we don’t agree with the behaviors in some use case. We can further discuss how to relax the measurement for higher priority frequencies (or whether to agree RAN4 ) in Q3. |
| ZTE | No | Use case A: NA is confusing because this criteria do make sense when Srxlev > SonoItraSearchP and Squal > SnonIntraSearchQ. Use case B: Because UE always fulfills not cell edge criterion when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, we do not think this use case should be listed in this situation. In short, there is only one triggering criterion in this case and the original use case A,B,C may be modified as below: Use case A’ : When UE fulfills low mobility criterion, UE may stop both equal/low priority and high priority inter-freq/inter-RAT measurements. Use case B’: When UE does not fulfills low mobility criterion, the UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133 on the high priority frequency. [Note: based on the RAN4’s perspective, when when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE always fulfills the not at cell edge criteria] |
| CATT | Yes | …with the correction highlighted above accounting for the minimum measurement time interval of 1 hour for RAN4 scenario 3. The only issue left is whether usecase A is valid for addressing the case when only “low mobility” criterion is configured. Strictly speaking, if we agree the below Q5 assumption (SSearchThresholdP < SIntraSearchP and SSearchThresholdQ < SIntraSearchQ), which is also the RAN4 assumption, it does not really matter if the “not at cell edge” criterion is configured or not, provided that for usecases A, B, C, it is always met. So we think RAN4 LS is quite clear that under this assumption: When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then - If “low mobility” criterion is configured and met: relax further the hi-prio frequencies compared with Rel-15 -> UE is allowed to not measure more often than every hour - Else: apply legacy (Thigher_priority_search) If companies have issues with the above relaxation beyond Rel-15, the highPriorityMeasRelax can be used to mitigate it (see Q5). |
| LG | Yes | We think it is correct that case A cannot exist. RAN4 assumes that when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, not-at-cell edge condition is already met because the threshold for the not-at-cell edge condition will be lower than the existing thresholds, i.e., SnonIntraSearchP and SnonIntraSearchQ. We could add description for this. For the cases E, F and G, as higher-priority frequencies are applied same measurement requirement with equal/lower-priority frequencies, so we are aligned with the interpretation on the table. By the way, as intra-frequency is treated as one of equal-priority frequency, do we really need to split the cases? |
| Intel | Partially | For the following cases, we do not share the explanation provided in the table: - Case A) We understand that this scenario was not addressed by RAN4 in their LS. - Case C) We are partially ok, as we understand that the following requirement also applies when both criterion (of low mobility and not at cell edge) are configured with an AND: “Less than 1 hour have passed since measurements for cell reselection were last performed” - Case G) and K) We have the same comment as for case C) i.e. UE can stop measurements with the 1 hour time interval rule that is explained in RAN4 LS when both criteria are configured and checked. - For other cases, we are OK with few minor clarifications to confirm: - We are ok with explanation on the table for cases B), E), F), I), J) understanding that they described the expected measurement relaxation based on RAN4 input on LS. - We are also ok with explanation on the table for case D) and H) if the understanding is that legacy operation applies where UE may choose not to perform measurement as explained in the table (as RAN4 LS did not address them). On summary, it may be good if RAN2 asks RAN4 for input on case A (i.e. when low mobility criterion is only configured or checked), and ask them to confirm RAN2 interpretation on other cases. |
| MediaTek | No | We suggest that the categorization be modified. 1. The “priority” separation should be “higher-priority” vs. “equal & lower priority”, instead of having “inter-frequency & inter-RAT” vs. “intra-frequency” and then consider priority in the former case. 2. Then for each of the two priority cases, we consider the low mobility & not at cell edge criteria and corresponding relaxation behaviors. For example, High priority NBR cell; Low mobility; Not low mobility. Cell center; Stop measurement for up to 1hr; No relaxation (measurement with Thigher_priority_search). Middle area; Stop measurement for up to 1hr; Relaxed measurements with longer intervals. Cell edge; Relaxed measurements with longer intervals; No relaxation. Equal/Low priority NBR cell; Low mobility; Not low mobility. Cell center; Stop measurement for up to 1hr; Stop measurement for up to 1hr. Middle area; Stop measurement for up to 1hr; Relaxed measurements with longer intervals. Cell edge; Relaxed measurements with longer intervals; No relaxation. Note: “Middle area” means that Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (not in cell center) but the not in cell edge criterion is met. |
| Huawei | Mostly yes | As commented above, for the case Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the only new behavior introduced is for high priority carriers when low mobility is detected. For the case Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then all carriers are treated in the same way regardless of priority. |
| Samsung | Mostly yes, but | We understand Panasonic and Chenli’s view that Case A is N/A. However, what if network configures only low-mobility criteria, and it is fulfilled? If so, Case A exists, since not-at-cell-edge is not examined, and thus not fulfilled. In this case, we think the desirable UE operation is same with Case C, i.e., stopping measurement. However, if needed, we can check RAN4 has the same view with us. |

Summary: 11 companies provided views.

7 companies (Panasonic, vivo, CATT, LG, Intel, Huawei, Samsung) think the above interpretation for the measurement relaxation is (mostly) aligned with the RAN4 reply LS.

- For CATT’s reply, some companies including I (vivo) thinks the new added highlighted part is not the RAN4 conclusion. That’s why some companies don’t agree RAN4 conclusion in the following Q3. Anyway, for this point, rapporteur suggests we can confirm RAN2’s understanding in the following Phase-2 discussion in Q3-2 (No matter what RAN4 LS means.)

- For Intel’s reply, rapporteur thinks you share the same view as CATT on use case C, G, K. For this point, rapporteur suggests we can confirm RAN2’s understanding in the following Phase-2 discussion in Q3-2 (No matter what RAN4 LS means.)

- For Samsung’s reply, this is a very good point. Rapporteur thinks we should jointly discuss the different configuration cases. Let’s discuss it in Phase-2 discussion in Q3-2.

4 companies (OPPO, Ericsson, ZTE, MediaTek) have different interpretation for the measurement relaxation is (mostly) aligned with the RAN4 reply LS

- For OPPO’s reply, rapporteur observed that the main difference is the understanding on the relaxation approach for the higher priority frequencies.

- After review the latest reply from Ericsson, rapporteur assumes latest understanding from Ericsson is almost aligned with the above summary.

- For ZTE’s reply, rapporteur assumes this understanding is another wording for the summary. There is no essential difference from the summary.

- For MediaTek’s reply, rapporteur assume this is different categorization for the summary. There is no essential difference from the meaning of the summary.

As companies have different understandings on the RAN4 conclusion, but some companies (even with different understandings on RAN4 conclusion) have similar preference. Rapporteur suggests we can move to phase-2 discussion on RAN2 understanding for all these cases by considering the following question.

Step 2: Whether to agree the conclusion from RAN4.

In the last several RAN2 meetings, we also had some discussion on the measurement relaxation mechanism for different scenarios. The above conclusion or interpretation from RAN4 may not align with some companies’ understanding. For example, some companies think that UE should not stop measurement for use case C and G. They think UE can perform measurements with the requirement of Thigher_priority_search according to TS 38.133, or UE can perform relaxed measurement but the relaxed requirement should not exceed the requirement of Thigher_priority_search, etc.

Q3: Companies are invited to provide opinions on whether to agree the conclusion from RAN4.

- Option 1: Yes, we should follow RAN4 conclusion.

- Option 2: No. Please indicate which use case (e.g. A, B, C, …) and the corresponding reasons.

- Option 3: We should further check with RAN4 for some use cases, on which we have concern. Please indicate which use case should be check with RAN4.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |
| OPPO | No | At least for C. EFG does not apply to higher priority frequency, i.e., higher priority frequency measurement relaxation should be independent of criterias. |
| Ericsson | - | It is not 100% clear what RAN4 agreed, and therefore it is not clear what to agree or disagree with. But RAN2 agreed that higher priority frequency relaxation is under NW control, which is on top of RAN4 agreements. |
| Vivo | Option 2 | At least for use case C and higher priority part for use case G. In our understanding, measurement on higher priority frequency should not be stopped roughly. - In use case C, the UE shall perform measurements for higher priority frequencies with the requirement of Thigher_priority_search according to TS 38.133. - In use case E, F, G, UE can perform relaxed measurement for higher priority frequencies, but the relaxed requirement should not exceed the requirement of Thigher_priority_search, |
| ZTE | -- | Use case C,D,G,K (stop measuring frequencies in these use cases) We do not think stopping either intra-frequency or inter-frequency measurement under some use cases is good for UE’s performance. For example, a UE camps in a FR2 cell and meets one of the use cases mentioned above. In this scenario, because the coverage of the FR2 cell is not as large as the coverage of FR1 cell. Hence, UE may have more risks to loss the NW if the UE suddenly change its movement status sometime. May be we can send a LS to RAN4 and let them know our concern. |
| CATT | Option 1 | There is no point in challenging RAN4 agreements. |
| LG | Option 1 | We are fine with RAN4 agreements. |
| Intel | Option 1/3 | As explained in Q2, we should ask for RAN4 input on case A) and follow RAN4 conclusions on other cases (which could be also included in a RAN2 LS to confirm RAN2 understanding of those conclusions included in RAN4 LS). |
| MediaTek | Option 1 | It’s about measurement requirements, RAN2 should not challenge RAN4 agreement unless it’s not feasible. |
| Huawei | Option 1 | This has been discussed extensively in RAN4 and we shouldn’t reopen that. |
| Samsung | Option 3 | We suggest to check the further classification according to our answer to Q2. |

Summary: 11 companies provided views.

6 companies prefer Option 1, i.e. we should follow RAN4 conclusion.

3 companies have concern on mechanism in use case C, G, or C, G, D, K.

2 or 3 companies prefer to send an LS to RAN4 for further check.

Rapporteur checked companies who prefer option 1 also have more or less different understanding on RAN4 conclusion. Thus, rapporteur suggest we can move to Phase 2 discussion in Q3-2.

Phase-2 discussion: measurement relaxation approach.

After considering the above inputs from majority companies and RAN4 conclusion, rapporteur summarize the RRM measurement relaxation approach for different scenarios as below:

| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) | Use case ID |
| --- | --- | --- | --- | --- |
| For inter-frequency / inter-RAT frequency measurement, this part is deduced from RAN4 LS |  |  |  |  |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (According to following proposal 5, not-at-cell-edge criterion is fulfilled always, if it is configured.) | Higher priority carriers | Low mobility (i.e. when NW only configures low mobility, and it is fulfilled) | Option 1: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Option 2: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. Note: when NW only configures low mobility criterion, this use case also exists. | A |
|  |  | Not at cell edge (i.e. when NW only configures not-at-cell-edge, and it is fulfilled) | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. Note: this criterion is always fulfilled, if it is configured. | B |
|  |  | Low mobility and Not at cell edge (i.e. when NW configure both, and both are fulfilled) | UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. | C |
|  | Equal/Lower priority carriers | All scenario | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) | D |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, Equal/Lower priority carriers | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | E |
|  |  | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | F |
|  |  | Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. (Note: for higher/equal/lower carriers) | G |
| For intra-frequency measurement, this part is deduced from RAN2/RAN4 conclusion |  |  |  |  |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) | H |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | I |
|  | N/A | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | J |
|  | N/A | Low mobility and Not at cell edge | No description in the RAN4 reply LS [2]. No conclusion in RAN2. But we can deduce from the conclusion in RAN4#93: UE can stop intra-frequency measurement with a minimum measurement time interval of 1 hour. | K |

For use case A:

- When NW configures low mobility criterion only, and when this criterion is fulfilled; and,

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, (according to following proposal 5, not-at-cell-edge criterion is fulfilled always, if it is configured),

According to the above companies’ inputs, there are different understandings on the measurement relaxation approach for higher priority inter-freq/inter-RAT measurements:

- Option 1: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133.

- Option 2: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour.

- Option 3: Others. Please specify. E.g. Further check with RAN4.

Rapporteur thinks this discussion is similar to the following Q9, and Phase-2 Q9-1. Companies can double check their understanding.

Q3-2-1: From RAN2 point of view, companies are invited to provide their views on which option is the preferred relaxation approach for higher priority inter-freq/inter-RAT measurement for use case A.

- Option 1: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (Some companies understand this is the RAN4 conclusion)

- Option 2: UE can stop high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour.

- Option 3: Others. Please specify. E.g. Further check with RAN4.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option1 | We think higher priority measurement is for load balancing purpose, it should be independent to the mobility state. So, even if network configures low mobility criterion, UE should still perform higher priority with the already relaxed enough measurement requirement even if this criterion is met. |
| Samsung | Option 2 | Even though not-at-cell-edge criterion is not examined in case A, UE definitely is not at cell edge. Given this UE location, since low-mobility criteria is fulfilled, UE can stop RRM measurement. |
| Ericsson | Option 3 | We agree that higher priority measurements are important for load balancing, and that these measurements every minute do not pose a serious power consumption thread, i.e. UE can continue to do them. PS: in our view, in REL-15, the UE is required to do higher priority measurements at least every minute, independent from the coverage conditions, but in bad coverage condition the UE is required to do them more often. But we also agree, that if the UE was mobile and did not find higher priority frequencies, that there is no point in continuing them when UE becomes stationary, i.e. apparently the UE is not in higher priority frequency coverage. However in such case the backup period of 1 hour should apply to ensure some minimum activity. Furthermore we should keep the agreed NW configuration for this case. We think that the above requirements are independent from the coverage conditions (good, bad, not-at-cell-edge), but they should be dependent on the mobility of the UE. |
| CATT | Option 3 | With P5, A = C. We don’t think it makes any difference whether “not-at-cell-edge” criterion is configured or not. UE can stop high priority inter-freq/inter-RAT measurements. No minimum measurement time interval applies in that case. Note: this is our understanding of current RAN4 status, but we also understand that the discussion may not be 100% complete in RAN4 so this status may be updated when RAN2#110-e PS session starts. |
| ZTE | Option 1/2 | Either of the 2 options is fine for us. In use case A, NW only configures the low mobility criterion and UE does not know the not-at-cell-edge criterion. For the power consumption perspective, it is good for UE to stop high priority inter-freq/inter-RAT measurements. For the deployment perspective, because not-at cell-edge criterion is not configured, it is reasonable for UE to support option 1. Though Q9-1 is similar to this question, they do not have the same precondition. In Q9-1, The discussing point is what shall UE perform when UE is configured two triggering criteria only and fulfills both of them. UE knows both of the triggering criteria in Q9-1. But in this question, UE does not know the cell edge criterion. Further discussion can be found in Q9-1. |

For use case C/G/K:

- When NW configures both low mobility and not-at-cell-edge criteria, and when both criteria are fulfilled;

According to the above understanding on RAN4 conclusion, UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. Rapporteur thinks there is no problem for equal/low priority inter-frequency measurement. But for high priority inter-frequency measurement, some companies think the legacy requirement of Thigher_priority_search according to TS 38.133 should not be exceeded.

Q3-2-2: From RAN2 point of view, companies are invited to provide their views on which option is the preferred relaxation approach for higher priority inter-freq/inter-RAT measurement for use case C/G/K.

- Option 1: UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. (Some companies understand this is the RAN4 conclusion)

- Option 2: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133.

- Option 3: Others. Please specify. E.g. Further check with RAN4.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option 2 | We are not sure why higher priority measurement should be further relaxed and why it should be related to the configured criterions. However, if this is the RAN4 conclusion, we can just follow. |
| Samsung | Option 1 | We think this issue is related to cases C/G, not K. |
| Ericsson | Option 3 | Same view as with Q3-2-1, i.e. we see two options: Option 1: UE is required to measure at least every minute higher priority frequencies independent from the coverage conditions (bad, good, not-at-cell-edge) Option 2: UE is required to measure at least every minute higher priority frequencies independent from the coverage conditions highPriorityMeasRelax not set or (highPriorityMeasRelax set and low mobility is not fulfilled) |
| CATT | Option 3 | Note usecase K is for intra-frequency, so the question on higher priority frequencies does not apply here. Again, our understanding of RAN4 agreement is that the minimum measurement time interval only applies to usecase G, not to usecases A/C. This is current RAN4 status and we don't think RAN2 should challenge RAN2 agreements. But same as above, we also understand that the discussion may not be 100% complete in RAN4 so this status may be updated when RAN2#110-e PS session starts. |
| ZTE | Option 1 | After further internal discussion, we think option 1 fulfills RAN4’s conclusion. So we respect to RAN4’s agreement. In our point of view, we prefer to unify the answer of Q3-2-1 and Q3-2-2 at the end of this email discussion. Otherwise,for example, if we select option 2 in both Q3-2-1 and Q-3-2-2, we have to explain the difference between case A and case C/G/K. More instruction may be needed. |

For other uses cases, rapporteur assumes companies (clear majority) agree the above summary. Let’s further confirm it.

Q3-2-3: Companies are invited to provide their views if they do not agree or want to suggest an update on above summary for other uses cases (other than A, C, G, K).

| Company | Comments, if any |
| --- | --- |
| OPPO | It should be noted that in RAN4 LS, it does not clearly say that the higher priority measurement relaxation should be dependent on configured criterions when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. In other words, one can say when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, UE performs higher priority mesaurment relaxation with the same relaxed measurement requirement for low/equal priority frequency. |

Step 3: For higher priority frequency, how to use the highPriorityMeasRelax indication.

In RAN2#108 and #109-e meeting, it was agreed to introduce an indication highPriorityMeasRelax to configure the measurement relaxation for higher priority frequencies. We had some discussion on the UE behaviors on how to apply this parameter for higher priority frequency, but no conclusion was made in RAN2. Thus, there is an Editor’s Note in current TS 38.304: FFS how to configure whether higher priority frequencies can be relaxed, and behaviour of relaxation of higher priority carriers pending RAN4 decisions.

As far as I know, there is no discussion and conclusion in RAN4 about this part. Here, the rapporteur suggests to discuss this issue based on the latest RAN4 progress. According to the above summary, only the use case C, E, F, and G have measurement relaxation. For use case A and B, there is no measurement relaxation. Thus, we should discuss how to use the highPriorityMeasRelax indication for use case C, E, F, and G.

During previous RAN2 discussion, there are several directions of understanding on how to use the indication highPriorityMeasRelax on the relaxation of higher priority carriers.

Q4: Companies are invited to provide views on which option is the preferred UE behavior of measurement relaxation for higher priority carriers:

Option 1: The behaviour for indication highPriorityMeasRelax is also controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True,

  - For use case C, E, F, G, when the criterion is fulfilled, the UE can perform relaxed measurement for higher priority frequency. How to relax measurement for higher priority frequency is up to the conclusion of above Q2 and Q3.

- Otherwise (i.e. highPriorityMeasRelax is not configured, or highPriorityMeasRelax is configured and set to True but the criterion is not fulfilled),

  - The UE will not perform relaxed measurement for higher priority frequency. The measurement requirement for higher priority frequency should follow the legacy defined in 38.133.

Option 2: The behaviour for indication highPriorityMeasRelax is not controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True,

  - For use case C, RAN4 already defined that the UE can perform relaxed measurement for higher priority frequency when criteria of low mobility and not in cell edge are both fulfilled.

  - For use case E, F, G, UE can perform relaxed measurement for higher priority frequency regardless whether the trigger criterion is fulfilled.

- Otherwise,

  - The UE will not perform relaxed measurement for higher priority frequency. The measurement requirement for higher priority frequency should follow the legacy defined in 38.133.

Option 3: The measurement relaxation of higher priority carriers is not configured by the indication highPriorityMeasRelax, i.e. the behavior of relaxation for higher priority frequency is just based on the conclusion of Q2 and Q3. Note: this option is contradict with previous RAN2 agreement made in RAN2#108.

Option 4: Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 3 | If we follow RAN4’s decision and also the table summarized in Q2, everything is clear and there is no need to use highPriorityMeasRelax. |
| OPPO | Option 2/3 | We in general think higher priority frequency measurement relaxation should be independent of the configured criterias, but open to whether we need an additional configuration to enable relaxation or not. |
| Ericsson | Option 1 | In our view we should not re-discuss RAN2 agreement that higher priority frequency relaxation is under NW control. Whether the UE is allowed to further relax higher priority frequency measurements every minute should be under NW control. When allowed by NW configuration, it should only be allowed when the low mobility criterion is fulfilled. |
| Vivo | Option 1 | We think RAN4 didn’t discuss the measurement relaxation for higher priority frequency with the highPriorityMeasRelax indication. This is RAN2 issue. In our understanding, how to relax measurement for higher priority frequencies should be under the network control. Meanwhile, the measurement relaxation should be also controlled by relaxation criteria, i.e. relaxation can be performed only when the corresponding criterion is fulfilled. |
| ZTE | Option 1 | We prefer to choose Option 1. Option 2: In option2, UE can perform the relaxed measurement on the high priority frequencies regardless of its environment. Sometimes,UE may not perform appropriately in some scenarios based on option 2. For example, a scenario in use case F: A UE is at the cell edge and is leaving the current serving cell slowly. If the highPriorityMeasRelax is set to True, the UE will never perform normal measurement on the high priority frequency until the UE camps to another cell and this is not good for the UE performance. If the same scenario happens in option1, because the UE does not fulfill the triggering criteria, normal measurement should be performed on the high priority frequency. Option 3: we can not find enough advantage to select this option and ignore our previous agreements. |
| CATT | Option 4 | It looks like companies’ main concern is about the “large” relaxation (no measurement) in usecase C. From our perspective, the relaxation of hi-prio frequencies when not in good coverage (usecases E, F, G) agreed by RAN4 is quite consistent with legacy and, similarly, should not have any system performance impact. So there is no reason to have it controlled by highPriorityMeasRelax. Hence our compromise is: - If highPriorityMeasRelax is configured - For use case C, UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. - Otherwise - For use case C, The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133 |
| LG | Option 3 | LS from RAN4 already describes how to relax the higher-priority frequencies in each scenario. So we think network configurability is not needed and we can just follow the RAN4’s agreement. If we make some exceptional cases, RAN4 needs additional discussion, but we are at the end of Release 16. |
| Intel | - | We understand that RAN4’s decisions (on how to relax measurements for high priority frequencies) are applicable when “highPriorityMeasRelax is configured and set to True” (as when highPriorityMeasRelax is not configured RAN2 already agreed to the compromise that no measurement relaxation were applicable (i.e. legacy operation is kept)). |
| MediaTek | Option 4 | We should not revert RAN2 agreements, so the measurement relaxation of higher priority frequencies should be controlled by network, i.e. configuration of highPriorityMeasRelax. However, even if highPriorityMeasRelax is configured, it indicates the scanrios allowed by RAN4. That is, relaxation is possible when UE is with low mobility and not at cell edge (cell center or middle area), or when UE is in middle area, but not possible when UE is at cell center but not with low mobility (UE should perform measurement with interval Thigher_priority_search even if highPriorityMeasRelax is configured). |
| Huawei | Option 3 | The parameter seems useless because RAN4 defined the behavior in each of the cases, and we have also pointed this out in previous meetings that it was too early to define a parameter to control behavior that has not yet been defined. |
| Samsung | Option 1 | highPriorityMeasRelax can be used for load balancing. Specifically, NW gives high priority to the under-loaded frequency and do not configure highPriorityMeasRelax. Then UEs do not relax RRM measurement for the frequency, and they are more likely to reselect the cell of that frequency. |
| Apple | Option 1 | Agree with Vivo |

Summary: 11 companies provided views.

5 companies prefer option 1, i.e. the behaviour for indication highPriorityMeasRelax is also controlled by the triggering criteria. Note: Rapporteur interprets Intel’s understanding is something as option 1.

3 companies (plus one prefer both option 2/3) prefer option 3, i.e. the measurement relaxation of higher priority carriers is not configured by the indication highPriorityMeasRelax. They think this indication is useless according to RAN4 conclusion.

1 company prefers option 2 or option 3.

1 company prefers not to change the previous RAN2 agreement. But have a slight different preference on the behavior.

1 company suggests a compromise:

- If highPriorityMeasRelax is configured

- For use case C, UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells.

- Otherwise

- For use case C, The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133

According to companies’ inputs, rapporteur suggests to exclude Option 2 first. Whether to discard our previous agreement (7 vs. 3 or 4) can be further discussed in phase-2 discussion in Q4-2.

Phase-2 discussion:

As rapporteur know, RAN4 didn’t discuss the measurement relaxation for higher priority frequency with the highPriorityMeasRelax indication. Thus, RAN4 did not have the intention to conclude this indication is useless. Companies can further check with their RAN4 colleagues, anyway.

Considering the majority preference in phase 1 discussion (not clear majority, 7 vs. 3 or 4), rapporteur suggests to first respect the previous RAN2 agreement to introduce an indication highPriorityMeasRelax to configure the measurement relaxation for higher priority frequencies. Companies can also express different views on this point.

Based on this RAN2 agreement and RAN4 conclusion, we can further discuss how to use this highPriorityMeasRelax indication. The compromise proposed by Pierre is also listed.

Q4-2: Companies are invited to provide views on which option is the preferred UE behavior of measurement relaxation for higher priority carriers:

Option 1: The behaviour for indication highPriorityMeasRelax is also controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True,

  - the UE can perform relaxed measurement for higher priority frequency. How to relax measurement for higher priority frequency is up to the conclusion of above Q3-2-1/2/3.

- Otherwise (i.e. highPriorityMeasRelax is not configured, or highPriorityMeasRelax is configured and set to True but the criterion is not fulfilled),

  - The UE will not perform relaxed measurement for higher priority frequency. The measurement requirement for higher priority frequency should follow the legacy defined in 38.133.

Option 3: The measurement relaxation of higher priority carriers is not configured by the indication highPriorityMeasRelax, i.e. the behavior of relaxation for higher priority frequency is just based on the conclusion of Q2 and Q3. Note: this option is contradict with previous RAN2 agreement made in RAN2#108.

Option 4: The behaviour for indication highPriorityMeasRelax is also controlled by the triggering criteria i.e.

- If highPriorityMeasRelax is configured and set to True and the criteria are fulfilled,

  - For use case A/C, and G UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells for higher priority frequencies.

- Otherwise (i.e. highPriorityMeasRelax is not configured, or highPriorityMeasRelax is configured and set to True but the criterion is not fulfilled),

  - For use case A/C, and G the UE shall perform measurements for higher priority frequencies with the requirement of Thigher_priority_search according to TS 38.133

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option1 | We can compromise to option 1 without violating previous RAN2 agreement. |
| Samsung | Option 1 | We still stick to Option 1. |
| Ericsson |  | Same view as with Q3-2-1, i.e. we see two options: Option 1: UE is required to measure at least every minute higher priority frequencies independent from the coverage conditions (bad, good, not-at-cell-edge) Option 2: UE is required to measure at least every minute higher priority frequencies independent from the coverage conditions highPriorityMeasRelax not set or (highPriorityMeasRelax set and low mobility is not fulfilled) |
| CATT | Option 4 | Again, we understand the relaxation from every 60s to “no measurement” seems the main concern to address. Note it also includes usecase A, per P5 (We don’t think it makes any difference whether “not-at-cell-edge” criterion is configured or not, so A=C). We also added usecase G which was missing in our first response. Note also that if the outcome of Q3-2-1/2 is Option 2 (no further relaxation beyond Thigher_priority_search) then above Option 1 only impacts use cases E/F, in which case we would be OK with Option 3. See also our comment to option 1. |
| ZTE | Option 1 | We prefer option 1. - This is our original discussed method to solve the higher priority frequency relaxation method. - As the rapporteur mentioned above, option 3 is contradict with our previous agreement in RAN2#108. - Whether higher priority frequencies can be relaxed is up to network configuration. FFS on how the configuration is done. - Besides this, The agreement we made in RAN2#109bis-e is built on the agreement shown above. - Global configuration of relaxation triggers is kept. No change is needed to the current specifications from this aspect. Differentiation of scenarios can be done via the high priority frequency indication framework and no further behaviour is expected to be specified. If we select option 3, i wonder whether we need to re-discuss the per-frequency configuration issue. - From our point of view, we prefer NW can configure not to relax the higher priority frequency when UE in bad coverage (use case, E,F). UE may have more possibility to perform cell reselection in bad coverage than in good coverage. Meanwhile, the indicator is not contradict with RAN4 agreement. Hence, we think using the indicator in use case E,F,G is good for UE performance and network flexibility in some scenarios. |

## Configuration of threshold for relaxation criteria

During the discussion on the CR on 38.304, some companies commented that the threshold for not-at-cell-edge criterion should be smaller than SIntraSearchP/SIntraSearchQ. We will discuss this issue.

In Rel-15, UE is not required to perform intra-frequency measurements if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ. It is reasonable to perform measurement relaxation only when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ. Thus, the measurement relaxation is applicable only when SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ. Based on this understanding, network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ.

On the other hand, we should not restrict the network configuration. When network configures SSearchThresholdP > SIntraSearchP, or SSearchThresholdQ > SIntraSearchQ, there will be no measurement when Srxlev > SIntraSearchP and Squal > SIntraSearchQ according to current mechanism in TS 38.304. So that there is also no measurement relaxation even the criteria are accordingly fulfilled (i.e. Srxlev > SSearchThresholdP and Squal > SSearchThresholdQ).

Rapporteur think the above two understanding can work well without any problem. For inter-frequency measurement, the situation is same as above.

Q5: Companies are invited to provide views on which option is the preferred understanding for the configuration of threshold for not-at-cell-edge:

- Option 1: Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ,

- Option 2: There should be no restriction to network configuration.

- Option 3: others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | Option 2 would need to specify that UE shall prioritize the SIntraSearchP/SIntraSearchQ checking over the SSearchThresholdP/SSearchThresholdQ checking. |
| OPPO | Option 1 | Option 1 results in the following case which should be reasonable: Otherwise, how to perform relaxation when UE does not required to perform neighbor cell measurement. |
| Ericsson | Option 1 | The specification would become more complicated to clarify which requirements apply. But typically SnonIntraSearchP ≤ SIntraSearchP, and SnonIntraSearchQ ≤ SIntraSearchQ, i.e. intra-frequency measurements are triggered before inter-frequency measurements are triggered, which are more power consuming. So in practice the inter-frequency measurement thresholds become the determining factor, i.e. the not-at-cell-edge equals the area between intra- and inter-frequency measurement threshold. We wonder if further clarification is needed for the case when SSearchThresholdP = SIntraSearchP but we can perhaps discuss that later. |
| Vivo | Option 1 | Otherwise, if SSearchThresholdP > SIntraSearchP, or SSearchThresholdQ > SIntraSearchQ, there is only measurement for higher priority frequencies, which have already relaxed requirement of Thigher_priority_search according to TS 38.133 in legacy. |
| ZTE | Option 1 | We share the views explained by other companies. |
| CATT | Option 1 |  |
| LG | Option 1 | We have same understanding with other companies, and RAN4 has same assumption. |
| Intel | Option 1 | We do not have strong view on whether details of the network configuration should be specified or not, although we have slightly preference for option 1 for simplicity. |
| MediaTek | Option 1 | We share the views of above companies |
| Huawei | Option 1 |  |
| Samsung | Option 1 | UE does not perform intra-frequency measurement when Srxlev > SIntraSearchP and Squal > SIntraSearchQ. Therefore, relaxation for intra-freq. measurement is possible only in Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ. So, the relaxation parameters (i.e., SsearchThresholdP and SsearchThresholdQ) should be less than SIntraSearchP and SIntraSearchQ |
| Apple | Option 1 | We share the view of the above companies. |

Summary: 11 companies provided views.

All companies agree that network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ.

Proposal 5: Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ.

Phase-2 discussion: whether agree the above proposal.

Q5-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal

| Company | Comments, if any |
| --- | --- |
| OPPO | Yes |

Q6: If the answer for the above question (Q5) is Option 1, whether need/where to capture this restriction?

- Option 1: Nothing need to capture in the specification. Only need to align the understanding in the chair’s minutes.

- Option 2: Add a note or something in the field description in TS 38.331.

- Option 3: Add a note or something in the description in TS 38.304.

- Option 4: others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option2 /Option 3 |  |
| OPPO | Option2 |  |
| Ericsson | Option 2 | This is a NW configuration requirement which fits in 38.331. |
| vivo | Option 2/3 | Both are OK for us. |
| ZTE | Option 2/3 |  |
| CATT | Option 2/3 |  |
| LG | Option 2 | Field description in 38.331 is enough. |
| Intel | Option 2 or 3 |  |
| MediaTek | Option 2/3 | A note would make thing clear, and it can be in either spec. |
| Huawei | Option 1 | If NW would configure the cell edge criteria to be higher than Ssearch then UE would simply follow Ssearch and hence stop measurements – i.e. it is a useless configuration and specification covers that without clarification. |
| Samsung | Option 2 |  |
| Apple | Option 2 or 3 | Either of option 2 or 3 is fine as long it is captured in a note for clarity purpose. |

Summary: 11 companies provided views.

10 companies agree to capture the restriction as a note or something in the field description in TS 38.331;

- 6 companies of them also agree to capture this restriction in TS 38.304.

1 company thinks nothing need to capture in the specification, only need to align the understanding in the chair’s minutes. They think if NW would configure the cell edge criteria to be higher than Ssearch, it is a useless configuration, and UE would simply follow Ssearch and hence stop measurements.

Based on the inputs from companies, Rapporteur suggests to go for clear majority.

Proposal 6: Capture the following restriction as a note or something in the field description in TS 38.331: Network configuration should ensure that SSearchThresholdP ≤ SIntraSearchP, and SSearchThresholdQ ≤ SIntraSearchQ, as well as, SSearchThresholdP ≤ SnonIntraSearchP, and SSearchThresholdQ ≤ SnonIntraSearchQ.

Phase-2 discussion: whether agree the above proposal.

Q6-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal

| Company | Comments, if any |
| --- | --- |
| OPPO | Yes |
| Ericsson | It should not be a NOTE, i.e. optional but a mandatory requirement. Otherwise the UE behavior would still not be fully specified. |

## Coordination between RAN2 and RAN4

- Capture the RAN4 conclusion on relaxation rule.

In legacy LTE, the relaxed monitoring measurement rule is captured in TS 36.304 as below:

5.2.4.12.0 Relaxed monitoring measurement rules When the UE is required to perform intra-frequency or inter-frequency measurement according to the measurement rules in sub-clause 5.2.4.2 or 5.2.4.2a, the UE may choose not to perform intra-frequency or inter-frequency measurements when: - The relaxed monitoring criterion in sub-clause 5.2.4.12.1 is fulfilled for a period of TSearchDeltaP, and - Less than 24 hours have passed since measurements for cell reselection were last performed, and - The UE has performed intra-frequency or inter-frequency measurements for at least TSearchDeltaP after selecting or reselecting a new cell.

Similar mechanism was agreed in RAN4, but with different values of Time Period. The detailed conclusion, which is highlighted, can be found in the LS [2] from RAN4 as below:

- UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements when: - both low mobility and not-at-cell-edge criteria are fulfilled, and - Less than 1 hour have passed since measurements for cell reselection were last performed.

Rapporteur would like to confirm with companies whether we agree to capture this in RAN2 specification (TS 38.304) as legacy.

Q7: Companies are invited to provide opinions on whether the RAN4 conclusion “Less than 1 hour have passed since measurements for cell reselection were last performed” needs to be captured in TS 38.304.

- Option 1: Yes, it needs to be captured in TS 38.304.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | However, we found it a bit difficult to implement the above RAN4 conclusion in section 5.2.4.9.0, as this section is describing the conditions when UE can “relax” the measurement but not “stop” the measurement. Maybe a new section is required to implement the above RAN4 conclusion. If a new section is created, then section 5.2.4.9.0 needs to remove the case where both “low mobility” and “not at cell edge” criteria are met. |
| OPPO | Option1 | We have the same concern as Panasonic, instead of creating a new section, how about capturing it in the 5.2.4.2 |
| Ericsson | Option 1 | We think this can be captured in 5.2.4.9.0: … - otherwise (i.e. both lowMobilityEvalutation and cellEdgeEvalutation are configured); - if combineRelaxedMeasCondition is configured and set to True, - the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - Less than 1 hour have passed since UE has performed normal intra-frequency or inter-frequency measurements; and, - the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and, - the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled |
| vivo | Option 1 | We agree with Ericsson’s proposal. |
| ZTE | Option 1 | Just as Ericsson mentioned, we also think 5.2.4.9.0 can capture this content. But we have a little concern about how to capture this part. We are not sure whether UE can stop performing intra/inter-frequency and inter-RAT neighbour cell measurement when network only configures both triggering criteria and UE fulfills less than 1 hour has passed since measurements for cell reselection were last performed(combineRelaxedMeasCondition is not configured by network). we may re-discuss this part after RAN4 has made the agreement in Q9. |
| CATT | Option 1 | Similar to legacy, it needs to be captured as a condition to trigger relaxed measurement in TS 38.304. |
| LG | Option 1 | We agree with Ericsson’s proposal, similar to what we did in “Relaxed monitoring” in 36.304 . |
| Intel | Option 1 | RAN2 CR should be updated to capture RAN4 inputs provided in the LS. - For the statement “Less than 1 hour have passed since measurements for cell reselection were last performed”, as explained in Q2, this rule needs to be applied when both lowMobilityEvalutation and cellEdgeEvalutation are configured and checked by UE (i.e. when UE does an “AND” between them, but not when UE does an “OR” between them. 38.304 CR captures that when low mobility criteria is configured or checked, “UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell”. However, we wonder whether this may not be needed based on RAN4’s input. |
| MediaTek | Option 1 | Agree to Ericsson’s proposal. |
| Huawei | Option 1 |  |
| Samsung | Option 1 | We share the view from Ericsson. |
| Apple | Option 1 |  |

Summary: 11 companies provided views.

All companies agree to capture the RAN4 conclusion “Less than 1 hour have passed since measurements for cell reselection were last performed” in TS 38.304.

Proposal 7: Capture in TS 38.304 that “Less than 1 hour have passed since measurements for cell reselection were last performed”. How to capture it can be discussed in CR discussion.

Phase-2 discussion: whether agree the above proposal.

Q7-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal.

| Company | Comments, if any |
| --- | --- |
| OPPO | Yes |

- Capture the RAN4 conclusion on detailed methods for relaxed measurements

There is an Editor’s Note in TS 38.304, “FFS whether detailed methods for relaxed measurements is captured in TS 38.133”. According to the RAN4 reply LS [2], RAN4 has concluded on the detailed methods for measurement relaxation. In legacy LTE, this part has been captured in RAN4 specification.

- For low mobility scenario, RRM measurement relaxation with longer measurement intervals is applied. - The scaling factor of measurement interval is fixed. - For not in cell-edge scenario, RRM measurement relaxation with longer measurement intervals is applied. - The scaling factor of measurement interval is fixed. - UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements when: - both low mobility and not-at-cell-edge criteria are fulfilled, and - Less than 1 hour have passed since measurements for cell reselection were last performed.

Rapporteur would like to confirm with companies whether we agree to capture this part in RAN4 specification (TS 38.133) as legacy. Note: the above highlighted part “Less than 1 hour have passed since measurements for cell reselection were last performed” has been discussed in above Q7.

Q8: Companies are invited to provide opinions on whether the RAN4 conclusion on the detailed methods for measurement relaxation need to be captured in RAN4 specification.

- Option 1: Yes, it should to be captured in RAN4 specification.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |
| OPPO | Option1 |  |
| Ericsson | - | We find the question unclear, i.e. the RRM measurement relaxation, i.e. scaling factor and no requirement to measure are captured in RAN4 specifications. The minimum requirement of 1 hour not. In our view, the yellow part in 38.304 should refer to the relevant section in 38.133: When the UE is required to perform measurements of intra-frequency or NR inter-frequencies or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2, the UE may choose to perform relaxed measurements [FFS according to TS 38.133 [8]] |
| vivo | Option 1 |  |
| ZTE | Option 1 |  |
| CATT | Both | 38.133 captures measurements methods. When there is no measurement, it should go in 38.304 (which is consistent with legacy where when the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the behavior that the UE is not required to perform intra-frequency measurements is already captured in TS 38.304). So: Whenever the UE may choose to not perform any NR inter/intra-frequencies or inter-RAT frequencies measurements, it is captured in 38.304. Whenever the UE may choose to perform relaxed NR inter/intra-frequencies or inter-RAT frequencies measurements, it is captured in 38.133. |
| LG | Option 2 | From Q7, if we agree to capture the agreement in 38.304, we don’t think it needs to be duplicated in RAN4 specification. |
| Intel | - | RAN2 should not discuss whether something is or not captured on RAN4 TS, but instead whether any of these aspects need or not to be captured on RAN2 TS. And if so, we can inform RAN4 to let them know (in case they want to add any TS cross-reference). In our understanding, it can be helpful if RAN2 TS describes the expected relaxation including the applicable RAN4 TS reference (e.g. similarly as Ericsson pointed “UE may choose to perform relaxed measurements according to TS 38.133). |
| MediaTek | Option 1 |  |
| Huawei | both | Agree with CATT. RAN2 specification defines the cases in which UE stops measurements, RAN4 defines the cases in which UE changes rate of measurements (And this is the case in all releases and RATs) |
| Samsung | - | We agree with Ericsson’s view |
| Apple | Both | Agree with CATT view |

Summary: 11 companies provided views.

5 companies agree the detailed methods for measurement relaxation need to be captured in RAN4 specification.

3 companies think RRM measurement relaxation (i.e. scaling factor and no requirement to measure) should be captured in RAN4 specification (TS 38.133). And RAN2 specification TS 38.304 should refer to the relevant section in TS 38.133. 1 company of them thinks it can be helpful if RAN2 TS describes the expected relaxation including the applicable RAN4 TS reference. Rapporteur thinks RAN4 has not captured any conclusion on the detailed relaxation approach in their specification by now. We can update TS 38.304 to refer to the relevant section in TS 38.133 during the meeting.

2 companies think TS 38.133 captures measurement relaxation method (i.e. scaling factor). RAN2 specification should capture the case whenever the UE may not choose to perform any NR inter/intra-frequencies or inter-RAT frequencies measurements. This is consistent with legacy where when the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the behavior that the UE is not required to perform intra-frequency measurements is already captured in TS 38.304. Rapporteur thinks we can further discuss this in the following phase 2.

Based on the inputs from companies, Rapporteur thinks clear majority agree that RRM measurement relaxation method with scaling factor should be captured in RAN4 specification (TS 38.133). And RAN2 specification (TS 38.304) should refer to the relevant section in TS 38.133.

Proposal 8: RAN2 understands that RRM measurement relaxation method with scaling factor should be captured in RAN4 specification (TS 38.133). And RAN2 specification (TS 38.304) should refer to the relevant section in TS 38.133.

Phase-2 discussion: whether agree the above proposal.

Q8-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal

| Company | Comments, if any |
| --- | --- |
| OPPO | Yes |

Phase-2 discussion: where to capture the use case, in which the UE may choose to not perform any NR inter/intra-frequencies or inter-RAT frequencies measurements, i.e. when both low mobility and not-at-cell-edge criteria are fulfilled. As explained by CATT, in legacy, when the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the behavior that the UE is not required to perform intra-frequency measurements is already captured in TS 38.304. Thus, it is consistent to capture this use case in TS 38.304.

Q8-2: Companies are invited to provide their views on where to capture the use case, in which the UE may choose to not perform any NR inter/intra-frequencies or inter-RAT frequencies measurements, i.e. when both low mobility and not-at-cell-edge criteria are fulfilled.

- Option 1: In TS 38.304.

- Option 2: In RAN4 specification.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option1 |  |
| Samsung | Option 1 | We share the view from CATT’ answer in Q8. |
| Ericsson | Option 1 |  |
| CATT | Option 1 | As mentioned by the rapporteur, this is consistent with the legacy 38.304 capturing UE behavior when the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ |
| ZTE | Option 1 |  |

- Measurement relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured

In RAN4 agreed WF [3], it was agreed that:

When network configures the parameters of both low mobility and not-at-cell-edge criteria, - If network indicates option a, (i.e. combineRelaxedMeasCondition is configured and set to TRUE) - the relaxation method corresponding to scenario #3 when both relaxation criteria have been fulfilled - If network indicates option b, (i.e. combineRelaxedMeasCondition is not configured) - the relaxation method corresponding to scenario #1 when only low mobility criteria is fulfilled - the relaxation method corresponding to scenario #2 when only not-at-cell-edge criteria is met. - FFS the relaxation method corresponding to scenario #3 when both relaxation criteria have been fulfilled

It is FFS in RAN4 that the relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured. This issue was also discussed in RAN2 email discussion, but had no conclusion. As far as I know, RAN4 will further discuss this issue to solve this FFS in the coming RAN4 meeting(s). Rapporteur would like to confirm with companies whether we agree to wait for RAN4 conclusion.

Q9: Companies are invited to provide opinions on whether we should to wait for RAN4 further conclusion on measurement relaxation method when both criteria are fulfilled if combineRelaxedMeasCondition is not configured.

- Option 1: Yes, it will be further discussed and concluded in RAN4.

- Option 2: No, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 |  |
| OPPO |  | We would say in this case, the relaxation method can be the same as that when the parameter is configured and set to true. But we can leave this to RAN4. |
| Ericsson | Option 1 | We assumed that Q9 assumes that combineRelaxedMeasCondition is set. It is our understanding that the UE behaviour is different when combineRelaxedMeasCondition is set or not set. Otherwise the configuration parameter would not be needed. |
| vivo | Option 1 | We share the same view as Ericsson. Or we can even agree in RAN2. |
| ZTE | Option 1 |  |
| CATT | Option 1 |  |
| LG | Option 1 | This issue is on the table in RAN4. |
| Intel | Option 1 |  |
| MediaTek | Option 1 |  |
| Huawei | Option 1 |  |
| Samsung | Option 1 | We think it seems desirable that UE does not perform measurement when both criteria are fulfilled if combineRelaxedMeasCondition is not configured. However, this issue is up to decision from RAN4. |

Summary: 11 companies provided views.

10 companies agree and 1 company can accept that it is up to RAN4 to conclude the measurement relaxation method to be adopted when both criteria are fulfilled if combineRelaxedMeasCondition is not configured. 2 companies of them think that the UE behaviour is different when combineRelaxedMeasCondition is set or not set. Otherwise, the configuration parameter would not be needed. While 2 companies of them think UE does not perform measurement when both criteria are fulfilled if combineRelaxedMeasCondition is not configured.

Based on the inputs from companies, Rapporteur suggests to go for clear majority.

Proposal 9X: It is up to RAN4 to conclude the measurement relaxation method to be adopted when both criteria are fulfilled if combineRelaxedMeasCondition is not configured.

Phase-2 discussion: whether agree the above proposal.

Q9-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal

| Company | Comments, if any |
| --- | --- |
| OPPO | Yes |

Phase-2 discussion: from RAN2 point of view, how to perform measurement relaxation when both criteria are fulfilled if “or” is configured?

As several companies think the decision on the measurement relaxation method when both criteria are fulfilled if “or” is configured will impact the usage of parameter combineRelaxedMeasCondition, rapporteur suggests to discuss this issue in phase-2 as below from RAN2 point of view. If there is conclusion, we will inform RAN4 our preference. Otherwise, we will leave this issue to RAN4 to conclude.

Rapporteur Note: I have to admit that some companies may misunderstand the assumption during the above discussion for Q9. According to the current ASN.1 structure, there are following cases for configuration. We are discussing Configuration 4 here.

| Configuration 1 | NW only configures Low mobility criterion |  |
| --- | --- | --- |
| Configuration 2 | NW only configures Not-at-cell-edge criterion |  |
| Configuration 3 | NW configures both criteria; and, combineRelaxedMeasCondition is configured as TRUE | This is “And” case |
| Configuration 4 | NW configures both criteria; and, combineRelaxedMeasCondition is not configured. | This is “Or” case |

In RAN2#109-e meeting, we agreed the LS to RAN4 [4], it is clearly stated that: when “or” is configured,

- Option b: UE uses either low mobility criterion or not-at cell-edge criterion (the selection can be left to UE implementation), i.e. UE can perform relaxation when either low mobility or not-at-cell-edge criterion is fulfilled. And detailed relaxation behaviours are same as case that network only configures the criterion fulfilled (either low mobility or not-at-cell-edge criterion);

As indicated from the reply of above questions by some companies, the RAN2 conclusion to make “AND” / “OR” configurable was intended to allow different UE behaviors in the two cases. Thus, the above RAN2 conclusion should be interpreted as: the UE should not perform “and” behavior when both criteria are fulfilled and “or” is configured. Otherwise, the configuration of parameter combineRelaxedMeasCondition would not be needed. Note: the “and” behavior is: UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements.

At the same time, some other companies have different understanding. They think the UE can stop performing measurement when both criteria are fulfilled and “or” is configured. Companies having such understanding can provide their views on how to use the parameter combineRelaxedMeasCondition.

Q9-2: From RAN2 point of view, companies are invited to provide their views on how to perform measurement relaxation when both criteria are fulfilled if “or” is configured.

- Option 1: the UE should not perform “and” behavior when both criteria are fulfilled and if “or” is configured. Note: the “and” behavior is: UE is not required to perform intra-frequency, inter-frequency and inter-RAT neighbour cell measurements.

- Option 2: UE is not required to perform measurements (i.e. the UE can stop performing measurement) when both criteria are fulfilled and if “or” is configured.

- Option 3: others. Please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option2 | Otherwise we would need to further specific which requirement should be used for this case, if the requirements are different for different criterion. |
| Samsung | Option 2 | Even with Option 2, “and” behavior is clearly different from “or” behavior. When either of two criteria is fulfilled, UE does not perform RRM measurement relaxation with “and” configuration while UE performs RRM measurement relaxation with “or” configuration. |
| Ericsson | Option 3 | In our view this is connected to the transition period that RAN4 is discussing. We prefer to only specify AND behavior and omit the NW configuration. But if we keep the NW configuration, we prefer clear distinctions between AND and OR configuration: AND: only when both criterion are fulfilled UE can relax according to RE-16 relaxation, otherwise legacy requirements apply OR: the UE is allowed to relax according to the criterion that is fulfilled. Because relaxation is optional to the UE, this implies that it is up to UE implementation which relaxation to apply when both are met. If neither of the criterion is met, the legacy requirements apply. |
| CATT | - | We understand this is a RAN4 FFS and, since it is in RAN4 domain, we prefer RAN4 to decide this. |
| ZTE | Option 2 | In our mind, it is benefit to UE power consumption if RAN2 can select option2. In option 2, UE can perform relaxed measurement (longer measurement interval) if NW does not configure combineRelaxedMeasCondition + either of the criteria is fulfilled. In other words, if NW does not configure combineRelaxedMeasCondition, UE have more chances to perform relaxed measurement than combineRelaxedMeasCondition is set as TRUE. We wonder why UE have to keep measuring when it fulfills both triggering criteria and without combineRelaxedMeasCondition. UE does not need to consume more resources to support the option 2 than option 1. In conclusion, UE may have better performance on power consumption in option 2 than that in option 1. And also, we agree current proposal 9. |

- Coordination with RAN4

Once we have the conclusion on the above questions, we should inform RAN4 to instruct them how to make futher discussion, and draft the specification, if we agreed something needs to confirmed/captured in RAN4. Thus, an LS should be sent to RAN4.

Q10. Whether an LS should be sent to RAN4 on the RRM measurement relaxation?

- Option 1: Yes, why;

- Option 2: No, why;

- Option 3: Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| Panasonic | Option 1 | At lease we shall update RAN4 regarding the RAN2 specification change (e.g., removal of highPriorityMeasRelax if RAN2 agree to remove it). We can also inform RAN4 how their agreements/conclusions are implemented in RAN2 specifications. |
| OPPO | Option3 | Depending on the discussion outcome of this email. |
| Ericsson | Option 3 | Depending on the outcome of the discussion about higher priority frequency relaxation in this RAN2 email discussion, we wonder if we need to inform RAN4. But it is premature to decide, i.e. we have to wait for the outcome of this email discussion. We wonder if the transition periods that RAN4 is considering, basically is removing the distinction that RAN2 intended with “OR” / “AND” configuration. We think RAN4 should remove the transition periods, when it blurs the distinction between “OR” and “AND” configuration and the UE behavior. |
| Vivo | Option 1 | Up to the outcome of this email. At least the conclusion for the following issues should be informed to RAN4: - The measurement relaxation for higher priority frequencies. - Conclusion of Q7 Q8, how to capture the RAN4 conclusion. - How to use the highPriorityMeasRelax indication? |
| ZTE | Option 1 | We prefer to send an LS to RAN4. The content depends on the result of this email discussion. |
| CATT | Option 2 | We should send an LS only if we have further question(s) to RAN4, which, at this very late stage we should try to avoid. |
| LG | Option 3 | It seems no need to send an LS to RAN4 yet. After discussion, we could decide whether to send LS. |
| Intel | Option 1 | As already explained (e.g. Q2/Q3), an LS could be helpful to confirm/align our understanding on the RAN4 LS and to also inform them of the agreed CR aiming for alignment across different TS. However, as RAN4’s meeting starts a week earlier than RAN2 one, therefore it would be helpful if companies can coordinate with their RAN4 colleagues internally so they can address corresponding open items. |
| MediaTek | Option 3 | It depends on RAN2’s conclusion on the interpretation about measurement relaxation on higher priority frequencies (mainly Q2 & Q4). With our interpretations, no LS is needed, but if RAN2 makes a different conclusion, or thinks there’s anything unclear, a LS to RAN4 may be needed. |
| Huawei | Option 2 | Agree with CATT. |
| Samsung | Option 3 | Let’s see the result of this email discussion first. |

Summary: 11 companies provided views.

5 companies think whether to send an LS depends on the result of the email discussion.

4 companies think an LS should be sent to RAN4 on the RRM measurement relaxation.

2 companies propose not to send an LS.

Based on the inputs from companies, Rapporteur suggests to go for majority.

Proposal 10: RAN2 to discuss whether an LS to RAN4 is needed based on the outputs of this email discussion.

Phase-2 discussion: whether agree the above proposal.

Q10-1: Companies are invited to provide their views if they do not agree or want to suggest an update on above Proposal

| Company | Comments, if any |
| --- | --- |
| OPPO | We agree to discuss whether to send an LS. |

## Other issues

- To be added

Q11. Any other issues that should be discussed in this email discussion? Please kindly specify, if any.

| Company | Comments |
| --- | --- |

# Summary and Proposals

This contribution summarizes the email discussion [Post109bis-e][940][PowSav] RRM open issues, and achieves the following proposals:

TO BE ADDED.

# References

- R2-2003959 CR for UE Power Saving in NR, vivo

- R4-2005331 Reply LS on RRM relaxation in power saving, RAN4.

- R4-2005330 WF on RRM measurement relaxation, RAN4.

- R2-2002394 LS to RAN4 on RRM relaxation in power saving, RAN2.