3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

# Online, June 01 – 12, 2020

Agenda item:	x.y.z

Source: 	Ericsson

Title: 	Email discussion report: [Post109bis-e][949][POS] Structure of UE-based assistance data (Ericsson)

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

- [Post109bis-e][949][POS] Structure of UE-based assistance data (Ericsson)

Scope: Discuss the structure of UE-based assistance data and determine if changes are needed to minimise repetition of information.

Intended outcome: Report to next meeting

Deadline:  Long

To allow a discussion of the summary as well, companies are asked to provide comments no later than Tuesday May 19th, 10.00 UTC.

From [1], [2] and [3], it was clear that the baseline representation of TRP location, beam info and possibly also RTD is not efficient in all cases. [1] analysed the PER-encoded number of bytes for two example scenarions, considering a matching hierarchy representation (baseline) and a liear list structure respresentation, where the letter was seen as much more efficient. During the email discussion [3], Qualcomm suggested a change to the baseline representation to make the representation more flexible, and in particular better at avoiding repeating UEB AD information that is the same. This one is referred to as a a matching structure with reference option representation.

In this email discussion, we will discuss these different representations to agree on an efficient structure for UEB AD.

# 2.	References

[1] R2-2003144, “Important LPP structural aspects”, Ericsson

[2]	R2-2003983, Email discussion report: [AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

[3]	R2-20xxxxx, "RAN2 Chairman’s Notes", RAN2#109bis-e.

# 3. 	Discussion

In order to analyze PER-encoded ASN.1 examples, we consider the two examples in [1] based on 3GPP IOO:

- IOO, FR1, 1 frequency layer, 12 TRPs, 1 Resource Set of 1 DL-PRS resource per TRP (minimalistic setup)

- IOO, FR2, 2 frequency layers, 12 3-sector nodes (36 TRPs), 1 resource set of 8 DL-PRS resources per TRP

For these two, we encode the NR-DL-PRS-AssistanceDataList and the UEB NR-TRP-LocationInfoLIst based on the matching hierarchies (baseline), matching hierarchy with a reference option and the flat linear list structure (FS – flat structure) respectively. The PER-encoded ASN.1 in bytes becomes:

Table 1. PER-encoded ASN.1 in bytes of the combination of two IEs for two key scenarios.

|  | Example 1, IOO FR1 |  |  | Example 2, IOO FR2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Matching hierarchies (baseline) | Matching hierarchies with reference option | Linear list structure | Matching hierarchies with reference option | Matching hierarchies (baseline) | Linear list structure |
| NR-DL-PRS-AssistanceDataList | 290 | 290 | 314 | 5205 | 5205 | 5350 |
| NR-TRP-LocationInfoLIst | 211 | 212 | 187 | 1189 | 393 | 187 |
| In total | 501 | 502 | 501 | 6394 | 5598 | 5537 |

In the email discussion [2], companies indicated a preference for a matching structure, but there were concerns raised about the overhead in some scenarios. With the matching structure with a reference option, most of the reduction in message size can be achieved, as seen from the table.

If also the beam information is the same for two TRPs at the same location but at different frequency layers, there can be additional savings in size due to the the beam information, as illustrated in [1].

In the following subsections, companies are asked to provide comments to the preferred representation for location, beam and RTD information. Furthermore, a text proposal adopting the proposal by Qualcomm to make the matching structure representation more efficient, can be found in the Annex.

## 3.1	Location information representation

As seen in Table 1, the matching structure with the TRP reference option is almost as efficient as the linear list. Given that companies indicated a preference for a matching structure in the email discussion, the addition of a TRP reference seem to be reasonable compromise. Essentially, it means that the location information of a TRP is either provided with details, or as a reference to another TRP with the same location information.

Companies are asked to provide comments to the suggested addition of a TRP reference to avoid duplication of location information when several TRPs are co-located, typically the case with more than one frequency layer.

| Table 3.1 Optional TRP reference in TRP location information IE to avoid data duplication |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | In our understanding, TRP ID (PRS ID) will be used to associate the same TRP in different frequency layers. In case the ARP of PRS resources are the same across frequency layers, can it be fixed by setting some fields optiional with need OP and add procedure text in the field description? |
| Qualcomm | Only a single "reference point" can be provided in IE NR-TRP-LocationInfo and all antenna reference points can be provided as delta relative to this "reference point". If the deltas are absent, the locations coincide (meaning delta=0; i.e., colocated). Since the TRP-ID is provided in each TRP-LocationInfoElement (and assuming the TRP-ID can include a global ID as currently defined), the NR-TRP-LocationInfo can be sorted as appropriate (i.e., does not need to have the same order as the DL-PRS assistance data). This should give an LMF enough freedom to avoid providing the same coordinates twice. Hence, we cannot see this "duplication issue". |
| Ericsson | In response to HW: TRP/PRS ID is to identify a TRP among the ones configured to allow matching in what is reported. The agreed RAN1 hierarchy is PFL(1..4) – TRP (1..64), which means that there are up to 256 TRPs that can be configured, each with a unique TRP ID 0..255 within the configuration scope between a UE and LMF. In response to QC It is a bit difficult to follow without an explicit example. Please provide fine details about how the IE NR-TRP-LocationInfo would be represented in the FR2 IOO scenario, I also understand that you are now open to use a structure that is not matching the DL-PRS structure, at least to reorder the TRP-entries in the list? |
| vivo | No need to change. It is necessary to support the case that each ARP location is different per each frequency layer. Therefore, we do not need optimize the UEB AD structure and the current way. |
| CATT | It seems the discussion is also related to TRP ID definition which is under another email discussion. If the TRP ID is not defined as (0..255), the optimization to use associated TRP ID may not save much signalling. Whether to optimize the signaling can be decided after we have the clear conclusion on the TRP ID definition. Considering this is the last meeting for ASN.1 frozen, it is better not to change the structure too much. |
| Ericsson | Good point, CATT – we agree A comment to vivo – yes, the case with different ARP locations per frequency layer is already supported. Here we want to avoid an inefficient representation in case they are the same, which seems to be a quite typical case. A comment to Qualcomm This is still broken – without the proposed TRP-ID-reference you still cannot encode the repetition of information properly. Note also that IOO FR2 is based on 3 sectors, so each TRP has three sectors at the same location per frequency layer, and with four frequency layers, that means 12 sectors per TRP that all have the same location. |

## 3.2	Beam information representation

As seen in [1], beam information can combine into a considerable amount of assistance data. If TRPs have the same beam configuration, for example two TRPs at the same location but at different frequency layers, then it is much more efficient to use a reference to the other TRP instead of repeating the beam information.

Companies are asked to provide comments to the suggested addition of a TRP reference to avoid duplication of beam information when several TRPs are co-located, typically the case with more than one frequency layer.

| Table 3.2 Optional TRP reference in TRP beam information IE to avoid data duplication |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | In our understanding, TRP ID (PRS ID) will be used to associate the same TRP in different frequency layers. In case the beam information of PRS resources are the same across frequency layers, can it be fixed by setting some fields optiional with need OP and add procedure text in the field description? |
| Qualcomm | Is the "for example two TRPs at the same location but at different frequency layers" the only case, or are there additional cases? I think the "same location" does not matter, since the Beam Info is provided for a TRP-ID (whose location is provided separately). If the DL-PRS-BeamInfoSet can be applicable to multiple TRPs, the trp-id in NR-DL-PRS-BeamInfoPerTRP could include a list of TRP-IDs (up to 4) for which the DL-PRS-BeamInfoSet is applicable. |
| Ericsson | In response to Qualcomm: It is a bit difficult to follow. There is no NR-DL-PRS-BeamInfoPerTRP in the ASN.1, but I guess you mean NR-DL-PRS-BeamInfo-r16. The trp-id field of that IE is currently a simple attribute, but you mean a SEQUENCE would be better, that could take up to 4 (as meany as the supported frequency layers) TRP-IDs? Yes – would be interesting to see what is more efficient. In RAN2#109bis, you suggested using a TRP ID reference to reduce overhead, and that is what we have provided encoding details for (for locationInfo). Why a new proposal again? The reference you suggested at the previous meeting and that we currently have in the text proposal in the Annex seems to address the overhead satisfactory. |
| CATT | Perhaps we needn’t spend much effort on multiple frequencies case, since at most, it is repeated 3 more times. At this stage, we’d better make the mechanism workable. |
| Ericsson | To repeat information 3 times can be critical, for example if the information is broadcasted it may prevent broadcasting due to its size. We believe that workable and efficient goes hand in hand here. |

## 3.3	RTD information representation

Depending on how TRPs are designed and deployed, it is possible that two TRPs have the same RTD configuration, for example two TRPs at the same location but operating at different frequency layers. In such cases, it is much more efficient to use a reference to the other TRP instead of repeating the RTD information.

Companies are asked to provide comments to the suggested addition of a TRP reference to avoid duplication of RTD information when several TRPs are co-located, typically the case with more than one frequency layer.

| Table 3.2 Optional TRP reference in TRP RTD information IE to avoid data duplication |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | In our understanding, TRP ID (PRS ID) will be used to associate the same TRP in different frequency layers. In case the RTD of PRS resources are the same across frequency layers, can it be fixed by setting some fields optiional with need OP and add procedure text in the field description? |
| Qualcomm | Its hard to believe that "two TRPs have the same RTD configuration". E.g., not even GPS SVs with a couple of atomic clocks on board of a satellite have the same clock-offsets. There may also be a "random" component on the RTDs which may affect the quality. (We also cannot see the principle difference to the DL-PRS assistance data (e.g., SFN0-Offset, expectedRSTD)). |
| Ericsson | We agree – it seems unlikely that two frequency layers would have the same RTD. One possibility could be if two or more frequency layers are within the same frequency band and the corresponding DL-PRS can be generated by the same transmitter chain and antenna panel, so I guess it can depend on the configuration. |
| CATT | It seems that this is not typical case. So we prefer not to optimize the signalling. |

## 3.4	Text Proposal

The optional TRP reference has been introduced in a text proposal in the Annex.

Companies are asked to provide comments to the text proposal with the suggested addition of a TRP reference to avoid duplication of assistance data when several TRPs are co-located, typically the case with more than one frequency layer.

| Table 3.4 Text proposal with an optional TRP reference in the Annex |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | This TP needs improvement. For example, dl-prs-BeamInfoSet-r16 DL-PRS-BeamInfoSet-r16, and subframeOffset-r16 INTEGER (0..1966079), are still mandatory, which means no overhead reduction could be achieved. We failed to see any need to introduced associatedTRP-ID-r16, as it is it can be simply resolved by setting trp-id-r16 to the desired value. Note that trp-id-r16 is the TRP identify defined across positioning frequency layers within the UE context, and we fail to see the need to assign different trp-id-r16 to the same TRP in different frequency layers. If it is related to different ARFCN number in the TRP-ID structure, we suggest to remove it as this field is not needed at all, since pointA of PRS is already indicated elsewhere. This is related to another email thread on TRP-ID. |
| Qualcomm | It seems the TP assumes one additional frequency layer only? I.e., shouldn’t the proposed associatedTRP-ID-r16 provide a list of up to 3 additional TRP-IDs (given the discsussion above)? For the IE NR-TRP-LocationInfo and NR-RTD-Info we cannot see the need for any associatedTRP-ID-r16 (see 3.1 and 3.3 above, respectively). For the beam info, it can be considered, but we believe the case discussed is not the most typical case. |
| Ericsson | It depends on the point of view. In RAN2#109bis, there was a majority of companies in favor of a matching structure between DL-PRS AD and TRP-LocationInfo, meaning that the two structures are matching in number of frequency layers, TRPs per FL, Res sets per TRP and res per res sets, as well as ordering the TRPs in the same order in both these structures. So if TRP identity 0, 64, 128, 192 points at the same TRP but at different frequency layers, TRP ID 64, 128 and 192 will all have an associatedTRP-ID equal to TRP ID 0. Furthermore, we need to further discuss the locationInfo, preferably by analyzing the FR2 IOO example, and the RTD, which could also have the optional field associatedTRP-ID included if we think the slight overhead is worth the benefit arising if indeed we will have cases where RTDs across frequency layers will be the same in practice. |
| CATT | For the TP itself, Huawei’s comment is valuable. Furthermore, since the TRP ID definition is still under discussion, the associated TRP ID should not be defined as TRP ID, perhaps a TRP index instead if needed. |
| Ericsson | The TP has been updated based on - comments from Huawei to ensure that the fields that risk to be duplicated are OPTIONAL - comments from CATT to use INTEGER (0..255) as TRP reference, at least until the TRP-ID discussion has converged. |

# 4. 	Conclusion

# Annex 1, Text proposal to 3GPP TS 37.355 for efficient representation of UEB AD

[…]

### 6.4.3	Common NR Positioning Information Elements

#### 6.4.3.1	Common NR assistance data Information Elements

[…]

#### –	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

associatedTRP-ID-r16			INTEGER (0..255),								OPTIONAL,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - trp-id: This field provides an identity of the TRP. - associatedTRP-ID: This field provides a reference to another TRP with the same location information. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	ReferencePoint

The IE ReferencePoint provides a well defined location relative to which other locations may be defined.

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

RelativeLocation-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255),

verticalConfidence-r15					INTEGER (0..100)

}

-- ASN1STOP

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 meter, and 1 meters, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [meters]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-DL-PRS-BeamInfoPerFreqLayer-r16

NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..64)) OF NR-DL-PRS-BeamInfo-r16

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

trp-id-r16							TRP-ID-r16,

associatedTRP-ID-r16				INTEGER (0..255)					OPTIONAL,

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16				OPTIONAL,

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..2)) OF DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..64)) OF DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

trp-id This field provides an identity of the TRP.

associatedTRP-ID This field provides a reference to another TRP with the same beam information.

lcs-gcs-translation-parameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field is absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-prs-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 0.1 degrees; range 0 to 180 degrees.

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

beta This field specifies the downtilts angle β for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs.

-- ASN1START

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

associatedRefTRP-ID-r16			INTEGER (0..255)						OPTIONAL,

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

}																		OPTIONAL,

rtd-RefQuality-r16				NR-TimingMeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..4)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..63)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

trp-id-r16					TRP-ID-r16,

associatedTRP-ID-r16		INTEGER (0..255)					OPTIONAL,

subframeOffset-r16			INTEGER (0..1966079) 				OPTIONAL,

rtd-Quality-r16				NR-TimingMeasQuality-r16			OPTIONAL,

...

}

-- ASN1STOP

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - ref-trp-id: This field specifies the identity of the reference TRP. - associatedRefTRP-ID: This field provides a reference to another TRP with the same reference RTD information. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

trp-id-r16 This fields provides the identity of the TRP for which the RTD-InfoElement is applicable.

associatedTRP-ID This field provides a reference to another TRP with the same RTD information.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [41]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.