3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

Electronic meeting, 1 June – 10 June 2020

Source:	CATT

Title:	[Post109bis-e][950][POS] Remaining issues on broadcast (CATT)

Agenda Item:	6.8.2.6

Document for:	Discussion and Decision

# Introduction

During RAN2#109bis-e, the remaining issues on broadcast will be concluded with the following email discussion:

- [Post109bis-e][950][POS] Remaining issues on broadcast (CATT)

Scope: Discuss the proposals from R2-2003607.

Intended outcome: Summary for next meeting

Deadline:  Long

In this discussion, we intend to progress the discussion on top of the summary for Broadcast assistance data which are not concluded in RAN2#109bis-e yet and the LS from RAN3 in RAN2#110-e:

- support the UE including GNSS ID/SBAS ID in the on-demand SI request

- support area scope

- whether a separate positioning system information area ID or reuse the existing area ID

- whether a cell list in NRPPa meta data

- whether a unicast tag for positioning si-BroadcastStatus

The left issues are categorized into

A)	A potential easy agreement

B)	Further discussion

# Discussion

The NOTE’s in this section are Rapporteur’s comments/understanding/questions.

## Potential easy agreement

## GNSS ID/SBAS ID in on-demand request

Issue#1: Support the UE including GNSS ID/SBAS ID in CONNECTED mode when on-demand SI request. R2-2002916

Observation 1: The on-demand SI request follows the same mechanism which is the decision in main session:

- The on-demand SI request message sent by the UE in RRC_CONNECTED is per SIB. A single message can request multiple SIBs.

Observation 2: GNSS id and SBAS id are optionally broadcasted together with posSIB type(as below), and the gNB knows which GNSS id or SBAS id (if available) a posSIB belongs to.

PosSIB-Type-r16 ::= SEQUENCE {

encrypted-r16		ENUMERATED { true }				OPTIONAL,		-- Need R

gnss-id-r16			GNSS-ID-r16						OPTIONAL,		-- Need R

sbas-id-r16			SBAS-ID-r16						OPTIONAL,		-- Need R

posSibType-r16		ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-

...

Proposal 1 in R2-2002916: Beside pos SIB type, the UE can optionally include GNSS id or SBAS id in on-demand SI request (per SIB) to assistance data in CONNECTED mode.

- Note 1: Some company was concerned if there would be an additional gNB requirement to maintain different versions of the SIB for different GNSSs/SBASs. This proposal is the request from UE perspective without additional gNB requirement to legacy behavior. If the gNB knows which GNSS/SBAS id a posSIB belong to according to the data via NRPPa, it will be shown in the PosSIB-Type.

- Note 2: Some company shows the concern of the Msg-1 based. This request is based on the RRC message for the on-demand SI in RRC CONNNECTED mode.

- Note 3: This proposal is the request enhancement when UE is sending the on-demand SI in RRC CONNECTED mode. There is no additional requirement to gNB. This enhancement saves the signaling overhead because the request from UE is more specific.

- Note 4: There was an email discussion and vote on this issue already, totally 12 companies providing answers. 8 companies support yes, 3 companies didn’t support, and 1 company not sure. Companies who had shown concern previously can go ahead share your concern and comments if still have. Companies who had supported the proposal will still support this proposal as default.

| Yes | Ericsson | Qualcomm | CATT | Huawei | LG | u-blox | ESA | Apple |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Nokia | Intel | Samsung |  |  |  |  |  |
| Not sure | Lenovo |  |  |  |  |  |  |  |

Proposal 1 by Rap: Agree with GNSS ID/SBAS ID in on-demand SI request (per SIB) to assistance data in CONNECTED mode and merge it into running CR 38.331 for ASN.1 check.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | Agree with most of the companies, this enhancement saves the signalling overhead. |
| Huawei | Yes | If gNB provides multiple posSIBs with same posSibType but different GNSS type, UE needs to inform gNB which GNSS type it really needs. |
| OPPO | Yes | GNSS ID/SBAS ID helps UE to request the on-demand SI precisely. |
| CATT | Yes | Support, because: - No additional gNB requirement to maintain different versions of the SIB for different GNSSs/SBASs. - Power saving from UE side. - Signalling overhead saving. |

## On support area Scope

Issue#2: The discussion on LS from RAN3 on support for Area Scope in Assistance Information metadata.

There is no agreement about the area ID and area scope in RAN3 yet. So RAN3 would like to collect the feedback on the two IEs from RAN2 [5].

RAN2 have agreed that area scope of a posSIB and the corresponding SI validity area are part of the NRPPa metadata in RAN2# 108 meeting as blow [2]:

Agreements:

2	The area scope mechanism in RRC is copied into the scheduling information for posSIBs.  FFS if there is a separate area ID for positioning.

3	The area scope of a posSIB and the corresponding SI validity area are part of the NRPPa metadata

Note 1: Only the LMF can decide whether a specific posSIB is cell specific or area specific.

Note 2: Area scope in NRPPa meta data can work together with area ID which was discussed in issue#3. Even if the area ID is not the sepearte ID which is decided by RAN O&M,  the area scope still works with the area ID for the validity of posSIB.

Proposal 2 by Rap: Agree to support on Area Scope in Assistance Information metadata and send the RAN2 agreement to RAN3.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | As we already support area scope mechanism for MDT in RRC, it is easy to support for posSIBs. |
| Huawei | Yes | LMF needs to inform gNB the area scope of one posSIB to facilitate gNB decide the content of PosSI-SchedulingInfoList. |
| OPPO | Yes |  |
| CATT | Yes | LMF decides the positioning assistance data cell specific or area specific. Then gNB can broadcast the area scope with area ID to UE for the validity of posSIB. |

## A separate positioning system information area ID

Issue#3: Whether a separate positioning system information area ID or reuse the existing area ID. R2-2002916

The positioning system information area ID is the enhancement of broadcast assistant data (e.g. the A-GNSS data).

- Note 1: The issue was discussed by email discussion in #109-e meeting but was postponed because of limited online time [1].

Yes reasons:

- The existed systemInformationAreaID is not an area suitable for RAT independent positioning,

- The benefits of power saving,

- More flexibility in handling HA-GNSS services,

- No big overhead is introduced

No reasons:

- It is not meant as a concept to manage the validity area of a SIB based on the contents of the SIB.

- System information area management is a RAN function

- Having per-SI message system information area ID increases the size of SIB1 (mind that each area ID is 24 bits)

Note 2: The separate area ID is the enhancement of broadcast assistant data. But this enhancement can be postponed to Rel-17 in order to make a way forward in Rel-16.

Proposal 3 by Rap: Postpone the separate positioning system information area ID to Rel-17.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | We can postpone it. |
| Huawei | yes | Agree to postpone the separate positioning area ID to Rel-17 with considering limited time and its large size consuming on SIB1. |
| OPPO | Yes, and see comments | By assuming the result of this proposal, that positioning SIB would reuse the same area ID together with other SIB as defined in R15. We are generally fine to discuss it in Rel-17 considering the limited time. |
| CATT | yes | The enhancement is good for UE powering saving in RAT-independent positioning. But we can postpone it to Rel-17 considering the time budget, reusing the existed area ID. |

## Need further discussion

## On support a cell list in NRPPa

Issue#4: The discussion on LS from RAN3 on adding a cell list in NRPPa associated with a particular posSIB.

The LS (R2-2004333/ R3-202749) [5] mentioned that: a proposal discussion for adding a cell list in NRPPa associated with a particular posSIB (independently of the Area Scope discussion) to enable the possibility to have different content in different cells of the gNB, but no consensus was reached in RAN3.

The proponent company provides the argumentation: associate NRPPa broadcast assistance data with a “target cell” by using a cell list.

Certain types of broadcast assistance can apply to an area (in fact that is when you can use area scope), while others may be cell specific.

Proposal 4 by Rap: Agree to support a cell list in NRPPa associated with a particular posSIB.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | agreed |
| Huawei |  | The discussion should continue in RAN3 and RAN2 cannot provide |
| OPPO |  | We prefer to discuss it after RAN3 reach consensus. |
| CATT | Yes | Support a cell list. The more specific assistance data from LMF, the more efficient broadcasting assistance data. |

## On support unicast tag

Issue#5: On the need of unicast tag for positioning si-BroadcastStatus. R2-2003810

The issue was discussed online in #109-e meeting but was postponed because of the limited online time. The proponent company provides the below argumentation: For posSIB delivery of positioning system information, introduce posSI-BroadcastStatus with enum {broadcasting, notBroadcasting, unicast}.

The main reason for this is to implement the below agreement made in RAN2-107.

- There is no requirement for a deployment to broadcast AD

Thus, there is one of the ways for UE to get the AD from gNB. Certain AD may be delivered by means of RRC dedicatedPosSysInfoDelivery; the NW will unicast the AD and this differs from the tag notBroadcasting: which implies:

- The UE shall not request it (neither by Msg3 nor Msg1), since there will be no broadcast based on such request;

- It should not be included in the SI scheduling. It would otherwise impact the scheduling of other SI messages that are actually broadcasted.

Note 1: Companies who had shown concern previously can go ahead share your concern and comments if still have. The others who have supported the proposal previously will be considered to support as default.

Proposal 5 by Rap: Discuss to support the need of unicast tag for positioning si-BroadcastStatus.

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes | It has benefit to optimise signalling and make it flexible for gNB deployment. |
| Huawei, HiSilicon | No | We acknowledge there may be some limited gains for doing this. But at this stage, we may not be able to finish all the necessary specs. we propose to postpone it to the next release. |
| OPPO | No | The “notBroadcasting” already allows NW to either Broadcast or to deliver dedicated SI. We do not see the necessity for “unicast” tag. |
| CATT | No | The reasons: 1.The info which gNB doesn’t plan to broadcast should not be shown in SIB1. 2.The use case what unicast tag is working for doesn’t always happen, but the impact on specifications is much for the last meeting. So we suggest postponing it to Rel-17 for further discussion. |

# Conclusion

In this contribution, the left issues were list and text proposals are shown:

Agreements proposed to be agreed in this meeting (from all sub-topics)

Open items proposed to be further discussed in this meeting (from all sub-topics)

Disc 1:

# References

[1]. R2-2001241	Summary of [108#88][NR/Rel-16] Remaining issues on broadcast assistance data	Ericsson

[2]. R2-2000009 Report of 3GPP TSG RAN2#108 meeting, Reno, USA  ETSI MCC

[3]. R2-2002916	Summary of the agreement and left issues on Broadcast Assistance Data	CATT

[4]. R2-2003810	On the need of unicast tag for positioning si-BroadcastStatus	    Ericsson

[5]. R2-2004333 LS on support for Area Scope in Assistance Information metadata       Ericsson

[6]. R2-2001950 [AT109e][621][POS] Open issues on broadcast assistance data        CATT