3GPP TSG-RAN WG2 #110-e	R2-2004403

E-meeting, June 2020

Agenda Item:	6.4.2.2

Source:	OPPO

Title:	Summary of sidelink capability related contributions

Document for:	Discussion, Decision

# Introduction

This is for the summary of sidelink capability related contributions.

# Discussion

This summary is to collect the left issues on SL capability based on the contribution submitted to RAN2#109bis-e meeting.

## Issue-1: Left issues on SL capability report via Uu-RRC

Q1: For peer UE SL capability report on Uu-RRC

| Tdoc | Company |  |
| --- | --- | --- |
| R2-2004597 | Nokia | Proposal 1: The initiating UE should not report its peer UE’s capability instead of its own to the network, since the network would interpret this erroneously as the TX UE capability. Proposal 2: The network should ensure that the PC5 configuration for sidelink unicast can be applied by both Tx UE and Rx UE. Proposal 3: The initiating UE (TX UE) that wants to establish unicast sideling shall report its peer UE capability as well as its own UE capability to the network. Proposal 4: The initiating UE (TX UE) should embed the peer UE capability into a container (carrying RX UE capability received via UECapabilityInformationSidelink) of a single message that is sent from the initiating UE to the network.Proposal 5: It’s up to the serving network to configure a PC5 unicast connection that both UEs can fulfil. Proposal 5: The UECapabilityInformation message is modified to carry the peer UE capability in a container that is uniquely associated with the peer UE identifier. Proposal 6: The network configuring the sidelink (i.e. the network of the initiating UE) creates a new cutset out of the two individual UE capabilities (TX-UE capability and RX-UE capability) and use this capability cutset for configuration of the sidelink. |

Considering the related discussion in [955], i.e., Q2.3-3, the related issues have been covered, so no need for further proposal.

Q2: For mixed LTE-PC5 and NR-PC5 band combination

| Tdoc | Company |  |
| --- | --- | --- |
| R2-2005296 | vivo | Proposal 1. Confirm the working assumption from RAN2#109bis-e, i.e. The band combination of mixed LTE-PC5 and NR-PC5 will be reported, in addition to pure LTE-PC5 band combination and NR-PC5 band combination. |

Considering the related RAN4 conclusion, it seems straightforward to confirm the WA

- Band combinations for each 2nd priority cases will be specified in Rel-16

  - For scenario of NR Uu + LTE SL, n71 (NR Uu) + B47 (LTE SL) is specified in Rel-16

  - For scenario of NR Uu + NR SL + LTE SL, n71 (NR Uu) + n47 (NR SL) +B47 (LTE SL) is specified in Rel-16

NR SL and LTE SL operate with TDM mode in the ITS band

- [Easy] RAN2 confirm the WA from RAN2#109bis-e, i.e. the band combination of mixed LTE-PC5 and NR-PC5 will be reported, in addition to pure LTE-PC5 band combination and NR-PC5 band combination.

If this WA is confirmed, as included in [109e#20], one left issue is as follows

Then w.r.t the concrete form of the reporting, one straightforward way is to reuse the way we adopted for pure LTE-PC5 (or NR-PC5) band combination, as agreed in R2#109e, i.e., to define separate list of band combination for mixed LTE-PC5 and NR-PC5 (separate from the band combination list of LTE-PC5 only, and band combination list of NR-PC5 only). Then for each Uu band combination, indicate the supported company LTE-PC5 and NR-PC5 band combination via bit-map;

2: In Uu-RRC, introduce supported LTE / NR PC5 band combination(s) for each NR Uu band combination by referring to a list of PC6 band combinations.

Q2.1-4: If yes to Q2.1-3, do you agree to introduce supported mixed LTE-PC5 and NR-PC5 band combination(s) for each Uu band combination by referring to a list of PC5 band combinations.

So the left issue needs to be confirmed as well.

- [FFS] If the WA is confirmed, RAN2 discuss to introduce supported mixed LTE-PC5 and NR-PC5 band combination(s) for each Uu band combination by referring to a list of PC5 band combinations.

Q3: For the NR-NR DC controlled sidelink

| Tdoc | Company |  |
| --- | --- | --- |
| R2-2005296 | vivo | Proposal 2.1: Rectify RAN2#109e agreement, i.e. "In Uu-RRC, when rat-Type=nr, UE reports NR-PC5 capability for NR standalone / NR-DC controlled NR-PC5 via UE-NR-Capability." Proposal 2.2 by rectifying the RAN2#109e agreement, RAN2 makes a new proposal as follows: “In Uu-RRC, not introduce supported for NR/LTE PC5 band combination(s) per Uu band combination for all MR-DC cases.” |

RAN2 need to further discuss whether to support sidelink BC report for (NG)EN/NE-DC and NR-DC. Rapporteur understand that:

- Since from RAN2 perspective, the functionality specified in R16 can already support MN-controlled sidelink since R16, to follow the release independent spirit of band combination, it is straightforward to define the sidelink band combination for MR-DC scenario (including NR-DC), limited to MN-controlled case.

- On the other hand, there is some further difference between (NG)EN/NE-DC and NR-DC

  - For the former one, not specifying SL BC in R16 means that we do not need to work on the UE-MRDC-Capability container;

  - For the latter one, not specifying SL BC in R16 does not remove our work on the UE-NR-Capability, which is anyway needed for NR SA scenario (please note that the difference between a NR BC supporting CA only and supporting CA + DC is just the presence of ca-ParametersNRDC).

So RAN2 need to further discuss the support of SL capability report for MR-DC scenario.

- [FFS] RAN2 further discuss the support of NR/LTE PC5 band combination(s) per Uu band combination for (NG)EN/NE/NR-DC scenario.

## Issue-2: Left issues on L2 feature list

Q4: For the L2 feature list

| Tdoc | Company |  |
| --- | --- | --- |
| R2-2004761 | Apple | Proposal 1 “Range-based HARQ feedback in Sidelink groupcast” is an optional UE capability. Proposal 2 No need to introduce capability signaling for SDAP in-sequence delivery in Uu or PC5. |
| R2-2005547 | Samsung | Proposal 1. maxNumberROHC-ContextSessions for sidelink should be contained in IE of UE capability PC5-RRC and Uu-RRC. Proposal 2. Supported ROHC profile can be SL capability for PC5-RRC. |

Considering the related discussion in [955], i.e., Q2.2-1, Q2.1-1, the related issues have been covered, so no need for further proposal.

# Conclusion

We have the following proposals:

Proposal 1	[Easy] RAN2 confirm the WA from RAN2#109bis-e, i.e. the band combination of mixed LTE-PC5 and NR-PC5 will be reported, in addition to pure LTE-PC5 band combination and NR-PC5 band combination.

Proposal 2	[FFS] If the WA is confirmed, RAN2 discuss to introduce supported mixed LTE-PC5 and NR-PC5 band combination(s) for each Uu band combination by referring to a list of PC5 band combinations.

Proposal 3	[FFS] RAN2 further discuss the support of NR/LTE PC5 band combination(s) per Uu band combination for (NG)EN/NE/NR-DC scenario.

# Reference

- R2-2004597	On the peer UE capability transfer in unicast sidelink	Nokia, Nokia Shanghai Bell	discussion	Rel-16	5G_V2X_NRSL-Core

- R2-2004761	Discussion on SL Capability	Apple	discussion	Rel-16	5G_V2X_NRSL-Core

- R2-2005296	Discussion on SL UE capability details	vivo	discussion

- R2-2005547	Discussion for Sidelink UE Capability	Samsung Electronics Co., Ltd	discussion	Rel-16	38.306	5G_V2X_NRSL-Core