3GPP TSG-RAN WG2 Meeting #110					R2-20xxxxx

Electronic, 1 June – 12 June 2020

Agenda Item:		6.12.1	General

Source:			Huawei

Title:				Summary of [Post109bis-e][960][MDTSON] Open issues on ASN1 RIL for MDT and SON

WI code(s):			NR_SON_MDT-Core

Document for:		Discussion and Decision

## 1	Introduction

This paper is to provide a summary of MDT ASN1 RILs.

- [Post109bis-e][960][MDTSON] Open issues on ASN1 RIL for MDT and SON (Huawei)

Scope: Continue the discussion on L2M open issues based on R2-2003486

Intended outcome: Summary with the following sets which should be identified

Try to change the issues marked as Cat b1 into proposals marked Cat b

Deadline: Next Meeting, ASN.1 review schedule

The input for ASN1 RIL discussion is [1]. To be more specific, MDT RILs_38.331 v64_v4.xlsx is the input for this email discussion, because 36.331 RIL XLS only has one RIL and that RIL has been agreed at RAN2-109b-e meeting.

## 2	Discussions

### 2.1	Guideline for RIL discussion

As discussed and agreed at RAN2-109b-e meeting, in XLS files for 38.331 and 36.331, cat a and b RILs were agreed, and cat c RILs were rejected. So cat b1 RILs are to be mainly discussed in this email discussion.

Section 2.2 is about cat b1 RILs with the column “Relations with others” filled, i.e. these RILs may be covered by previous or ongoing discussions.

Section 2.3 is for cat b1 RILs other than section 2.2, i.e. with the column “Relations with others” empty, i.e. these RILs may not be covered by any discussions yet.

Section 2.4 is for RILs which may impact two or more WIs, i.e. RILs in label 2, 3, 4, 5, and 6.

For some RILs, tables are put in order to collect companies’ opinions. If any RIL is missing, please companies indicate it timely.

### 2.2	Cat b1 RILs related with others

Filter condtions in XLS:

- In label “1_MDT, NR_SON_MDT-Core”

- The Column M Category = b1

- The Column N Relations with others = not empty

There are in total 58 RILs in the XLS.

#### 2.2.1	RILs (Related to N006)

1 RIL.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| N008 | 75 | In section 2.3, two RILs may be related to N006: - N006, line73 - N009, line77 - N010, line79 Suggest to discuss all 4 RILs in the table below. |  |

Issue: For N008, line75, N006, line73, N009, line77, and N010, line79, the details are as below.

N008:

| Description | Proposed Change |
| --- | --- |
| See above –there could be an IE for these (see N006) | See N006 for the IE – this part would become RRCReconfigurationComplete-v16xy-IEs ::= SEQUENCE { mdt-Availability-r16 MDT-Availability-r16 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |

N006:

| Description | Proposed Change |
| --- | --- |
| These fields seem to be in ReestablishmentComplete, ReconfigurationComplete, SetupComplete and ResumeComplete. Since all of them relate to MDT, might be useful to group them into one IE and assign one optionality bit to that. While that does consume 1 more bit in case a field is present, in many cases the information is not available and this would save some bits from all those cases. | Define new IE MDT-Availability-r16 to contain these flags as shown below: MDT-Availability-r16 ::= SEQUENCE { logMeasAvailable-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableBT-r16 ENUMERATED {true} OPTIONAL, logMeasAvailableWLAN-r16 ENUMERATED {true} OPTIONAL, connEstFailInfoAvailable-r16 ENUMERATED {true} OPTIONAL, rlf-InfoAvailable-r16 ENUMERATED {true} OPTIONAL } and use that to refer to these fields. If It could even be extendible to retain the MDT information all together for future use if the overhead is seen acceptable. For example: ReestablishmentComplete would become as shown below, which is significantly easier to read and better indicates the usage: RRCReestablishmentComplete-v16xy-IEs ::= SEQUENCE { mdt-Availability-r16 MDT-Availability-r16 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |

N009:

| Description | Proposed Change |
| --- | --- |
| See N006 – could create an IE for the MDT parts. | See N006 – this part would become as shown below: RRCResumeComplete-v16xy-IEs ::= SEQUENCE { idleMeasAvailable-r16 ENUMERATED {true} OPTIONAL, measResultIdleEUTRA-r16 MeasResultIdleEUTRA-r16 OPTIONAL, measResultIdleNR-r16 MeasResultIdleNR-r16 OPTIONAL, scg-Response CHOICE { nr-SCG-Response OCTET STRING (CONTAINING RRCReconfigurationComplete), eutra-SCG-Response OCTET STRING } OPTIONAL, mdt-Availability-r16 MDT-Availability-r16 OPTIONAL, mobilityHistoryAvail-r16 ENUMERATED {true} OPTIONAL, mobilityState-r16 ENUMERATED {normal, medium, high, spare} OPTIONAL, nonCriticalExtension SEQUENCE{} OPTIONAL } |

N010:

| Description | Proposed Change |
| --- | --- |
| Could create an IE for the MDT-specific indications (see N006 for details). | See N006 – this part would become as shown below: RRCSetupComplete-v16xy-IEs ::= SEQUENCE { iab-NodeIndication-r16 ENUMERATED {true} OPTIONAL, idleMeasAvailable-r16 ENUMERATED {true} OPTIONAL, mdt-Availability-r16 MDT-Availability-r16 OPTIONAL, mobilityHistoryAvail-r16 ENUMERATED {true} OPTIONAL, mobilityState-r16 ENUMERATED {normal, medium, high, spare} OPTIONAL, nonCriticalExtension SEQUENCE{} OPTIONAL } |

For the above issues and proposed changes, please companies provide comments if any.

Note1: The column Yes/No means do companies agree with the issue and solution/proposed changes for a RIL, and it also applies to all following tables.

Note2: For some RILs that need comments in tables, the content in the column Comments in XLS is also provided for reference.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | It seems beneficial in signalling overhead aspect. |

#### 2.2.2	RILs (Related to offline 801)

2 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| E026 | 88 | RAN2-109b-e agreed: 2: For capturing the measurement quantities included in the logged MDT, re-use the existing IE MeasQuantityResults and if required, add a text in the field description of the IE MeasQuantityResults, e.g. sinr is not included when it is used for MDT. Suggest to put PropReject. | [Samsung] We would like to keep current RAN2 agreement. Need to minimize to create new IE. |
| E031 | 107 | Ok with the new text for the field description of numberOfConnFail, and suggest to put cat b. numberOfConnFail This field is used to indicate the number of failed RRCSetup or RRCResume procedures. | [Samsung] Fine with the suggestion. On the other hand, we could slightly further update it as follows (it’s related to S490): numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures. |

#### 2.2.3	RILs (Related to offline 802)

20 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| N016 | 5 | RAN2-109b-e agreed: 7 RAN2 confirms that no explicit availability indicator is included (e.g. ra-ReportAvailable) in RRCSetupComplete, RRCResumeComplete, RRCreestablishmentComplete and RRCReconfigurationComplete messages for availability RA-report. Suggest to put PropReject. | [Samsung] We would like to keep current RAN2 agreement, i.e. no explicit availability indicator |
| E009 | 31 | Maybe Ericsson can check it as all are related to [802] SON offline discussion. In addition, if it is related to SON email [961], maybe we can rely on that. | [Samsung] It seems strange. I assume that these procedural texts are related to inter-RAT HO from E-UTRA to NR. However, how can UE consider RRCReconfiguration including reconfigurationWithSync? In my understanding, UE should receive RRCConnectionReconfiguration including mobilityControlInfo when it is in E-UTRA? |
| N019 | 32 | Same comment to E009. |  |
| E012 | 39 | We have agreed to have a uniform CGI-Info-Logging for NR MDT and SON, and TAC is included. Suggest to put cat b. | [Samsung] In detail, the field trackingAreaCode should be added as optional, to the IE CGI-Info-Logging. If needed, the field description may be updated to indicate when TAC is configured. |
| E021 | 51 | Same comment to E009 |  |
| qS478 | 57 | Same comment to E009 |  |
| C265 | 58 | Same comment to E009 |  |
| N017 | 63 | Same comment to E009 |  |
| S473 | 66 | Same comment to E009 |  |
| E023 | 80 | RAN2-109b-e agreed: Include beamFailureRecoveryFailure as a failureType in SCGFailureInformation (NR RRC spec) and SCGFailureInformationNR (LTE RRC spec) messages. Suggest to put cat b. |  |
| Z152 | 91 | Same comment to E009 |  |
| S475 | 92 | Same comment to E009 |  |
| S482 | 93 | RAN2-109b-e agreed: 3 If Msg1 SCS for contention free BFR is configured, Msg1 SCS for both contention based and contention free PRACH transmission occasions are reported in RA report. Suggest to put cat b. |  |
| S483 | 94 | Same comment to E009 |  |
| S481 | 95 | Same comment to E009 |  |
| S480 | 96 | RAN2-109b-e agreed: 1 For CSI-RS based RA attempt contentionDetected-r16 and dlRSRPAboveThreshold-r16 are not included in PerRAInfoList-r16. Suggest to put cat b. |  |
| S485 | 97 | Same comment to E009 |  |
| S484 | 98 | RAN2-109b-e agreed: 4 Clarify the following in field description of dlRSRPAboveThreshold a. This field is used to indicate if SS-RSRP of selected SSB is above or below the rsrp-ThresholdSSB. b. For random access procedure initiated for beam failure recovery, rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure is used to set parameter dlRSRPAboveThreshold. Otherwise, rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure is used to set parameter dlRSRPAboveThreshold. Suggest to put cat b. |  |
| S477 | 99 | Same comment to E009 |  |
| S490 | 106 | Same comment to E009 |  |

#### 2.2.4	RILs (Related to offline 888)

5 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| C252 | 15 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C252 | 17 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C201 | 72 | Not very clear about the issue and proposed solution. Maybe CATT can clarify it. In addition, in section 2.2.5 there are some RILs may be relatated to this RIL, so it may be good to discuss them together. |  |
| B003 | 130 | RAN2-109b-e agreed: 10: use setupRelease structure for BT/Sensor/WLAN location configurations. Suggest to put cat b. |  |
| B005 | 143 | Same comment to B003 and Suggest to put cat b. |  |

#### 2.2.5	RILs (Related to R2-2002731 in offline 888)

14 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| C201 | 4 | This was part of offline 888, and it is suggested to further it in the next table as below. |  |
| C201 | 16 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 24 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 35 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C204 | 53 | We think it is related to MDT summary paper R2-2004003, i.e. discussion on P1. Copmanies’ comments were collected and there was no consensus, so it is suggested to put PropReject. |  |
| C201 | 59 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 74 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 76 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 78 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C201 | 82 | Seems relating to C201, line72 (also captured in section 2.2.3), so it may be good to discuss them together. |  |
| C204 | 119 | Proposed change is empty, so Maybe CATT can clarify it. |  |
| C203 | 134 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C203 | 135 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C203 | 138 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |

Issue: For C201, line4, the detailed information is as below.

| Description | Proposed Change |
| --- | --- |
| Bluetooth/WLAN measurements results are not independent with log measurements collection. Instead, Bluetooth/WLAN measurements results are embedded into each log MDT entry. what’s the use case to introduce such independent available indicator for Bluetooth/WLAN measurements results? More addition, Sensor measurements results are also part of locationInfo, why there is no independent available indicator for Sensor measurements results? | 2> if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport: 3> include the logMeasAvailableBT in the RRCSetupComplete message; 2> if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport: 3> include the logMeasAvailableWLAN in the RRCSetupComplete; |

We think it is related to ASN1 summary paper R2-2004002, and the related proposal is shown as below:

Proposal 1:RAN2 to discuss which of the option is more desirable from RAN2 perspective:	(R2-2002731)

Option1: Remove the availability indicators for Bluetooth/WLAN measurements results from RRCReestablishmentComplete, RRCReconfigurationComplete, RRCResumeComplete, RRCSetupComplete, UEInformationResponse message. If so, the same change should also be done for 36.331.

Option2: Add an extra availability indicator for Sensor measurements results in RRCReestablishmentComplete, RRCReconfigurationComplete, RRCResumeComplete, RRCSetupComplete, UEInformationResponse message to align with Bluetooth/WLAN measurements results.

Option3：Nothing should be done for Bluetooth/WLAN/Sensor measurement results availability indicator feature.

For the above P1, please companies provide comments if any.

| Company | Which option is preferred? | Comments |
| --- | --- | --- |
| Samsung | Option 1 | Currently, NW cannot request selective retrieval. Not sure if there is any significant benefit with separate availability indicator. |

#### 2.2.6	RILs (Related to R2-2002733 in offline 888)

4 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| C253 | 8 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C257 | 11 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C256 | 14 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C256 | 85 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |

#### 2.2.7	RILs (Related to R2-2002826 in offline 888)

7 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| S471 | 38 | It is related to P13 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, P13 needs more discussions. So it is suggested to further discuss it in tables as below. |  |
| S468 | 48 | It is related to P11 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, P11 was agreed, so suggest to put cat b. |  |
| S472 | 65 | It is related to SON paper R2-2002827, so maybe Ericsson can check it. |  |
| S461 | 69 | It is related to P1 and P2 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, P1 needs more discussions, and lots of companies put cat c to P2. So it is suggested to further discuss P1 in tables as below, and put PropReject to P2. |  |
| S465 | 81 | It is related to P8 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, lots of companies put cat c to P8, so suggest to put PropReject. |  |
| S469 | 127 | It is related to P12 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, P12 needs more discussions. So it is suggested to further discuss P12 in tables as below. |  |
| S474 | 128 | It is related to P5 and P6 in R2-2002826. Based on the Offline 888 summary R2-2004002 and the MDT minutes, P5 and P6 need more discussions. So it is suggested to further discuss P5 and P6 in tables as below. |  |

Issue: S471, line 38. The proposal is as below (details can be found  in R2-2002826)

Proposal 13: Upon setup failure or resume failure, UE sets the plmn-Identity to RPLMN if available.

For the above proposal, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Except for initial access, UE always has RPLMN upon setup failure and resume failure. It is reasonable to store RPLMN if available. |

Issue: S461, line69. The proposal is as below (details can be found  in R2-2002826)

Proposal 1	Add the nonCriticalExtension and laterNonCriticalExtension fields to the LoggedMeasurementConfiguration message.

For the above proposal, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | It’s a general mean for future enhancement. |

Issue: S469, line127. The proposal is as below (details can be found  in R2-2002826)

Proposal 12: RAN2 clarifies how to determine whether a cell is part of the area Indicated by AreaConfiguration.

For the above proposal, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | SIB1 includes multiple PLMN-IdentityInfo, each of which includes multiple PLMN-Identity. We need to clarify how to determine whether a cell is part of the area Indicated by AreaConfiguration. A simple option is to consider only first PLMN-Identity in first PLMN-IdentityInfo of the PLMN-IdentityInfoList, and cellIdentity corresponding to the first PLMN-IdentityInfo. |

Issue: S474, line128. The proposals are as below (details can be found  in R2-2002826)

Proposal 5	Change the name and field descriptions of areaConfigForServing and areaConfigForNeighbour as to areaConfig and interFreqTargetList

Proposal 6	Change the interFreqTargetList to be a list of frequencies, with for each a cell list.

For the above proposals, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | In RAN2#107bis, RAN2 made the following agreement: Introduce the logged MDT measurement frequencies and cell IDs (i.e. PCI) for neighbour cell measurement in the area configuration. UE only need to log and report measurement results for the configured frequencies if the results available. However, current CR has indicated frequency band List, instead of frequencies. It is not in line with the RAN2 agreement. |

#### 2.2.8	RILs (Related to R2-2003160 in offline 888)

4 RILs.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| N011 | 111 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| N012 | 112 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| N013 | 113 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| N014 | 114 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |

#### 2.2.9	RILs (Reated to Z157)

1 RIL.

The email rapporteur has provided initial analysis on the RILs. Please companies check and provide comments if any. If there are no concerns, it is suggested to go with the email rapporteur’s comments/suggestions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| S496 | 129 | Z157 was agreed at the last RAN2 meeting, so suggest to put cat b. |  |

### 2.3	Cat b1 RILs not related with others

Filter condtions in XLS:

- In label “1_MDT, NR_SON_MDT-Core”

- The Column M Category = b1

- The Column N Relations with others = empty

There are in total 22 RILs in the XLS.

The following table provides an overview on RILs. For some RILs, more tables are provided as below in order to collect companies’ opinions.

| RIL No | Line | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| S470 | 10 | It should be put in section 2.2.7 as it is related to R2-2002826. However, we can discuss it here. In R2-2002826, P13 is for both S470 and S471. Since section 2.2.7 is discussing P13, so there is no need to re-discuss P13 here. Suggest to wait for P13 discussion in section 2.2.7. |  |
| C202 | 19 | In the column Comments, one company did not agree on the change. So suggest to put PropReject. Ericsson_Pradeepa: This is not a must as this does not indicate how these measurements are performed for which we have to again refer to LPP spec. Instead of doing that, we can leave the specifcation as is. |  |
| E005 | 20 | Suggest to discuss it in tables below. |  |
| C203 | 21 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| C202 | 29 | Similar comments to C202, line19, one company did not agree on the change, so suggest to put PropReject. |  |
| C202 | 41 | Similar comments to C202, line19, one company did not agree on the change, so suggest to put PropReject. |  |
| C202 | 54 | Similar comments to C202, line19, one company did not agree on the change, so suggest to put PropReject. |  |
| C202 | 55 | Similar comments to C202, line19, one company did not agree on the change, so suggest to put PropReject. |  |
| Z155 | 56 | Suggest to discuss it in tables below. |  |
| E013 | 60 | Suggest to discuss it in tables below. |  |
| Z156 | 71 | Suggest to discuss it in tables below. |  |
| N006 | 73 | Section 2.2.1 covers a similar issue, so suggest to discuss all related RILs in section 2.2.1. |  |
| N009 | 77 | Section 2.2.1 covers a similar issue, so suggest to discuss all related RILs in section 2.2.1. |  |
| N010 | 79 | Section 2.2.1 covers a similar issue, so suggest to discuss all related RILs in section 2.2.1. |  |
| S491 | 83 | In the column Comments (shown below), it seems that there are some disagreements. So suggest to put PropReject. Ericsson_Pradeepa: In our understanding this is already included as part of the serving cell information CATT_Jayson: the same view with Ericsson Huawei_Jun: it seems that nothing is needed. Samsung_Sangbum: can clarify that the serving cell info in last log entry is first serving cell after leaving? If so, we can need a NOTE to indicate it? |  |
| H017 | 110 | Suggest to discuss it in tables below. |  |
| C260 | 116 | It has been captured in the latest 38.331 CR, so suggest to put cat b. |  |
| S457 | 122 | Discuss together with S458. For the RIL, the Comments are as below: Ericsson_Pradeepa: Agree CATT_Jayson: Agree Intel_Candy: seems reasonable When implementing the RIL into the 38.331 CR (related to [Post109bis-e][900][MDTSON] CR to 38.331), the email rapporteurs found the following issue: Huawei_Jun: wonder whether it is possible, as ReportConfigNR does not have extension mark. Ericsson_Pradeepa: Agree with Huawei. It is better to keep the existing text as the MDT measurements related configurations are only applicable for periodical and event driven immediate MDT measurements. So S457 and S458 are changed to cat b1. Suggest to discuss both RILs in tables below. |  |
| S458 | 123 | See comments to S457. |  |
| E028 | 125 | This RIL was initially agreeable, however, when implementing the RIL into the 38.331 CR (related to [Post109bis-e][900][MDTSON] CR to 38.331), the email rapporteurs found the following issue: Huawei_Jun: we checked 38.306 v16.0.0 and there are no MDT/SON UE capabilities. If so, we think maybe we can revisit UE capabilities at RAN2-110 meeting, e.g. discuss both 38.306 CR and 38.331 CR together. Ericsson_Pradeepa: Okay to wait. We should change the category to b1 The email rapporteur Huawei (Jun) checked this with the WI rapporteur CMCC (Ningyu), and here are suggestions: - NR UE MDT capabilities can be put in “big CR” for MDT - At RAN2-109b-e meeting, there was a running 38.306 CR, and ths CR will be re-submitted to 110-e meeting (may be updated). So 38.331 changes are to be aligned with 38.306 CR R2-2002896 Running CR to 38.306 for NR_SON_MDT vivo, CMCC draftCR Rel-16 38.306 16.0.0 NR_SON_MDT-Core If above suggestions are agreeable, the RIL can be put cat b. |  |
| C208 | 132 | Suggest to discuss it in tables below. |  |
| C262 | 148 | Suggest to discuss it in tables below. |  |

Issue: E005, line20, the details are as below:

| Description | Proposed Change |
| --- | --- |
| From the procedural text, it is unclear if the random-access resources referred in the text is related to the one that is used by the UE for reconfigurationWithSync or if they correspond to some other RA reosurces. | Original text: 3> set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources; 3> set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources; 3> set the msg1-FrequencyStart, msg1-FDM and msg1-SubcarrierSpacing associated to the random-access resources; Proposed text: 3> set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources used in reconfigurationWithSync procedure; 3> set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources used in reconfigurationWithSync procedure; 3> set the msg1-FrequencyStart, msg1-FDM and msg1-SubcarrierSpacing associated to the random-access resources used in reconfigurationWithSync procedure; |

In XLS, the Comments for the RIL are:

Huawei_Jun: seems reasonable.

Ericsson_Pradeepa: Proponent

Intel_Candy: We think this change may not be correct. RA resource may be either contention free or contention based. So it doesn't necessary to be in RRC configure.

Huawei_Jun2: ok to change it from a to b1.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

Issue: Z155, line56, the details are as below:

| Description | Proposed Change |
| --- | --- |
| location information is introduced in SCG Failure information to help locate the place where the SCG failure happens and therefore assist the optimization of SN configuration, for similar reason location information shall also be included in MCG failure information to help optimize MN configuration. | Proposed text : 1> if SRB1 is configured as split SRB and pdcp-Duplication is not configured: 2> if primaryPath refers to the MCG: 3> set primaryPath to refer to the SCG; 1> if available, set the locationInfo as follows: 2> if available, set the commonLocationInfo to include the detailed location information; 2> if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 2> if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs. 2> if available, set the sensor-LocationInfo to include the sensor measurement results. |

In XLS, the Comments for the RIL are:

Ericsson_Pradeepa: We support this proposal although this is related to another ongoing rel-16 work item

Intel_Candy: we think this needs further discussion.

Huawei_Jun: ok to change it from b to b1

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | It seems too late to discuss it. Also, we have assumed that the information related to Rel-16 features will be considered in next release. |

Issue: E013, line60, the details are as below:

| Description | Proposed Change |
| --- | --- |
| It is not clear if the UE includes the content of NR RRC spec relared RLF report variable or LTE spec related RLF report variable during the cross RAT RLF reporting of LTE RLF report to the NR node. | Existing text: 2> else if the UE has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]: 3> set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link or handover failure in EUTRA; 3> set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report; 3> discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers; 3> discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers; Proposed text: 2> else if the UE has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]: 3> set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link or handover failure in EUTRA; 3> set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10]; 3> discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers; 3> discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers; |

In XLS, the Comments for the RIL are:

Ericsson_Pradeepa: Proponent

CATT_Jayson: We agree this proposal, think it's more like a Cat a issue.

Intel_Candy: We think this needs further discussion. We think the UE should disgard the record once RLFreport has been successful delivered.

Huawei_Jun: ok to change it from a to b1.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | It seems more editorial. |

Issue: Z156, line71, the details are as below:

| Description | Proposed Change |
| --- | --- |
| As described in Z155, location information shall also be included in MCG failure information to assist the optimization of MN configuration. | measResultSCG-EUTRA-r16 OCTET STRING OPTIONAL, ... [[ locationInfo-r16 LocationInfo-r16 OPTIONAL ]] |

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | It seems too late to discuss it. Also, we have assumed that the information related to Rel-16 features will be considered in next release. |

Issue: H017, line110, the details are as below:

| Description | Proposed Change |
| --- | --- |
| This is used only for E-UTRA RLF reporting but in 36.331, for the field RLF-Report-r9 in TS 36.331, it is FFS whether it can support LTE/5GC and in the procedural text in TS 36.331, the UE is able to report RLF report only for LTE/EPC. Therefore, it is unclear whether the 5GC identities are relevant here. | First discuss whether the RLF reporting when connected to LTE/5GC is supported. If not, 5GC identities could be removed. |

In XLS, the Comments for the RIL are:

Huawei_Jun: there is a discussion paper R2-2003584 on RIL.

ZTE_Zhihong: We prefer to fix it in LTE specs.We agreed on LTE RLF to be reported in NR is to support MRO.If we don't support RLF on LTE-5GC case then many scenarios in MRO cannot be supported, which is against the original intention.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | Shared with ZTE’s view above |

Issue: S457, line122, S458, line123, the details are as below:

| Description | Proposed Change |
| --- | --- |
| includeCommonLocationInfo, includeBT-Meas, includeWLAN-Meas and includeSensor-Meas are applicable for both periodical reporting and event-triggered reporting | remove above IEs from the current place and move them up directly under ReportConfigNR |
| Same issue as S457. includeCommonLocationInfo, includeBT-Meas, includeWLAN-Meas and includeSensor-Meas are applicable for both periodical reporting and event-triggered reporting | Remove above IEs from the current place |

For the above RILs, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

Issue: C208, line132, the details are as below:

| Description | Proposed Change |
| --- | --- |
| duplicated field description for rssiWLAN | rssiWLAN Measured WLAN RSSI result in dBm. The IE WLAN-RSSI-Range specifies the value range used in WLAN RSSI measurements and thresholds. Integer value for WLAN RSSI measurements is according to mapping table in TS 36.133 [40]. Value 0 corresponds to –infinity, value 1 to -100dBm, value 2 to -99dBm, and so on (i.e. in steps of 1dBm) until value 140, which corresponds to 39dBm, while value 141 corresponds to +infinity. Rtt-WLAN This field provides the measured round trip time between the target device and WLAN AP and optionally the accuracy expressed as the standard deviation of the delay. Units for each of these are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 37.355 [49]. rttValue This field specifies the Round Trip Time (RTT) measurement between the target device and WLAN AP in units given by the field rttUnits as defined in TS 37.355 [49]. rttUnits This field specifies the Units for the fields rttValue and rttAccuracy. The available Units are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 37.355 [49]. rttAccuracy This field provides the estimated accuracy of the provided rttValue expressed as the standard deviation in units given by the field rttUnits as defined in TS 37.355 [49]. Ssid Service Set Identifier (SSID) defined in IEEE 802.11-2012 [50]. Wlan-Identifiers Indicates the WLAN parameters used for identification of the WLAN for which the measurement results are applicable. WLAN-RSSI-Range The IE WLAN-RSSI-Range specifies the value range used in WLAN RSSI measurements and thresholds. Integer value for WLAN RSSI measurements is according to mapping table in TS 36.133 [40]. Value 0 corresponds to –infinity, value 1 to -100dBm, value 2 to -99dBm, and so on (i.e. in steps of 1dBm) until value 140, which corresponds to 39dBm, while value 141 corresponds to +infinity. |

In XLS, the Comments for the RIL are:

Ericsson_Pradeepa: Agree. Just one question related to the reference. Shall we continue to use 36.133 as the reference or is there any update in 38.133?

CATT_Jayson: I'm  not sure now, maybe Huawei can answer the question.

Huawei_Jun: We will double check it ASAP.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

Issue: C262, line148, the details are as below:

| Description | Proposed Change |
| --- | --- |
| This PLMN Id list should be removed from VarLogMeasConfig since the PLMN Id list received from the NW is directly stored in VarLogMeasReport, which is aligned with the text procedure and follow 36.331. | Remove the field of “plmn-IdentityList-r16” |

In XLS, the Comments for the RIL are:

Ericsson_Pradeepa: We believe this is required as the UE checks the PLMN of the new cell before logging the new MDT measurements based on the PLMN identity list stored in varLogMeasConfig instead of varLogMeasReport.

ZTE_Zhihong: Not agree with the proposal. This field is needed for UE to perform PLMN checking to see if it is required to perform the logging in the selected PLMN. Suggest to change to cat b1.

Huawei_Jun: ok to change it from b to b1.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | Shared with Ericsson and ZTE’s opinion. We called it MDT PLMN list in LTE. |

### 2.4	Cat b1 RILs with relating two or more WIs

Filter condtions in XLS:

- In label “2_DCCA, MDT”, “3_MDT, 2StepRA”, “4_MDT, NR-U”, “5_MDT, TEI”,  and “6_MobEnh, MDT”

- The Column M Category = b1

There are in total 3 RILs in the XLS.

The following table provides an overview on RILs. For some RILs, more tables are provided as below in order to collect companies’ opinions.

| RIL No | Label | Email Rapp’s comments | Other comments |
| --- | --- | --- | --- |
| E051 | 3_MDT, 2StepRA | Maybe Ericsson can check it as it may be related to [802] SON offline discussion. |  |
| E008 | 4_MDT, NR-U | Maybe Ericsson can check it as it may be related to [802] SON offline discussion. |  |
| E039 | 5_MDT, TEI | Suggest to discuss it in tables below. |  |

Issue: E039, 5_MDT, TEI, the details are as below:

| Description | Proposed Change |
| --- | --- |
| Upon detecting radio link failure on the MCG, according to current procedure the UE should discard any segments of the segmented RRC message and store various link failure information in the VarRLF-Report variable. However, this should be done only if before triggering the RRC restablishment procedure. | We propose to move the following text just before triggering the RRC reestablishment procedure. A possible implementation is as follow: 5> else: 6> discard any segments of segmented RRC messages received ……(until) 11> set the dlRSRPAboveThreshold to false; 6> initiate the connection re-establishment procedure as specified in 5.3.7. |

In XLS, the Comments for the RIL are:

[Samsung] We think that UE should discard the RRC segments for some other re-establishment cases as well. So, it would be better to move this sentence to the clause 5.3.7.2 (initiation of RRC connection re-establishment).

ZTE_Zhihong: Please note that with the proposed change, if a RLF occurs and UE goes in to idle as indicated in the following two bullet, "4> if AS security has not been activated:

5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4> else if AS security has been activated but SRB2 and at least one DRB have not been setup:

5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; ", UE won't log any RLF information, this is different behavior as in LTE and requires further discussion.

Also for better readibility we suggest to have a separated subclause 5.3.11.x for setting detailed RLF content, not to mix it with the procedure defined in 5.3.11.3.

For the above RIL, please companies provide comments if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

### 2.6	Others

#### 2.6.1	RILs from email discussion [Post109bis-e][900][MDTSON] CR to 38.331

In [1], some MDT RILs are changed into cat b1 due to some reasons, and details can be found in section 2.4 in [1]. These RILs have been captured in section 2.2 and 2.3, and companies can check and provide comments if any.

#### 2.6.2	Two MDT RILs without Work Item code in XLS

In the XLS, the following two RILs are not included as the column Work Item is empty for both RILs. It is suggested to rely on ASN1 main session discussion. If needed, we may treat them in MDT session.

| ID | Delegate | Work Item | Class | TDoc | Status |
| --- | --- | --- | --- | --- | --- |
| N040 | Nokia (Tero) |  | 2 | None | PropAgree |
| N002 | Nokia (Tero) |  | 2 | None | PropAgree |

#### 2.6.3	MDT RILs treated in MailDisc

In the XLS, the following two RILs (related to MDT and other WIs) are under ASN1 main session discussion.

| ID | Delegate | Work Item | Class | TDoc | Status |
| --- | --- | --- | --- | --- | --- |
| E008 | Ericsson (Pradeepa) | MDT, NR-U | 2 | R2-2003078 | DiscMail11 |
| M005 | MediaTek (Felix) | DCCA/MDT | 2 | R2-2003654 | DiscMail9 |

## 3	Conclusion

## 4	Reference

[1]	R2-2003871, Summary of [Post109bis-e][900][MDTSON] CR to 38.331, Huawei, Ericsson