3GPP TSG-RAN WG2 Meeting #110-e					R2-20xxxxx

Electronic, 1 June – 12 June 2020

Agenda Item:		6.12.4	ASN1 review

Source:			Huawei (Summary rapporteur)

Title:				Summary on ASN1 review

WI code(s):			NR_SON_MDT-Core

Document for:		Discussion and Decision

## 1	Introduction

This paper is to provide a summary on 6.12.4 ASN1 review topic for the WI SON/MDT support in NR. All related Tdocs are listed in section 4.

In previous RAN2 meeting, some categories were allocated for proposals in order for efficient discussions. So this paper is to use the same method,

- (a) a potential easy agreement, e.g. Proposals where consensus exists, that seem straightforward to agree

- (b) need further discussion. These should be tagged with e.g. [FFS] so they are clearly visible, and should indicate what the primary controversy is

- (c) a candidate for immediate postpone, e.g. issues that may require other WG discussions or is contentious such that it is unlikely to converge at e-Meeting.

- (d) Issues on ASN.1 – or specifications-specific CR proposals will go directly to email discussion on the corresponding CRs.

## 2	Issue summary

### 2.1	Guideline

Section 2.1 to 2.11 are discussions based on the RIL papers under the AI 6.12.4.

For ASN1 XLS, compared with v64, v164 has 54 new RILs. There are some RILs with no RIL papers, so section 2.12 are to check them. In XLS, the Status and Cat handling are still following the legacy rules.

### 2.2	CEF report

[1], ZTE, [Z162-Z166]

Proposal 1: Modify “if the UE has connection establishment failure informaton available in VarConnEstFailReport” in 5.3.3.7 and 5.3.13.5 to as ““if the UE has connection establishment failure information or resume failure information available in VarConnEstFailReport”.	// (a)

Proposal 2: UE compares the cell identity to determine the numberOfConnFail before it clears the available information except numberOfConnFail in VarConnEstFailReport. 	// (b)

Proposal 3: UE shall reset the numberOfConnFail to “0” when this is the first connection failure since UE transits from connected to idle state. 	// (b)

Proposal 4: UE reset numberOfConnFail to “1” when storing resume failure information. And remove the part UE compare the numberOfConnFail from subclause 5.3.13.5. 	// (b)

Proposal 5: Update the field description of to numberOfConnFail to “the latest number of consecutive failed connection establishment or resume attempt in the same cell.’’ 	// (a). Covered by the email 960

Proposal 5: It is confirmed in RAN2 that following RA resource related information is included in CEF report and the related description is added in TS 38.331:			// (b)

- absoluteFrequencyPointA (e.g., in FrequencyInfoUL)

b. locationAndBandwidth (e.g., in UL BWP)

c. subcarrierSpacing (e.g., in UL BWP)

d. msg1-FDM (e.g., in RACH-ConfigGeneric)

e. msg1-FrequencyStart (e.g., in RACH-ConfigGeneric)

f. msg1-SubcarrierSpacing  (e.g., in RACH-ConfigCommon)

[8], Ericsson, [E012]

In the procedural text, set the measResultFailedCell to also include tracking area code. 			// (a). In the last RAN2 meeting, this RIL had been already agreed.

### 2.3	RLF report

[2], ZTE, [Z167][Z169]

Proposal 1: In 5.3.5.8.3, move the RLF report content setting procedure to right before the UE initiate re-establishment procedure. 	// (b)

Proposal 2: In 5.3.10.3, in order to allow store RLF information when RLF happens and UE goes to idle, the RLF content setting procedure shall be put in the corresponding section when UE goes to idle with release cause ‘other ’ or ‘RRC connection failure’ as well. 	// (b)

Proposal 3: To create an independent section in 5.3.10.3 for RLF report content setting. 	// (b)

[5], Samsung, [S951][S952]

Proposal 1: For the case UE declares radio link failure due to the random access problem indication from MCG MAC: if the random access procedure was initiated for beam failure recovery, rlf-Cause is set to beamFailureRecoveryFailure, otherwise rlf-Cause is set to randomAccessProblem. 	// (a)

Proposal 2: rlf-cause is set to randomAccessProblem or beamFailureRecoveryFailure if the radio link failure is declared due to the random access problem indication from MCG MAC for 4 step random access procedure. 	// (a)

[6], Ericsson, [E008]

Proposal 1	Include lbtFailure as an option in rlfCause in RLF report.

Proposal 2	Include lbtFailure as a failureType in SCGFailureInfomationNR in LTE RRC specification.

Proposal 3	RAN2 is kindly requested to agree the TPs in this document.

// (b). Under [961] discussion.

[7], Ericsson, [E009]

Based on the outcome of email discussion Post109bis-e#961 Error: Reference source not found, it was agreed to include previousPCellID in the NR RLF report. This contribution provides the text proposal for the same. 	// (a). The proposal is following agreements made in [961]

[11], Ericsson, [E200]

Proposal 1	Include T312-expiry as an option in rlfCause in RLF report.	// (b)

[19], Huawei, [H365]

There are only observations in this paper.

[21], Huawei, [H367]

It is suggested to add the following text regarding UE including failedPCellId-EUTRA in LTE RLF report:  3> set failedPCellId-EUTRA to the PCell in which RLF is detected or the target PCell of the failed handover stored in VarRLF-Report of TS 36.331 [10];		// (a)

[22], Huawei, [H368]

Proposal: The UE should set the measResult-RLF-Report-EUTRA in rlf-Report for LTE RLF report purpose.	// (a)

### 2.4	RACH report

[3], ZTE, [Z170][Z171][Z173]

Proposal 1: If both CB/CF RA resource with different configuration is used in RA procedure, one common IE is used to indicate the CBRA resource configuration, while another dedicated IE is used to indicate the CFRA resource parameters with different value of that in the common IE. 	// (b)

Proposal 2: contentionDetected flag is no required to be included when the ra-Purpose is set to requestForOtherSI. 	// (b)

[13], Ericsson, [???]

Proposal 1	The UE includes beamFailureRecovery in raPurpose in case of beam failure recovery failure in the SpCell.

Proposal 2	The UE includes reconfigurationWithSync in raPurpose when RA is triggered due to reconfiguration with sync.

Proposal 3	The UE includes ulUnSynchronized in raPurpose when RA is triggered by a PDCCH order, irrespective of whether the PDCCH order is for the PTAG or STAG.

Proposal 4	Delete sCellAdditionTAAdjestment from RA-purpose.

Proposal 5	The UE includes either the SpCell or the SCell in which RA occurred when the raPurpose is set to ulUnSynchronized.

Proposal 6	The UE includes schedulingRequestFailure in raPurpose when RA is triggered due to scheduling request failure.

Proposal 7	The UE includes noSRPUCCHResourceAvailable in raPurpose when RA is triggered due to lack of PUCCH SR resources configured at the UE.

Proposal 8	The UE includes requestForOtherSI in raPurpose when RA is triggered to request on demand system information.

Proposal 9	The UE includes reestablishment in raPurpose when RA is triggered to perform reestablishment.

//	put (b) to all above proposals

[16], Samsung, [S481]

For SSB based RA attempt based on contention free random-access resources contentionDetected-r16 is not included in PerRAInfoList-r16.		// (b). Seems to be related to [961]

[27], ZTE, [Z168][Z172]

Proposal 1: One common IE, e.g. RA-InformationCommon is used to indicate the RA related information, i.e., BWP configuration,RA resoource information and perRA-InfoList.	// (b)

Proposal 2: To create a new subclause 5.7.10.5 for determination of RA information in RA report or RLF report, and put the reference at corresponding section.	// (b)

### 2.5	Sensor measurements

[4], CATT, [???]

Proposal1: Add an extra availability indicator for Sensor measurements results in RRCReestablishmentComplete, RRCReconfigurationComplete, RRCResumeComplete, RRCSetupComplete, UEInformationResponse messages to align with Bluetooth/WLAN measurements results. 	// (a). In the summary of email 960, this proposal is agreeable.

Proposal2: RAN2 is kindly asked to agree the corresponding TP. 	// (a)

### 2.6	Logged MDT

[9], Ericsson, [E021]

Proposal 1	Modify the procedural text to indicate that the UE shall log anyCellSelectionDetected flag indication and the last serving cell related measurements upon entering any cell selection state only when the UE is configured with periodical logged MDT. 	// (b). At the last RAN2 meeting, this proposal was discussed in MDT and the summary paper can be found inR2-2004003. Generally, 8 companies agreed but 4 companies disagreed. If there is no change on companies’ opinions, it is suggested to not pursue this proposal.

[12], Ericsson, [E235]

Proposal 1	The UE shall include a flag in the logged MDT report to indicate whether the UE is performing RRM measurements based on relaxed RRM measurement policy or normal RRM measurement policy.		// (b)

[15], Samsung, [S954]

Proposal 1: The UE does not store logged measurement configurations in the UE Inactive AS Context.	// (b). RAN2 agreed that logged MDT is also applicable in RRC Inactive state, so does this proposal contradict the RAN2 agreement?

[18], Huawei, [H363]

change "2> when performing the logging:"  into:  "2> when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:"		// (b)

[24], Huawei, [H371]

Proposal: It is proposed to add UE behaviours regarding the field InterFreqTargetList in the procedural text.	// (b)

[25], Samsung, [S959]

Proposal: IE InterFreqTargetList moves out of IE AreaConfiguration.	// (b)

### 2.7	UE capabilities

[10], Ericsson, [E028]

TP for 38.331 on UE capabilities.	// (a)

### 2.8	Mobility History Information

[14], Samsung, [S953]

Proposal 1: The UE does not include mobilityState in the RRCSetupComplete message if RRCSetup is received in response to an RRCReestablishmentRequest.	// (b)

### 2.9	Processing delay requirements

[20], Huawei, [H366]

Proposal: Suggest to add UE information procedure in processing delay requirements and the value could be one of the following options:	// (b)

- (1) 15 ms. This is following LTE definition

- (2) NA

- (3) a specific value different from 15ms

### 2.10	CGI info

[23], Huawei, [H369][H370]

It is suggested to add Optional for trackingAreaCode-r16 in CGI-Info-Logging-r16.	// (a)

In order to be aligned with the procedural text, in the field description of trackingAreaCode, it is suggested to add the following text:  This field should be included if CGI-Info-Logging-r16 is under RA-Raport-r16 or under ConnEstFailReport-r16.		// (b)

### 2.11	TP related to SON email 961

[26], ZTE, [???]

TP		// (b). Or maybe we can agree to it and review changes in the updated 38.331 CR.

### 2.12	New RILs that are not covered by RIL papers

| ID | Delegate | Description | Proposed Change | Agreeable | Comments |
| --- | --- | --- | --- | --- | --- |
| H360 | Jun Chen (Huawei) | Missing description on SON and MDT. In comparison, TS 36.331 has defined the following: - Support of self-configuration and self-optimisation (not applicable for NB-IoT); - Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [60] (not applicable for NB-IoT); | It is suggested to add similar description as LTE, e.g.: - Support of self-configuration and self-optimisation; - Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [xx]; | (a) |  |
| H361 | Jun Chen (Huawei) | For the following description, it is missing "or connection resume failure" in the last part: The UE may discard the connection establishment failure or connection resume failure information , i.e. release the UE variable VarConnEsFailReport, 48 hours after the last connection establishment failure is detected. | Add "or connection resume failure" in the description "48 hours after the last connection establishment failure is detected." | (a) |  |
| Z161 | Z(Zhihong) | Missing procedure to release btNameList,wlanNameList and sensorNameList, which is “need M” in OtherConfig | Upon initiation of the procedure, the UE shall: /*Partially omitted*/ 1> stop timer T342, if running; 1> stop timer T345, if running; 1> release idc-AssistanceConfig, if configured; 1> release btNameList, if configured; 1> release wlanNameList, if configured; 1> release sensorNameList, if configured; | (a) |  |
| E201 | Ericsson (Pradeepa) | Wrong reference to the section 5.7.10.4 instead of 5.3.10.4 | Existing text: 5> set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.7.10.4; Proposed text: 5> set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4; | (a) |  |
| H362 | Jun Chen (Huawei) | The 1st, 2nd and 3rd "2>" are for three logging cases, and regarding UE behaviours on "loggingInterval", there are the following text: (1) for periodical MDT, there is " ... by the loggingInterval in the LoggedPeriodicalReportConfig" (2) for event triggered MDT, there is "....by the loggingInterval in VarLogMeasConfig" | Suggest to update the UE behaviours on the loggingInterval for event triggered MDT. For example: 3> perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig (changed into LoggedEventTriggerConfig) only when the UE is in any cell selection state; 4> perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig (changed into LoggedEventTriggerConfig) only when the conditions indicated by the eventL1 are met; | (a) |  |
| E239 |  |  |  | (b) | Will be handled in main room |
| E206 |  |  |  | (b) | Will be handled in main room |
| H364 | Jun Chen (Huawei) | There are duplicated description on discarding the LTE RLF report (as below). 3> discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers; 3> discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers; | Suggest to keep the second "3>" and remove the first one. | (a) |  |
| N040 | Nokia (Tero) | Consider grouping the mandatory fields under an IE – these are the fields needed to distinguish the Trace context at network side. | Create IE for these and use it to identify the Trace identity for the UE: NR-Trace-r16 ::= SEQUENCE { traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), ... } and LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE { traceIdentity-r16 NR-TraceId-r16 absoluteTimeInfo-r16 AbsoluteTimeInfo-r16, | (b) |  |
| Z175 | Z(Zhihong) | According to ASN.1 coding guideline, the need code for SetupRelease structure is “need M” | LoggedMeasurementConfiguration message -- ASN1START -- TAG-LOGGEDMEASUREMENTCONFIGURATION-START LoggedMeasurementConfiguration-r16 ::= SEQUENCE { criticalExtensions CHOICE { loggedMeasurementConfiguration-r16 LoggedMeasurementConfiguration-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE { traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), absoluteTimeInfo-r16 AbsoluteTimeInfo-r16, areaConfiguration-r16 AreaConfiguration-r16 OPTIONAL, --Need R plmn-IdentityList-r16 PLMN-IdentityList2-r16 OPTIONAL, --Need R bt-NameList-r16 SetupRelease {BT-NameList-r16} OPTIONAL, --Need M R wlan-NameList-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, --Need M R sensor-NameList-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL, --Need M R | (a) |  |
| N002 | Nokia (Tero) | This is a mandatory field but not extendible. If we ever have to extend it, there will need to be discussion on what is filled in for the legacy field. Hence, it would be cleaner to just add ellipsis to the reportType CHOICE definition. | Add ellipsis to the CHOICE. | (a) |  |
| I680 | Intel (Sudeep) | Need late and non critical extensions | Add late and noncritical extensions. | (a) |  |
| I672 | Intel (Sudeep) | Could be useful to add an extension marker for future. | Add extension marker. Also for LoggedEventTriggerConfig-r16. | (a) |  |
| X003 | NEC (Hisashi) | As legacy network may receive the SCGFailureInformation from Rel-16 UE, the legacy failureType shall be set in backward compatible manner, i.e. randomAccessProblem when the new failureTypeExt is set to beamFailureRecoveryFailure. Otherwise, the network cannot identify the failure cause appropriately. This is not only SON/MDT issue, but also NR genral issue. | Change the field description to “The field contains the reason for declaring the SCG failure. When the UE includes failureTypeExt set to beamFailureRecoveryFailure, the UE shall set the failureType to randomAccessProblem. then the network discards the contents of the field failureTyp i.e., Otherwise, the UE can chose any of the option for failureType is failureTypeExt is included.” Then, Remove the corresponding NOTE in 5.7.3.3 and 5.7.3.5. | (a) |  |
| I671 | Intel (Sudeep) | Missing Need codes. All of them seem to be one-shot processing in the UE. | Add Need N for all these fields. | (a) |  |
| E203 | Ericsson (Pradeepa) | The presence of ‘…’ in the MeasResultServingCell will create a lot of additional overhead. MeasResultServingCell is included in every sample of logged MDT report and thus this extension marker should be removed to reduce the logged MDT report size. | Remove ‘…’ in MeasResultServingCell-r16 in UEInformationResponse message. | (a) |  |
| Z152 | Z(QZH) | Current RA report only include one set of RA resource configuration, i.e., msg1-FDM, msg1-SubcarrierSpacing and msg1-FrequencyStart, while during one RA procedure UE might use different RA resource with different value of parameters mentioned above, e.g., when switching between CBRA and CFRA, therefore RA report shall be enhanced to be able to include more than one RA resource configuration. The same problem also appears in RLF report involving RA. Also the ‘msg1-” is updated to “prach-” to cover both 2-step/4stepRA case. For more details please refer to the tdoc provided. Some changes are also needed to the procedure text in sections 5.3.10.3, 5.7.10.4 and 6.4 | Please refer to the contribution R2-2002923. | (b) | It seems that R2-2002923 is not available. |
| E202 | Ericsson (Marco) | The field raPurpose is included as part of the RAReport. But it is not clear as to what value is set by the UE for different RA procedures initiated by the UE | Update the field description as provided in R2-20xxxxx to explicitly capture the UE behaviour related to what value if set by the UE for the field raPurpose when the UE performs an RA procedure. | (b) |  |
| Z174 | Z(Zhihong) | The definition of msg1-FrequencyStart and msg1-SubcarrierSpacing in RA-report is the same as the one defined in RACH-ConfigGeneric and RACH- ConfigCommon, no additional information is given, the field description here is unnecessary. | Delete the field description of msg1-FrequencyStart and msg1-SubcarrierSpacing in RA-Report field description. | (a) |  |
| C210 | CATT(Jayson) | For condition handover, there may have some gap between HO initialization and the point where the UE starts to access the candidate Cell(UE will check whether the trigger condition is fulfilled or not during the gap), in our view, it doesn’t make sense to consider this gap in timeConnFailure because this gap is not related to HO failure itself, so we prefer to redefine a new starting point for CHO case when recording timeConnFailure. More addition, UE may try several candicate cell before declaring HO failure, so it’s better to clarify the ending point for CHO case when recording timeConnFailure to avoid any confusion. | Redefine a new starting point and ending point for CHO case when recording timeConnFailure. One contribution is requsted from CATT to give more detailed discussion for this class 2 issue. | (b) | This RIL is about CEF and CHO, and maybe it can be put to R17. |
| I635 | Intel (Sudeep) | This seems to be an uplink IE. Need codes should not be used. | Delete the Need codes for all fields. | (a) |  |
| N037 | Nokia (Tero) | These fields are not following ASN.1 guidelines: BOOLEAN + Need R is unnecessary – either we use BOOLEAN + Need M (or even mandatory fields) or ENUMERATED {true} + Need R. | Use mandatory fields with BOOLEAN, i.e. as shown below: Sensor-NameList-r16 ::= SEQUENCE { measUncomBarPre-r16 BOOLEAN, measUeSpeed BOOLEAN, measUeOrientation BOOLEAN } | (a) |  |
| B201 | Lenovo (Hyung-Nam) | Need codes for all optional fields are not needed. Furthermore, no Setup/Release funcction is needed for bt-NameList-r16, wlan-NameList-r16, sensor-NameList-r16 in the variable. We can simply refer to BT-NameList-r16, WLAN-NameList-r16, Sensor-NameList-r16. | Remove need codes for all optional fields and remove SetupRelease function call for IE BT-NameList-r16, WLAN-NameList-r16, Sensor-NameList-r16. | (a) | This has been discussed in email 960 (raised from Hakan), and has been implemented in the latest 38.331 CR. |
| S955 | Samsung (Sangbum Kim) | There is no such resultSSB-Indexes IE in ASN.1 so we can remove it. | to include neighbouring cell measurements (excluding the resultsSSB-Indexes IE) that became available [Comments]: | (a) |  |
| S956 | Samsung (Sangbum Kim) | Unlike SIB1, the plmn-IdentityList in LoggedMeasurementConfiguration is just one single list and this sentence is already mentioned in SIB1. So we can remove it. | Delete "A PLMN-identity can be included only once, and in only one entry of the PLMN-IdentityInfoList." [Comments]: | (a) |  |
| S957 | Samsung (Sangbum Kim) | According to the ASN.1 structure, there is no resultSSB-Indexes IE within the measResultListNR IE in the LogMeasInfo-r16 IE. So the whole sentence can be removed. | Delete "UE does not include the resultsSSB-Indexes IE, if the measResultListNR IE is included in the LogMeasInfo-r16." | (a) |  |
| S958 | Samsung (Sangbum Kim) | In order to have delta signalling i.e. avoid that entire list has to be included, need M is more appropriate, which is aligned with LTE. | includeBT-Meas-r16 SetupRelease {BT-NameList-r16} OPTIONAL, -- Need RM includeWLAN-Meas-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, -- Need RM includeSensor-Meas-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL -- Need RM ]] [Comments]: | (a) |  |
| B204 | Lenovo (Hyung-Nam) | Need codes for the optional fields interFreqTargetList-r16, cellist (in InterFreqTargetList-r16) are missing. Since IE AreaConfiguration is optional with Need R, Need R for both fields looks appropriate. | Add Need R for the optional fields interFreqTargetList, cellList. | (a) |  |

## 3	Conclusion

Based on section 2 discussions, the following proposals are made as below.

Agreeable proposals are:

[To be added]

Proposals that need more discussions are:

[To be added]

## 4	Tdocs under AI 6.12.4 ASN1 review

- R2-2004409	[Z162-Z166] Correction to connection establishment failure report	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core	Late

- R2-2004410	[Z167][Z169] Correction to RLF report	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core

- R2-2004411	[Z170-171][Z173] Correction to RACH report 	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core

- R2-2004417	Corrections on Sensor Measurement	CATT	discussion	Rel-16	38.331	NR_SON_MDT-Core

- R2-2004528	Corrections to RA/RLF Report_S951_S952	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_SON_MDT-Core

- R2-2004717	[E008] On adding LBTFailure as SCG Failure cause and RLF cause	Ericsson	discussion

- R2-2004718	[E009] On EUTRA previousPCellID in NR RLF report	Ericsson	discussion

- R2-2004719	[E012] On logging TAC in CEF report	Ericsson	discussion

- R2-2004720	[E021] Any cell selection state related logging for OOC event	Ericsson	discussion

- R2-2004721	[E028] On SON-MDT related UE capabilities addition	Ericsson	discussion

- R2-2004722	[E200] On T312 expiry related RLF cause	Ericsson	discussion

- R2-2004723	[E235] UE power savings impact on MDT	Ericsson, CMCC, Samsung	discussion

- R2-2004733	Clarification to RA-report purposes	Ericsson	discussion

- R2-2004884	[S953] Mobility state reporting in RRC connection re-establishment	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_SON_MDT-Core

- R2-2004886	[S954] Logged MDT configuration in UE Inactive AS Context	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_SON_MDT-Core

- R2-2004902	Text Proposal_for_RIL_S481	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005371	Summary of [Post109bis-e][960] ASN1 RIL discussion	Huawei	discussion	Rel-16	NR_SON_MDT-Core	Late

- R2-2005372	[H363] Discussion on UE logging of a MDT entry	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005373	[H365] Discussion on conditions for RLF report	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005374	[H366] Discussion on processing delay requirements	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005375	[H367] Discussion on failedPcellId-EUTRA	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005376	[H368] Discussion on measResult-RLF-Report-EUTRA	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005377	[H369][H370] Discussion on corrections of TAC	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005378	[H371] Discussion on applying the field interFreqTargetList	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2005416	Correction on MDT Configuration [S959]	Samsung	discussion	NR_SON_MDT-Core

- R2-2005468	TP on cat-a proposal2/3 of SON emailDisc[961]	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core	Late

- R2-2005469	[Z168][Z172] Alignment of RA informatiom	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core