3GPP TSG RAN WG2 Meeting #109bis-e                       Draft R2-2003935

E-Meeting, April 20th-30th,2020

Title:	[Draft] LS on early UE capability retrieval for eMTC

Release:	Release 16

Work Item:	LTE_eMTC5-core

Source:	Qualcomm Inc [To be RAN2]

To:	SA2, RAN3, CT1

CC:                          SA3

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

In case of CP CIoT EPS/5GS optimization for NB-IoT, SA2/RAN3 has specified a procedure for (ng-)eNB to retrieve the NB-IoT UE Priority that allows retrieval of additional parameters such as UE radio capabilities from MME/AMF during RRC Connection Setup procedure i.e. upon receiving RRCConnectionRequest-NB in Msg3 from the UE and before sending RRCConnectionSetup-NB in Msg4 to the UE.

RAN2 discussed and concluded the same feature would be beneficial for configuring eMTC UEs which support only CP IoT optimization. RAN2 concluded it would be beneficial if the (ng-)eNB is aware of UE radio capabilities sufficiently early, i.e. after receiving Msg3 before Msg4 transmission, so that the radio configuration for the features supported by the UE can be included in Msg4.

In case of CP CIoT 5GS optimization for eMTC, due to Msg3 size limitation, UE can only send 40 bit LSB of 5G-S-TMSI in RRCConnectionRequest message and with this limitation ng-eNB may not be able to uniquely identify AMF for early retrieval of UE capability information from AMF upon receiving Msg3 from UE. SA2/RAN3/CT1 already introduced truncated 5G-S-TMSI solution for NB-IoT CP CIoT 5GC Optimization Re-establishment. RAN2 would like to ask SA2/RAN3/CT1 on the feasibility of using the same truncated 5G-S-TMSI solution for early UE capability retrieval in case of eMTC connected to 5GC. RAN2 would like to further highlight that there is not enough space to indicate which UE identity (5G-S-TMSI or truncated 5G-S-TMSI) is used in Msg3 thus the same UE identity shall be used by all UEs in the cell.

# 2	Actions

ACTION to SA2, RAN3, CT1:

RAN2 respectfully asks SA2, CT1 and RAN3

- To consider feasibility of introducing enhancements to enable the (ng-)eNB for early UE capability retrieval for eMTC UEs connected to EPC and 5GC in Rel-16, and if confirmed feasible, specify the required changes based on truncated 5G-S-TMSI solution in their specifications.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #110e	01 – 12 June 2020, E-Meeting

TSG RAN WG2 Meeting #111	24 – 28 August 2020, E-Meeting