3GPP TSG RAN WG2 Meeting #109bis-e                                Draft R2-2003935

E-Meeting, April 20th-30th,2020

Title:	[Draft] LS on early UE capability retrieval for eMTC

Release:	Release 16

Work Item:	LTE_eMTC5-core

Source:	Qualcomm Inc [To be RAN2]

To:	SA2, RAN3

CC:                          SA3, CT1

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

In case of CP CIoT EPS/5GS optimization for NB-IoT, SA2/RAN3 has already specified procedure for (ng-)eNB to retrieve UE radio capabilities from MME/AMF during RRC Connection Setup procedure i.e. upon receiving RRCConnectionRequest-NB in Msg3 from the UE and before sending RRCConnectionSetup-NB in Msg4 to the UE.

RAN2 discussed and concluded the same feature would be beneficial for configuring eMTC UEs which support only CP IoT optimization. RAN2 concluded it would be beneficial if the (ng-)eNB is aware of UE radio capabilities sufficiently early, i.e. after receiving Msg3 before Msg4 transmission, so that the radio configuration for the features supported by the UE can be included in Msg4.

In case of CP CIoT 5GS optimization for eMTC, due to Msg3 size limitation, UE can only send 40 bit LSB of 5G-S-TMSI in RRCConnectionRequest message and with this limitation ng-eNB may not be able to uniquely identify AMF for early retrieval of UE capability information from AMF upon receiving Msg3 from UE. SA2/RAN3 already introduced truncated 5G-S-TMSI solution for NB-IoT CP CIoT 5GC Optimization Re-establishment use case. RAN2 requests SA2/RAN3 to consider feasibility of using the same truncated 5G-S-TMSI solution for early UE capability retrieval in case of eMTC connected to 5GC as well.

# 2	Actions

ACTION to SA2, RAN3:

RAN2 respectfully asks SA2 and RAN3

- To consider feasibility of introducing enhancements to enable the (ng-)eNB for early UE capability retrieval for eMTC UEs connected to EPC and 5GC in Rel-16, and if confirmed feasible, specify the required changes based on truncated 5G-S-TMSI solution in their specifications.

- 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #110e	01 – 12 June 2020, E-Meeting

TSG RAN WG2 Meeting #111	24 – 28 August 2020, Toulouse, France