3GPP TSG RAN WG2 Meeting #109bis-e                                Draft R2-2003936

E-Meeting, April 20th-30th,2020

Title:	[Draft] reply LS on suspend indication 5G NAS

Response to:	     LS on suspend indication to NAS (R2-2002214/ C1-201040)

Release:	Release 16

Work Item:	5G_CIoT, LTE_eMTC5-core, NB-IoTEnh3-core

Source:	Qualcomm Inc [To be RAN2]

To:	CT1

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 R2-2003923

# 1	Overall description

RAN2 would like to thank CT1 for their LS on suspend indication to NAS.

RAN2 has discussed the issue and the following was agreed to be captured in TS 36.331 for UEs connected to 5GC:

For UEconnected to 5GC, when UE enters RRC_INACTIVE state, RRC layer indicates “suspend indication” to upper layers [as it is for eLTE in Rel-15]. When UE enters RRC_IDLE in suspended state, i.e., 5GC UP optimization, RRC indicates “idle suspend indication” to upper layers.

In the endorsed RRC CR (R2-2003923), the following text is captured that applies to eMTC and NB-IoT UEs.

5.3.12	UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE

Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall:

……………………

2>	if the UE is connected to 5GC:

3>	indicate the idle suspension of the RRC connection to upper layers;

2> else:

3>	indicate the suspension of the RRC connection to upper layers;

……………………..

# 2	Actions

To: CT1

ACTION: RAN2 respectfully asks CT1 to take above RAN2 agreement into consideration.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #110e	01 – 12 June 2020, E-Meeting

TSG RAN WG2 Meeting #111	24 – 28 August 2020, E-Meeting