3GPP TSG RAN WG2 Meeting #109bis-e                                Draft R2-2003936

E-Meeting, April 20th-30th,2020

Title:	[Draft] reply LS on suspend indication 5G NAS

Response to:	     LS on suspend indication to NAS (R2-2002214/ C1-201040)

Release:	Release 16

Work Item:	LTE_eMTC5-core

Source:	Qualcomm Inc [To be RAN2]

To:	CT1

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 would like to thank CT1 for their LS.

RAN2 has discussed the issue and following was agreed.

For eMTC connected to 5GC, when UE enters RRC_INACTIVE state, RRC layer indicates “suspend indication” to upper layers [as it is for eLTE in Rel-15]. When UE enters RRC_IDLE in suspended state, i.e., 5GC UP optimization, RRC indicates “idle suspend indication” to upper layers.

# 2	Actions

To: CT1

ACTION: RAN2 respectfully asks CT1 to take above RAN2 agreement into consideration.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #110e	01 – 12 June 2020, E-Meeting

TSG RAN WG2 Meeting #111	24 – 28 August 2020, Toulouse, France