3GPP TSG-RAN WG2 #112-e	R2-200xxxx

E-meeting, November 2020

Agenda Item:	6.4.4

Source:	OPPO

Title:	Summary of [Post111-e][708][V2X]: Update of capability CRs (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion of open issues for sidelink capability.

- [POST111-e][708][V2X] Update of capability CRs (OPPO)

Discuss updated RAN1/4 features included in the late incoming RAN1/4 LS, and prepare agreeable CRs.

Intended outcome: Report

Deadline:  Long

# Discussion

In R1-2007326 (NR) / 7328 (LTE), the updated L1 feature list introduce another FG for the simultaneous reception of Uu and PC5

| 15-24 | Simultaneous reception of downlink and sidelink | UE supports simultaneous reception of NR downlink and NR sidelink in a band combination for which the UE indicated simultaneous sidelink and downlink support in a band combination. | 15-1 | Yes | No |  | Per feature set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5-18 | Simultaneous reception of downlink and sidelink | UE supports simultaneous reception of LTE downlink and NR sidelink in a band combination for which the UE indicated simultaneous sidelink and downlink support in a band combination. | 5-1 | Yes | N/A |  | Per feature set |

The ambiguity comes from the granularity, i.e., per feature-set, which is different from the granularity of per-band combination for simultaneous transmission (15-16/5-11).

In LTE-V2X, the simultaneous operation of Uu and PC5 is captured in the following way

BandCombinationParameters-v1430 ::= SEQUENCE {

bandParameterList-v1430			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1430		OPTIONAL,

v2x-SupportedTxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL,

v2x-SupportedRxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL

}

I.e., it is a bit per-Uu-BC-and-per-PC5-BC, for both transmission and reception, i.e., no difference;

Furthermore, on top of the per-Uu/PC5-BC bit, there is a further per-PC5-band indicator, to indicate the Tx/Rx capability of each PC5 band within a PC5 BC

V2X-BandParameters-r14 ::= SEQUENCE {

v2x-FreqBandEUTRA-r14			FreqBandIndicator-r11,

bandParametersTxSL-r14			BandParametersTxSL-r14				OPTIONAL,

bandParametersRxSL-r14			BandParametersRxSL-r14				OPTIONAL

}

I.e., the optionality of the bandParametersT/RxSL-r14 would be used to indicate the capability of per-BC-per-band Tx/Rx capability of PC5.

So in order to clarify the capability granularity of simultaneous transmission/reception:

Q1a: For 15-16/5-11, what is the granularity of the capability?

- Option-1: as in LTE, capture it as a per-Uu-BC-per-PC5-BC bit, and introduce aper-PC5-BC-per-PC5-band bit to indicate whether a PC5 band in a PC5 BC is capable to do TX;

- Option-2: other (if this option is selected, please clarify the granularity)

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Option-1 | We have the same view as Rapporteur that the optionality of bandParametersTxSL/bandParametersRxSL would be used to indicate the capability of per-BC-per-band TX/RX capability. |

Q1b: For 15-24/5-18, what is the granularity of the capability?

- Option-1: as in LTE, capture it as a per-Uu-BC-per-PC5-BC bit, and introduce aper-PC5-BC-per-PC5-band bit to indicate whether a PC5 band in a PC5 BC is capable to do RX;

- Option-2: other (if this option is selected, please clarify the granularity)

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Option-1 | We have the same view as Rapporteur that the optionality of bandParametersTxSL/bandParametersRxSL would be used to indicate the capability of per-BC-per-band TX/RX capability. |

In R1-2007353, RAN1 indicate the need of capability signalling for max data rate

By checking the formula, rapporteur understands the only parameter that is not covered by the existing L1 feature list is the scaling factor, for which the granularity is not very clear, i.e., whether the per-BC is per-PC5-BC or per-Uu-BC-per-PC5-BC;

Q2a: For capability of scaling factor used in the formula of SL max data rate, what is the granularity of the capability?

- Option-1: per-PC5-BC;

- Option-2: per-Uu-BC-per-PC5-BC;

- Option-3: other (if this option is selected, please clarify the granularity)

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Option-1 | It would be simple to define this capability with only SL band consideration for scheduling or implementing UE. |

Q2b: For SL max data rate, is there any other capability to capture besides scaling factor?

- Yes (if this option is selected, please clarify the capability to capture);

- No

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No |  |

- xxx.

- Another issue that may be worth confirming by companies is how to give an appropriate definition for the PC5 fallback band combination introduced earlier into the Spec. The Uu fallback band combination, as per TS 38.306, is defined as “A band combination that would result from another band combination by releasing at least one SCell or uplink configuration of SCell, or SCG”. It seems that this definition in Uu may not be completely applied to PC5 directly, as the concept of “Cell” usually makes no sense in sidelink. On the other hand, it still looks possible to reuse most of the definition in Uu, by simply replacing “Scell/SCG” in Uu definition by “sidelink carrier” in PC5 (considering that “cell” in Uu usually corresponds to a carrier). Thus, the following question is asked to request companies’ check.

Q3: Do companies agree that a PC5 fallback band combination is defined as a band combination that would result from another band combination by releasing at least one “sidelink carrier”?

- Yes;

- No (if this option is selected, please give a specific definition of PC5 fallback band combination)

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Yes |  |

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference