3GPP TSG-RAN WG2 Meeting #111-e		R2-200xxxx

Electronic, 17th Aug. – 28th Aug. 2020

Title:	[Draft] LS to RAN1 on beamSwitchTiming

Response to:	-

Release:	Rel-16

Work Item:	TEI16

Source:	vivo [to be RAN2]

To:	RAN1

Cc:

- Contact Person:

- Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

- 1. Overall Description:

RAN2 has discussed the capability signaling design for TEI16 on “Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of sym224 and sym336”. RAN2 has introduced a new capability signaling for beamSwitchTiming-r16 values of sym224 and sym336 (i.e. Rel-16 capability). Based on RAN2 understanding,

- For Rel-15 UEs,

- if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”.

- if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling,  there is no specified beahviour for value of 224 and 336.

- If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing.

- For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional.

- If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above.

- If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration:

  - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above.

  - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following:

    - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48?

    - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

    - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or is the signalled value in Rel-15 capability signaling?

2. Actions:

- To RAN WG1

- RAN2 kindly requests RAN1 to:

- Confirm whether the above RAN2 understanding is correct or not.

- Provide response to the above Questions.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #112e		2 – 13 Nov. 2020				e-Meeting

TSG RAN WG2 Meeting #113		12 – 16 Feb. 2021				e-Meeting (TBD)