3GPP TSG-RAN2 Meeting # 111-e	R2-2008646

- Electronic, 17– 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1961 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for Early Implementation in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Huawei, CATT, Ericsson, Samsung Electronics Co., Ltd , MediaTek Inc., British Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no description of the UE requirements regarding how the ASN.1 definitions can be comprehended by the UE in the TS 38.331 as well, which is similar to that in Annex F of TS 36.331. And regarding the UE requirements on transfer syntax (ASN.1) comprehension, some misalignment exists between broadcast signalling and dedicated signalling. In addition the previous agreed under-reporting codebook capability should be captured in the annex as it is concluded that these features should be recorded in the Annex C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add description in the Annex on ASN.1 handling to allow early implementation. To capture the previous agreed function of under-reporting codebook capability reporting into the Annex C to record what is allowed to be early implemented. Impact analysis Impacted functionality: Early implementation of features Inter-operability: As the description clarification is similar as in TS 36.331, and so far early implementation is not agreed for features in case of dedicated configuration , no interoperability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of the UE requirements regarding how the ASN.1 definitions can be comprehended by the UE in the TS 38.331 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new), Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR… |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

Annex X (normative):	UE requirements on ASN.1 comprehension

This clause specifies UE requirements regarding the ASN.1 transfer syntax support, i.e. the ASN.1 definitions to be comprehended by the UE.

A UE that indicates release X in field accessStratumRelease shall comprehend the entire transfer syntax (ASN.1) of release X, in particular at least the first version upon ASN.1 freeze. The UE is however not required to support dedicated signalling related transfer syntax associated with optional features it does not support.

In case a UE that indicates release X in field accessStratumRelease supports a feature specified in release Y, which is later than release X, (i.e. early UE implementation) additional requirements apply.

Critical extensions (dedicated signaling)

If the early implemented feature involves one or more non-critical extensions in system information, the UE shall comprehend the parts of the transfer syntax (ASN.1) of release Y that are related to the feature implemented early. This in particular concerns the ASN.1 parts related to configuration of the feature. The UE obviously also has to support the ASN.1 parts related to indicating support of the feature (in UE capabilities).

If configuration of an early implemented feature introduced in release Y involves a message or field that has been critically extended, the UE shall support configuration of all features supported by the UE that are associated with sub-fields of this critical extension. Apart from the early implemented feature(s), the UE need however not support functionality beyond what is defined in the release the UE indicates in access stratum release.

Let's consider the example of a UE indicating value X in field accessStratumRelease that supports the features associated with fields A1, A3 and A5 of InformationElementA (see ASN.1 below).

The feature implemented early is associated with field A5, and can only be configured by the –rY version of InformationElementA. In such case, the UE should support configuration of the features associated with fields A1, A3 and A5 by the –rY version of InformationElementA.

If, however, one of the features was modified, e.g. the feature associated with fieldA3, the network should assume the UE only supports the feature according to the release it indicated in field accessStratumRelease (X).

The UE is neither required to support the additional code-point (n80-vY0) nor the additional sub-field (fieldA3c-rY).

InformationElementA-rX ::=		SEQUENCE {

fieldA1-rX						InformationElementA1-rX				OPTIONAL,	-- Need N

fieldA2-rX						InformationElementA2-rX				OPTIONAL,	-- Need R

fieldA3-rX						InformationElementA3-rX				OPTIONAL	-- Need R

}

InformationElementA-rY ::=	SEQUENCE {

fieldA1-rY					InformationElementA1-rX				OPTIONAL,	-- Need N

fieldA2-rY					InformationElementA2-rX				OPTIONAL,	-- Need R

fieldA3-rY					InformationElementA3-rY			OPTIONAL,	-- Need R

fieldA4-rY					InformationElementA4-rY			OPTIONAL,	-- Need R

fieldA5-rY					InformationElementA5-rY			OPTIONAL	-- Need R

}

InformationElementA3-rY ::=	SEQUENCE {

FieldA3a-rY					InformationElementA3a-rX			OPTIONAL,	-- Need N

FieldA3b-rY					ENUMERATED {n10, n20, n40,

n80-vY0}						OPTIONAL,	-- Need R

fieldA3c-rY					InformationElementA3c-rY			OPTIONAL	-- Need R

}

Non-critical extensions (dedicated and broadcast signaling)

If the early implemented feature involves one or more non-critical extensions, the UE shall comprehend the parts of the transfer syntax (ASN.1) of release Y that are related to the feature implemented early.

If the early implemented feature involves one or more non-critical extensions in dedicated signaling, the network does not include extensions introduced after the release X that are not the parts related to the feature which the UE indicates early support of in UE capabilities. The UE shall anyway comprehend the parts of the transfer syntax (ASN.1) which indicate absence of such extensions.

If the early implemented feature involves one or more non-critical extensions in system information, the SIB(s) containing the release Y fields related to the early implemented features may also include other extensions introduced after the release X that are not the parts related to the feature which the UE supports. The UE shall comprehend such intermediate fields (but again is not required to support the functionality associated with these intermediate fields, in case this concerns optional features not supported by the UE).

## << Start of changes >>

- Annex C (normative): List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table C-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-200335: Correction on usage of access category 2 for UAC for RNA update | 1141 | 2 | Release 15 |  |
| RP-201185: Introduction of signalling for high-speed train scenarios | 1464 | 5 | Release 15 |  |
| RP-201216: Release-16 UE capabilities based on RAN1, RAN4 feature lists and RAN2 | 1665 | 2 | Release 15 | Early implementation part is referring to the aspect covered by RP-201216: Extension of CSI-RS capabilities per codebook type |

- << End of change >>