3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Electronic, 2nd – 13th Nov, 2020

Agenda Item:	8.x

Source: 	Huawei, HiSilicon

Title:	Summary email discussion [621][Relay] of Service continuity

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of the following email discussion.

[Post111-e][621][Relay] Service continuity (Huawei)

Scope: Determine agreeable requirements and scenarios for service continuity, and progress the understanding of service continuity procedures for L2 and L3 relays.

Intended outcome: Summary to next meeting

Deadline:  Long

# Discussion in Phase 1

Below, rapporteur clarifies the terminologies used in this discussion:

Mobility scenario: the scenario due to the mobility of remote UE and/or relay UE.

Path switching procedure: the procedure performed by UE and NW in the mobility scenario.

Service continuity: the requirement of the SID during the path switching procedure in mobility scenario.

- 2.1: Common requirements and scenarios of service continuity

In this section, we discuss the common requirements and scenarios of service continuity for both L2 relay and L3 relay architectures.

- Service continuity requirement

Based on the discussion in Question 28 from offline 603 R2-2008264, the majority see no need to support the service continuity for UE to UE relay. The key point is there is no clear requirement in SA2 for service continuity in U2U relay. Therefore, rapporteur proposes to confirm following majority understanding.

Q1-1: Do you confirm the requirement of service continuity is only for U2N relay, but not for U2U relay, during mobility?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes | Same understanding as rapporteur |
| Xiaomi | Yes | We could focus on U2N in R17 |
| CATT | Yes |  |
| vivo | Yes |  |
| Ericsson | Maybe | We believe that the requirements should not be set by SA2, but we are fine to consider service continuity only for the U2N case. However, we want to clarify that this does not imply, in general, that we prioritize U2N over U2U. [Rapp]: This is only for service continuity. |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |
| Interdigital | Yes | UE to UE can be considered in a future release. |
| Intel | Yes | Agree with majority. |
| Samsung | Yes | Since there is no clear requirement of service continuity for U2U relay, RAN2 should focus on U2N relay. |
| Lenovo, MotM | Yes |  |
| Convida | Yes |  |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | Maybe | Service continuity is a RAN topic, and is also relevant for UE-to-UE relay in case of mobility scenarios. However, we are ok if UE-to-UE service continuity will be considered in future release. |
| Nokia | Yes |  |
| ZTE | Yes |  |

- Mobility scenario: “between indirect and direct” vs. “between indirect and indirect”

Four scenarios are identified as the mobility scenario in U2N relay:

Scenario 1: Mobility between direct (Uu) path and indirect (via the relay) path, in intra-gNB;

Scenario 2: Mobility between indirect (via a first relay UE) and indirect (via a second relay UE), in intra-gNB;

Scenario 3: Mobility between direct (Uu) path and indirect (via the relay) path, in inter-gNB;

Scenario 4: Mobility between indirect (via a first relay UE) and indirect (via a second relay UE), in inter-gNB;

Based on the discussion in Question 27 from offline 603 R2-2008264, the majority see the need to support at least the scenario “between direct (Uu) path and indirect (via the relay) path”. Companies have slightly different views on if the scenario “between indirect (via a first relay UE) and indirect (via a second relay UE)” can be low priority.

Therefore, rapporteur proposes to confirm following majority understanding.

Uu

SL

gNB

Relay UE1

Remote UE

Uu

Uu

SL

gNB

Relay UE1

Remote UE

Uu

Relay UE2

SL

Scenario 1: between indirect and direct                   Scenario 2: between indirect and indirect

Uu

SL

gNB1

Relay UE1

Remote UE

Uu

gNB2

Uu

SL

Relay UE1

Remote UE

Uu

Relay UE2

SL

gNB1

gNB2

Scenario 3: between indirect and direct in inter-gNB  Scenario 4: between indirect and indirect in inter-gNB

Q1-2: Do you confirm R2 should study the mobility scenario of “between direct (Uu) path and indirect (via the relay) path” for U2N relay?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes | We also agree that “between direct and indirect” should have higher priority than “between indirect paths”. |
| Xiaomi | Yes | Due to limited time, prioritization is needed. |
| CATT | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |
| Interdigital | Yes |  |
| Intel | Yes | This is a primary scenario to be considered for service continuity. |
| Samsung | Yes | RAN2 should prioritize the scenario of “between direct path and indirect path” for U2N relay. |
| Lenovo, MotM | Yes |  |
| Convida | Yes |  |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |

Note that the mobility scenario 2 can be considered as a similar case to one of the cases (i.e. from direct path to indirect path) of scenario 1, since the target path in those two are same, i.e. “to indirect path”. If we focus on the scenario 1 to discuss the solutions to cover: case 1) switching to direct Uu cell, and case 2) switching to indirect relay UE, then we almost cover the solutions for both scenario 1 and 2.

Therefore, rapporteur proposes to deprioritize the scenario of “between indirect and indirect”, which seems fine to many companies in the last meeting discussion.

Q1-3: Do you think R2 should deprioritize the mobility scenario of “between indirect (via a first relay UE) and indirect (via a second relay UE)” in the SI phase, which can be studied in the WI phase, if needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes with comments | Our understanding is that few extra specifications is required to support scenario 2/4 on top of scenario 1/3. Then, “de-prioritization” here just means RAN2 starts study from scenario 1/3 and then reuse most design for scenario 1/3 to scenario 2/4, i.e. not intended to skip study of scenario 2/4. Not sure whether rapporteur have the same understanding. [Rapp] Yes, de-prioritization means “not excluded now but to be discussed in WI phase” |
| Xiaomi | Yes |  |
| CATT | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes with comment | We are fine to deprioritize this use case for now and we have also the same understanding as QC. |
| Fraunhofer | Yes, with comment | From our understanding, configurations “between indirect and indirect” might need to be setup differently as compared to the “between direct and indirect”. Further optimizations might also depend on the relaying solution. It would beneficial to identify the differences in design as a part of the study. |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes | We can start from indirect-direct scenarios in SI. |
| Interdigital | Yes | We are fine to go with the majority view at the last meeting. |
| Intel | Yes | No strong view, okay to go with majority. It should be captured in the TR that the scenario is still valid and to be considered in detail during the WI phase. We also think that it is tied to the relay (re)selection criteria aspects. |
| Samsung | Yes | We are fine to deprioritize the scenario of “between indirect and indirect” in SI phase and to figure out that the scenario 2 can be covered with the study of scenario 1 in the WI phase. |
| Lenovo, MotM | Yes but | Indeed, like Rapp and others (QC) we expect to take care of this directly in the WI part assuming no separate study is needed. |
| Convida | Yes | We have the same understanding as QC |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | Yes, with comment | Scenarios 2/4 can be deprioritized for the study phase, but have to be addressed in the work item phase, otherwise you end up with a half baked solution. |
| Nokia | Yes, with comment | Note that Relay reselection, which is an important step for path switch scenarios between indirect paths, is in the scope of the study item (see email discussion [622]) |
| ZTE | Yes |  |

- Mobility scenario: intra-gNB vs. inter-gNB

Compared to the intra-gNB mobility scenario, the inter-gNB mobility scenario may requires the Xn interface coordination and signaling. However, from R2 procedure perspective, the solution for intra-gNB and inter-gNB should be common. For the inter-gNB specific discussion with R3 impacts, it can be done in the WI phase.

Therefore, rapporteur proposes to confirm following approach.

Q1-4: Do you confirm R2 should focus on the mobility scenarios of intra-gNB cases in the study phase, assuming the major difference with inter-gNB cases is R3 issue and to be studied in WI phase, if needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | Essentially, during SI phase, RAN2 can focus on Uu interface which is common for intra/inter-gNB mobility case. And leave any possible RAN3 impact (if any) to WI phase. |
| MediaTek | - | The needed signalling procedure for service continuity is mainly for L2 based relaying. We believe most of the air interface impact should be common for intra/inter-gNB mobility case, even though the inter-gNB mobility may needs some Xn based signalling, which are legacy procedures managed by RAN3: (1)Handover admission signalling on Xn (2)Data forwarding (3)Path switch There is no harm for us to try to document both intra-gNB and inter-gNB mobility procedure in the TR in one shot if the general principle is agreed during the discussion. With this saying, we assume that during the discussion of inter-gNB mobility case, RAN2 should not spend too much time in discussing Xn based signalling, which can be informative from RAN2 perspective. Otherwise our answer to question Q1-4 is yes. [Rapp]: Yes, the baseline with reuse legacy Xn assuming R2 SI not to touch R3 discussion. |
| Qualcomm | Please see comments | We agree RAN2 solution should be common for intra-gNB and inter-gNB case. And we agree that main difference is that inter-gNB has extra RAN3 impacts as list by MediaTek. However, from functionality, inter-gNB case is also a valid/practical case. In our understanding, RAN2 should decide prioritization based on importance of issue in deployment, instead of whether it needs more spec work. If without support of inter-gNB in this release, it seems not a good restriction on relay’s deployment. Thus, we agree with MediaTek that we can try to document both intra-gNB and inter-gNB mobility in the RAN2 TR (i.e. we understand RAN2 TR should not only document intra-gNB mobility). Meanwhile, we are wondering whether rapporteur has plan/consideration to trigger RAN3 discussion. [Rapp]: I assume R3 discussion will be left to WI phase. We may try to summary in phase2 to check companies’ view as: “R2 focus on the mobility scenarios of intra-gNB cases in the study phase. R2 assume the inter-gNB cases can also be supported, considering the major difference with intra-gNB case is the R3 issue and to be studied in WI phase” |
| Xiaomi | Yes | Inter-gNB mobility requires RAN3 study, but there is no TU allocated in RAN3. RAN2 should focus on intra-gNB mobility. |
| CATT | See comments | Inter-gNB case is a general mobility scenario.RAN2 should research intra- and inter- solutions in one shot, and we reckon that the detail RAN3 related work can be left to WI stage. So we prefer to treat them equally. [Rapp]: See the updates to clarify the R3 |
| vivo | Yes | We share the similar view with rapporteur that from RAN2 point of view, there is no obvious difference between intra-gNB and inter-gNB case. And we should focus on Uu interface discussion in SI stage and RAN3 can discuss the necessary Xn interface part in the WI stage, since there is no TU of RAN3 in SI. |
| Ericsson | Yes for intra-gNB but with comment for inter-gNB case. | RAN2 took the agreement that the solution should be common for the inter-gNB and intra-gNB case. However, the inter-gNB case may have an impact also on the Uu interface as when the handover between gNB is performed you may also change the relay UE. Further, the path switch and how you get the PDCP status report is yet not clear to us. All these details need to be clear out and we do see there is not enough time to do it. In addition, for the inter-gNB case the RAN3 impact for the inter-gNB case may be quite substantial. For these reasons, and since we should be also realistic in what we can do with the limited time we have, we prefer to focus only on the intra-gNB case scenario and we may leave the standardization of the inter-gNB during the normative work, if time allows. [Ericsson2] I think that assuming that the impact for the inter-gNB case is only on R3 is not correct as we believe that some further work is also needed on the Uu interface. We are not okay then with the new question as is formulated now. |
| Fraunhofer | Yes, with comment | Any de-prioritization should be contingent on commonality of solutions between intra-gNB and inter-gNB. Nevertheless, we are okay with the updates from the Rapporteur. |
| Sony | Yes | We are ok to start with the intra gNB case even though RAN2 impacts should not be different for both intra and inter gNB case. |
| Spreadtrum | See comments | We think the TR should document intra and inter-gNB mobility scenario, since inter-gNB mobility is also a valid and important case. Thus, we prefer to consider both intra and inter mobility in SI, at least for the common parts. |
| Apple | Yes | But we believe the inter-gNB mobility still be part of work in WI phase once this SI converts to a WI and RAN3 needs to be included. |
| Interdigital | See comments | We should be consistent with what was agreed last meeting – “RAN2 will strive for a common solution between same cell and different cell cases for this scenario. If a common solution is not possible and impacts are found to supporting different cell case, RAN2 works on the same cell case with higher priority.” There are clearly RAN3 impacts to the inter-gNB case, but discussion of these during the SI phase should be limited to stage 2-level discussions only. |
| Intel | See comment | Similar to Ericsson comment above, Aas per the agreement in last meeting, RAN2 will strive to reach a common solution for both intra-gNB and inter gNB case. In order to understand the differences in the solutions, inter-gNB needs to be studied (at a high-level) to be documented like some other companies have noted above. In addition to the RAN3 impacts mentioned, we wonder if there may be minor RAN2 impacts as well. For example, if the relay that the remote UE discovers is supported under a different gNB, we wonder if the remote UE should also move to the different gNB even if it is not the strongest cell [also related to group mobility]. |
| Samsung | Yes | We are fine to focus on the scenario of intra-gNB and figure out the common parts for intra-gNB and inter-gNB. |
| Lenovo, MotM | Yes (for U2N Relay); No (for U2U) | We assume that this question (and others in the section) are for both U2N and U2U relay scenarios. We also assume we discuss “mobility” irrespective of RRC state here and the most general case is OOC. Also, IC for RRC Idle remote UE needs to be accounted. For RRC Connected of course the network controls the decision to change the serving relay. For U2N, the serving cell of both remote and relay UEs must be same since we abide to have only one RRC Connection. For U2U, we think a UE AS protocol should not be required to distinguish between intra and inter-gNB Relays at least for the mobility reasons – this is following Uu principle where the gNB identity is not a decision point for mobility, the UE is not even required to be aware of this. Ericsson’s argument if we understand is that: “when the handover between gNB is performed you (i.e. the remote UE) may also change the relay UE” – we think this is the other way around. If we do not make (remote) UE oblivious of the Relay UE’s serving gNB Id, it will need to take notice post-handover and must change its serving Relay – to another relay that may not be the best radio Relay. We assume Uu AND PC5 are independent interfaces and there is no compelling reason to affect one based on the other. |
| Convida | Yes | We share the same view as Oppo |
| ETRI | Yes | We agree to focus primarily on the Uu interface common in both cases. |
| Huawei | Yes |  |
| Philips | See Comments | Both intra-gNB and inter-gNB should be studied, at least to identify and document the potential differences between the two cases, that can then be further worked upon in detail together with RAN3 during the work item stage. |
| Nokia | No | The decision to deprioritize a scenario should be based on the importance, and not whether it may be difficult to support it. Having the Remote UE being connected to a different gNB than the Relay UE is a real-life scenario, occuring when e.g. a) Relay UE is connected to the network via a different band b) Relay UE is at the edge of its serving cell and the Remote UE moves further away to the coverage of a different gNB We support to work on a common solution for inter-gNB and intra-gNB scenarios. We are not convinced that this has only minor RAN2 impacts, especially in the L2 relay case. RAN3 impacts are out of scope of RAN2. |
| ZTE | See comments | Inter-gNB is also a common case for UE mobility. Although there is not enough time for SI, it doesn’t make sense to de-prioritize one of mobility scenario. And from RAN2’s perspective, the general steps for inter and intra gNB are common, the main difference can be left to RAN3 in WI, RAN2 can take intra-gNB as baseline to discuss UE mobility. |

- Mobility scenario: Group mobility

Based on the discussion in Question 29 from offline 603 R2-2008264, the majority see no need to support the group mobility scenario at least in SI phase. One of the reasons seems it has been deprioritized in SA2 and already touched by other WI (e.g. eIAB).

In addition, in case of the connected relay UE1 moving away, we can treat it as the scenario where remote UE switches from “relay UE1” to “another relay UE2” (or to direct path), which can be covered in the above non-group mobility scenarios.

Therefore, rapporteur propose to confirm following majority understanding.

Q1-5: Do you confirm R2 should deprioritize the group mobility scenario in the SI phase, with that case to be discussed in WI phase, if needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes with comments | In case there is serving gNB change for Relay UE, the remote UE’s context by default would remain at the source gNB if there is no support for group handover. Hence, the target gNB would not be able to deliver traffic or signalling to the Remote UE via the Relay UE as there is no RRC reconfiguration from target gNB to Remote UE. Theoretically, the previous gNB can forward the traffic or signalling to the Remote UE via target gNB and then via the Relay UE, but such complicated operation may be not expected. RAN2 can agree the general principle to support group based mobility for scenario of UE-to-Network relay in case of layer 2 relaying architecture and postpone the work on the detailed mechanism to WI stage considering the limited time for NR Sidelink Relay SI. |
| Qualcomm | Yes with comments | We agree to deprioritize group mobility in SI. However, if RAN2 don’t study it in SI, it shouldn’t be covered in WI as well. In our understanding, there shouldn’t be anything in the WI that is not properly studied and discussed and concluded in SI. Note that different from “switch between indirect and indirect”, grouping mobility needs SI study on its feasibility and conclusion, if RAN2 think it is possible to support it in Rel-17. Thus, if RAN2 agree to deprioritize group mobility in SI, we would suggest to also agree to deprioritize it in WI phase. [Rapp]: In Q1-5, with “that case to be considered in WI phase, if needed”, it means we need to fully discuss the group mobility in WI phase before conclude to support this or not. I guess we cannot make the WI decision in SI phase anyway. So, maybe it is safe to leave it vague as “to be discussed, if needed”. |
| Xiaomi | Yes | We see some benefit of group mobility in wearable device scenario. But this SI mainly focus on coverage extension scenario, where the benefit is vague. |
| CATT | Yes with comments | It has been deprioritized in SA2 and we also have alternative solution proposed by rapporteur, so we’d like to deprioritize it in the SI phase. For WI phase, we have the same view with QC. |
| vivo | Yes, with comments | We agree to deprioritize group mobility in SI. In our understanding, there is no need for L3 relay architecture to support group mobility since only relay UE is in RRC-connected mode and legacy mobility procedure may be enough. But for L2 relay architecture, both relay UE and remote UE are in RRC-connected mode. When relay UE and remote UE move together, e.g. my phone and my wearable devices, it may be difficult to guarantee a good user experience for relayed services since relay UE and remote UE will perform HO procedure independently. |
| Ericsson | Yes with comment | Our understanding is the same as QC [Ericsson2] Given the new formulation on the question, our preference would be to say that group mobility “may” be discussed in WI phase, if needed. This is because we do not want to hint that group mobility need to be added in the WI phase by default. |
| Fraunhofer | Yes |  |
| Sony | Yes with comments | We have same understanding as Mediatek. |
| Spreadtrum | Yes with comment | We have the same view with vivo. We agree to deprioritize group mobility in SI. But we think support of group mobility is essential for L2 relay for some cases. |
| Apple | Yes | Agree with MediaTek and we can postpone this to WI phase. |
| Interdigital | Yes | We think the scenario is an important one for the case of UE to NW relay and maintaining service continuity when relay UE performs HO. However, we agree to deprioritize this in the SI phase, and come back to it in the WI phase if needed. |
| Intel | Yes, with comment | We are OK to go with majority view. However, we think this is somewhat tied to the previous question. We may need group mobility as one of the solutions to support inter-gNB case as described above. |
| Samsung | Yes with comment | We prefer to deprioritize the group mobility scenario in this release. |
| Lenovo, MotM | Yes | Group mobility can be deprioritized. |
| Convida | Yes | We share the same view as QC |
| ETRI | Yes | We agree to deprioritize group mobility in SI. However, group mobility is a necessary feature for the optimization of signalling overhead. |
| Huawei | Yes |  |
| Philips | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |

- Mobility scenario: Dual connection

Based on the discussion in Question 13 from offline 603 R2-2008264, the majority prefer to focus on the single connection case in U2N relay in SI phase. This is somehow related to the pending proposal in last meeting.

Revised Proposal 11: For UE to NW relay, RAN2 assumes the remote UE has an active end-to-end connection via only a single relay UE or via Uu at a given time. The remote UE can have a direct Uu connection or a connection via a single relay UE, but these two connections should not be active at the same time. Mechanisms for ensuring service continuity (e.g. during path switch) are not precluded.

Please note that, in this email discussion, we only intend to handle the mobility/service continuity related proposals, rather than the general scenario on this.

As to the point of not excluding “dual connection during path switching procedure”, rapporteur would like to clarify before companies input their views. In 3GPP, when we achieved the “service continuity in mobility”, we first try to “avoid data loss/guarantee in-order delivery” in the initial release (see R15 NR mobility procedure). Then, we can further enhance the “0ms interruption” in the further release (see R16 NR mobility enhancement).

For L2 relay, if we use the HO procedure as baseline for mobility in relay, the DAPS can be considered as baseline for the dual connection scenario, if needed. For L3 relay, in case direct to indirect switching, it is not clear how to coordinate between gNB and remote UE on the time to releasing the link with source cell.

Therefore, with many issues to be resolved, rapporteur proposes to go with the majority’s view at last meeting discussion.

Q1-6: Do you confirm R2 should deprioritize the scenario where remote UE has the simultaneous transmission of one QoS flow with both source and target path for the optimization of “almost 0ms interruption” (e.g. DC-like mobility, DAPS-like mobility and make-before-break-like mobility), in both L2 and L3 relay?

NOTE 1: This does not exclude the case remote UE has one CP RRC connection with direct link and one UP data connection via relay at the same time, for L3 relay.

NOTE 2: For “make-before-break-like mobility”, we here refer to the AS layer MBB defined in LTE mobility (i.e. UE breaks the source link after connects to the target link), rather than the PDU session level “make-before-break” which is in SA2 scope.

NOTE 3: This is the scenario/requirement discussion of “simultaneous transmission for 0ms interruption”, which should be common for L2/L3.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes | We think this question may be only applicable to L2 Relay. We assume the dual connection operation from AS layer perspective should be transparent to L3 Relay operation. |
| Qualcomm | Please See comments | First, we are a bit confused by rapporteur’s description on relationship of “dual connection” and “almost 0ms interruption”. We don’t support DAPS in DC in this release, and we don’t support “dual connection” for SRB in DAPS. We understand rapporteur’s intention, but think the term “dual connection scenario” may cause unnecessary discussion. Secondly, we agree with MediaTek that this question is better to separate between L2 and L3 relay: - For L2 relays, agree to not consider DC and to prioritize normal HO in SI. DAPS-like HO and CHO-like can be discussed later if time permits in Rel-17. - For L3 relays, we think it is allowed to have “dual connection” with both Uu link and PC5 link for a CONNECTED remote UE in L3 relay, which is a common scenario in NR Rel-16 V2X (i.e. one CONNECTED sidelink UE has connections to both gNB and one or more peer UE). Thus, we think it is not correct to preclude “dual connection” for L3 relay. Meanwhile, we are not sure whether there is handover notion for L3 relays. If this is company’s common understanding, we don’t need to discuss this question for L3 relay. [Rapp]: See NOTE1 above and the updates on the question. For L3 relay, in intra-gNB direct to indirect switching, this question is to exclude the case one QoS flow is transmitted on both the source direct link and target indirect link to the same gNB. Based on above, we suggest to remove “dual connection scenario” and only mention L2 relay in summary proposal, e.g. RAN2 deprioritize the DAPS-like (and/or CHO-like) handover between source and target path in L2 relay, including HO between direct and indirect path and HO between indirect paths. [Rapp]: Please see NOTE3 above. |
| Xiaomi | Yes | Due to the half-duplex mode, 0ms interruption may not be possible even with dual connectivity. So we don’t see the need to introduce dual connectivity. |
| CATT | Yes | We have the same view with MTK. |
| vivo | See comments | We share similar view with Qualcomm. Dual connection scenario will have different use cases for L2 and L3 relay. - For L2 relay, dual connection is mainly for 0-ms HO interruption time like DAPS, which is an optimization and can be considered in later release. - For L3 relay, an RRC-Connected remote UE can also have a relay link which is similar as legacy R16 V2X UE and different services can be carried on these two links separately. [Rapp]: See NOTE1 above. Based on above comments, we propose to deprioritize dual connection scenarios for only L2 relay. |
| Ericsson | See comment | Our understanding is that make-before-break is essential for L3 and useful for L2. It should be rather simple to apply to sidelink what has been already specified for MBB in 23.501. [Rapp]: See NOTE2 above. Please note that “PDU session level MBB” in 23.501 should be SA2 issue. Here we only discuss the AS layer make-before-break mechanism, which is a different thing as in 23.501. For these reasons, we should not deprioritize/exclude MBB from the TR. |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |
| Interdigital | Yes | Agree with Mediatek |
| Intel | Yes | We think this is optimization and is not necessarily needed for the purpose of supporting service continuity and hence can be deprioritized. |
| Samsung | Yes | We are fine to deprioritize dual connection scenarios. |
| Lenovo, MotM | Yes | Agree with the update (of question and notes) from Rapp. |
| Convida | Yes | Share the same view with Mediatek |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | See comments | Agree with Vivo |
| Nokia | Yes | We are OK with the deprioritization, but the NOTES shall be part of the agreement. |
| ZTE | Yes | We should focus on the basic procedure (normal HO procedure) and left the optimization(CHO, DAPS) to WI phase or future release, if possible. |

- 2.2: L2 relay: solution for service continuity

NOTE: we only discuss the mobility case when relaying data, rather than the case with remote UE in IDLE, which falls into the scope of relay selection discussion.

NOTE: Following questions focus on the intra-gNB scenario. The inter-gNB scenario can be considered after some progress on Q1-4.

- General approach

Base on the SA2 TR 23.752 (Sec. 6.29, Solution #29: Service continuity via L2 UE-to-Network Relay), “The access stratum procedure that the Remote UE, UE-to-Network Relay and served NG-RAN node for handover is defined by RAN2.”

In L2 U2N relay, from remote UE and gNB point of view, there is RRC connection during relaying data, the handover procedure defined in NR seems the straight forward approach. With the PDCP re-establishment or data recovery procedure and the End-Marker in SDAP, the lossless and in-order data delivery are maintained during path switching procedure.

Therefore, rapporteur asks if companies can confirm the handover procedure as baseline for the approach for service continuity. This leaves the mobility of L2 relay as NW control, and excludes the case remote UE switches to target cell or target relay UE without the target configuration provided by gNB.

Q2-1: Do you confirm that L2 U2N relay uses the R15 NR HO procedure as the baseline AS layer solution to guarantee service continuity (i.e. gNB hands over the remote UE to a target cell or target relay UE, including the HO preparation (if needed), HO command to remote UE, remote UE switching to the target, and HO complete message, similar to legacy procedure)?

Note: this excludes the case that remote UE executes the path switching procedure without NW control/decision, but does not exclude the possible enhancement like CHO.

| Company | Yes/No? | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes | Suggest to make it clear to use Rel-15 “normal HO” as baseline (i.e. not DAPS-like or CHO-like), and capture a flow chart in TR [Rapp]: we consider R15 as baseline here, but not explicitly exclude CHO for now. Please see the udpates. |
| Xiaomi | Yes | We think R16 mobility mechanism should also be considered, e.g. CHO. [Rapp]: Can be considered but not baseline. |
| CATT | Yes | In addition, gNB shall send the RRC reconfiguration message to the relay UE. And for the case that the remote UE and relay UE are in the same cell, there isn’t HO preparation procedure. [Rapp]: See the updates. Even in that case, gNB does prepare something but without Xn signalling. |
| vivo | Yes |  |
| Ericsson | Yes | Agree with QC that Rel-15 handover should be used and we should leave out for the time being all the optimization we did for Rel-16 e.g., CHO. However, when capturing the procedure in the TR, we may need also to cover the case where the source and target nodes are within the same cell and in different cells. Based on the cases the legacy handover procedure may not be applied fully but some steps can be make optional (e.g., handover preparation). [Ericsson2] For the case of path switch, we believe that there is no handover procedure as such since, for the single gNB case (scenario 1 and 2), the procedure is a simple RRCReconfiguration and there is no HO command. |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes |  |
| Interdigital | Yes with comments | Rel15 HO can be a baseline, but CHO should also be considered. Similar to the traditional HO procedure being very similar in the context of relays, the CHO procedure should also be very similar to Rel16 Uu CHO procedure. |
| Intel | Yes | For L2 U2N relay, the NR HO procedure can be used. |
| Samsung | Yes |  |
| Lenovo, MotM | Yes |  |
| Convida | Yes |  |
| ETRI | Yes | We agree the NR15 NR HO procedure as a baseline. Additionally, mobility enhancements like CHOs help improve service continuity. |
| Huawei | Yes |  |
| Philips | Yes, with comments | Agree with Interdigital |
| Nokia | Yes |  |
| ZTE | Yes |  |

With the above question, companies are invited to provide the views on the basic steps of the path switching procedure for L2 U2N relay.

- Procedure for Case 1: Switching to direct Uu cell

In the case 1, where remote UE switches from indirect to direct link, the legacy NR Uu HO procedure and legacy RRC message can be reused.

Q2-2: For service continuity for L2 U2N relay, do you agree with the following procedure, in case of remote UE switching to direct Uu cell?

  - Step 1: Measurement and reporting

  - Step 2: HO decision of switching to a target cell by gNB

  - Step 3: RRC Reconfiguration message as the HO command to remote UE

  - Step 4: Remote UE performs RA to target cell

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in HO command.

  - Step 6: The data path switching

| Company | Yes/No? | Comments (please indicate which step may need more discussion/clarification) |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Qualcomm | Yes | Suggest to capture a flow chart in TR [Rapp]: Sure. |
| Xiaomi | No | These steps are only applicable to non-CHO like switching procedure. We think CHO like switching procedure should be considered especially in U2N case. The indirect link is more vulnerable than direct link, due to the smaller coverage of sidelink. Furthermore, step 1 may not be essential, if we allow gNB trigger the switching blindly as in Uu. |
| CATT | Yes with comments | For details, we propose below suggestions: 1)Step 2 is optional. [Rapp]: This is the gNB implementation to select the target cell. Even in intra-gNB case, the decision is still required. 2)After step 5, gNB shall send the RRC reconfiguration message to the relay UE. [Rapp]: One possible way is relay UE can detect the leaving of remote UE anyway, then relay UE will release this remote UE’s context/configuration itself. But, we can discuss the details in WI phase. Let’s see if other companies prefer this also. |
| vivo | Yes |  |
| Ericsson | Yes with comment | General procedure is okay, and we can further work on the details in the next meeting e.g., like the flow chart to add in the TR. See comment in Q2-1. [Ericsson2] For the case of path switch, we believe that there is no handover procedure as such since, for the single gNB case (scenario 1 and 2), the procedure is a simple RRCReconfiguration and there is no HO command. Further, one missing step is that the network may decide to reconfigure also the Uu link with the relay UE and thus it may send a separate RRCReconfiguration. Another missing step is the release of the PC5 link that it may (or not happen). Therefore, this it should be like an optional step. |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes with comment | In general, we agree with the sequence of HO operations. But for SI, we can further clarify how the indirect path is cleaned up (e.g., which UE decides to release the PC5 link) and how to handle the packets already delivered to the relay UE when path switch happens. |
| Interdigital | Yes with comments | This can serve as a baseline. However, CHO based switching can be covered with minor differences, and has some benefits (as mentioned by Xiomi) |
| Intel | Yes |  |
| Samsung | Yes |  |
| Lenovo, MotM | Yes | We also agree with the questions raised by Apple. |
| Convida | Yes | Suggest to add a call flow in the TR |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | Yes, with comments | We agree with Interdigital. |
| Nokia | Yes | Fine as general procedure |
| ZTE | Yes with comments | From our perspective, HO decision can also be made by remote UE itself, if configured by network. |

- Procedure for Case 2: Switching to indirect relay UE

In the case 2, where remote UE switches indirect link, this procedure covers both the mobility scenario from direct path to indirect path and the mobility scenario from indirect path to indirect path. If we can agree to reuse the basic HO procedure for L2 relay, following procedure seems straightforward as the baseline.

Q2-3: For service continuity for L2 U2N relay, do you agree with the following procedure as baseline, in case of remote UE switching to indirect relay UE?

  - Step 1: Remote UE reports one or multiple candidate relay UE(s), after measures/discoveries the candidate relay UE(s).

    - remote UE may filter the appropriate relay UE(s) meeting higher layer criteria when reporting, which is studied by separate topic.

    - This may include the relay UE’s ID and SL RSRP information, where the measurement on PC5 details can be left to WI phase.

  - Step 2: HO decision of switching to a target relay UE by gNB, and target (re)configuration on relay UE optionally (like HO preparation).

  - Step 3: RRC Reconfiguration message as the HO command to remote UE

  - Step 4: Remote UE establishes PC5 connection with target relay UE, if the connection has not been setup yet.

    - The exact time to execute this step is not restricted (e.g. can be also before step1)

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in HO command.

  - Step 6: The data path switching

Note that the CHO manner can be considered as one enhancement of this procedure, which can be discussed in WI phase. Note that the similar solution was captured in the LTE TR 36.746 sec. 5.1.2.5.

| Company | Yes/No? | Comments (please indicate which step may need more discussion/clarification) |
| --- | --- | --- |
| OPPO | Yes | The description above is general enough to avoid discussion on specific issues, so can be used to conclude the text for SI/TR. Further discussion can be left to WI phase. |
| MediaTek | Yes |  |
| Qualcomm | Yes with one question | We have one question for clarification: for step 4, do we need to establish PC5 security and authentication? Our understanding is Yes, but not sure whether each company has the same understanding. [Rapp]: Yes. Those essential steps (e.g. security) as in R16 PC5 connection establishment are covered in step 4. |
| Xiaomi | No | CHO was introduced in Uu to improve robustness in R16. Similar problem could also be observed during switching to indirect relay UE. If RLF is triggered, there is no re-establishment procedure via relay. UE could not recover connection with NW and service continuity could not be supported. We think CHO like switching procedure should also be considered in SI as baseline. [Rapp]: Note that CHO is not excluded in the above procedure, but not considered as baseline. |
| CATT | Yes with comments | For details, we have below questions: 1) Remote UE reports one or multiple candidate relay UE(s). whether it means relay selection is performed by gNB? [Rapp]: Yes. But, we’d better call it target decision. 2) For step2, we wonder why target (re)configuration on relay UE is optional? [Rapp]: The target (re)configuration on relay UE could be send later steps, as long as before relay UE forwarding remote UE’s RRCReconfigurationComplete. |
| vivo | Yes, with comments | In step 1, the candidate relay UE(s) reported by the remote UE should have the admission ability for all or most of remote UE services, e.g. which can be interacted via discovery procedure. But our concern is whether discovery procedure should also consider RAN ability, e.g. how much RAN processing/buffer ability is needed for a specific relayed service. We agree that detailed solutions can be further discussed in WI phase. |
| Ericsson | Yes with comment | We generally agree with it but we think that step 4 cannot be done in parallel with step 1 because the UE did not get yet the HO command from the gNB. Since is the gNB that decides, it would be quite inefficient to setup a possible PC5-RRC connection with all the possible relay UE candidate in the meanwhile to know which one has been chosen by the gNB. Maybe good to remove the note on step 4 or clarify that if the PC5 connection between remote UE and relay UE is already existing, then step 4 is optional in the procedure. [Rapp]: Thanks for the comments. Please see the updates. For now, the possibility of step4 is in parallel with step1 is not excluded. Let’s wait for more companies’ view on this. [Ericsson2] For the case of path switch, we believe that there is no handover procedure as such since, for the single gNB case (scenario 1 and 2), the procedure is a simple RRCReconfiguration and there is no HO command. Regarding step 4, how the step 4 can be done together with step 1? When the UE report the measurement or the candidates relay UE, the network will start the path switch procedure and, at the same time, will send multiple configuration to allow the remote UE establishing PC5 connection with all the possible relay UE? Or? This does not look that feasible to us. |
| Fraunhofer | Yes |  |
| Sony | Yes |  |
| Spreadtrum | Yes |  |
| Apple | Yes with comments | We think the steps are in general fine. We have the following two comments: 1_Reading the timing of PC5-RRC connection establishment, we agree with the text originally proposed by the rapporteur. The remote UE does not need to wait for HO command to do this. It is possible that in step 1 UE only reports one relay UE candidate to the gNB, so UE can establish PC5 link in parallel or even before reporting. 2. Regarding step 2, for the optional relay UE configuration, I think this is possible if the chosen relay UE is in RRC_CONNECTED. Otherwise, if the relay UE is still unknow to gNB, then this configuration cannot be done. |
| Interdigital | Yes | Agree with Ericsson, that if step 4 is performed before, it would mean that a PC5-RRC connection already existed. |
| Intel | Yes |  |
| Samsung | Yes |  |
| Lenovo, MotM | Yes | We think that Step 1 and 4 are independent. The remote UE had established a PC5 RRC connection with a future relay for any reason like two SL UEs not necessarily for remote-relay relationship. |
| Convida | Yes | Further discussion can be left to WI phase. |
| ETRI | Yes |  |
| Huawei | Yes |  |
| Philips | Yes, with comments | We agree with Apple with respect to timing of step 1. In addition, we think that some reporting of the candidate relay UEs will need be taken into account as well, incl. RAN abilities. |
| Nokia | Yes | We generally agree to having this as baseline, and then to discuss actual functionality for the WI |
| ZTE | Yes with comments | See our comments in Q2-2 |

In the HO procedure, the HO command from gNB to remote UE should at least include the target identity and the RRC configuration at target path. The target identify is used for remote UE to determine the target relay UE to initiate the PC5 connection. The target configuration is used for remote UE to setup the connection and radio/RLC bearer at the target path directly, instead of using the initial connection setup procedure. Those target configuration may include target PC5 link configuration, as needed. The details can be further discussed in WI.

Q2-4: Which information should be included in the HO command from gNB to remote UE, in case remote UE switching to indirect relay UE?

- Part 1: Identity of the target relay UE;

- Part 2: Target configuration, e.g. including the Uu/PC5 L2 configuration and adaptation layer configuration, if needed (with details to be discussed in WI phase);

- Part 3: (Please add if any) HO condition, which is used to trigger switching to indirect relay UE;

| Company | Which parts? | Comments |
| --- | --- | --- |
| OPPO | 1/2 | For the configuration in Part-2, it may further include the configuration of Uu-bearer (PDCP, SDAP). Other details can be left to WI phase. [Rapp]: Sure, Uu L2 config is included. |
| MediaTek | 1/2 |  |
| Qualcomm | 1/2 | Beside OPPO’s points, we think RAN2 may need to study basic security (e.g. key refresh) mechanism during HO with indirect path in study item. [Rapp]: The security related can be covered by the Uu RB configuration (e.g. reestablishPDCP¸ and SecurityConfig in inter-gNB case). |
| Xiaomi | 1,2,3 | 3 is essential to support CHO like switching procedure. |
| CATT | 1 and 2 with comments | For “Part 2: Target configuration, e.g. include the PC5 L2 configuration and adaptation layer configuration, if needed;”. We reckon that the adaptation layer configuration should be configured to relay UE, not to remote UE. [Rapp] Here we are talking about the HO command, which is send to remote UE. For adaptation configuration to remote UE, we use “if needed” to clarify this depends on the protocol stack discussion. |
| vivo | 1/2, also see comments | Additionally, part 2 should also include the configuration of Uu bearer(s), e.g. PDCP and SDAP, and potential Uu dedicated configurations, e.g. measurement and security. |
| Ericsson | 1,2 | For the target configuration, we can keep it general and leave the details on what “target configuration” means in the normative phase. |
| Fraunhofer | 1,2 |  |
| Sony | 1,2 |  |
| Spreadtrum | 1,2 |  |
| Apple | 1,2 |  |
| Interdigital | 1, 2, 3 |  |
| Intel | Parts 1, 2 |  |
| Samsung | 1, 2 |  |
| Lenovo, MotM | 1, 2 |  |
| Convida | 1/2, see comment | This should not exclude additional information for consideration in the WI phase. |
| ETRI | 1,2 | Parameters for improving service continuity can be added at the baseline. |
| Huawei | 1,2 |  |
| Philips | 1,2,3 |  |
| Nokia | Postpone | The question is strongly related to the discussion and assumption in email discussion #622 (Relay selection and reselection), where the relay reselection may be left up to UE implementation. The reasons for triggering path switching are manifold, e.g. triggered at relay UE if Uu link is degrading or triggered at remote-UE if PC5 link is degrading. More study is needed and it’s too early to decide without knowing the impact (e.g. security context while path switching etc.) and without coordination with #622 Relay selection and reselection |
| ZTE | 1/2 |  |

- 2.3: L3 relay: solution for service continuity

NOTE: we only discuss the mobility case when relaying data, rather than the case with remote UE in IDLE, which falls into the scope of relay selection discussion.

- General approach

For L3 relay, there are two general approaches to proceed on the service continuity. Since the gNB is not aware of the relayed remote UE, upper layer (e.g. by application layer) solution can be the baseline anyway. In addition, we may need to achieve some AS layer solution, like L2 relay, for service continuity.

Therefore, before we discuss the detailed AS layer solution, rapporteur would like to ask companies’ view on the general approaches.

Q3-1: Which approach do you prefer for service continuity in L3 U2N relay?

- Approach 1: No AS layer solution to guarantee the service continuity, and leave it to the upper layer (e.g. application layer) solution

- Approach 2: R2 study the AS layer solution to guarantee the service continuity

| Company | Approach? | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| MediaTek | 2 | We think that only AS layer approach can have the possibility to meet the requirement of service continuity. The upper layer approach (e.g. IMS session continuity) is actually UE centric handover and transparent to network. As such, there is no guaranteeing for the service continuity. Meanwhile we see a need to clarify the requirement of service continuity, e.g. lossless data during handover. |
| Qualcomm | At least 1 (as baseline) | We believe approach 1 is sufficient for L3 relay. Meanwhile, we also understand some companies want to study some of its enhancements (i.e. approach 2). If companies really want to study, we can accept approach 2 if Approach 1 is agreed and approach 2 is agreed as enhancement with low priority. |
| Xiaomi | 2 | We agree major solution should be specified in upper layer. But it’s too early to exclude AS involvement. AS may provide support to upper layer solution. |
| CATT | 1 | The lasted SA2 TR clarified the definition of service continuity as below:” According to the definition of service continuity in TS 22.261 [3] and TS 23.501 [6], it can be seen that "service continuity" is different from "session continuity" by definition, and service continuity can be achieved at application layer regardless of IP address preservation: - For Mission Critical Service in Public Safety, service continuity can be achieved by the application layer mechanism, e.g. as described in Annex B in TS 23.280 [29]. - For commercial IMS use cases, service continuity can be achieved using mechanisms described in TS 23.237 [30]. - For commercial use cases with application layer out of 3GPP scope (e.g. non IMS), service continuity can be achieved using similar way, e.g. QUIC. It is noted that all of the above application layer mechanisms can be reused for Layer-3 UE-to-Network Relay without any enhancements in this study item.” From our current analysis, approach 1 is feasible for the current stage. And in consideration of time limitation, we reckon it is a better choice to deprioritize approach 2. |
| vivo | 1 | We think approach 1 is enough for L3 relay, at least in this first release. With further progress of SA2, RAN based solutions may be discussed, if most companies want to do it. |
| Ericsson | 1 (baseline) but ok with 2 only for necessary solutions | SA2 already have solutions to allow service continuity for L3 and thus this should be the baseline. For the approach 2, we are fine study/capture already some solutions now (e.g., MBB for L3 and L2) but those should be only limited to what is really necessary. |
| Fraunhofer | 1, 2 | Agree with QC, Ericsson |
| Sony | 1 | We think 1 should be sufficient but ok to study further. |
| Spreadtrum | 1 |  |
| Apple | 1 |  |
| Interdigital | 1 | We think 1 should be a baseline for initial SI phase. |
| Intel | 2 | As per TR 23.752, service continuity can be achieved at application layer regardless of IP address preservation, however, the AS layer solution for service continuity could be studied by R2. |
| Samsung | 1 | We think that the service continuity should be rely on the application layer solution which works with any L3 U2N relay architecture. |
| Lenovo, MotM | 1 |  |
| Convida | 1 |  |
| ETRI | 1 |  |
| Huawei | 1 |  |
| Philips | 2 | Also layer-3 relays suffer from mobility/signal degradation issues, which may lead to switching between direct and indirect path or relay reselection. It should be studied which mechanisms are needed for this. |
| Nokia | 2, but comments | It is not clear what approach 2 means here. Our understanding is that in L3 solutions the service continuity is provided by upper layers. However, RAN2 should investigate if AS can provide assistance to enhance the performance (e.g. with measurements). If Approach 2 (AS layer solution) means this, then we support Approach 2. |
| ZTE | 1 |  |

- Detailed solution

In case R2 agrees to work on the AS layer solution to achieve better performance on the service continuity, followings are to discuss if any AS layer solution can be used and agreed for the service continuity in L3 U2N relay.

Regardless if any AS layer solution is used, we should first clarify the basic path switching procedure, even without any service continuity specific solution/enhancement.

The assumed basic path switching procedure for L3 U2N relay in AS layer (with no service continuity specific enhancement):

- Step 1: Remote UE (re)selects the target cell or target relay UE (where the discovery procedure before target selection is studied by separate topic); (where the selection procedure/criteria among candidate cells and candidate relay UEs is studied by separate topic) - Step 2: Remote UE establishes the new connection at the selected target cell or target relay UE, like the initial connection establishment (after releasing the source path); - Step 3: The data path switching.

Taking the above basic procedure into account, the AS solution/enhancement can be discussed to modify this procedure somehow for the purpose of service continuity, if R2 agrees approach 2 in Q3-1.

Q3-2: If R2 agrees on Approach 2, which AS layer solution can be used to guarantee the service continuity, by modifying the above basic path switching procedure for L3 U2N relay?

- Option 1: relay UE acts as the “gNB” in N3IWF L3 architecture

  - Proposed in R2-2007041;

  - Taking indirect to direct switching as example, the peer PDCP entity of remote UE is located at relay UE. Source relay UE may perform SN Status Transfer and Data Forwarding procedure to gNB.

  - Note that the measurement report from remote UE to relay UE, and the HO request and data forwarding from relay UE to gNB is newly introduced compared to the legacy HO concept. And, the RRC message from gNB to remote UE via relay UE needs to be clarified on the feasibility for L3 relay.

- Option 2: (Please add if any)relay/gNB could provide AS control parameters to trigger switching, similar to discovery control.

- Option 3: Study/capture make-before-break like solutions for L3.

- …

| Company | None or Option x? | Please provide comments to explain whether/how the option achieves the service continuity |
| --- | --- | --- |
| MediaTek | Option 1 alike | A likely enhanced AS layer procedure for service continuity at N3IWF based L3 Relay can be: Step 1: Radio measurement is configured from Relay UE to Remote UE over PC5-RRC. Remote UE performs measurements following that measurement configuration. Step 2: Remote UE performs measurement report to Relay UE over PC5-RRC. Step 3: Relay UE sends to HO request to gNB over Uu, which like the Xn based HO request. Step 4: gNB acknowledges the Relay UE by HO ACK over Uu, which like the Xn based HO ACK. Step 5: Relay UE sends to HO command to the Remote UE over PC5 via PC5 RRC, which like the Uu based HO command. In the rest steps, Remote UE follows the legacy procedure to reach the gNB The above description may be the similar to option 1. |
| Qualcomm | None | We are not sure how option 1 can work: 1) Relay PDCP (PC5) and gNB PDCP (Uu) are different. 2) the forwarding of the RRC message from gNB via Relay is essentially making it a L2 Relay. For MediaTek’s solution, we are not sure how data forwarding can work to ensure loss-less because relay’s PDCP and gNB’s PDCP are different. |
| Xiaomi | Option 2 | Relay/gNB could provide AS control parameters to trigger switching, similar to discovery control. Due to lack of radio condition information, upper layer may trigger switching too late, which may result in connection failure. AS control parameters could avoid this problem. |
| CATT | See comments | See our answer for Q3-1. |
| vivo | None | Option 1 and proposals by MediaTek and Xiaomi will require relay UE as a gNB role, e.g. control remote UE completely, support Xn interface of CP and UP and so on, which is a little complicated for this first release. |
| Ericsson | Option 1, Option 3 | Regarding Option 1, our understanding is that the solution proposed is agnostic to the presence of the N3IWF. Therefore, it would be good to delete the word “N3IWF” from the option. |
| Fraunhofer | Option 1 |  |
| Sony | Option 1 | We think that option 1 can work but it will require guidance from SA2. |
| Apple | None | We do not see any clear solution to make this Approach 2 work. |
| Interdigital | None | We think approach 2 should be down-prioritized, given the complexity involved with the solutions. |
| Intel | Option 2/3 | We think that some form of options 2 and 3 can be supported to aid in service continuity. The remote UE could maintain both connections (over Uu and relay paths) and switch to the other link in a make-before-break manner. As per agreement in last RAN2 meeting, “RAN2 leaves control plane protocol stacks of L3 UE-to-NW relay to SA2. And RAN2 TR adds a reference to SA2 TR. FFS if there is RAN2 impact to support the related control plane procedures.” Therefore, control plane procedures for L3 U2N relay path switching could be studied for RAN2 impact. |
| Samsung | None | We think Approach 2 should be deprioritized. |
| Lenovo, MotM | None | Agree with Interdigital. |
| ETRI | None |  |
| Huawei | None |  |
| Philips | Option 1/2/3/… | We don’t have a clear preference yet. Needs further study. |
| Nokia | Option 2 and 3 | We think Option 1 would have significant AS impacts, making L3 relay almost a L2 relay. We think that some "light versions" of option 2 and 3 will have minimal AS level impacts, and can help improving L3 path switch performance |
| ZTE | None |  |

# Discussion in Phase 2

Based on the phase 1 inputs from 21 companies, we have the below discussion in phase 2:

- 3.1: Common requirements and scenarios of service continuity

- Service continuity requirement

Q1-1: Do you confirm the requirement of service continuity is only for U2N relay, but not for U2U relay, during mobility?

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 19 companies

- Maybe: 2 companies

Based on the majority views and with the understanding this does not imply the general prioritization for U2N over U2U and the U2U service continuity may be considered in later release, we have the following proposals.

Draft Proposal 1-1 (19/21): The requirement of service continuity is only for U2N relay, but not for U2U relay, during mobility.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

- Mobility scenario: “between indirect and direct” vs. “between indirect and indirect”

Q1-2: Do you confirm R2 should study the mobility scenario of “between direct (Uu) path and indirect (via the relay) path” for U2N relay?

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 21 companies

All companies have the same views. Then, we have the following proposals.

Draft Proposal 1-2 (21/21): R2 should study the mobility scenario of “between direct (Uu) path and indirect (via the relay) path” for U2N relay.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

Q1-3: Do you think R2 should deprioritize the mobility scenario of “between indirect (via a first relay UE) and indirect (via a second relay UE)” in the SI phase, which can be studied in the WI phase, if needed?

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 15 companies

- Yes with comments: 6 companies

Majority companies are generally fine to deprioritize the discussion in service continuity for scenario “between indirect and indirect”. It is indeed worth to clarify this has no impact to the relay reselection discussion in [622]. Then, we have the following proposals.

Draft Proposal 1-3 (21/21): R2 deprioritize the mobility scenario of “between indirect (via a first relay UE) and indirect (via a second relay UE)” for path switching in the SI phase, which can be studied in the WI phase, if needed.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

- Mobility scenario: intra-gNB vs. inter-gNB

Q1-4: Do you confirm R2 should focus on the mobility scenarios of intra-gNB cases in the study phase, assuming the major difference with inter-gNB cases is R3 issue and to be studied in WI phase, if needed?

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 9 companies

- Yes with comments: 2 companies

- See comment: 8 companies

- No: 2 companies

Besides the intra-gNB case, we see some supporters for capture the feasibility to also support the inter-gNB case. For the delta part on inter-gNB case in R3 scope, companies are fine to postpone to WI phase. R2 can focus on the common R2 part for both intra- and inter-gNB cases. It seems we have no clear majority views on if there is some/minor difference on R2 scope. But, the stage3 details should also be studied in WI phase. There, we have the following draft proposal for companies to review.

Draft Proposal 1-4 (x/21):

R2 focus on the mobility scenarios of intra-gNB cases in the study phase.

R2 assume the inter-gNB cases can also be supported by the similar solution as the intra-gNB cases.

For the inter-gNB cases, compared to intra-gNB case, the different parts on R3 issues and potential different parts on R2 Uu interface in details are to be studied in WI phase.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

- Mobility scenario: Group mobility

Q1-5: Do you confirm R2 should deprioritize the group mobility scenario in the SI phase, with that case to be discussed in WI phase, if needed?

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 12 companies

- Yes with comments: 9 companies

Majority companies are generally fine to deprioritize the group mobility in SI phase at least. For the WI phase, it seems some companies assume the solution will be studied in WI phase, while some companies think whether group mobility will be supported/studied in WI should be is still FFS. Therefore, rapporteur proposes the compromised wording.

Draft Proposal 1-5 (12+/21): R2 deprioritize the group mobility scenario in the SI phase, which may be discussed in WI phase, if needed.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

- Mobility scenario: Dual connection

Q1-6: Do you confirm R2 should deprioritize the scenario where remote UE has the simultaneous transmission of one QoS flow with both source and target path for the optimization of “almost 0ms interruption” (e.g. DC-like mobility, DAPS-like mobility and make-before-break-like mobility), in both L2 and L3 relay?

NOTE 1: This does not exclude the case remote UE has one CP RRC connection with direct link and one UP data connection via relay at the same time, for L3 relay.

NOTE 2: For “make-before-break-like mobility”, we here refer to the AS layer MBB defined in LTE mobility (i.e. UE breaks the source link after connects to the target link), rather than the PDU session level “make-before-break” which is in SA2 scope.

NOTE 3: This is the scenario/requirement discussion of “simultaneous transmission for 0ms interruption”, which should be common for L2/L3.

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 17 companies

- See comments: 4 companies

Majority companies are generally fine to deprioritize the UP simultaneous transmission with both source and target path. As the comments from other companies: Rapporteur changes the “dual connection” to the simultaneous data transmission for clarification to the comments from MediaTek. Rapporteur also clarify the intention of this question with above NOTE1-3 for other comment. Please note the NOTE1 is clear from the draft proposal itself and NOTE2/3 are clarified in the highlight part in below draft proposal.

Draft Proposal 1-6 (17+/21): R2 deprioritize the scenario where remote UE has the simultaneous transmission of one QoS flow with both source and target path for the optimization of “almost 0ms interruption” (e.g. DC-like mobility, DAPS-like mobility and AS layer make-before-break-like mobility), in both L2 and L3 relay.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |
| Qualcomm | Thanks Rapporteur for trying to find compromised wording. However, we still prefer not to combine this proposal for L2 and L3: - As draft Proposal 3-1 indicates, majority companies think L3 relay has no HO (i.e. left to higher layer solution) and hence the DAPS HO is not relevant. In current draft Proposal 1-6 used terminology “source/target” which seems to imply concept of HO. - As Ericsson mentioned, it is feasible to use higher layer MBB for L3 relay. Thus, the simultaneous link for L3 relay is mostly an upper layer aspect and not an AS procedure. So, it is not necessary to mention L3 relay in Proposal. We appreciate Rapporteur to list “AS layer MBB-like mobility” as example in Proposal. However, it is just an example to de-prioritize, and we think we still have ambiguity whether it de-prioritize higher solutions because example can’t be exhaustive. We are not sure whether it has implication to other WGs. Based on above, we think the below combination of updated proposal 1-6 and proposal 3-1 make everything clear: Draft Proposal 1-6: R2 deprioritize the scenario where remote UE has the simultaneous transmission of one QoS flow with both source and target path for the optimization of “almost 0ms interruption” (e.g. DC-like mobility, DAPS-like mobility and AS layer make-before-break-like mobility), in L2 relay Draft Proposal 3-1: Working Assumption: For service continuity in L3 U2N relay, R2 assume it is left to the upper layer (e.g. application layer) solution. |
| OPPO | Although we share the intention of this proposal, the wording on “the simultaneous transmission of one QoS flow with” may cause further debate, like 1. Whether it hints that simultaneous transmission for different QoS flows is fine? 2. Whether simultaneous reception is fine? 3. Whether simultaneous CP connection is fine? Our understanding was that this wording is mainly to exclude the case for L3 relay, then would it be clear to differentiate L2/L3 relay here, e.g., R2 deprioritize the scenario for L2 relay where remote UE has the simultaneous transmission of one QoS flow connection with both source and target path for the optimization of “almost 0ms interruption” (e.g. DC-like mobility, DAPS-like mobility and AS layer make-before-break-like mobility), in both L2 and L3 relay. |

- 3.2: L2 relay: solution for service continuity

- General approach

Q2-1: Do you confirm that L2 U2N relay uses the R15 NR HO procedure as the baseline AS layer solution to guarantee service continuity (i.e. gNB hands over the remote UE to a target cell or target relay UE, including the HO preparation (if needed), HO command to remote UE, remote UE switching to the target, and HO complete message, similar to legacy procedure)?

Note: this excludes the case that remote UE executes the path switching procedure without NW control/decision, but does not exclude the possible enhancement like CHO.

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 18 companies

- Yes with comments: 3 companies

Majority companies are generally fine to reuse the NR R15 HO procedure. For comments on CHO, the proposal below itself does not exclude the CHO since we only say “baseline”. For other comments, rapporteur address the wording concern as below:

Draft Proposal 2-1 (18+/21): L2 U2N relay uses the R15 NR HO procedure as the baseline AS layer solution to guarantee service continuity (i.e. gNB hands over the remote UE to a target cell or target relay UE, including the HO preparation (if needed), HO command/RRCReconfiguration to remote UE, remote UE switching to the target, and HO complete message, similar to the legacy procedure).

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |

- Procedure for Case 1: Switching to direct Uu cell

Q2-2: For service continuity for L2 U2N relay, do you agree with the following procedure, in case of remote UE switching to direct Uu cell?

  - Step 1: Measurement and reporting

  - Step 2: HO decision of switching to a target cell by gNB

  - Step 3: RRC Reconfiguration message as the HO command to remote UE

  - Step 4: Remote UE performs RA to target cell

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in HO command.

  - Step 6: The data path switching

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 14 companies

- Yes with comments: 6 companies

- No: 1 companies

Majority companies are generally fine with the above procedure for case 1. For the comments on CHO (e.g. Xiaomi, Interdigital, Philips, ZTE), we clarify this is just the baseline procedure in SI phase, with any CHO like enhancement to be discussed in WI phase if needed. Based on the comments from CATT and Ericsson, one more step 6 is added for NW to inform or reconfigure the relay UE after the leaving of remote UE. For the source PC5 link release, the details can be discussed in WI phase, with one high level step added. The understanding should be that the order of step 6/7/8 is not restricted and should be further discussed in stage3. Also, the flow chart is added below.

Draft Proposal 2-2 (20/21): For service continuity of L2 U2N relay, the following baseline procedure is used, in case of remote UE switching to direct Uu cell:

  - Step 1: Measurement and reporting

  - Step 2: HO decision of switching to a target cell by gNB

  - Step 3: RRC Reconfiguration message to remote UE

  - Step 4: Remote UE performs RA to target cell

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in HO command.

  - Step 6: RRC Reconfiguration message to relay UE

  - Step 7: The PC5 link is released between remote UE and the relay UE

  - Step 8: The data path switching

Figure 2-2: Procedure for remote UE switching to direct Uu cell

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |
| Qualcomm | Step-1: why it is bi-directional arrow? Measurement reporting is from UE to NW. The current double arrow is confusing. Step-3 and 7: there are two connections for remote UE: one is E2E Uu connection with gNB and another one is PC5 connection with relay. Our understanding is Remote UE has to suspend Uu RRC connection and data via relay link after step 3. And Remote UE can maintain the PC5 link until either release after step3 or after 5 or in step 7. Step-6: it seems its response message is missing (i.e. RRC Reconfiguration complete). Maybe a simple way is to use bi-directional arrow for it. Hence, suggest to capture some notes like: 1. “Step 7 may happen after step 3”, or add a new step after step3 on release PC5 link. 2. “The order of step 6/7/8 is not restricted and should be further discussed in stage3” 3. “Remote UE suspends Uu RRC connection and data via relay link after step 3” |
| OPPO | We share the understanding of the rapporteur that “the order of step 6/7/8 is not restricted and should be further discussed in stage3”, e.g., step-6 can even happen earlier than step-4, together with step-3, and also we tend to share the understanding of Ericsson that the necessity of step-7 needs to be further checked (e.g., together with SA2 during WI-phase), e.g., step-7 might be only needed if the non-relayed PC5 link is not necessary, and that is an issue independent from the reconfiguration of the relayed PC5 link. Given that, it is suggested to add some description like “the order of step 6/7/8 is not restricted and should be further discussed in stage3, together with the necessity of step-7” For the UP behaviour details like suspension, it is suggested to leave to WI-phase. |
| Xiaomi | We wonder what is the intention of step 6? In our understanding, the path switch and data forwarding should be done by gNB. Relay could confirm successful handover by step 7. |

- Procedure for Case 2: Switching to indirect relay UE

Q2-3: For service continuity for L2 U2N relay, do you agree with the following procedure as baseline, in case of remote UE switching to indirect relay UE?

  - Step 1: Remote UE reports one or multiple candidate relay UE(s), after measures/discoveries the candidate relay UE(s).

    - remote UE may filter the appropriate relay UE(s) meeting higher layer criteria when reporting, which is studied by separate topic.

    - This may include the relay UE’s ID and SL RSRP information, where the measurement on PC5 details can be left to WI phase.

  - Step 2: HO decision of switching to a target relay UE by gNB, and target (re)configuration on relay UE optionally (like HO preparation).

  - Step 3: RRC Reconfiguration message as the HO command to remote UE

  - Step 4: Remote UE establishes PC5 connection with target relay UE, if the connection has not been setup yet.

    - The exact time to execute this step is not restricted (e.g. can be also before step1)

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in HO command.

  - Step 6: The data path switching

Note that the CHO manner can be considered as one enhancement of this procedure, which can be discussed in WI phase. Note that the similar solution was captured in the LTE TR 36.746 sec. 5.1.2.5.

Summary: Based on the phase 1 inputs, we have following companies’ view

- Yes: 13 companies

- Yes with comments: 7 companies

- No: 1 companies

Majority companies are generally fine with the above procedure for case 2. For the comments on CHO, again, we clarify this is just the baseline procedure in SI phase, with any CHO like enhancement to be discussed in WI phase if needed. As to the comments on step4, we now leave it as vague on whether the PC5 connection is established in step 1. Also, the flow chart is added below.

Draft Proposal 2-3 (20/21): For service continuity of L2 U2N relay, the following baseline procedure is used, in case of remote UE switching to indirect relay UE:

  - Step 1: Remote UE reports one or multiple candidate relay UE(s), after remote UE measures/discoveries the candidate relay UE(s).

    - Remote UE may filter the appropriate relay UE(s) meeting higher layer criteria when reporting, in step 1.

    - The reporting may include the relay UE’s ID and SL RSRP information, where the measurement on PC5 details can be left to WI phase, in step 1.

  - Step 2: HO decision of switching to a target relay UE by gNB, and target (re)configuration on relay UE optionally (like HO preparation).

  - Step 3: RRC Reconfiguration message to remote UE

  - Step 4: Remote UE establishes PC5 connection with target relay UE, if the connection has not been setup yet.

  - Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in RRCReconfiguration.

  - Step 6: The data path switching.

Figure 2-3: Procedure for remote UE switching to indirect relay UE

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |
| Qualcomm | Step-1: why it is bi-directional arrow? Measurement reporting is from UE to NW. The current double arrow is confusing. Step-2: it seems its response message is missing (i.e. RRC Reconfiguration complete). Maybe a simple way is to use bi-directional arrow for it. Step-2/3: suggest to make it more clear in the flow chat what is difference between step 2 and step 3. They look confusing. |
| OPPO | We share the understanding of the rapporteur that “As to the comments on step4, we now leave it as vague on whether the PC5 connection is established in step 1”, it is suggested that to add that description (similar to Q2-2 above), e.g., “the order of step 4 is not restricted and should be further discussed in stage3” |
| Xiaomi | Step 2 should be optional, if we only allow relay UE to be in IDLE and INACTIVE. |

Q2-4: Which information should be included in the HO command from gNB to remote UE, in case remote UE switching to indirect relay UE?

- Part 1: Identity of the target relay UE;

- Part 2: Target configuration, e.g. including the Uu/PC5 L2 configuration and adaptation layer configuration, if needed (with details to be discussed in WI phase);

- Part 3: (Please add if any) HO condition, which is used to trigger switching to indirect relay UE;

Summary: Based on the phase 1 inputs, we have following companies’ view

- Part1+2: 17 companies

- Part1+2+3: 3 companies

- Postpone: 1 companies

Majority companies are generally fine with the above procedure for case 2. For the comments on CHO, again, we clarify this is just the baseline procedure in SI phase, with any CHO like enhancement to be discussed in WI phase if needed. Then the part 3 is pending on the decision on CHO. For the comments from Nokia, even if the candidate target relay UE is selected by remote UE, it is still the HO command from NW to initiate remote UE’s switching. Anyway, the target relay UE ID has to be included to associate with the target configuration. Therefore, we have the following updated proposals based on the comments.

Draft Proposal 2-4 (20/21): For L2 U2N relay, following information may be included in the HO command from gNB to remote UE, in case remote UE switching to indirect relay UE: 1) Identity of the target relay UE; 2) Target configuration, e.g. including the Uu/PC5 L2 configuration and adaptation layer configuration, if needed (with details to be discussed in WI phase).

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |
| Qualcomm | We fail to understand why it is just “Uu/PC5 L2 configuration and adaptation layer configuration”. As we comment during phase 1: 1. Uu configuration may include Uu bearer configuration (e.g. security), which is not L2 configuration. 2. It is relay needs to get adaptation layer configuration. We are not sure whether remote UE needs it. Thus, we think the “e.g…part” may have some unnecessary implication. Thus, we suggest to modify it to: “For L2 U2N relay, following information may be included in the HO command from gNB to remote UE, in case remote UE switching to indirect relay UE: 1) Identity of the target relay UE; 2) Target configuration, e.g. including the Uu/PC5 L2 configuration and adaptation layer configuration, if needed (with details to be discussed in WI phase). |

- 3.3: L3 relay: solution for service continuity

- General approach

Q3-1: Which approach do you prefer for service continuity in L3 U2N relay?

- Approach 1: No AS layer solution to guarantee the service continuity, and leave it to the upper layer (e.g. application layer) solution

- Approach 2: R2 study the AS layer solution to guarantee the service continuity

Summary: Based on the phase 1 inputs, we have following companies’ view

- Approach 1: 13 companies

- Approach 1 as baseline, also OK with approach 2: 3 companies

- Approach 2: 5 companies

Majority companies prefer the approach 1, while some companies are fine to attempt some AS layer solution if possible. But this may depends if we can achieve agreeable AS layer solution in Q3-2. Therefore, in order to make the progress, rapporteur propose below as the R2 assumption based on the majority view. We may continue the discussion on Q3-2. In case R2 can agree on some AS layer solutions, we can revisit this working assumption then.

Draft Proposal 3-1 (13+/21): Working Assumption: For service continuity in L3 U2N relay, R2 assume no AS layer solution to guarantee the service continuity, and leave it to the upper layer (e.g. application layer) solution.

Please companies review the above proposal. If you have any concern, please input your views below. Otherwise, you may skip below box.

| Company | Please provide your concerns to the above proposal, if any |
| --- | --- |
| Qualcomm | Because 5 companies still prefer to study AS solution as enhancement and it is still being discussed in Q3-2, we think it is better not to say “R2 assume no AS layer solution to guarantee the service continuity”. Hence, we suggest to remove this sentence, i.e. we suggest to modify this proposal as: “Working Assumption: For service continuity in L3 U2N relay, R2 assume it is left to the upper layer (e.g. application layer) solution” |
| Xiaomi | We may need to send LS to SA to request confirmation. |

- Detailed solution

Q3-2: If R2 agrees on Approach 2, which AS layer solution can be used to guarantee the service continuity, by modifying the above basic path switching procedure for L3 U2N relay?

- Option 1: relay UE acts as the “gNB” in N3IWF L3 architecture

  - Proposed in R2-2007041;

  - Taking indirect to direct switching as example, the peer PDCP entity of remote UE is located at relay UE. Source relay UE may perform SN Status Transfer and Data Forwarding procedure to gNB.

  - Note that the measurement report from remote UE to relay UE, and the HO request and data forwarding from relay UE to gNB is newly introduced compared to the legacy HO concept. And, the RRC message from gNB to remote UE via relay UE needs to be clarified on the feasibility for L3 relay.

- Option 2: relay/gNB could provide AS control parameters to trigger switching, similar to discovery control.

  - Relay/gNB could provide AS control parameters to trigger switching, similar to discovery control. Due to lack of radio condition information, upper layer may trigger switching too late, which may result in connection failure. AS control parameters could avoid this problem.

- Option 3: Study/capture make-before-break like solutions for L3.

Summary: Based on the phase 1 inputs, we have following companies’ view

- None: 9 companies

- Option 1: 4 companies

- Option 2: 3 companies

- Option 3: 2 companies

Even though majority prefer not to discuss or agree any of the option 1-3, it is still helpful to continue the discussion on option 1-3, at least to clarify the common understanding on the solution rather than just making decision based on the high level “concept”.

If you prefer the approach 1 in Q3-1 or none of the option 1-3 in Q3-2, you may skip this question box. For the proponents of option 2/3, please clarify how the option(s) works in details and how to ensure the service continuity, so that companies can have the common understanding.

| Company | Clarify how option 2 ensure the service continuity (e.g. lossless and in-order-delivery), if you prefer this option | Clarify how option 3 works in details, and how to ensure the service continuity (e.g. lossless and in-order-delivery), if you prefer this option |
| --- | --- | --- |
| Xiaomi | Due to lack of radio condition information, upper layer may trigger switching too late, which may result in connection failure. UE can’t re-establish Uu connection via relay and has to establish connection via relay, which would result in data loss. AS control parameters could avoid the data loss due to connection failure to ensure service continuity. |  |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2008264	[AT111-e][603] Scope, requirements, and scenarios (Interdigital) 	InterDigital

- R2-2008048	Service continuity for L2 UE-to-Network relay	Huawei, HiSilicon

- R2-2006723	Service Continuity with Sidelink Relay	Futurewei

- R2-2007041	Protocol stack and service continuity for L2 and L3 relay	vivo

- R2-2007816	Considerations on UE-to-NW Relay	ETRI

- R2-2008066	Discussion on service continuity from Uu to relay	Xiaomi communications

- R2-2006641	L2 vs L3 - Relay/Remote UE Authorization, Service Continuity	Fraunhofer HHI, Fraunhofer IIS

- R2-2006609	Clarification on the Scenarios for NR Sidelink Relay	CATT

- R2-2006717	Requirements, Assumptions and Supported Scenarios for NR Sidelink Relay Intel Corporation

- R2-2007099	Discussion on NR Sidelink Relay Scenarios		Apple, Convida Wireless

- R2-2006611	L2/L3 UE-to-NW Relay Comparison	CATT

- R2-2006737	Discussion on NR SL Relay Architecture		ZTE Corporation, Sanechips

- R2-2006718	Characteristics of L2 and L3 based Sidelink relaying	Intel Corporation

- R2-2007290	Service continuity scenarios for sidelink relay	Ericsson

# Annex: Copy related discussion in [AT111-e][603]

Below some related discussions in offline [603] are copied, R2-2008264:

==================================Start======================================

In addition to MR-DC, multiconnectivity of the relayed link is also discussed in Error: Reference source not foundError: Reference source not found.  For the UE to NW relay, in stage 1 description of TS 22.261, the connectivity models for the remote UE are as follows:

6.9	Connectivity models

6.9.1	Description

The UE can connect to the network directly (direct network connection), connect using another UE as a relay UE (indirect network connection), or connect using both types of connections. […]

Although connection to the network by the remote UE with both types of connections (direct or indirect) are possible, Error: Reference source not found prefer not to consider simultaneous Uu and PC5 connection, as was assumed in FeD2D.

Question 13: Which connectivity scenarios should be supported for the remote UE in UE to NW relaying?

- Active link with only the relay or directly with Uu, but not both.

- Active link with both the relay UE and with Uu supported simultaneously

- Active links with different relay Ues supported simultaneously

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | A | It is preferred to simplify the dimension of the scenarios, in order to focus on the comparison of L23 solution during the study, considering the limited timefor this study. |
| Ericsson (Tony) | a) |  |
| Qualcomm | a) |  |
| MediaTek | a) |  |
| Lenovo, MotM | a) |  |
| Huawei | a) |  |
| FirstNet | See comments | a)Yes, b)No-already connected to network, c)yes |
| Interdigital | a) | Single link should be preferred, as in FeD2D. |
| Kyocera | b and c | For reliability, we think connections with multiple connections will be needed. |
| Vivo | a) | Keep the design simple in this release |
| Intel (Rafia) | a) | We think that (b) is a temporary state in service continuity scenario when performing path switching between Uu and relay PC5. |
| Xiaomi | A | We think U2N relay is used to provide coverage extension in this study item. It’s strange to keep relay connection when there is available Uu connection. |
| CATT | a) |  |
| Sony | a |  |
| ZTE | A with comment | If we just focus on relay scenario and exclude normal sidelink scenario. |
| Nokia | a) but see comment | We assume that make-before-break type of mobility between direct and relay connected connections should be supported |
| Fraunhofer | a) | Option a has to be supported. All 3 options are possible. However, studying b and c may depend on the timeline of the SI. |
| Samsung | a) |  |
| Convida | A |  |
| Futurewei | A | a) is the intention of this study. |
| Spreadtrum | a) |  |
| Fujitsu | a), c) | a) is the general MR-DC, and we need to support. In addtion, c) can be used to further improve the coverage up to 1000m, for instance. |
| ETRI | a) |  |
| Apple | a) |  |
| LG | a), c) | Regading the c) we can consider active link only the one relay UE while a PC5 connection is established with aonther relay UE. |
| AT&T | Yes | A-1st Priority; C 2nd Priority; B as time permits |

Summary of Q13:

Majority companies selected a) only.  Some companies mentioned that some transition cases (such as make before break, or establishing another PC5-RRC connection, should not be excluded.

Proposal 11: For UE to NW relay, RAN2 assumes the remote UE has an active connection with only a single relay UE or to Uu at a given time.  The remote UE can have a direct Uu connection or a connection via a single relay UE, but these two connections should not be active at the same time.

One of the objectives for the SID is to study the mechanisms for L2/L3 relay to support service continuity.  To study such mechanisms, the mobility scenarios in which service continuity is assumed should first be defined.  A number of company contributions described the scenarios that should be considered for UE to NW relay and UE to UE relay in where support service continuity is assumed Error: Reference source not foundError: Reference source not found Error: Reference source not foundError: Reference source not foundError: Reference source not foundError: Reference source not foundError: Reference source not foundError: Reference source not found.  Specifically, for UE to NW relay, the possible mobility scenarios are i) changes between direct path (via Uu) and relayed path (via a UE to NW relay) and vice versa; ii) change from a relayed path via a first relay to a second relay.  Similarly, for UE to UE relay, the direct (SL with no relay) to indirect (SL via a relay) change and indirect/indirect change is also possible.

Question 27: Which mobility scenarios should be considered for support of service continuity in UE to NW relay?

- Direct (Uu) path to indirect (via the relay) and vice versa

- Indirect (via a first UE to NW relay) to Indirect (via a second UE to NW relay)

| Company | Response | Comments |
| --- | --- | --- |
| Spreadtrum | a) and b) | b) can be de-prioritized. |
| OPPO | a) | Given the limited time of the study phase, to study both a and b is not feasible, so a) should be prioritized since it is of more interest by companies. |
| Huawei | a+b | a and b in intra-gNB case are a good starting point. If time is limited, we can de-prioritize the inter-gNB case. We are also fine, if majority prefer to select one of them. |
| ETRI | a) and b) |  |
| Apple | a+b | If time is an issue, we are fine to deprioritze b |
| Ericsson (Tony) | a) | Agree with OPPO. We can leave the study of b) in the WI phase if time allows. |
| LG | a) ,b) |  |
| CATT | a), b) |  |
| Qualcomm | a) b) is low priority | Agree with OPPO and Ericsson that b) is low priority. It is fine if b) is not studied in this release. |
| Samsung | a), b) |  |

| Nokia | a) and b) | We also think that a) should have higher priority |
| --- | --- | --- |
| Convida | a+b | Agree with Apple |
| AT&T | a) + b) | OK to prioritize a) |
| Intel (Rafia) | a), b) |  |
| Interdigital | a), b) |  |
| vivo | a). | b) can be deprioritized for this Release |
| Xiaomi | a) | a) is of high priority. |
| Futurewei | a > b | They both should be supported. For study purpose, a has higher priority given the limited time. |
| Fraunhofer | a), b) | We are also okay with deprioritizing b) for this release |

Question 28: Which mobility scenarios should be considered for support of service continuity in UE to UE relay?

- Direct (SL with no relay) path to indirect (SL via a UE to UE relay) and vice versa

- Indirect (via a first UE to UE relay) to Indirect (via a second UE to UE relay)

| Company | Response | Comments |
| --- | --- | --- |
| Spreadtrum | No for both | Mobility senarios in UE to UE relay are supported by connection release and setup procedure witout service continuity. |
| OPPO | No | There is no requirement in SA2 for service continuity in U2U relay, so there is no need to consider in RAN either. |
| Huawei | None | Service continuity for U2U seems not in the scope. This has been also assumed in SA2. |
| ETRI | No |  |
| Apple | None | Agree with OPPO and Huawei |
| Ericsson (Tony) | None | Agree with OPPO and Huawei |
| CATT | None | Agree with OPPO and Huawei |
| Qualcomm | None | Agree with OPPO and Huawei. We don’t understand why U2U has requirement of servie continuty |
| Samsung | None |  |
| Nokia | None |  |
| Convida | None | Agree with OPPO and Huawei |
| AT&T | None |  |
| Intel (Rafia) | See comments | Need clarification: Scenarios a) and b) are covered for relay (re)selection, but RLF handling to support service continuity for such scenarios is off the table? In either case, it would depend on Q24, if U2U is under discussion, then service continuity scenarios a), b) can be discussed. But we have no strong view and are okay to go with majority. |
| Interdigital | a), b) – see comments | Agree with Intel – although SA2 does not have explicit requirement for service continuiuty, path switch functionality that is applicable to UE to NW relay should also be studied for UE to UE relay (e.g. RLF). This is inline with the common architecture objective. Similarly, our understanding is that solutions to enable service continuiy for SA2 in UE to UE relay are also being considered under KI#4 (even at this meeting). |
| vivo | None | Agree with OPPO |
| Xiaomi | None |  |
| Futurewei | None |  |
| Fraunhofer | None | Agree with OPPO and Huawei. |

In addition, some papers talked specifically about the scenario of group mobility, which was discussed in FeD2D Error: Reference source not foundError: Reference source not found.  In these papers, it was proposed to down-prioritize or not study the group mobility scenario, considering it is an optimization with little benefit.

Question 29: Do you agree to exclude the group mobility scenario in the SI when studying service continuity?

| Company | Response | Comments |
| --- | --- | --- |
| Spreadtrum | Yes with comments | Group mobility should be de-prioritized. If time does not allow, the study of group mobility can be excluded. |
| OPPO | Yes | Since there is no RAN3 TU allocated, one cannot study group-HO topic which is highly rely on RAN3. |
| Huawei | Yes | This can be done in WI pahse, if needed. Also, R17 IAB WI is working on this group mobility procedure, we can reuse the basic idea after IAB complete their work. |
| ETRI | Yes |  |
| Apple | Yes | Agree to deprioritize this in SI phase. |
| Ericsson (Tony) | Yes | Agree to downprioritize group mobility. |
| LG | Yes | It is better to deproritize in this SI phase. Regading group mobility, SA2 group has big condern. We should check with SA2. |
| CATT | Yes | Agree with OPPO. |
| Qualcomm | Yes | As we know, group mobility has been down-prioritized by SA2. Then, we don’t think it is possible in RAN2 in this release. |
| Samsung | Yes |  |
| Nokia | No | Our view is that supporting the mobility of a Relay UE in an efficient manner is important. |
| Convida | Yes | Agree with Apple |
| AT&T | Yes | Can consider this in the WI phase considering IAB and other solutions if applicable |
| Intel (Rafia) | Yes | We think support of group mobility enhancements in FeD2D hinged on the assumption of a ‘linked‘ state between the Remote UE and the Relay UE, which is decidedly not the case here. |
| Interdigital | Yes |  |
| vivo | Yes | Group mobility would complicate the design. And for L2 group mobilitywould require too much specification effort. So we prefer not to support group mobility, at least for this release. It can be considered in next release. |
| Xiaomi | Yes |  |
| Futurewei | Yes |  |
| Fraunhofer | Yes | Agree to downprioritze in SI phase, but could be added to the WI. |

Rapporteur Summary of Q27-Q29 (Service continuity scenarios)

For UE to NW relay, majority of companies are ok to support both option a) and b) in Q27, but would be ok to deprioiritize b).  All companies are ok to exclude the group mobility scenario.  Finally, most companies note that there is no need to discuss any service continuity scenarios for UE to UE relay, since it is not required in SA2.  Rapporteur thinks the following proposal can be easy to agree, as there are no companies objecting:

Proposal 25: For UE to NW relay, RAN2 supports the following scenarios for service continuiuty i) Direct (SL with no relay) path to indirect (SL via a UE to UE relay) and vice versa ii) Indirect (via a first UE to UE relay) to Indirect (via a second UE to UE relay).  Scenario ii) is down-prioritized.

Proposal 26: Group mobility scenario is excluded in this SI/WI.

==================================End======================================