3GPP TSG-RAN WG2 #111-e	R2-20xxxxx

E-meeting, November 2020

Agenda Item:

Source:	OPPO (rapporteur)

Title:	Remaining issues on relay discovery

Document for:	Discussion, Decision

# Introduction

There are some issues for further study and hence we have following email discussion.

- [Post111-e][623][Relay] Remaining issues on relay discovery (OPPO)

Scope: Discuss the remaining issues on relay discovery:

- Need for MAC and/or PHY solution to differentiate discovery messages

- Need for separate resource pool for discovery messages

- Handling of potential cases where the serving gNB is not sidelink-capable

- Conditions for discovery for UE-to-UE relay

- FFS points in the discovery conclusions from RAN2#111-e

Intended outcome: Summary to next meeting

Deadline:  Long

Issues discussed in this paper are assumed to be common for both layer2 and layer3 solution unless described explicitly. Technical issues are categorized into 3 types i.e. common for U2N and U2U, U2N specific and U2U specific issues in section 2.1, 2.2 and 2.3 respectively.

# Discussion

# Issues common for U2N and U2U relay operation

One left issue is whether a new SL SRB is introduced for discovery message. SA2 agreed to contain type of discovery message (e.g. Model A Announcement, Model B Solicitation or Response, Group member discovery, or Relay discovery) in document [26]. It means discovery message will not be taken as a PC5-S signalling because otherwise these new message types will be introduced as PC5-S signalling. So the protocol stack can be actually refreshed as following:

Figure 2.1-1 protocol stack of discovery message

Question1-1: Do you agree the protocol stack of discovery message as indicated in Figure 2.1-1?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | See comments | SA2 only agree the content of the discovery message, did not design a new bearing signalling.It had better send LS to SA2 to check whether the discovery message can be included in PC5-S signalling or not. |
| Qualcomm | See comments | Same understanding as CATT. Another possibility is to reuse protocol stacks of LTE discovery (i.e. only with PHY/MAC/Discovery). Agree with CATT that RAN2 can send LS to SA2 to decide. |
| Huawei | See comments | Agree with CATT and Qualcomm that there is no conclusion from SA2 on whether the discovery message is a PC5-S message or other protocol message. In SA2 LS, it only says “PC5 communication channel is assumed to carry the discovery message over PC5”. The AS protocol stack can be decided later based on a clear SA2 decision. We prefer to wait for SA2 conclusion to see if any update is needed in our TR, but do not see the need to trigger a LS. |
| Ericsson (Min) | Yes | However, we are also okay to consult SA2, if majority of companies prefer so. If we go in this direction, we think that it would also be good to inform SA2 about what is the RAN2 preference about this. |
| Intel | See comment | We agree with Huawei that it the conclusion from SA2 does not seem to conclude on whether or not PC5-S can be used to carry SL discovery message. So, we also propose to wait for further discussion and are fine to send and LS to confirm if the majority wants to do so. |
| Samsung | Yes | We are okay to define a protocol stack for discovery message which is independent from PC5-S signalling. |
| Apple | Yes | We are fine with the proposed protocol stack. This figure can be further updated if SA2 decision is different. |
| Lenovo, MotM | See comment | It will be better to ask SA2 and then the remaining decision on the protocol stack should be a quick one. |
| Kyocera | See comment | We have the same view as CATT. |
| OPPO | Yes | We are fine to send LS to SA2 to confirm |
| ZTE | See comments | Agree with CATT and Qualcomm that there is no conclusion from SA2 that discovery message will not be taken as a PC5-S signalling. And it is better to send a LS to SA2 to check whether the discovery message can be included in PC5-S signalling or not. |
| Xiaomi | Yes | Fine with consultant with SA2. |
| Nokia | No with comments | Based on the previous discussion in last RAN2#111 meeting none of the 4 discovery models have been agreed nor the signalling of the discovery message itself. Under understanding is that SA2 dif NOT infer any signaling for discovery message and above statement “discovery message will not be taken as a PC5-S signalling” is not correct. SA2 did not decide or agree on a protocol stack for discovery message. |
| Interdigital | Yes | We would also be ok to ask SA2. |
| vivo | See comments | In last meeting, the majority’s view is that the discovery message could be taken as a kind of PC5-S message and it is only FFS if separate SL-SRB would be used for it. As companies mentioned that this is not clarified yet in SA2 we agree to send an LS to SA2 for confirmation and thus no need to rush to agree with the protocol stack showed in the figure above. |
| Philips | See comments | We agree with Lenovo |
| MediaTek | See comments | Prefer to wait for SA2 conclusion. |
| Fraunhofer | See comments | Agree with other companies to send an LS to SA2 as to our understanding whether discovery message is a PC5 message is not yet concluded. |
| Futurewei | See comments | Prefer to wait for SA2 to conclude. |
| ETRI | Yes |  |
| Sharp | Yes | We are fine to check with SA2 |
| Convida | Yes | We are fine to send LS to SA2 to confirm |
| Spreadtrum | See comments | Prefer to to check with SA2 whether the discovery message can be included in PC5-S signalling or not. |

RAN2 agreed following proposal:

Proposal6: Solution is needed to differentiate discovery message in AS layer from existing SL signalling or traffic

During email discussion “[AT111-e][606][Relay] Discovery model and procedure (OPPO)” it is clarified that this agreement is only applicable for U2N relay operation since at that time it is not very clear what is discovery model for U2U relay operation. And during RAN2#111e meeting RAN2 also had following agreement:

Proposal2: Model A/ B discovery model similar to LTE is reused for U2U relay also

The agreed proposal6 is related to transmission and/or reception of discovery message. Rapporteur think it can be applied for PC5 interface between remote UE and relay UE U2U relay operation also since they are common issues. Hence rapporteur would like to confirm whether this is also companies understanding.

Question 1-2: Whether agreed proposal6 is applicable for U2U relay operation too?

Option1: Yes

Option2: No i.e. for U2U relay operation discovery message is not visible in AS layer

| Company name | Chosen option | More comments or reasoning |
| --- | --- | --- |
| CATT | Option 1 |  |
| Qualcomm | Option 1 (Yes) | We believe the issue is common for U2N and U2U relay |
| Huawei | Option1 | We understand for U2U remote UE need to do relay (re)selection based on discovery message as well. |
| Ericsson (Min) | Option 1 |  |
| Intel | Option 1 |  |
| Samsung | Option1 | The same solution can be applied for both. |
| Apple | Option 1 |  |
| Lenovo, MotM | Option 1 |  |
| Kyocera | Option 1 |  |
| OPPO | Option 1 |  |
| ZTE | Option 1 |  |
| Xiaomi | Option 1 |  |
| Nokia | Option 1 |  |
| Interdigital | Option 1 |  |
| vivo | Option-1 | Still we may need to reconsider this after SA2’s final decision about the discovery modelling for U2N and U2U relay. |
| Philips | See comments | Too early to decide. SA2 may select e.g. solution #8 and/or #9 (in TR 23.752) which reuses existing Direct Communication Request over PC5 |
| MediaTek | Option 1 |  |
| Fraunhofer | Option 1 | We also think the similar principle as U2N relay should be used here for U2U relay |
| Futurewei | Option 1 | It’d be better to have uniform design for the remote UE. |
| ETRI | Option 1 |  |
| Sharp | Option1 |  |
| Convida | Option 1 |  |
| Spreadtrum | Option 1 |  |

If it is concluded that agreed proposal6 is also applicable for U2U relay operation, following issues are common for both U2N and U2U relay operation, otherwise only applicable for U2N relay operation and please check further question3-3 in section 2.3.

As for the solution to differentiate discovery message majority companies would like to focus on solution in MAC layer or PHY layer. On the other hand some companies also want to introduce separate resource pool which can address part of the issue e.g filtering discovery message in RX side. So it seems beneficial to confirm whether separate resource pool is needed at first.

The pros and cons of separate and shared resource pool are listed in table 2.4-1[23]. But rapporteur would suggest to reconsider them again after RAN2 making some progress at last meeting. In discovery model A, discovery message supposes to transmit periodically. In case of shared resource pool, discovery message will share the reservation period with sidelink communication. So it seems separate resource pool could be rather a sparser resource pool than resource pool for sidelink communication considering the periodicity of discovery message is likely longer than sidelink communication. But RRC signalling can actually configure a list of reservation period by IE “sl-ResourceReservePeriodList”. If RRC signalling configure a long period in the list for shared pool, then there is no problem for TX UE to select such long period when it tries to send discovery message. In addition for discovery model B TX UE should be allowed to reserve one shot resource. It means sparse resource pool could result in more delay. And no matter how sparse the resource is, it is true additional resource increase resource fragment. Even separate resource pool is split from resource pool for sidelink communication, still it will hurt sidelink communication since its resource pool becomes sparser. Note if a longer reservation period is needed then the impact on specification will be there regardless separate or shared resource pool. And another issue mentioned by one company is resource granularity in frequency domain. In LTE it seems 2 PRBs are sufficient to send discovery message. In NR the message size of discovery message could be different. But even the size is doubled roughly including protocol overhead 4 PRBs are still much lower than current minimum sub-channel size of resource pool i.e. 10 PRB.

Observation1: separate resource pool causes resource fragment

2 companies argues that separate resource pool could help saving more power consumption because RX UE can reduce monitoring discovery message assuming it is a sparse resource pool. But it is also not difficult for shared resource pool as long as RX UE knows how often it should monitor discovery message. It is already feasible since current SCI will contain a parameter called “resource reservation period” which basically indicates periodical resources in future. So there is no advantage from power saving point of view. On contrary shared resource pool can help reduce delay to send discovery message in model B.

Observation2: there is no difference in terms of power saving between separate and shared resource pool

Another power relevant issue is power control. The main criteria for remote UE to select or reselect a relay is measured PC5 link quality based on received discovery message. In shared resource pool relay UE need follow same power control parameters and procedures to transmit discovery message. In short ,transmission power could be changed with the change of e.g. CBR measurement result. Then it is difficult for remote UE to tell which relay UE is better because a high SL RSRP could be either due to high CBR measured by relay UE or short distance between relay UE and remote UE. In case of separate resource pool such situation can be improved by configuring different power control parameters or even introduce different procedure.

Observation3: separate resource pool could help to configure different power control parameters or scheme

It is obvious that separate resource pool can already differentiate discovery message from 1st place implicitly. And

Observation4: separate resource pool could differentiate discovery message implicitly

Some company also mentioned that separate resource pool may have RAN1 impact. Company’s input is needed to clarify this aspect. From rapporteur point of view, all sidelink communication parameters and procedures can be just reused unless some optimization is introduced i.e. no RAN1 impact is foreseen. Note more than one resource pools are already supported by RAN1 spec.

Observation5: separate resource pool has no RAN1 impact.

Companies are invited to comment on listed observations and chose one of options listed below.

Question1-3: which options do you prefer for transmission of discovery message?

Option1: separate resource pool only

Option2: shared resource pool only

Option3: both separate and shared resource pool in study phase and leave the final decision in WI phase

| Company name | chosen option | comments |
| --- | --- | --- |
| CATT | Option 3 | The aim of this study item is to select one between L2 and L3 relay, how to design the resource pool does is too detail, and has no impact on the L2/L3 relay selection. |
| Qualcomm | Option 1 | For observation 1, we failed to see why resource fragment is an essential issue: - We only have up to two separate resource pool types (discovery and communication). - One UE is not expected to transmit many discovery messages and the resources for discovery PSSCH should be quite small comparing with communication PSSCH. Therefore, the impact to resource utilization/fragment won’t be significant. In addition, it seems Rapporteur’s argument / solution is to always tie discovery transmission with slot reservation. However, we don’t think slot resource reserved is designed for discovery, and it is not a clean solution: - it restricts resource pool configuration flexibility for communication PSSCH. Note that NR V2X only allows up to 3 slot reservation patterns in SCI, which will be shared with both discovery and communication PSSCH if shared resource pool is agreed. - It will further cause resource wastage for the resource pool with long periodic slot reservation, especially for OOC UE. - It implies shared reserved slots among discovery and communication. We think it will complicate resource selection and reselection procedure: - Example1: TX UE may need to determine whether the reselected resource pool has the same type (disc or comm). - Example 2: Tx UE can reserve resources for retransmissions or a new transmission at a time, this could be more complicated or limiting if a Relay UE needs to reserve resources for both data and discovery in a shared pool. - It can’t work for mode 1 transmission. - [Rapporteur]: can you clarify why mode 1 transmission doesn’t work for shared resource pool ? For Observation 2, we also don’t agree with Rapporteur’s solution to always tie discovery message and slot reservation. In addition, during Relay (re)selection, remote UE needs to perform relay RSRP measurement which only includes discovery message. Note the relay UE may send both discovery and communication. Then, the remote UE has to decode the message until higher layer to divide whether it is discovery message in shared resource pool. So, it still can’t achieve power saving. We agree with Observation 3-5, and think observation 3 provides a clear benefit of separate resource pool. Power control can only be configured as resource pool specific in current spec. Thus, separate resource pool has benefit to allow discovery transmission to have its own required transmitting power. Based on above analysis, we think separate resource pool is a cleaner solution and provide further flexibility. |
| Huawei | Option1 | We do not agree with observation2. If the shared resource pool is used for discovery message and communication, even if we assume a RX UE can know the resource reserved by SCI from one Tx UE, it still needs to monitor the whole resource pool for other discovery messages from other UEs. So the power consumption on monitoring discovery messages in case of shared RP is more than that of dedicated RP. |
| Ericsson (Min) | Option 2 | Discovery messages are transmitted over a same SL communication channel as the data, therefore, one common resource pool should be enough. In addition, discovery and data transmission are typically performed in different time phases, meaning that discovery message is typically transmitted alone, so, it is feasible to apply particular treatment for discovery if it is justified. |
| Intel | Option 2 | Regarding the set of observations listed by the rapporteur, we agree with observation 1 fully; in fact, this was one of the main arguments for not having separate resource pools for different cast types for Rel-16 and we think the same principle applies here. Regarding observation 2, while it really depends on the design of the separate resource pool itself, we somewhat agree with the general observation. Regarding observation 3, again it depends on the ultimate design of the resource pool and the associated procedure itself. We agree with observation 4. We do not agree with observation 5 since once RAN2 decides that separate resource pool is supported, RAN1 will have to define the physical structure and overall design and it is unfair to say that there is no RAN1 impact. We strongly believe that RAN1 should be included in the discussion if a separate pool is to be defined. In general, we are not sold on the need for having a dedicated pool for discovery, especially given the kind of periodic and relatively infrequent transmissions characteristic of discovery models A/B. These can be easily handled by the Rel-16 SL resource reservation design and so, do not warrant the need for a dedicated resource pool. |
| Samsung | Option2 | Discovery message is sparse comparing with normal communication so shared pool is better for sidelink resource utilization. |
| Apple | Option 3 | We agree with the observations provided by the rapporteur. We think both solutions can be listed as the outcome of the study. |
| Lenovo, MotM | Option 1 | Option 1 helps dimensioning the resource pool for discovery for a certain performance (avoiding more than a certain level of collisions) level. Discovery is important. In addition, if shared resource pool is supported, UE is expected to check if the received data is discovery message, which will increase UE complexity. |
| Kyocera | Option 1 | In addition to the advantages pointed out in Observations 3 to 5 which we agree, it should also be pointed out that in TR 23.752 it is stated as part of the architecture requirement that, it is a SA system requirement to enable the direct discovery of the ProSe-enabled UE by other ProSe-enabled UEs within the same PLMNs or different PLMNs. We don’t think combining discovery and communication resources in one pool works for supporting inter-PLMN discovery. [rapporteur]: can you elaborate why shared resource pool is not feasible for inter-PLMN discovery case? I assume as long as resource pool configuration is aligned between PLMNs then it doesn’t matter whether it is separate or shared. But I may miss something here. |
| OPPO | Option3 | Separate and shared resource pool have their pros and cons. We are fine to study them both in SI phase. We slightly prefer shared resource pool. |
| ZTE | Option 1 | We agree with the observations 3-5, the benefit of separate resource pool is obvious. |
| Xiaomi | Option 2 | Furthermore, if the separate resource has sparse resource to fit long periodicity of discovery message, discovery may be delayed. If remote is only allowed to trigger discovery procedure when the Uu rsrp is lower than a threshold, it is important for remote UE to discover relay early. Otherwise RLF may happen. |
| Nokia | Option 3 | We have some sympathy with option-1, but think it’s too early to make a strict decision right now without thorough analysis and without detailed feedback from SA2 (characteristics of discovery message e.g. message size, periodicity, transmission occasion etc. are not defined yet). |
| Interdigital | Option 2 | We agree with Observation 1, 2, and 4 We are not sure about the issue mentioned for Observation 3. It may be desirable for remote UE to not select the relay that has a high CBR and consequently low discovery transmit power, so there may be no additional work needed. For Observation 5 it may be too early to conclude whether there is RAN1 impact. One additional observation that could be made is that separate resource pool (based on input on subsequent questions) requires a new MAC procedure for transmission on the discovery pool, whereas, share resource pool can reuse the existing LCP procedure. |
| vivo | Option-3 | It is a simple way to differentiate discovery message by using different resource pools (option-1), with some benefits such as different power control level as rapporteur mentioned. And separate resource pools may have benefits for sidelink measurements. At the same time option-2 can also work well since discovery message is carried on communication channel now. In our opinion we think it is fine to leave the final decision and details of pool configuration in WI, since this also seems to be a tough decision for now. |
| Philips | Option 1 | A separate resource pool is cleaner and has some benefits as explained e.g. by Qualcomm. |
| MediaTek | Option 1 | We agree with observation 3 – 5. For observation 2, we tend to agree with Huawei that separate resource pools can have power saving gain. |
| Fraunhofer | Option 1 | We agree with Rapporteurs Observation 3-5. Additionally, we also think a separate resource pool is beneficial for discovery message transmission in terms of power saving because the UE does not need to monitor complete shared resource pool. |
| Futurewei | Option 3 | Given the tight schedule to complete the study, we don’t think RAN2 should spend more time on this issue in SI. |
| ETRI | Option 3 | We have similar view with Observation 1, 3 and 4. But since it is not possible to make decision whether which option is better than the other, we think more analysis seems to be needed. |
| Sharp | Option 3 | We are open to include both solutions in SI phase. |
| Convida | Option 3 | We are fine to study them both in SI phase. |
| Spreadtrum | Option 1 | As the rapporteur has analysed above, there are at least three advantages (observations 3-5) to using a separate resource pool for transmission discovery information. |

Assuming conclusion of question1-3 is option1 or option3, then from TX UE point of view it is not clear whether LCP procedure is still needed. From remote UE point of view it is not necessary to transmit more than one discovery messages simultaneously to multiple relay UEs. Actually it is likely solicitation message from remote UE is a broadcast message. It means no collision is possible at remote UE side. In relay UE side collision is possible e.g. when relay UE is answering solicitation messages from more than one remote UEs. But regardless how many discovery messages will be transmitted simultaneously they should be treated equally because they will be transmitted in same logical channel. In this sense within the resource pool for discovery message, current LCP procedure in MAC specification is not necessary.

Question1-3.1: Do you agree that discovery messages should be treated equally in terms of channel prioritization within the separate resource pool if necessary?

| Company name | Position (yes or no) | More comments |
| --- | --- | --- |
| CATT | Yes, but see the comment | This is too detail to discuss in SI stage. |
| Qualcomm | Yes | We are a little confused by Rapporteur’s description. So, we will try to share our understanding. First, we agree with Rapporteur that one SL UE may send multiple discovery messages, e.g. in below cases: - One for relay discovery and another for commercial use (e.g. advertisement). - Response messages for different remote UEs as rapporteur mentioned. Secondly, if one SL UE needs to send multiple discovery messages in above cases, we think it should be sufficient to be differentiated in higher layer (i.e. Discovery layer) because RAN2 has agreed to send discovery message via SL SRB. Then, we don’t see much benefit to differentiate them in AS layer and spec change in LCP. |
| Huawei | Postpone to WI phase if needed | We understand this is not essential, can be discussed in WI phase if companies consider this is as an issue. |
| Ericsson (Min) | Yes |  |
| Intel | See comment | We agree with CATT and HW that this can be discussed later, once a clear need for having separate resource pool is identified |
| Samsung | See comment | If no prioritization i.e., no separate logical channels among discovery messages, then no LCP is needed. Otherwise existing LCP can be applied. |
| Apple | Yes |  |
| Lenovo, MotM | Yes | Agree with QC analysis. |
| Kyocera | Yes |  |
| OPPO | Yes |  |
| ZTE | Yes | It make sense that discovery messages is treated equally in terms of channel prioritization. If more than one discovery messages simultaneously are transmitted, it is up to UE implementation to decide which is prioritized. |
| Nokia | comment | Up to UE implementation |
| Interdigital | Yes |  |
| vivo | Yes | This can be left to UE implementation as well. |
| Philips | Yes |  |
| MediaTek | Yes |  |
| Fraunhofer | Yes | Detailed discussion can be discussed in the WI phase. |
| Futurewei | Yes | This can also be postponed to WI, if a quick agreement is not reachable. |
| ETRI | Yes | We agree with Qualcomm and Ericsson’s view. |
| Sharp | Yes |  |
| Convida | Yes |  |
| Spreadtrum | Yes |  |

If the answer to question1-3 is no i.e. resource pool is shared between sidelink communication and discovery message transmission, then at least some solution is needed to filter discovery message from RX UE point of view. For UEs which is either not interested in or not capable of relay operation e.g. Rel16 UE to filter discovery message in physical layer is beneficial because further decoding and treatment above PHY layer could be saved. For example destination id or some information in SCI can already indicate discovery message then UE can skip decoding PSSCH channel. In case MAC layer solution e.g. a new logical channel is introduced, then UE can drop the discovery message at least after decoding of PSSCH channel. So from RX UE point of view, physical layer solution is better than other solution above physical layer.

From RX UE point of view another issue is how to interact with upper layer because upper layer will take discovery message differently from PC5-S signalling. One clean approach is to introduce another SL SRB for discovery message in order not to mix discovery message and PC5-S signalling into same SL SRB.

On the other hand MAC layer solution may be also needed from TX UE point of view. This is because LCP procedure is necessary in case of shared resource pool. And if different logical channel priority is necessary for LCP procedure then at least a new LCID should be introduced. Similarly if different channel priority is needed for SL-UL prioritization, then new LCID should be introduced too.

In case PHY and/or MAC solution is introduced then in AS layer discovery message can be already differentiated from TX and/or RX UE point of view, so it seems not necessary to introduce solution in e.g. PDCP layer. And this also reflects the majority view during email discussion “[AT111-e][606][Relay] Discovery model and procedure (OPPO)”.

Question1-3.2 in case of shared resource pool, which solution option do you prefer?

Option1: PHY layer solution only

Option2: MAC layer solution only

Option3: both PHY and MAC solutions

Regardless which option do you choose, please elaborate the details of the solution in 3rd column

| Company name | Chosen solution direction | More details of chose solution |
| --- | --- | --- |
| CATT | Option 2 | We suggest option2 and new SL-SRB. |
| Qualcomm | Option 2 | It is confused to us what is specific solution of Option 1? It seems a new bit is introduced in SCI so that RX UE can filter packet in PHY, right? If correct, we think it obviously needs RAN1 impact, which is conflicted with scoping SID. We believe MAC solution (i.e. a new LCID) is a good solution because it can further help to separate the radio resources management for discovery, existing SL SRB (PC5-S and/or PC5-RRC), and other communication traffic. For example, we think it is valid scenario that one relay may have to handle the case to broadcast discovery and send dedicated PC5-S signalling to another peer UE. In this case, it makes sense to have different logic channel priority for discovery and PC5-S. |
| Huawei | Option3 | Regardless of dedicated resource pool/shared resource pool, we agree a new LCID is needed for discovery message as Qualcomm commented. On top of that, we think using the L1 ID in SCI could be worthy to look at if shared RP is agreed. |
| Ericsson (Min) | Option 2 or Option 1 | Agree with QC that, Option 1 would impact RAN1, which is not in the study scope of the SID. However, it is feasible to minimize RAN1 impact, e.g., define a special destination ID to indicate discovery message. In that case, Option 1 is also acceptable. However, there is no need to support both options. Also agree with CATT that a new SL SRB would be needed in order to distinguish discovery from PC5-S and PC5-RRC. |
| Intel | Option 1 or 2 | We think both options are viable, but we prefer option 2. |
| Samsung | Option3 | We can consider separate SL-SRB from PC5-S signalling, new LCID. We are fine to consider using SCI to indicate discovery message. |
| Apple | Option 2 | Unless we cannot solve the issue, we prefer to not involve RAN1 for discovery design. Layer-1 filtering of SL discovery messages is an optimizaton, which is not worth pursuing in the SI phase. We think besides using a new LCID, defining discovery-specific L2 address is also acceptable as MAC layer solutions, |
| Kyocera | Option 2 | We think a new LCID as QC described makes sense, but we could also consider if a new SL SRB can be helpful. |
| OPPO | Option3 | We also agree that a new LCID is necessary. We also think some particular destination ID make sense for both PHY layer and MAC layer in order to filter out discovery message and no additional PHY layer solution is necessary. |
| ZTE | Option 1 or option 2 | Considering that discovery message may need different transmission mechanism, e.g. constant Tx power, thus, PHY layer needs to distinguish whether it is a normal TB or discovery message. Also, MAC layer needs to perform particular LCP procedure targeting on discovery message, thus, it is also necessary for MAC layer to be aware of discovery message. |
| Xiaomi | Option 2 | New SL-SRB is a clean solution. UE could multiplex the discovery message with other SLRB if the destination is the same. |
| Nokia | Option 2 | We share Qualcomm’s view that option 1 is not clearly described and it seems a new SCI format is necessary, which will have RAN1 impact. For option 2 several alternative solutions (new LCID, new SL-SRB) need to be discussed. From the divergent answers here we see that a shared resource pool for discovery message is not the optimal solution, hence option 1 in question 1-3 seems appropriate way forward. |
| Interdigital | Option 3 | We think that a new LCID is necessary in MAC to reuse the existing LCP procedure. Using SCI to indicate the discovery message is necessary for the receiver to determine which message to measure RSRP on. We wonder, however, whether separate resource pool option can be done without any MAC impacts. |
| vivo | Option-2 | If shared pool is agreed then option-2 is a clear and simple solution. Otherwise why not just apply separate resource pools? |
| Philips | Option 3 | Agree with Interdigital |
| MediaTek | Option 3 | We can consider to use separate SL-SRB and new SL LCID for discovery message. Besides, we can consider a specific (new) destination ID dedicated for discovery message, which requires both MAC and PHY for packet filtering. |
| Fraunhofer | Option 3 | We think that a MAC solution i.e. based on LCID is good, as it will help in resource separation for discovery. In case, the shared resource pool is agreed we agree with Huawei it might be beneficial to look at the L1 ID in SCI. |
| Futurewei | Option 3 | It seems premature to limit solution to MAC layer only. |
| ETRI | Option 3 | From MAC point view, new LCID is necessary for prioritized handling of discovery message. In addition, PHY also need to identify whether a TB is discovery message or not. |
| Sharp | Option 3 | PHY layer indication is useful for RSRP measurement of discovery message, and MAC layer is useful for LCP. |
| Convida | Option 2 | Layer-1 filtering of SL discovery messages is a RAN 1 topic. |
| Spreadtrum | Option 2 | We prefer option 2 because option 1 may has an impact on RAN1. |

Regardless what is the answer to question1-3, one issue is common for both approaches i.e. from TX UE point of view whether SL/UL prioritization is needed? And if needed, whether different logical channel priority is necessary. The collision between SL and UL is possible for both separate and shared resource pool. So it is apparently SL/UL prioritization is always needed. And in current specification priority of SL SRB is 1 i.e. the highest priority. If different logical priority is introduced for discovery message, it means its priority value should be lower than 1. And it implies that discovery message has less chance to be transmitted compared to other SL SRBs. RAN2 had following agreement at last RAN2 meeting:

Proposal5: Discovery message is carried over SL SRB with control plane protocol stack similar or identical to PC5-S (PC5-S/PDCP/RLC/MAC/PHY). FFS whether new SL SRB is introduced for discovery message

So from AS layer point of view there is no strong motivation to introduce different logical channel priority for discovery message.

Question1-4: which options do you prefer about logical priority of discovery message?

Option1: same as existing SL SRBs i.e. priority value=1

Option2: different from existing SL SRB i.e. priority value>1

| Company name | Chosen option | More comments |
| --- | --- | --- |
| CATT | See comments | Whether to use a new SRB to carry discovery message is uncertain now, so shall we leave it to WI stage? |
| Qualcomm | Option 2 | As we mentioned in Q1-3, we think it is a valid scenario that one relay may have to handle the case to broadcast discovery message and send dedicated PC5-S signalling to another peer UE at the same shared resource pool. In this case, it makes sense to have different logic channel priority for discovery and PC5-S. And discovery should have lower priority than PC5-S. Thus, we think Option 2 is more reasonable and flexible. |
| Huawei | Postpone to WI phase | We understand this is not a critical issue, we can discuss it in WI phase. |
| Ericsson (Min) | Option 1 | It is enough to share the same priority as other SL SRBs. In case there are both discovery and other SL SRBs, they should be served equally. |
| Intel | Option 2 | We agree that option 2 seems more flexible compared to Option1, even though both options can work ok. |
| Samsung | See comment | We think this is a bit early stage to decide the priority value of discovery message. We do not know yet whether all discovery messages should have the same priority or the priority value should be differentiated among discovery messages. |
| Apple | Too early to decide |  |
| Lenovo, MotM | Postpone to WI phase |  |
| Kyocera | Option 1 | We consider discovery message to be of high importance which would also be used for relay reselection, so our preference is that it should have high priority. |
| OPPO | Option1 |  |
| ZTE | See comments | During LTE sidelink, the PC5 sidelink discovery announcement/monitoring is always has the lowest priority. So we think it is not reasonable to take the priority value as 1 for NR sidelink relay discovery. However, it is a bit early stage to decide this question. |
| Xiaomi | Option1 | It’s up to UE’s implementation to prioritize discovery message or other SL-SRB. |
| Nokia | postpone | Too early to discuss priority values since introduction of new SL-SRB has so far just been discussed as one out of many options for carrying discovery message. |
| Interdigital | See comment | This can be discussed in the WI phase. |
| vivo | Option-1 | Discovery message could be used in cases such as relay selection/reselection which is important for the remote UE to perform communications so the same priority as SL-SRB would be OK. Moreover, this may be related to which SL-SRB is used for discovery messages, e.g. if SL-SRB0 is reused for discovery message then of course the priority value would be 1. |
| Philips | Postpone to WI phase |  |
| MediaTek | Postpone to WI phase |  |
| Fraunhofer |  | This can be left to WI phase. |
| Futurewei | Postpone to WI phase |  |
| ETRI | See comment | Not critical issue at this moment, this issue can be deferred to WI phase. |
| Sharp |  | Can be discussed after there is more conclusion on new SL-SRB. |
| Convida | Too early to decide |  |
| Spreadtrum | Postpone to WI phase |  |

# U2N specific issues

One left issue for U2N relay is whether gNB is SL-capable when discussing radio condition for both remote UE and relay UE when they are connected to gNB. Here is the agreement of last RAN2 meeting:

Proposal12: For U2N relay, relay UE in CONNECTED state is allowed to transmit/receive discovery message if sidelink communication configuration is provided from network.  FFS for the case that the serving gNB is not SL-capable (if applicable)

Here SL capable means whether gNB can provide necessary radio configuration and parameters for both sidelink communication and transmission of discovery message. For L3 solution logically gNB is not aware of existence of remote UE. So if gNB is not SL capable, still relay operation is feasible. The difference is that even relay UE is connected to gNB it can rely on pre-configuration to transmit discovery message. It still aligns with Rel16 design where UE could be out of coverage for NR sidelink communication when it is connected to network. On the other hand if some resource coordination is needed between PC5 and Uu interface e.g. for intra-carrier case, then gNB should be SL-capable otherwise coordination can’t be done properly.

For L2 solution technically gNB can be still not SL capable. But remote UE and relay UE are actually “visible” from gNB point of view since E2E protocol is Uu PDCP protocol layer. So it will be strange that gNB support such E2E relay protocol stack, relay connection and potential service continuity etc. relay operation but it will not provide radio configuration for sidelink communication and transmission of discovery message since anyway it need upgrade to support such relay operation.

Question 2-1: for L3 U2N relay,do you agree relay UE is allowed to transmit discovery message based on pre-configuration when it is connected to a non-SL-Capable gNB?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes | We want to clarify the below two cases: Case1: gNB provide V2X anchor carrier information, relay UE can transmit discovery message based on configuration form anchor carrier of gNB. Case2: gNB cannot provide any radio configuration for SL, relay UE may use preconfiguration for discovery message transition. We reckon that for case1 relay UE transmit discovery message based on configuration from anchor carrier of gNB.That’s to say the Question 2-1 is just only to case2. |
| Qualcomm | Yes | We believe it is a practical scenario which may happen in real deployment (i.e. gNB is in Rel-15 but relay/remote UE has upgraded to Rel-17). Agree with Rapporteur that remote UE is invisible to gNB in L3 relay, and then the Uu link and PC5 links can be regarded as two independent links in AS layer. Therefore, it is the same case we discussed in NR V2X in Rel-16. And relay operation can be supported when relay connecting to Rel-15 gNB without further spec work and any cost, i.e. no need of the network to do anything, not even announcing configurations. For CATT’s comments, please note that NR V2X has specified that the UE decides whether a gNB supports SL based on absence/presence of SIB12 in 38.331. The same condition can be reused in Discovery. |
| Huawei | Postpone to WI phase | We are not quite sure about the meaning of “SL-Capable”. - Does it mean support of sidelink configuration for U2N relay communication (with support of R17 U2N relay function but possibly without support of R16 V2X function)? - Or does it mean support of R16 V2X communication (but without support of R17 U2N relay function)? At this stage we do not know the difference on capability between R17 U2N relay function and R16 V2X function. Thus we suggest to postpone this discussion to WI phase. And if we only discuss the first part of the question “for L3 U2N relay, do you agree relay UE is allowed to transmit discovery message based on pre-configuration”. We do not think it is reasonable that network provides thresholds configuration for discovery but not resource configuration of discovery message. So maybe there is no need to discuss the case that relay UE transmits discovery message based on pre-configuration |
| Ericsson (Min) | Yes | Agree with Qualcomm. |
| Intel | Yes | We think it makes sense to support this scenario, i.e. when the gNB does not support SL operation and relying on pre-configuration for discovery operation can be supported in this case. |
| Samsung | Yes |  |
| Apple | Yes |  |
| Lenovo, MotM | Yes | Agree with Qualcomm. |
| Kyocera | Yes | We agree that this is a realistic scenario and one that can and should be supported using pre-configuration. |
| OPPO | Yes |  |
| ZTE | Yes | Agree with Qualcomm. In order to aviod interference, we think Relay UE can be allowed to transmit discovery message based on pre-configuration only when the UE is out of coverage for NR sidelink carrier. |
| Xiaomi | Yes | Even gNB is SL capable, remote UE could not obtain the configuration from gNB in L3 relay, when it’s out of coverage. |
| Nokia | postpone | We think that the use of “non-SL-Capable gNB” is confusing. Our interpretation is that it means that the gNB does not support Rel-16 V2X function; e.g. it is a Rel-15 gNB. However, L3 relay may work without any gNB impacts and it makes sense to enable L3 Relay operation without any gNB support (using OOC resources for PC5). We think that more analysis is needed and the design of discovery procedure can progress without concluding this issue, thus we propose to postpone this question. |
| Interdigital | Yes | Agree with Qualcomm |
| vivo | Postpone to WI | If we consider LTE, a relay UE have to be configured with resources by gNB for relay PS related sidelink discovery transmission when the UE is RRC_CONNECTED. For the ‘non-SL-Capable gNB’ case if we refer to the out-of-coverage case in LTE, the relay UE may not be allowed to transmit discovery message based on pre-configuration. But We agree with Huawei and Nokia that this ‘non-SL-capable gNB’ case is complicated and ambiguous and may not be the most critical issue so we can postpone it to WI phase. |
| Philips | No | We think it may lead to resource conflicts/interference if the Relay UE starts to operate in sidelink whilst the gNB is not SL capable. In addition, we think that we should have a common solution between L2 and L3. |
| MediaTek | Yes | Agree with Qualcomm |
| Fraunhofer | Yes |  |
| Futurewei | Postpone to WI | We are uncomfortable with the assumption that a UE can advertise and accept relay service without authorization/configuration from its serving gNB. |
| ETRI | Yes |  |
| Sharp | Yes |  |
| Convida | Yes |  |
| Spreadtrum | Yes |  |

Question 2-2: for L2 U2N relay, do you agree relay UE should be always connected to a SL-Capable gNB for relay operation including transmission of discovery message?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | No | gNB only needs to support relay function, supporting SL is optional. |
| Qualcomm | Yes | Agree with Rapporteur that it sounds strange that the relay UE connects to a SL non-capable gNB which however supports L2 relay operations. |
| Huawei | Postpone to WI phase if needed | Similar to Q2-1. We agree that in L2 U2N relay, the network should be able to provide configuration of SL communication between remote UE and relay UE. However, whether this means the network must support R16 V2X still needs FFS. We prefer to discuss this in WI phase together with UE capability. |
| Ericsson (Min) | Yes | Agree with rapporteur and Qualcomm. gNB need to be SL capable. |
| Intel | No | We think this depends on what exactly is meant by ‘SL-capable’ gNB and how the configuration for transmission of discovery messages is defined. Given that sidelink discovery in NR has so far only been discussed in the context of relaying, it should be considered a part of relay functionality. In other words, if a gNB ‘supports’ SL relay functionality, it should provide the configuration necessary for SL discovery operation as well, even if it does not support R16 SL V2X operation. |
| Samsung | Yes |  |
| Apple | Yes | I think here “SL-capable” means capable of “R17 SL relay” functions. Legacy rel-16 SL gNB does not satisfy the condition, and the L2 U2N relay cannot operate as U2N relay if it camps on a legacy R16 gNB. |
| Lenovo, MotM | Yes | We assume that a gNB supporting Relay function supports minimum SL infrastructure for the same including relay related configuration for relay UE as well as remote UE, PC5 resources required for discovery as well as communication etc. This may be called “SL support” or not – matter of terminology. |
| Kyocera | Yes | Agree with QC. |
| OPPO | Yes |  |
| ZTE | Yes | Agree with Rapporteur. If the gNB is non-SL-Capable, it seems strange it can provide L2 U2N relay service. |
| Xiaomi | Yes | We understand only the gNB which is SL capable could support relay operation. |
| Nokia | Yes |  |
| Interdigital | Yes | Agree with the rapporteur-if the gNB supports E2E relaying at L2, it should also support sidelink configuration. |
| vivo | Yes | For L2 relay it is more reasonable to support relay operation only by SL-capable gNB. |
| Philips | Yes | Agree with Apple. In addition, being connected to a SL-capable gNB is essential for avoiding resource conflicts and for service continuity. |
| MediaTek | Yes | Agree with the rapporteur. And, if needed, maybe we can clarify that the “SL-capable” here means “supporting R17 SL relay” rather than “supporting R16 NR V2X”. |
| Fraunhofer | Yes |  |
| Futurewei | See comment | We’d agree that relay UE should be connected to a “SL relay capable” gNB. And this can be required for both L2 and L3 U2N relay. |
| ETRI | Yes |  |
| Sharp | Yes |  |
| Convida | Yes | Agree with the rapporteur |
| Spreadtrum | Yes |  |

Another similar agreement is about remote UE:

Proposal15: for U2N relay, whether remote UE in CONNECTED state is allowed to transmit/receive discovery is based on configuration provided by serving gNB and detail is FFS. FFS for the case that the serving gNB is not SL-capable (if applicable).

The story for remote UE is bit different for L2 and L3 solution. During discussion of [25] one pending proposal is as following:

Revised Proposal 11: For UE to NW relay, RAN2 assumes the remote UE has an active end-to-end connection via only a single relay UE or via Uu at a given time.  The remote UE can have a direct Uu connection or a connection via a single relay UE, but these two connections should not be active at the same time.  Mechanisms for ensuring service continuity (e.g. during path switch) are not precluded.

This proposal is being discussed in post email discussion “[621][Relay] of Service continuity”. At last meeting majority companies prefer not to have dual E2E connection for both L2 and L3 solution. For L2 solution E2E connection actually refers to RRC connection. So if remote UE is to connected a non-SL-Capable gNB directly, remote UE should not be allowed to transmit discovery message unless current Uu connection is lost e.g. due to RLF.

For L3 solution, if same assumption is hold, the E2E connection is in IP layer or even higher than IP layer. So UE should be allowed to (re)select relay UE or even establish PC5 link when remote UE is still connected with network regardless via relay UE or not as long as there is only one E2E connection. It means remote UE can transmit discovery message based on its own decision and rely on pre-configuration.

Question 2-3: for L2 solution, do you agree remote UE connected to a non-SL-Capable gNB directly is not allowed to transmit discovery message unless current Uu connection is lost e.g. due to RLF?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | No | It’s ok if there is no interfere to Uu. |
| Qualcomm | Yes | Agree with the justification of rapporteur on not allowing two E2E connection for L2 relay. Furthermore, we think it also sounds strange that the remote UE connects to a SL non-capable gNB which however supports L2 relay operations. |
| Huawei | Postpone to WI phase if needed | The handling of RLF can be discussed in WI phase. We’d better to focus on the essential case for now. |
| Ericsson (Min) | Yes |  |
| Intel | No (see comment) | Based on our response to the previous question, we think a non-SL-capable gNB does not necessarily means that discovery configuration for relaying is not provided to the UE. So, a more precise question to ask should be whether a remote UE should be allowed to transmit discovery messages if the gNB it is connected to does not provide necessary configuration for transmission of discovery message? In this case, from a dual E2E connection perspective, we don’t think the remote UE should be prohibited from transmitting discovery messages since the remote UE performing SL discovery while having an active Uu connection in this case does not necessarily mean having dual E2E connections. We do however understand that there may be interference issue when the remote UE uses pre-configuration for transmitting discovery message. |
| Samsung | Yes |  |
| Apple | No | It is a valid scenario that UE connects to gNB1 has a weak Uu connection can switch to a relay link to gNB2. So, SL discovery is allowed in this case for remote UE (based on preconfiguration). |
| Lenovo, MotM | Question background is not clear | Unclear question/ scenario: Are we talking a about a gNB that does not support SL operation but however supports L2 relay operations? If so, why would it not provide discovery resources? Further, we assume that any SL operation and relay/ remote operation can be performed on a non-serving-OOC frequency using preconfiguration, if the serving cell does not support SL/ relay operation. |
| Kyocera | No | For the case when the remote UE is directly connected to the non-SL-capable gNB, we don’t think it makes much difference whether it’s for L2 or L3 relaying. So, just as in the case for Q2-1 for relay UE’s discovery transmission in L3, we think the remote UE should also be allowed to transmit discovery message in this scenario. |
| OPPO | Yes | In this case Uu connection should be controlled by gNB completely. So even remote UE is allowed to discovery and select a relay UE, it should not be allowed to leave when Uu connection is still there. |
| ZTE | No | It is valid scenario for remote UE to transmit discovery message when its serving gNB is not SL-capable. More important is whether relay UE’s serving gNB is SL-capable. |
| Xiaomi | No | Although we agree two connection should be avoided, it’s beneficial for remote UE to discover the relay UE early. Remote UE could discover relay but postpone the relay connection establishment. |
| Nokia | Yes |  |
| Interdigital | Yes |  |
| vivo | Postpone | We still don’t reach any agreement on whether two ‘active’ links are possible and if yes in which scenarios. And whether the Uu connection has to be lost is a more detailed issue and we can discuss this after we have consensus on other essential issues. |
| Philips | No | Agree with Apple. In the scenario as described by Apple, it would be best for service continuity if the Remote UE can already start discovery of a relay UEs, before the signal to gNB1 is completely lost, since the discovery process may take some time. Note that this assumes that some conditions can be pre-configured under which it does not result in resource conflicts/interference with the non-SL gNB1. |
| MediaTek | No | We don’t think the restriction is really needed. A remote UE should be able to transmit discovery message as early as possible if Uu connection is weak. |
| Fraunhofer | Yes |  |
| Futurewei | No | It depends on if the serving gNB supports U2N relay (in particular, handing UE off to a relay UE in this case). That is, what matters is if the serving gNB is “SL relay capable”. And this applies to both L2 and L3 relay model. |
| ETRI | Yes |  |
| Sharp | Yes |  |
| Convida | No | Agree with Apple |
| Spreadtrum | No |  |

Question 2-4: for L3 solution, do you agree remote UE connected to a non-SL-Capable gNB is allowed to transmit discovery message on its own based on pre-configuration?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes |  |
| Qualcomm | Yes | Agree with the justification of rapporteur. Furthermore, similar to our comments in Q2-3, we believe remote UE upgrading to Rel-17 and gNB in Rel-15 is a practical scenario which may happen in real deployment. Because Uu link and PC5 links are independent from AS layer perspective, relay operation can be supported when remote UE connecting to Rel-15 gNB without further spec work and any cost. |
| Huawei | No | Similar to Q2-1. But we think it is strange if the network can provide the threshold related configuration for relay and remote UE, but cannot provide radio configuration for discovery message. |
| Ericsson (Min) | Yes | Agree with Qualcomm |
| Intel | Yes |  |
| Samsung | Yes |  |
| Apple | Yes |  |
| Lenovo, MotM | Question background is not clear | Which (discovery) resources are we talking about? Who’s providing them? Please see our previous reply (to 2-3). What are we missing? |
| Kyocera | Yes |  |
| OPPO | Yes |  |
| ZTE | Yes | Agree with Qualcomm. Moreover, in order to avoid interference, remote UE can be allowed to transmit discovery message based on pre-configuration only when the UE is out of coverage for NR sidelink carrier. |
| Xiaomi | Yes |  |
| Nokia | Yes |  |
| Interdigital | Yes |  |
| vivo | Yes or postpone | Similar as Q2-1, we can postpone this discussion to WI phase. But in LTE a UE capable of sidelink remote UE operation can use SL-Preconfiguration including discTxPoolList and preconfigRelay for relay PS related sidelink discovery transmission when out-of-coverage. So it is acceptable to us for the SL-non-capable case. And for the case when gNB is non-SL-capable, we may pursue a unified solution for both L2 and L3. |
| Philips | Depends (see comments) | We think that the behaviour of remote UE should be the same when connected to a non-SL capable gNB, regardless of L2 or L3 solutions. A remote UE may have a weak Uu link and may decide to discover other relays around while it is still connected to the non-SL capable gNB. Also for service continuity it would be best if the Remote UE can already start discovery of a relay UEs, before the signal to non-SL gNB is completely lost, given that the discovery process may take some time. Note that this assumes that some conditions can be pre-configured under which it does not result in resource conflicts/ interference with the non-SL gNB. |
| MediaTek | Yes |  |
| Fraunhofer | Yes |  |
| Futurewei | No | The UE can’t just disappear from the serving gNB. The serving gNB would need to go through proper clean-up for UE to be handed off to U2N relay. Hence, what matters is if the serving gNB is “SL relay capable”. And this applies to both L2 and L3 relay model. |
| ETRI | Yes |  |
| Sharp | Yes |  |
| Convida | Yes |  |
| Spreadtrum | Yes |  |

One editor note is as following in agreed TR [27]

Editor note: For Remote UE out of coverage, it is FFS whether transmission of discovery message is based on configuration from network if the Remote UE is already connected with network through a Relay UE.

When remote UE is connected to gNB through relay UE, it is possible to configure remote UE with radio configuration for sidelink communication and transmission of discovery message in case of U2N L2 relay. But for U2N L3 relay, again remote UE is not visible from gNB technically. And forwarding SIB is only feasible for U2N L2 relay. So in case of U2N L3 relay remote UE out of coverage should always rely on pre-configuration even it is connected to gNB via relay UE. But in case of U2N L2 relay it is feasible to configure remote UE in such case. The left question is whether it is really necessary.

Question 2-5: for L2 solution, is it necessary for gNB to configure an out of coverage remote UE with radio configuration for transmission of discovery message via indirect Uu connection?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | No | We reckon that this is one enhancement and we wouldn’t like to support it. |
| Qualcomm | No strong opinion | We agree that it is feasible in L2 relay. Although we are not sure whether it is really necessary (i.e. it also works if remote UE only relies on pre-configuration), we also don’t see the harm to allow it. If majority prefer to preclude it, we are also fine. |
| Huawei | Yes | We would like to check whether the question is “for L2 solution, an out of coverage remote UE can only transmit discovery message based on network configuration of discovery message after the remote UE connected to the gNB via indirect connection”. If so we support it. |
| Ericsson (Min) | Yes | It would be beneficial to involve gNB’s control. gNB is able to give better configuration for discovery and communication than pre-configuration in terms of resource utilization efficiency and improved discovery performance. |
| Intel | No with comment | We also think this is an enhancement and since we can always rely on pre-configuration, it is not particularly essential to support at this stage. If needed, we can study this aspect with lower priority. |
| Samsung | No | OOC remote UE should use pre-configuration regardless of its connection via relay to NW. |
| Apple | No strong view | This is not essential. Actually, we tend to think TX resource in preconfiguraiton and NW configuration have to be compatible in a certain geographical area, so there seems no difference regarding the contents of configuration. |
| Lenovo, MotM | Yes | We agree it is not essential but will make the configuration consistence and relevant in a larger area – this should be part of the aim (for coverage extension). Furthermore, the OOC remote UE is considered by gNB as normal UE after the OOC remote UE accesses gNB via L2 relay. |
| Kyocera | No | We think the remote UE should just depend on pre-configuration for sending discovery messages. If it depends of gNB’s configuration for the OoC remote UE, then which configuration (pre-configuration or gNB’s configuration) the remote UE uses will depend on its PC5 link status. |
| OPPO | No | Even it is technical feasible, we don’ t think such kind of optimization is necessary. |
| ZTE | Yes | It is reasonable and feasible for the relay UE to forward not only the SIB but also the dedicated sidelink resource configuration to the remote UE. |
| Xiaomi | No | We don’t see the benefit of this operation. |
| Nokia | No | It’s enough if the remote-UE is sending discovery message based on pre-configuration – that works fine already now and there is no time in the SID to discuss additional fancy procedures for an ooc remote-UE. |
| Interdigital | No | While this is possibly in theory, we don’t see a direct benefit of doing this compared to relying on pre-configuration. |
| vivo | Yes | For L2 solution we think the configuration from gNB (e.g. forwarding SIB) is necessary no matter the remote UE is OOC or not. |
| Philips | Yes | In case the UE has been out of coverage for long time or wakes up in different geographical area, some reconfiguration may need to be done via the indirect connection. |
| MediaTek | No strong opinion | Although we don't see a clear benefit, we are fine to follow the majority view. |
| Fraunhofer | Yes | Agree with Lenovo. |
| Futurewei | Yes | Network should be able to control the transmission of discovery message, when UE is connected. This should be consistent, for the sake of network and UE implementation, to direct and indirect connections. |
| ETRI | Yes | Agree with ZTE’s view |
| Sharp | No strong view | It is feasible but the benefit seems not so clear comparing with pre-configuration. |
| Convida | Yes | It would be beneficial to involve gNB’s control. |
| Spreadtrum | No | For OOC remote UE, it should use pre-configuration to transmission discovery message. |

Question 2-6: for L3 solution, can you confirm that it is not feasible for serving gNB to configure an out of coverage remote UE with radio configuration for transmission of discovery message?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes |  |
| Qualcomm | Yes | Because remote UE is not visible to gNB in L3 relay, it is not feasible for gNB to configure OOC remote UE on discovery via relay. |
| Huawei | Yes | It is not feasible even after the remote UE connected to the gNB via a L3 relay UE. |
| Ericsson (Min) | Yes | As Qualcomm said, remote UE is invisible to gNB. it is unnecessary to involve gNB’s control in this case. |
| Intel | Yes |  |
| Samsung | Yes | OOC remote UE should use pre-configuration. |
| Apple | Yes |  |
| Lenovo, MotM | Not sure | Clearly, the gNB does not see the L3 remote UE but it can know (or should expect) that there are such remote UEs – simply since the relay needs some support/ configuration/ resources for the same. If so, it should be possible to provide a “common configuration” via the relay UE(s). |
| Kyocera | Yes | Agree with QC. |
| OPPO | Yes |  |
| ZTE | No | Although the gNB may be not aware of remote UE, but the relay UE can forward the SIB including the sidelink resource configuration to the remote UE. |
| Xiaomi | Yes |  |
| Nokia | Yes | We think that this is not feasible and not necessary either. |
| Interdigital | Yes | Relaying of configuration to the remote UE should only be possible if the configuration is used for a CONNECTION by the remote UE where the gNB is aware. This is not the case for L2 relay. |
| vivo | No | Agree with Lenovo that it is possible for gNB to provide configuration via L3 relay UE for OOC UEs. We may not exclude this case in study phase. |
| Philips | Not sure | We agree with Lenovo that it is not impossible for the gNB to find out but would probably lead to extra complexity to make it work. |
| MediaTek | Yes |  |
| Fraunhofer | Yes |  |
| Futurewei | Yes | Dedicated control is not possible. |
| ETRI | Yes | It is feasilbe to use pre-configuration. |
| Sharp | Yes |  |
| Convida | Yes | Remote UE is not visible to gNB in L3 relay |
| Spreadtrum | Yes |  |

Another editor note is as following in agreed TR[27]

Editor note: For Remote UE in RRC_IDLE or RRC_INACTIVE state, the details of the idle measurements and possible additional network configuration is FFS.

When remote UE camps on a suitable cell, remote UE should conduct measurement for normal cell reselection. And the measurement of serving cell must be there otherwise cell reselection is not feasible. The Uu radio condition for remote UE is measurement result of its serving cell. So technically such measurement can be reused without change. So it is rapporteur’s understanding no additional network configuration is needed.

Question 2-7: Do you agree no additional network configuration is needed for remote UE in RRC_IDLE or RRC_INACTIVE

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes |  |
| Qualcomm | Yes | We have the same understanding as rapporteur that IDLE/INACTIVE remote UE will perform legacy IDLE measurement without spec impacts. Note that in LTE Rel-13 Prose relay, there is also no additional enhancement on Uu measurement for IDLE remote UE. |
| Huawei | Yes | The evaluation on Uu RSRP of camping cell can be based on the idle/inactive measurement for cell (re)selection, no additional work is needed. |
| Ericsson (Min) | Yes | We agree that there is no additional network configuration needed. In addition, we would like to point out that all Uu measurement features in Rel-16 should be equally applicable to remote UE, in case this is in coverage. |
| Intel | Yes |  |
| Samsung | Yes | Existing idle measurement can be used. |
| Apple | Yes |  |
| Lenovo, MotM |  | Existing measurement mechanism for idle UE can be reused. The idle/inactive UE is expected to perform registration update and RAN based notification area update. For example, each of inactive remote UE needs to perform RNAU based on timer. It could be beneficial that relay UE periodically indicate to gNB for a group of the served inactive remote UEs. |
| Kyocera | Yes |  |
| OPPO | Yes |  |
| ZTE | Yes | No additional network configuration is needed for remote UE in RRC_IDLE or RRC_INACTIVE. |
| Xiaomi | Yes |  |
| Nokia | Yes |  |
| Interdigital | Yes |  |
| vivo | Yes with comments | The measurement configuration mechanism for IDLE/INACTIVE UE can be reused of course, but as the measurement result may be used for cell reselection or Relay (re)selection, the concrete measurement configuration content can be different (e.g. different reporting threshold, different measurement quantities, etc.) |
| Philips | Depends (see comments) | Assuming that this question is only about measurements of gNBs, then yes. For performing measurements on discovery messages and other sidelink measurements a separate discussion is happening in [622] |
| MediaTek | Yes |  |
| Fraunhofer | Yes |  |
| Futurewei | Yes |  |
| ETRI | Yes |  |
| Sharp | Yes |  |
| Convida | Yes |  |
| Spreadtrum | Yes |  |

# U2U specific issues

At last meeting RAN2 agreed:

Proposal2: Model A/ B discovery model similar to LTE is reused for U2U relay also

And RAN2 also agreed radio condition to allow remote or relay UE to transmit discovery message. This section mainly examine whether any radio condition is necessary for U2U relay operation w.r.t transmission of discovery message.

There are several agreements regarding scenarios of U2U relay operation as listed below:

For UE to UE relays, any of the UEs involved in relaying can be either in coverage or out of coverage.  RAN2 will strive for a common solution to the in- and out-of-coverage cases.

Revised Proposal 7: For the UE to UE relay, RAN2 supports the scenario that UEs can be in coverage of the different cell.  RAN2 will strive for a common solution between same cell and different cell cases for this scenario.  If a common solution is not possible and impacts are found to supporting different cell case, RAN2 works on the same cell case with higher priority.

Proposal 22: RAN2 assumes no restrictions on the RRC states of any UEs involved in UE to UE relaying.

Listed agreements basically say no limitation is necessary on coverage, serving cell and RRC states for both remote UE and relay UE. Such agreements are natural outcome because the main motivation of U2U relay is to extend PC5 coverage i.e. nothing to do with network coverage. For U2N relay operation, apart from upper layer triggering the radio condition to allow transmission of discovery message are all related to signal quality of Uu interface. For U2U relay such radio condition is not applied i.e. as long as remote or relay UE is triggered by upper layer, they should be allowed to transmit discovery message. On the other hand network coverage matters for radio configuration for transmission of discovery message. Similar to sidelink communication both remote UE or relay UE can rely on pre-configuration unless relevant radio configuration is provided by network, either via system information or dedicated signalling which is subject to UE’s RRC state and gNB’s capability.

Question 3-1: Do you agree relay UE or remote UE is allowed to transmit discovery message when it is triggered only by upper layer?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes | Discovery model is decided by upper layer, hence it is not feasible for AS layer to trigger the discovery message transmission. |
| Qualcomm | Yes | Agree with Rapporteur’s understanding that radio condition is not applied for U2U relay because main use scenario of U2U relay is all UEs (source, destination and relay) in OOC. |
| Huawei | Yes with comments | Our understanding is that upper layer request is one precondition, the other is discovery related configuration from network configuration for IC case and per-configuration for OOC case. |
| Ericsson (Min) | Yes with comments | For relay UE reselection, remote UE needs to consider the PC5 signal strength of current relay UE, which is the same as for U2N case. For relay UE reselection, remote UE may or may not transmit discovery message depending on whether stored information (obtained from previous discovery procedure) is still valid/relevant. |
| Intel | Yes with comment | We also agree that it is primarily the upper layer that triggers the discovery procedure. If we want to aim for a common solution as listed in the discussion above, this is the most straightforward way to move forward. At the same time, we assume that some AS involvement in terms of informing/assisting the upper layer about the status of the direct PC5 link (e.g. RLF indication) is needed to trigger discovery procedure. |
| Samsung | No | From relay UE perspective, the upper layer triggers discovery message transmission. But from remote UE perspective, the discovery can be triggered based on radio condition between the two UEs as well as upper layer. |
| Apple | Yes | Upper layer triggering is a necessary condition. |
| Lenovo, MotM | No (but early to decide) | For a number of reasons: - UE needs to maintain a reliable link on U2U and until we are sure that there will be a RLM procedure (HARQ based or otherwise), we can’t rule out the discovery procedure to do this job as well. - We assume that an OOC UE may need certain SIB from a reliable relay’s serving cell. It’s too early to rule this out as well. - For one relay case, the relay UE will indicate to the transmitting UE when the link between relay UE and the receiving UE fails. The transmitting UE may perform relay reselection. For multi-hop case, the relay UE may indicate to the transmitting UE/receiving UE when the intermediate hop between two relays fails. The transmitting UE/receiving UE may perform relay reselection when receiving the notification. |
| Kyocera | Yes | For all UEs that are OoC, since pre-configured resources will be used, transmission of discovery message can be decided by the upper layer. In case the UE is in-coverage, it will also depend on the available discovery resources configured by the gNB. |
| OPPO | Yes |  |
| ZTE | Yes | Since the radio condition of Uu has no impacts on U2U relay operation, so the Uu radio condition will have no impact on whether the discovery message can be triggered. But if the remote UE is in coverage, whether the discovery message can be triggered depends on the radio configuration including whether the U2U relay is allowed and SL relay resource is provided by network. |
| Xiaomi | Yes with comment | Upper layer request should not be the only condition to trigger U2U discovery message transmission. |
| Nokia | Yes | Trigger to request discovery message comes from upper layer. |
| Interdigital | Yes with comment | Discovery message should be triggered by upper layers but there should be some assistance by AS layer (e.g. providing conditions of the PC5-link). |
| vivo | Yes with comments | Agree with Huawei and ZTE. And we think it is clear that the main principle here is that in U2U relay, the remote UE or relay UE can be allowed to transmit discovery message without considering network coverage, but by saying it is triggered only by upper layer we just exclude other potential triggering condition which may be supported in the future. |
| Philips | No | Agree with Samsung |
| MediaTek | Yes with comments | The trigger for discovery message transmission is from upper layer, but whether a UE is allowed to transmit may also depend on AS layer condition such as measurement results of PC5 links. |
| Fraunhofer | Yes | We agree that the triggering condition of discovery message is based on upper layers. |
| Futurewei | Yes | Upper layer can trigger transmission of discovery message. |
| ETRI | Yes |  |
| Sharp | Yes with comments | The discovery message transmission/reception is triggered by upper layer, and whether the discovery message is allowed to transmit/receive also consider the PC5 link condition. |
| Convida | Yes with comments | Upper layer request should not be the only condition to trigger U2U discovery message transmission. |
| Spreadtrum | Yes |  |

Question 3-2: For relay UE and remote UE, do you agree radio configuration used for transmission of discovery message follows same principle as defined for sidelink communication?

| Company name | Position (yes, no) | More comments |
| --- | --- | --- |
| CATT | Yes |  |
| Qualcomm | Yes | Agree with Rapporteur’s understanding. However, we think what is “same principle” may not be clear. Thus, we suggest making it clear in proposal, e.g. “As U2N relay, both remote UE and relay UE in U2U relay can rely on pre-configuration unless relevant radio configuration is provided by network, either via system information or dedicated signalling” |
| Huawei | Yes | We understand the same principle as SL communication means network configuration for IC case and pre-configuration for OOC case. |
| Ericsson (Min) | Yes | Agree with Qualcomm’s suggestion. |
| Intel | Yes (see comment) | The question is not clear without some context. We think it is better to ask whether the following statement (listed above by rapporteur) is agreeable: “Similar to sidelink communication, both remote UE or relay UE can rely on pre-configuration unless relevant radio configuration is provided by network, either via system information or dedicated signalling which is subject to UE’s RRC state and gNB’s capability” If that is indeed the intention, we think it is agreeable as first principle. |
| Samsung | Yes | Depending on its RRC state (connected, idle, inactive, out of coverage), relay UE and remote UE can use the configuration as defined for sidelink communication. |
| Apple | Yes |  |
| Lenovo, MotM | Yes | Agree with QC/ Intel |
| Kyocera | Yes | Agree with QC |
| OPPO | Yes | QC’s version is fine for us |
| ZTE | Yes |  |
| Xiaomi | Yes | Does this proposal imply shared resource pool is used for discovery and communication? |
| Nokia | Yes | Agree with Qualcomm’s proposal for rewording as meaning of “same principle” is not clear. |
| Interdigital | Yes | Agree with Huawei on the interpretation of “same principle”. |
| vivo | Yes | Agree with QC. |
| Philips | Yes | Agree with Qualcomm |
| MediaTek | Yes | Agree with QC/ Intel. |
| Fraunhofer | Yes | Agree with Qualcomm. |
| Futurewei | Yes |  |
| ETRI | Yes | Agree with Qualcomm |
| Sharp | Yes |  |
| Convida | Yes | Agree with QC |
| Spreadtrum | Yes |  |

In case answer to question 1-2 is No, and since RAN2 agreed the protocol stack of discovery message is identical or similar to SL SRB for PC5-S, then only approach is to reuse one of the existing SL SRBs.

Question 3-3: for U2U relay, if discovery message is not visible in AS layer, which SL SRB is reused for discovery message?

| Company name | Chosen SL SRB | More comments |
| --- | --- | --- |

Note: if you answer yes to question 1-2, please ignore this question.

# Reference

- 3GPP RP-193253 "New SID: Study on NR sidelink relay".

- 23.752 Study on system enhancement for Proximity based Services v0.4.0

- R2-2006556	Discussion on relay discovery model / procedure   	Qualcomm Incorporated	discussion	Rel-17	FS_NR_SL_relay

- R2-2006573	Initiation of relaying operation	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

- R2-2006612	Discovery Model/Procedure for NR Sidelink Relay	CATT	discussion	Rel-17	FS_NR_SL_relay

- R2-2006738	Discussion on relay discovery and link management	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

- R2-2006761	Discovery Procedure for SL Relaying	InterDigital	discussion	Rel-17	FS_NR_SL_relay

- R2-2006771	Discussion on SL relay discovery procedure	OPPO	discussion	Rel-17	FS_NR_SL_relay

- R2-2006862	NR Sidelink Relaying Discovery	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

- R2-2006868	Discovery Model and Procedure in NR Sidelink Relaying	Fujitsu	discussion	Rel-17	FS_NR_SL_relay

- R2-2006931	On Sidelink Discovery for Relaying	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

- R2-2006969	Sidelink relay discovery model and procedure	Samsung Electronics Co., Ltd	discussion	Rel-17	FS_NR_SL_relay

- R2-2007042	Discussion of Relay UE discovery	vivo	discussion	Rel-17

- R2-2007045	Discussion on discovery procedure for sidelink relay	Spreadtrum Communications	discussion

- R2-2007098	Discussion on NR Sidelink Relay Discovery	Apple, Convida Wireless	discussion	Rel-17	FS_NR_SL_relay

- R2-2007291	Discovery aspects for NR sidelink relay	Ericsson	discussion	Rel-17	FS_NR_SL_relay

- R2-2007476	Considerations on discovery procedure for sidelink relay	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2008045	Consideration of discovery model/procedure for sidelink relay	LG Electronics Inc.	discussion	Rel-17	FS_NR_SL_relay

- R2-2008049	Common aspects for L2 and L3 UE-to-Network relay	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

- R2-2006641	L2 vs L3 - Relay/Remote UE Authorization, Service Continuity	Fraunhofer HHI, Fraunhofer IIS	discussion

- R2-2006770	 Discussion on SL relay (re)selection and authorization	OPPO	discussion	Rel-17

- R2-2007040	Selection/Authorization and Security for L2 and L3 relay	vivo	discussion	Rel-17

- R2-2008255	Discovery model and procedure	OPPO	discussion	Rel-17	FS_NR_SL_relay

- R2-2008252	[AT111-e][603] Scope, requirements, and scenarios (Interdigital)	InterDigital	discussion	Rel-17	FS_NR_SL_relay

- R2-2008264	[AT111-e][603] Scope, requirements, and scenarios (Interdigital)	InterDigital	discussion	Rel-17	FS_NR_SL_relay

- S2-2006553 was 5839r07 Key Issue#1 Interim conclusions on message contents_QL

- R2-2008274 TR38.836 V010