3GPP TSG-RAN WG2 Meeting #111-e	R2-200xxxx

Online, 17 – 28 August 2020

Agenda item:	6.4.3

Source: 	Huawei (Rapporteur)

Title: 	Discussion of [POST111-e][705][V2X] Proposal 2-1 and 2-2  (Huawei)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [POST111-e][705][V2X] Proposal 2-1 and 2-2  (Huawei)

Discuss the proposal 2-1 and 2-2, and if possible prepare agreeable 38.321 CR in R2-2008583 (discussion in R2-2008584 if needed). Note cover-page should be written well (with reasonable impact analysis). CR will be approved by email. Deadline is 9/3 10:00am (UTC).

Proposal 2-1: timeReferenceSFN is captured in ASN.1. Or, timeReferenceSFN is removed from 38.321. Proposal 2-2: the definition of N and numberOfSLSlotsPerFrame in the CR to 38.321 are agreed.

Companies are requested to provide their views on the issues listed in this document.

In current TS 38.321, the equation for the sidelink configured grant type 1 occasion is shown as below:

After a sidelink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the first slot of the Sth sidelink grant occurs in the logical slot for which:

[(SFN × numberOfSLSlotsPerFrame) + logical slot number in the frame] =
 (timeReferenceSFN × numberOfSLSlotsPerFrame + sl-TimeOffsetCGType1+ S × PeriodicitySL) modulo (1024 × numberOfSLSlotsPerFrame).

where , and numberOfSLSlotsPerFrame and N refer to the number of logical slots that can be used for SL transmsission in the frame and 20ms, respectively, as specified in clause 8.1.7 of TS 38.214 [7].

# Remaining issues on calculation of sidelink configured grant occasion

The leftover issues for calculation of sidelink configured grant occasion are listed below.

#### Issue 1: Necessity of timeReferenceSFN

In R2-2007919, it has been suggested that the parameter timeReferenceSFN should be reused to determine the occasion of SL CG type-1 in the MAC formula for the following reasons:

- When it comes to SL CG type-1, it was agreed by RAN1 in #100bis meeting that the UL formula shall be reused for determining the occasion of SL CG type-1 with several changes (e.g. using logical indexes, using slot-level granularity and so on). And SFN is employed to derive slots in the formula as mentioned in the introduction section. Therefore, it is reasonable to inherit the time reference for SL CG type-1 and utilize it to calculate the Sth SL grant in MAC specification.

In R2-2006704, it has been suggested to remove the parameter from the equation in 5.8.3 for the following reason:

- timeReferenceSFN is not supported in RRC ASN.1.

Thus, RAN2 can discuss whether to keep the parameter timeReferenceSFN in the equation for sidelink configured grant occasion or not.

Note that in RAN1#100bis meeting, RAN1 achieved the following agreement for sidelink configured grant. RAN1 agreed to use the Uu formula with the following changes as listed below. However, RAN1 never agreed to remove the timeReferenceSFN from the formular.

Agreements: - The slots for sidelink transmission for CG type-1 are determined using the UL formula in 38.321 with the following changes: - Using slot-level granularity instead of symbol-level granularity (i.e., remove numberOfSymbolsPerSlot, “symbol number in the slot”, S in the formula) - periodicity is in number of slots - timeDomainOffset is expressed in number of slots - FFS: frame indexing (e.g., SFN, or virtual frame number) - FFS: whether logical or physical slots are used

Question 1A:	Do companies agree to keep the parameter timeReferenceSFN in the equation for sidelink configured grant occasion in section 5.8.3 in TS 38.321?

- Yes.

- No.

| Company | Yes/No | Comment |
| --- | --- | --- |

In R2-2007919, it has been suggested that a separate sl-TimeReferenceSFN-Type1 should be defined in RRC configuration for SL CG type-1 for the following reasons:

- The use of time reference timeReferenceSFN has been captured in above equation, but its definition and/or explanation is absent, and it is unclear whether the parameter in formula for SL CG is the one indicated in the UL CG configuration because the name of these two parameters are the same. Therefore, we propose that a separate sidelink-specific parameter, sl-TimeReferenceSFN-Type1 should be introduced in the RRC massage for SL CG type-1, like in R16 Uu.

Thus, RAN2 can discuss whether to define a separate sl-TimeReferenceSFN-Type1 in RRC configuration for sidelink configured grant type 1 or not.

Question 1B:	If the answer to Question 1A is yes, do companies agree to define a separate sl-TimeReferenceSFN-Type1 in RRC configuration for sidelink configured grant type 1?

- Yes.

- No.

| Company | Yes/No | Comment |
| --- | --- | --- |

In R2-2007919, it has been suggested to clarify that sl-TimeOffsetCGType1 should be the offset of a resource with respect to the frame of which SFN = sl-TimeReferenceSFN-Type1 for the following reasons:

- With the introduction of sl-TimeReferenceSFN-Type1, SFN0 or SFN512 is configured explicitly to the UE for the purpose of determining each SL CG occasion. For instance, if the parameter indicates that SFN512 is used, the first available SL CG occasion occurs after an offset sl-TimeOffsetCGType1 corresponding to SFN512; otherwise, sl-TimeOffsetCGType1 is considered by taking SFN0 as reference. In a word, the time offset should take the boundary given by sl-TimeReferenceSFN-Type1 as a reference, not always the SFN0 as captured in the current MAC spec. To align with the principle defined for UL CG type-1, sl-TimeOffsetCGType1 should be the offset of a resource with respect to the frame of which SFN = sl-TimeReferenceSFN-Type1.

Thus, RAN2 can discuss whether sl-TimeOffsetCGType1 should be the offset of a resource with respect to the frame of which SFN = sl-TimeReferenceSFN-Type1.

Question 1C:	If the answer to Question 1B is yes, do companies agree sl-TimeOffsetCGType1 should be the offset of a resource with respect to SFN = sl-TimeReferenceSFN-Type1, which is similar as in Uu?

- Yes.

- No.

| Company | Yes/No | Comment |
| --- | --- | --- |

#### Issue 2: Definition of N

In R2-2006704, it has been suggested to clarify that N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration, if configured, for the following reason:

- It is currently specified that numberOfSLSlotsPerFrame and N refer to the number of logical slots that can be used for SL transmsission in the frame. However, accoding to 38.214 subcaluse 8.1.7, N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration. Thus, the definition of N should be changed according to 38.214.

Thus, RAN2 can discuss whether to some clarification on the definition of N is needed or not.

Question 2A:	Do you agree that RAN2 needs to clarify the definition of N?

- Yes (The definition of N is not clear in the current specification )

- No, (The definition of N is already clear in the current specification).

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 2B:	If the answer to Question 2A is yes, do you agree with the following change on the definition of N?

N refers to the number of logical slots that can be used for SL transmsission in the frame and within 20ms, respectivelyif configured, of TDD-UL-DL-ConfigCommon as specified in TS 38.331 [5] and clause 8.1.7 of TS 38.214 [7]

- Yes

- No, (please provide your suggestion if any).

| Company | Yes/No | Comment |
| --- | --- | --- |

#### Issue 3: Definition of numberOfSLSlotsPerFrame

In R2-2007919, it has been suggested to clarify that numberOfSLSlotsPerFrame refers to the number of logical slots that can be used for SL transmsission within the associated resource pool in the frame for the reason that:

- RAN1 has agreed that logical instead of physical index is adopted in the SL CG formula. In addition, a SL CG is allowed to be associated with only one transmitting resource pool. This means that SL grants occur periodically and logically in single transmitting resource pool. Therefore, when deriving SL CG occasion, only slots used for sidelink within the same resource pool are counted, i.e. numberOfSLSlotsPerFrame refers to the number of logical slots that can be used for SL transmission(s) within the associated resource pool in the frame.

Note in this RAN2 meeting, RAN2 agreed with the addition of the missing resource pool ID associated with each configured sidelink type-1 configuration to reflect that the configured sidelink grant is assocaited with a single transmit resource pool.

Thus, RAN2 can discuss whether to some clarification on the definition of numberOfSLSlotsPerFrame is needed or not.

Question 3A:	Do you agree that RAN2 needs to clarify the definition of numberOfSLSlotsPerFrame?

- Yes (The definition of numberOfSLSlotsPerFrame is not clear in the current specification )

- No, (The definition of numberOfSLSlotsPerFrame is already clear in the current specification).

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 3B:	If the answer to Question 3A is yes, do you agree with the following change on the definition of numberOfSLSlotsPerFrame?

numberOfSLSlotsPerFrame refers to the number of logical slots that can be used for SL transmsission within the associated resource pool in the frame.

- Yes

- No, (please provide your suggestion if any).

| Company | Yes/No | Comment |
| --- | --- | --- |

#### Issue 4: Definition of sl-TimeOffsetCGType1

In R2-2006704, it has been suggested to clarify that sl-TimeOffsetCGType1 refers to the number of logical slots that can be used for SL transmsission.

Thus, RAN2 can discuss whether to some clarification on the definition of sl-TimeOffsetCGType1 is needed or not.

Question 4A:	Do you agree that RAN2 needs to clarify the definition of sl-TimeOffsetCGType1?

- Yes (The definition of sl-TimeOffsetCGType1 is not clear in the current specification )

- No, (The definition of sl-TimeOffsetCGType1 is already clear in the current specification).

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 4B:	If the answer to Question 4A is yes, do you agree to clarify that sl-TimeOffsetCG-Type1 refers to the number of logical slots that can be used for SL transmission?

- Yes

- No, (please provide your suggestion if any).

| Company | Yes/No | Comment |
| --- | --- | --- |

# Conclusion

# Reference

[1] RAN2#111e meeting minutes

[2] RAN1#100bis meeting minutes

[3] R2-2007919 Discussion on the calculation of SL CG occasion, Huawei, Hisilicon

[4] R2-2006704 Corrections to 5G V2X with NR Sidelink, LG Electronics Inc.

[5] TS 38.321 specification