3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Online, 2 – 13 November 2020

Title:	[DRAFT] LS on RAN1 agreement on pre-emption

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	LG Electronics [To be RAN2]

To:	RAN1

Cc:

Contact Person:

#### Name:	Youngdae LEE

Tel. Number:

E-mail Address:	youngdae.lee@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 previously sent the LS to RAN1 in R2-2008585 including the issue on whether to perform reselection for dropped retransmission after checking whether there is(are) pre-empted resource(s). Meanwhile, RAN2 recognized that RAN1 made a new agreement on pre-emption in RAN1#102-e as follows:

- If periodic reservation is in use by a UE, and if pre-emption is enabled in a resource pool, the UE checks pre-emption for resources provided by MAC layer to L1, according to specified procedures

  - L1 expects that MAC layer provides resources intended for transmission of one TB, which can fit to resource selection window of current TB of the UE, and for which the relevant priority is available

  - If a resource is pre-empted, a re-selection for the pre-empted resource is triggered based on the specified step 1 and step 2 procedures,

    - with details up to UE implementations, including whether/how to set the reservation period in the re-selected resource

  - FFS in TP phase how/where to capture this in specification

    - During the pre-emption checking, j is up to Cresel-1

RAN2 understands that the above agreement is related to the step of checking whether there is(are) pre-empted resource(s) which has been specified in 38.321. RAN2 would like to ask RAN1 to clarify the above agreements that may impact MAC specification:

Q1: Does the TB in the above agreement correspond to only MAC PDU created for transmission?

Q2: Is it correct understanding that whenever a MAC PDU is created, UE checks whether there is(are) pre-empted resource(s) among all remaining periodic reserved resources assuming that the periodic reserved resources are used for transmissions of potential MAC PDUs which have the same priority with the created MAC PDU?

Q3: Does RAN1 assume that only retransmission resources in periodic reservation are checked for pre-emption?

2. Actions:

To RAN1 group

RAN2 respectfully requests RAN1 to provide their answers to the above questions.

3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#113-e	25 Jan - 5 Feb 2021   	Online

3GPPRAN2#114-e	19 – 27 May 2021   	Online