3GPP TSG-RAN WG2 #112-e	R2-20xxxxx

E-meeting, November 2020

Agenda item:	8.4.1

Source: 	Samsung

Title: 	Report from email discussion [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation (Samsung)

Document for:	Discussion and decision

# Introduction

This submission contains the report from the following post-RAN2#111-e email discussion:

- [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation (Samsung)

Scope: Aim to clarify the scope. Determine which technical issues to address as a part of this WI objective. For better understanding and illustration of the issues only also solutions can be discussed, but solution decisions will NOT be taken based on this email discussion.

Intended Outcome: Report

Deadline: long

As a reminder, the following has been agreed as one of the objectives of the Rel-17 IAB WI:

Topology, routing and transport enhancements [RAN2-led, RAN3]:

- Specifications of enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation

Starting from guidelines provided by the Chair in the email discussion description above, and the submissions on the relevant issues to RAN2#111-e (none of them were treated as there were no TUs allocated to Rel-17 IAB WI at RAN2#111-e, but are a very useful and valid starting point nonetheless) this document aims to clarify the scope by identifying specific technical issues to be addressed.

# Phase-1: clarifying the scope

## Topology-wide fairness

This is perhaps the aspect of the Rel-17 IAB WI where we have least Rel-16 work to fall back on – although the issue was addressed briefly during the SI phase and some aspects are captured in the IAB TR. The following is a list of issues identified by the rapporteur of the discussion, based on submissions to RAN2#111-e:

- Fairness definition and metrics:

  - Focus of fairness (e.g. from a single-node point of view; fairness across different nodes; across different paths – i.e. load balancing across different paths and nodes to achieve resource utilization fairness, etc.)

  - Granularity of fairness (e.g. UE bearer level, BH RLC channel level, BH link level, fairness between access and backhaul links, etc.)

  - Metrics used to evaluate fairness (e.g. resource usage, fair and efficient use of scheduling opportunities, meeting QoS requirements in a uniform manner, etc.)

- Whether there are any differences in approaches to fairness for 1:1 and N:1 mapping cases, and whether the 1:1 case at all requires any fairness-related enhancements

- Who enforces fairness (Donor CU, parent IAB node(s), …) and whether we should support local decision-making for cases other than RLF

- Whether additional information exchange is needed on top of Rel-16 baseline to ensure topology-wide fairness

- If additional information exchange and additional procedures are needed on top of Rel-16 baseline, what are they? Examples include (but are not limited to):

  - Whether additional measurement reports are needed

  - Which layer (or layers) is used for exchange of additional information (if any)

  - What is the flow direction of the additional information exchange

  - Whether the additional information (if any) introduces unacceptable levels of overhead

  - Whether to allow splitting data of a radio bearer into two or more paths

Question 1: Do you agree that the above is a comprehensive list of high-level issues impacting topology-wide fairness? If you feel certain issues have been missed, please add them below (but bear in mind they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |
| Kyocera | We agree with the rapporteur’s view. |
| Intel | Yes, we agree that the above list has covered well for fairness discussion. We would like to propose to add UE logical channel level as a candidate in granularity of fairness discussion list, where more accurate scheduling can be considered. Besides, item 5/e “splitting data of a radio bearer into two or more paths” can bring benefit to many areas, including fairness and multi-hop latency reduction. So, we propose to consider this point in Q2 as well. |
| Huawei | The summary looks comprehensive. Before we agree to consider/study something in the WI, we need to first clarify what’s the issue which cannot be solved by R16. Please note in the R16 F1AP signaling, we do have the per Hop QoS configuration on BH RLC from CU to DU. This will help the CU implementation to control the per hop BH RLC QoS parameters in the DU scheduler, which will take into account all the topology related fairness by CU. That means regardless the topology, CU will instruct DU to guarantee all UE traffic’s QoS. So, we prefer to first discuss “What fairness cannot be supported by R16 spec/implementation and needs to be specified in R17”. At last, “whether we should support local decision-making for cases other than RLF” in 3 and 5-e can be discussed in another topic, which seems not tight related to the “fairness”. |
| CATT | We agree with rapporteur’s views about the topology-wide fairness. |
| LG | Generally fine with following comments. Since the bullet 5 depends on the solution and the guided email discussion scope discourages specifying specific solution, we think that it may be too early to discuss the bullet 5 (probably bullet 4 as well) in this email discussion. We may revisit this point after determining more concrete candidate solutions. In addition, it would be good to clarify what the topology-wide fairness means from RAN2 point of view which may be different from RAN3 perspective. |
| ZTE | We agree with the majority issues issues to be discussed for fairness enhancement. For 5-e, it is agreed in last RAN3 meeting to deprioritize Multi-Route Support with data split in IAB. We think the data split into two or more paths could also be deprioritized in RAN2. |
| Nokia, Nokia Shanghai Bell | In this area we think issues = shortcomings justifying enhancements have been pointed out. The above list seems comprehensive. |
| Ericsson | We agree with the list of topics proposed by the Rapporteur for the Rel.17 scope. However, RAN2 should first discuss what is not covered by Rel.16 specification and what can be achieved via implementation. Hence, we believe that before pursuing a certain enhancement, RAN2 should evaluate pros and cons of each enhancement, as well as the level of complexity. |
| Fujitsu | We generally agree with the rapporteur’s view, with some comments on item 3: For uplink, the fairness can be enforced to specific logical channels via LCP procedure enhancement. It can be done at each IAB node. Local route selection is being discussed in another thread [903] and not so related to fairness. Based on the above, we would like to change item 3 to: 3. Who enforces fairness (Donor CU, parent IAB node(s), …) and whether we should support local decision-making for cases other than RLF |
| AT&T | For 1b this should also consider fairness over multiple hops For 5a the additional information could be measurement reports or could be more generic scheduling/benefit metrics |
| Futurewei | We have a few comments on the issues above: - Fairness definition and metrics: - We don’t understand why we would need to consider fairness across different routing paths. Our understanding is that different BH routing paths are typically defined so as to support different QoS requirements (e.g. one path may have few hops resulting in lower end-to-end latency, while another may support higher bandwidth, but with higher latency). The donor would map a UE bearer to an appropriate path that best supports its QoS needs. Hence, we would not expect necessarily expect to have “fairness” across different routing paths towards the same access IAB node as an objective. - Regarding granularity, it seems that only UE bearer level is useful to consider when assessing fairness. BH links and RLC channels are not visible to the end user, and do not directly contribute to user experience. Hence, it is not particularly meaningful to study fairness for the BH link. - Who enforces fairness… It is not clear to us how RLF is related to fairness. Can the rapporteur please elaborate on the intent here? |
| Sony | Agree with the rapporteur’s view |
| Qualcomm | We have a different view on fairness vs. load balancing, since these are very different objectives: - Fairness: Resources are allocated so that UEs receiving same service have same or comparable QoS. - Load balancing: Load is distributed over different channels/links/paths available to maximize throughput. These objectives can usually not be simultaneously optimized. In fact, the load distribution across the topology may be very unbalanced in order to achieve fairness. As discussed in Rel-15 SI, the important issue for IAB is to achieve fairness across the topology, i.e., UEs receiving same services at different access points in the topology have same or comparable QoS. For IAB, this implies granularity of UE-bearer. Further, we should first converge on the problem before we discuss potential solutions, e.g., additional information that needs to be exchanged, local vs. global decision making, etc. |
| Apple | Though the list looks comprehensive, we agree with Qualcomm and AT&Ts sentiments here. Fairness should be considered across multiple hops. Devices that are “x” hops away from the donor should not be penalized due to the architecture. This is even more concerning for UEs with BE flows since the presence of QoS on some UEs shouldn’t mean that the UEs. Also |
| vivo | We agree with the rapporteur’s view. |
| NEC | Agree with the rapporteur’s view |
| OPPO | We basically agree with the rapporteur’s view. We also share the comments from Huawei and Ericsson. We can list as: - What was done in R16 and what is not covered by Rel.16 specification. - what can be achieved via implementation. Then we can decide what can be done in R17. |
| Turkcell | Agree with the rapporteur’s view |

## Multi-hop latency

The MT part of an IAB node can currently request uplink resources for the UL data transmission after it actually receives the data to be transmitted from its child node, but also before it receives actual data, as it may already have knowledge of incoming data from the child node based on received BSR (Buffer Status Report). In a multi-hop network, the delays are likely to accumulate due to number of hops and aggregated volume of data at IAB nodes, so pre-emptive scheduling has the potential to reduce this significantly. This is why pre-emptive BSR was introduced in NR IAB Rel-16.

Further enhancements have been discussed in submissions to RAN2#111-e and the following is a list of high-level issues:

- What are the latency/QoS metrics we would like to optimize under this WID objective?

- Whether additional procedures are needed on top of Rel-16 baseline in order for this to happen

- If additional procedures are needed on top of Rel-16 baseline, what are they? Examples include (but are not limited to):

  - Whether to allow the Donor CU to choose a different serving cell (access node/parent node) for a UE or an IAB-MT (such as one which is fewer hops away from the Donor CU)

  - Whether and how to enhance scheduling decisions of parent IAB nodes; examples include:

    - allowing the reporting by a node to its parent node of status of backhaul links to its own child nodes

    - allowing the reporting by a node to its parent node of specific channels that will e.g. experience delay etc.

  - Whether to further enhance the pre-emptive BSR procedure

  - Whether and how to take into account the additional latency components in PDB management – things to take into account may include:

    - switching between transmit and receive modes of an IAB node

    - processing times along the chain, etc.

  - Whether to increase the number of LCGs per BH link

Question 2: Do you agree that the above is a comprehensive list of high-level issues impacting multi-hop latency? If you feel certain issues have been missed, please add them below (but bear in mind they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |
| Kyocera | We agree with the rapporteur’s view. |
| Intel | As mentioned in Q1, we suggest to consider item 5/e “data split” in sub-question 3 “additional procedure needed on top of Rel-16 baseline” as well. From our understanding, splitting data into multiple paths can also help to improve resource utilization as well as reduce multi-hop latency. |
| Huawei | The summary looks comprehensive. Maybe we can directly discuss part 3 to focus on the issues. For the PDB related discussion, e.g. in 3d, the per hop PDB related QoS can be configured on per BH RLC channel by CU in R16 IAB. This already considers the hop number related impacts on latency. So, we see no further need on the optimization on the PDB related management. For 3a, we need to clarify the difference with the HO/redirection for UE/MT to another cell/node by CU implementation. For 3b-i, we need to clarify the difference with flow control, which is a separate topic. For 3e, we proposed this to be discuss in sec 2.1 as fairness issue, since this is to support more accurate scheduling among LCHs of one UE/MT, to provide finer QoS provision for the fairness. |
| CATT | We are general fine with rapporteur’s view. For 3b, we think whether some additional information provided by donor CU to the IAB node is necessary to enhance IAB node scheduling decisions can also be discussed. Thus, we prefer to add 3biii, i.e., allowing the additional information provided by donor CU to the IAB node. |
| LG | Generally fine with following comments. We wonder whether the bullet 3.b, i.e., “Whether and how to enhance scheduling decisions of parent IAB nodes”, is out of scope for multi-hop latency or not because it seems to try to optimize one hop scheduling decision, not multi-hop latency. |
| ZTE | We generally agree with the list if the following issues can be added. The detailed descriptions for these two issues could be found in R2-2007312. 3f. Whether and how to allow the intermediate IAB node to discard the data packet if the PDB of the data packet could not be guaranteed during forwarding. 3g. Whether and how to enable the donor CU/IAB DU to configure/choose a routing path for time-critical packets from/to a UE based on the experienced delay of the path. |
| Nokia, Nokia Shanghai Bell | We are not sure what is the intention of example 3a., isn’t this up to Donor CU already in Rel-16 through topology adaptation decisions? Also, we are not sure enough justification would have been provided to enhance pre-emptive BSR procedure / number of LCGs in improving multi-hop latency. |
| Ericsson | We agree with the list of topics proposed by the Rapporteur for the Rel.17 scope. Bullet 3b seems overlapping with bullet 3 in Section 2.3. In general, RAN2 should first discuss what is not covered by Rel.16 specification and what can be achieved via implementation. Hence, we believe that before pursuing a certain enhancement, RAN2 should evaluate pros and cons of each enhancement, as well as the level of complexity. |
| Fujitsu | We generally agree with the rapporteur’s view, with a minor comment on item 3b-i: The sentence “status of backhaul links to its own child nodes” looks unclear. It may give more examples, such as number of UE/IAB-MTs served by child IAB-nodes and their subtending IAB-nodes, windowed throughput, congestion, etc. In addition, this status may apply to its access links as well. |
| AT&T | For 1, performance during mobility events for UEs or topology adaptation events for IAB-MTs (e.g. to reduce interruption time) should also be considered for this topic For 3b, aspects related to cell selection in IDLE mode as well could also be included |
| Futurewei | We have the following comment on the issues listed above: 3. Fairness definition and metrics: a) We are not sure how the Donor CU choosing a different serving cell for a UE or an IAB-MT is an additional procedure on top of Rel-16. Our understanding is that Rel-16 does not limit the Donor CU triggering a HO of a UE or IAB-MT to another node and/or cell. |
| Sony | We generally agree with the rapporteur’s view and in favor of RAN2 evaluating the aspects which can be left to implementation and the others requiring standardization. |
| Qualcomm | The above bullets emphasize on “what features would we like to add” rather than “what latency-related shortcomings does Rel-16 IAB have”. We see the following latency-related shortcomings of Rel-16 IAB: - The CU cannot determine how to set the latency budget for each hop since it doesn’t have the wireline latency value, for instance. - The IAB-node does not know when to discard a packet based on the total delay accumulated over multiple hops. - There is unnecessary buffering delay due to misalignment of BH throughput to the dynamic throughput variations of access links. - Topology-wide optimization of routes and resources based on latency criteria is not possible since buffer load is not reported to CU-CP. |
| Apple | We agree with rapporteur’s views. We also think additions need to be made at RLC/PDCP especially in 1:N mapping scenarios esp. when QoS is involved. |
| vivo | We agree with the rapporteur’s view. |
| NEC | Agree with the rapporteur’s view |
| OPPO | We basically agree with the rapporteur’s view and also agree Ericsson’s suggestion. |
| Turkcell | We agree with the rapporteur’s view. |

## Congestion mitigation

Flow control is needed in IAB networks to prevent congestion occurring. There are two main types of flow control in relay networks: end-to-end and hop-by-hop. On the uplink (UL), resource allocation serves as a form of flow control (the parent node has full control over UL transmissions of its child nodes). Therefore, Rel-16 IAB work on congestion mitigation has focused on the DL.

In NR Rel-16 IAB, HbH flow control feedback is limited to single-hop, and includes available or desired buffer size (in absolute terms, rather than relative terms e.g. percentage). Additionally, the flow control feedback can only be reported for a subset of bearers with the same routing ID (basically bearers heading to the same final destination), or for the entire channel (total buffer status of a channel of the link). Moreover, reporting based on polling and threshold-based reporting are both introduced.

Further enhancements have been discussed in submissions to RAN2#111-e and the following is a list of high-level issues:

- Whether to introduce HbH flow control for upstream traffic as well

  - If HbH flow control is introduced for upstream traffic, which layer (or layers) should be responsible for it

- Whether any enhancements are needed to E2E flow control (and how much of this is within RAN2 remit)

- Whether to enhance existing DL HbH flow – examples include:

  - Enhancements to control feedback content (e.g. information on links to child nodes, information on buffer status of child nodes, time validity of this information etc.)

  - Enhancements to triggering conditions for self-reporting and/or triggering conditions for polling

  - Introducing additional reporting granularity options

- Whether the additional information (if any) introduces unacceptable levels of overhead

Question 3: Do you agree that the above is a comprehensive list of high-level issues impacting congestion mitigation? If you feel certain issues have been missed, please add them below (but bear in mind that they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |
| Kyocera | We agree with the rapporteur’s view. |
| Intel | We generally agree the above list should be discussed with following change proposals from our side. 1) We think “enhancement to triggering frequency for self-reporting and/or triggering conditions for polling” can also be discussed under sub-question 3. 2) Enhancement to E2E flow control should be low priority for RAN2, as DL E2E flow control is now being actively discussed in RAN3, and RAN3 has deprioritized UL E2E flow control in Rel-17 during RAN3-109e. |
| Huawei | The summary looks comprehensive. For UL flow control in bullet 1, the R16 implementation manner to throttle the UL grant scheduling is per LCG per UE level. We may want to have finer granularity as per LCH level flow control, similar to the DL. The E2E flow control in bullet 2 should be purely R3 issue. |
| CATT | We generally agree the above views. Besides that, we think the following proposals should also be considered. 1) Re-routing can reduce the congestion of IAB-node by change the congested path. Thus, we think “whether any enhancements for local re-routing are needed due to the congestion” can also be discussed. 2) “Whether any enhancements are needed to E2E flow control” is in RAN3 scope. Thus it should be discussed base on the progress of RAN3’s discussion. |
| LG | Agree with the rapporteur’s view except the bullet 1. In Rel-16, when a child IAB node suffers DL data congestion, if the parent IAB node keep transmitting DL traffic toward the congested child IAB node, unnecessary packet drop/overflow is inevitable in the congested child IAB node and the only way to avoid this packet drop problem is to make the parent IAB node throttle down the DL traffic toward this congestion child IAB node. For this, DL HbH flow control is introduced in Rel-16. As RAN2 already identified that the IAB node has full control of UL scheduling and uplink data congestion can be resolved by itself. Thus, we still think that packet drop due to uplink data congestion may not happen because the IAB node does not allocate too much UL grants, which may cause uplink buffer overflow, to child IAB nodes or UEs. Thus, the bullet 1 for UL HbH flow control may not be needed. |
| ZTE | We generally agree to discuss the above issues, and we agree with Intel that the UL E2E flow control enhancements need to be deprioritized. |
| Nokia, Nokia Shanghai Bell | In this area we think issues = shortcomings justifying enhancements to DL HbH flow control have not been pointed out. E2E flow control falls under RAN3. The yet non-existing HbH flow control for upstream traffic could bring benefits in aiding on scheduling the UL traffic by the child nodes down to the scheduling of the data sources. |
| Ericsson | We agree with the list of topics proposed by the Rapporteur for the Rel.17 scope. In general, RAN2 should first discuss what is not covered by Rel.16 specification and what can be achieved via implementation. For example, as LG mentioned the objective of bullet 1 can be already achieved via Rel.16 implementation. Hence, we believe that before pursuing a certain enhancement, RAN2 should evaluate pros and cons of each enhancement, as well as the level of complexity. |
| Fujitsu | We agree with the rapporteur’s view. |
| Futurewei | Generally, we agree with the rapporteur’s list of issues In addition, similar to CATT’s comment, local routing decisions (under control of CU configuration) could be an effective mechanism to address congestion. A secondary benefit of enabling local routing decisions would be to reduce multi-hop latency in the case of link congestion, so this may also be related to 2.2 |
| Sony | We agree with the rapporteur’s view. |
| Qualcomm | The above bullets emphasize on “what features would we like to add” rather than “what problem does present Rel-16 congestion control have”. We see the following congestion-control-related shortcomings of Rel-16 IAB: - DL single-hop flow control on BAP layer cannot reach up to the CU-UP. Therefore, the congestion point can only be moved up to the donor-DU where packets will have to be ultimately dropped. Consequently, single-hop flow control can at best mitigate short term load congestion. It is therefore questionable if additional features can provide any benefit. - DL E2E flow control via DDDS/NUPP relies on packets to be lost to trigger feedback, which adds latency due to retransmission. Obviously, it would be better to have E2E indication of congestion to the CU-UP to occur before packets have been dropped. - UL single-hop flow control should be adequate. It is not clear why any improvement is needed. |
| Apple | We agree in general with the rapporteur’s views. We prefer to also have E2E flow control on top of hop by hop to ensure efficient QoS |
| vivo | We agree with the rapporteur’s view in principle, but additionally the following aspects are also considered beneficial from our perspective. 1) Enhancements to control feedback content are necessary, except for the examples given in 3/a, we believe the format design of control feedback can be investigated further to improve the size efficiency. Besides, enhancements to avoid intensive feedbacks of DL HbH flow control scheme can also be added as 3/d, as intensive FC feedback does not help the congestion resolution but to increase the FC feedback overhead., 2) On top of the above 3 mechanisms for congestion mitigation, we’d also like to study the 4th mechanism termed as radio resource re-allocation. In our understanding, congestion risk is the product of a mismatch between ingress and egress data rate, which can be mitigated by the resource re-allocation mechanism without sacrificing spectrum efficiency. |
| NEC | We agree to discuss the above issues. For UL E2E flow control enhancements, this could be done by rerouting, so it can to be deprioritized. |
| OPPO | We basically agree with the rapporteur’s view and also agree Ericsson’s suggestion. |
| Turkcell | We agree with the rapporteur’s view and Ericsson comment |

## Others

[QC] We are adding this bullet to address other issues not considered above.

Question 4: Are there other topics that need to be addressed on the main objective of “Topology, routing and transport enhancements”.

| Company | Comments |
| --- | --- |
| Qualcomm | We see the following additional issues to be addressed: - The CU-CP has limited insight into load conditions to optimize topology, routing and resource allocation. |

# Phase-II: consolidating/confirming the scope

Based on the input received in Phase-I, the rapporteur would like to first make a couple of observations, before making proposals for a way forward:

- The input received was quite detailed, and it is clear a lot of thought went into the responses, with companies sharing their views not just on issues, but also on the structure of the discussion and the directions the solutions could take.

- While we will not be agreeing the specific solutions in this discussion, many companies have shared proposed solutions to certain perceived issues; the rapporteur advises everyone participating to carefully consider these – even though we do not need to agree on solutions, the responses received reveal companies’ viewpoints, and it will save us time in future if those intentions and views are recognized now and meeting submissions tailored to address certain recurring issues.

- Some companies felt that some of rapporteur’s Phase-1 observations were too solutions focused; while this was a minority view, it is nevertheless acknowledged and taken into account – the proposals that follow are now focused on issues and structuring the future discussion on solutions, rather than anticipating solutions (except where the rapporteur feels is appropriate, and even then only at a high level), so in a sense the proposals that follow are less solutions-heavy than the discussion (nevertheless very important) in Phase-1.

- From the input received in Phase-1, the majority seem to agree that for Topology-wide fairness it is yet unclear whether any enhancements are needed on top of Rel-16 IAB (and any implementation-specific solutions), whereas for Multi-hop latency and Congestion control, the majority view seems to be that Rel-16 baseline does not offer solutions to several of the issues companies would like to see fixed in Rel-17; this is taken into account when drafting proposals.

Bearing the above in mind, and the detailed comments received, the rapporteur would like to make the following proposals for RAN2 consideration:

Topology-wide fairness

- Topology-wide fairness is focused wholly on end-user experience (i.e. it is not based on e.g. fair use of processing resources of IAB nodes, equitable use of paths available, and so on).

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - The proposal is misleading. Fairness relates to the relative difference of QoS or performance experienced by multiple users receiving the same service. It may be possible that fairly treated users suffer from poor end-user experience (low QoS). It may further be possible that end-user experience of non-fairly treated users is generally good (e.g. they all receive high but very different TP). |
| - Kyocera | - We agree with Proposal 1. |
| - Ericsson | - We agree with the intention of the proposal, but we also agree with QC that the proposal might be a bit misleading. - We suggest the following rephrase: “Topology-wise fairness should focus on facilitating/providing the intended end-user experience for a service throughout the network” |
| - NEC | - We agree the end user experience is the first prioritized target, but we also think the IAB node resource usage should also be considered as much as possible. |
| - OPPO | - We agree with proposal 1. We are also ok with the wording improvement from Ericsson. |
| - Nokia, Nokia Shanghai Bell | - This seems a bit unclear formulation. E.g. why is “equitable use of paths available” not focused on end-user experience..? |
| - CATT | - We agree with Ericsson’s suggestion. |
| - Intel | - No, we think topology-wide fairness should also include topology-wide fairness for different paths to avoid congestion. Considering load balance between different paths, it may allow users with similar QoS to map into different paths (with similar QoS). In other words, load balancing can help to avoid congestion and hence lead end user fairness. |
| - Futurewei | - We agree with Proposal 1. We are also fine to further clarify the language of the agreement, if there is a consensus view. |
| - AT&T | - We are OK with Ericsson’s suggested wording |
| - ZTE | - Tend to agree with Ericsson’s comments. |
| - Huawei | - We need to clarify if “focused wholly on end-user experience” means the discussion is from UE experience perspective, rather than NW perspective. If the answer is yes, it seems the proposal is straight forward. |
| - Apple | - We agree with proposal 1. Ok with Ericsson’s word changes. |
| - LG | - We are ok with Ericsson’s modification. |
| - Turkcell | - We agree with proposal . And support Ericsson’s change. |
| - Sony | - We agree with Ericsson. |

- Topology-wide fairness is achieved when UEs receiving the same service have the same (within a margin) QoS, regardless of the number of hops a given UE is away from the Donor DU.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - The definition is better. However, we propose a rewording to “topology-wide equal fairness is achieved,,,, regardless where the UE connects to the topology “. - For access-only systems (i.e. no backhaul), the term “proportional fairness” does not imply “equal fairness”. It is therefore perceivable that multiple topology-wide fairness schemes are considered which may achieve different distributions of QoS over the Ues in the topology that consume the same service. - Further, this is not only about hop count but also about aggregation of traffic on some BH links. - We may rather define the term of topology fairness as: “Topology-wide fairness mechanism aim to control the distribution of service-specific QoS experienced by the Ues across the topology”. |
| - Kyocera | - We agree with Proposal 2. |
| - Ericsson | - We prefer the definition provided by QC at the end of their reply. - Different fairness schemes might be foreseen locally at an IAB node, which may not necessarily provide the same perceived QoS at the Ues for a given service. Additionally, the perceived QoS will not only depend on whether a UE has been treated fairly, but also on a number of other factors such as the local radio conditions at the various hops, the amount of served BH RLC channels, the radio resource availability, etc. RAN2 should just aim at providing tools to ensure fairness locally at the IAB for the various served traffic. |
| - NEC | - We agree with Proposal 2. |
| - OPPO | - We agree with Proposal 2. But term “margin” is not clear. |
| - Nokia, Nokia Shanghai Bell | - Seems agreeable. |
| - CATT | - We agree with Proposal 2. UE with different hops should have same QoS for same service. We also think the “margin” in the proposal is not so clear. |
| - Intel | - Agree |
| - Futurewei | - We agree in principle with the proposal, but it is a bit tricky to define what “the same QoS” means exactly. As already mentioned by QCM and E/// above, with a proportional fair scheduler, different users will get different average throughput dependent on their respective channels. This is still “proportionally fair”. In other words “fairness” should not be understood to imply Equal-throughput scheduling. - I think want we want is for users with the same QoS class to have similar performance statistics across the network, regardless of which IAB node is serving the UE. - If this definition is a bit “fuzzy”, that is because it is intended to be. What are the appropriate objectives of scheduling and resource allocation are very subjective, and so I don’t think we should spend too much time trying to nail-down a strict definition of “fairness” that all have to agree on, before we can move forward. |
| - AT&T | - We are not sure how useful Proposal 2 is without clarity on the QoS “margin,” so we don’t see a need to agree on it. If companies really prefer to have an agreement like Proposal 2, perhaps the wording from QC is more clear, but at the same time we want to emphasize that Topology-wide fairness cannot exist in a vacuum but may also coexist with other equally important objectives like system stability and resource efficiency. |
| - ZTE | - We agree with proposal 2. |
| - Huawei | “when UEs…have the same (within a margin) QoS” I guess it means “similar QoS” because we cannot guarantee the 100% same in realistic network. We also need to clarify whether this is for QoS requirement or for QoS fairness. First, the UE’s E2E QoS requirement should be ensured anyway. So, if UEs receiving the same service all can met their QoS requirement, the “margin” seems for the optimization of balancing the different margin among UEs. In that case, it is extremely difficult to evaluate if we have done good enough for the margin balancing. Whether the following is the intention? - “Topology-wide fairness is achieved when UEs receiving the same service have the balanced/similar QoS, regardless of the number of hops a given UE is away from the Donor DU. “ |
| - Apple | Agree with proposal 1 since our view has always been that a network coverage extension should not limit user experience in any way. |
| - LG | It seems agreeable, but ‘within a margin’ may not be needed in the proposal 2. |
| - Turkcell | We prefer the definition provided by Qualcomm |
| - Sony | Agree with rapporteur’s proposal. |

- RAN2 to discuss further the metrics to be used quantify the topology-wide fairness. Open issues include (but are not limited to) the "margin" from previous proposal, and the granularity of fairness (e.g. overall/average QoS across all bearers of a UE, or per UE bearer).

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - We need to first converge on the metrics. The margin, i.e., how wide this metric is spread over all UEs, may be up to operation and implementation. Example: the proportional fair scheme, for instance, aims to maximize the geometric mean TP over all UEs, i.e., it does not aim to minimize the margin or set any bound to the margin. - The concept of fairness was discussed in the context of BE services when broadband moved to wireless. For these services, the main metric is per user throughput. Latency was initially not considered very critical, and even if it was, it could be assumed comparable for all UEs receiving same service at the same cell. For multi-hop, latency may have to be considered as an additional metric. Alternatively, latency could be captured by a “guaranteed max bound”. - For RT services using GBR, the guarantee on the bitrate already provides a minimum throughput fairness over all UEs. It is somewhat assumed that higher TP does not (significantly) improve QoS (e.g. as for voice services). Latency criteria may need to be considered for topology-wide fairness as discussed for BE services. |
| - Kyocera | - We agree with Proposal 3. |
| - Ericsson | - We are not ok with this proposal. - It is not clear what is the purpose of such “metrics”. The 5G QoS framework already provides all the necessary metrics to enforce the QoS over the RAN, and to properly handle the different services. If the intention is to introduce some KPIs on QoS fulfillment, we do not believe that this aspect should be specified. - RAN2 should instead focus on the tools that should be specified in order to allow the IAB network to fulfill the legacy QoS requirements. |
| - NEC | - We also think the term “metrics” should be clarified. |
| - OPPO | - We also think more clarifications are needed on term “margin” and “metrics”. |
| - Nokia, Nokia Shanghai Bell | - OK to discuss. |
| - CATT | - We don’t agree with proposal 3. - We think 5G QoS can be used quantify the topology-wide fairness. We don’t need to discuss any other metrics. |
| - Intel | - Agree |
| - Futurewei | - I’m afraid we may unnecessarily spend a lot of time trying to define what “margin” means, and then on what an acceptable “margin” is. - As far as granularity is concerned, average QoS across different bearers of a given UE seems to be meaningless. QoS is always defined on a per bearer basis. - What metric would we average across the UE’s bearers? Mean packet delay achieved for each bearer? |
| - AT&T | - Agree with Ericsson, we don’t see the need to quantify exact metrics or introduce new metrics. |
| - ZTE | - Tend to agree with this proposal. However, it is suggested to give some concrete examples of “metric” which may be used for evaluate the topology-wide fairness. |
| - Huawei | - In our understanding, if all UE’s E2E QoS requirements can be met, it is the best efforts by implementation rather than some requirement, as to the “fairness/balancing/margin”. So, why do we need to specify the metric for a best effort thing? |
| - Apple | - Agree |
| - LG | - We also have same concerns on “margin” and “metric” |
| - Turkcell | - Agree with Ericsson and AT&T |
| - Sony | - We also share the same concern on what’s the meaning of metric. |

- RAN2 to discuss whether topology-wide fairness can be met using Rel-16 IAB baseline and/or through implementation.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - No, under no circumstances. Rel-16 IAB baseline cannot achieve topology-wide fairness. This is a fact and we should not waste our time discussing facts. - Parties interested in this topic can read the TR, which establishes the problem, or R2-2006965 to last meeting which explains the issue and shows detailed simulation results. |
| - Kyocera | - We agree with Proposal 4. |
| - Ericsson | - OK. - We are fine to discuss what can be already achieved by implementation in order to have better focus on what to introduce in Rel.17. |
| - NEC | - We agree with Proposal 4. |
| - OPPO | - We agree with Proposal 4. |
| - Nokia, Nokia Shanghai Bell | - OK to discuss. - From our point of view, it cannot be met at least with N:1 mapping, and as pointed out in contributions, the standardized mechanisms do not inform an IAB node whether 1:1 or N:1 mapping applies. |
| - CATT | - We aren’t fine with Proposal 4 - Base on previous proposals, we think the enhancements of topology-wide fairness should be further discussed in R17 |
| - Intel | - We agree to identify which enhancement should be left for implementation and to be specified, and discuss those specified issues in RAN2 |
| - Futurewei | - Agree with the proposal. - We certainly need to first understand whether there is a problem, before we discuss potential solutions to a problem |
| - AT&T | - Tend to agree with QC in that we prefer not to repeat discussions from the SI or Rel-16 WI if possible. At the same time, of course enhancements should provide some benefits/functionality beyond what is possible in Rel-16. |
| - ZTE | - We agree with proposal 4. In addition, we think Rel-16 IAB can be used as baseline to evaluate whether the topology-wide fairness is improved in Rel-17 IAB. |
| - Huawei | - Agree. We prefer to discuss this first to see if there is any requirement cannot be met by R16. - We’d like to clarify the issue captured in the TR is the one in SI phase. Please note we already completed the R16 WI phase, where, we believe, the issue was already solved by the per hop QoS configuration. As to the R2-2006965 mentioned by Qualcomm, the rapporteur’s proposal covers the discussion on the issue (e.g. in sec 2.1 of R2-2006965). |
| - Apple | - We also agree with the proposal and Rel-16 IAB should be used to enhance Rel-17 topology wide fairnesss |
| - LG | - We are ok with the proposal. |
| - Turkcell | - Agree |
| - Sony | - Agree |

- If additional information exchange and additional procedures are needed on top of Rel-16 baseline, RAN2 to discuss what they are, using the following as starting point:

Whether additional measurement reports are needed

Whether additional scheduling/benefit metrics are needed

Which layer (or layers) is used for exchange of additional information (if any)

What is the flow direction of the additional information exchange

Whether the additional information (if any) introduces acceptable levels of overhead

Who enforces farness (CU, intermediate IAB nodes, …)

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - None of these topics should be discussed before we have discussed what additional information the scheduler needs to apply topology-wide fairness criteria. - For TP-related fairness (e.g. BE services), the scheduler needs to know the number of DRBs within an RLC channel. - For latency-related fairness, the scheduler needs to know the latency budget available for each packet. This depends on each packet’s hop count number. It could be provided by bundling all traffic onto a common RLC channel that have same number of hops, and by assigning this latency to the RLC channel QoS. - Again, it is primarily about the information needed by the scheduler, and secondarily where it comes from. |
| - Kyocera | - We agree with Proposal 5. |
| - Ericsson | - This proposal seems to already go in the solution direction. In our view, RAN2 should first start with the Rel16 baseline, then look into what cannot be achieved via Rel.16, and finally dig into the necessary procedures/signaling to enhance the Rel.16. |
| - NEC | - We agree with Proposal 5. |
| - OPPO | - We share the same view with Ericsson. |
| - Nokia, Nokia Shanghai Bell | - OK to discuss, we favor making an IAB node aware of the number of DRBs mapped on each BH RLC channel. |
| - CATT | - We don’t need to discuss these topics in this e-mail discussion - All of these topics are the solutions for topology-wide fairness. We should focus on the discussion of the issues of topology-wide fairness in this e-mail discussion. |
| - Intel | - Agree |
| - Futurewei | - We have sympathy for E///’s point of view. This proposal may be pointing to solutions before we have even agreed that there is an issue to address. However, the rapporteur did provide the preface: - If additional information exchange and additional procedures are needed on top of Rel-16 baseline, - So, we are generally fine with the proposal. It seems that the issues cited are quite reasonable to take as a starting point, but only after we determine if there is a problem to be addressed. - We don’t agree with QCM that the need for enhancements is a foregone conclusion at this stage. |
| - AT&T | - Ok to take these as a starting point, but details should be further discussed. |
| - ZTE | - We also agree with Ericsson’s view. |
| - Huawei | - Share the similar view as Ericsson. We should first clarify the issue and then work on the solution, rather than just go through all the proposed solutions without common understanding on the issues not covered by R16. |
| - Apple | - Agree with proposal 5. |
| - LG | - We also share Ericsson’s view. |
| - Turkcell | - We also agree with Ericsson’s view. |
| - Sony | - Agree |

Multi-hop latency

- RAN2 agrees that certain issues to do with multi-hop latency cannot be met through implementation and/or using Rel-16 IAB baseline. Examples include (but are not limited to):

Inability to discard packets at intermediate nodes once they have "expired"

Inability to discard packets at intermediate nodes if PDB cannot be met

Inability by the CU to determine how to set the latency budget for each hop

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - Agree on proposal 6. All examples should be considered. |
| - Kyocera | - We’re fine with Proposal 6. - However, we don’t have strong view if the inability of packet discarding is a critical issue. |
| - Ericsson | - We do not agree with the formulation of this proposal. - The CU knows the topology of the IAB network and it can already today configure the PDB hop-by-hop at each IAB node accordingly, e.g. taking into account the IAB topology. Similarly, it is not clear from this proposal why the IAB node is not able discard a packet on the basis of the configured PDB. Since the scheduling of a given packet is fully in control of the IAB node, that should be possible. - We suggest first to clarify what are the issues with the Rel.16 baseline and agree on that. Then RAN2 can address the Rel.17 enhancements. |
| - NEC | - We agree with the observations in proposal 6. |
| - OPPO | - We agree with the observations in proposal 6. |
| - Nokia, Nokia Shanghai Bell | - What is the significance of agreeing on the above-listed examples? That as a minimum they will be addressed in Rel.17? Some of the issues may not be able to be met by Rel-16 IAB baseline but it is unclear whether they need to be addressed. |
| - CATT | - We agree with Proposal 6 |
| - Intel | - Agree |
| - Futurewei | - Just to confirm our common understanding, by an “expired” packet, we assume this means a packet that has exceeded the PDB for the corresponding UE DRB. Is this correct? - Note that there is no requirement that a packet that exceed its PDB be discarded. TS 23.501 states the following in section 5.7.3.4: - The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, for a Delay critical GBR resource type, packets delayed more than the PDB are added to the PER and can be discarded or delivered depending on local decision. - Also, even without IAB, our understanding is that the 3GPP spec do not currently provide a standardized method to assess the PDB for a particular packet. For example, as QCM pointed out in their Phase I comment, the “CU doesn’t have the wireline latency”. However, this is also true in the purely wireline backhaul case. Therefore, our assumption is that the network may estimate the backhaul delay between CU and DU by implementation and utilize this estimate for appropriate decisions at the DU scheduler. - It is not clear why such an implementation-specific approach could not also be used in the case of IAB to assess the latency of a packet between donor CU and IAB-DU (including for example for packet discard, if so desired). - That said, we do agree that there are limitations to what can be currently supported by the spec. Therefore, think that this topic may be worth further discussion and exploration. |
| - AT&T | - Agree with the proposal |
| - ZTE | - We agree with proposal 6. |
| - Huawei | - Not sure. - For “discard packets”, we do not think it is related to the latency enhancement. This is more like the radio resource efficiency, right? - For the 3rd bullet, it seems a R3 issue on some feedback from IAB-DU to donor-CU on per hop status. We are not sure if it is already supported, which should be discussed in R3 instead. - Also agree with Nokia that it seems those example bullets are not even needed to be met. |
| - Apple | - Agree with proposal 6. |
| - LG | - We share views from Nokia and Huawei. |
| - Turkcell | - Agree |
| - Sony | - Agree |

- RAN2 will discuss enhancements using following as a starting point:

Whether and how to enhance scheduling decisions of parent IAB nodes

Whether and how to further enhance the pre-emptive BSR procedure

Whether and how to ensure PDB management, including enabling packet discarding by intermediate nodes

Whether to increase the number of LCGs per BH link

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - On scheduling decisions of parent IAB-nodes: It is not clear why we have the prefix “parent” here. This sub-bullet is further too broad, i.e., it is not clear what it refers to (fairness? Latency? Load balancing? Etc.) - On enhance the pre-emptive BSR procedure: Pre-emptive BSR was thoroughly handled in Rel-16 and we don’t believe that any further improvement can be made without proper simulation work. - On PDB management: This topic needs more discussion. Presently, proper PDB management is not possible, e.g., because the IAB-donor does not know what the latency is on the wireline section of the RAN. - On LCGs per BH link: We don’t believe any increase in LCG space provides significant benefits. |
| - Kyocera | - We agree with Proposal 7. |
| - Ericsson | - On scheduling decisions of parent IAB-nodes: we agree with QC. It is not clear what should be enhanced, but we assume that this should be part of the discussion? - On enhance the pre-emptive BSR procedure: As pointed out by QC, pre-emptive BSR was discussed extensively in Rel-16, hence, RAN2 should probably prioritize other topics. - On PDB management: We are ok to discuss it, but as pointed out in the reply to the previous question we should first focus on the issue with the Rel.16 baseline. - On LCGs per BH link: OK, that can be considered when discussing UL enhancements. |
| - NEC | - We agree with Proposal 7. |
| - OPPO | - We agree with proposal 7. |
| - Nokia | - OK to discuss. |
| - CATT | - We generally agree with P7. - Scheduling decisions of IAB-nodes needs to be discussed. We prefer to delete the “parent”. We think generally the scheduling of IAB node can be discussed. - Pre-emptive BSR is well done in R16, We don’t think there should be any enhancement in R17. - PDB management is needed to discuss in R17. We think it has benefits in scheduling. - Increasing the number of LCG per BH link can refine the scheduling. |
| - Intel | - Agree |
| - Futurewei | - Similar to QCM and CATT, we are not sure why we need to include the prefix “parent” when discussing enhancements for support of scheduling decisions. Also, as E///, we believe that at this stage we have not yet determined that any enhancements for support of scheduling are really needed. However, this is within the scope of our discussions. - Generally, we think the discussion should first focus on “Whether” part for these topics. Only moving on to the “How” part iff there is consensus on the necessity of certain enhancements to address specific issues. |
| - AT&T | - Agree with the proposal |
| - ZTE | - Basically agree with proposal 7. But we think the PDB management should be prioritized. |
| - Huawei | - Agree with bullet 2/4 |
| - Apple | - Agree with proposal 7. |
| - LG | - Agree with bullet 2 and 4. |
| - Turkcell | - Agree |
| - Sony | - Agree |

Congestion mitigation

- RAN2 agrees that certain issues to do with congestion control in IAB networks cannot be met through implementation and/or using Rel-16 IAB baseline. Examples include (but are not limited to):

Inability to do local routing

Inability to mitigate long-term congestion using Rel-16 HbH design

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - On inability to do local routing: We disagree. Rerouting may mitigate congestion (e.g. via load balancing) but this can be done via central routing decisions as well. Local routing decisions are not better suited since the local node needs to have extended visibility of congestion points along the route. This information transfer erases the benefit local decision making has over centralized solutions. - On inability to mitigate long-term congestion via HbH design: This is correct. |
| - Kyocera | - We agree with Proposal 8. |
| - Ericsson | - On local routing: We agree with QC’s comment about local routing. We should first discuss why the IAB node is not able to do local routing, and if RAN2 sees benefits what is needed to enable a proper local rerouting. - On HbH congestion: the purpose of HbH was to solve the temporary congestion on a local BH link, not long-term congestion that should be under the domain of the E2E flow control mechanism. So, we don’t understand the purpose of this proposal. |
| - NEC | - Local routing should be carefully done. In Rel_16 IAB, local routing can only be done in case of RLF, this is because RLF occurs very quickly. But the CU knows the congestion status, so CU can re-configure the routing. |
| - OPPO | - We think we should fist confirm the issues and decide to have the two enhancement. |
| - Nokia, Nokia Shanghai Bell | - Agree with sentence #1. If the purpose of the listed examples in sentence #2 is that as a minimum they will be addressed in Rel.17, we disagree with sentence #2. |
| - CATT | - Local routing: We support it. The local routing can be done due to the congestion control, which is similar as ‎ the local routing mechanism in case of RLF in R16‎. To involve local routing can quickly reduce the mitigation compared with donor CU configuration based on E2E flow control‎. - HbH congetion: It is correct. It can’t mitigate long-term congestion in R16. |
| - Intel | - Agree |
| - Futurewei | - These proposals seem rather vague to me. It seems technically incorrect to state that Rel. 16 IAB baseline can not do local routing, as local routing is already supported in Rel. 16 (albeit under specific circumstances). Perhaps it would be better to say something like: - Observation: Rel. 16 does not support features to enable and optimize local routing decisions - Proposal: RAN2 to discuss the need to enable and optimize local routing decisions for Rel. 17 - Regarding mitigating long-term congestion, we agree that Rel. 16 HbH flow control can not achieve this. However, we also agree with E/// that this was never the objective of HbH flow control. - On the other hand, if companies have proposals to enhance HbH flow control for the purpose of enhancing or improving the performance of the E2E flow control mechanism, then we are open to discuss such proposals. |
| - AT&T | - Agree with the proposal |
| - ZTE | - We think the long term congestion mitigation should be considered. For the local re-routing, we should be cautious of its usage since it may interfere with the global-optimized routing configuration by donor CU. |
| - Huawei | - For bullet 1, we prefer “Inability to do local re-routing in case of congestion” to limit the use case under the WID scope of “congestion mitigation” - For bullet 2, long-term congestion should be handled by E2E flow control, which is R3 issue. |
| - Apple | - Agree with proposal 8. |
| - LG | - Agree with bullet 1 in the proposal 8. |
| - Agree |  |
| - Sony | - Agree |

- In light of recent RAN3 agreements (that End-to-end UL flow control is deprioritized in Rel-17), RAN2 will deprioritize UL E2E flow control.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - Yes |
| - Kyocera | - We agree with Proposal 9. |
| - Ericsson | - Agree |
| - NEC | - Yes |
| - OPPO | - Agree |
| - Nokia, Nokia Shanghai Bel | - Seems outside RAN2 domain. |
| - CATT | - Agree |
| - Intel | - We are ok with the proposal. |
| - Futurewei | - Generally, E2E flow control seems to fall into the scope of RAN3. So, it’s not clear if there is anything for RAN2 to prioritize or deprioritize regarding this topic |
| - ZTE | - Agree |
| - Huawei | - Agree. But seems no need of the redundant agreement with R3. |
| - Apple | - Agree |
| - LG | - Agree |
| - Turkcell | - Agree |
| - Sony | - Agree |

- RAN2 to discuss whether UL HbH flow control is a useful addition to the commonly used scheduling procedures on the UL.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - We propose to rewording: - Proposal 10’: RAN2 to discuss what benefits UL HbH flow control can have over the commonly used scheduling procedures. |
| - Kyocera | - We’re fine with Proposal 10. - However, we think Rel-16 implementation still works well, as LG and Ericsson pointed out in Phase 1 discussion. |
| - Ericsson | - We agree with reworded proposal from QC. First, we should discuss the benefits. |
| - NEC | - QC’s rewording is better. |
| - OPPO | - Agree with QC’s revision. |
| - Nokia, Nokia Shanghai Bell | - OK. - Current UL scheduling procedures are insufficient in the case that one cell group of an IAB node in DC is congested while the other one is not. |
| - CATT | - We agree with QC that we should first discuss whether there is any benefit of UL HbH flow control |
| - Intel | - Agree. |
| - Futurewei | - Agree with the proposal. - “whether …is a useful addition to” or “what benefits …can have over” seem to be saying the same thing. |
| - ZTE | - We agree with the QC’s proposal. |
| - Huawei | - Agree. See no difference from QC and rapporteur’s wording. Can we just say “RAN2 to discuss whether to introduce UL HbH flow control in R17.”? |
| - Apple | - Ok with this proposal. |
| - LG | - Ok with QC’s modification, benefit should be discussed first. |
| - Turkcell | - Agree with Qualcomm changes |
| - Sony | - Agree with Qualcomm |

- RAN2 to discuss whether any aspects of DL E2E flow control enhancements are within its remit.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - This is mainly in RAN3 space. |
| - Kyocera | - We’re fine with Proposal 11. |
| - Ericsson | - E2E flow control is RAN3 domain. There might be RAN2 impact, but RAN3 should provide guidance first. So RAN2 it is suggested waiting for more inputs from RAN3 before discussing this topic. |
| - NEC | - We’re fine with Proposal 11. |
| - OPPO | - Agree |
| - Nokia, Nokia Shanghai Bell | - We have not seen such aspects identified so we’d prefer not to discuss this. |
| - CATT | - It’s the RAN3 issue. We should wait for the guideline from RAN3 if there is any work in RAN2. |
| - Intel | - No, we should leave it for RAN3 discussion first |
| - Futurewei | - Similar to proposal 9 above, the topic of E2E flow control seems to mostly be in the scope of RAN3. - However, as stated in our response to Proposal 8, we are open to discussing enhancements to HbH flow control for the purpose of enhancing or improving the performance of the E2E flow control mechanism. - Furthermore, if RAN3 agrees something on E2E flow control that requires action from RAN2, we would of course adderss this at the appropriate time. |
| - ZTE | - We agree with proposal 11. |
| - Huawei | - We need first clarify what’s the R2 impact for DL E2E flow control. |
| - Apple | - Agree that shi should first be a RAN3 issue. |
| - LG | - We think that DL E2E flow control enhancements should be discussed in RAN3 first. |
| - Turkcell | - It”s in the scope of RAN3 |
| - Sony | - Agree |

.

- RAN2 will focus on enhancements to Rel-16 DL HbH flow control using following as a starting point:

Enhancements to control feedback content

Enhancements to triggering conditions for self-reporting and/or triggering conditions for polling

Introducing additional reporting granularity options

Enhancements to minimize flow control feedback overhead

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - We should not add more functionality since it is not certain if HbH flow control can do much since it never reaches the CU-UP. The last point, enhancements to minimize flow control feedback overhead may make sense. |
| - Kyocera | - We’re fine with Proposal 12. - However, we think RAN2 should first identify the problematic scenario in Rel-16. |
| - Ericsson | - We share the same view with QC and Kyocera. We are open to discuss possible enhancements, but first we should agree on what are the Rel.16 issues on the HbH flow control. For example, some of the proposed enhancements above can be already achieved by implementation. |
| - NEC | - We’re fine with Proposal 12. We should discuss whether the Rel_16 implementation solution is not satisfied. |
| - OPPO | - We share the same view with Ericsson. |
| - Nokia, Nokia Shanghai Bell | - We don’t really see a need to enhance DL HbH flow control – why this proposal is not equally to previous proposals, ie., “To discuss” whether to support? |
| - CATT | - We agree with proposal 12. DL HbH flow control need some enhancements in R17. For example, DL HbH flow control of R16 may cause parent IAB node congestion since parent IAB node may reduce the transmission. Thus, we think it should be discussed whether the IAB node needs to forward its child node’s flow control information to its parent node to further relieve the child node’s congestion at the ancestor nodes. Moreover, besides the current triggering conditions, whether it’s necessary to introduce more triggering conditions to report flow control information can be further discussed. |
| - Intel | - Agree. |
| - Futurewei | - Agree with other QCM, Kyocera, E///, and Nokia. First identify what enhancements are useful, if any, and then discuss how to achieve these. |
| - ZTE | - We think the HbH flow control mechanism in Rel-16 works well. Unless critical issues are identified, it is not necessary to further optimize it. |
| - Huawei | - We are open on this, since regardless whether those are necessary, R2 needs to discuss those anyway. So, maybe we can say “RAN2 discuss the need on enhancements…” |
| - Apple | - We should look at if Rel-16 is not sufficient here. |
| - LG | - We are not sure whether this enhancement is needed for now, but ok to discuss if there are critical problems. |
| - Turkcell | - We agree with Qualcomm |
| - Sony | - Agree |

Miscellaneous

- In light of recent RAN3 agreements (to deprioritize Multi-Route Support with data split in IAB), RAN2 will deprioritize splitting data of a radio bearer into two or more paths.

| - Company | - Comments |
| --- | --- |
| - Qualcomm | - Yes. There is a good reason for RAN3’s decision. If the F1-U is spread over multiple paths with different delay significant reordering issues may arise which affect IPsec and PDCP on top. Note that this delay spread would add to that created by ARQ on BH RLC CH. |
| - Kyocera | - We agree with Proposal 13. |
| - Ericsson | - Agree. |
| - NEC | - We agree with Proposal 13. |
| - OPPO | - Agree |
| - Nokia, Nokia Shanghai Bell | - OK. At the same time, we wonder if there is a real difference between the local routing in Proposal 8 and Multi-Route Support of this proposal, i.e. why the former should be prioritized while deprioritizing the latter. |
| - CATT | - Agree |
| - Intel | - We disagree to deprioritize data splitting in RAN2. - Data split can help to improve resource utilization and enable higher data rate as well as lower latency, though RAN3 has discussed this issue and didn’t see benefit for congestion control. Besides, considering IAB nodes at branching also have visibility to channel condition, which can assist to guarantee UE’s QoS and fairness. Moreover, when splitting data, IAB nodes can adopt certain ratio on different split paths to reduce the impact caused by spread delay. |
| - Futurewei | - Similar view as Nokia. We wonder if Multi-Route Support is not just a special case of local routing decisions? Are these fundamentally different in some way? Therefore, our view is that we don’t need to discuss this as a separate topic. We can just include it within the local routing decisions discussion. |
| - AT&T | - Would be Ok to discuss this jointly with Proposal 8 as they seem to be related as noted by Nokia |
| - ZTE | - We agree with proposal 13. |
| - Huawei | - Similar view as above comments. We’d better just skip this proposal and discuss the local rerouting with the R3 agreement in our mind. |
| - Apple | - Agree |
| - LG | - Agree |
| - Turkcell | - Agree |
| - Sony | - Similar view as Nokia and Futurewei. |

# Conclusions

In the present tdoc, …