3GPP TSG-RAN WG2 #112-e	R2-20xxxxx

E-meeting, November 2020

Agenda item:	8.4.1

Source: 	Samsung

Title: 	Report from email discussion [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation (Samsung)

Document for:	Discussion and decision

# Introduction

This submission contains the report from the following post-RAN2#111-e email discussion:

- [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation (Samsung)

Scope: Aim to clarify the scope. Determine which technical issues to address as a part of this WI objective. For better understanding and illustration of the issues only also solutions can be discussed, but solution decisions will NOT be taken based on this email discussion.

Intended Outcome: Report

Deadline: long

As a reminder, the following has been agreed as one of the objectives of the Rel-17 IAB WI:

Topology, routing and transport enhancements [RAN2-led, RAN3]:

- Specifications of enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation

Starting from guidelines provided by the Chair in the email discussion description above, and the submissions on the relevant issues to RAN2#111-e (none of them were treated as there were no TUs allocated to Rel-17 IAB WI at RAN2#111-e, but are a very useful and valid starting point nonetheless) this document aims to clarify the scope by identifying specific technical issues to be addressed.

# Phase-1: clarifying the scope

## Topology-wide fairness

This is perhaps the aspect of the Rel-17 IAB WI where we have least Rel-16 work to fall back on – although the issue was addressed briefly during the SI phase and some aspects are captured in the IAB TR. The following is a list of issues identified by the rapporteur of the discussion, based on submissions to RAN2#111-e:

- Fairness definition and metrics:

  - Focus of fairness (e.g. from a single-node point of view; fairness across different nodes; across different paths – i.e. load balancing across different paths and nodes to achieve resource utilization fairness, etc.)

  - Granularity of fairness (e.g. UE bearer level, BH RLC channel level, BH link level, fairness between access and backhaul links, etc.)

  - Metrics used to evaluate fairness (e.g. resource usage, fair and efficient use of scheduling opportunities, meeting QoS requirements in a uniform manner, etc.)

- Whether there are any differences in approaches to fairness for 1:1 and N:1 mapping cases, and whether the 1:1 case at all requires any fairness-related enhancements

- Who enforces fairness (Donor CU, parent IAB node(s), …) and whether we should support local decision-making for cases other than RLF

- Whether additional information exchange is needed on top of Rel-16 baseline to ensure topology-wide fairness

- If additional information exchange and additional procedures are needed on top of Rel-16 baseline, what are they? Examples include (but are not limited to):

  - Whether additional measurement reports are needed

  - Which layer (or layers) is used for exchange of additional information (if any)

  - What is the flow direction of the additional information exchange

  - Whether the additional information (if any) introduces unacceptable levels of overhead

  - Whether to allow splitting data of a radio bearer into two or more paths

Question 1: Do you agree that the above is a comprehensive list of high-level issues impacting topology-wide fairness? If you feel certain issues have been missed, please add them below (but bear in mind they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |

## Multi-hop latency

The MT part of an IAB node can currently request uplink resources for the UL data transmission after it actually receives the data to be transmitted from its child node, but also before it receives actual data, as it may already have knowledge of incoming data from the child node based on received BSR (Buffer Status Report). In a multi-hop network, the delays are likely to accumulate due to number of hops and aggregated volume of data at IAB nodes, so pre-emptive scheduling has the potential to reduce this significantly. This is why pre-emptive BSR was introduced in NR IAB Rel-16.

Further enhancements have been discussed in submissions to RAN2#111-e and the following is a list of high-level issues:

- What are the latency/QoS metrics we would like to optimize under this WID objective?

- Whether additional procedures are needed on top of Rel-16 baseline in order for this to happen

- If additional procedures are needed on top of Rel-16 baseline, what are they? Examples include (but are not limited to):

  - Whether to allow the Donor CU to choose a different serving cell (access node/parent node) for a UE or an IAB-MT (such as one which is fewer hops away from the Donor CU)

  - Whether and how to enhance scheduling decisions of parent IAB nodes; examples include:

    - allowing the reporting by a node to its parent node of status of backhaul links to its own child nodes

    - allowing the reporting by a node to its parent node of specific channels that will e.g. experience delay etc.

  - Whether to further enhance the pre-emptive BSR procedure

  - Whether and how to take into account the additional latency components in PDB management – things to take into account may include:

    - switching between transmit and receive modes of an IAB node

    - processing times along the chain, etc.

  - Whether to increase the number of LCGs per BH link

Question 2: Do you agree that the above is a comprehensive list of high-level issues impacting multi-hop latency? If you feel certain issues have been missed, please add them below (but bear in mind they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |

## Congestion mitigation

Flow control is needed in IAB networks to prevent congestion occurring. There are two main types of flow control in relay networks: end-to-end and hop-by-hop. On the uplink (UL), resource allocation serves as a form of flow control (the parent node has full control over UL transmissions of its child nodes). Therefore, Rel-16 IAB work on congestion mitigation has focused on the DL.

In NR Rel-16 IAB, HbH flow control feedback is limited to single-hop, and includes available or desired buffer size (in absolute terms, rather than relative terms e.g. percentage). Additionally, the flow control feedback can only be reported for a subset of bearers with the same routing ID (basically bearers heading to the same final destination), or for the entire channel (total buffer status of a channel of the link). Moreover, reporting based on polling and threshold-based reporting are both introduced.

Further enhancements have been discussed in submissions to RAN2#111-e and the following is a list of high-level issues:

- Whether to introduce HbH flow control for upstream traffic as well

  - If HbH flow control is introduced for upstream traffic, which layer (or layers) should be responsible for it

- Whether any enhancements are needed to E2E flow control (and how much of this is within RAN2 remit)

- Whether to enhance existing DL HbH flow – examples include:

  - Enhancements to control feedback content (e.g. information on links to child nodes, information on buffer status of child nodes, time validity of this information etc.)

  - Enhancements to triggering conditions for self-reporting and/or triggering conditions for polling

  - Introducing additional reporting granularity options

- Whether the additional information (if any) introduces unacceptable levels of overhead

Question 3: Do you agree that the above is a comprehensive list of high-level issues impacting congestion mitigation? If you feel certain issues have been missed, please add them below (but bear in mind that they could be sub-issues of the above issues). If you feel some of the above should not be studied, please say so, and explain why. Please focus on identifying issues at this stage and not proposing solutions.

| Company | Comments |
| --- | --- |

# Phase-II: consolidating/confirming the scope and introducing example solutions

…

# Conclusions

In the present tdoc, …