3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Electronic, 02nd – 13th November 2020

Agenda item:	x.x.x

Source: 	Huawei

Title: 	Email discussion [Post111-e][904][MBS] L2 Architecture

Document for:	Discussion and Decision

# Introduction

This paper is aimed at discussing the following topic.

- [Post111-e][904][MBS] L2 Architecture (Huawei)

Scope: L2 architecture, have proposals on the table, find potential agreements. Note that Architecture = function allocation, the aim is to understand a) what functionality we need or potentially need and b) then what protocol layer/entity houses this.

Intended outcome: Report, preparation for decisions.

Deadline: Long

The relevant contributions submitted to RAN2#111-e meeting are reviewed and proposals on L2 architecture/functions are summarized as below:

| Contribution Number | Proposals on L2 architecture/functions |
| --- | --- |
| R2-2007124 | Proposal 3: NR PTM transmission scheme should support the L1 A/N feedback and HARQ retransmission mechanism. Proposal 4: The UP architecture for unicast transmission is reused for the MBMS service transmission, which includes MAC, RLC, PDCP and SDAP |
| R2-2006793 | Proposal 3. MRB will support both L1 HARQ and L2 ARQ reliability. Proposal 4. MRB user plane AS protocol stack includes SDAP to support Multicast QoS flow to MRB/DRB mapping. Details of SDAP is FFS. Proposal 5. MRB user plane AS protocol stack includes PDCP to support loss-less HO, data recovery etc. It is FFS to support RoHC and Security aspects based on SA3 decision. Proposal 6. FFS whether to support single PDCP associated with multicast RLC entity and unicast RLC entity to enable dynamic switching between multicast and unicast RLC legs. Proposal 9. If Multicast RLC entity is configured in UM, allow PDCP entity to support L2 ARQ reliability. PDCP Re-transmission can be either via Multicast RLC entity or unicast entity up to network implementation. |
| R2-2006804 | Proposal 1: the SDAP, PDCP, RLC, MAC, PHY will exist for MBS service from gNB perspective no matter MBS is transmitted via PTM or PTP. The PDCP, RLC, MAC, PHY will exist for MBS service from UE perspective no matter MBS is received via PTM or PTP. Proposal 2: the option 1 (i.e. PTM and PTP share the SDAP/PDCP/RLC/MAC) and option 2 (i.e. PTM and PTP share the SDAP/PDCP) are agreed as baseline. Proposal 4: there is only one MAC entity shared with MBS reception and eMBB reception for simplicity. |
| R2-2006952 | Proposal 2: SDAP for MBS bearer uses SDAP Data PDU format without SDAP header, SDAP Control PDU are FFS depending on solution for service continuity in mobility. Proposal 5: RoHC and security is not supported in PDCP for PTM delivery of MBS. Other PDCP functionality for NR MBS service, especially for reordering, duplication, SN continuity, re-establishment, etc. are FFS depending on solution for service continuity. |
| R2-2007025 | Proposal 3: whether SDAP is needed or not can be decided based on SA2 inputs. Proposal 4: PDCP sublayer is used for NR MBS, and reordering and duplicate detection are supported. Proposal 5: RAN2 can further discuss if RoHC is supported in PDCP for MBS. Proposal 6: Whether security functions are supported in PDCP is subject to SA3. |
| R2-2006574 | Proposal 1: specify both SDAP layer and PDCP layer over Uu interface for NR downlink multicast/broadcast transmission. Proposal 2: Ask SA3 to understand the consideration of the security aspects for both PTM delivery mode and PTP delivery mode. Proposal 3: When PTM RB is established to deliver Multicast/broadcast data, unidirectional mode (U-mode) ROHC can be configured for NR multicast/broadcast PDCP packets if there is no support of uplink feedback at PDCP layer. |
| R2-2007442 | Proposal 15 SDAP is needed in NR MBS, functions as the mapping of QoS flows to radio bearers. Proposal 17 For each data radio bearer that is associated with the MBS session, there is a corresponding PDCP entity and RLC entity. Proposal 18 LS SA3 if ciphering for MBS session in access network is needed. Proposal 19 No MAC layer multiplexing among service data from different MBS session. |
| R2-2007550 | Proposal 1: Support having simplified PDCP for shared MBS bearer. Proposal 4: Adopt RLC, MAC, PHY and SYNC functions similar to MBMS in the protocol stack for MBS. |
| R2-2007672 | Proposal 10: NR PDCP is kept for PTM bearer. ROHC can be supported for MBS. Proposal 11: If UL feedback mechanism is introduced, PDCP reordering is reused for MBS. Proposal 12: RLC segmentation/ re-assembly is supported for PTM bearer. |
| R2-2007774 | Proposal 7 RAN2 should discuss if HARQ feedback/retransmission is useful on multicast in NR MBS, for UEs in RRC IDLE, INACTIVE and Connected. Proposal 8 RAN2 should discuss if RLC AM mode is supported for multicast in NR MBS, at least for UEs in RRC Connected. Proposal 9 RAN2 should discuss if PDCP layer is supported for groupcast in NR MBS, at least for UEs in RRC Connected. |
| R2-2008031 | Proposal 2. A PTM radio bearer has a PDCP entity. Proposal 3. A PTM radio bearer is configured with UM RLC. Proposal 4. A PTP radio bearer can be configured with either UM RLC or AM RLC. Proposal 5. A PTM radio bearer and the corresponding PTP radio bearer can be bound in the similar manner with user-plane structure for PDCP CA duplication. The PDCP entity can be associated with two RLC entities with different RLC modes. |
| R2-2006794 | Proposal 3. FFS based on SA3 multicast security discussion whether to use common PDCP entity between multicast and unicast radio bearers to enable lossless switching. Proposal 6. Introduce PDCP status reporting enhancements to enable dynamic switching between multicast bearer and unicast in lossless manner. |
| R2-2007015 | Proposal 2: RAN2 to introduce duplication transmission of multicast and unicast to enhance the reliability of multicast transmission. |
| R2-2007631 | Proposal 3 A common PDCP entity for both MRB and DRB delivering an MBS session is defined. Proposal 4 No need for SDAP, for an MBS session, a single QoS flow is mapped to MRB/DRB. |
| R2-2007026 | Proposal 2: Dynamic switch between PTP and PTM is up to the gNB and transparent to the UE, i.e. no signalling is needed. Proposal 3: Support dynamic switch between PTP and PTM within one bearer and one PDCP entity. |
| R2-2006982 | Proposal 3: RAN2 should study how to support the switching between multicast (PTM) and unicast (PTP) based on two alternatives for placement of switching function in the L2 architecture, with a switching function above PDCP and below PDCP. |
| R2-2006594 | Proposal 5: Discuss whether a given UE can receive the specific MBS service via PTM and PTP over radio interface simultaneously. Proposal 10: Discuss the necessity of functions provided by PDCP one by one and discuss the need of PDCP for MBS. Proposal 11: Discuss whether RLC AM mode should also be supported for M Proposal 13: Discuss whether data from different MBS traffic logical channels belonging the same MBS session could be multiplexed in MAC. |
| R2-2007637 | Proposal 2: Introducing the mapping of QoS Flows of an MBS session to different MBS/ unicast radio bearers should be studied in R17. Proposal 3: Supporting of at least one of the following PDCP reliability mechanisms; PDCP data recovery, PDCP re-establishment or loss-less switching between multicast bearers should be studied in R17. Proposal 4: Supporting of RLC-AM mode and RLC-TM mode by a multicast radio bearer, logical multicast traffic and/or control channels should be also studied in R17. |
| R2-2006803 | Proposal 6: the UE can receive the PTM and PTP simultaneously at least in a period during MBS switching between PTP and PTM in order to improve the reliability of MBS. |
| R2-2007034 | Proposal 4: The SDAP header is not configured for the MBS session. Proposal 5: RAN2 is kindly request to discuss whether the PDCP entity is configurable for MRB or DRB. Proposal 6: For MRB or the multicast leg of the split MRB, the RLC UM is supported. Proposal 8: RAN2 is kindly requested to discuss whether to support lossless switching and in-order delivery when the MBS service is switched between PTM and PTP. |
| R2-2007413 | Proposal 3: RAN2 should study the latency requirements of MBS services to decide which way is used to perform dynamic delivery mode switch: - Two protocol stacks are set in UE and network, and related resources are configured, and network could active/deactive one protocol stack via MAC CE or DCI; - Only one protocol stack is set in UE and network, once the network decides to switch the delivery mode, it sends RRCReconfiguration message with corresponding configurations to UE. |
| R2-2007443 | Proposal 11 The protocol stacks for PTP and PTM delivery mode share the PDCP entity. Proposal 12 For PTM delivery mode, the corresponding RLC entity is of UM mode; for PTP delivery mode, the corresponding RLC entity can be of both modes. |
| R2-2007466 | Proposal 1 A Multicast Radio Bearer (MRB) is configured to UE for 5G MBS, which includes a common PDCP layer associated with one RLC entity for PTM mode (PTM RLC Bearer) and/or one RLC entity for PTP mode (PTP RLC Bearer). Proposal 4 the SDAP entity is not needed in UE side for 5G MBS. Proposal 7 Sequence Numbering, Routing/Duplication, Reordering and Duplicate Discard functions are needed in PDCP layer. Proposal 8 It is assumed that ROCH is not supported for 5G MBS. Proposal 9 Whether security function (ciphering and/or integrity protection) is needed and whether same security function is used for PTM and PTP modes is pending to SA3. Proposal 10 RLC AM mode does not apply to PTM RLC bearer. RLC AM mode is supported for PTP RLC bearer. Proposal 11 Multiplexing for 5G MBS services should be supported. Proposal 12 How to support HARQ feedback and retransmission is pending to RAN1 discussion. |
| R2-2007551 | Proposal 2: Discuss and consider dual PTP/PTM protocol stack pre-configuration for fast dynamic PTP/PTM switch. |
| R2-2007633 | Proposal 2 HARQ feedback and corresponding retransmissions are supported. |
| R2-2008032 | Proposal 1. RAN2 discuss the transmission of the uplink feedback using a layer 2 signaling. |
| R2-2006596 | Proposal 3: In the scenario of multicast only, HARQ retransmission can be considered and the feedback/retransmission mechanism should be decided in RAN1 mostly. |
| R2-2007415 | Proposal 4: It is preferred to utilize the HARQ feedback and CSI reporting to realize the link adaptation and improve the reliability, especially for the UE in the cell edge. |
| R2-2006576 | Proposal 1: specify HARQ feedback from UE to network to enable reliable NR multicast transmission. Proposal 2: specify RLC layer feedback from UE to network to enable reliable NR multicast transmission. Proposal 3: support higher layer signalling to indicate the resource assignment for uplink HARQ feedback from UE to network. Proposal 4: specify a separate RLC channel for transmission of uplink feedback from UE to network to support reliable NR multicast transmission. |
| R2-2008062 | Proposal 1. RLC AM is not supported for MBS. Proposal 2. Whether and how to support HARQ retransmission is up to RAN1. |
| R2-2007028 | Proposal 1: HARQ feedback and retransmission should be discussed in RAN1 first, and RAN2 can discuss the support of PDCP feedback for NR MBS. |
| R2-2007445 | Proposal 6: NR MRB for PTM delivery mode should be configured with RLC UM mode. Proposal 7: NR MRB for PTP delivery mode could be configured with RLC UM or RLC AM. Proposal 8: To improve the reliability of NR broadcast/multicast services, HARQ feedback should be supported for NR MBS. |

The following discussions are conducted in accordance with the agreed scope and the above contributions.

# Discussion

- 2.1 NR L2 architecture overview

According to TS 38.300, the overall NR L2 protocol architecture is illustrated as below:

Figure: User Plane Protocol Stack

According to TS 37.324 and TS 38.32x series, the functions of each L2 sublayer are listed as below:

SDAP Sublayer:

-	transfer of user plane data;

-	mapping between a QoS flow and a DRB for both DL and UL;

-	mapping between a PC5 QoS flow and a SL-DRB for NR SL communication;

-	marking QoS flow ID in both DL and UL packets;

-	marking PC5 QoS flow ID in unicast of NR SL communication packets;

-	reflective QoS flow to DRB mapping for the UL SDAP data PDUs.

PDCP Sublayer:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	header compression and decompression using the EHC protocol;

-	ciphering and deciphering;

-	integrity protection and integrity verification;

-	timer based SDU discard;

-	for split bearers and DAPS bearer, routing;

-	duplication;

-	reordering and in-order delivery;

-	out-of-order delivery;

-	duplicate discarding.

-	feedback and retransmission (which has not been listed in sub-clause 4.4 of TS 38.323).

RLC Sublayer:

-	transfer of upper layer PDUs;

-	error correction through ARQ (only for AM data transfer);

-	segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

-	re-segmentation of RLC SDU segments (only for AM data transfer);

-	duplicate detection (only for AM data transfer);

-	RLC SDU discard (only for UM and AM data transfer);

-	RLC re-establishment;

-	Protocol error detection (only for AM data transfer).

MAC Sublayer

-	mapping between logical channels and transport channels;

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

-	scheduling information reporting;

-	error correction through HARQ;

-	logical channel prioritisation;

-	priority handling between overlapping resources of one UE;

-	radio resource selection.

For NR unicast transmission, the above functions of each sublayer are more or less useful at least in some cases. For NR MBS transmission, however, the legacy functions may not be totally applicable, which is the focus of this email discussion. In the following sections, companies are invited to give their opinions on what functions to layout for MBS transmission and which sublayer to accommodate the functions. After these two issues are settled, the L2 architecture will be almost in shape.

- 2.2 L2 functions for MBS

- 2.2.1 SDAP functions

- Mapping from QoS flows to radio bearers

This function is responsible for mapping QoS flows from CN to multicast radio bearer in RAN. Although QoS modelling for MBS should be finally concluded by SA2, RAN3 has made the following working assumptions:

Working Assumptions (by RAN3)： - One or more QoS flows may be used within a single MBS session. - Each MB QoS flow belongs to one MBS Session. - Each MB QoS flow is associated with a QoS profile. - NR MBS supports both GBR and non-GBR QoS. - One Shared NG-U tunnel is used per MBS session. For multicast, same QoS requirements are applicable regardless of whether PtP or PtM is selected by NG-RAN.

RAN2 can further discuss this issue based on the working assumption that the QoS flow concept will be reused for NR MBS. Companies are invited to give answers to the following question:

Q1: Do companies agree that the function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | We prefer to reuse the legacy unicast approach as much as possible |

- Other SDAP functions

There are some other functions in SDAP as listed in section 2.1. Of course, “transfer of user plane data” should be naturally supported if SDAP sublayer is concluded to be needed. Companies are invited to provide views on whether any other functions in SDAP (other than mapping from QoS flows to radio bearers and transfer of user plane data) are needed for NR MBS.

Q2: Do companies think that any functions in SDAP other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes for part of them | The rest SDAP functions other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are the followings: - marking QoS flow ID in both DL and UL packets; - reflective QoS flow to DRB mapping for the UL SDAP data PDUs. We see the need to have the function of “marking QoS flow ID in DL packets” but there is no need to take “reflective QoS flow to DRB mapping for the UL SDAP data PDUs” due to lack of the UL data flow |

- 2.2.2 PDCP functions

- Security

Security function includes ciphering/deciphering and integrity protection/ verification. Note that security function is very much related to the parallel discussion in SA3 under their study on security aspects of 5G multicast-broadcast services. RAN2 discussion on security may need to wait for SA3 progress first. Based on the information, companies are invited to give answers to the following questions:

Q3a: Do companies agree that RAN2 discussion on security function should wait for SA3 progress first?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | SA3 input helps |

Q3b: If the answer to Q2a is no, what can be discussed in RAN2 before inputs from SA3?

| Company | Comments |
| --- | --- |

- Header compression/decompression

In UMTS and LTE, the header compression/decompression function is supported for MBMS data in BM-SC. This function is especially efficient for voice traffic involved in mission critical services. Based on the information, companies are invited to give answers to the following questions:

Q4a: Do companies agree that the header compression/decompression function is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | RoHC should be always supported for IP packets over the air and one further discussion can be which RoHC mode should be selected for the DL MBS traffic. |

Q4b: If the answer to Q3a is yes, companies are further invited to give opinions on which sublayer/entity (e.g. PDCP or an entity in CN) to accommodate this function.

| Company | Answer (e.g. PDCP or an entity in CN) | Comments |
| --- | --- | --- |
| MediaTek | PDCP | PDCP should be the choice considering that the PTP may be used for the transmission over the air for MBS RB. |

- Reordering and in-order delivery

This function is to guarantee that packets are delivered to upper layer in the right order. Companies are invited to give answers to the following question:

Q5: Do companies agree that the reordering and in-order delivery function in PDCP is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | The need of reordering and in-order delivery function in PDCP for MBS RB is the same as for legacy unicast based transmission |

- PDCP feedback (i.e. status reporting) and retransmission

According to the WID [1], reliability is a significant requirement for NR MBS and reliability can be achieved via feedback and retransmission. Traditional feedback and retransmission mechanisms consist of: HARQ, ARQ and PDCP status reporting. During the 111-e meeting, it is agreed that HARQ is up to RAN1. Based on the information, companies are invited to give answers to the following question:

Q6: Do companies think that PDCP feedback and retransmission mechanism is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | The feedback and retransmission mechanism should be support in either RLC layer or PDPC layer other than HARQ layer feedback and retransmission |

- Other PDCP functions

There are some other functions in PDCP as listed in section 2.1. Of course, “transfer of data” should be naturally supported if PDCP sublayer is concluded as needed. Companies are invited to provide views on whether any other functions in PDCP (other than those discussed above in 2.2.2 and transfer of data) are needed for NR MBS.

Q7: Do companies think that any functions in PDCP other than those discussed above and “transfer of data” are needed for NR MBS?

| Company | Comments |
| --- | --- |
| MediaTek | There may be a need to keep the legacy PDPC function of both maintenance of PDCP SNs and duplicate discarding for NR MBS, since PDCP SN should be consistent during dynamic PTP/PTM switch and the duplicate packets received should be discarded e.g. when PTP/PTM is simultaneously active. |

- 2.2.3 RLC functions

For RLC, there are three transmission modes:

- Acknowledged Mode (AM);

- Unacknowledged Mode (UM); and

- Transparent Mode (TM).

It is assumed that the functions in each transmission mode should be supported as a whole, so we may not need to discuss each functions as listed in 2.1 one by one.

According to proposals submitted in the last meeting, for PTP and PTM transmission, companies have different views on which RLC transmission mode(s) should be supported. Therefore, companies are expected to provide views on the supported RLC transmission mode(s) for NR MBS for PTP and PTM separately.

Q8: Do companies think that RLC AM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | It should be needed to enable reliable transmission |

Q9: Do companies think that RLC AM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No | It would be too much complicated to maintain the PTM Tx windows for RLC |

Q10: Do companies think that RLC UM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | It should be needed dependent on the characteristics of the MBS RB. |

Q11: Do companies think that RLC UM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |

Q12: Do companies think that RLC TM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No |  |

Q13: Do companies think that RLC TM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No |  |

- 2.2.4 MAC functions

Some of the MAC functions as listed in 2.1 are for UL only, e.g. scheduling information reporting and logical channel prioritisation, which do not need to be discussed for NR MBS. Some functions should be naturally supported in MAC, e.g. mapping between logical channels and transport channels. According to the proposals submitted to RAN2#111-E, the functions that deserve to be specifically discussed here are as follows:

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

Whether to allow multiplexing of MAC SDUs from different logical channels onto a same TB is also related to discussion in 2.2.1 about QoS flow mapping, as well as whether different logical channels can be linked to the same G-RNTI. Companies are invited to provide views on the following question:

Q14: Do companies think that multiplexing/de-multiplexing of different logical channels should be supported in MAC for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No | We assume there is no need to link different logical channels to the same G-RNTI. One UE interested in only one MBS traffic (carried by one logical chanel) needs to decode a large MAC PDU if multiplexing of MAC SDUs from different logical channels are supported during TB assembly, which is suboptimal. |

- 2.3 Dynamic switch between PTP and PTM

According to the WID [1], dynamic switch between PTP and PTM is a function to be supported for NR MBS. During the 111-e meeting, it is agreed that the gNB makes the decision of the dynamic switch, while which sublayer handles the details is FFS. Companies are invited to give answers to the following question:

Q15: Which sublayer acts as the anchor of PTP and PTM dynamic switch, i.e. the splitting and converging sublayer for MBS traffic transmitted via PTP and PTM?

| Company | Answer (e.g. SDAP/PDCP/RLC/MAC) | Comments |
| --- | --- | --- |
| MediaTek | PDCP or RLC | We assume the discussion of PTP and PTM dynamic switch can be subject to the modelling of the MBS MRB. So far we see two alternatives, one is PDCP based and the other is RLC based. So accordingly, the PTP and PTM dynamic switch can be conducted at either PDCP or RLC layer. |

- 2.4 Phase-2 discussion: L2 architecture for MBS

(To be discussed in Phase II of this email discussion)

# Conclusion

# Reference

- RP-201038 WID revision: NR Multicast and Broadcast Services, Huawei, HiSilicon