3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Electronic, 02nd – 13th November 2020

Agenda item:	x.x.x

Source: 	Huawei

Title: 	Email discussion [Post111-e][904][MBS] L2 Architecture

Document for:	Discussion and Decision

# Introduction

This paper is aimed at discussing the following topic.

- [Post111-e][904][MBS] L2 Architecture (Huawei)

Scope: L2 architecture, have proposals on the table, find potential agreements. Note that Architecture = function allocation, the aim is to understand a) what functionality we need or potentially need and b) then what protocol layer/entity houses this.

Intended outcome: Report, preparation for decisions.

Deadline: Long

The relevant contributions submitted to RAN2#111-e meeting are reviewed and proposals on L2 architecture/functions are summarized as below:

| Contribution Number | Proposals on L2 architecture/functions |
| --- | --- |
| R2-2007124 | Proposal 3: NR PTM transmission scheme should support the L1 A/N feedback and HARQ retransmission mechanism. Proposal 4: The UP architecture for unicast transmission is reused for the MBMS service transmission, which includes MAC, RLC, PDCP and SDAP |
| R2-2006793 | Proposal 3. MRB will support both L1 HARQ and L2 ARQ reliability. Proposal 4. MRB user plane AS protocol stack includes SDAP to support Multicast QoS flow to MRB/DRB mapping. Details of SDAP is FFS. Proposal 5. MRB user plane AS protocol stack includes PDCP to support loss-less HO, data recovery etc. It is FFS to support RoHC and Security aspects based on SA3 decision. Proposal 6. FFS whether to support single PDCP associated with multicast RLC entity and unicast RLC entity to enable dynamic switching between multicast and unicast RLC legs. Proposal 9. If Multicast RLC entity is configured in UM, allow PDCP entity to support L2 ARQ reliability. PDCP Re-transmission can be either via Multicast RLC entity or unicast entity up to network implementation. |
| R2-2006804 | Proposal 1: the SDAP, PDCP, RLC, MAC, PHY will exist for MBS service from gNB perspective no matter MBS is transmitted via PTM or PTP. The PDCP, RLC, MAC, PHY will exist for MBS service from UE perspective no matter MBS is received via PTM or PTP. Proposal 2: the option 1 (i.e. PTM and PTP share the SDAP/PDCP/RLC/MAC) and option 2 (i.e. PTM and PTP share the SDAP/PDCP) are agreed as baseline. Proposal 4: there is only one MAC entity shared with MBS reception and eMBB reception for simplicity. |
| R2-2006952 | Proposal 2: SDAP for MBS bearer uses SDAP Data PDU format without SDAP header, SDAP Control PDU are FFS depending on solution for service continuity in mobility. Proposal 5: RoHC and security is not supported in PDCP for PTM delivery of MBS. Other PDCP functionality for NR MBS service, especially for reordering, duplication, SN continuity, re-establishment, etc. are FFS depending on solution for service continuity. |
| R2-2007025 | Proposal 3: whether SDAP is needed or not can be decided based on SA2 inputs. Proposal 4: PDCP sublayer is used for NR MBS, and reordering and duplicate detection are supported. Proposal 5: RAN2 can further discuss if RoHC is supported in PDCP for MBS. Proposal 6: Whether security functions are supported in PDCP is subject to SA3. |
| R2-2006574 | Proposal 1: specify both SDAP layer and PDCP layer over Uu interface for NR downlink multicast/broadcast transmission. Proposal 2: Ask SA3 to understand the consideration of the security aspects for both PTM delivery mode and PTP delivery mode. Proposal 3: When PTM RB is established to deliver Multicast/broadcast data, unidirectional mode (U-mode) ROHC can be configured for NR multicast/broadcast PDCP packets if there is no support of uplink feedback at PDCP layer. |
| R2-2007442 | Proposal 15 SDAP is needed in NR MBS, functions as the mapping of QoS flows to radio bearers. Proposal 17 For each data radio bearer that is associated with the MBS session, there is a corresponding PDCP entity and RLC entity. Proposal 18 LS SA3 if ciphering for MBS session in access network is needed. Proposal 19 No MAC layer multiplexing among service data from different MBS session. |
| R2-2007550 | Proposal 1: Support having simplified PDCP for shared MBS bearer. Proposal 4: Adopt RLC, MAC, PHY and SYNC functions similar to MBMS in the protocol stack for MBS. |
| R2-2007672 | Proposal 10: NR PDCP is kept for PTM bearer. ROHC can be supported for MBS. Proposal 11: If UL feedback mechanism is introduced, PDCP reordering is reused for MBS. Proposal 12: RLC segmentation/ re-assembly is supported for PTM bearer. |
| R2-2007774 | Proposal 7 RAN2 should discuss if HARQ feedback/retransmission is useful on multicast in NR MBS, for UEs in RRC IDLE, INACTIVE and Connected. Proposal 8 RAN2 should discuss if RLC AM mode is supported for multicast in NR MBS, at least for UEs in RRC Connected. Proposal 9 RAN2 should discuss if PDCP layer is supported for groupcast in NR MBS, at least for UEs in RRC Connected. |
| R2-2008031 | Proposal 2. A PTM radio bearer has a PDCP entity. Proposal 3. A PTM radio bearer is configured with UM RLC. Proposal 4. A PTP radio bearer can be configured with either UM RLC or AM RLC. Proposal 5. A PTM radio bearer and the corresponding PTP radio bearer can be bound in the similar manner with user-plane structure for PDCP CA duplication. The PDCP entity can be associated with two RLC entities with different RLC modes. |
| R2-2006794 | Proposal 3. FFS based on SA3 multicast security discussion whether to use common PDCP entity between multicast and unicast radio bearers to enable lossless switching. Proposal 6. Introduce PDCP status reporting enhancements to enable dynamic switching between multicast bearer and unicast in lossless manner. |
| R2-2007015 | Proposal 2: RAN2 to introduce duplication transmission of multicast and unicast to enhance the reliability of multicast transmission. |
| R2-2007631 | Proposal 3 A common PDCP entity for both MRB and DRB delivering an MBS session is defined. Proposal 4 No need for SDAP, for an MBS session, a single QoS flow is mapped to MRB/DRB. |
| R2-2007026 | Proposal 2: Dynamic switch between PTP and PTM is up to the gNB and transparent to the UE, i.e. no signalling is needed. Proposal 3: Support dynamic switch between PTP and PTM within one bearer and one PDCP entity. |
| R2-2006982 | Proposal 3: RAN2 should study how to support the switching between multicast (PTM) and unicast (PTP) based on two alternatives for placement of switching function in the L2 architecture, with a switching function above PDCP and below PDCP. |
| R2-2006594 | Proposal 5: Discuss whether a given UE can receive the specific MBS service via PTM and PTP over radio interface simultaneously. Proposal 10: Discuss the necessity of functions provided by PDCP one by one and discuss the need of PDCP for MBS. Proposal 11: Discuss whether RLC AM mode should also be supported for M Proposal 13: Discuss whether data from different MBS traffic logical channels belonging the same MBS session could be multiplexed in MAC. |
| R2-2007637 | Proposal 2: Introducing the mapping of QoS Flows of an MBS session to different MBS/ unicast radio bearers should be studied in R17. Proposal 3: Supporting of at least one of the following PDCP reliability mechanisms; PDCP data recovery, PDCP re-establishment or loss-less switching between multicast bearers should be studied in R17. Proposal 4: Supporting of RLC-AM mode and RLC-TM mode by a multicast radio bearer, logical multicast traffic and/or control channels should be also studied in R17. |
| R2-2006803 | Proposal 6: the UE can receive the PTM and PTP simultaneously at least in a period during MBS switching between PTP and PTM in order to improve the reliability of MBS. |
| R2-2007034 | Proposal 4: The SDAP header is not configured for the MBS session. Proposal 5: RAN2 is kindly request to discuss whether the PDCP entity is configurable for MRB or DRB. Proposal 6: For MRB or the multicast leg of the split MRB, the RLC UM is supported. Proposal 8: RAN2 is kindly requested to discuss whether to support lossless switching and in-order delivery when the MBS service is switched between PTM and PTP. |
| R2-2007413 | Proposal 3: RAN2 should study the latency requirements of MBS services to decide which way is used to perform dynamic delivery mode switch: - Two protocol stacks are set in UE and network, and related resources are configured, and network could active/deactive one protocol stack via MAC CE or DCI; - Only one protocol stack is set in UE and network, once the network decides to switch the delivery mode, it sends RRCReconfiguration message with corresponding configurations to UE. |
| R2-2007443 | Proposal 11 The protocol stacks for PTP and PTM delivery mode share the PDCP entity. Proposal 12 For PTM delivery mode, the corresponding RLC entity is of UM mode; for PTP delivery mode, the corresponding RLC entity can be of both modes. |
| R2-2007466 | Proposal 1 A Multicast Radio Bearer (MRB) is configured to UE for 5G MBS, which includes a common PDCP layer associated with one RLC entity for PTM mode (PTM RLC Bearer) and/or one RLC entity for PTP mode (PTP RLC Bearer). Proposal 4 the SDAP entity is not needed in UE side for 5G MBS. Proposal 7 Sequence Numbering, Routing/Duplication, Reordering and Duplicate Discard functions are needed in PDCP layer. Proposal 8 It is assumed that ROCH is not supported for 5G MBS. Proposal 9 Whether security function (ciphering and/or integrity protection) is needed and whether same security function is used for PTM and PTP modes is pending to SA3. Proposal 10 RLC AM mode does not apply to PTM RLC bearer. RLC AM mode is supported for PTP RLC bearer. Proposal 11 Multiplexing for 5G MBS services should be supported. Proposal 12 How to support HARQ feedback and retransmission is pending to RAN1 discussion. |
| R2-2007551 | Proposal 2: Discuss and consider dual PTP/PTM protocol stack pre-configuration for fast dynamic PTP/PTM switch. |
| R2-2007633 | Proposal 2 HARQ feedback and corresponding retransmissions are supported. |
| R2-2008032 | Proposal 1. RAN2 discuss the transmission of the uplink feedback using a layer 2 signaling. |
| R2-2006596 | Proposal 3: In the scenario of multicast only, HARQ retransmission can be considered and the feedback/retransmission mechanism should be decided in RAN1 mostly. |
| R2-2007415 | Proposal 4: It is preferred to utilize the HARQ feedback and CSI reporting to realize the link adaptation and improve the reliability, especially for the UE in the cell edge. |
| R2-2006576 | Proposal 1: specify HARQ feedback from UE to network to enable reliable NR multicast transmission. Proposal 2: specify RLC layer feedback from UE to network to enable reliable NR multicast transmission. Proposal 3: support higher layer signalling to indicate the resource assignment for uplink HARQ feedback from UE to network. Proposal 4: specify a separate RLC channel for transmission of uplink feedback from UE to network to support reliable NR multicast transmission. |
| R2-2008062 | Proposal 1. RLC AM is not supported for MBS. Proposal 2. Whether and how to support HARQ retransmission is up to RAN1. |
| R2-2007028 | Proposal 1: HARQ feedback and retransmission should be discussed in RAN1 first, and RAN2 can discuss the support of PDCP feedback for NR MBS. |
| R2-2007445 | Proposal 6: NR MRB for PTM delivery mode should be configured with RLC UM mode. Proposal 7: NR MRB for PTP delivery mode could be configured with RLC UM or RLC AM. Proposal 8: To improve the reliability of NR broadcast/multicast services, HARQ feedback should be supported for NR MBS. |

The following discussions are conducted in accordance with the agreed scope and the above contributions.

# Discussion

- 2.1 NR L2 architecture overview

According to TS 38.300, the overall NR L2 protocol architecture is illustrated as below:

Figure: User Plane Protocol Stack

According to TS 37.324 and TS 38.32x series, the functions of each L2 sublayer are listed as below:

SDAP Sublayer:

-	transfer of user plane data;

-	mapping between a QoS flow and a DRB for both DL and UL;

-	mapping between a PC5 QoS flow and a SL-DRB for NR SL communication;

-	marking QoS flow ID in both DL and UL packets;

-	marking PC5 QoS flow ID in unicast of NR SL communication packets;

-	reflective QoS flow to DRB mapping for the UL SDAP data PDUs.

PDCP Sublayer:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	header compression and decompression using the EHC protocol;

-	ciphering and deciphering;

-	integrity protection and integrity verification;

-	timer based SDU discard;

-	for split bearers and DAPS bearer, routing;

-	duplication;

-	reordering and in-order delivery;

-	out-of-order delivery;

-	duplicate discarding.

-	feedback and retransmission (which has not been listed in sub-clause 4.4 of TS 38.323).

RLC Sublayer:

-	transfer of upper layer PDUs;

-	error correction through ARQ (only for AM data transfer);

-	segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

-	re-segmentation of RLC SDU segments (only for AM data transfer);

-	duplicate detection (only for AM data transfer);

-	RLC SDU discard (only for UM and AM data transfer);

-	RLC re-establishment;

-	Protocol error detection (only for AM data transfer).

MAC Sublayer

-	mapping between logical channels and transport channels;

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

-	scheduling information reporting;

-	error correction through HARQ;

-	logical channel prioritisation;

-	priority handling between overlapping resources of one UE;

-	radio resource selection.

For NR unicast transmission, the above functions of each sublayer are more or less useful at least in some cases. For NR MBS transmission, however, the legacy functions may not be totally applicable, which is the focus of this email discussion. In the following sections, companies are invited to give their opinions on what functions to layout for MBS transmission and which sublayer to accommodate the functions. After these two issues are settled, the L2 architecture will be almost in shape.

- 2.2 L2 functions for MBS

- 2.2.1 SDAP functions

- Mapping from QoS flows to radio bearers

This function is responsible for mapping QoS flows from CN to multicast radio bearer in RAN. Although QoS modelling for MBS should be finally concluded by SA2, RAN3 has made the following working assumptions:

Working Assumptions (by RAN3)： - One or more QoS flows may be used within a single MBS session. - Each MB QoS flow belongs to one MBS Session. - Each MB QoS flow is associated with a QoS profile. - NR MBS supports both GBR and non-GBR QoS. - One Shared NG-U tunnel is used per MBS session. For multicast, same QoS requirements are applicable regardless of whether PtP or PtM is selected by NG-RAN.

RAN2 can further discuss this issue based on the working assumption that the QoS flow concept will be reused for NR MBS. Companies are invited to give answers to the following question:

Q1: Do companies agree that the function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | We prefer to reuse the legacy unicast approach as much as possible |
| Lenovo, Motorola Mobility | Yes | For downlink, when receiving data of a 5G MBS session from core network, the gNB still needs to perform the QoS flows to DRB mapping in SDAP layer. The same principle of unicast service should be applied to 5G MBS. The SDAP header may not be needed for 5G MBS. |
| OPPO | Yes | Due to the agreements made in RAN3, i.e. One or more QoS flows may be used within a single MBS session, we think the SDAP is necessary for Qos flow to DRB mapping purpose in gNB side. The SDAP header may be not needed due to no UL data. The SDAP layer is not needed in UE side. |
| ZTE | Yes | We suggest following the design in NR unicast and NR V2X (for Broadcast, Groupcast and Unicast) for the QoS model with SDAP mapping QoS flow(s) to radio bearer(s). Let's make it a working assumption for NR MBS and see what SA2 has to offer in QoS model. |
| NEC | Yes, with additional comment | We agree with the above SDAP functions of mapping from QoS flow to MBS RB, but given the QoS flow of the shared NG-U tunnel is mapped to RB, the reflective QoS is not needed for MBS scenario, so the SDAP header is not needed for MBS. It should be noted that it is FFS whether SDAP control PDU is needed for MBS. |
| Samsung | Yes | gNB should be responsible for mapping from QF to MBS RB. Reflective QoS is not necessary for MBS, so SDAP header is not necessary. |
| Kyocera | Yes, if SA2 decides | We think it’s reasonable to reuse the existing function in SDAP, if SA2 decides QoS flow for MBS. |
| QC | Yes | In DL, for a given shared MBS session carrying multiple MB QoS flows need to be mapped to either PTP/PTM bearer in SDAP sublayer. However, since there is no UL data, there is no need of AS reflective QoS and no need of SDAP header. |
| CATT | Yes | As there may be one or more QoS flows within one MBS session according to working assumption by RAN3, mapping from QoS flows to MBS RBs in downlink should be supported in SDAP Besides this function may only be needed on NG-RAN side, and no SDAP header is needed for MBS. |
| Huawei, HiSilicon | Yes | Yes, the function of mapping from QoS flows to MBS RBs should be supported in RAN as different RBs may be needed to satisfy the requirement of different QoS flows. Since MBS is only for downlink transmission, gNB implementation can handle the mapping for simplicity. |
| Spreadtrum | Yes | The mapping between the QoS flow of MBS session and MRB is needed in gNB and this function should locate in SDAP layer as in legacy unicast. |
| LG | Yes | We think that reflective QoS is not necessary. However, the need for SDAP header needs further discussion because QFI value in downlink SDAP header may be helpful to classify downlink QoS flows. |
| CMCC | Yes | We prefer to following the legacy unicast approach of mapping from QoS flows to MBS RBs in SDAP. |
| Nokia | Yes | SDAP can be kept transparent to the UE though. |
| Sony | Yes | We think it is reasonable because RAN3 working assumption has a dependency on the presence of SDAP layer and aligns the protocol stack with unicast. |

- Other SDAP functions

There are some other functions in SDAP as listed in section 2.1. Of course, “transfer of user plane data” should be naturally supported if SDAP sublayer is concluded to be needed. Companies are invited to provide views on whether any other functions in SDAP (other than mapping from QoS flows to radio bearers and transfer of user plane data) are needed for NR MBS.

Q2: Do companies think that any functions in SDAP other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes for part of them | The rest SDAP functions other than “mapping from QoS flows to radio bearers” and “transfer of user plane data” are the followings: - marking QoS flow ID in both DL and UL packets; - reflective QoS flow to DRB mapping for the UL SDAP data PDUs. We see the need to have the function of “marking QoS flow ID in DL packets” but there is no need to take “reflective QoS flow to DRB mapping for the UL SDAP data PDUs” due to lack of the UL data flow |
| Lenovo, Motorola Mobility | See comments | The functionality of mapping between a QoS flow and a DRB for downlink is needed. Since there is no uplink 5G MBS traffic, the following functionalities may not be needed: - mapping between a QoS flow and a DRB for UL; - marking QoS flow ID in both DL and UL packets; - reflective QoS flow to DRB mapping for the UL SDAP data PDUs. whether to support QoS flows to DRB remapping is to be discussed. To reduce the standard effort, it can be assumed that QoS flows to DRB remapping is not supported for 5G MBS. |
| OPPO | No |  |
| ZTE | No | There are other two functions in SDAP sublayer: Reflective QoS, and QoS flow remapping. - Reflective QoS is definitely not needed for downlink only MBS. - QoS flow remapping can be done by network implementation without relying on SDAP header of QFI, as in NR V2X Groupcast/Broadcast. Therefore, the answer is no for Q2 from spec impacts perspective. |
| Samsung | Yes | Agree with Lenovo |
| Kyocera | No | We assume MBS data is conveyed on “Data PDU without SDAP header” as specified in section 6.2.2.1 of TS37.324. |
| QC | No | No need for Reflective QoS, No need of DL header. As there is no UL data, there is no need of any of UL SDAP functions. Like ZTE mentioned, it should be allowed to re-mapping of DL QoS flow to radio bearers as NW implementation. |
| CATT | No | All the other functions are used for uplink. They are not needed as there is only downlink data on MRB. |
| Huawei, HiSilicon | No | The motivation to mark QoS flow ID in DL packets is to realize reflective QoS flow to DRB mapping for the UL packets. Thus no other functions are needed for MBS. |
| Spreadtrum | No | For the MBS session, there is no UL QoS flow. So the Reflective QoS function and UL QoS flow remapping function in SDAP are not needed. The DL QoS flow remapping can be done by network implementation if QoS flow remapping is supported. |
| LG | Yes | Marking QoS flow ID in DL packets may be beneficial. |
| CMCC | Yes, part of them | Since there’s no UL traffic for MBS, the function of making QoS flow ID in UL packets and reflective QoS flew to DRB mapping for the UL SDAP data PDUs may be not needed, while the function of making QoS flow ID in DL packets could be supported if more than one QoS flow is used within a single MBS session as RAN3 assumed. |
| Nokia | No | We haven’t identified the need for other functions and thus SDAP can be kept transparent to the UE. |
| Sony | No | We think QoS flow to DRB mapping in DL should be sufficient in this release. |

- 2.2.2 PDCP functions

- Security

Security function includes ciphering/deciphering and integrity protection/ verification. Note that security function is very much related to the parallel discussion in SA3 under their study on security aspects of 5G multicast-broadcast services. RAN2 discussion on security may need to wait for SA3 progress first. Based on the information, companies are invited to give answers to the following questions:

Q3a: Do companies agree that RAN2 discussion on security function should wait for SA3 progress first?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | SA3 input helps |
| Lenovo, Motorola Mobility | Yes | We agree with waiting for SA3 progress first. Whether security function (ciphering and/or integrity protection) is needed and whether same security function is used for PTM and PTP modes depends on SA3’s progress. |
| OPPO | Yes | We can wait for SA3’s inputs. |
| ZTE | Yes | Yes. The decision on whether ciphering in the air interface is needed is up to SA3. LS to SA3 as companies suggested in RAN2 #111e can be sent out asap. |
| NEC | Yes | We can wait for SA3’s inputs. |
| Samsung | Yes |  |
| Kyocera | Yes | We agree with the rapporteur’s view. |
| QC | Yes | Wait for SA3 progress. We need to understand whether same or different security need to be used for PTM and PTP cases if independent PDCP used? |
| CATT | Yes | We should wait for SA3’s decision on whether the security function will be located in RAN |
| Huawei, HiSilicon | Yes | RAN2 should be involved only if SA3 decides to locate security function in RAN. Regarding Qualcomm’s question, our understanding is that PTM and PTP are only different in terms of scheduling RNTIs. The security function for MBS service should be independent from PTM or PTP. RAN2 can make this working assumption and if there is any doubt SA3 can be consulted with to confirm this working assumption. |
| Spreadtrum | Yes | We can wait for SA3’s inputs. |
| LG | Yes |  |
| CMCC | Yes | Wait for SA3’s progress. |
| Nokia | Yes |  |
| Sony | Yes |  |

Q3b: If the answer to Q3a is no, what can be discussed in RAN2 before inputs from SA3?

| Company | Comments |
| --- | --- |

- Header compression/decompression

In UMTS and LTE, the header compression/decompression function is supported for MBMS data in BM-SC. This function is especially efficient for voice traffic involved in mission critical services. Based on the information, companies are invited to give answers to the following questions:

Q4a: Do companies agree that the header compression/decompression function is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | RoHC should be always supported for IP packets over the air and one further discussion can be which RoHC mode should be selected for the DL MBS traffic. |
| Lenovo, Motorola Mobility | FFS | It is clear that ROHC is beneficial for header compression of small data as VoIP. However, there may be no UL feedback, and only unidirectional mode can be used. The other way is to introduce UL feedback to support more RoHC modes e.g. Bi-directional RoHC mode. Furthermore, the potential support of header compression/decompression in RAN also depends on the outcome of SA2’s architecture enhancements for MBS. If it will be supported in core network, then there is no need to duplicate the function in RAN. |
| OPPO | Yes | The ROHC is beneficial for small IP data. For MBS, we can discuss whether it is necessary for MBS data. The header compression/decompression function is supported for MBMS data in BM-SC and ROHC is supported for unicsst/multicast/broadcast in V2X. So it seems it is reasonable to support header compression/decompression function. |
| ZTE | Yes | Uni-directional ROHC can be at least considered. |
| NEC | Yes, it can be configurable | In LTE MBMS, there is no PDCP entity at all, namely ROHC is not supported. And it is quite clear that ROHC is usually used for IP packets compression. Given the diversity of NR MBMS service, we think it should be configurable by the network whether this MBS service is supportive for ROHC. |
| Samsung | Yes | ROHC is useful for small-sized IP packet. It should be one of major use cases of NR MBS. |
| Kyocera | Yes | We think the header (de)compression is still useful for NR MBS. |
| QC | Yes | May be unidirectional DL only ROHC as optional can be considered as baseline function. FFS whether to support ROHC feedback or not ? |
| CATT | Yes but | ROHC is efficient for services like voice call, Maybe we could limit the discussion on whether to support bidirectional mode ROHC as ROHC in unidirectional mode is not efficient and may result in additional delay. For MBS services received in connected mode only, Bidirectional mode is possible as UL feedback channel could exists, so ROHC could be supported for service received in connected only. |
| Huawei, HiSilicon | Yes | ROHC is especially beneficial for public safety services such as voice which is usually of small packets. |
| Spreadtrum | Yes | The ROHC is beneficial to the services with small data. |
| LG | Yes | It is assumed that it is supported in a limited manner such as supporting U-mode only. |
| CMCC | Yes | The header compression/decompression is beneficial in some cases. But we need to wait for SA2’s progress to decide whether it is supported in RAN. |
| Nokia | FFS | Agree with Lenovo. |
| Sony | Yes | At least Unidirectional mode can be supported for DL MBS traffic. |

Q4b: If the answer to Q4a is yes, companies are further invited to give opinions on which sublayer/entity (e.g. PDCP or an entity in CN) to accommodate this function.

| Company | Answer (e.g. PDCP or an entity in CN) | Comments |
| --- | --- | --- |
| MediaTek | PDCP | PDCP should be the choice considering that the PTP may be used for the transmission over the air for MBS RB. |
| Lenovo, Motorola Mobility | PDCP | If header compression/decompression in RAN is needed, then PDCP is the entity which is responsible for RoHC of Uu interface. |
| OPPO | PDCP | If the PDCP is supported in MBS, and ROHC is in PDCP. So, it is reasonable to support ROGC in PDCP. |
| ZTE | PDCP | Current design of PDCP just works. |
| NEC | PDCP | CN makes no sense. |
| Samsung | PDCP | We do not need BM-SC-like entity only for ROHC. PDCP seems to be necessary for reordering function. We can just reuse PDCP similar to Uu ROHC. |
| Kyocera | PDCP | We think RAN2 should consider the case where the UEs late joins the MBS session. We assume to take RoHC as baseline for header (de)compression. |
| QC | PDCP |  |
| CATT | PDCP | It is natural for ROHC to be located in PDCP. |
| Huawei, HiSilicon | PDCP | Maybe the simplest way is to reuse PDCP. |
| Spreadtrum | PDCP | ROHC should locate in PDCP as in NR. |
| LG | PDCP | The ROHC function of PDCP can be used for this. |
| CMCC | PDCP |  |
| Sony | PDCP |  |

- Reordering and in-order delivery

This function is to guarantee that packets are delivered to upper layer in the right order. Companies are invited to give answers to the following question:

Q5: Do companies agree that the reordering and in-order delivery function in PDCP is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | The need of reordering and in-order delivery function in PDCP for MBS RB is the same as for legacy unicast based transmission |
| Lenovo, Motorola Mobility | Yes | In-order delivery is important for TCP based traffic. 5G MBS supports high reliability, in-order delivery should be supported in PDCP. And considering the service continuity during mobility and during dynamic switching between PTM and PTP, re-ordering and in-order delivery function in PDCP is needed. |
| OPPO | Yes | NR RLC may not deliver data in order; it is different from LTE RLC (e.g UM mode RLC). The NR PDCP reordering function and in order delivery function are necessary. |
| ZTE | Yes | Considering the diverse scenarios/requirements of NR MBS, in-order delivery shall be supported first. |
| NEC | Yes, but configurable | We agree all the scenarios supportive for in-order delivery and reordering. But we believe there are still some delay sensitive scenarios for out-of-order delivery. So we think it should be like unicast, in-order delivery and reordering is configurable. |
| Samsung | Yes | Since HARQ feedback was agreed to introduce, in-order delivery is essential. |
| Kyocera | Yes | We think the reordering/in-order delivery is needed at least for dynamic PTM PTP switch. |
| QC | Yes | Same view as Lenovo. Like unicast both in-order and out of order delivery to be supported based on configuration choice. |
| CATT | Yes | Reordering function is necessary for services like TCP applications. So in-order delivery is needed at least for normal PTM transmission. And FFS for dynamic PTM/PTP switch. Both in-order delivery and out of order delivery should be supported. |
| Huawei, HiSilicon | Yes | As a basic AS function, in-order delivery of upper packets should be guaranteed, which is especially beneficial for MBS services like V2X applications and software update. |
| Spreadtrum | Yes | The reordering and in-order delivery function is necessary to some MBS services, and it should be supported. |
| LG | Yes | The reordering and in-order delivery functions in PDCP can be used in MBS transmission and PTM/PTP switching. |
| CMCC | Yes | Share similar view with Media Tek. |
| Nokia | Yes |  |
| Sony | Yes | We see no motivation to deviate from unicast |

- PDCP feedback (i.e. status reporting) and retransmission

According to the WID [1], reliability is a significant requirement for NR MBS and reliability can be achieved via feedback and retransmission. Traditional feedback and retransmission mechanisms consist of: HARQ, ARQ and PDCP status reporting. During the 111-e meeting, it is agreed that HARQ is up to RAN1. Based on the information, companies are invited to give answers to the following question:

Q6: Do companies think that PDCP feedback and retransmission mechanism is needed for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | The feedback and retransmission mechanism should be support in either RLC layer or PDPC layer other than HARQ layer feedback and retransmission |
| Lenovo, Motorola Mobility | Yes | PDCP status reporting is needed at least for mobility case. Whether enhancements of PDCP status reporting for PTP and PTM switching needs further discussion. |
| OPPO | Yes | Considering the MBS mobility and dynamic switching between PTP and PTM, the PDCP feedback and retransmission mechanisms are needed for NR MBS. |
| ZTE | Yes | PDCP SR is already there or will be beneficial in cases of mobility (especially in intra-CU scenarios) and mode switching. PDCP retransmission as one of the L2 retranmission schemes can be applied without much standardization effort. |
| NEC | Yes | In order to ensure the reliability for some service, the PDCP feedback and RLC/HARQ retransmission is needed. |
| Samsung | No | In unicast, PDCP status report and retransmission are supported only for handover/bearer type change of AM bearer. We think AM can be supported only for PTP bearer. This means that PDCP status report for MBS can be supported only for PTP AM -> PTP AM switching. This is just a unicast handover/BTC. We don’t need to introduce any new usage. |
| Kyocera | Yes | We think the PDCP feedback and retransmission mechanism is still useful e.g., for handover. |
| QC | Yes | PDCP status reporting is needed to support loss-less RLC AM HO and loss-less switching between PTM and PTP bearer. In order to support high reliability services, we think both RLC AM and HARQ to be supported. R16 DAPS HO supports PDCP status reporting for RLC UM as well. If DAPS is supported for Multicast radio bearer HO, then PDPC SR is need to be supported for RLC UM DAPS bearer as well. |
| CATT | Maybe | Whether PDCP feedback is needed depends on requirement on data lossless delivery. Besides, as mentioned by Samsung, In unicast, PDCP status report and retransmission are supported only RLC AM bearer. So maybe PDCP feedback and retransmission should be discussed later after it is concluded on whether to support RLC AM. |
| Huawei, HiSilicon | Yes | At least for handover, PDCP retransmission is useful to eliminate packet loss, which is not a new use case. Whether to extend PDCP status reporting to normal transmission can be further discussed, but it seems the extension is simple and will not introduce many specification changes. |
| Spreadtrum | Yes | PDCP feedback and retransmission mechanism is needed at least for the mobility case. |
| LG | Yes | PDCP feedback and retransmission mechanism will provide enhanced reliability in MBS transmission and PTM/PTP switching together with the reordering and in-order delivery functions in PDCP. |
| CMCC | Yes | It’s reasonable to support the PDCP feedback and retransmission, considering the mobility scenario and dynamic delivery mode switch. |
| Nokia | No in the sense of ARQ like operation | PDCP Status reports are provided to reduce duplicates over the air, not to trigger retransmissions. Retransmissions of PDUs “for which the successful delivery has not been confirmed by lower layers” should be limited to the same scenarios as in legacy i.e. re-establishment and recovery |
| Sony | Yes |  |

- Other PDCP functions

There are some other functions in PDCP as listed in section 2.1. Of course, “transfer of data” should be naturally supported if PDCP sublayer is concluded as needed. Companies are invited to provide views on whether any other functions in PDCP (other than those discussed above in 2.2.2 and transfer of data) are needed for NR MBS.

Q7: Do companies think that any functions in PDCP other than those discussed above and “transfer of data” are needed for NR MBS?

| Company | Comments |
| --- | --- |
| MediaTek | There may be a need to keep the legacy PDPC function of both maintenance of PDCP SNs and duplicate discarding for NR MBS, since PDCP SN should be consistent during dynamic PTP/PTM switch and the duplicate packets received should be discarded e.g. when PTP/PTM is simultaneously active. |
| Lenovo, Motorola Mobility | We prefer to have a common PDCP layer for PTM/PTP switching: Based on above assumption, the sequence Numbering, Routing/Duplication, and Duplicate Discard functions are needed in PDCP layer. |
| OPPO | The PDCP layer supports the following functions: - transfer of data (user plane or control plane); - maintenance of PDCP SNs; - header compression and decompression using the ROHC protocol; - ciphering and deciphering; - integrity protection and integrity verification; - timer based SDU discard; - for split bearers, routing; - duplication; - reordering and in-order delivery; - out-of-order delivery; - duplicate discarding. We think all the functions except the security function (wait for SA3) should be supported for NR MBS. |
| ZTE | Be aligned with NR design. If PDCP is the anchor layer for mode switching, current duplication function (in NW side) will work. |
| NEC | The legacy unicast PDCP functions are the baseline of MBS service. The above questions clarified the controversial PDCP functions. |
| Kyocera | We think the “maintenance of PDCP SNs” and “duplicate discarding” will be necessary depending on the solution for dynamic PTM PTP switch. |
| QC | We think common PDCP with 2 RLC legs (one RLC leg for Multicast and other RLC leg for unicast can be considered). Security is FFS based on SA3 progress and all other functions needed. From R2-2006793 |
| CATT | Basic functions like PDCP SN maintenance should be supported. Functions like for split bearer is not needed. |
| Huawei, HiSilicon | It seems that any functions involving downlink, like“maintenance of PDCP SNs” and “duplicate discarding” should be supported. |
| Spreadtrum | The PDCP functions in NR should be the baseline for MBS. |
| LG | As commented in Q6, PDCP feedback and retransmission is needed. Packet loss received by PTM RLC entity is detected by PDCP entity and uplink feedback is transmitted via PTP RLC entity. Retransmission is received by PTP RLC entity. |
| CMCC | We prefer to align with the legacy functions as much as possible, at least maintenance of PDCP SNs and duplication related functions. |
| Nokia | An easier question to answer would have been which functions are not needed :) |
| Sony | NR PDCP should be the baseline |

- 2.2.3 RLC functions

For RLC, there are three transmission modes:

- Acknowledged Mode (AM);

- Unacknowledged Mode (UM); and

- Transparent Mode (TM).

It is assumed that the functions in each transmission mode should be supported as a whole, so we may not need to discuss each functions as listed in 2.1 one by one.

According to proposals submitted in the last meeting, for PTP and PTM transmission, companies have different views on which RLC transmission mode(s) should be supported. Therefore, companies are expected to provide views on the supported RLC transmission mode(s) for NR MBS for PTP and PTM separately.

Q8: Do companies think that RLC AM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | It should be needed to enable reliable transmission |
| Lenovo, Motorola Mobility | Yes | RLC AM mode is supported for PTP transmission as unicast. |
| OPPO | Yes | it is necessary for reliability transmission. |
| ZTE | Yes | Network decides (AM or UM for PTP transmission) based on QoS requirement. |
| NEC | Yes | It is necessary for reliability transmission. |
| Samsung | Yes |  |
| Kyocera | Yes | We think the PTP transmission should be the same with the current unicast from the RLC point of view. |
| QC | Yes |  |
| CATT | Yes | Support RLC AM for PTP transmission will be simple by reusing legacy unicast function |
| Huawei, HiSilicon | Yes | There should be no specification impact to support this function. |
| Spreadtrum | Yes | It is necessary to achieve the reliability transmission. |
| LG | Yes |  |
| CMCC | Yes | It helps to the reliability transmission. |
| Nokia | Yes |  |
| Sony | Yes |  |

Q9: Do companies think that RLC AM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No | It would be too much complicated to maintain the PTM Tx windows for RLC |
| Lenovo, Motorola Mobility | No | If RLC AM is used for a PTM transmission, how to transmit the RLC status report and retransmission data should be addressed. One possible way is to transmit the RLC status report and retransmission data in a PTP way, which seems a little bit complicated. Since there is already HARQ function in lower layer and if there is higher requirement the PTM transmission can be switched to PTP transmission. The RLC AM for PTM transmission is not needed. |
| OPPO | Maybe no | If support, the RLC should be enhanced, e.g. Tx window and Rx window maintenance. |
| ZTE | No | Considering we will have - PDCP layer reliability (as explained above in PDCP) and - L1 HARQ, and - mode switching to RLC AM based PTP transmision if it has to, we suggest not touching RLC AM based PTM transmission which potentially complicates the design in both NW and UE. |
| NEC | Yes | If reliability is only guaranteed for PTP, this WI doesn’t make any progressive enhancement compared to LTE MBMS/SC-PTM. Some scenarios like V2X, IOT transmission, PTM is mandated and reliability requirement is necessary. A complementary unicast channel can be established for every UE for the transmission of RLC status report and the re-transmission. |
| Samsung | No |  |
| Kyocera | Yes | We think the AM mode for PTM transmission will offer the option for more reliable PTM delivery. |
| QC | Yes | Main goal of Multicast is to support a similar user experience as if the traffic was mapped to unicast. We should support improved radio efficiency and also meet various high reliability application requirements (Ex: s/w download etc). We think HARQ reliability is not sufficient (e.g. due to NAK->ACK errors) and RLC AM is needed for applications requiring high reliability as configuration choice. As some companies commented for applications requiring high reliability, PTP with RLC AM can be used. If RLC AM is not supported, these high reliability services cannot served by using PTM and motivation of reliable multicast is gone. PDCP re-transmission based reliability based on PDCP status report mechanism means moving RLC AM reliability functionality one layer above, which is not efficient. As we move re-transmissions from RLC to PDCP, it adds more overhead ,adds more delay and we need to make additional changes to PDCP SR reporting triggers etc, which is not efficient way. |
| CATT | Maybe No | It seems RLC UM is sufficient to meet the reliability requirement of MBS services. For MCPTT service with high reliability requirement(99.9999%), But it is carried in RLC UM mode in SC-PTM. And it seems no critical issues found. Besides, complexity of introducing RLC AM for PTM should also be considered, the channel for UL feedback and the channel for retransmission need further. Therefore, to avoid over design and increase of complexity, no RLC AM for PTM transmission is needed if there is no clear requirement. |
| Huawei, HiSilicon | No | First, we agree that Multicast should be supported with a similar performance as unicast. In NR MBS, we will support L1 HARQ and repetition, which can basically support reliability requirement of the MBS services in most cases. Note that in Rel-15/16 even for URLLC services such as V2X, physical layer itself which relies on retransmission and repetition can already meet the requirement of ultra-high reliability. In Rel-15, when defining packet duplication for URLLC, the baseline was to support two RLC legs with RLC UM, and RLC AM was agreed only at the very end as it has few specification impacts. Basically we think RLC AM will not bring much gain, and if introducing it for PTM, RLC functions will have to be modified significantly. Note that different from PDCP which only specify UE’s behaviours, RLC should specify both the transmitting and receiving behaviours. If there is a need to further improve reliability in some cases, it can be considered to extend PDCP status reporting to normal transmission for simplicity. |
| Spreadtrum | Maybe no | The complexity of introducing RLC AM for PTM should be considered carefully. |
| LG | No | We think that it is difficult to make PTM receive feedbacks from a set of UEs, which are in different situations and send different feedbacks. Furthermore, to retransmit accordingly will make PTM very complicated. We are negative in supporting RLC AM for PTM. |
| CMCC | Maybe no | It could be complicated to design RLC AM for PTM transmission. And one potential drawback is the delay could be large, which is not suitable for some scenarios. |
| Nokia | No | Would be very complex to operate. |
| Sony |  | We prefer if MBS design is kept closer to the unicast and in this case, have no strong opinion and would be interesting to see the performance gain and complexity analysis of supporting RLC-AM further. |

Q10: Do companies think that RLC UM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes | It should be needed dependent on the characteristics of the MBS RB. |
| Lenovo, Motorola Mobility | Yes | According to the QoS requirements, RLC UM may be configured for the PTP of NR MBS. |
| OPPO | Yes |  |
| ZTE | Yes | Network decides (AM or UM for PTP transmission) based on QoS requirement. |
| NEC | Yes |  |
| Samsung | Yes |  |
| Kyocera | Yes | We think the PTP transmission should be the same with the current unicast from the RLC point of view. |
| QC | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| Sony | Yes |  |

Q11: Do companies think that RLC UM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Lenovo, Motorola Mobility | Yes | Segmentation function is beneficial. |
| OPPO | Yes |  |
| ZTE | Yes | RLC UM only for PTM transmission is preferred as explained above. |
| NEC | Yes |  |
| Samsung | Yes |  |
| Kyocera | Yes | We think the UM mode is the baseline for PTM transmission since it’s same with eMBMS. |
| QC | Yes | Both RLC AM and UM as configuration choice to be supported for PTM. |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| Sony | Yes |  |

Q12: Do companies think that RLC TM is supported for PTP transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | No | Segmentation function is beneficial and needed. |
| OPPO | NO |  |
| ZTE | No |  |
| NEC | No |  |
| Samsung | No |  |
| Kyocera | No | We don’t think the TM mode is used for MBS data transmission. |
| QC | No | There is no segmentation with TM mode and is not efficient for scheduling as well. |
| CATT | No | No clear MBS use cases for RLC TM mode. |
| Huawei, HiSilicon | No |  |
| Spreadtrum | No |  |
| LG | Yes | The difference between RLC UM and RLC TM is whether segmentation is supported or not. For RLC UM, one byte header is always included for the provision of potential segmentation. However, in some MBS service packet size may be small so that segmentation is never needed. In this case, using RLC TM is beneficial. Therefore, RLC TM should be allowed for MBS service. |
| CMCC | No |  |
| Nokia | No |  |
| Sony | No | This seems like agreeing a restriction |

Q13: Do companies think that RLC TM is supported for PTM transmission of NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | No | Segmentation function is beneficial and needed. |
| OPPO | No |  |
| ZTE | No |  |
| NEC | No |  |
| Samsung | No |  |
| Kyocera | No | We don’t think the TM mode is used for MBS data transmission. |
| QC | No |  |
| CATT | No | No clear MBS use cases for RLC TM mode. |
| Huawei, HiSilicon | No |  |
| Spreadtrum | No |  |
| LG | Yes | Please find our comment of Q12. |
| CMCC | No |  |
| Nokia | No |  |
| Sony | No |  |

- 2.2.4 MAC functions

Some of the MAC functions as listed in 2.1 are for UL only, e.g. scheduling information reporting and logical channel prioritisation, which do not need to be discussed for NR MBS. Some functions should be naturally supported in MAC, e.g. mapping between logical channels and transport channels. According to the proposals submitted to RAN2#111-E, the functions that deserve to be specifically discussed here are as follows:

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

Whether to allow multiplexing of MAC SDUs from different logical channels onto a same TB is also related to discussion in 2.2.1 about QoS flow mapping, as well as whether different logical channels can be linked to the same G-RNTI. Companies are invited to provide views on the following question:

Q14: Do companies think that multiplexing/de-multiplexing of different logical channels should be supported in MAC for NR MBS?

| Company | Answer | Comments |
| --- | --- | --- |
| MediaTek | No | We assume there is no need to link different logical channels to the same G-RNTI. One UE interested in only one MBS traffic (carried by one logical chanel) needs to decode a large MAC PDU if multiplexing of MAC SDUs from different logical channels are supported during TB assembly, which is suboptimal. |
| Lenovo, Motorola Mobility | Yes or No | It depends on QoS flow to DRB mapping and G-RNTI allocation. If there are multiple DRB/logical channels linked to the same G-RNTI, it is beneficial to support multiplexing of MAC SDUs of the logical channels. Multiplexing of MAC SDUs of PTM and PTP transmission is FFS. Multiplexing of MAC SDUs of PTM with different G-RNTIs is FFS. |
| OPPO | Not sure | It is up to the mapping relationship among MBS session, MRB and G-RNTI. |
| ZTE | See comments. | For PTM transmission, multiplexing among logical channels associated with same MBS service, if there are any, shall be allowed to enable flexible scheduling. For PTM transmission, multiplexing among logical channels associated with different MBS service shall NOT be allowed from both UE power consumption and spec impacts perspective. That is to say, one G-RNTI/transport block corresponds to one MBS/MBS session. For PTP transmission, it is per UE and the multiplexing/de-multiplexing of different LCHs shall be supported. |
| NEC | No | Different logical channels should be mapped different G-RNTI. |
| Samsung | Yes/No | When a UE is interested in multiple flows, multiplexing may be beneficial. But it depends on QF to RB mapping discussion. Note that we think only data from PTM bearer should be transmitted by G-RNTI. PTP bearer should be treated as a unicast bearer. |
| Kyocera | Yes | We think at least the multiplexing/de-multiplexing will be needed for “PTP-leg” in dynamic PTM PTP switch since we assume it uses C-RNTI. For PTM transmission, the rapporteur pointed out it’s related to QoS flow mapping and G-RNTI assignment. In LTE SC-PTM, a specific MBMS session (TMGI and optionally Session ID) is associated with a G-RNTI. So, in NR MBS we think the different logical channels can be linked with a G-RNTI as long as these logical channels are associated with a specific MBS session. |
| QC | Yes | For certain applications like IIoT, where different set of UEs may be subscribed to different services (different services can be mapped to different LCIDs) can be multiplexed into same TB and scheduled using same G-RNTI. This avoids UEs subscribed to multiple services (mapped to different LCIDs) are not required to monitor multiple G-RNTIs which will impact UE power consumption. From specification perspective, we think it should be allowed to support multiplexing of different LCIDs into same TB and each LCID to be mapped to a single TB as well. |
| CATT | Yes | As discussed in 2.2.1. One or more QoS flows may be used within a single MBS session. logical channels carrying data from different MBS flows (mapping to different MRBs) of one MBS session could be multiplexed in MAC,in case there is one to one mapping between MBS session and G-RNTI. |
| Huawei, HiSilicon | Yes | Agree with Kyocera’s analysis. What Qualcomm pointed out could be another use case, which would allow the network to configure logical channels of different MBS sessions to use the same G-RNTI. |
| Spreadtrum | Not sure | It should be guaranteed that UE can only receive the service data it interested in to avoid the resource waste and unnecessary power consumption. |
| LG | No | Logical channel can be identified by using G-RNTI. So there is no need of multiplexing. |
| CMCC | Not sure | In the last RAN1 meeting, it was agreed that: For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. o FFS: whether to support UE-specific PDCCH to schedule a PDSCH for MBS. If UE-specific PDCCH to schedule a PDSCH for MBS, it’s naturally to support multiplexing/de-multiplexing of different logical channels. Maybe we can wait for RAN1’s progress on this FFS. And for G-RNTI scheduling, we share similar view with other companies that it depends on the mapping relationship among MBS session, MRB and G-RNTI. |
| Nokia | Yes | We do not see a reason to exclude it. |
| Sony | No | We think that a logical channel should be associated with a unique G-RNTI. However, one logical channel may carry multiplexed QoS flows. |

- 2.3 Dynamic switch between PTP and PTM

According to the WID [1], dynamic switch between PTP and PTM is a function to be supported for NR MBS. During the 111-e meeting, it is agreed that the gNB makes the decision of the dynamic switch, while which sublayer handles the details is FFS. Companies are invited to give answers to the following question:

Q15: Which sublayer acts as the anchor of PTP and PTM dynamic switch, i.e. the splitting and converging sublayer for MBS traffic transmitted via PTP and PTM?

| Company | Answer (e.g. SDAP/PDCP/RLC/MAC) | Comments |
| --- | --- | --- |
| MediaTek | PDCP or RLC | We assume the discussion of PTP and PTM dynamic switch can be subject to the modelling of the MBS MRB. So far we see two alternatives, one is PDCP based and the other is RLC based. So accordingly, the PTP and PTM dynamic switch can be conducted at either PDCP or RLC layer. |
| Lenovo, Motorola Mobility | PDCP | In order to support service continuity, a common sequence numbering function is needed between PTM and PTP. As MR-DC, it is straight forward to use a common PDCP layer for both PTM and PTP modes since the re-ordering function resided in PDCP layer in NR. In this layer 2 protocol stacks, PTP transmission with RLC AM can be configured to improve the reliability. |
| OPPO | PDCP and MAC | - The common PDCP will make the same packet own the SN number of PTM and PTP. It will help to increase the reliability reception. - The common MAC has benefit of soft combination of PTM and PTP. It will also help to increase the reliability reception. If PTP is only for retransmission of PTM, the common MAC is necessary. |
| ZTE | PDCP | PDCP as the anchor layer naturally fits into the current RAN architecture design with CU/DU split and bearer type change design; while PDCP as the anchor layer introduces least spec impacts in lossless support compared to other solutions. We suggest separating mode switching and re-transmission schemes in future discussion, although they share some kind of commonality. For mode switching, only one anchor layer is needed, while for retransmission it can be studied in Reliability Enhancement part. |
| NEC | PDCP | For service continuity and DC, it is always the PDCP entity which acts like an anchor. |
| Samsung | No need (RRC switching) | We do not think “dynamic switch” mandates a common sublayer. We think dynamic switch can be accomplished by RRC message indicating switch between PTM and PTP. In this case, split structure is not needed. Under the split bearer Lenovo is referring to, PDCP SN for all UEs receiving the same MBS bearer should be synchronized, even if UE receives the data via PTP bearer. It does not have a value. We think lossless can be achieved by classical AM unicast bearer. Such complicated structure is not necessary. |
| Kyocera | PDCP (or RLC) | We think it’s straightforward to reuse the existing split bearer functions in PDCP for MBS PTP PTM switch. If RLC AM is supported, we think it may also be possible to do this at the RLC layer, but there will likely be more specification impact. |
| QC | PDCP is main anchor and RLC/MAC dynamic re-transmissions to be allowed. | From above architecture, PDCP can be used as anchor for switching between PTP and PTM legs. One RLC leg is meant for Multicast, which can operate in AM/UM mode based on configuration and other RLC leg is meant for specific UE, which can be configured in either AM/UM modes. Multicast RLC AM leg can re-transmit either by using multicast LCID or unicast LCID for a specific UE. At PHY/MAC, for multicast leg, HARQ original transmission can be based on G-RNTI and re-transmission can be either by using G-RNTI or C-RNTI and is dynamically decided by GNB based on whether re-transmissions targeted for one UE or multiple UEs. In cased on PTP leg, original and re-transmissions are based on C-RNTI. This gives flexibility at different levels. RRC based switching needed during handover cases. |
| CATT | SDAP,or PDCP or RLC | Not sure about the detailed function of the “anchor”. We are open to both common PDCP and separate PDCP. Either solution has both advantages and disadvantages. It seems common PDCP entity is beneficial for reordering handling during mode switch .but with common PDCP, PTM and PTP of the same MBS session should have the same PDCP configuration. This will put a restriction on the PDCP configuration of PTP transmission. so the question is, for a MBS session, do we assume that the same PDCP functions or configuration are to be used for PTM and PTP? if no, maybe common PDCP is not helpful to the reordering during mode switch because anyway the PDCP will be reconfigured during switching. For separate PDCP solution, no such limitation, but SN synchronization between PDCPs needs to be maintained between 2 PDCP entities for reordering. |
| Huawei, HiSilicon | PDCP | Agree with ZTE’s view that dynamic switch should be decoupled with the discussion on retransmission which can be performed at MAC or RLC. Dynamic switch between PTP and PTM will almost inevitably lead to out-of-order reception of MBS data, which makes PDCP the most suitable anchor entity as reordering function is at PDCP in NR. Besides, PDCP acting as the splitting and converging sublayer is much similar to the current split bearer architecture and thus can save a lot of specs effort. |
| Spreadtrum | PDCP | Considering the service continuity during the switch PTP and PTM, the duplication detection and reordering function is needed. The PDCP layer has these functions. Therefore, it is natural for PDCP layer to act as anchor layer. And we agree with CATT that the whether common PDCP or separate PDCP is applied needed further discussions. |
| LG | PDCP | The PDCP sublayer should be the anchor point of PTM and PTM dynamic switch. The PDCP entity is associated with a PTM RLC entity and a PTP RLC entity. |
| CMCC | PDCP and/or MAC | - Using PDCP as the anchor also suits for the mobility scenario, it is benefit for the service continuity. Also, in the discussion above, the RLC mode for PTM and PTP leg could be different, which could be achieved by common PDCP. - As we mentioned in Q14, RAN1 is discussing about using C-RNTI to schedule a PDSCH for MBS, which means PTM leg could be scheduled by C-RNTI or G-RNTI. If it’s supported, there could be a PTM leg and PTP leg in PHY naturally. Maybe we need to wait RAN1’s progress or at least reach a consensus on the basic definition of PTP leg and PTM leg. |
| Nokia | PDCP | Service continuity will rely on SN and PDCP is an obvious choice. Note that if duplication is required, it would also be located at PDCP. |
| Sony | PDCP |  |

- 2.4 Phase-2 discussion: L2 architecture for MBS

(To be discussed in Phase II of this email discussion)

# Conclusion

# Reference

- RP-201038 WID revision: NR Multicast and Broadcast Services, Huawei, HiSilicon