3GPP TSG-RAN WG2 Meeting #112-e........................................................... R2-20XXXX

- Electronic Meeting, 2-13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | TBD | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for Unaligned CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-10-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conclusion from RAN #89 (RP-202030), the Rel-16 inter-band CA with non-aligned frame boundaries support the following cases - Case A: Within the same cell group, the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for PCell/PSCell is smaller than or equal to the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for each of the Scells - Case B: Within the same cell group, the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for PCell/PSCell is larger than the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for at least one of the Scells The CR proposes to add the capability for case B. In addition, the ambiguities in SMTC window in async CA should be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In IE SS-RSSI-Measurement, clarify that only complete slots inside the SMTC window are indicated by the bitmap in measurementSlots in case of intra- frequency measurement. - In 6.3.3, add the ASN.1 definition for case B. Impact analysis: Impacted 5G architecture options: Standalone, EN-DC, NGEN-DC, NR-DC Impacted functionality: Inter-band CA with non-aligned frame boundaries Inter-operability: If the UE implements the CR but the network does not or vice versa, the case B of non-aligned CA is not supported and it is unclear whether the partial overlap slot in SMTC window is used for RSSI measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether a partially overlapped slot with SMTC window should be treated as “a slot in the SMTC window”, in case of intra-frequency measurement of Rel-16 async CA; And the UE capability for Case B is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.306 CRxxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

### 6.3.2	Radio resource control information elements

/************************************************************No change omitting**************************************************************/

- –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement is used to configure RSSI measurements based on synchronization reference signals.

SS-RSSI-Measurement information element

-- ASN1START

-- TAG-SS-RSSI-MEASUREMENT-START

SS-RSSI-Measurement ::=             SEQUENCE {

measurementSlots                    BIT STRING (SIZE (1..80)),

endSymbol                           INTEGER(0..3)

}

-- TAG-SS-RSSI-MEASUREMENT-STOP

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.3-1 in TS 38.215 [9], which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the duration and by the subcarrierSpacing). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1. In case this field is configured for a SCell with ca-SlotOffset-r16, the bits in the bitmap corresponds to the slots that are fully contained in the SMTC window.

### 6.3.3	UE capability information elements

/************************************************************No change omitting**************************************************************/

- –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombinationList-v1610 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610

BandCombinationList-v16xy ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16xy

BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1610 ::=          SEQUENCE {

bandList-v1610                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610  OPTIONAL,

ca-ParametersNR-v1610               CA-ParametersNR-v1610                  OPTIONAL,

ca-ParametersNRDC-v1610             CA-ParametersNRDC-v1610                OPTIONAL,

powerClass-v1610                    ENUMERATED {pc1dot5}                   OPTIONAL

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

... ,

[[

bandCombination-v16xy               BandCombination-v16xy                      OPTIONAL

]]

}

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1610 ::=         SEQUENCE {

srs-TxSwitch-v1610               SEQUENCE {

supportedSRS-TxPortSwitch-v1610  ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL,

intraFreqDAPS-Parameters-r16      SEQUENCE {

intraFreqDiffSCS-DAPS-r16                        ENUMERATED {supported}    OPTIONAL,

intraFreqDAPS-r16                                ENUMERATED {supported}    OPTIONAL,

intraFreqAsyncDAPS-r16                           ENUMERATED {supported}    OPTIONAL,

intraFreqMultiUL-TransmissionDAPS-r16            ENUMERATED {supported}    OPTIONAL,

intraFreqTwoTAGs-DAPS-r16                        ENUMERATED {supported}    OPTIONAL,

intraFreqSemiStaticPowerSharingDAPS-Mode1-r16    ENUMERATED {supported}    OPTIONAL,

intraFreqSemiStaticPowerSharingDAPS-Mode2-r16    ENUMERATED {supported}    OPTIONAL,

intraFreqDynamicPowersharingDAPS-r16             ENUMERATED {short, long}  OPTIONAL

}                                                                              OPTIONAL

}

BandCombination-v16xy::=            SEQUENCE {

ca-ParametersNR-v16xy                   CA-ParametersNR-v16xy                  OPTIONAL

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

| BandCombination field descriptions |  |
| --- | --- |
| BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590, BandCombinationList-r16, BandCombinationList-v16xy The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix). If the field is included in supportedBandCombinationListNEDC-Only-v1610, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList of supportedBandCombinationListNEDC-Only (without suffix) field. If the field is included in supportedBandCombinationListNEDC-Only-v15a0, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix) of supportedBandCombinationListNEDC-Only (without suffix) field. |  |
| ca-ParametersNRDC If the field is included for a band combination in the NR capability container, the field indicates support of NR-DC. Otherwise, the field is absent. |  |
| ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent. |  |
| srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on |  |
| srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on |  |
| srs-TxSwitch Indicates supported SRS antenna switch capability for the associated band. If the UE indicates support of SRS-SwitchingTimeNR, the UE is allowed to set this field for a band with associated FeatureSetUplinkId set to 0 for SRS carrier switching. |  |

/************************************************************No change omitting**************************************************************/

- –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)         OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)        OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}                OPTIONAL

}

CA-ParametersNR-v1610 ::=           SEQUENCE {

-- R1 9-3: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA

parallelTxMsgA-SRS-PUCCH-PUSCH-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 9-4: MsgA operation in a band combination including SUL

msgA-SUL-r16                                      ENUMERATED {supported}            OPTIONAL,

-- R1 10-9c: Joint search space group switching across multiple cells

jointSearchSpaceGroupSwitchingAcrossCells-r16     ENUMERATED {supported}            OPTIONAL,

-- R1 14-5: Half-duplex UE behaviour in TDD CA for same SCS

half-DuplexTDD-CA-SameSCS-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 18-4: SCell dormancy within active time

scellDormancyWithinActiveTime-r16                 ENUMERATED {supported}            OPTIONAL,

-- R1 18-4a: SCell dormancy outside active time

scellDormancyOutsideActiveTime-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 18-6: Cross-carrier A-CSI RS triggering with different SCS

crossCarrierA-CSI-trigDiffSCS-r16                 ENUMERATED {higherA-CSI-SCS,lowerA-CSI-SCS,both}    OPTIONAL,

-- R1 18-6a: Default QCL assumption for cross-carrier A-CSI-RS triggering

defaultQCL-CrossCarrierA-CSI-Trig-r16             ENUMERATED {supported}            OPTIONAL,

-- R1 18-7: CA with non-aligned frame boundaries for inter-band CA

interCA-NonAlignedFrame-r16                       ENUMERATED {supported}            OPTIONAL,

simul-SRS-Trans-InterBandCA-r16                   INTEGER (1..2)                    OPTIONAL,

daps-Parameters-r16                   SEQUENCE {

asyncDAPS-r16                           ENUMERATED {supported}                  OPTIONAL,

interFreqDAPS-r16                       ENUMERATED {supported}                  OPTIONAL,

interFreqDiffSCS-DAPS-r16               ENUMERATED {supported}                  OPTIONAL,

multiUL-TransmissionDAPS-r16            ENUMERATED {supported}                  OPTIONAL,

semiStaticPowerSharingDAPS-Mode1-r16    ENUMERATED {supported}                  OPTIONAL,

semiStaticPowerSharingDAPS-Mode2-r16    ENUMERATED {supported}                  OPTIONAL,

dynamicPowersharingDAPS-r16             ENUMERATED {short, long}                OPTIONAL,

ul-TransCancellationDAPS-r16            ENUMERATED {supported}                  OPTIONAL

}                                                                                   OPTIONAL,

codebookParametersPerBC-r16           CodebookParameters-v1610                      OPTIONAL

}

CA-ParametersNR-v16xy ::=           SEQUENCE {

-- Case B: The lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for PCell/PSCell is larger than the lowest subcarrier spacing of the subcarrier spacings given in scs-SpecificCarrierList for each of the Scells

interCA-NonAlignedFrame-B-r16                       ENUMERATED {supported}            OPTIONAL

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

/************************************************************No change omitting**************************************************************/

- –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                   SEQUENCE {

supportedBandListNR                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v1610  BandCombinationList-v1610                   OPTIONAL,

supportedBandCombinationListSidelink-r16  BandCombinationListSidelink-r16       OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16  BandCombinationList-UplinkTxSwitch-r16 OPTIONAL

]] ,

[[

supportedBandCombinationList-v16xy  BandCombinationList-v16xy                   OPTIONAL

]]

}

- << End of change >>