3GPP TSG-RAN WG2#112-e	R2-200xxxx

Online, November 2 - 13, 2020

Source: 	Huawei

Title: 	[Post111-e][919][eDCCA] Efficient activation deactivation of SCG Discussion on SCG deactivation and activation

Agenda Item:	x.x.x

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

- [Post111-e][919][eDCCA] Efficient activation deactivation of SCG (Huawei)

Scope: a) Cover common grounds, e.g. find “easy” agreements, and items that might need limited discussion. b) Identify key performance trade-off decisions that will need to be taken.

Intended outcome: Report to next meeting

Deadline: Long

RAN2 had initial discussions on the WI “Further Multi-RAT Dual-Connectivity enhancements” with the following conclusions:

FFS how signalling and inter-node interaction works at activation deactivation (e.g. MN triggered, SN triggered, UE triggered, signalling mechanism, which node is in control etc) FFS if for deactived SCG, the UE stop monitoring PDCCH for PSCell and SCells of the SCG FFS if For the PSCell in deactivated SCG, the UE performs CSI/RRM measurement and report; AGC; beam management; RLM

# 2	Discussion

## 2.1	Previous Rel-16 discussions

The following agreements were reached during Rel-16 discussions:

- The UE supports network-controlled suspension of the SCG in RRC_CONNECTED.

- UE behaviour for a suspended SCG is FFS

- The UE supports at most one SCG configuration, suspended or not suspended, in Rel16.

- In RRC_CONNECTED upon addition of the SCG, the SCG can be either suspended or not suspended by configuration.

The two first agreements are somehow covered by the WID but it can be checked whether the twoother agreements can be confirmed.

Q1: Do companies agree that the UE supports at most one SCG configuration, suspended or not suspended?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | The only one SCG can be the baseline for SCG suspension. But considering the benefit of SCG suspension and UE mobility, more than one SCG can be considered in SCG suspension topic. |
| MediaTek | Yes | We prefer to follow the R16 agreement. So far, it is not clear to us whether multiple suspended SCG configuration will be beneficial. |
| Nokia | Yes (at least as baseline) | At least as baseline only one concurrent SCG configuration needs to be supported. If there is lots of time after completing this behaviour we can consider more complex scenarios. BTW, should we use terminology deactivated here as well based on WI description? |
| NEC | Yes | our understanding of the text below in the WID is to clarify only one SCG regardless its state (i.e. activated or deactivated) after extensive discussions in RANP. No need to open this question again in Rel-17. - Support efficient activation/de-activation mechanism for one SCG and SCells - Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4] On terminology, we agree that “deactivated” at least for now. |
| Samsung | Yes | Agree UE is configured with at most one SCG, that can be deactivated or not |
| Ericsson | Yes | For Rel-17 we can start with this assumption. |
| Apple | Yes | We think this is already present in the WI description although ‘activation/de-activation’ is used instead of suspension. Same view as NEC |
| Intel | Yes (but open) | We would like to prioritize determining UE behavior and signalling when SCG is suspended while MCG is active. After then, we can further consider whether more than one SCG configuration is necessary. |
| LG | Yes | We prefer to discuss a simple scenario firstly i.e. single SCG configuration regardless of suspension. More complicated scenarios i.e. can be discussed after discussion of the single SCG configuration. |
| KDDI | Yes | Share the comments with other companies, we are fine to discuss additional scenario if there is more time |
| Sharp | Yes |  |
| Lenovo and Motorola Mobility | Yes | Take Rel16 agreements as start point. |
| Spreadtrum | Yes | It is the baseline for R17 discussion. |
| DOCOMO | Yes | we are also fine to discuss additional scenario if time allows |
| Interdigital | Yes | Priority is to have the SCG suspension/activation behaviour specified. Agree with other companies above that if more than one SCG can be considered can be discussed later. |
| CMCC | Open | In our understanding, only one SCG can be activated. So one SCG configuration can be discussed first. However, we can have more de-activated SCGs prepared for the scenarios like mobility. For the terminology, we prefer activation/de-activation which is used in the WID. |
| vivo | Yes | The UE behavior when SCG is deactivated and the corresponding control signalling need to be first discussed. |
| Qualcomm | Yes | We prefer the SCG activated/deactivated terminology since it is used in the WID. |
| Futurewei | Yes | It is can be the baseline for R17. |
| ZTE | Yes | We think it is beneficial to support multiple SCGs. However, considering the limited time in Rel-17, it can be discussed with low priority if time allows. In addition, we also prefer to use SCG activated/deactivated terminologies same as in the WID. |
| CATT | Yes | The UE can be provided with more than one SCG for CPAC. However the UE can only support at most one activated SCG and the UE should only be able to perform CPC while there is an active SCG. From this point of view, there is at most one activated or suspended SCG at a time. Considering the possible UE behaviour for suspended SCG, e.g. CSI/RRM measurement and report; AGC; beam management; RLM, more than one SCG configuration to be suspended or activation will introduced the UE complexity. |

Q2: Do companies agree that in RRC_CONNECTED upon addition of the SCG, the SCG can be either suspended or not suspended by configuration?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | Yes | The SCG can be either suspended or not suspended by RRC configuration upon addition of the SCG in RRC_CONNECTED. |
| MediaTek | Yes |  |
| Nokia | Yes (partially) | “not suspended” is rel-16 behaviour so at least that is supported. Whether it can be directly suspended can be discussed further once we know what “suspended” is. |
| NEC | No (preference) | To make it clear, the agreements listed above had been agreed as “assumptions” and not confirmed due to following agreement that RAN2 does not progress this topic. We feel that “SCG is suspended upon addition” (= directly suspended) may be over specification for unclear benefit at least for PSCell. In addition, in Rel-17, RAN2 is going to introduce the CPA, which should be sufficient for quick configuration (activation) of SCG upon necessity. For example, we understand that “SCG is suspended upon addition” means the UL synchronization is NOT performed yet. In other words, if it is assumed that the UL synchronization is to be done upon SCG addition, the SCG can be considered as not-suspended at first and then the SCG is somehow suspended. With this understanding, the SCG activation delay will not be shortened so much compared with the CPA. Therefore, if companies want to support the SCG suspended upon addition, it should be clarified what it really means and in which scenario this is beneficial with considering UE’s mobility (e.g. when activating later, UE may be far away from PSCell). |
| Samsung | Yes | We assume that in general it should be possible for network to control SCG deactivation state i.e. would not really require additional changes. |
| Ericsson | Yes |  |
| Apple | Yes partially | Similar views as Nokia, and deactivated at configuration should be possible (would depend on how we model the activation/deactivation). |
| Intel | Yes |  |
| LG | No | Basically we think this is stage-3 detail to be discussed later, and the definition of deactivated SCG should be clarified first. Moreover, we do not see meaningful gain of supporting deactivation upon addition of CG. |
| KDDI | Yes | Share the comments with other companies |
| Sharp | Yes |  |
| Lenovo and Motorola Mobility | Yes |  |
| Spreadtrum | Yes | It can be controlled by the network. |
| DOCOMO | Yes |  |
| Interdigital | Yes (but no strong view) |  |
| CMCC | Yes | Deactivation upon addition of the SCG can be the network decision and will decrease the delay when the SCG is needed to be activated. |
| vivo | Yes | It should be possible for network to deactivate the SCG upon addition of SCG. |
| Qualcomm | Yes |  |
| Futurewei | Yes | Pre-configure a deactivated SCG allows the network to activate it quickly if needed. |
| ZTE | Yes |  |
| CATT | Yes (partially) | Stage -3 details can be discussed later when the stage-2 level procedure is clear. For RRC signalling point of view, suspend/not suspend at SCG addition can be supported. However, benefit of SCG suspension at SCG addition is not very clear. The issue should be discussed for both SCG addition and SCG change. |

## 2.2	UE behaviour

One key aspect is the UE behaviour while the SCG is deactivated. Here, the discussion is about the PSCell only.

### 2.2.1	PDCCH Monitoring

PDCCH is the main fast control mechanism but also a major source of power consumption. There seems to be no company proposing that the UE monitors PDCCH on the PSCell while the SCG is deactivated.

Q3: Do companies agree that the UE does not monitor PDCCH on the PSCell when the SCG is deactivated?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | Yes | It is obvious agreement. |
| MediaTek | Yes |  |
| Nokia | Yes | In order to get some benefits this seems to be natural |
| NEC | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes | We agree, but we need to discuss the consequences of that. The primary thinking was that SCG would not schedule data, which makes sense. However, we do believe that activation of a deactivated SCG needs to be faster than SCG addition. Otherwise there is no benefit of deactivating the SCG. Thus, the UE should be as ready as possible to re-start PDCCH monitoring upon SCG activation and we need to investigate possibilities for RLM, beam failure detection, beam management, TCI state updates, etc even with absence of PDCCH reception in deactivated SCG. More on this in below comments. |
| Apple | Atleast not need to monitor as frequently as in activated state. | While we agree that UE does not need to monitor PDCCH atleast as frequent as the case when the SCG is activated, agreeing to “does not need to monitor” at all in deactivated state is a big statement. Need to discuss on complete absence of monitoring or not. |
| Intel | Yes |  |
| LG | Yes |  |
| KDDI | Yes | Share the comment with Ericsson, we are also open to discuss fast activation mechanism |
| BT | Yes but | We agree it makes no sense the UE monitors the PDCCH when the SCG is deactivated but the SCG activation should be more efficient than the SCG addition. |
| Sharp | Yes |  |
| Lenovo and Motorola Mobility | Yes |  |
| Spreadtrum | Yes |  |
| DOCOMO | Yes |  |
| Interdigital | Yes | Not performing PDCCH monitoring is the main area where UE power savings gains are achieved. |
| CMCC | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes | This is the main power saving mechanism in SCG deactivated state. |
| Futurewei | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |

### 2.2.2	RRM measurements

RRM measurements: are based on serving cell MO (i.e. performed regardless of measIDs) and on measID. Reports could be provided to the SN via the MN, as is already the case when there is no SRB3. They can allow mobility and, for serving cells, provide some hints about best beams (which might be used e.g. to allocate RACH resources to access the cells again). If there is any RRM measurement configured not for the purpose of mobility (e.g. CGI reporting, positioning), it may be better to stop it while the SCG is deactivated. Regarding measurements for mobility, dropping or modifying some of them when the SCG is deactivated could save power but at the cost of reduced mobility performance.

There seems to be many companies thinking that the UE should perform RRM measurements based on SCG serving and neighbour cells while the SCG is deactivated.

Q4: Do companies want that the UE performs RRM measurements based on SCG serving and neighbour cells while the SCG is deactivated?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO |  | The common understanding that the UE will perform the RRM based on the measurement configuration from MN side. For SCG deactivation case, we think there are 4 options to define the RRM for SCG. Option 1: continue performing RRM based on the measurement configuration from SN if possible. Option 2: continue performing RRM based on SCG serving frequencies if possible. Option 3: continue performing RRM based on SCG serving cells if possible. Option 4: continue performing RRM based on frequency based on the frequency configuration from MN SIB 4. We prefer option 2 and 4. |
| MediaTek | Yes | We consider that the suspend PSCell is still a configured servicing cell, and thus we would prefer to continue the RRM (configured by MN and SN) to maintain the basic SCG mobility. |
| Nokia | Yes | At least some level of RRM measurements are needed for serving cel (which PSCell is). Then details can be discussed later but as UE most likely can have regular measConfig configured probably this will without too much RAN2 impacts – but of course some optimizations can be considered but regular behaviour should be considered as baseline |
| NEC | Yes, basically | our assumption is no impact to MN configured measurements for SCG (apart from details, e.g. MG aspects). Regarding the SN configured measurements, the UE should keep performing measurements somehow for intra-SN mobility or decision of releasing SCG at all due to moving out of coverage. A question is whether such measurements should be the same as those in activated SCG or can be relaxed e.g. with longer periodicity like measCycleSCell? |
| Samsung | Yes | Some RRM seems required to ensure UE has valid PSCell configuration to be used upon SCG resumption. We note that it is SN handles SCG mobility |
| Ericsson | Yes | We agree to support RRM measurements. Regarding what RRM measurements to support, we need to discuss proposals stepwise, considering what is most important to be supported and possible cost in terms of power consumption, such as: - 1) At least PSCell measurements; - 2) 1+ SCG SCell measurements; - 3) 2+ Neighbours on PSCell frequency. - 4) 3+ Neighbours on all serving frequencies. - 5) 4+ All measurements configured according to measIds of the SCG MeasConfig. Also other approaches relaxing the requirements on RRM measurements can be considered. Beam level reporting could also be considered. |
| Apple | Yes to atleast some RRM measurements | Details need to be discussed on what level of RRM is to be agreed. |
| Intel | Neutral | We think it could be better if RRM for the suspended SCG is configurable from NW, rather than shall or not. |
| LG | Yes | The UE needs to perform RRM to maintain SCG/SCells for fast CG activation. Without RRM on the deactivated SCG, the UE may have invalid SCG/SCells while the SCG is deactivated. Reduction of power consumption on the deactivated SCG may be considered. |
| KDDI | Yes | Pcell obtain the status of Scell via RRM , therefore it is needed to have RRM |
| BT | Yes | We consider RRM measurements may be required for a fast SCG activation and those measurements should be configured by the MN. The UE may inform the network about its preferences for power efficiency. |
| Sharp | Yes | As clarified by Oppo, Option 1 is preferred. RRM based on the measurement configuration from SN should be performed and special handlings (e.g. limitation of MO) are not needed. NW can reconfigure MeasConfig if needed. |
| Lenovo and Motorola Mobility | Yes |  |
| Spreadtrum | Yes | We think the PSCell change related RRM should be considered and it can be configured by MN. |
| DOCOMO | Yes | Agree with LG |
| Interdigital | Yes | RRM, at least on the PSCell and neighbour cells on the PSCell’s frequency, should be supported. |
| CMCC | Yes | RRM is needed for fast activation of valid SCG. Considering the power consumption, we think that specific level of RRM needs further discussion. |
| vivo | Yes | RRM measurements should be supported during SCG deactivation. The measurement on SCG serving cells should be at least performed to avoid the SN to activate an invalid SCG. The measurements on the neighbour cells can be configured by SN, and should not cause too much UE power consumption. |
| Qualcomm | Yes | UE should perform RRM measurements on SCG serving and neighbor cells. Measurements on PSCell and its neighbour frequencies should be mandatory. Measurements on SCG SCells are useful. When data arrives and SCG resources are needed upon activation, network can determine which SCG SCells to activate besides the PSCell. Relaxed RRM measurements, e.g., for stationary UEs, can be performed to further lower UE power consumption. For UEs in RRC_CONNECTED, some RRM relaxation mechanisms are in Rel-17 RAN2 RedCap power saving WI scope, and we can wait for the developments in that WI. |
| Futurewei | Yes | RRM measurements in general are needed for possibility of SCG change and for preparing the activation of the deactivated SCG. |
| ZTE | Yes | To ensure the UE has valid PSCell configuration for fast SCG activation, at least RRM measurements for PSCell change (and SCell change) are needed, and these measurements are configured by SN. We can further discuss how to handle the measurements configured for other purposes. In addition, we think there should be no impact to the RRM measurements configured by MN, but we are open to discuss possible optimizations, e.g. measCycleSCell. |
| CATT | Yes | Considering the mobility, the RRM measurement results can help the NW to make the decision for SN mobility. As for the 4 options mentioned by OPPO, further discussion is needed |

### 2.2.3	Radio link monitoring

Radio link monitoring consists in detecting BLER above 10% or below 2% on a number of periodic RS, lower than the number used for RRM measurements on the PSCell.

For RRM measurements, depending on the DRX cycle length, the measurement period is either 200ms or a multiple of the SMTC period (which is likely the same as the SSB periodicity). For RLM, depending on the DRX cycle length, the period to detect BLER < 2% or BLER > 10% is either 200ms or a multiple of the SSB periodicity. For BLER < 2%, the multiple for RLM is half of the multiple for RRM measurements, for BLER>10% it is the same value (see 38.133 tables 8.1.2.2 and 9.2.5.2).

The arguments that were raised in favour or against performing RLM were:

-	if RLM is not performed, the network may activate a non-functional SCG [11]. Still, what was not discussed is how likely is this to occur, what would be the consequence and whether the network behaviour could not minimise the risk and/or mitigate the consequences. For instance, RRM reports based on A2 or A3 events may already help detecting that the PSCell is no more suitable.

-	if RLM is performed, as indicated in [3], it may be necessary to sometimes update the RS for RLM as their number is limited to 2 or 4 for FR1.This may require some information to reported by the UE, for instance CSI/RS (in case this is reported) or the content of SCGFailureInformation message reported after SCG RLF was detected. The UE behaviour after SCG RLF has occurred while the SCG is deactivated may need to be different from the existing behaviour, so this also has some specification impact. Besides, performing RLM also increases UE power consumption, in a manner that depends on UE implementation. If the UE remains a long time with a deactived SCG, the impact of that additional power consumption will be higher and it may be better to release the SCG.

Q5: Do companies want that the UE performs radio link monitoring on the PSCell while the SCG is deactivated? If your answer is yes, please provide your views on the above-discussed issues (as raised in [3]).

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | There are 3 models to define SCG suspension. Model 1: based on RRC_INACTIVE Model 2: based on dormancy Scell Model 3: based on SCell deactivation Only when model 2 is chosen, then RLM will be supported for SCG suspension. The purpose for SCG suspension is power saving. We wonder how much gain we can get if we only further make the PSCell stay in dormancy compared with R16 dormancy scell mechanism, i.e all the SCells in dormancy. If model 2 is chosen, I wonder how to define the SCG state when a UE configured with MR-DC configuration enter RRC_INACTIVE and the data arrive only on the DRB configured with MCG bearer. It seems it is not necessary to active the SCG due to SCG suspension concept introduced in R17 for power saving purpose. Based on discussion above, we prefer model 1. So the RLM is not supported. |
| MediaTek | No | For suspended SCG, we think that RRM is enough. The NW could understand the quality of PSCell based on the RRM reporting. The RLM increase the power consumption and this is not justified while there is no ongoing data transmission. We prefer no RLM for suspended SCG. |
| Nokia | Maybe | Maybe RRM measurements are sufficient for handling deactivated SCG but as we already now have SCG failure behaviour defined it is worthwhile to consider would it be easier just to support RLM without impacts to specification. But of course also UE power consumption needs to be considered as well but likely that is minimal as same resources can be used for RLM and RRM evaluations. Also already defined UE actions upon RLF may be necessary once SCG gets in very bad radio condition. |
| NEC | No | Given that the SCG can be deactivated, there seems to be no urgency to detect RLF based on RLM which will take more UE power. Only if RAN2 agree not to perform RRM measurement in deactivated SCG, the RLM on PSCell may be useful. |
| Samsung | No | We prefer not to perform RLM in deactivated state and assume RRM should be sufficient to handle UE mobility in normal cases. |
| Ericsson | Yes | RLM and SCG Failure reporting are both beneficial, otherwise network may not detect that a deactivated SCG is not suitable any longer. If this is not supported there is a risk that the network tries to activate an SCG that is not even within the coverage. The result will be SCG RLF after the SCG activation, which would just prolong the procedure for the network to look for a new target cell suitable for SCG operation and adding it. On the other hand, as RLM measurements may increase the UE’s power consumption, means to relax RLM requirements for deactivated SCG can be discussed (to be coordinated with RAN4). |
| Apple | Maybe | It depends on RAN2 deciding on the importance of low latency in getting back to activated state from deactivated SCG. |
| Intel | Neutral | We think it could be better if RLM for the suspended SCG is configurable from NW, rather than shall or not. |
| LG | No | RRM may be sufficient to detect and handle problems of deactivated SCG. RLM would require additional monitoring of RLM-related reference signal, which increase power consumption. |
| KDDI | No | RLM is not mandated in deactive state but we may want to discuss RLM in deactive state as an additional feature to improve the latency for fast activation |
| BT | Maybe | The risk to not perform RLM is to try to activate the SCG when it is no longer available. This functionality should be configured by the network rather than yes or no. The network may require UE feedback to take the decision. |
| Sharp | No | Considering the power consumption of RLM and the intention of SCG deactivation, RLM is preferred not to be applied when SCG is deactivated. |
| Lenovo and Motorola Mobility | No | We tend to agree the benefit of performing RLM can be to some extent achieved by the RRM report. Besides, if RACH procedure is anyway necessary to get time aligned (see Q8) when activating the SCG, the actual link quality can be determined at that time. |
| Spreadtrum | No | When the SCG is deactivated, SCG failure report is not needed. The RRM in MN is enough to ensure whether the PSCell change is necessary or not. |
| DOCOMO | No | We assume that RACH procedure is performed when activating the SCG and RLM is not needed before RACH is performed. |
| Interdigital | Yes | RLM is beneficial in reducing the activation latency of a suspended SCG as it ensures that the UE will not be instructed to activate a PSCell that it is not able to use. Some level of RLM measurement relaxation can be discussed to overcome the possible downside (from power consumption point of view). |
| CMCC | Maybe | Although power saving is one important purpose of this WI, we should increase the reliability of the SCG activation mechanism and try to achieve a better trade-off between the power efficiency and availability of the de-activated SCG. RLM is beneficial for valid activations and measurement relaxation should be discussed for de-activated SCGs. |
| vivo | Yes | Since SCG may experience blocking during SCG deactivation, performing RLM on PSCell enables the detection of S-RLF. However, performing RLM on PSCell will cause more power consumption in case of that the radio link quality of PSCell is still stable. It seems that whether the UE perform RLM can be left to network implementation, i.e., UE will perform RLM when the associated RLM-RS is configured for the PSCell during SCG deactivation. |
| Qualcomm | No | We do not think UE should perform RLM on PSCell in SCG deactivated because of the following: - RRM measurements should suffice for the network to command UE to perform PSCell change before radio conditions deteriorate significantly. As pointed out above, RRM reports based on A2 event can help detect that PSCell is no longer suitable, and network can initiate PSCell change upon receiving such a report. - Performing RLM consumes energy. |
| Futurewei | Yes | Can be further discussed. Maintain the RLM in deactivated SCG is beneficial for the network/MN to take action in time when the link condition of the deactivated SCG is really bad, the UE can report to MN right way. It is inline with the motivation of quick activation upon request by the application. There could be trade-off between UE power consumption and RLM with a “light” RLM for deactivated SCG. |
| ZTE | Maybe | RRM measurement can be used to update PSCell when UE moves, but the drawback is that network may not notice the PSCell becomes bad timely, thus network cannot take appropriate action timely. We are open to further discuss this (e.g. how to cope with the limited configured resources), and it is acceptable to us if it is configurable from network. |
| CATT | Yes | To our understanding SCG suspension is decided based on the dormancy of the cell. One motivation of the suspended SCG is fast SCG activation. If the RLM is not performed and if the SCG is activated while the SCG RLF occurred, the network activates a non-functional SCG, the NW will handle the failure SCG first, which will increase the delay for SCG transmission. We think SCG Failure reporting procedure can be reused. |

### 2.2.4	Beam failure detection/recovery

Beam failure detection is based on L1-RSRP measurements and is another type of periodic RS monitoring. For the PSCell, bam failure recovery is based on RACH, or on the report of a MAC CE. Reporting a MAC CE would require getting a grant, i.e. start monitoring PDCCH and then transmit PUSCH. So this would probably involve RACH and require the UE to monitor PDCCH again, possibly leaving the deactivated state.

Q6: Do companies want that the UE performs beam failure detection/recovery on the PSCell while the SCG is deactivated? If your answer is yes, please provide your views on how the recovery would be performed.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | Same comments as that for Question 5. |
| MediaTek | No | Similar comment in Q5. BFR is also kind of RLM and we think it is not necessary for suspended PSCell. |
| Nokia | No | At least so far we have not identified use cases to keep beam failure detection/recovery for deactivated SCG. |
| NEC | No | same as Q5 |
| Samsung | No | See our previous response |
| Ericsson | Yes for BFD FFS for BFR | Beam Failure Detection (BFD) should be supported, otherwise UE would always require RACH upon SCG activation (which may defeat the purpose of the feature which is saving power but coming back as fast as possible). However, it can be FFS whether the UE performs Beam Failure Recovery (BFR) or not upon BFD. One could for example simply define that if BFD is detected while SCG is deactivated the UE performs RACH upon resumption, otherwise it could assume previous state(s) for PDCCH monitoring with the SCG. |
| Apple | Depends | It depends on how fast the UE should move from deactivated SCG to activated SCG. We should compare this with SCG in DRX operation, and if deactivated SCG is better in terms of latency in bringing back to activated state. |
| Intel | No |  |
| LG | No | If BFR is allowed for deactivated SCG, too frequent BFR procedures are expected to be triggered, which then would increase UE power consumption but with no gain (since the beam may remain unused ). |
| KDDI | NO | Same as RLF, BFD could also be discussed as an additional feature to improve the latency for fast activation |
| BT | No | We consider RRM and RLM is enough. |
| Sharp | No |  |
| Lenovo and Motorola Mobility | No | Similar as Q5 |
| Spreadtrum | No | Not necessary as no data transmission is performed in SCG. |
| DOCOMO | No |  |
| Interdigital | Yes | BFD can be considered as part of the RLM procedure, and as such it is beneficial for a faster activation, as we have mentioned above. As Ericsson pointed out, it can be discussed, in case of failure, whether the BFR can be postponed until the UE receives an instruction to activate the suspended SCG. |
| CMCC | Maybe | If we prefer a more efficient SCG activation mechanism, BFD is needed. |
| vivo | No |  |
| Qualcomm | No | The purpose of BFD/BFR is to restore lost connectivity if there is degradation in the serving best beam pairs and tries to ensure minimum disruption in data transmission. BFR would be performed by UE initiating RACH on the PSCell. We think that BFD/BFR is not required because the procedure consumes power and since there is no ongoing data transfer in SCG deactivated state. |
| Futurewei | FFS | Seems much more additional efforts with less benefit added. |
| ZTE | No | We think RRM and RLM would be sufficient. |
| CATT | Yes for BFD | We think BFD should be supported. This would enable fast SCG activation for the UEs who has not gone through Beam Failure. We don’t see BFR while in SCG suspension brings advantage as it would anyway require performing RACH. |

### 2.2.5	CSI-RS measurements and reporting

Some companies suggest that the UE performs CSI-RS measurements and reporting while the UE is deactivated [1][7] but possibly with relaxed requirements [6]. Other companies suggested no to perform CSI measurements and reporting. If the UE does not monitor PDCCH and cannot receive any MAC CE from the SCG, the measured CSI-RS would probably be periodic and the reporting would need either to use PUCCH (supposing the TAT is not expired) or to be via the MCG. Overall, this seems to have significant specification impact.

Q7: Do companies want that the UE performs CSI-RS measurements on the PSCell while the SCG is deactivated? If your answer is yes, please provide your views on how the reporting would be performed.

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | Same comments as that for Question 5. |
| MediaTek | No | Similar comment in Q5. Again we think that performing CSI-RS measurement increasing the power consumption and the SPEC complexity. We do not see enough benefit to have this. |
| Nokia | Yes | Why would we not support CSI-RS measurements as they are part of regular RRM measurements. CSI reporting can be discussed. |
| NEC | No | we do not see a need of CSI measurements and reporting for deactivated SCG. |
| Samsung | No | See our previous response |
| Ericsson | Yes | We agree with Nokia that CSI-RS measurements should be supported, but that the reporting needs further discussion. It should be possible to find some middle ground between complexity and gain for the reporting. In our view, we should be careful not to defeat the purpose of the feature, which is save power by deactivating SCG, but make sure the SCG can be quickly activated again. Therefore, we think that CSI-RS measurements should at least not be precluded at this point. |
| Apple | We assume CSI-RS towards channel feedback…? If so tending towards yes | CSI-RS towards channel feedback is helpful for PSCell scheduling after re-activation. If the CSI-RS is towards RRM, then this can be discussed. |
| Intel | No |  |
| LG | Yes | It is important to balance between power-efficient and time-efficient activation. Hence, we think CSI measurement and reporting with a long periodicity for deactivated SCG are beneficial. |
| KDDI | Yes | Share the comment with Nokia |
| BT | Maybe | For UE power consumption reasons, we should try to reduce the amount of measurements done in the UE, so we tent to say no unless is a clear benefit to do this for SCG activation. This can be also configurable by the network based on the service. |
| Sharp | No |  |
| Lenovo and Motorola Mobility | No | It is different from the dormant SCell case, where the CSI report can be sent per PSCell or per non-dormant SCell. For deactivated SCG, it can be complicated to support CSI report, e.g. UE sends CSI report to SN per MN. Considering the CSI report is essentially for link adaptation and time sensitive, it’s less useful if the CSI report could be outdated already when reaching SN. |
| Spreadtrum | No | It is only needed for RRM measurement in MN. CSI reporting is not supported in PSCell when SCG is deactivated. |
| DOCOMO | No |  |
| Interdigital | Yes | Waiting for CSI measurements/reports until the SCG activation command is received will increase the actual time before the PSCell is activated and ready to be scheduled on. For example, that was the reason why the SCell dormant state was introduced in LTE euCA, wherein CSI measurements were performed even though PDCCH is not being monitored. As others have also pointed out, the reporting aspect can be further discussed. |
| CMCC | Yes | Share the same view with Nokia. CSI-RS measurement and report are beneficial for activating SCG. |
| vivo | Yes | In our understanding, quick start SCG scheduling is also required upon SCG activation, thus CSI-RS measurements/reporting is beneficial. We also agree that long periodicity CSI-RS measurements for SCG deactivation can be considered. |
| Qualcomm | No | Besides the issues mentioned above with CSI reporting, the following are additional points to consider. Upon SCG activation, it is likely that the UE has to perform RACH on the SN in order to acquire UL timing. Since the RACH procedure delay is expected to be higher than the delay if UE performs CSI-RS measurements and reporting after SCG activation, the gain in terms of delay from performing CSI-RS measurements and reporting before SCG activation, i.e., in SCG deactivated state, does not seem significant. To be able to perform CSI reporting, UE needs to maintain UL timing with the SN. Maintaining UL timing requires UE to be able to receive Timing Adjustment commands from SN through either of the following means, one of which impacts power consumption and the other involves significant specification changes. - MAC CE commands sent on the SCG, which will require UE to monitor PDCCH. - Timing Adjustment commands received via MCG, that may involve significant specification changes (impacts on Xn/X2 specifications as well as MAC specification). |
| Futurewei | Yes | With scheme trade-off for power consumption. It will help for the beam level operation for access. |
| ZTE | Maybe | We understand this question does not involve CSI-RS based RRM measurement, CSI-RS based RRM measurement should be allowed as long as RRM is supported. For CSI reporting, we understand it is beneficial for fast PSCell activation, so we are open to further discuss it. |
| CATT | Yes | The CSI-RS measurements can be provided via the MCG or via the SCG by periodical PUSCH resources. Similar with the dormancy BWP, the CSI-RS measurements can reduce the delay for activating SCG for data transmission |

### 2.2.6	Timing advance

If the TAT is not expired, the UE could access the PSCell again without the need for RACH, but this can only work if there is no issue to keep the same TAT value while the UE does not monitor PDCCH.

Q8: Do companies want that the UE keep the TAT running, as well as PUCCH/SRS resources, while the SCG is deactivated?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | Same comments as that for Question 5. |
| MediaTek | No | It is unclear that whether this TA maintenance request frequently RACH procedure for suspended PSCell. We prefer just perform RACH procedure while transition from suspended to normal activate state. |
| Nokia | No as baseline but consider also whether not stopping would be easy to achieve. | As baseline stopping is OK option but there are many scenarios where stopping is not very useful/optimal e.g. small cells and also in case of larger cells fast reactivation of SCG would cause less resource if TA keeps running. |
| NEC | FFS | if reusing handling of pTAG TAT and SCell deactivation together, the UE can keep the TAT running. Once it expires, the UE shall release PUCCH and SRS resources. we want some more time to think about.. |
| Samung | No | We assume UE performs RA upon SCG activation |
| Ericsson | Yes | Same discussion about readiness. If we define that UE completely stops, there might be no point to use the feature compared to the legacy SCG release and add. |
| Apple | Depends | It depends again on how quickly the UE should get back to activated state from SCG deactivation. If it’s with RACH, then we have to compare this entire SCG activation/deactivation with DRX operation. |
| Intel | No |  |
| LG | Yes | We think the maintaining a TAT timer while CG is deactivated is beneficial for time-efficient CG activation. |
| KDDI | Yes | We think keeping TAT could help to reduce the RACH process when scell transits from deactive to active. |
| BT | FFS | We see the benefit to keep the TAT if we assume that most of the times, it will be still valid when required. If this is not the case, then it is better to directly use a normal RACH process. |
| Sharp | No |  |
| Lenovo and Motorola Mobility | No | RACH seems necessary for UE and gNB to get time aligned again. During SCG deactivation, there is no clear way for gNB to determine the time difference and send a timing advance command before TAT expires. |
| Spreadtrum | Yes | UE can keep the SRS resource when TAT is running. SN can adjust the TA value through MN. Once it is expired, the related resources shall be released. |
| DOCOMO | No | We are same view as Samsung |
| Interdigital | Yes | If the UE keeps the TAT running, if the SCG activation message is received before the TAT has run out, it does not have to perform RA. It can be discussed whether the UE can defer the RA upon TAT expiry until SCG activation. |
| CMCC | Yes | Same comments as that for Question 5. |
| vivo | Yes | It is helpful for fast SCG activation. |
| Qualcomm | Yes to keeping TAT running, No to keeping PUCCH/SRS resources | As mentioned, the UE could keep the TAT running upon entering SCG deactivated state, so that if SCG is activated while the timer is running UE can access PSCell without performing RACH. If TAT expires before SCG is activated, there does not seem to be any need to define additional procedures by which UE maintains UL timing with the SN in SCG deactivated since there is no identified need for any UL transmissions (please see our responses to questions Q7 and Q9). Regarding keeping PUCCH/SRS resources while the SCG is deactivated, there is no need (please see our responses to questions Q7 and Q9). We would like to confirm that since SCG configuration is retained while SCG is deactivated, PUCCH/SRS configuration which is a part of it, is also retained. |
| Futurewei | Yes | With scheme trade-off for power consumption. It will be beneficial for fast SCG activation. |
| ZTE | Yes with comment | We think the UE can keep TAT running, so that RACH can be avoid when performing SCG activation before TAT expires. But we perfer not to discuss the complex inter-node operation just to support sending TA command to UE via MN. |
| CATT | Yes | Keeping the UL sync can reduce the delay for SCG data transmission as it could avoid performing RACH at SCG activation. Fast SCG activation is also important in design of SCG suspension. |

### 2.2.7	SRS transmission

SRS transmission could be beneficial for faster activation but would increase the power consumption, which makes it not nice if the SCG remains deactivated for some time.

Q9: Do companies want that the UE performs SRS transmission while the TAT running (or some other criteria, please explain)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| OPPO | No | Same comments as that for Question 5. |
| MediaTek | No | Prefer to save more power. |
| Nokia | No | UL activity will cause massive power consumption and this is not supported in deactivated/dormant SCell either. Would be quite weird no starting to support for deactivated SCG. |
| NEC | No | this should never happen in deactivated SCG. So, keeping SRS resources while TAT is running and performing SRS transmission are totally different issue. |
| Samsung | No |  |
| Ericsson | No | We do not see a use case for this right now. |
| Apple | Maybe | While we were partial towards this, RANP decided to not have SRS atleast for SCell dormancy, and we think the same can be applied to PSCell as well.. but we are also ok to discuss this. |
| Intel | No |  |
| LG | No | SRS transmission for TAT is not essential, since TAT can be maintained by other ways (e.g. CSI reporting). We do not see much gain of SRS transmission for other purposes. |
| KDDI | Maybe | We are open to discuss how much energy consumption could be saved without SRS transmission and how faster the transition to active could be achieve. |
| BT | No | It is preferred to save power. |
| Sharp | No |  |
| Lenovo and Motorola Mobility | No | See the comment for Q8. |
| Spreadtrum | Maybe | SRS transmission is benefit for fast activation SCG and it can be controlled by the network. But it will affect UE power consumption. We are open to discuss it. |
| DOCOMO | No |  |
| Interdigital | No | This will undo the power savings gained by suspending the SCG to some extent. |
| CMCC | No |  |
| vivo | No |  |
| Qualcomm | No | Our primary concern with SRS transmissions is power consumption. The benefits of SRS transmission are enabling SN to perform UL channel estimation and to help maintain UL timing alignment (TA). Regarding UL TA, it seems that, even with SRS transmissions, UE may still need to RACH on SN upon activation if the TAT has expired but the UE has not received TA command to correct UL timing. The probability of this occurring can be lowered, for example, by reducing the periodicity of SRS transmissions (more frequent transmissions), but that increases power consumption. |
| Futurewei | No |  |
| ZTE | No |  |
| CATT |  | We are open to discuss this. For dormancy SCell, the TA of the dormancy SCell is consistent with the other cell which in the same TA group, so the dormancy SCell doesn’t need to send SRS. But for dormancy PSCell, if the PSCell stop the SRS transmission, but keep the TAT running, upon the TAT expiry, the PSCell will lose UL sync. Long periodic SRS transmission can be taken into consideration to keep UL sync and reduce power consumption. |

# 3	Conclusion

…

# 4	References

[1]	R2-2006756	On Support of Activation/Deactivation for SCG	InterDigital

[2]	R2-2006806	Discussion on SCG suspension	OPPO

[3]	R2-2006900	Framework of SCG deactivation and activation	ZTE Corporation, Sanechips

[4]	R2-2007009	Efficient Activation/Deactivation Mechanism for SCG and Scells	CATT

[5]	R2-2007046	Discussion on efficient activation mechanism for one SCG	Spreadtrum Communications

[6]	R2-2007068	On fast deactivation and activation of one SG and SCells	Nokia, Nokia Shanghai Bell

[7]	R2-2007109	Scoping the usage of SCG suspension	Apple

[8]	R2-2007215	Efficient activation and deactivation mechanism for SCG and SCells	vivo

[9]	R2-2007236	Enhancements for Rel-17 efficient activation/de-activation	Intel Corporation

[10]	R2-2007439	Consideration on dormant SCG	CMCC

[11]	R2-2007598	Efficient SCG/SCell (de)activation	Ericsson

[12]	R2-2007623	Further enhancements regarding deactivation and resumption for R17	Samsung Telecommunications

[13]	R2-2007678	Discussion on SCG deactivation and activation	Huawei, HiSilicon

[14]	R2-2007748	Efficient SCG activation/deactivation in MR-DC	Qualcomm Incorporated

[15]	R2-2007867	Discussion on SCG suspension	MediaTek Inc.

[16]	R2-2007986	Time-efficient SCG Activation mechanism	LG Electronics

[18]	R2-2007994	Discussion of SCG activation/deactivation	SHARP CorporationRP-193249 New WID on further enhancements on Multi-Radio Dual-Connectivity, Huawei