Agenda Item:	TBD

Source: 	Qualcomm Incorporated (email discussion rapporteur)

Title:    	(draft) Report for [Post111-e][923][NBIOT R17] RLF Enhancements (Qualcomm)

Document for:		Discussion and decision

# Introduction

The purpose of this email discussion is to study the NB-IoT RLF procedure and provide summary as per below email discussion:

- [Post111-e][923][NBIOT R17] RLF Enhancements (Qualcomm)

Scope: Study current RLF procedure to understand the time taken to select the cell for access. Start and end points FFS.

Intended outcome: report to next meeting

Deadline: Long

# NB-IoT RLF procedure overview

This section describes the RLF procedure according to Release 16 specification [2]. The following timers and counters are applicable in NB-IoT for RLF detection: T310, N310.

Note: Early detection of physical layer problems is not applicable to NB-IoT.

Figure 1 Overview of RLF

Question 1: 	Do you agree with the RLF procedure as depicted in Figure 1?

| Company | Yes/No | If No, suggest changes |
| --- | --- | --- |
| ZTE | Yes but | Generally fine. But we assume the data still can be transmitted before expiration of T310 (the failure or loss of some packets may be possible), so we think the total interruption time should be from Ref point C to Ref point D, not from Ref point A as mentioned in the Figure 1. |
| Huawei | No | In our understanding, the RLF procedure terminates when the UE declares RLF, i.e. when T310 expires, thus cell selection is not part of it. |
| RadiSys | No | Agree with Huawei. RLF procedure ends at Ref point C when T310 timer expires. |

The objective of the WID [1] is to reduce the total data transfer interruption time by reducing the time for cell reselection between Ref point C and Ref point D. RAN4 specifies the period between Ref point C and Ref point D as TUE-re-establish_delay_NB-IoT [6].

Question 2: 	Do you agree the WID objective is to reduce the time between Ref point C and Ref point D as depicted in Figure 1?

| Company | Yes/No | If No, provide your view on the reference points |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei | No | We agree it is part of the objective but we think that the WI description includes the time between Ref point B and Ref point C, ‘Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF’ |
| RadiSys | Yes | We agree that objective is to have a faster cell selection upon RLF. But UE should be equipped to have a faster cell selection between Ref point B and Ref point C. Network should provide mechanism for target cell selection during connected mode and it should be left to the UE implementation to make best use of the network provided information upon RLF for faster Cell Selection |

Question 3: 	If answer to above question is Yes, then do you agree the time between Ref point C and Ref point D is used to measure the performance of the solutions to WID objective?

| Company | Yes/No |  |
| --- | --- | --- |
| ZTE | May be No? | According to RAN4 spec, the time between Ref point C and Ref point D can be separated to several parts as following: TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-NC + TSI_NB1-NC + TPRACH_NB-IoT The WID emphasize the optimization on neighbor cell measurements and corresponding measurement triggering before RLF while the other parts, e.g., the optimization on SI acquisition is not covered by WID scope. Moreover, we worry that optimization on SI acquisition may introduce too much complexity but will not benefit much. Therefore, we suggest to focus on reduction of the time for neighbor cells detection between Ref point C and Ref point D, not all the parts in the above formula. |
| Huawei, HiSilicon | No | See answer to Q2. we think we should consider the timer between Ref point B and Ref point D. |
| RadiSys | No | See answer to Q2 |

# Cell selection upon RLF

In TS 36.304 [3], three different categories of cell selection are defined:

- Category 1: Initial cell selection: UE has no knowledge of what frequency bands are used in the area.

- Category 2: Stored information cell selection: UE knows what frequency bands are used in the area and in some cases centre frequencies.

- Category 3: Cell selection when leaving connected mode: UE knows frequency of the serving and neighbour cells.

Generally, category 1 takes the longest time to select a suitable cell while category 3 takes the shortest time to select a suitable cell and UE generally follows category 3 for cell selection upon RLF i.e. UE knows upon releasing dedicated resources which cell(s) to attempt measurements and then camp on.

Question 4: 	Do you agree following RLF the UE follows Cell selection when leaving connected mode?

| Company | Yes/No | Comments to support your answer |
| --- | --- | --- |
| ZTE | No | We understand Category 1 is mainly for the UE just power up. Category 3 is only used for the procedure of leaving RRC_CONNECTED, e.g., on reception of RRCConnectionRelease-NB message or RRCEarlyDataComplete-NB message. So in our understanding, Category 2 is performed during RRC re-establishment procedure. |
| Huawei, HiSilicon | No | We are confused about the introduction of these three categories when TS 36.304 section 5.2.3.1 only defines the two first ones. We think category 3 is a subset of category 2 and there is no need to introduce a new category. |
| RadiSys | Yes | Agree with Huawei. Category 3 is a subset of Category 2. UE will perform Cell Selection upon RLF, and will fall into Category 2 |

Cell selection generally involves the following steps for each NB-IoT carrier frequency:

- Detect and measure NB-IoT cell (i.e. NPSS/NSSS detection and RSRP/RSRQ measurement [4])

- MIB-NB acquisition

- SIB1-NB acquisition

Only after performing all these 3 steps can UE camp on the cell, acquire SIB2-NB, optionally SIB22-NB and initiate random access. After step 1 UE knows whether a carrier is a NB-IoT carrier and meets the minimum RF requirements and after step 3 UE knows whether it can (e.g. cell is not barred) camp on this cell for normal service.

The periodicity for the various signals/messages used for cell selection are shown in Table 1.

Table 1 Periodicity for NPSS, NSSS, MIB-NB and SIB1-NB

| Step | Signal/ Message | Periodicity (ms) |
| --- | --- | --- |
| 1 | NPSS | 10 [4] |
| 1 | NSSS | 20 [4] |
| 2 | MIB-NB | 640 [2] |
| 3 | SIB1-NB | 2560 [2] |

For each NB-IoT carrier, the three steps are done in a sequence hence the total time taken is simply aggregation of time take by step 1, 2 and 3. The total time to receive both NPSS and NSSS is shown in Figure 2.

Figure 2 NPSS/NSSS timing and example maximum detection time

The actual time to synchronize to the cell (i.e. decode NPSS and NSSS) depends on radio condition and whether cell is known. RAN4 has specified timing requirements for cell detection (Tsearch_NB1-NC) considering these two aspects and these are shown in Table 2 [6].

Table 2 Time to detect a NB-IoT cell

| Step | Tsearch_NB1-NC (ms) |
| --- | --- |
| Target cell known, any coverage | 0 |
| Target cell unknown, any coverage, decode on first attempt | 80 |
| Target cell unknown – normal coverage | 1400 |
| Target cell unknown – enhanced coverage | 14800 |

For eMTC, the definition of ‘target cell known’ is: a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal to or longer than the time duration required for the cell identification. But there is no such definition provided for NB-IoT and propose the same definition can be used in NB-IoT.

Question 5: 	For NB-IoT, assume definition of ‘target cell known’ as: a cell is known if it has been meeting the relevant cell identification requirement for a time duration equal to or longer than the time duration required for the cell identification?

| Company | Yes/No | If No, provide alternative definition |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon | No | This is specific to eMTC. In LTE, the target PCell is known if it has been measured by the UE in the last 5 seconds. For NB-IoT, no additional condition to UE identification has been specified, thus the cell is known when the UE has successfully decoded NSS and PSS. |
| RadiSys | No | This needs FFS |

Question 6: 	The time to synchronize to the cell upon RLF depends on coverage level?

| Company | Yes/No | Comments to support your answer |
| --- | --- | --- |
| ZTE | Yes | RLF occurs more frequently for the UEs in enhanced coverage and those UE would need more times to detect target cell. Therefore, we should pay (more) attention to the enhanced coverage case. We also think reducing the time for neighbor cell detection would benefit more for UEs in bad coverage level. Moreover, we may need to have some common assumptions for further evaluation, e.g., whether we should assume maximum value for T310 (8s for worst coverage) or a moderate value would be preferred (2s or 4s)? We think the maximum value may be seldom used as it makes the connection maintained in a poor status for some kind of long time, which is unexpected from the perspective of user experience and UE power consumption. So we think a moderate value (2s or 4s) for T310 may be enough. |
| Huawei, HiSilicon | Maybe | Yes, if we consider the coverage level in the neighbour cell. Not really otherwise. |
| RadiSys | Yes |  |

If a cell is known then total time to synchronize is 0 ms and as long as this cell fulfils the cell selection criteria defined in 36.304 [3] (i.e. Srxlev and Srxqual > 0) then UE can attempt to camp on the cell.

Assuming UE knows frequency of NB-IoT cells but all cells are unknown then the total time to detect all cells is aggregation of the time taken to synchronize to each NB-IoT cell.

Question 7: 	Do you agree for the purpose of this analysis, all cells are assumed to be unknown i.e. no cell meets the definition of ‘target cell is known’?

| Company | Yes/No | Comments to support your answer |
| --- | --- | --- |
| ZTE | Yes | According to current spec, NB-IoT UE cannot support any neighbor cell measurement in RRC connected mode (there previously was some clarification on that SIBs cannot be used for measurement in RRC connected mode), therefore we agree, upon RLF, all cells are assumed to be unknown. |
| Huawei, HiSilicon | No | At least we need to consider the case of ‘unknown’ cell. Then, if the purpose is only ‘cell selection’, there is no need to detect ‘all’ cells on all carriers as suggested multiple times above. |
| RadiSys | Yes | Since UE does not perform any measurements in RRC Connected Mode, UE need to synchronise with different cells to find any recommended cell (known cell) |

Question 8: 	Any other aspects of RLF detection and re-establishment?

| Company | Aspect (classify whether it as RLF detection or re-establishment) |
| --- | --- |
| ZTE | In last meeting, there was contribution to suggest using cell reselection upon RLF. Per our understanding, in current spec, upon RLF, UE will perform cell selection, e.g., after UE detects the target cell, as long as the cell fulfill the S criterion, the UE would select this cell for further access. Such process is already helpful for reducing the interruption time and no need to be further enhanced. |
| Huawei, HiSilicon | Not quite sure if the question is about the current procedure or the enhancements to introduce. If we have measurements in connected mode, we think we should introduce support for early RLF in case a target cell is found and suitable. |
| RadiSys | Connected mode procedure should be enhanced for faster cell selection upon RLF |

# Summary

TBD

- References

- RP-201064, WID revision: Additional enhancements for NB-IoT and LTE-MTC, RAN#88e, Reno, June 2020

- 3GPP TS 36.331, Radio Resource Control Protocol Specification, Release 16, June 2020.

- 3GPP TS 36.304, User Equipment procedures in idle mode, Release 16, June 2020.

- 3GPP TS 36.213, Physical layer procedures, Release 16, June 2020.

- 3GPP TS 36.211, Physical channels and modulation, Release 16, June 2020.

- 3GPP TS 36.133, Requirement for support of radio resource management, Release 16, June 2020.