3GPP TSG-RAN2 Meeting #112e	R2-20xxxxx

Electronic Meeting, 2 – 13 Nov 2020

Title:	draft LS on the use of simultaneous CSI-RS resources and ports

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN WG2)

To:	RAN WG1

Cc:	-

Contact Person:

#### Name:                   Lian Araujo

E-mail Address:   lian.araujo@ericsson.com

1. Overall Description:

RAN2 has discussed the use of simultaneous CSI-RS resources and ports, and would like to ask for clarification for the following aspect:

In RAN2 understanding, the field maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the number of active CSI-RS resources. Furthermore, the field totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC is supposed to indicate a restriction on the total number of ports to be used in active CSI-RS resources.

RAN2 would like to ask whether the fields maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC and totalNumberPortsSimultaneousNZP-CSI-RS are supposed to indicate: the number of active CSI-RS resources and ports, respectively; or the number of configured CSI-RS resources and ports (including active CSI-RS resources and ports), respectively?

The reason for the question is that the current field description for maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC in TS 38.306 includes “This parameter limits the total number of NZP-CSI-RS resources that the NW may configure across all CCs, and across MCG and SCG in case of NR-DC (irrespective of the associated codebook type).”; and the current field description for totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC  in TS 38.306 includes “This parameter limits the total number of ports that the NW may configure across all NZP-CSI-RS resources across all CCs”.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to answer to the above question.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #113e		25 Jan - 05 Feb 2020			Electronic Meeting

TSG-RAN2 Meeting #104		12-20 April 2020			Electronic Meeting