3GPP TSG-RAN WG2 Meeting #113-e	 R2-21xxxxx

Electronic, Jan 25th – Feb 5th, 2021

Agenda Item:	8.10.x

Source:	OPPO

Title:	Report of [Post112-e][152][NTN] UL scheduling enhancements [OPPO]

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion, Decision

# Introduction

This document is to collect companies’ views on UL scheduling enhancements in NTN.

- [POST112-e][1xx][NTN] UL scheduling enhancements (Oppo)

Scope: Discuss UL scheduling enhancements based on proposals in R2-2009064 and R2-2009109

Intended outcome: email discussion report

Deadline:  Long

# Discussion

On UL scheduling enhancements in NTN, RAN2#111e meeting has made some initial agreements, which are provided below for information.

Agreements via email - from offline 107:

- At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded)

With these two methods (configured grant and BSR over 2-step RACH) on the table, we will discuss them in detail in the rest part of this document.

# Configured Grant

In NR, there are two types of configured grant configuration, i.e. Type 1 and Type 2. Configured grant Type 1 is fully RRC-configured, while Type 2 is configured by RRC but needs to be activated/deactivated via DCI. In [1], it was stated that Type 2 is UE specific due to the use of CS-RNTI and due to large number of UEs in NTN cell, it will result in a large amount of required resources. Therefore, it was proposed that the preferred configured grant is Type 1, which is configurable for a group of UEs. However, rapporteur has different understanding on the statement that Type 2 is UE specific, and thinks that both Type 1 and Type 2 can be shared resources for a group of UEs. Anyway, it would be good to check company’s understandings.

| Question 1: Which type of configured grant is preferred in NTN? - Option A: Type 1 is preferred. - Option B: both Type 1 and Type 2 are feasible. |  |  |
| --- | --- | --- |
| Company | Option A/B | Comments |
| APT | Option B | Whether the CG resource is shared or dedicated seems not the difference of different CG types. Both types can be shared or dedicated based on NW configuration. The biggest difference between type 1 and type 2 is that type 2 can be more dynamically activated/deactivated via DCI. In NTN, we consider both types can work well according to smart NW configuration. |
| Huawei, HiSilicon | Option B | We share the same view with the rapporteur. CG type 2 can also configure shared resources for UEs. No need to restrict it to Type 1. |
| Xiaomi | Option B | We think the main difference of the two types is that type 2 is more dynamic than type 1. As to whether configured resources can be shared or not, it is network implementation to decide. Type 2 is still useful for cases that RTT is comparable to or smaller than RRC processing delay (10ms), e.g. LEO 600km with RTT = 2ms in order to be more dynamic than type 1. |
| CATT | Option B | Type1 or Type 2 configured grant can be configured by network, e.g. according to different services. For example, VoIP service works within type2 and latency sensitive service works within type2 in TN. It seems that both Type 1 and Type 2 are still feasible in NTN. |
| OPPO | Option B | Both types are feasible and there is no need to restrict network’s configuration. |
| Intel | Option A is preferred but B is ok | Our understanding is that both options are feasible. Type 2 allow the network dynamically activate and deactivate with DCI. However, due to long propagation delay, we slightly prefer option A. |
| Lenovo | Option B | Type 2 CG can also be shared and there is no need to limit to Type 1 CG. |
| Turkcell | Option B | We don’t need to restrict network configuration. Type 1 and Type 2 can be feasible in NTN |
| Qualcomm | Option B | We also think there is no need for restriction for now. |
| Ericsson | Option B | Type 2 can use any wanted resources just as Type 1. No need to limit to one or the other without any good technical reason. Type 1 CG uses CS-RNTIs for retransmissions, thus no difference in number of RNTIs between Type 1 and Type 2. RNTIs will anyway not limit the maximum number of supported users. Type 2 is faster to reallocate resources between users, in case of changes in propagation or in the traffic, using DCI instead of the RRC signalling needed for Type 1. |

For non-geostationary satellites, due to the movement of the satellites and the resulting changes in elevation angle, the channel quality or path loss may change drastically in a short period of time and may require a timely reconfiguration or activation/deactivation of the configured grant. The long propagation delay in NTN may have impact on frequent reconfiguration as well as activation/ deactivation of configured grant and thus enhancement to reduce the signaling overhead was proposed in [1]. Companies are invited to provide views on whether the issue is valid and whether enhancement is needed.

| Question 2: Do companies agree that “Enhancement to reduce the signaling overhead on configuration as well as activation/deactivation of configured grant should be discussed for NTN”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments (Possible enhancements can be provided here if your answer is “agree”) |
| APT | Disagree | Legacy mechanisms may be enough. |
| Huawei, HiSilicon | Disagree | We don't see a strong motivation to have any enhancement. |
| Xiaomi | Require RAN1 decision | It is RAN1 to decide whether frequent update of configuration of configured grant is needed or can be avoided. If the issue exist, RAN1 to decide how to solve the issue. |
| CATT | Disagree | From RAN2’s perspective, the legacy mechanism is enough. |
| OPPO | Require RAN1 discussion | It is not clear how fast the radio condition changes and whether the CG reconfiguration or activation/deactivation would be frequent. We think this should be discussed in RAN1 first. |
| Intel | Disagree | We are not sure the motivation for enhancement. If the signalling overhead is due to drastically changing, then dynamic grant should not be configured. |
| Lenovo | Disagree | Legacy mechanism is sufficient unless RAN1 identify any new issue. |
| Turkcell | Require RAN1 discussion | This item should be discussed in RAN1. Then we can decide the legacy mechanism is enough or not. |
| Qualcomm | Partly agree | For activation and de-activation, further discussion is needed when exactly network/UE assumes the resource de-activated. |
| Ericsson | Disagree | This is nothing special for NTNs. This is a general optimization that is not in the scope of this WI. |

For both types of configured grant, a periodicity of the configured UL grant is configurable. The supported periodicities are depending on the configured subcarrier spacing, e.g. from 15kHz to 120kHz. Taking subcarrier spacing of 15kHz as an example, the value range is from 2 symbols to 640*14symbols = 8960 symbols, i.e. from below 1ms to 640ms. Rapporteur understands that configuration of configured grant periodicity is mainly to map to the target service’s periodicity and is less relevant to propagation delay. Companies are invited to comment whether existing value ranges are sufficient for NTN or any modification is needed.

| Question 3: Do companies agree that “There is no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments (Please indicate the extended values if your answer is “disagree”) |
| APT | Agree |  |
| Huawei, HiSilicon | Agree | The periodicity is mostly related to the service, not the propagation delay. |
| Xiaomi | Depends on SA2 | Similar discussion with PDCP discard timer/t-reordering timer, periodicity is related to whether new 5QI is defined for NTN. It requires input from SA2. |
| CATT | Agree | The value range (e.g. from 1ms to 640ms) still works well in NTN. |
| OPPO | Agree | At least for now, we don’t see any need to modify. |
| Intel | Agree |  |
| Lenovo | Agree | No need to modify unless SA2 define new 5QI for NTN and require modification. |
| Turkcell | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree | If new 5QIs are defined we may return to check if they are compatible with the periodicity. |

Another parameter, configurable regarding configured grant is the maximum number of configured grant configurations per BWP maxNrofConfiguredGrantConfig-r16 and the maximum number of configured grant configurations per MAC entity maxNrofConfiguredGrantConfigMAC-r16. In Rel-16, maxNrofConfiguredGrantConfig-r16 is equal to 12 and maxNrofConfiguredGrantConfigMAC-r16 is equal to 32. As in NTN, the goal is more to provide coverage everywhere and support high mobility than to provide high peak data rate, there is no need to configure a lot of UEs with a large number of configured grants. Therefore, it was proposed in [1] that there is no need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN.

| Question 4: Do companies agree that “There is no need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments (Please indicate the extended values if your answer is “disagree”) |
| APT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| Xiaomi | Agree |  |
| CATT | Agree | Usually maxNrofConfiguredGrantConfig and maxNrofConfiguredGrantConfigMAC are configured by network according to the services of this UE. So there is no need to modify these parameters considering no special service types in NTN. The main goal of NTN still is eMBB which requires peak data rate. However the value of these parameters is not related with peak data rate. |
| OPPO | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Turkcell | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree |  |

As RAN2#111e’s agreements simply stated that configured grant will be further studied in NTN but the support for it was not formally agreed, it would be good to collect company’s opinions on whether configured grant is supported in NTN.

| Question 5: Do companies agree that “RAN2 support configured grant in NTN for UL scheduling”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments |
| APT | Agree |  |
| Huawei, HiSilicon | Agree | Configured grant helps to reduce overall the transmission latency, therefore is suitable for NTN. |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| OPPO | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Turkcell | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree | Yes, CG can be an important tool to decrease the delays when data arrives in the UE uplink buffer. All functionalities shall be supported unless specifically stated to not be supported, thus no spec change due to this. |

# BSR over 2-step RACH

RAN2#111e meeting has agreed to further study BSR over 2-step RACH in NTN. However, it is not crystal clear whether it means that BSR is sent over 2-step RACH triggered by any existing events in RRC Connected mode (e.g. SR failure, beam failure recovery, etc.) captured in TS 38.300 so far, or that RAN2 will introduce a new trigger (e.g. BSR) for 2-step RACH. It would be good to clarify company’s understandings on this terminology.

| Question 6: How do companies understand “BSR over 2-step RACH”? - Option A: BSR can be sent over 2-step RACH which is triggered by existing events in RRC Connected mode. - Option B: BSR can trigger 2-step RACH. |  |  |
| --- | --- | --- |
| Company | Option A/B | Comments |
| APT | Options A and B | In the beginning, we need to clarify whether “BSR over 2-step RACH” implies the BSR MAC CE transmitted via the MsgA PUSCH? If yes, option A is already supported now. Based on LCP, UE can multiplex the BSR MAC CE into a TB transmitted by MsgA PUSCH. For option B, it’s a new triggering of RA procedure. We think it’s helpful to trigger a 2-step RACH if the UE has triggered a BSR and there is no available UL resource, i.e., the UE does not trigger SR in this case. If the UE triggers the BSR but does not trigger the 2-step RACH, the UE may need to trigger SR based on legacy mechanism, which would suffer from propagation delay. |
| Huawei, HiSilicon | Option A | Option A has no spec impact. The current spec does not exclude transmitting BSR MAC CE in MSGA PUSCH, therefore Option A is already supported. We prefer not to have Option B (introducing a new triggering condition) because it brings much spec impact. |
| Xiaomi | Option B | During RAN2 #111e email discussion, the following 5 options were listed: - Option 1: SR-BSR procedure; - Option 2: Sending large grant in response to SR; - Option 3: Configured Grant; - Option 4: BSR-indication in SR; - Option 5: BSR over 2-step RACH. Option 1 is legacy procedure where BSR->SR-> RA(either 4-Step RA or 2-Step RA). The enhancement of the selection between 2-step RA and 4-step RA can be seen as a follow-up enhancement to option 1. Option 5 is the procedure where BSR triggers 2-step RACH directly, i.e. omitting SR. So, option A equals to option 1, option B equals to option 5. |
| CATT | Option A and B | - BSR MAC CE can be carried on MsgA PUSCH according to the legacy mechanism. So agree with option A. - It is possible for BSR to trigger 2-step RACH under some specific condition. |
| OPPO | Option A | For option A, it is already supported by the current spec. For option B, actually it can be achieved by network configuration instead of introducing spec impact. For example, for some logical channel, network does not configure any SR resource. If no UL resources are available, BSR triggered by this logical channel will trigger SR and will then automatically trigger RACH due to no valid PUCCH resource. |
| Intel | Option A | This is already supported today. |
| Lenovo | Option A | Already supported so no spec impact. |
| Turkcell | Optian A and B | Option A is already supported. Option B can be useful for specific cases in NTN. |
| Qualcomm | Option A + Option B | We also think Option B can be useful for some cases. |
| Ericsson |  | Difference between A and B are not clear to us. In Rel-15, the UE will trigger a BSR at new data arrival and if there is no UL-SCH resource available this will trigger an SR (or when the data cannot be transmitted on the UL-SCH, e.g., because allowedCG-List disallows it). If either a maximum number of SRs are sent or if no PUCCH SR resources are allocated, then the UE will trigger random access which may or may not be 2-step RACH. This RA will (if all data do not fit in the TBS) result in a BSR sent in msg3 or msgA if the msg3/msgA TBS is large enough to accommodate it after including the C-RNTI MAC CE. Thus, both Option A and option B are possible in legacy. What RAN2 can discuss is: - if it shall be possible for the UE to trigger a 2-step RA immediately for an SR even though the UE has PUCCH SR resources, need spec change - Or if UE itself may select between using an SR resource or a 2-step RA resource, need spec change - Or if it is enough that we do not configure PUCCH SR resources, no spec change needed - deciding if we need to differentiate collision probability for UEs in CONNECTED mode and UEs in IDLE/INACTIVE - then configure UEs with separate 2-step RA resources that is only used in CONNECTED mode, no spec change needed - possibly only use 4-step for UEs in IDLE/INACTIVE mode, no spec change needed - if UE has resources for both 2-step and 4-step RA, shall it select only the 2-step resources when wanting to send a BSR, spec change needed |

It was mentioned in [1] that using BSR over 2-step RACH will increase the collision probability of RACH procedure, since the number of UEs per cell/beam could become quite large. Compared to achieving low latency for NTN services, RACH resource shortage would be more important to be avoided. In some case, low latency is not so important for all NTN services. For example, if a UE wants to transmit huge amount of data, the delay until the first data transmission will not play such an important role and the usage of BSR over 2-step RACH could be avoided. Therefore, it was proposed to limit the usage of BSR over 2-step RACH in NTN.

| Question 7: Do companies agree that “In NTN, use BSR over 2-step RACH only to a limited level. FFS whether a level should be specified or is up to network implementation”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments |
| APT | Neutral | It depends on whether the BSR (over 2-step RACH) would be triggered too frequently in NTN. Note that in current behavior, the UE will only trigger the RACH if there is no available UL resource (and no SR). |
| Huawei, HiSilicon | Disagree | If we don’t introduce new triggering condition for 2-step RACH (i.e. Option B is not adopted in Q6), then UEs will not use extra 2-step RACH resources, thus RACH resource shortage is not an issue. |
| Xiaomi | Agree | BSR over 2-step RACH configured per LCH is enough, no need to further distinguish different data situation. |
| CATT | Neutral | We can discuss if there is RACH resources shortage at first. If there is, FFS whether a specified a level or up to network implementation. |
| OPPO | Agree, but up to network implementation | BSR over 2-step RACH configured per LCH can be implemented by not configuring SR resource for those LCHs. |
| Intel | agree | Up to network implemnetation |
| Lenovo | Agree | NW implementation is OK. |
| Turkcell | Neutral | We need to clarify RACH resource shortage. |
| Qualcomm | Up to network | Question is not clear what the limitation is. It is always up to network how to minimize the collision (2 step RACH may have different PRACH resource). There is no need to introduce the limitation. |
| Ericsson |  | Sending BSR when the UE has no grant is a rare event. Only if traffic has many bursts with no data in between there will be many BSRs per user. The network can manage the load on the 2-step resources by allocating more 2-step resources or by limiting which LCHs may trigger 2-step RA (by allocating PUCCH SR resources for the LCH). Only reason to separate 2-step between regular (from IDLE/INACTIVE UEs) and BSR RA (from CONNECTED UEs) is that we would like lower collision probability for one of the types of RA. No spec change needed for this. |

Similar as configured grant, since RAN2#111e’s agreements simply stated that BSR over 2-step RACH will be further studied in NTN but the support for it was not formally agreed, it would be good to collect company’s opinions on whether BSR over 2-step RACH is supported in NTN. Of course, the exact meaning of BSR over 2-step RACH would depend on company’s answers to Question 6.

| Question 8: Do companies agree that “RAN2 support BSR over 2-step RACH in NTN for UL scheduling”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments |
| APT | Agree |  |
| Huawei, HiSilicon | Agree if no extra spec impact is introduced | Agree if no extra spec impact is introduced. |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| OPPO | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Turkcell | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree, this is possible in Rel-15 already. | This is already part of baseline in Rel-15, by not configuring PUCCH SR resources to the UE. We see no reason not to support this functionality. |

# Co-existence issue

Before addressing the specific issue, it would be good to first clarify company’s assumption on whether UE in NTN can have both 2-step RACH and configured grant as valid configurations.

| Question 9: Do companies agree that “UE in NTN can have both 2-step RACH and configured grant as valid configurations”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments |
| APT | Agree |  |
| Huawei, HiSilicon | Agree | Same as R16. |
| Xiaomi | Agree |  |
| CATT | Agree |  |
| OPPO | Agree |  |
| Intel | Agree |  |
| Lenovo | Agree |  |
| Turkcell | Agree |  |
| Qualcomm | Agree |  |
| Ericsson | Agree | We can have both configured at the same time. |

If the answer to Question 9 is “agree”, then there may be a co-existence issue between 2-step RACH and configured grant. For a UE configured with both CG and 2-step RACH, at T1 (see Figure 1 below), the UE has UL new data and BSR is triggered. The next 2-step RACH occasion is at T2 and the next CG occasion is at T3. One issue is that if the time interval between T1 and T3 is much larger than the time interval between T1 and T2, whether the UE should trigger 2-step RACH or wait until the next CG occasion to send BSR. Note that, using CG as available UL resources to transmit BSR has been supported by the current MAC spec, but from low latency’s perspective, triggering 2-step RACH for BSR transmission is also beneficial in this case. However, 2-step CBRA may suffer RACH failure due to collision, which also makes the ultimate latency less predictable. Since it would be difficult to specify the selection rule, it was proposed in [2] to leave this to UE implementation.

Figure 1 resource selection for BSR

| Question 10: Do companies agree that “For a UE configured with both CG and 2-step RACH, whether the UE triggers 2-step RACH for BSR transmission is up to UE implementation”? |  |  |
| --- | --- | --- |
| Company | Agree/Disagree | Comments |
| APT | Disagree | It can be FFS. Even we go for UE implementation, there are some impacts if the UE decides to trigger/prioritize 2-step RACH in this case. |
| Huawei, HiSilicon | Disagree | At T2, if the 2-step RACH is triggered by existing triggering conditions (not introducing BSR as a new triggering condition), then UE is able to send BSR in MSGA at T2 according to the current spec. |
| Xiaomi | Disagree | If the configured grant doesn’t meet the meet the LCP mapping restrictions configured for the logical channel that triggered the BSR, 2-Step RACH will be triggered; Otherwise, 2-Step RACH is not triggered. This is the same mechanism adopted by BSR triggered SR. |
| CATT | Disagree | It is hard to guarantee the QoS if up to UE implementation. We can discuss it further how to solve the issue. |
| OPPO | Agree | Proponent company. As commented by Xiaomi, if CG becomes unavailable due to LCP mapping restriction, then certainly UE has no choice left other than SR (or RACH). Here we are considering the case where CG is available. According to current spec, UE will never trigger RACH, no matter how far the next CG occasion will be. We think this may not benefit the scheduling delay and allowing UE to trigger 2-step RACH can be considered in such case. Meanwhile, we don’t think it can be easy to specify any rules here, and thus propose to leave it to UE implementation. |
| Intel | Neutral | We can up to the UE implementation but further discuss if there is other impacts. |
| Lenovo | Neutral | We have concerns on UE implementation but we should be careful on making new rules (condition to use CG or 2-step RA) |
| Turkcell | Neutral | It’s UE implementation. |
| Qualcomm | Disagree | We would like to if some clarification is needed. |
| Ericsson | Disagree | This configuration is for an artificial scenario. Is the CG to be used for a traffic with known packet arrivals like VoIP while supporting some even higher prio service? Anyway, this can be achieved in legacy Rel-16 by configuring allowedCG-List to exclude the CG for the LCH that we want to trigger a 2-step RA for and then not configuring PUCCH SR resources for the LCH. |

# Summary and Proposals

This section summarizes the discussion and reports the following proposals:

Proposal 1	xxx.

# Reference

- R2-2009064, Enhancements on UL scheduling for NTN, Nomor Research GmbH, Thales

- R2-2009109, Discussion on other MAC issues in NTN, OPPO

# Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Huawei, HiSilicon | Lili Zheng (zhenglili4@huawei.com) |
| Xiaomi | xiaowei jiang(jiangxiaowei@xiaomi.com) |
| CATT | Jianxiang Li (lijianxiang@datangmobile.cn) |
| OPPO | Haitao Li (lihaitao@oppo.com) |
| Intel | Candy Yiu (candy.yiu@intel.com) |
| Lenovo | Min Xu (xumin13@lenovo.com) |
| Turkcell | Izzet.saglam@turkcell.com.tr |
| Qualcomm | Bharat Shrestha (bshrestha@qti.qualcomm.com) |
| Ericsson | robert.s.karlsson AT ericsson.com |