3GPP TSG-RAN WG2 Meeting #113-bis-e	                                                                                  R2-21xxxxx

Electronic meeting, 12th April – 20th April 2021

Agenda item:	8.13.3.2

Source:	CMCC

Title:	Report of [Post113-e][854][NR17 SON/MDT]  Logged MDT (CMCC)

Document for:	Discussion, Decision

# Introduction

This is to report the result of the following email discussion:

- [Post113-e][854][NR17 SON/MDT]  Logged MDT (CMCC)

-	Scope, In R2-2102250 (MDT summary), cat (b) proposals in 2.3 should be progressed.

Intended outcome: Report

Deadline: Long

According to the chair’s guidance, this email discussion and report will be based on the cat (b) proposals in 2.3 of R2-2102250 [1]. Please provide your comments by Wednesday 24/03/2021 23:59 UTC so that we have time to prepare the summary.

# Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| CMCC | xiefang@chinamobile.com |
| oppo | liuyangbj@oppo.com |
| Ericsson | pradeepa.ramachandra@ericsson.com |
| Samsung | sb07.kim@samsung.com |
| Qualcomm | rkum@qti.qualcomm.com |
| CATT | erlin.zeng@catt.cn |
| Huawei, HiSilicon | jun.chen@huawei.com |
| vivo | Ming.wen@vivo.com |
| Sharp | ningjuan.chang@cn.sharp-world.com |
| CMCC | xiefang@chinamobile.com |
| ZTE | Qiu.zhihong@zte.com.cn |
| Intel | Yi.guo@intel.com |
| Nokia | malgorzata.tomala@nokia.com |

# Discussion

## Logging early measurements

[2], OPPO

Proposal 1: RAN2 to agree that down selection from following three options should be made to let the network clearly understand the reason of expiration of the T331 or the mismatch between the camped cell and the validity area list why early measurement results are not included in the logged measurement report anymore:

- Option A: the configuration of the validityAreaList and/or the validityCellList and measIdleDuration-r16 IE is included in the logged measurement report.

- Option B: include an indication in a new LogMeasInfo-r16 entry in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report from now on (corresponding to the time stamp included in the entry) due to a reason other than cell-level signal not hearable.

- Option C: include indications in all related LogMeasInfo-r16 entries in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report due to a reason other than cell-level signal not hearable.

Proposal 2: RAN2 to agree that down selection from following two options should be made to let the network clearly understand the reason why some previously recorded early measurement results are missed in the following logged measurement entries is the updating of the early measurement configuration:

- Option A: include the updated carrier frequency information (measIdleCarrierListEUTRA and/or measIdleCarrierListNR IE) in the logged measurement report and add a new indication to mark the timestamp from which the updated configuration becomes valid.

- Option B: include an indication for the carrier frequencies for which the early measurements are no longer performed due to updating of the configuration of the early measurement.

[3], Qualcomm

Proposal 1: Introduce the UE capability to indicate whether UE supports logging of non-SIB4 frequencies, i.e., UE capability to indicate UE can perform logging of early measurements.

Proposal 2: Allow the logging and reporting of early measurements for logged MDT purposes. RAN 2 is requested to discuss the following choices:

- The network configures the location information in the early measurements. UE reports the early measurements together with location information. Note that, in the early measurement/idle mode measurements, UE reports a single latest measurement instance.

- The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

- The network may configure a separate report for logging early measurements. This report logs the measurements on carrier frequencies configured in early/idle mode measurements. This can also have different logging periodicity as compared to loggedMeasurementConfiguration.

During the email discussion “[AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I [4]”, most of companies think that P1&P2 of [2] are details so that [3] can be discussed first. So, it was suggested to not make any conclusions regarding [2] for last RAN2#113 E-meeting, and following agreements achieved based on [3]:

Agreement:

The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies. Whether a flag is needed should be FFS.

In this email discussion, we would like to first discuss the intention of the proposals from [2], further details of the solution are FFS (can be postponed to the next meeting online discussion). We understand the intention of the proposals is to help the network to justify the reason why early measurement results are missed in some part of logged measurement reports, either due to the reason of coverage restriction or other reason e.g., UE walks out the validitiy area, T331 timer expires, etc. It seems that such classification is benificial for the network in terms of coverage analysis of the early measurement frequiecies. Suppose T331 expires at a specific time moment A, the UE will no longer need to make measurement on the early measurement frequencies, natually, the corresponding early measurement results will be missed in the subsequent logged measurement report. Without the information of ‘T331 expiration at time moment A’ known by the netowork, after obtaining and anaylsis of the logged measurement report, the network may make a wrong decision to improve the coverage of the early measurent frequencies at the targeted location.

Companies are invited to provide views on the intention of the proposals from [2], OPPO:

Proposal 1: To know the reason why early measurement results are missed in some part of logged measurement report is beneficial for the network in terms of coverage analysis of early measurement frequencies.

Q1: Do you support above proposal to let the network to understand the reason why early measurement results are not included in the logged measurement report anymore is due to coverage hole or not?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| oppo | Agree | The intention for UE to report something in the logged measurement report for the convenience of the network for coverage analysis of early measurement related frequencies is crystal clear. |
| Ericsson | Agree | Upon expiry of T331, there is a sudden lack of measurements associated to certain frequencies in the MDT sample. So, having an indication in MDT sample allows the operators to identify the reason for ‘lack’ of measurements associated to certain frequencies would be good. We believe this could be implicitly derived depending on how early measurement results are introduced within the logged MDT report. This implicit vs explicit indication can be discussed once we start the running CR. |
| Samsung | No | There may be various reasons why UE does not have EMR results available for a frequency, some related to measurement performance/ RAN4 related. We think there is no real need for network to know the details. Only thing that might be considered is some way to distinguish between: a) UE recently performed EMR measurement but did not detect any sufficiently good cells b) UE did not recently perform EMR measurement for concerned frequency (due to whatever reason) |
| Qualcomm | No | In our understanding, by capturing non-cellreselction frequency, UE provides the network additional information on the frequency other than SIB4 and SIB5. Network should use this information as the additional information instead of mandatory information. |
| CATT | No | We understand the intention, but we are not sure if this is needed. There is similar situation for ‎areaConfigure in logged MDT, i.e., UE performs the measurement ‎according to the areaConfigure if configured, but will not report the areaConfigure to the network. [oppo]: these two are in different situations. Note that the areaconfiguration for logged MDT could be, at least up to network implementation, stored in the CN (it comes from the CN). When the UE sends the logged measurement report to the CN, CN could know if the missed measurements logging is due to coverage hole problem or UE being out of configured area (CN can check location information included in the measurement report). However, for the early measurement logging, it is the RAN to configure the early measurement related configuration, which is not known to the CN at all. So sending related information to the CN is needed. |
| Huawei, HiSilicon | No | We share similar view as Samsung. |
| vivo | No | The configuration of EMR to UE is not mandatory, therefore it is possible that no EMR measurements recorded at all from the UE side but nothing is wrong with the coverage issue. We tend to agree with Samsung that maybe it is needed to exclude the non-configured EMR case from the case in which EMR is configured but UE fails to perform any EMR measurements due to various reasons. |
| Sharp | No | Agree with Samsung’s view |
| CMCC | Yes | Agree with OPPO and Ericsson. |
| ZTE | No | Don’t know how this information can be used at NW’s side. |
| Intel | No | We are not sure whether it is needed. |
| Nokia | No | It is unclear to us what UE could do with those early measurement results that are not included in the logged measurement report. Absence of such measurements can be due to several reasons |

Summary:

We received 12 company inputs, where 3 companies support the proposal, and 9 companies don’t agree.  Therefore, the proposal is not pursued based on the majority view.

## MDT and On-demand SI

[5], CMCC

Proposal 1: UE reports the on demand SI related information even when the triggered random access procedure fails.

Proposal 2: UE reports its requested notBroadcasting SI message to help the network side to optimize the transmission of the SI message.

Proposal 3: One specific raPurpose is introduced for MSG3 based on demand SI request.

Proposal 4: UE reports the list of requested SI messages that request by MSG3 to the network.

[2], OPPO

Proposal 3: RAN2 to agree that on-demand SI related information shall be also included in logged MDT, in order to help the network optimize the SIB information broadcast periodically.

Proposal 4: RAN2 to discuss the cases for UE to record the on-demand SI related information. Options are provided as follows:

- When the maximum attempts of RACH for SI request are reached.

- When cell reselection occurs during the RACH for SI request.

- When the required SI is already broadcast periodically by network

[6], Samsung

Proposal 9	For on-demand SI optimization, extend RA report to include the following information:

- Time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure

- Msg3-based SI request related information

After the email discussion “[AT113-e][845] [NR/R17 SON/MDT] Logged MDT part II (CMCC) [7]”, following agreements were achieved:

=>	 UE records the on demand SI related information for following scenarios:

1. Failed on-demand SI request

2. Successful on-demand SI request

Agreements:

1 	One specific raPurpose is introduced for MSG3 based on demand SI request.

All the following proposals can be discussed through post meeting email discussion.

FFS:	UE reports its requested notBroadcasting SI message. It is FFS to only report the SIBs UE actually intends to request.

Proposal 2: It is FFS to consider following scenarios:

3. Cell reselection occurs during the RACH for SI request.

4. The required SI is already broadcast periodically by network

5.  Detecting geographic areas that are (unintentionally) covered by a non-desired SIA

6.  Connected on-demand SI request cases

Proposal 4: It is FFS for UE to report Time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure.

Proposal 6: It is FFS whether only Msg3-based SI request related information is reported.

Proposal 7: It is FFS whether to extend current RA-report to include the on demand SI information.

Therefore, companies are invited to provide views on above FFS and proposals:

Q2: Do you support that UE reports its requested notBroadcasting SI message or only report the SIBs UE actually intends to request?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| OPPO | Both | We think both these two information is useful for the network to make the optimization. In our understanding, the requested notBroadcasting SI message is subject to the mapping list provided by the network. But it is also possible that the required SI is not included in the list, for which case UE can not trigger the SI acquisition procedure. In order to assist the network to get a full picture, we think it is beneficial to capture both these two cases. |
| Ericsson | Report the SIBs UE actually intends to request | In the case of msg1 based on demand SI, the UE can send only one msg1 per RA procedure and thus the UE can obtain only the corresponding (associated to that msg1) SIB. However, if the UE needs two different SIBs which are allocated to different msg1, then the UE needs to perform two successive RA procedure. As we do not include any time stamp in the RA report, the information that’ the UE wanted to get two SIBs which are allocated to different msg1’ will get lost. Therefore, we support the inclusion of ‘SIBs UE actually intends to request’. By including this, the UE by default reports ‘its requested notBroadcasting SI message’. |
| Samsung | Agree | It could be more useful to only report SIBs UE actually intends to request via on-demand, so that the network can optimize the SIBs included in each SI message. |
| Qualcomm | May be none | NotBroadcasting SI information is available at the network, therefore, there is no need for UE to report it. I believe the argument behind adding the intended SI is to optimize set of broadcasting SIBs based on information from UE. However, in the success cases, network knows what are the SIs that UE have requested. Therefore, it may not be needed. |
| CATT | Both | It seems both information would be useful for the network. |
| Huawei, HiSilicon | Report the SIBs UE actually intends to request | This information is useful for the network to optimize on demand SI mechanism. |
| vivo | Report the SIBs UE actually intends to request | In case the RACH procedure fails, NW cannot find out what SIBs are requested from UE, thus it might be beneficial if the requested SIBs information can be reported. |
| Sharp | May be none | We also not quite clear about real benefit of report the requested SIBs or Sis. We tend to agree with Qualcomm. The network can know what Sis are requested by UEs from the success cases. |
| CMCC | Both | Both information would be useful for the network. |
| ZTE | The SIBs UE actually intends to request | UE can only based on the SI list configured in the si-SchedulingInfo to request SIBs, which might not perfectly matched with the SIBs UE actually intends to request, therefore it is useful to let UE report the SIBs it intends to request, which can help NW to optimize the broadcast status of SIBs as well as the mapping between SI message and SIBs. Since the mapping between SIBs and SI message is known by NW, with the SIBs reported by UE, NW shall be able to deduce the SI message UE requested. |
| Intel | May be none | Tend to agree with Qualcomm that it is not needed since the network can know this based on success cases. |
| Nokia | Both in case of failure | In this case, both notBroadcasting SI corresponding to MSG1 based SI Request as well as the SIBs that the UE requests in MSG3 based SI request could help the network learn which are the SIBs that a given UE is interested in |

Summary:

We received 12 company inputs, where 3 companies deem that none need to be report, 9 companies support to report the SIBs UE actually intends to request. Furthermore, 3 of the 9 companies support that UE also reports its requested notBroadcasting SI message.

Therefore, the majority view is that UE reports the SIBs that UE actually intends to request.

Proposal 1: UE reports the SIBs that UE actually intends to request.

Q3: Do you support to consider following one or more scenarios?

3. Cell reselection occurs during the RACH for SI request.

4. The required SI is already broadcast periodically by network

5.  Detecting geographic areas that are (unintentionally) covered by a non-desired SIA

6.  Connected on-demand SI request cases

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| OPPO | 3,4 | We agree to consider 3 because in this case the RACH procedure for SI request would be terminated and the requirement can not be delivered to network successfully. The information collected by 4 is useful for the network to determine whether some of SI in current broadcast list is actually not needed by most of UEs. From the perspective to make the optimization, we think it is also important to reduce those SI that is broadcasted periodically but not in great need. No strong on 5. We agree with it if it is majority view. For 6, we think network can play the role to collect this information. |
| Ericsson | Agree to Scenario-4 and Scenario-6 | Scenario-3 is not critical in our understanding as this is not very common. Therefore, we don’t think we need to treat it now. Scenario-4 could occur more frequently. Consider the scenario of a cell in rural area that performs on demand SI transmission. This cell might receive a group of users all at once (e.g., passing train scenario) wherein only one UE might actually transmit the msg1/msg3 based on demand SI request. Once the network starts broadcasting the SI which could benefit all the UEs, those that needed the SI but did not request for it (UEs could read the SI already) although they know that it is an on demand SI. In such scenario, it is beneficial for the network to know how many UEs benefit from the transmission of SI so that this can be taken in deciding whether the permanently broadcast the SI or keep it on-demand. Scenario-5 should be visible from the logged MDT reports i.e., if there is a serving cell for a very short span of time then the OAM realizes that this cell is a overshooting cell and thus if this cell transmits on demand SI, that could result in UEs ending up in scenario-3 wherein the UE might perform cell reselection before even receiving the on demand SI. Therefore, we believe scenario-5 is not required. Scenario-6 could also be looked at as the request-response procedure associated to acquisition of on demand SI is different in connected mode. |
| Samsung | Need to first clarify | We would like to clarify the scenarios: For 3), the intention seems unclear, i.e. it’s unclear on the relationship between cell reselection and SI request optimization. For 4), it’s unclear if it’s valid, i.e. UE can identify if SI is periodically broadcast or not via SIB1, and then UE will not request the SI. For 6), it’s unclear to need anything to be specified since the network would already identify the case. In Rel-17, we may focus on SI request in idle or inactive. |
| Qualcomm | Agree for 5. | 3 seems is a subcase of 5. Don’t see a use case for 4. Does network transmit different SIs for UE in connected, and in IDLE/INACTIVE states. In my understanding set of broadcasts and nonBroadcasts SIs are the same for UEs in connected and IDLE/INACTIVE, therefore, it is not needed to study in connected state. However, network should obtain the statistics about on-demand SIs requests received from the UE in IDLE/INACTIVE and connected states, such that it broadcast SIs minimizes UE to transmit msg1 or msg3 for acquisition of on-demand SI. |
| CATT | 3 | We agree with 3, and think it can be considered as a failure case for on-demand SI. For 6 we agree with some previous comments that network may be aware of this information. For the others we do not have strong view. |
| Huawei, HiSilicon | S4 | For S4, we think that the information could help network decide on transmission of SI via broadcast or on-demand ways. |
| vivo | 3 | 3) might be a rare case but it is still valid. Assume previously the UE performs RACH for SI request at cell A but ends up performs cell reselection at cell B, where the RACH procedure is terminated, thus the requested SI cannot be indicated to NW. In this case the requested SI should be recorded by UE and sent to NW afterwards. 4) is confusing to us as the si-BroadcastStatus and si-Periodicity carried in SIB1 already indicates whether the SI is periodically broadcast or not. No strong view on 5). For 6), agree with QC that it is not needed. |
| Sharp |  | For 3, the relation between RA and cell reselection may be that RA procedure for SI request can be stopped by cell reselection. But we are not sure the information in this case can help the network for RACH optimization or SI setting. For 4, same concern as Samsung, not sure this is a valid scenario. For 5, no strong option. For 6, no need for connected case. |
| CMCC | 4 | No strong view on other scenarios. |
| ZTE | 6 | Scenario 3 is not clear to us, it seems to fall into the case Msg1/Msg3 on-demand SI fails, why we need to highlight this situation? Scenario 4 is invalid, in our understanding UE can only request SIBs with status set as notBroadcasting. Scenario 5 also is confusing to us. I wonder at which circumstance will UE receive SI that is out of SI area, and how NW can deal with this situation based on this info? As commented in Q1, the information known by NW is the SI message that’s requested by UE but not the actual SIBs UE intends to request. Considering UE in connected state and inactive/idle state use the same si-SchedulingInfo to request SIs, it is necessary to also consider connected on-demand SI, so that NW can optimize the si-SchedulingInfo with complete knowledge. |
| Intel | Need to first clarify | Tend to agree with Samsung. Further clarifications on the benefits are needed. |
| Nokia | Not agree | We don’t see the need really to support these scenarios. |

Summary:

We received 12 company inputs, where 3 companies support scenario-3, and 4 companies support scenario-4, 1 company supports scenario-5, 2 companies support scenario-6. And, 3 companies think further clarifications on the benefits are needed.

Therefore, none of the option obtains enough support, and further discussion is needed.

Q4: Do you agree for UE to report Time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| OPPO | No | We do not think this information is necessary. |
| Ericsson | Depends on Q2 outcome | In our understanding, this is related to Q2. If all companies agree that the UE ‘reports the SIBs UE actually intends to request’ in Q2, then the network can crudely derive the time elapsed from the time of wanting the SI to the time of receiving it. This can be calculated based on the number of RA attempts and the number of successive RA procedures used to obtain those SIBs. However, if ‘Report the SIBs UE actually intends to request’ is not agreed in Q2, then we would support the inclusion of this time information as phrased for Q4. |
| Samsung | Agree | It would be important how long it takes to get the requested SI. Depending on the time info, the network may assign more radio resources for SI request. |
| Qualcomm | No | Don’t see how this information is useful to the network |
| CATT | Agree, and see comments | We agree to report time elapsed since the SI request initiation for on ‎demand SI until the successful SI acquisition, but we do not see the ‎need for UE to report other time.‎ |
| Huawei, HiSilicon | No | Do not see a strong need of it. |
| vivo | No | Do not see a strong need of it. |
| Sharp | No | This may not be needed. It there any requirement about the delay of SI acquisition? |
| CMCC | Agree | Share the view with Ericsson. |
| ZTE | Maybe No | In case SI request failure whether to initiate another SI request is up to UE’s implementation, don’t know how this information can be used at NW’s side. |
| Intel | No | Same question as Sharp. |
| Nokia | Not agree | We do not see the need why UE needs to log the time that has passed since SI Request and until successful SI acquisition. Obtaining of on demand system information is not mandatory or urgent to be provided by the network. |

Summary:

Since majority companies agree that UE ‘reports the SIBs UE actually intends to request’ in Q2, then the network can crudely derive the time elapsed from the time of wanting the SI to the time of receiving it. Therefore, for Q4, only one company supports to report the time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure, and the majority view is not to report it.

Proposal 2: UE does not report the time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure.

Q5: Do you agree that only Msg3-based SI request related information is reported?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| OPPO | No | We do not think we need to distinguish whether the SI acquisition procedure is based on msg1 or msg3. If we get the intention to collect the SI related information correctly, it is mainly used to let the network know which SI is in great need and which is not. Therefore, we think it is critical for the UE to provide the required SI information but not which procedure it has chosen. Both Msg1-based and Msg3-based SI request related information should be reported. |
| Ericsson | No | Msg3-based and msg-1 based SI request enhancements should be supported as the parameters to optimize related to each of them could be different. msg-3 based SI request is already supported to some extent as part of RA report but there could be missing information like which SIBs were requested etc. Such additions could be done for RA report. |
| Samsung | No | We have suggested new info (e.g. new timer) used for SON purpose, and it can be introduced for both msg1- and msg3-based SI request. |
| Qualcomm | Agree | I that we need to extend for msg-3 based SI request. However, I am wondering previously in the accessRelated was previously covering information regarding msg3-based SI, what additional information may be required. Network knows in the case of successful RA procedure; what SI was requested. So, I believe no additional information may be needed apart from “mag3 based SI” that we included as the ra-purpose. |
| CATT | No. | It is mainly for the network to know the requested SI information and times ‎no matter it’s based on Msg1 or Msg3. Both Msg1-based and Msg3-based SI ‎request related information should be logged and reported.‎ |
| Huawei, HiSilicon | No | We are open for Msg1-based SI request related information. |
| vivo | No | The SI optimization could be benefited from both cases. |
| Sharp | Agree | Share Qualcomm’s view. |
| CMCC | No | Both Msg1-based and Msg 3-bsed SI request related information should be supported. |
| ZTE | No | Msg1-based SI request information can also be considered. |
| Intel | No | Agree with other companies, the information for both MSG1 based and MSG3 based request should be considered. |
| Nokia | No | Both Msg1- and Msg3-based SI should be reported. |

Summary:

We received 12 company inputs, where 10 companies support that both Msg1-based and Msg3-based SI request related information should be supported, and 2 companies do not agree.

Therefore:

Proposal 3: Both Msg1-based and Msg3-based SI request related information are supported.

Q6: Do you agree to extend current RA-report to include the on demand SI information?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| OPPO | Maybe | If we agree to also capture some scenarios discussed in Q2 and Q3, extending RA-report may not be that suitable to make the report since the scenarios are not limited to RACH related procedure any more. New type of report might need to be introduced. |
| Ericsson | Agree | There are already most of the components available in RA report to enable this. We would also want to highlight that the CEF report should be used for failed on demand SI. |
| Samsung | Agree | Since the RA related info has been already included in RA report, it’s a good option to extend current RA report. |
| Qualcomm | No | In the success scenario, network already know what SI information is requested by UE. Therefore, ra-report need not be extended. |
| CATT | Maybe | In some failure scenarios, extending current RA-report seems not be ‎suitable, and the new report type or new mechanism may be needed.‎ |
| Huawei, HiSilicon | Suggest to further discuss | In Rel-16 SON features, CEF and RLF are for failure reports, and some RA information is included inside. In addition, Rel-16 RA report was separately defined and it was for successful RA procedures. Follow the above logic, we think the RRC design could be separated between failure and successful cases. For example: For failures of on demand SI, it is FFS whether existing failure reports can cover it. If not, perhaps a new failure report is needed. For successful on demand SI, it is an option to directly extend RA report to support it. In general, Q6 depends on scenarios, so it is suggested to seprate scenarios for discussions. |
| vivo | Suggest to further discuss | Share the similar view with HW. |
| Sharp | Suggest to further discuss | Share the similar view with HW. |
| CMCC | Suggest to further discuss | Share the similar view with HW. |
| ZTE | Further discussion is needed | Agree with other companies which report or even new report is needed require further discussion and depends on the scenario we intends to support. |
| Intel | Further discussion | Share the same view with Huawei. |
| Nokia | Not agree | The purpose of logging on demand SI would be to determine which SI has failed to be retrieved by a UE.RACH Report logs only RACH procedures that end with success. A request for On Demand SI may not end in success and therefore RACH Report would fail to log this information. |

Summary:

Most companies deem that further discussion is needed, since which report or even new report is needed require further discussion and depends on the scenario we intend to support.

Proposal 4: It is FFS whether to extend current RA-report to include the on demand SI related information.

## Logged MDT in EN-DC

There are the following options for logged MDT in EN-DC:

(1) LTE and NR logged MDT configurations are independent, and UE performs logging based on the logged MDT configuration of the same RAT it camps. (CATT, Qualcomm, Ericsson, ZTE)

(2) In EN-DC where UE cannot camp on NR cells, UE logs the NR measurements based on network configurations, i.e. for the NR logged MDT measurements in NSA, LTE SIB24 broadcasts the NR frequencies and indicates the UE does not need to reselect to NR cells. (Huawei)

(3) Do not introduce SN configuration for logged MDT (neither for camping nor for non-camping/ EMR specific frequencies). (Samsung)

(4) Enhanced areaConfiguration wherein a list of areaConfig could be provided to the UE wherein one list is associated to LTE and the other is associated is NR. (Vivo, Ericsson)

Companies are invited to provide views on above options for logged MDT in EN-DC:

Q7: Which option do you support for logged MDT in EN-DC?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| oppo | 1 or 4, 2 may be OK | For 2, it seems only workable in restricted scenario, e.g., NSA-only coverage area. For the scenario of UE capable of reselecting to NR cell from LTE cell, after the UE reselects to the NR cell, the UE has no NR logged MDT measurement configuration stored, and therefore could not perform NR logged MDT measurement. But we are Ok if majority think that it is sufficient that the NR logged MDT report could only come from the UEs served by NR cell when it was In RRC_connected state. For 3, the UE only reports the early measurement results to the base station, which is not used for OAM for coverage analysis. |
| Ericsson | Option-4 | We believe option-4 provides the most flexible possibility of configuring both NR and LTE related serving cell measurements while the UE camps in NR or LTE. |
| Samsung | 3) | As indicated before, MN can easily set logged measurement configuration for EMR specific frequencies (not more difficult than e.g. setting SIB24) We would like proponents to clarify option 1) and its relation to MRDC. Our assumption is that this option merely comprises: a) Per RAT configuration i.e. at T1 UE receives a configuration in RAT1 and at T2, after IRAT reselection, UE can receive logMDT config in RAT2 in which case UE does not overwrite config received in RAT1 b) When camping in RATN, UE behaves according to RATN config (can include target freqs for RAT2) i.e. performs logging, reports availability and provides results (if network initiates retrieval) Hence, option 1 is merely an alternative to 4) and independent of 3) We see no real need to enhance logged MDT procedures for IRAT continuation. As indicated, it does not relate to MRDC and we would like why this is now needed as so far we did not introduce this. If we have to choose, we somewhat prefer option 4) above option 1) as it is more consistent with current framework |
| Qualcomm | Option 1 | There are several benefits of option 1: 1. Provide flexibility in measurement logging, when UE moves from one RAT to another. 2. Both logged measurement configuration can be prepared considering individual RATs, i.e. a single measurement config may not have visibility to optimization objectives over both RAT. This becomes further exaggerated in the inter-RAT inter-system scenario. 3. Option 4 is difficult at the UE end, as UE has to SYNC T330 timer and other measurements. Therefore, our preferred choice is option 1. |
| CATT | Option 1 | Option 1 is more flexible. And, we see two issues for O4, 1) spec impact to ASN, and for both measurement configuration and reporting, 2) The MN and the SN may need to negotiate the logged MDT ‎configuration, which will increase the signalling overhead and ‎complexity.‎ |
| Huawei, HiSilicon | Option 2 | Option 1 addresses the scenario where the UE can camp on the NR cell. For this option, the UE is to store both LTE and NR logged MDT configuration, and thus it may bring lots of complexites (in legacy definition, the UE overrides the configuration when a new configuration is received). In our opinion, there should be enough samples due to NR logged measurements for SA UEs, so we do not see a strong need to have option 1. Option 2 addresses a scenario different from option 1, so it could be independent from option 1. For option 4, we see some complexities at both network and UE sides, and similar as our comments for option 1, NR SA UEs should be sufficient to do logging. |
| vivo | Option 4 | 1) Option 1 - Pros: Simple and elegant from the NW’s perspective; - Cons: stringent requirements to UE regarding storage, power consumption, and UE complexity; need to introduce the mechanism for the retrieval of SN RACH report. 2) Option 2 - Pros: low operational complexity for UE; fewer specification efforts - Cons: leads to extra power consumption; not applicable to other MR-DC scenarios 3) Option 3 - Pros: Re-use the early measurement framework in MDT (if specified in future) - Cons: UE can only measure part of the configured NR measurements. 4) Option 4 - Pros: low operational complexity for UE (UE maintains a single logged MDT report); fewer specification efforts are foreseen since the current reporting mechanism can be re-used. - Cons: lack of reporting flexibility. Cannot camp on NR cell in NSA (this can be solved by including option 2-like configuration) In summary, our preference is Option4. |
| Sharp | (3) | We agree with Samsung’s view. |
| ZTE | Option 1 | Option 1 allows the continuity of logging at both RAT while guarantee the flexibility since NW can based on the requirement to configure separate logged MDT configuration to MN and SN; Option 4 only allow the same configuration for different RAT which limit the flexibility. |
| Nokia | Option 1 or Option 3 | For Option 1: If the SRBs that triggers configurations for different RATs are differentiated, it would help to address the requirement to not overwrite Signalling based MDT by Management based MDT. Option 4: limits UE’s complexity but would require RAN nodes coordination |

Summary:

5 companies support option 1 (OPPO, QC, CATT, ZTE, NOKIA), 2 companies support option 2 (OPPO, HW), 3 companies support option 3 (Samsung, Sharp, NOKIA), 3 companies support option 4 (OPPO, Ericsson, VIVO).

Therefore, none of the option obtains enough support, and further discussion is needed.

Proposal 5: It is FFS which option is used for logged MDT in EN-DC.

## Logged MDT in other MR-DC

As suggested in [1], it was proposed to firstly discuss EN-DC case and then other MR-DC cases for logged MDT. So we will not discuss other MR-DC cases at this email discussion.

## Override discussion (based on RAN3 LS R3-207176)

[6], Samsung

Proposal 3	RAN2 is requested to clarify whether there is a need to avoid logged MDT configuration in the following cases:

- Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Logged MDT configuration is released, but UE still has un-retreived results that would be discarded upon accepting a new configuration

Proposal 4	Before agreeing to revert the previous RAN2 agreements to adopt a network based approach, RAN2 should conclude which UE based mechanism would be appropriate i.e. whether

- UE rejects/ does not accept the MDT configuration

- UE provides assistance by which network can avoid overwriting of an MDT configuration

Proposal 5	When considering a UE based mechanism, adopt the approach in which UE provides assistance by which network can avoid overwriting of an MDT configuration

[8], Huawei

Proposal 2: Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration. The UE reports the logged MDT type in the RRCConnectionComplete/RRCResumeComplete/ RRCReconfigurationComplete /RRCRestablishmentCompelete messages if the UE has the logged MDT configuration.

[9] Ericsson

Proposal 1	UE needs to be informed regarding the type of logged MDT configuration.

Proposal 2	UE needs to store the flag information until logged MDT report are collected by the network or till 48 hours of T330 expiry.

Proposal 3	A UE configured with signalling-based MDT sends an explicit reject message to RAN if it receives a management-based MDT configuration.

In last RAN2#113 E-meeting, following agreements were achieved:

=>	Introduce UE based solutions in Rel17 to fulfil the requirement that management based logged MDT should not overwrite signalling based logged MDT. FFS the details.

Companies are invited to provide views on following details about UE based solutions:

Q8: Do you think there is a need to avoid logged MDT configuration in the following cases?

- Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Logged MDT configuration is released, but UE still has un-retreived results that would be discarded upon accepting a new configuration

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| oppo | no | Prefer to follow the legacy operation. No need for enhancement. |
| Ericsson | Yes | Both are scenarios when signaling based MDT related information is still available at the UE. Therefore, the UE should not discard the logs (for 2)) and the configuration (for 1)). We believe the following proposal (proposal-2) in R2-2101424 includes both scenarios. Proposal 2 UE needs to store the flag information until logged MDT report are collected by the network or till 48 hours of T330 expiry. |
| Samsung | 2) | We think we should avoid discarding of signalling based MDT results |
| Qualcomm | No | We agreed that management-based MDT shouldn’t overwrite the signalling-based MDT configuration. This should be true irrespective of measurements have been collected or not. No measurement collected so far doesn’t mean that there will be no measurement in the future. Upon T330 timer expiry, the ValLogMeasConfig is released. It should be the network responsibility to extract the logged measurement when availability is indicated by the UE. Therefore, we do not support a need for avoiding logged MDT configuration in above scenarios. |
| CATT | No | We don’t see strong need for this. |
| Huawei, HiSilicon | Yes |  |
| vivo | No | See no strong need for further enhancements under such scenarios. |
| Sharp | Yes |  |
| CMCC |  | No strong view. |
| ZTE | No strong view |  |
| Intel |  | No Strong view. |
| Nokia | No | There should be mechanism to avoid overwriting Signalling MDT bu Management MDT, but not necessarily mechanism to avoid losing the data. If we adopt mechanism to ensure Signalling based MDT gets priority (by UE reject or network negotiation), there is no need for overcomplex handling of data retrieval. |

Summary:

4 companies support a need for avoiding logged MDT configuration in above scenarios, and another 4 companies do not support; 2 companies have no strong view. Therefore, further discussion is needed.

Proposal 6: It is FFS whether there is a need to avoid logged MDT configuration in the following cases:

- Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Logged MDT configuration is released, but UE still has un-retreived results that would be discarded upon accepting a new configuration

Q9: Which UE based mechanism would be appropriate?

- UE rejects/ does not accept the MDT configuration

- UE provides assistance by which network can avoid overwriting of an MDT configuration

- A UE configured with signalling-based MDT sends an explicit reject message to RAN if it receives a management-based MDT configuration.

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| Oppo | 2 | The assumption that network configuration is always proper should be maintained. Otherwise, additional work is required for the UE to judge if the configuration is proper or not, which we think is not feasible. |
| Ericsson | Option-3 | In option-1, the network does not get to know whether the UE has received the MDT configuration or not. Therefore, this is not optimal. In option-2, for every RRCxxComplete message, the UE needs to include a flag that it has immediate MDT configuration. As a UE might perform many RRC communications, we believe this is a large unnecessary overhead. Option-3 ensures that the network realizes that this UE is configured with signalling based MDT as the UE sends an explicit reject message indicating that the incoming management based MDT config cannot be accepted due to the currently ongoing signalling based MDT configuration. As this solution does not introduce overhead to the existing RRCxxCOmplete messages, we believe option-3 should be standardized. |
| Samsung | 2) | We are quite reluctant to introduce the option that UE rejects configuration requested by network as violates our general framework and fear it may open door for more cases |
| Qualcomm | Option 2 | UE indicates that signalling based MDT is configured in the RRCSetupComplete/RRCReconfigurationComplete/RRCResumeComplete/ RRCReeastablishmentComplete. Network should configure the loggedMDT based on this information. We want to avoid unnecessary signalling. |
| CATT | Option 2 | Agree with QC. |
| Huawei, HiSilicon | Option 2 | We think Option 1) and 3) enable rejection procedure, and then it may lead to extra signallings. We prefer to avoid such inefficient interactions between UE and RAN sides. |
| Vivo | Option 2 | Agree with QC and HW. |
| Sharp | Option 1 | If a UE that has been configured with signalling-based MDT receives a management-based MDT configuration, the UE can just ignore and do not accept the received configuration. We suppose this is no big impact on the network, as the network may anyway configure management-based MDT to many Ues. |
| CMCC | Option 2 |  |
| ZTE | Option 2 | Share the same view as Samsung |
| Intel | Option 1 | Tend to agree with Sharp, the UE can simply ignore the management based MDT configuration. |
| Nokia | Potentially Option 1 or Option 3 | Currently, only the RAN node knows whether the MDT configuration is for Signalling or Management based option. There is no need to assist the network, if the network takes care of the proper handling at the point of configuration. However, overall mechanism depends on whether we agree to have two UE contexts (for different RATs). In case, the UE transitions to RRC_IDLE and reconnects to other cell it could simply reject another MDT config from the same RAT, if there is one MDT session ongoing. |

Summary: 3

8 companies support option 2, 3 companies support option 1, and 2 companies support option 3. Therefore, majority view is to prefer option 2: UE provides assistance by which network can avoid overwriting of an MDT configuration.

Proposal 7: UE provides assistance by which network can avoid overwriting of an MDT configuration.

Q10: Do you agree to introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |
| oppo |  | No strong view |
| Ericsson | Agree | This is required for any of the UE based solution |
| Samsung | No | We understand this information is already available. |
| Qualcomm | Agree | This may be required for a UE based solution. |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| vivo | Agree |  |
| Sharp | Agree |  |
| CMCC | Agree |  |
| ZTE | Maybe | Perhaps it is simpler to only introduce one-bit indication to indicate whether the MDT configuration is a signalling based MDT or not. |
| Intel | Agree |  |
| Nokia | Need further clarification | This is available information in the network, by proper handling of the prioritized configuration there may be simpler mechanism adopted |

Summary:

9 companies support to introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration, 1 company shows no strong view, 1 company deems further clarification is needed, and 1 company says no. Therefore, majority companies support the proposal.

Proposal 8: Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

# Conclusions

Following proposals are made based on above discussion:

Proposal 1: UE reports the SIBs that UE actually intends to request.

Proposal 2: UE does not report the time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure.

Proposal 3: Both Msg1-based and Msg3-based SI request related information are supported.

Proposal 4: It is FFS whether to extend current RA-report to include the on demand SI related information.

Proposal 5: It is FFS which option is used for logged MDT in EN-DC.

Proposal 6: It is FFS whether there is a need to avoid logged MDT configuration in the following cases:

- Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Logged MDT configuration is released, but UE still has un-retreived results that would be discarded upon accepting a new configuration

Proposal 7: UE provides assistance by which network can avoid overwriting of an MDT configuration.

Proposal 8: Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

# References

- R2-2102250	Summary on 8.13.3 MDT	Huawei	discussion

- R2-2101960	Consideration of logged MDT enhancements	 OPPO discussion

- R2-2101341	Logged measurement Enhancements	QUALCOMM INCORPORATED	discussion

- R2-2102143	Report of [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I		Huawei

- R2-2101642	MDT enhancement for on-demand SI	CMCC	discussion

- R2-2101945	Progressing Logged MDT for R17 concerning MR-DC, IRAT and IDC	Samsung Telecommunications	discussion

- R2-2102142	Report of [AT113-e][845] [NR/R17 SON/MDT] Logged MDT part II (CMCC)	CMCC

- R2-2101697	Discussion on logged MDT enhancements	Huawei, HiSilicon	discussion´

- R2-2101424	On UE based solution related to Logged MDT (reply LS to R3-207176)	, Ericsson