3GPP TSG-RAN WG2 Meeting #113bis-e	R2-210xxxx

Electronic meeting, 12th -20th April 2021

Agenda Item:	8.10.2.1

Source: 	OPPO

Title:  	Discussion for [Post113bis-e][103][NTN] LS on TA pre-compensation (OPPO)

Document for:	Discussion and Decision

# Introduction

This document is to collect companies’ comments to the following discussion.

- [Post113bis-e][103][NTN] LS on TA-precompensation (Oppo)

Scope: Discuss the content of an LS to RAN1 on TA-precompensation aspects and determination of UE-gNB RTT, based on meeting agreements.

Intended outcome: Approved LS

Deadline (for companies' feedback): Monday 2021-04-26 16.00 UTC

Deadline (for final LS in R2-2104376../../../../../../../Data/3GPP/archive/RAN2/RAN2%23112/Tdocs/R2-2010761.zip): Tuesday 2021-04-27 16.00 UTC

# Discussion

Discussion point 1: The following text is included in the draft LS to ask RAN1 about the TA pre-compensation parameters.

RAN2 has briefly discussed how to broadcast the TA pre-compensation parameters during RAN2#113bis-e, but did not reach any agreements due to the lack of RAN1’s input. To facilitate future RAN2 discussion, RAN2 would like to ask RAN1 to prioritize the TA pre-compensation work on: (i) whether and/or what parameters to broadcast for TA pre-compensation, and (ii) when broadcasted, how often the broadcasted parameters are expected to change over time.

Please provide your comments or suggestions (if any) in the below table:

| Company | DP1. Comments on the question about TA pre-compensation parameters |
| --- | --- |
| Samsung | The first two points are fine. Please add the following two items: (iii) If RAN1 would consider any alternatives to the common feeder link delay if RAN2 were to suggest those such as NTN-GW coordinates (to improve accuracy and reduce the frequency of updates) and total processing delay (to improve accuracy instead of considering pure propagation delays only). (iv) When GNSS visibility is poor, would RAN1 consider using the Reference Point coordinates (e.g., cell center) at the UE? |
| CATT | According to the latest agreement of RAN1 on common TA, the value of 0 is supported. We are fine to ask the two questions. - is network-controlled common TA, and may include any timing offset considered necessary by the network. - with value of 0 is supported. - FFS: details of signaling including granularity. |
| Xiaomi | Agree with the text |

Discussion point 2: The following text is included in the draft LS to ask RAN1 about determination of UE-gNB RTT.

RAN2 has agreed to use UE-gNB RTT as the offset to start some UP timers (e.g. drx-HARQ-RTT-TimerDL for the HARQ process with HARQ feedback enabled). RAN2 understands that it is not assumed in RAN1 that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Therefore, RAN2 would like to ask RAN1 to provide input on: (i) how UE determines UE-gNB RTT, and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

Please provide your comments or suggestions (if any) in the below table:

| Company | DP2. Comments on the question about determination of UE-gNB RTT |
| --- | --- |
| Samsung | I think (i) is adequate. (ii) is unclear and can be removed in our view. |
| CATT | We are fine to ask “how UE determines UE-gNB RTT” to RAN1. TA pre-compensation is different from the delay. However there is an assumption that UE-gNB RTT is calculated by the broadcasted parameter for TA pre-compensation in the question “(ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.” So we suggest to delete the assumption which is not the agreement in RAN1/2 and update as: (ii) what information needs to be broadcasted for UE-gNB RTT in UE. |
| Xiaomi | Agree with the text |

Discussion point 3: The following text is included in the draft LS to ask RAN1 about TA reporting.

RAN2 has discussed TA reporting and agreed that at least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation. RAN2 also agreed that UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE. RAN2 would like to ask RAN1 to provide input on the exact content and frequency of TA reporting.

Please provide your comments or suggestions (if any) in the below table:

| Company | DP3. Comments on the question about TA reporting |
| --- | --- |
| Samsung | We like enhanced clarity of this proposal by Robert/Ericsson. RAN2 has discussed TA pre-compensation reporting and agreed that at least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation (FFS when/how to report). RAN2 also agreed that if the UE reports the UE specific TA pre-compensation during RACH procedure, it useing a MAC CE. RAN2 would like to ask RAN1 to provide input on the exact content and frequency of TA UE reporting of information about the UE specific TA pre-compensation for uplink scheduling adaptation. |
| CATT | The proposal from Ericsson looks good to CATT. |
| Xiaomi | Agree with the text |

Please provide any other comments related to the LS content in the below table:

| Company | Any other comments |
| --- | --- |
| Samsung | Contributors can submit examples of information that could be useful to UEs for TA pre-compensation so RAN1 can provide a preliminary view on those. We have done so in our Discussion Point 1 response. This can potentially accelerate the overall progress. |

# 3. Summary

To be updated

# Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Sasmung | Nishith Tripathi (nishith.t@samsung.com) |
| Xiaomi | xiaowei jiang (jiangxiaowei@xiaomi.com) |