3GPP TSG-RAN WG2 Meeting #113bis-e			                DRAFT R2-210xxxx

Electronic meeting, 12th - 20th  April 2021

Title:	[DRAFT] LS on TA pre-compensation

Response to:	-

Release:	Rel-17

Work Item:	NR_NTN_solutions-Core

Source:	OPPO (to be RAN2)

To:	RAN1

Cc:	-

Contact person:	Haitao Li

lihaitao at oppo dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 has briefly discussed how to broadcast the TA pre-compensation parameters during RAN2#113bis-e, but did not reach any agreements due to the lack of RAN1’s input. To facilitate future RAN2 discussion, RAN2 would like to ask RAN1 to prioritize the TA pre-compensation work on: (i) whether and/or what parameters to broadcast for TA pre-compensation, and (ii) when broadcasted, how often the broadcasted parameters are expected to change over time.

RAN2 has a working assumption to use UE-gNB RTT as the offset to start one UP timer (drx-HARQ-RTT-TimerDL for the HARQ processes with HARQ feedback enabled). RAN2 understands that it is not assumed in RAN1 that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Therefore, RAN2 would like to ask RAN1 to provide input on: (i) how UE determines UE-gNB RTT, and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

RAN2 has discussed TA pre-compensation reporting and agreed that at least for uplink scheduling adaptation, the UE may report information about the UE specific TA pre-compensation (FFS when/how to report). RAN2 also agreed that UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE (FFS if this needs to be configured). RAN2 would like to ask RAN1 to provide input on the exact content and frequency of UE reporting of information about the UE specific TA pre-compensation at least for uplink scheduling adaptation.

2. Actions:

To RAN1 group:

ACTION:   1) RAN2 respectfully requests RAN1 to prioritize the TA pre-compensation work on: (i) whether and/or what parameters to broadcast for TA pre-compensation, and (ii) when broadcasted, how often the broadcasted parameters are expected to change over time.

2) RAN2 respectfully requests RAN1 to provide input on: (i) how UE determines UE-gNB RTT, and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

3) RAN2 respectfully requests RAN1 to provide input on the exact content and frequency of UE reporting of information about the UE specific TA pre-compensation at least for uplink scheduling adaptation.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#114-e		19th – 27th May 2021			Electronic Meeting

3GPP RAN2#115-e		16th – 27th August 2021		Electronic Meeting