3GPP TSG-RAN WG2 Meeting #114 electronic	DRAFT R2-21xxxxxx

- Online, 19 – 27 May, 2021

Title	Summary of [POST113bis-e][505][R17 IIoT] URLLC in UCE

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	8.5.3

WID/SID	NR_IIOT_URLLC_enh-Core

# 1	Introduction

This document is the report of the following email discussion:

- [POST113bis-e][505][R17 IIoT]  URLLC in UCE (LG)

Scope:

    - Discuss remaining open issues related to URLLC in UCE based on inputs submitted to 8.5.3.

    - Agreeable Proposals

Deadline: same as submission deadline

Note that the INACTIVE period is 28/April – 5/May.

# 2	Contact Information

Please enter the contact information in the following table:

| Company | Name (Email Address) |
| --- | --- |
| LG | SunYoung LEE (ssunyoung.lee@lge.com) |

# 3	Prioritization of retransmission over initial transmission

In RAN2#113 e-mail discussion 505 [R2-2102087], it was discussed whether to prioritize retransmission over initial transmission on a CG by taking the LCH priority into account if cg-RetransmissionTimer and lch-basedPrioritization are configured, which was left as FFS:

- FFS With cg-RetransmissionTimer and LCH-based prioritization configured, the MAC entity can prioritize between initial transmissions and retransmissions on a CG based on priority of multiplexed LCH(s) -or to be multiplexed

Despite of support from majority companies (17/22) to prioritize between initial transmission and retransmission by taking LCH priority into account, it was left as FFS due to different understanding of the problematic cases and/or specification impact.

Case1:  Between different but overlapping CG configurations

Case1 is that autonomous retransmission due to LBT failure on a CG (blue box) collies with an initial transmission of higher priority data on different but overlapping CG (green box). In [8], it is proposed to prioritize the initial transmission of higher priority data (green box) in this case. In the meanwhile, in [2][9], it is explained that prioritizing the initial transmission of higher priority data for different but overlapping CGs is already supported by intra-UE prioritization.

Figure 1. Case1 when low priority data is mapped to CG1 while high priority data is mapped to CG2

Question 1: (1) Do companies agree that, for overlapping CGs, the MAC entity should prioritize the initial transmission of higher priority data over autonomous retransmission of lower priority data? If Yes, (2) do companies agree that it is already supported?

| Company | (1) Yes/No | (2) If yes, do companies agree that it is already supported? | Comment |
| --- | --- | --- | --- |

Case2: Within a singles CG configuration

Case2 is that, for a single CG configuration, an uplink data arrives and its priority is higher than that of a MAC PDU waiting for autonomous retransmission.

It is proposed, in [1][3][5][12][14][15][17], that an initial transmission of higher priority data should be prioritized over autonomous retransmission of lower priority data. It is concerned that lower priority data delays higher priority data while they are assigned to the same CG configuration.

It is argued in [6][9][13][16] that the network would not assign logical channels having different QoS to the same CG configuration, hence intra-UE prioritization within the single CG configuration is considered as a unnecessary optimization. Also, it is pointed out in [2] that, even for Rel-16 IIOT, the MAC entity transmits lower priority of deprioritized data on the next available CG with the same HARQ process regardless of whether higher priority of new data arrives or not. It is concerned in [9] that prioritizing initial transmission or autonomous retransmission within the same CG configuration confuses the network regarding proper configuration configuredGrantTimer and cg-RetransmissionTimer. In addition, [9] raises an issue that lower priority data may be a MAC PDU containing only MAC CE(s) and loss of such data is not desirable because it can be recovered neither by the UE nor by the network. Similarly, in [3], it is further proposed not to prioritize if the transmission is for a UCI-only MAC PDU.

Question 2: Do companies agree that, within a single CG configuration, the MAC entity should be able to prioritize the initial transmission of higher priority data over autonomous retransmission of lower priority data?

| Company | Agree /Disgaree | Comment |
| --- | --- | --- |

In [6][10], it is proposed not to introduce prioritization between initial transmissions and re-transmissions on the same CG configuration based on priority of multiplexed LCH(s) or LCH(s) to be multiplexed in Rel-17, i.e., leave it up to UE implementation which to prioritize. Contrary, in [4], it is proposed that network can control the selection rule for a CG, e.g. the retransmission is prioritized, or the initial transmission is prioritized, or the transmission with highest LCH priority is prioritized.

Question 3: If the MAC entity should be able to prioritize the initial transmission of higher priority data over autonomous retransmission of lower priority data for a single CG configuration, which solution do companies prefer?

- Option1. The UE always prioritizes initial transmission of higher priority data over autonomous retransmission of lower priority.

- Option 2. It is UE implementation which transmission is to be prioritized [6][10]

- Option 3. The network configures prioritization rule, i.e., the retransmission is prioritized, or the initial transmission is prioritized, or the transmission with highest LCH priority is prioritized [4]

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |

# 4	HARQ sharing and lch-basedPrioritization

So far, RAN2 agreed the following combinations:

- When cg-RetransmissionTimer is configured, HARQ processes sharing between multiple CGs are allowed (RAN2#112)

- When cg-RetransmissionTimer is not configured, HARQ processes sharing between multiple CGs are not allowed. (RAN2#112)

- lch-basedPrioritization and cg-RetransmissionTimer can be configured together in Rel-17 (RAN2#113)

In [9][13], it is proposed that when lch-basedPrioritization is configured, HARQ process is not shared between CG configurations in order to avoid mixture of CG selection mechanism from NR-U and IIOT, i.e., autonomous retransmission is allowed across different CG configurations whereas autonomous transmission is limited to the same CG configuration. Whether to have a specification restriction or rely on network implementation may need to be discussed further.

On the other hand, in [2], it is proposed to confirm that HARQ process sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured, i.e., even when lch-basedPrioritization is configured, as the benefit is foreseen in terms of latency.

- Option 1. HARQ process is not shared between CG configurations when lch-based Prioritization and cg-RetransmissionTimer are both configured [9][13]

- Option 2. HARQ process is shared between CG configurations when lch-based Prioritization and cg-RetransmissionTimer are both configured [2]

Question 4: When both of lch-based Prioritization and cg-RetransmissionTimer are configured, which option do companies prefer?

| Company | Option 1 or 2 | Comment |
| --- | --- | --- |

# 5	Harmonization operation

In this clause, we look into the issues in configuration of AutonomousTx and cg-RetransmissionTimer for the scenarios listed below:

| Scenario | Description |
| --- | --- |
| 1 | UL grant is prioritized and LBT fails |
| 2 | UL grant is de-prioritized including the case where UL grant is de-prioritized by e.g., CI-RNTI |

## 5.1	Scenario 1: UL grant is prioritized and LBT fails

### Case 1. AutonomousTx is configured and cg-RetransmissionTimer is not configured

The MAC entity triggers a new transmission for the next available CG (when not filtered out by running configuredGrantTimer). Consequently, neither autonomous transmission nor autonomous retransmission is triggered [2].

Question 5: Do companies agree that the above operation is the correct understanding of the current specification?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |

### Case 2. AutonomousTx is not configured and cg-RetransmissionTimer is configured

The MAC entity changes the HARQ status to pending and does not start cg-RetransmissionTimer. The MAC entity triggers a retransmission for the next available CG. Consequently, autonomous retransmission is triggered [2].

Question 6: Do companies agree that the above operation is the correct understanding of the current specification?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |

### Case 3. AutonomousTx and cg-RetransmissionTimer are configured

Neither cg-RetransmissionTimer nor configuredGrantTimer is started.

If configuredGrantTimer is already running, the configuredGrantTimer is kept running. As configuredGrantTimer is running, it means that the HARQ state was not pending and not changed. If configuredGrantTimer is not running, the HARQ state becomes or remains pending. As a result, the MAC entity triggers a retransmission for the next available CG. Accordingly, autonomous retransmission is triggered [2][7][16].

Question 7: Do companies agree that the above operation is the correct understanding of the current specification?

| Company | Agree/Disagree | If Yes, which option do companies prefer |
| --- | --- | --- |

## 5.2	Scenario 2: UL grant is de-prioritized

### Case 1. AutonomousTx is configured and cg-RetransmissionTimer is not configured

The MAC entity triggers a new transmission for the next available CG (when not filtered out by running configuredGrantTimer) [2]. Consequently, autonomous transmission is triggered.

Question 6: Do companies agree that the above operation is the correct understanding of the current specification?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |

### Case 2. AutonomousTx is not configured and cg-RetransmissionTimer is configured

In RAN2#113, it is agreed that:

- the MAC entity stops cg-RetransmissionTimer when the CG resource associated with the timer is deprioritized due to LCH-based prioritization.

Currently, configuredGrantTimer is stopped for the corresponding HARQ process if the deprioritized uplink grant is configured with autonomousTx.

In scenario 2, the MAC entity triggers a retransmission for the next available CG because configuredGrantTimer is kept running [2][7]. This is not aligned with the RAN2 agreement that If AutoTx is not configured, deprioritized MAC PDU is not retransmitted. For this reason, the following options are proposed:

- Option1. Even if a CG is not configured with autonomousTx, the configuredGrantTimer is stopped when the associated CG is deprioritized [7]

- Option 2. If a CG is not configured with autonomousTx, the cg-RetransmissionTimer is not stopped when the associated CG is deprioritized [13]

- Option 3. If a CG is not configured with autonomousTx and the previous CG for the same HARQ process was deprioritized, it is routed to a new transmission [2]. The suggested Text Proposal is in the table below:

1> if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured (i.e. new transmission): 3> consider the NDI bit for the corresponding HARQ process to have been toggled; 3> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 2> else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process: 3> if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink which was deprioritized and autonomousTx is not configured; or 3> if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant which was not deprioritized (i.e. retransmission on configured grant): 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

Question 8: Which option should be considered?

| Company | Option 1/2/3 or No option necessary | Comment |
| --- | --- | --- |

### Case 3. AutonomousTx and cg-RetransmissionTimer are configured

Neither configuredGrantTimer nor cg-RetransmissionTimer is started, or both timers are stopped, if running. The HARQ state is not changed.

If HARQ state is not pending and a CG is de-prioritized, the MAC entity triggers a new transmission for the next available CG [2][4][7][16]. However, there are different understanding of the current specification in detailed HARQ operation.

- Option 1. In HARQ operation, the MAC entity performs autonomous transmission [2][16].

- Option 2. In HARQ operation, the MAC entity obtains a new MAC PDU and autonomous transmission is not possible [4][7]. Some solutions may be needed, e.g., not stopping configuredGrantTimer.

Question 10: Which option is the correct understanding of the current specification?

| Company | Option 1 or 2 | Comment |
| --- | --- | --- |

If the HARQ state is pending due to previous LBT failure and a CG is de-prioritized, the MAC entity triggers a retransmission at the next available CG. Accordingly, autonomous retransmission is triggered, which is not aligned with the agreement in RAN2#113 that AutoTx and CGRT are responsible for deprioritized MAC PDU and LBT-failed MAC PDU, respectively. For this reason, it is proposed in [13] to consider the HARQ process as not pending so that the MAC entity triggers a new transmission and performs autonomous transmission.

Question 11: Do companies agree that the HARQ process should be considered as not pending if CG is de-prioritized while the HARQ state is pending?

| Company | Agree/Disagree | Comment |
| --- | --- | --- |

# 6	HARQ process ID selection

In RAN2#112, it was agreed:

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

RAN1 is currently discussing how to enable/disable CG-UCI, i.e., configurable depending on RRC parameters or always enabled. Given that HARQ process ID selection is impacted by CG-UCI, it was discussed further in RAN2#113-e how to select HARQ process ID and RV when cg-RetransmissionTimer is not configured, where no agreement was made.

At that time, half of companies (11/22) prefer Option 3, i.e., no enhancements needed, i.e., if cg-RetransmissionTimer is not configured, the UE uses the existing formula to determine the HARQ PID according to timing of the CG occasion, while there were some support to wait for RAN1 decision (8/22). The options provided in RAN2#113bis-e are listed below:

- Option 1. If cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism is used for HARQ process ID, i.e., confirm the RAN2 agreement in RAN2#112e [2]

- Option 2. Wait for RAN1 conclusion [9]

- Option 3. If cg-RetransmissionTimer is not configured and CG-UCI is not configured, Rel-16 URLLC mechanism is used for HARQ process ID [1][5]

Question 12: When cg-RetransmissionTimer is not configured, which option do companies prefer for HARQ process ID selection?

| Company | Preferred option | Comment |
| --- | --- | --- |

# 7	Other issues

If companies have any other issues that should be discussed in this e-mail discussion, please fill that in the table below:

| Company | Tdoc Number | Issue |
| --- | --- | --- |

# 8	References

[1] R2-2102685, CG Harmonization for Unlicensed Controlled Environment, Qualcomm

[2] R2-2102725, URLLC in UCE, CATT

[3] R2-2102992, HARQ Process Prioritization of Configured Grant for IIoT in NR-U, Nokia

[4] R2-2103059, Remaining issues about uplink enhancements for URLLC in UCE, Huawei, HiSilicon

[5] R2-2103072, Uplink enhancements for URLLC in unlicensed controlled environments, Intel

[6] R2-2103126, Issue of Prioritizing Initial Transmission over Retransmission on a CG, VIVO

[7] R2-2103211, Consideration on URLLC over NRU, OPPO

[8] R2-2103297, Enhancements for URLLC in unlicensed controlled environments, Lenovo, Motorola Mobility

[9] R2-2103428, Harmonizing UL CG enhancements in NR-U and URLLC, Ericsson

[10] R2-2103441, Further Consideration on the UL transmission in UCE, ZTE

[11] R2-2103492, CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments, III

[12] R2-2103566, Prioritization of UL transmissions in unlicensed URLLC, Sony Europe B.V.

[13] R2-2103648, CG harmonization for UCE, Samsung

[14] R2-2103688, Discussion on the remaining issue for uplink enhancements for URLLC in UCE, CMCC

[15] R2-2103797, IIoT operation in unlicensed controlled environments, InterDigital

[16] R2-2104103, Further details on harmonization, LG

[17] R2-2104288, Remaining issues of CG harmonization, Xiaomi Communications