3GPP TSG-RAN WG2 #115-e                                                                                   R2-21xxxxx

Online, 16-27 August 2021

Agenda Item:	    xx

Source: 	    MediaTek Inc.

Title:		Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)

Document for:	    Discussion, decision

# 1 Introduction

This document is a report on the following email discussion, initiated after RAN2#114-e:

[Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)

Scope: how UE shall handle the extension field of candidateBeamRSList. The intention is to agree a 38.331 clarification CR in next meeting. Could consider option 2 and option 3 proposed in R2-2106115 as a starting point. This was also discussed in [AT114-e][022].

Intended outcome: Report, agreeable CR.

Deadline: Long

The discussion will proceed in two phases, first to determine an agreeable mechanism for handling the extension field and second to converge on an agreeable CR.  The deadlines are as follows:

Phase 1: Friday 2 July 1700 UTC

Phase 2: Friday 6 August 0900 UTC

# 2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (rapporteur) | Nathan Tenny | nathan.tenny@mediatek.com |

# 3 Background

## 3.1	Original options

The discussion from [1] considered three options drawn from the discussion in [2]:

- Option 1: The UE releases the entire concatenated list, both the entries configured with candidateBeamRSList and the entries configured with candidateBeamRSListExt-v1610.

- Option 2: The UE releases only the extended entries that were configured with candidateBeamRSListExt-v1610.

- Option 3: The release branch is not used, and the UE treats candidateBeamRSList and candidateBeamRSListExt-v1610 as a single concatenated field with Need M.  The extended list candidateBeamRSListExt-v1610 is only included when candidateBeamRSList is included and fully populated.

## 3.2	Updated options for this discussion

In the discussion, option 1 had less support compared to options 2 and 3.  Rapporteur also understands that in continued offline discussion (separate from the official email discussion), a network-based restriction was proposed, in which the network is required to signal the extension (candidateBeamRSListExt-v1610) whenever it wants the extension entries to remain unchanged in the UE, and the “release” option on the extension list is used only when the network intends to reconfigure the UE to a number of entries fitting within the original list.  This option (option C below) disambiguates the UE behaviour by having the network always indicate explicitly the fate of the extension entries.

Accordingly, this discussion considers three options:

- Option A: When candidateBeamRSListExt-v1610 is set to release, the UE releases only the extended entries that were configured with candidateBeamRSListExt-v1610.

- Option B: The release branch is not used, and the UE treats candidateBeamRSList and candidateBeamRSListExt-v1610 as a single concatenated field with Need M.  The extended list candidateBeamRSListExt-v1610 is only included when candidateBeamRSList is included and fully populated.

- Option C: The network is required to signal the extension (candidateBeamRSListExt-v1610) whenever it wants the extension entries to remain unchanged in the UE, and the release option on the extension list is used only when the network intends to reconfigure the UE to a number of entries fitting within the original list.

The details of the options may require some clarification (e.g. how to define the “extended entries” in option A), so the following discussion subsections include space for discussion of the details of each option.

## 3.3	Examples

For clarity, this section illustrates how options A/B/C would operate in two example scenarios.

Example 1: The network reduces the list size while extension entries are configured, and the resulting list is still larger than the legacy list size:

- Network sends a BeamFailureRecoveryConfig containing a fully populated candidateBeamRSList (16 entries) and a partly populated candidateBeamRSListExt-v1610 (2 entries).

- UE concatenates the fields into a single list of 18 entries.

- Network sends a BeamFailureRecoveryConfig containing a partly populated candidateBeamRSList (15 entries) and omitting the candidateBeamRSListExt-v1610.

  - With option A, the UE populates a list of 17 entries, and the handling of a future release indication depends on the interpretation of the option (see section 4.1 below).

  - With option B, the UE populates a list of 15 entries; the 2 entries from candidateBeamRSListExt-v1610 are released.

  - With option C, this step is not allowed; if the network intends to reconfigure the UE to a list of 15 entries, it needs to include the 15 entries explicitly along with the candidateBeamRSListExt-v1610 set to release, and if it intends to reconfigure the UE to a list of 17 entries, it needs to include the 17 entries explicitly.

Example 2: The network reduces the list size while extension entries are configured, and the resulting list fits inside the legacy list size:

- Network sends a BeamFailureRecoveryConfig containing a fully populated candidateBeamRSList (16 entries) and a partly populated candidateBeamRSListExt-v1610 (2 entries).

- UE concatenates the fields into a single list of 18 entries.

- Network sends a BeamFailureRecoveryConfig containing a partly populated candidateBeamRSList (10 entries) and omitting the candidateBeamRSListExt-v1610.

  - With option A, the UE populates a list of 12 entries, and the handling of a future release indication depends on the interpretation of the option (see section 4.1 below).

  - With option B, the UE populates a list of 10 entries; the 2 entries from candidateBeamRSListExt-v1610 are released.

  - With option C, this step is not allowed; if the network intends to reconfigure the UE to a list of either r10 or 12 entries, it needs to include the entries explicitly along with the candidateBeamRSListExt-v1610 set to release.

# 4 Discussion (Phase 1)

## 4.1 Details of option A

During the previous discussion of option A, it became clear that there are two potential understandings of the definition of “only the extended entries”.

- Approach A.1: The UE remembers which list entries were initially configured by candidateBeamRSListExt-v1610, and subsequently treats these as being the extension entries.  Thus, even if the list is later shortened to a length that fits within the original list size, some entries may be marked as “extended entries” and can be released with the extension field.

  - In Example 1 from section 3.3, sending a release would release the 2 entries that were initially configured by candidateBeamRSListExt-v1610, leaving the UE with a list of 15 entries.

  - In Example 2 from section 3.3, sending a release would release the 2 entries that were initially configured by candidateBeamRSListExt-v1610, leaving the UE with a list of 10 entries.

- Approach A.2: The UE treats the entries from the two list fields as a single undifferentiated list (as usual for lists without ToAddMod structure), and the extension field only addresses entries beyond the size of the original list.  Thus, if the list is shortened to a length that fits within the original list size, the UE considers that it has no more extended entries, and setting candidateBeamRSListExt-v1610 to release becomes vacuous.

  - In Example 1 from section 3.3, sending a release would release the 1 entry that exceeds the legacy list size, leaving the UE with a list of 16 entries.

  - In Example 2 from section 3.3, sending a release would have no effect.

Question 1.1: Which of the two approaches do companies prefer, within the scope of option A?

| Company | Preferred Approach | Comments |
| --- | --- | --- |
| MediaTek | A.1 | The extension field is Need M, which means that if it is omitted rather than set to release, it should be considered to maintain its previous contents. This suggests that we should view candidateBeamRSListExt-v1610 as a field separate from candidateBeamRSList, and the UE should be aware of which entries were configured with the extended list (so that it knows what to maintain in case the field is omitted). We don’t find this to be a problem for the implementation to remember, |

Question 1.2: Any other comment on the details of option A?

| Company | Comments |
| --- | --- |

## 4.2 Details of option B

Rapporteur understands that option B is fairly straightforward and there may not be many details that need clarification.  This section is provided for any comments on the details of option B.

Question 2.1: Any comment on the details of option B?

| Company | Comments |
| --- | --- |

## 4.3 Details of option C

Option C was introduced in informal discussion and may need some analysis to make sure that all the implications are understood.  Rapporteur understanding is that this option is intended to have no UE impact (i.e., to be compatible with any UE handling of the release branch), since the release branch is only used when the original field is populated and the list fits within the original entries.

Question 3.1: Companies are invited to provide details of their understanding of option C.

| Company | Comments |
| --- | --- |
| MediaTek | Our understanding is described in the paragraph above; this option should be possible to specify without UE impact. As in the examples from section 3.3, we think it successfully disambiguates the handling of the extended entries. |

Question 3.2: Do companies understand that option C can be specified/implemented without UE impact?

| Company | Y/N | Comments |
| --- | --- | --- |
| MediaTek | Y |  |

Question 3.3: Any other comment on the details of option C?

| Company | Comments |
| --- | --- |

## 4.4 Preferred option

Companies are invited to indicate their preferred option (A/B/C).

Question 4.1: Which option do companies prefer among options A/B/C?

| Company | Preferred Option | Comments |
| --- | --- | --- |
| MediaTek | A or C | We have some concern about option B, as it effectively changes the need code of candidateBeamRSListExt-v1610 from Need M to Need R. We are fine with the other two options. |

## 4.5 Text for the general case

The difficulty of this example seems to suggest that we should have some general guidance in the spec for extending a list without ToAddMod, e.g. in a new section A.4.3.7 or by expanding on the existing example in section A.3.10.  Any general text to be captured will depend on what approach we take to solving the specific example, but companies are invited to provide candidate text or general guidance for discussion.

Question 5.1: What guidance should we provide for the general case of extending lists without ToAddMod?

| Company | Comments |
| --- | --- |
| MediaTek | This ASN.1 idiom is definitely unfortunate, but it may not always be avoidable. Basically, we think the agreed-upon option should be documented as a general practice for the case that such an extension is necessary. A couple of principles that should be captured: - If option A is selected, we should document that the UE needs to remember which entries were configured by the extension field, as this is a departure from the usual extension practice of considering the fields as a single combined list. - If option B is selected, we should make the extension field Need R instead of Need M. Some clarification is still needed about which entries are released, but the combination of Need M with “release when absent” behaviour is confusing. - If option C is selected, the field description should clarify the network behaviour—we shouldn’t rely only on the general guidance to specify what is expected, as it’s easy for general principles like this to be overlooked in specific cases. - In general, we don’t see that any of these solutions are really in conflict with the principle that lists without ToAddMod are always replaced when signalled. The base and extension lists are separate fields with separate need codes, and we understand the existing guideline (section A.3.10) as applying to a single field. It might be good to change section A.3.10 to state this unambiguously (in the first sentence, “the contents of the field are always replaced”). We think clarification of how the extension works should go in a new subsection of A.4.3 (the non-critical extension section), with a reference from section A.3.10 (the “lists without ToAddMod” section). |

# 5 Discussion (Phase 2)

[To be populated]

# 6 Conclusions

Based on the discussion in sections 4 and 5 above, we propose the following outcomes:

Phase 1

Phase 2

# 7 References

[1]	R2-2106736: “Report of e-mail discussion [AT114-e][022][NR16] RRC II (MediaTek), MediaTek Inc., RAN2#114-e

[2]	R2-2106115: “Extension of candidateBeamRSList set to ‘release’”, MediaTek Inc./Intel Corporation, RAN2#114-e