3GPP TSG-RAN WG2 #115-e                                                                                   R2-21xxxxx

Online, 16-27 August 2021

Agenda Item:	    xx

Source: 	    MediaTek Inc.

Title:		Report of email discussion [Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)

Document for:	    Discussion, decision

# 1 Introduction

This document is a report on the following email discussion, initiated after RAN2#114-e:

[Post114-e][071][NR16] CandidateBeamRSList set to release (MediaTek)

Scope: how UE shall handle the extension field of candidateBeamRSList. The intention is to agree a 38.331 clarification CR in next meeting. Could consider option 2 and option 3 proposed in R2-2106115 as a starting point. This was also discussed in [AT114-e][022].

Intended outcome: Report, agreeable CR.

Deadline: Long

The discussion will proceed in two phases, first to determine an agreeable mechanism for handling the extension field and second to converge on an agreeable CR.  The deadlines are as follows:

Phase 1: Friday 2 July 1700 UTC

Phase 2: Friday 6 August 0900 UTC

# 2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (rapporteur) | Nathan Tenny | nathan.tenny@mediatek.com |
| ZTE | Liu yu | liu.yu3@zte.com.cn |
| Intel | Sudeep Palat | Sudeep.k.palat@intel.com |
| Nokia, Nokia Shanhhai Bell | Tero Henttonen | tero.henttonen@nokia.com |
| Ericsson | Håkan Palm | hakan.l.palm@ericsson.com |

# 3 Background

## 3.1	Original options

The discussion from [1] considered three options drawn from the discussion in [2]:

- Option 1: The UE releases the entire concatenated list, both the entries configured with candidateBeamRSList and the entries configured with candidateBeamRSListExt-v1610.

- Option 2: The UE releases only the extended entries that were configured with candidateBeamRSListExt-v1610.

- Option 3: The release branch is not used, and the UE treats candidateBeamRSList and candidateBeamRSListExt-v1610 as a single concatenated field with Need M.  The extended list candidateBeamRSListExt-v1610 is only included when candidateBeamRSList is included and fully populated.

## 3.2	Updated options for this discussion

In the discussion, option 1 had less support compared to options 2 and 3.  Rapporteur also understands that in continued offline discussion (separate from the official email discussion), a network-based restriction was proposed, in which the network is required to signal the extension (candidateBeamRSListExt-v1610) whenever it wants the extension entries to remain unchanged in the UE, and the “release” option on the extension list is used only when the network intends to reconfigure the UE to a number of entries fitting within the original list.  This option (option C below) disambiguates the UE behaviour by having the network always indicate explicitly the fate of the extension entries.

Accordingly, this discussion considers three options:

- Option A: When candidateBeamRSListExt-v1610 is set to release, the UE releases only the extended entries that were configured with candidateBeamRSListExt-v1610.

- Option B: The release branch is not used, and the UE treats candidateBeamRSList and candidateBeamRSListExt-v1610 as a single concatenated field with Need M.  The extended list candidateBeamRSListExt-v1610 is only included when candidateBeamRSList is included and fully populated.

- Option C: The network is required to signal the extension (candidateBeamRSListExt-v1610) whenever it wants the extension entries to remain unchanged in the UE, and the release option on the extension list is used only when the network intends to reconfigure the UE to a number of entries fitting within the original list.

The details of the options may require some clarification (e.g. how to define the “extended entries” in option A), so the following discussion subsections include space for discussion of the details of each option.

## 3.3	Examples

For clarity, this section illustrates how options A/B/C would operate in two example scenarios.

Example 1: The network reduces the list size while extension entries are configured, and the resulting list is still larger than the legacy list size:

- Network sends a BeamFailureRecoveryConfig containing a fully populated candidateBeamRSList (16 entries) and a partly populated candidateBeamRSListExt-v1610 (2 entries).

- UE concatenates the fields into a single list of 18 entries.

- Network sends a BeamFailureRecoveryConfig containing a partly populated candidateBeamRSList (15 entries) and omitting the candidateBeamRSListExt-v1610.

  - With option A, the UE populates a list of 17 entries, and the handling of a future release indication depends on the interpretation of the option (see section 4.1 below).

  - With option B, the UE populates a list of 15 entries; the 2 entries from candidateBeamRSListExt-v1610 are released.

  - With option C, this step is not allowed; if the network intends to reconfigure the UE to a list of 15 entries, it needs to include the 15 entries explicitly along with the candidateBeamRSListExt-v1610 set to release, and if it intends to reconfigure the UE to a list of 17 entries, it needs to include the 17 entries explicitly.

Example 2: The network reduces the list size while extension entries are configured, and the resulting list fits inside the legacy list size:

- Network sends a BeamFailureRecoveryConfig containing a fully populated candidateBeamRSList (16 entries) and a partly populated candidateBeamRSListExt-v1610 (2 entries).

- UE concatenates the fields into a single list of 18 entries.

- Network sends a BeamFailureRecoveryConfig containing a partly populated candidateBeamRSList (10 entries) and omitting the candidateBeamRSListExt-v1610.

  - With option A, the UE populates a list of 12 entries, and the handling of a future release indication depends on the interpretation of the option (see section 4.1 below).

  - With option B, the UE populates a list of 10 entries; the 2 entries from candidateBeamRSListExt-v1610 are released.

  - With option C, this step is not allowed; if the network intends to reconfigure the UE to a list of either r10 or 12 entries, it needs to include the entries explicitly along with the candidateBeamRSListExt-v1610 set to release.

# 4 Discussion (Phase 1)

## 4.1 Details of option A

During the previous discussion of option A, it became clear that there are two potential understandings of the definition of “only the extended entries”.

- Approach A.1: The UE remembers which list entries were initially configured by candidateBeamRSListExt-v1610, and subsequently treats these as being the extension entries.  Thus, even if the list is later shortened to a length that fits within the original list size, some entries may be marked as “extended entries” and can be released with the extension field.

  - In Example 1 from section 3.3, sending a release would release the 2 entries that were initially configured by candidateBeamRSListExt-v1610, leaving the UE with a list of 15 entries.

  - In Example 2 from section 3.3, sending a release would release the 2 entries that were initially configured by candidateBeamRSListExt-v1610, leaving the UE with a list of 10 entries.

- Approach A.2: The UE treats the entries from the two list fields as a single undifferentiated list (as usual for lists without ToAddMod structure), and the extension field only addresses entries beyond the size of the original list.  Thus, if the list is shortened to a length that fits within the original list size, the UE considers that it has no more extended entries, and setting candidateBeamRSListExt-v1610 to release becomes vacuous.

  - In Example 1 from section 3.3, sending a release would release the 1 entry that exceeds the legacy list size, leaving the UE with a list of 16 entries.

  - In Example 2 from section 3.3, sending a release would have no effect.

Question 1.1: Which of the two approaches do companies prefer, within the scope of option A?

| Company | Preferred Approach | Comments |
| --- | --- | --- |
| MediaTek | A.1 | The extension field is Need M, which means that if it is omitted rather than set to release, it should be considered to maintain its previous contents. This suggests that we should view candidateBeamRSListExt-v1610 as a field separate from candidateBeamRSList, and the UE should be aware of which entries were configured with the extended list (so that it knows what to maintain in case the field is omitted). We don’t find this to be a problem for the implementation to remember, |
| ZTE | A.1 | If UE and NW store the R15/R16 list separately, NW can reconfigure the R15 list and R16 list separately, and this can save the signaling length effectively. |
| Intel | A2 | This is based on our understanding of A1 and A2 as given in response to Q1.2. |
| CATT | A.1 |  |
| Nokia, Nokia Shanghai Bell | A2 but no strong view | Generally, the UE is supposed to concatenate the entries to a single list so A.2 seems most aligned with that option. However, it has also some issues as per our replies for Q1.2 (see below). |
| Ericsson | A.1 | A.1 is more aligned with the “separated list” view, as indicated by MediaTek and Intel. But there are open ends as explained by Intel. |

Question 1.2: Any other comment on the details of option A?

| Company | Comments |
| --- | --- |
| Intel | I tried to summarise my understanding of A1 and A2 but some of it is still not clear to me (shown with ?). Initial condition: # of entries >16 Option A1; Option A2. Legacy list without extension list; Replaces only the elements that was previously signalled by original list?; Replaces only the original list. Extension list without legacy list; Replaces entries signalled previously by ext list?; Replaces entries >16?. Ext list with release; Release entries that were previously signalled by extension list; Release entries >16. Original+ext list (ext list configures new elements); Replaces entries previously signalled by the original list and entries signalled by the ext list; Replaces both lists (conf of ext list allowed if signalled original list is more than 16?). Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases entries signalled by the ext list?; Replaces entries <16 and releases entries >16 ?. Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option A1; Option A2. Legacy list without extension list; Replaces the entire list? Including entries previously signalled by ext list?; Replaces the entries signalled by original list?. Extension list without legacy list; Replaces entries previously signalled by ext list?; N/A. Ext list with release; Release entries that were previously signalled by extension list; N/A. Original+ext list (ext list configures new elements); Replaces both lists (is it allowed if original list is less than 16?); Replaces both lists (allowed if original list is more than 16?). Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases entries signalled by the ext list?; Replaces entries <16 and releases entries >16 ?. |
| Nokia, Nokia Shanghai Bell | Below is our interpretation of the options A1/A2 (red text shows differences to Intel version) Initial condition: # of entries >16 Option A1; Option A2. Legacy list without extension list; Replaces only the elements that was previously signalled by original list, extension list elements remain; Replaces the entire list, i.e. releases also the extended list?. Extension list without legacy list; Adds or modifies entries signalled previously by ext list (i.e. CANNOT be used to modify legacy list entries!); Adds or modifies entries signalled previously either list (i.e. CAN be used to modify legacy list entries!). Ext list with release; Release entries that were previously signalled by extension list (e.g. what if there are 17 entries, 15 of which were originally added with extension list, all 15 of those are released and only 2 remain); Release entries >16, but it's unclear which entries (e.g. what if there are 17 entries, 15 of which were originally added with extension list?). Original+ext list (ext list configures new elements); Replaces entries previously signalled by the original list and entries signalled by the ext list; Replaces both lists (conf of ext list is only allowed if signalled original list is more than 16). Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases (any) entries signalled by the ext list?; Replaces entries <16 and releases entries >16, but it's unclear which entries (e.g. what if there are 17 entries, 15 of which were originally added with extension list?). Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option A1; Option A2. Legacy list without extension list; Replaces the entire legacy list, (i.e. only entries configured by the legacy list, extended elements remain); Replaces the entire list (regardless of where the entries were signalled). Extension list without legacy list; Only allowed when legacy list size = 16 (i.e. CANNOT be used to modify legacy list entries); Only allowed when legacy list size = 16, but only to add new entries. Ext list with release; Release entries that were previously signalled by extension list; N/A (if list size < 16). Original+ext list (ext list configures new elements); Replaces legacy list and adds to the extended list (only allowed if legacy list size = 16 and new list size > 1); Replaces both lists (only allowed if new list size is more than 16). Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases entries signalled by the ext list (the resulting list size is always the same as the newly signalled legacy list size, i.e. only those entries remain after this operation); Releases the whole list (i.e. legacy entries and extended list entries) and adds the new legacy list (the resulting list size is always the same as the newly signalled legacy list size, i.e. only those entries remain after this operation). We would note that this is our first assessment on how the options work - there are still questions and ambiguities with both A1 and A2, so we are not sure these are fully correct. |
| MediaTek (2) | Thanks to Intel for the table. Our understanding of the options is as follows in the table format (generally aligned with Intel’s): Initial condition: # of entries >16 Option A1; Option A2. Legacy list without extension list; Replaces only the elements that was previously signalled by original list (entries that were configured by the extension list are preserved); Replaces only the first 16 elements (irrespective of whether they were configured by the original list or the extension list). Extension list without legacy list; Replaces entries signalled previously by ext list (entries that were configured by the original list are preserved); Replaces entries >16 (note: in line with Nokia’s comments above, this assumes that the UE keeps the entries in a known order—e.g., if there are 17 entries, 15 of which were originally configured by the extension list, the UE stores first the two “legacy” entries and then the 15 “ext” entries). Ext list with release; Release entries that were previously signalled by extension list; Release entries >16. Original+ext list (ext list configures new elements); Replaces entries previously signalled by the original list and entries signalled by the ext list; Replaces both lists (conf of ext list allowed if signalled original list is more than 16?) [MTK: We understand this would be possible in principle, but there seems to be no advantage in doing it, and it would be OK to prohibit this configuration to simplify implementations and guard against implementation divergence]. Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases entries signalled by the ext list; Replaces entries <16 and releases entries >16. Initial condition: # of entries <=16 (may have been signalled as original or ext list) Note: After discussion with Intel, we understand that the entries in the first line of this table were accidentally switched in Intel’s comment—we have corrected this in the version below. Option A1; Option A2. Legacy list without extension list; Replaces the entries signalled by original list; Replaces the first 16 entries, irrespective of whether they were originally configured by the original list or the ext list; entries above 16 are preserved. Extension list without legacy list; Replaces entries previously signalled by ext list (entries that were configured by the original list are preserved); N/A. Ext list with release; Release entries that were previously signalled by extension list (entries that were configured by the original list are preserved); N/A. Original+ext list (ext list configures new elements); Replaces both lists (is it allowed if original list is less than 16?) [MTK: See above comment—we think the case when the original list is included with <16 entries could be excluded.]; Replaces both lists (allowed if original list is more than 16?) [MTK: See above comment—we think the case when the original list is included with <16 entries could be excluded. We assume “more than” is a typo here for “less than”, as the original list can’t be more than 16 entries.]. Original+ext list (ext list set to release); Replaces entries previously signalled by the original list and releases entries signalled by the ext list; Replaces entries <16 and releases entries >16. |

## 4.2 Details of option B

Rapporteur understands that option B is fairly straightforward and there may not be many details that need clarification.  This section is provided for any comments on the details of option B.

Question 2.1: Any comment on the details of option B?

| Company | Comments |
| --- | --- |
| ZTE | We think the release branch(i.e. release in SetupRelease) can be used in option B, and the need code of candidateBeamRSListExt-v1610 can be ‘Need M’. For option B ,we think the key concerns are: - The UE treats candidateBeamRSList and candidateBeamRSListExt-v1610 as a single concatenated list; 2) The first 16 entries are configured by candidateBeamRSList, and the entries over 16 are configured by candidateBeamRSListExt-v1610; 3) The need codes of candidateBeamRSList and candidateBeamRSListExt-v1610 all are ‘Need M’; 4) If NW wants to release the entries over 16, NW uses release branch of candidateBeamRSListExt-v1610. For example, Step1 (NW wants to add 16 entries): NW only includes 16 entries by candidateBeamRSList; Step2(NW wants to add 6 more entries, and maintains the first 16 entries unchanged): NW only includes 6 entries by candidateBeamRSListExt-v1610(setup), does not include candidateBeamRSList( because the field is ‘Need M’, when the UE receives the second message, the UE stores the new 6 entries concatenated to the first 16 entries. After processing the second message, the UE has 22 entries. ) Step3(NW only wants to modify any entry from the first 16 entries): Because the later 6 entries are unchanged, NW only includes the first 16 entries by candidateBeamRSList; Step4(NW only wants to modify any entry from the later 6 entries): Because the first 16 entries are unchanged, NW only includes the later 6 entries by candidateBeamRSListExt-v1610(setup); Step5(NW wants to delete any 10 entries): Because there are 12 remaining entries after 10 entries are deleted, NW should include the 12 remaining entries by candidateBeamRSList, and simultaneously include release command by candidateBeamRSListExt-v1610(release). After processing this message, the UE stores the 12 remaining entries. |
| Intel | The main “motivation” of option B is that it follows the principle we had previously agreed that non-AddMod lists are always replaced and there is no delta configuration of partial replacement or release of the elements. It is also simple as there is only one list in the UE and network and behaviour is common for the whole list. There is no release mechanism but then there was no release of the original list anyway. I tried to summarise my understanding of option B: Initial condition: # of entries >16 Option B. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; Replaces entries >16. Ext list with release; N/A. Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); N/A. Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option B. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; N/A. Ext list with release; N/A. Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); N/A. |
| Nokia, Nokia Shanghai Bell | Below is our interpretation of the option B (red text shows differences to Intel version) Initial condition: # of entries >16 Option B. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; Adds or modifies entries when list size >16 (otherwise not allowed). Ext list with release; N/A (i.e. the list can never be released fully and will always contain at least one entry). Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list only allowed if signalled original list is more than 16). Original+ext list (ext list set to release); Either 1) N/A OR 2) Replaces the entire list (i.e. UE releases the entire list and replaces it with the legacy list). Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option B. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; N/A (only allowed if legacy list size == 16). Ext list with release; N/A (list size < 17). Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list only allowed if signalled legacy list size = 16). Original+ext list (ext list set to release); Either 1) N/A OR 2) Replaces the entire list (i.e. UE releases the entire list and replaces it with the legacy list). |
| MediaTek (2) | To ZTE’s comment, we have some confusion about taking option B but also allowing the release branch to be used. Doesn’t that bring us back to the question that options A.1 and A.2 attempt to answer—which entries exactly should be released when the extension list is set to release? Nokia’s table above also seems to reflect some cases in which the release branch would be allowed. We understood that the guiding point of option B was to avoid using the release branch. |

## 4.3 Details of option C

Option C was introduced in informal discussion and may need some analysis to make sure that all the implications are understood.  Rapporteur understanding is that this option is intended to have no UE impact (i.e., to be compatible with any UE handling of the release branch), since the release branch is only used when the original field is populated and the list fits within the original entries.

Question 3.1: Companies are invited to provide details of their understanding of option C.

| Company | Comments |
| --- | --- |
| MediaTek | Our understanding is described in the paragraph above; this option should be possible to specify without UE impact. As in the examples from section 3.3, we think it successfully disambiguates the handling of the extended entries. |
| ZTE | For option C, when NW only wants to modify R16 list, whether NW needs to include R15 list or not? (i.e. Do the UE and NW need to store the R15 list and R16 list separately? If yes, we think option A.1 is better than option C) In addition, the behavior that ‘the network is required to signal the extension (candidateBeamRSListExt-v1610) whenever it wants the extension entries to remain unchanged in the UE’ conflicts with the need code ‘Need M’ of the field candidateBeamRSListExt-v1610. |
| Intel | The description of C in the introduction is not fully self contained in our view. We think option C is similar to option B in that a common list is maintained in the network and UE except that the release is still applicable for option C and it avoids the signalling scenarios which could avoid some ambiguous scenarios based on network restrictions. I tried to summarise my understanding of option C: Initial condition: # of entries >16 Option C. Legacy list without extension list; N/A. Extension list without legacy list; N/A. Ext list with release; Release entries >16. Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); Replaces entries <16 and releases entries >16. Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option C. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; N/A. Ext list with release; N/A. Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); Replaces entries <16 and releases entries >16. |
| Nokia, Nokia Shanghai Bell | Below is our interpretation of the option C (red text shows differences to Intel version): Like Intel, we also see similarities to option B, and also to option A in some sense. Initial condition: # of entries >16 Option C. Legacy list without extension list; N/A (network has to explicitly release the ext list entries!). Extension list without legacy list; Either 1) N/A or 2) allowed, and can modify any field entry. Ext list with release; Either 1) Release entries >16 (signalled via extension) or 2) N/A (network has to signal the original list together with "release" to avoid ambiguities). Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); Replace both lists (i.e. legacy list replaces entries <16 and all other entires are released). Initial condition: # of entries <=16 (may have been signalled as original or ext list) Option C. Legacy list without extension list; Replaces the entire list. Extension list without legacy list; N/A (unless legacy list size == 16, in which case new entries can be added). Ext list with release; N/A (legacy list is not affected by the ext list). Original+ext list (ext list configures new elements); Replaces both lists (conf of ext list allowed if signalled original list is more than 16). Original+ext list (ext list set to release); N/A (since the extension list was not present!). As with other options, there are a few unclarities also here that need more discussion. |

Question 3.2: Do companies understand that option C can be specified/implemented without UE impact?

| Company | Y/N | Comments |
| --- | --- | --- |
| MediaTek | Y |  |
| Intel | Most likely | It depends on exact definition of C. Based on our understanding of C as captured above, it is quite likely that it is aligned with any possible UE implementation. |
| Nokia, Nokia Shanghai Bell | Depends | We had slightly different interpretation on all of the options, so would like to hear the UE vendor views. What matters the most is that 3GPP makes it clear how the extension works and ensures there are no inter-operability issues. |
| Ericsson | Y | We also do not see any case (possible UE impl) that is not covered by Option C |

Question 3.3: Any other comment on the details of option C?

| Company | Comments |
| --- | --- |

## 4.4 Preferred option

Companies are invited to indicate their preferred option (A/B/C).

Question 4.1: Which option do companies prefer among options A/B/C?

| Company | Preferred Option | Comments |
| --- | --- | --- |
| MediaTek | A or C | We have some concern about option B, as it effectively changes the need code of candidateBeamRSListExt-v1610 from Need M to Need R. We are fine with the other two options. |
| ZTE | prefer A.1, then B | We prefer option A.1. But if companies think option A.1 has NBC issue, option B is ok to us. @ MediaTek: For option B, we think it is unnecessary to change the need code of candidateBeamRSListExt-v1610 from Need M to Need R. We can use release branch also. Please see our comments in 4.2. |
| Intel | B, C or combination of B and C (see comments) | We particularly don’t like option A1 as it seems quite complex for the UE to remember which list an entry was signalled in. We also have several open points with regard to option A1 which is not clear to us as identified in the table in Q1.2. We support UE has a single list containing entries signalled by both lists without any additional differentiation. Option B and C are similar in our understanding in terms of how the list is maintained by the UE. Option B does not use Release branch, while option C restricts signalling just the ext. A combination of B and C could also be considered. A2 is not entirely clear to us (as indicated in the table on A2 behaviour) – if it is clarified, it may also be acceptable. |
| CATT | A.1 | From our point of view A.1 is reasonable and simpler. The ext list introduced in R16 was so that if it released the UE only keeps the R15 list. |
| Nokia, Nokia Shanghai Bell | B or C, but depends on exact details | We don't like option A1/A2 as they seem to have several open questions. B seemed preferable initially, but it's true that C could have some benefits in simplicity. But no matter what, we think it's important to make this topic clear. |
| Ericsson | B/C | Conceptually, we agree with Intel comments above on B/C above, on single list containing entries signalled by both lists. And aligns to what we have agreed earlier. Option A depends on UE impl. |

## 4.5 Text for the general case

The difficulty of this example seems to suggest that we should have some general guidance in the spec for extending a list without ToAddMod, e.g. in a new section A.4.3.7 or by expanding on the existing example in section A.3.10.  Any general text to be captured will depend on what approach we take to solving the specific example, but companies are invited to provide candidate text or general guidance for discussion.

Question 5.1: What guidance should we provide for the general case of extending lists without ToAddMod?

| Company | Comments |
| --- | --- |
| MediaTek | This ASN.1 idiom is definitely unfortunate, but it may not always be avoidable. Basically, we think the agreed-upon option should be documented as a general practice for the case that such an extension is necessary. A couple of principles that should be captured: - If option A is selected, we should document that the UE needs to remember which entries were configured by the extension field, as this is a departure from the usual extension practice of considering the fields as a single combined list. - If option B is selected, we should make the extension field Need R instead of Need M. Some clarification is still needed about which entries are released, but the combination of Need M with “release when absent” behaviour is confusing. - If option C is selected, the field description should clarify the network behaviour—we shouldn’t rely only on the general guidance to specify what is expected, as it’s easy for general principles like this to be overlooked in specific cases. - In general, we don’t see that any of these solutions are really in conflict with the principle that lists without ToAddMod are always replaced when signalled. The base and extension lists are separate fields with separate need codes, and we understand the existing guideline (section A.3.10) as applying to a single field. It might be good to change section A.3.10 to state this unambiguously (in the first sentence, “the contents of the field are always replaced”). We think clarification of how the extension works should go in a new subsection of A.4.3 (the non-critical extension section), with a reference from section A.3.10 (the “lists without ToAddMod” section). |
| Intel | I think we should differentiate the general case going forward from what is agreed for this particular case. We have already captured the following: Upon reception of a list not using ToAddModList and ToReleaseList structure, the UE shall delete all entries of the list currently in the UE configuration before applying the received list and shall consider each entry as newly created. This applies also to lists whose size is extended (i.e. with a second list structure in the ASN.1 comprising additional entries). This implies that Need M should not be used for fields in the entries of these lists; if used, UE will handle such fields equivalent to a Need R. Non-AddMod lists will always be fully replaced and there is no mechanism for partial release or update of the entries. The issue for this specific field came about because we didn’t have a release for the original list and had a release for the ext. That won’t happen for the future releases. In the future, we won’t have a scenario where we have an original list that cannot be released. And we won’t have the scenario where there is ambiguity on release of the extension list. The current text also implies the full list (org and ext) always has to be signalled. |
| Nokia, Nokia Shanghai Bell | In hindsight, it really seems like we should simply NOT use AddMod to extend plain lists: This discussion shows that brings some troubles (despite all the good intentions when we agreed to do so, when we also supported doing that). As Intel says, to some extent following the "legacy list" rule (i.e. always re-signal everything) fits the AddMod-list paradigm as well (where everything is just "one list"). |
| MediaTek (2) | We think Intel’s statement that “we won’t have a scenario where we have an original list that cannot be released” may be a little bit optimistic. It seems difficult to exclude the possibility that a field with no release mechanism is extended in some future release, and even more difficult to guarantee that no one ever again introduces a list without a release mechanism (that could then be extended in the future). So it seems necessary to have some guidance in case this problem arises again (hopefully we can forestall it by having a clear guideline for extending lists without ToAddMod). The existing guideline in 6.1.3 is in conflict with the use of Need M for the extension list, so we think it’s not enough to rely on this guideline to disambiguate the expected UE behaviour. |
| Ericsson | We agree with Intel that what we agree for this particular case need not mean we have to use the same approach in future. |

# 5 Discussion (Phase 2)

[To be populated]

# 6 Conclusions

Based on the discussion in sections 4 and 5 above, we propose the following outcomes:

Phase 1

Phase 2

# 7 References

[1]	R2-2106736: “Report of e-mail discussion [AT114-e][022][NR16] RRC II (MediaTek), MediaTek Inc., RAN2#114-e

[2]	R2-2106115: “Extension of candidateBeamRSList set to ‘release’”, MediaTek Inc./Intel Corporation, RAN2#114-e