3GPP TSG-RAN WG2#115-e	R2-210xxxx

Online, August 9 - 27, 2021

Source: 	Huawei

Title: 	[Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

Agenda Item:	x.x.x

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

Scope: Discuss options based on R2-2106505. Can have multiple phases and ask questions how the solutions work, should discuss technical aspects.

Intended outcome: Report

Deadline: Long

# 2	Discussion

## 2.1	Network-triggered SCG activation

### 2.1.1	With RACH

Among other cases, if the TAT associated with the PTAG of the SCG already expired before entering the deactivated SCG state, it is necessary to perform RACH at SCG activation.

One aspect is which RACH resources to use. Possible solutions are to use:

1)	common RACH resources;

2)	dedicated RACH resources indicated when going to the SCG deactivated state;

3)	dedicated RACH resources indicated in the SCG activation indication.

Q1: Do companies agree with the above descriptions of candidate solutions for RACH resources used at SCG activation? (please add any solution if needed)

| Company | Comments |
| --- | --- |
| OPPO | We agree bullet 1) and 3). For bullet 2), we think it is not good because the dedicated RACH resource will be kept for a long time without using. It is not sure when the dedicated RACH resource will be used, i.e. it is not sure when the SCG is required to be activated. |
| CATT | We agree that the three options listed above can be the RACH resources used to activate the deactivated SCG. For 1), at least it can be used as a fallback method, e.g. when the dedicated RACH resource cannot be used due to the chose beam is not configured with dedicated RACH resource. For 2) and 3), collision can be avoided and delay can be reduced. Out of the three solutions, we prefer option 1 and option 3. |
| Apple | All 3 solutions are possible. However, we have 2 comments: - TAT expiring before UE going into SCG deactivated state: If the TAT expires for the PSCell, then we are wondering about the logistics of the SCG deactivation RRC transaction between the NW and the UE. The UE would be RACHing on the PSCell, while the MCG provides the SCG deactivation RRC message (is our assumption correct)? - We are not sure if it’s efficient to associate TA/TAT with RACH usage at SCG re-activaiton. Pls see our resposnes below for further details. |
| Futurewei | We think at the activation of the deactivated SCG, if TAT already expired, performing RACH is required. Option 1) or 3) should work. The concern with option 2) is that it may unnecessarily lock the dedicated resource for long time. |
| Ericsson | We agree with the listed alternatives. We also think solutions 2) and 3) can be used in combination, i.e. network signals dedicated resources upon SCG deactivation (solution 2), but if SCG deactivation is long and network needs to free the resources, the networks also has the option to reconfigure upon activation (solution 3). Considering that reconfiguration of the dedicated resources is made with RRC reconfiguration, there should from specification point of view be no need to limit when network may perform such reconfiguration, i.e. it can happen at SCG deactivation, SCG activation, or any point of time while SCG is deactivated. |
| NEC | Basically we agree with those as possible solutions (regardless of our preference). For 3), as the SCG activation indication is sent by the MN and the dedicated RACH resource can be configured only by the SN, we assume 3) would mean that the MN needs to wait for SN to prepare the dedicated RACH resource when activating the SCG. |
| Lenovo, Motorola Mobility | The description is fair. Option 1) should be taken as the baseline Option 2) works if we support UE triggered SCG activation via RACH directly, instead of sending indication via MCG. Option 3) is also feasible. |
| MediaTek | We agree the descriptions. We think that option 1 should be the baseline. And we are open for option 2 and 3. |
| Interdigital | We agree with the listed three options, and the network could configure the UE to apply any of them, for example, depending on the resources available. |
| Samsung | We agree to the listed options. All the options are possible. Option 2 or Option 3 would be up to network, i.e. we wonder if either option should be restricted. |
| Qualcomm | Agree with the above candidate solutions. We propose to add the following solution: 4) dedicated RACH resources indicated during the time UE is in SCG deactivated state. This offers flexibility and is useful in case a MAC CE based activation indication is used in which information about dedicated RACH resources cannot be included. |
| LGE | We think all options are possible with comments: If TA expired before entering SCG deactivation state, there is no other option to send UL data w/o RACH procedure. Thus, option 1 is reasonable when reactivating SCG in this scenario. Also, the network definitely can provide the dedicated RACH configuration helping collision avoidance for synchronisation. Thus, option 3 is also reasonable. However, since the network doesn’t know exactly when the UE needs to reactivate SCG, option 2 is less efficient than other options from the perspective of network resource handling. |

|  | 1) Use common RACH resources | 2) Use dedicated resources indicated when going to deactivated SCG state | 3) Use dedicated resources indicated in the SCG activation indication | 4) Other |
| --- | --- | --- | --- | --- |
| Benefits | - If there is no need to reconfigure anything, processing delay could be reduced by using DCI or MAC CE (without any information) instead of RRC message | - If there is no need to reconfigure anything, processing delay could be reduced by using DCI or MAC CE (without any information) instead of RRC message - the dedicated RACH resources could be used for other scenarios, such as fall back to RACH (see solution 2 and 3 in 2.1.2) or UE-triggered SCG activation | - Fully flexible |  |
| Drawbacks | - Risk of collision depending on RACH load for common resources | - Dedicated RACH resources are reserved permanently, which reduces capacity | - It is necessary to use an RRC message, which implies more processing delay than DCI or MAC CE |  |

Q2: Do companies agree with the above comparison of candidate solutions for RACH resources used at SCG activation? (please add any solution if needed)?

| Company | Comments |
| --- | --- |
| OPPO | For the “Drawbacks” for bullet 3), we think the MAC CE or DCI command for SCG activation can also include dedicated RACH resource. Currently, the DCI can indicate dedicated RACH resource to trigger CFRA in some cases, e.g. DL data arrival with UL async. |
| CATT | It is possible to support that the dedicated RACH resource is included in MAC CE/DCI as mentioned by OPPO. There may be some spec impact to support this solution. |
| Apple | We agree with the summary. We also see that the processing delay as not critical. |
| Futurewei | Agree with rapporteur’s analysis. Since the option 2) requires locking more resources for long time, it should only be applied to the services with high delay requirement and high grade of the service. Option 3) is for the RRC activation command, the effect is similar to HO command. |
| Ericsson | - Regarding the processing delay, we also need to list the option of reducing RRC processing time for SCG activation with limited changes to the RRC configuration, which causes less protocol impact compared to DCI or MAC CE. - Bullet 2) drawbacks currently reads “Dedicated RACH resources are reserved permanently”, but if resources are signalled when SCG becomes deactivated, then we assume they are reserved only during when SCG is deactivated. Furthermore, by using a combination of 2) and 3) as we explained above, the network can free the resources at any time if needed. |
| NEC | Yes, agree with Rapporteur summary |
| Lenovo, Motorola Mobility | The comparison is fair. For option 2) it works if we support UE triggered SCG activation via RACH directly, instead of sending indication via MCG. For option 3) we don’t think the delay is big issue though. |
| MediaTek | Yes. The comparison looks reasonable. |
| Interdigital | Agree with the summary, but support Ericsson’s view that the network could revert the decision to reserve the RACH resources for another purpose, if the need arises while the SCG is deactivated, and inform the UE to use other resources or contention based access in the activation command. |
| Samsung | The comparison looks fine but “permanently” for Option 2 seems not appropriate since the network would reconfigure it even for deactivated SCG as Ericsson mentioned. |
| Qualcomm | Agree in general. A few comments: - We should probably not consider DCI activation at this stage since it requires RAN1 involvement and there doesn’t seem to be much support for it. - We support MAC CE based activation. - Our proposed solution 4) (see response to Q1) has similar benefits and drawbacks as solution 2) with an additional benefit that if dedicated resources become available during SCG deactivated, network can indicate them to the UE. |
| LGE | Agree with the rapporteur’s analysis generally. Also agree with OPPO that DCI command can indicates dedicated RACH resource for random access procedure which is initiated by PDCCH order. |

### 2.1.2	Without RACH

In Rel-15/16, at PSCell addition (but also at PSCell change), the UE performs random access towards the PSCell.

By receiving the RA, the network can know that the UE is ready to access the PSCell but the random access procedure also serves other purposes:

-	the UE performs Candidate Beam Detection (CBD) and uses a RA resources associated with the best DL beam, so the network can use this DL beam for initial PDCCH/PDSCH transmissions and the UE will use the corresponding RS as QCL reference for reception;

-	the network can send a TA command in the RAR, which is an offset determined based on timing of the reception of the random access preamble.

In the case of the deactivated SCG, it is almost agreed that the UE is not transmitting/receiving via the PSCell (no formal agreement for PUCCH though).

There are existing scenarios where the UE in RRC_CONNECTED is temporarily not transmitting/receiving via a serving cell but can resume activity towards that serving cell without RA, e.g.:

-	when the UE is in DRX and resumes PDCCH monitoring at the next on-duration period;

-	when the activated BWP for an SCell configured is switched from dormant to non-dormant BWP;

-	when a deactivated SCell configured is activated and the UE starts PDCCH monitoring on the SCell and other activities.

In these scenarios:

-	the timing of resume is known by the network either as a fixed time (DRX case) or as delay requirements with respect to the BWP switching (DCI) or SCell activation (MAC CE) indication;

-	TA is considered accurate if the associated TAT is running;

-	the network chooses TCI states either:

-	the same as before entering DRX;

-	determined based on L1 beam management (using L1 reports/MAC CEs transmitted via other serving cells) for switching from dormant to non-dormant BWP;

-	determined based on recent L3 RRM reports for activation of a deactivated SCell.

Resuming from DRX occurs at a fixed time, not upon a network indication.

The delay for BP switching from a dormant to a non-dormant BWP is between 0.75ms and 3ms depending on the case (see TS 38.133 table 8.6.2-1). This is thanks to low processing delay of L1 indication, no configuration change, maintenance of accurate DL beams and TCI states using transmission of reports/MAC CEs via other serving cells of the same CG.

For activation of a deactivated SCell, the delay requirements is THARQ + Tactivation_time + TCSI_Reporting for activation with MAC CE (see TS 38.133 clause 8.3.2) or TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3ms (see TS 38.133 clause 8.3.4) for activation at SCell addition. For activation at handover, the time for TA adjustment comes in addition to that.

Tactivation_time depends on parameters of the SCell to be activated: FR, measurement cycle, time of the next complete SSB burst, SMTC period and whether it is "known" or "unknown". This last aspect depends on whether the UE has sent a valid L3-RSRP report (with SSB index for FR2) within a certain time period before activation (FR1) or before the last MAC CE for TCI state activation and semi-persistent CSI-RS for CQI reporting (F2) and the same beams remain detectable.

For RACH-less SCG activation:

The following solution was proposed to ensure the TA is valid:

1)	continue running the TAT for the PTAG when entering SCG deactivated state, RACH-less activation is not allowed if the TAT for the PTAG has expired.

The following solutions were proposed to allow the network to select DL beams/TCI states at SCG activation:

1)	the UE performs BFD and RLM based on previously activated TCI states ("implicit configuration") while the SCG is deactivated and reports in case of beam/radio link failure. The network can use these DL beams/TCI states at SCG activation, as long as beam/radio link failure was not reported by the UE (similar to operation when resuming from DRX)

2)	the network uses information from L3 measurement reports (similar to SCell activation)

3)	the network uses L1 measurement reports (similar to switching from dormant to non-dormant BWP)

In option 1) the UE can resume the same TCI states used before SCG is deactivated without the need for the network to indicate any TCI state in the SCG activation indication, while in options 2) and 3), the network provides an indication of TCI state in the SCG activation indication.

Q3: Do companies agree with the above descriptions of candidate solutions for RACH-less SCG activation? (please add any solution if needed)?

| Company | Comments |
| --- | --- |
| OPPO | - The TCI state is configured per BWP. If the network provides an indication of TCI state in the SCG activation indication, the network should first indicate the activated BWP and Serving cell activation state. - The issue is related to the PSCell sate during SCG deactivation. If PSCell is deactivated state, we cannot see the necessary to perform RACH after each PSCell change. After SCG activation, the UE should perform RACH to the PSCell to obtain the synchronization with PSCell. If the PSCell is dormant state, we think the UE will maintain the CSI measurement, beam management and AGC setting. In this case, the UE will also perform RACH after SCG activation because the UE will lose the UL synchronization. Some companies intend to introduce a timer to control TA validity and decide whether RACH procedure is necessary or not. however, we do not think a timer can decide the TA validity, i.e. the TA is valid when the timer is running. In TA maintain procedure, the network will decide to adjust the TA value according to uplink measurement and send delta TA to the UE if the network evaluate the change of TA. The main purpose of TA timer is to control UE whether to lose UL synchronization and also including TA validity. For SCG deactivation, the network cannot send TA command to the UE even if the network evaluates the TA change and furthermore it is also not clear where there is UL signal for the network to evaluate the TA change. So the timer is not enough to decide the TA validity. Even if the TA is validity, the RACH should also be performed even if the TA timer is running if there is BFD declare. The UE will perform RACH to get good beam. If ReconfigurationWithSync is provided by network (SN), RACH to PSCell is also should be performed no matter the TA is valid or not. Proposal: The RACH is always performed to PSCell only when the SCG is activated |
| CATT | We agree that the options mentioned above can achieve RACH-less. For fast activation, we prefer Option 1 and 3. |
| Apple | For TCI state aspect, we see that all 3 options can be used. However, UE performing RLM/BFD is SCG deactivated state is an overkill in trying to speed up SCG re-activation, as there are delays system-wide (Xn delays etc). Op2 (NW using L3 meas) is a simpler and more practical approach and can also help with mobility in SCG. Op3 is also practical in that the NW can request L1 meas at the SCG re-activation. So for us, Op2 and Op3 are both valid and it would be upto the NW to decide. We can also decide on UE assuming the TCI state for PDCCH monitoring at SCG re-activation, based on the RRM UE does on the SCG. On using TA/TAT for RACH-less, we strongly think that this is in-efficient. We are aligned a bit on Oppo’s views on this. TAT is used in CONNECTED mode where the expiry of this timer is meant as a watchdog for NW on not adjusting UE’s TA, all the while in CONNECTED mode with UE’s UL Tx happening. In SCG deactivated state, there is no UL. And NW can keep the SCG deactivated till the right conditions where the SCG is needed, are triggered. This should not be constrained by the TAT. While we do agree that there is some spec changed needed in MAC with respect to TAT and SCG deactivated state, we think it’s better to design the solution based on the merit of the procedure. It is our view that UE can base the validity of whether to do RACH or not based on the RRM activity the UE does in SCG deactivated state rather than the TAT. |
| Futurewei | We in principle agree on the general motivations of the solutions listed above. For fast activation, we think RACH-less is desirable in scenarios it is allowed. We have some comments on TA validation solution: If TAT timer was expired, the TA previously configured by the network is more likely not valid. But there are exceptions, e.g., the UE is in very low mobility and the TA may be still valid after expiry of TAT, or in high mobility high frequency environment, UL sync may be already lost before the expiry of TAT. Additional information should be helpful to improve the reliability of the RACH decision. The network may be able to provide such information/ instruction. Another option is the network based on its knowledge instructs the UE whether to perform RACH, including the network provide updated TA if the network can estimate the TA based on its knowledge on the UE. |
| Ericsson | - RACH-less SCG activation is feasible and reduces the SCG activation delay for cases where SCG was deactivated only for a short time, such that the TAT has not expired, and no beam failure was detected. We also note that options 1) and 2) can be used combined, i.e. network can use information received from L3 measurement reports. - Regarding the comment from Oppo, we consider PSCell to be in deactivated state for deactivated SCG. There is no need to introduce a new timer to control the TA validity. The current timeAlignmentTimer can be reused. We agree that TA adjustment for deactivated SCG is difficult or not even possible if there is no uplink transmission in the deactivated SCG, but it is also not required for short periods of SCG deactivation. By keeping the existing timeAlignmentTimer running, which was restarted when the UE received the last update of the TA value, the timer indicates how long the UE thereafter can consider itself to be UL synchronized. - Regarding the comment from Apple regarding TAT, we are open towards having only network control of whether SCG activation is RACH or RACH-less, but note that this works only if UE triggered SCG activation via SCG is not supported, since in that case the UE needs some means to determine whether RACH or RACH-less SCG activation is to be used. |
| NEC | For 1), it would be good to discuss/clarify which BWP is used/camped while the SCG is deactivated and when the SCG is to be activated. It seems that the current description means it is the same as the latest active BWP. For 2), we understand this L3 measurement report is received via the MN while the SCG is deactivated. For 3), from this description, it is not very clear how this is received by SN. We understand from the next question that it may be received by PUCCH directly even while the SCG is deactivated or via MN (like L3 meas report). To our understanding on the current status/agreements, PUCCH on deactivated SCG has been precluded, whereas only RACH for SCG activation request remains as possible option. |
| Lenovo, Motorola Mobility | When TAT is running and no BFD, UE can of course activate the SCG without doing RACH. Don’t see the reason why not. |
| MediaTek | We agree the descriptions. We understand there is desire to support RACH-less operation for latency reduction although we think that always triggering RACH is fine. Among all the 3 options to indicate TCI state, we prefer option 2 as it is the simplest solution without increasing UE power consumption. |
| Interdigital | RACH less activation should be supported. In addition to just considering that RACH based activation should be applied, we also think it should be possible for the network to configure the UE to keep the UL in sync (i.e. initiate a RACH just to update the TA in case the TAT expires), because that would ensure the SCG activation will be fast enough. One drawback of this is that if the SCG remains activated for a very long time, there may be several RACH procedures initiated to get UL sync again each time the TAT expires. However, our understanding is that proper network behaviour will ensure that an SCG will be released rather than kept hanging in SCG deactivated state for a long time (i.e. when/if not needed) We also support activation based on L1 or L3 measurements, and a dormant BW like approach that we have standardized in rel-16 for dormant SCells. |
| Samsung | We don’t see critical problems with the listed options. However, we wonder if there is any benefit of SCG deactivation except signalling overhead if RACH-less SCG activation is not supported, i.e. it would be almost similar to legacy SCG addition and release. |
| Qualcomm | Agree with the above candidate solutions. We propose to add the following which is a variant of solution 1): 4) UE performs BFD and RLM and reports beam/radio link failure as in solution 1). UE waits for network to reconfigure it with updated beams, RLM RSs, BFD RSs, TCI states. Upon receiving updated configuration from network, UE resumes BFD, RLM. |
| LGE | Agree with the analysis. For DL beam/ TCI state selection, we think it needs to have time to discuss more because there may be a way that network doesn’t need to provide TCI state if the UE can perform beam measurement for TCI selection in SCG deactivation state. But this may require RAN1 input to conclude. |

Below is a tentative comparison of the solutions, in terms of activation delay, power consumption in SCG deactivated state, specification impact and issues or limitations.

|  | 1) Use RS on which the UE performs BFD/RLM | 2) Use L3 measurement reports to select DL beam / TCI state | 3) Use L1 measurement reports to select DL beam / TCI state | 4) Other |
| --- | --- | --- | --- | --- |
| Delay (to be confirmed by RAN4) | - Much lower than RACH (probably similar to previous solution) | - Lower than RACH | - Much lower than RACH |  |
| Power consumption | - Medium | - Low | - High if PUCCH is used for reporting, medium for reporting via MCG |  |
| Specification impact | - need to define RRC report for beam failure - RAN4 needs to define delay requirements | - activation indication may need to indicate TCI state for PDCCH reception - RAN4 needs to define delay requirements | - activation indication may need to indicate TCI state for PDCCH reception - if reporting via MCG, define reporting and transport between MN and SN - RAN4 needs to define delay requirements |  |
| Issues / limitations | - If configured RS for BFD/RLM are not suitable anymore, need to reconfigure them or fall back to RACH at SCG activation | - L3 measurement reporting is triggered by cell results so detectable PSCell beams may change without triggering any L3 measurement report. In that case, the network may select beams/TCI states that the UE cannot detect anymore | - Only periodic L1 reporting is possible on PUCCH, so frequent reports may be necessary to ensure that in most cases, last reported DL beams are still detectable |  |

Q4: Do companies agree with the above comparison of candidate solutions for RACH-less SCG activation? (please provide comments)

| Company | Comments |
| --- | --- |
| OPPO | We do not support RACH-less SCG activation. See comments above. |
| CATT | For 3), the content included in the bullet of issue/limitations should only be applied to the option that the L1 reporting on the SCG side. |
| Apple | We did not agree on UE performing RLM/BFD in SCG deactivated state yet. For us, option-1 is not power efficient for the UE. For op2, if the UE can assume the TCI from previous config, or based on the RS that UE uses for RRM, then TCI state does not need to given as part of activation of SCG (can be upto NW to decide) For op3, the UE can report L1 ‘after’ the SCG activation, which can be much simpler (avoids any change to MCG PUCCH etc..) and during SCG activation, the NW can be defensive (use SSB as TCI ref) for PDCCH decoding, and then request UE to report L1 for better beams…all of this as part of SCG activation (instead of UE reporting these during SCG deactivated state). |
| Futurewei | Agree with some comments: In option 3), if measurement reporting to MN and the backhaul delay between MN and SN is large, a possible alternative is the MN instructs the UE the updated TCI and candidate beam(s) with the SCG, the UE itself determines best beam and send the beam ID in the first message to the SCG. The first message is carried by preconfigured PUSCH resource as for RACH-less HO in LTE. |
| Ericsson | As commented above, we consider RACH-less SCG activation feasible with solution 1) or solutions 1&2) combined. - Regarding power consumption, RAN2 agreed already that UE performs RRM measurements for deactivated SCG, so 2) is baseline. Then the additional power consumption for performing 1) has been shown to be very small, see R2-2101871, UE behaviour in SCG deactivated state, Qualcomm, 3GPP TSG-RAN WG2 #113e, Electronic meeting, January 25th – February 5th, 2021. - Regarding specification impact, SCGFailureInformation message can be updated with limited specification impact to support 1). - Regarding issues/limitations, if configured RS are not suitable anymore, UE will detect this and report BFD to the network, so that network is aware and can reconfigure the UE or RA is triggered upon SCG activation. We don’t see this as an issue, rather as an added alternative compared to always performing RA. |
| NEC | Regarding the power consumption, performing BFD/RLM on deactivated SCG in 1) would cause power consumption. It may be comparable with 3) depending on e.g. L1 measurement report on PUCCH? |
| Lenovo, Motorola Mobility | 1) is the most straight forward way and should be taken as the baseline. |
| MediaTek | Yes. The comparison looks reasonable. For 3), the additional inter-node delay should also be listed as an issue (if sending L1 report via MCG). |
| Interdigital | We think the comparison looks reasonable and we think all the 3 options could be used, based on network configuration. The use of UL SRS to keep the UL in sync while SCG is deactivated, as well as UL sync maintenance when TAT expires (as we have commented in previous question) could also be considered. |
| Samsung | 2) is the baseline. 1) is further optimized one at the cost of UE power, which we are open to discuss. |
| Qualcomm | Agree in general. A few comments: - Though our proposed solution 4) in response to Q3 is mentioned in Issues/Limitations for solution 1), we mention it separately since power consumption may be higher than 1) because UE resumes RLM, BFD upon reconfiguration, but delay may be lower since it is more likely for UE to have a usable beam. - Solution 3): If PUCCH reporting is used and PSCell TA timer expires, there is additional spec impact due to sending of TA commands via MCG to maintain TA with SN. Power consumption in 3) may also be high for periodic reporting via MCG (depends on frequency of reporting). - Solution 2): The limitation as stated can be less severe since in L3 measurement reports it is possible to include beam measurements. Power consumption in 2) may be medium or high if periodic reporting is used (even if MCG is used; there is impact on overall UE power consumption). |

## 2.2	UE-triggered SCG activation

RAN2 agreed to support UE-triggered SCG activation and the following solutions were suggested:

1)	There are no SCG bearers, only split bearers, the primary path is MCG so when the SCG is deactivated, the UE sends BSR/UL data on the MCG leg and the network decides to trigger SCG activation if needed

2)	In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG, then the network can trigger SCG activation

3)	In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG then the SN requests SCG activation to the MN using the SN-initiated modification procedure.

Q5: Do companies agree with the above descriptions of candidate solutions forUE-triggered SCG activation? (please add any solution if needed)?

| Company | Comments |
| --- | --- |
| OPPO | Both split bearer and DC based PDCP duplication cases should be considered. |
| CATT | For 1), the PDCP duplication should be deactivated when deactivate the SCG if the PDCP duplication are configured. In addition, it should be “no SCG RLC bearer which supports UL transmission” instead of “no SCG bearers” in 1). |
| Apple | We agree that all 3 options are possible. Op2 requires additional spec changes. In addition, the UE can trigger RACH for SCG re-activation for MCG recovery. |
| Futurewei | Agree with the description on above 3 candidate solutions. |
| Ericsson | For solution 3) it is not clear to us at which point the UE considers the SCG to be activated. The above description suggests the UE "requests" SCG activation, implying that it is only activated based on a response from the network. If the UE then should wait for an activation command via the MCG, there does not seem to be any benefit for the UE to go directly to the SCG compared to going via the MCG. Another alternative is that the UE considers the SCG as activated after e.g. successful RACH in the SCG as at that point the UE is already performing PDCCH monitoring. Actually, this boils down to how we define "SCG/PSCell deactivated state". In addition, to be aligned with alternatives for network-triggered SCG activation, solution 3) should include the alternative where the UE performs a scheduling request towards the SCG without a prior random access. This is also clear from Proposal 6 on UE-triggered activation in R2-2104317 Summary of AI 8.2.2.3: Activation of deactivated SCG, which states "FFS whether SR can be used instead of RACH (e.g. when TAT is running)". |
| NEC | agree with possible solutions. One point to be clarified for 2) and 3) is whether it is network choice/control for applying either 2) or 3), or intention is to specify only one of two? |
| Lenovo, Motorola Mobility | Yes, the description is fine. Actually, we think all scenarios are reasonable and can be supported. Option 3) also implies when SN receives RACH request from UE of deactivated SCG, SN needs to first confirm with MN before replying RAR to UE. |
| MediaTek | We agree the description. |
| Interdigital | #1 could be a baseline behaviour for handling a UE with only split bearers. However, if 3 is supported (i.e. UE can directly trigger SCG activation), then we think triggering SCG activation directly by the UE even with only split bearers should be supported. For example, when the ul buffer threshold is exceeded for the split bearer is exceeded. We prefer #3 over #2, as direct activation triggering to the SCG will lead to a faster activation As Ericsson has also indicated, we also think that it should be possible to have the SCG activated immediately without the need for inter-node messaging (e.g. UE sends an SR to the SCG if it is still in UL sync with it, and SCG can immediately schedule the UE). |
| Samsung | Yes, but 3) needs to be clarified further. SR (Scheduling Request) can be considered for this purpose since we have not agreed to PUCCH transmission for deactivated SCG yet. |
| Qualcomm | Agree with the above candidate solutions. Comments: - Solution 1): There can be SCG bearers as well as split bearers. We don’t see a need for the restriction of not having SCG bearers. For split bearers, as in 1), network (MN) can decide to trigger SCG activation based on received BSR or UL data from UE. - Solution 3): In this case, it is not clear why SN needs to confirm SCG activation with the MN since SN manages resources for SCG bearers. |
| LGE | For 1), we agree but we also need to consider the case that the primary path is SCG because UE can also initiate SCG (re)activation in this case. For 2 and 3), we don’t think it is necessary to send an indication to MCG since sending via MCG requires additional time to delay for inter-node signalling. If PUCCH transmission is allowed on SCG when the UE wants to reactivate SCG, it is more efficient to send an indication to SCG directly to support fast activation. Since RAN2 hasn’t agreed yet that the UE can perform PUCCH transmission in the deactivated SCG, RAN2 also needs to discuss supporting PUCCH transmission together with this issue. For 3), since there is another option to skip the random access procedure when SCG activation, the UE may also skip the random access procedure in case of UL data arrival on SCG bearers. Thus, in our view, the UE can simply send SR to send UL data to indicate that the UE wants to reactivate SCG. |

Below is a tentative comparison of the solutions, in terms of activation delay, , specification impact and issues or limitations.

|  | 1) There are no SCG bearers | 2) The UE sends an indication to the MCG | 3) The UE initiates random access towards the SCG | 4) Other |
| --- | --- | --- | --- | --- |
| Delay | - No delay to UL data - SCG activation delay is the same as network-triggered SCG activation + sending a notification via the MCG | - UL data and SCG activation delay are the same as network-triggered SCG activation + sending a notification via the MCG | - If SN-initiated modification is used and the SN waits for the MN's response before allocating UL grants, similar to 2 (larger if RACH-less activation as in 2.1.1 is possible) |  |
| Specification impact | -None (SCG bearers are suspended, like | - Define the UL data arrival notification towards the MCG | - Limited if the SN initiates SCG activation and waits for the MN response to allocate UL grants |  |
| Issues / limitations | - The network must allocate an MCG leg for all DRBs |  | - RACH resources must either be common, with a risk of collision, or permanently allocated, which reduces capacity |  |

Q6: Do companies agree with the above comparison of candidate solutions for UE-triggered SCG activation? (please provide comments)

| Company | Comments |
| --- | --- |
| OPPO | Bullet 2 is preferred. |
| CATT | For the specification impact of 1), we think a note can be added in the spec to indicate that the network needs to ensure that no SCG RLC bearers supporting UL transmission is configured when deactivating the SCG. For the “the network must allocate an MCG leg for all DRBs” mentioned in table above, we think it is also only for DRB that supporting UL transmission. |
| Apple | For (3), the UE can perform CBRA (but use the C-RNTI of the SCG), in which case, this is not a limitation anymore? |
| Futurewei | Agree with the comparison in general. In 3) for SCG bearer, wondering the possibility that upon the UE successfully access to the SN, the SN notify the MN while scheduling the UE transmission in parallel. If this is not possible, we agree with that the MN/SN negotiation will introduce more delay. In this case, if random access is required, it normally should be contention based. Without the network instruction before conducting the random access, the chance of allowing RACH-less is low. |
| Ericsson | - For solution 1) the issue is really that if there is UL data arrival on any SCG bearers while SCG is deactivated, UE cannot report it. As a result this may mean that the network does not configure any SCG bearers. We think this is an acceptable limitation, which helps to limit the complexity for SCG deactivation. And also, for solution 1, the text in the specification impact above that the "SCG bearers are suspended" seems not applicable since in solution 1) there are no SCG bearers. - The bullets for Solution 3) above seem to include some assumptions we have not agreed upon yet. So as solution 3) is a bit unclear how it works the issues may be different depending on how solution 3) is assumed to work. For example, in the email discussion report R2-2104317 this solution was described including this proposal: " Proposal 7: For solution 1 in Proposal 6, network cannot reject the SCG activation request from UE ". This suggests a different interpretation of solution 3). So, depending on how the solution is designed, a possible limitation would be that the network cannot reject. Moreover, we have multiple concerns on the solution 3) as described above, which deserves to be mentioned. For example, the RACH procedure combined with the SN/MN interaction seems unclear. When should the UE here consider the SCG as activated? Based on activation command from the MN or when receiving RAR or Msg4 (contention resolution completion) from the SN? And what happens with the SCG activation state if the UE does not receive the confirmation from the network that the SCG is activated? There might therefore be quite some specification impact for this option if it is designed as described above. |
| NEC | Regarding the Delay in 3), it would be good to discuss whether the SN can decide whether to activate SCG or not upon receiving the request directly from the UE. When the SN decides to activate, the SN should inform the MN. This looks different from the network triggered SCG activation (i.e. MN/SN can reject the request from SN/MN). Otherwise, the UE’s direct SCG activation request towards the SN would not be so attractive in terms of the delay. |
| Lenovo, Motorola Mobility | Option 1) is the simplest scenario. Option 2) is similar as the idea of sending a preferred SCG state in UE assistance information Option 3), it can reduce the delay comparing to option 2, if UE needs to perform RACH upon SCG activation. - In option 2, delay comes from UL RRC message + MN SN interaction + DL SCG activation + RACH - In option 3, delay comes from UL RACH request + MN SN interaction + DL RAR Besides, option 3 can work under MCG failure scenario for the purpose of MCG fast recovery. Another option shall be also considered, if TAT has not expired and no BFD, UE may activate SCG by sending data/SR/BSR to SN directly without doing a RACH. |
| MediaTek | The comparison looks reasonable. We think that solution 1) should be baseline. For solution 3), we need further discuss whether the SN has to wait MN response before providing UL grant. If yes, it seems that solution 3) does not provide too much benefit than solution 2). |
| Interdigital | We think a unified solution that is applicable for both 1 (split bearers only) and for also SCG bearer could lead to less standardization work and also faster activation time. Basically, the UE will proceed as legacy, and on finding out that the SCG is suspended, will send a SR if UL is in sync or otherwise initiates a RACH. |
| Samsung | Yes. It seems fine to us. 1) can be implemented by network configuration and 2) can be considered as an extension of UAI message. However, 3) seems not control of network considering that the network should accept UE request regardless of network status. |
| Qualcomm | Agree mostly. - Solution 3): On issues/limitations, since the UE may use SR as well, PUCCH resources may be needed. |
| LGE | Agree with the comparison generally. Also, we think there is an additional option to consider that the UE sends an indication to SCG w/o random access procedure according to the conclusion of 2.1.2. This option may have the same on the aspects of (3) except the limitations. |

Another usage of UE-triggered SCG activation could for fast MCG link recovery for MCG RLF while the SCG is deactivated.

In this scenario, only solution 3) above is suitable. However, it would not be as fast as fast MCG link recovery in Rel-16 because:

-	RACH is needed (while in Rel-16 it is likely that SR can be used)

-	the SN may need to wait for the MN to confirm SCG activation before allocating grants to transmit the MCGFailureInformation message

Q7: Do companies agree with the above observations regarding the use of the deactivated SCG for fast MCG link recovery? (please provide comments)

| Company | Comments |
| --- | --- |
| OPPO | The configuration of fast MCG recovery and SCG deactivation simultaneously should be allowed. The next question is how to support fast MCG recovery for SCG deactivation. Option 1: support fast MCG recovery, then rapporteur’s observations are correct. Option 2: ignore the fast MCG recovery if SCG deactivation and the UE behaviour should be clear, i.e. - If SCG is deactivated and RLF is detected on MCG, the UE trigger RRC Reestablishment procedure even if T316 timer is configured. - If SCG deactivation is received while T316 is running, the T316 is stop and trigger RRC reestablishment procedure immediately. |
| CATT | The above analysis is correct. It can be seen that the delay includes the delay waiting to activate SCG + the delay sending MCG failure information + the feedback delay of the network. It can be seen that the benefit is not obvious compared with directly triggering RRC reestablishment. Therefore, the simplest way is to trigger reestablishment procedure when RLF is detected on MCG while SCG is deactivated or SCG deactivation is received while T316 is running. |
| Apple | We agree. But we do not see it as a limitation that the SN needs to wait for MN to confirm the SCG activation before allocating grants to transmit MCG failure. This is still better than UE releasing the connection and triggering RLF. |
| Futurewei | Agree with the rapporteur’s observation. It can be considered as reestablishment taking deactivated SCG as candidate with priority (similar as CHO candidates). At least there are some advantages to take as the UE context is already established at the SCG. There is still chance of RACH-less. |
| Ericsson | We agree with the observations of the rapporteur. Considering the complexity and open issues around solution 3 discussed above, we think RRC re-establishment can be triggered for the case when SCG is deactivated. |
| NEC | Observations look valid. However, we think this should be re-discussed once the UE triggered SCG activation is confirmed, as this is not really the intention of SCG deactivation. |
| MediaTek | We agree the observations. Triggering MCG failure recovery while SCG is deactivated seems complicate and the gain is unclear to us. So, we prefer just trigger re-establishment in this case. |
| Interdigital | We agree that RACH may be need if the UL with the SCG is not in sync. However, as Apple has also indicated, the SN may allocate the resources immediately without the need to communicate with the MN. |
| Samsung | We agree with the Rapp’s observations. The simplest way would be to trigger RRC re-establishment for MCG failure when SCG is deactivated as the latency from the approach like Rel-16 MCG failure recovery seems not that much. |
| Qualcomm | - We support UE-triggered SCG activation for fast MCG recovery. The comparison should not be with R16 fast MCG recovery but with RRC re-establishment procedure which would have to be performed otherwise upon MCG RLF. - Not in all cases would RACH be needed. E.g., if TAT has not expired and UE has a usable beam, UE could send an SR to SN to trigger activation. - Even if SN seeks MN confirmation for SCG activation before allocating grant to transmit MCGFailureInformation message, the resulting procedure would be faster than RRC re-establishment. - An added benefit of SCG activation is that if MN decides to keep the SN and data becomes available for a bearer requiring SCG resources after activation, the activated SN connection can cater to it immediately. |
| LGE | Agree with observation. However, we are not sure triggering RRC Reestablishment is a good solution from a mobility robustness perspective. Since there is still an available link to recover MCG, there is a benefit not to declare connection failure. Also, since the UE must have cell selection which may require the longest time in the RRC Reestablishment procedure, time delay more increases than the fast MCG recovery procedure in most cases. Lastly, since we think performing the fast MCG recovery is one of the scenarios sending UL data while SCG deactivation, the fast MCG recovery doesn’t need to be differentiated with other scenarios of UL data arrival on SCG. |

## 2.3	SCG deactivation

SCG deactivation can be triggered at any time by the network, by sending an indication to the UE.

The following additional solutions were proposed:

1)	Assistance information: the UE reports that it would like the SCG to be deactivated.

2)	Deactivation request / response: the UE reports that it would like the SCG to be deactivated and the network replies to the UE whether it accepts or rejects the request.

3)	Report preference between deactivation and release: the network can configure the UE to indicate its preference between SCG deactivation and SCG release.

4)	Inactivity timer: the UE can be configured with an inactivity timer and the SCG is deactivated if the timer expires, i.e. no traffic for a certain period (note: unlike 1, 2 and 3, there is no notification to the network).

The above solutions are not mutually exclusive.

Q8: Do companies agree with the above descriptions of candidate solutions for SCG deactivation? (please add any solution if needed)?

| Company | Comments |
| --- | --- |
| OPPO | The SCG deactivation can be up to MN decision without any new information from SN and UE. |
| CATT | Agree with the options listed above, but 3) can also reuse the assistance information message. The indication provided by UE can assist MN make decisions, but it is up to MN to make the decision. |
| Apple | We agree with the descriptions. |
| Futurewei | It appears that the network has enough information on arrival of DL data and UL data (with UE reporting for UL scheduling). The network should be able to make the SCG deactivation decision promptly. Deactivation decision should be made by MN and no additional UE reporting/request is needed. |
| Ericsson | - On solution 1) – this is not a solution proposed but rather an already agreed baseline in RAN2#113-bis-e, i.e. “The UE can indicate to the MN that the UE would like the SCG to be deactivated. FFS on the details (e.g. reusing UAI or existing messages, information included, etc.). Network can configure whether UE is allowed to do the indication”. - On solution 2), it seems to deviate from the baseline of solution 1) and would be built on a completely different principle. Hence higher impact with questionable benefits (see comments on next question). - On solution 3, we think this would be built on the baseline that we already agreed, which is solution one, hence could be considered. - On solution 4, it seems a bit unclear on the use for a shorter timer compared to the DataInactivityTimer. |
| NEC | For 4), it should be clarified who (MN or SN) configures the inactivity timer? probably SN? Then, it should be also highlighted that the MN who does not configure the inactivity timer is not aware of the SCG deactivation unless the SN informs the MN upon timer expiry. This is something new. |
| Lenovo, Motorola Mobility | Description is fair. |
| MediaTek | We wonder the difference between solution 1 and solution 3, it seems that we can combine these two solutions. Solution 3 looks just further reporting granularity compared to solution 1. |
| Interdigital | We agree with the description of the different alternatives. |
| Samsung | Yes. 1), 2), and, 3) would be one of the options to specify the previous agreement on SCG deactivation indication. However, 4) seems not. For 4), if the network configure a short value for inactivity timer, it should do something to keep SCG alive for data inactivity period while UE preferring SCG deactivation may wait for the expiry of inactivity timer if configured with a large value. In this regards, there seems no clear benefit. |
| Qualcomm | Agree with the above candidate solutions. |
| LGE | All options are possible. However, we don’t think sending a request for ‘SCG release’ is necessary. The UE may request SCG due to power consumption or overheating problem which are legacy problems. The network already can handle those problems by the reception of UE assistance information messages. Whether to release SCG is just up to the network decision and this isn’t dependent on the UE’s decision. For the inactivity timer, we think that is one of optimisation. Currently, sending UAI is enough at least in this release. |

The table below tries to capture the benefits, specification impacts and drawbacks of the solutions.

|  | 1) UE assistance information | 2) Deactivation request / response | 3) Report preference between deactivation and release | 4) Inactivity timer |
| --- | --- | --- | --- | --- |
| Benefits | - Allows the UE to reduce resources, e.g. to save power | - Allows the UE to use SCG resources for another purpose (e.g. another subscription) | - Allows the UE to indicate the best method to save power according to UE implementation or current preference | - Saves DL signalling for SCG deactivation |
| Specification impact | - Existing framework could be reused - Needs to decide the details of the indication | - Need to specify configuration, indication and response | - Need to decide whether to use the UE assistance information or a new indication, and specify the details | - Existing sCellDeactivationTimer could be reused, with little modifications |
| Drawbacks | - This method is unsuitable if the UE wishes to make a decision depending whether the network accepts the request now or not (e.g. to determine whether it is possible to use SCG resources for another subscription) | - This method is less appropriate than 1) if the UE can wait an undetermined time for the SCG deactivation | - This method does not say whether the UE actually wishes to save power at the moment, so it less appropriate than | - Does not allow to modify the UE configuration at SCG deactivation, unless that configuration is signalled previously and stored |

Q9: Do companies agree with the above mentioned benefits, specification impacts and drawbacks of candidate solutions for UE-triggered SCG deactivation? (please provide comments, or missing information in your view)

| Company | Comments |
| --- | --- |
| OPPO | For bullet 1), 2), 3), we can not see any necessary to make the UE request the SCG deactivation. For bullet 4), it is too complex for both UE and network side. The SCG deactivation can be up to MN decision without any new information from SN and UE. |
| CATT | We would like the rapporteur to clarify that what “SCG resource” is in “Allows the UE to use SCG resources for another purpose” which is mentioned in the table above. |
| Apple | We agree with the summary. |
| Futurewei | We agree the analysis on the drawbacks. We see the benefits can be achieved without additional UE reporting/request. |
| Ericsson | - For the specification impact on 1), it would be good to clarify what is implied by reusing the existing framework – does it mean to use UE assistance information concept or any particular IE use for e.g. overheating or power saving? - For drawbacks in 1) and benefits of 2), it would be good to further detail the intention/scenario with the UE use of SCG resources for another purpose. - For specification impact on 4), it is not clear to us how sCellDeactivationTimer would be used. |
| NEC | For Drawbacks in 1), we do not think this is the drawback, because it is not really clear what is “use the SCG resources for another subscription”? A possible drawback in 1) would be some more bits are to be used than 2) or 3). Otherwise, the solutions are very similar, except for 4). |
| Lenovo, Motorola Mobility | Comparison is fair. |
| MediaTek | For solution 3), whether it use UE assistance information or other RRC message does not change the functionality. As commented in previous question, it looks like same as solution 1. The problem of assistance information is that the network may not response it and it may just result in useless indicator. For solution 2, we agree that it could be benefit for MUSIM purpose so we prefer to have this solution. For solution 4, the sCellDeactivationTimer is rarely used in current network and we prefer NOT to extend this for PSCell. The only condition to entering deactivation is “no data transmission” in this method. But we believe that there should be other triggering condition (e.g. MUSIM, overheating) to request deactivation. |
| Interdigital | In general, we agree with most of the comparison. |
| Samsung | Yes. It looks fine for comparison. |
| Qualcomm | Solution 1): Our understanding is that network responds immediately indicating accept/reject upon receiving UAI message from UE with deactivation indication. Therefore, the mentioned drawback should not arise. Solution 2): If the request is accepted by the network then it seems UE state does not transition to SCG deactivated as we understand it. If SCG resources are used for another subscription, this looks more like sharing, i.e., TDM, of SCG resources among subscriptions which seems out of scope of this WI. Solution 3): The UE intention is clear – whether it is requesting SCG release or deactivation. The mentioned drawback is not applicable. |
| LGE | We disagree on reporting preference to SCG release. This isn’t a solution that has the same goal for efficient SCG state transition. A way for SCG release would be better to be excluded from this analysis. For the drawback of bullet 1), since the UE cannot make a decision and cannot enter SCG deactivation state autonomously, the description may not be correct. This is because, to deactivate SCG, we think the network always should make a final decision considering MN-SN interaction. |

# 3	Conclusion

…