3GPP TSG-RAN WG2 #115-e	R2-21xxxxx

E-meeting, 09 – 27 August 2021

Agenda item:	8.5.4

Source: 	Samsung

Title: 	Report from email discussion [Post114-e][511][URLLC/IIoT] QoS Solutions (Samsung)

Document for:	Discussion and decision

# Introduction

Following the email discussion [POST113bis-e][506][R17 IIoT] Enhancements based on QoS (CATT), captured in R2-2104897, the following agreements were made at RAN2#114-e (May 2021):

Agreement:

- RAN2 does not consider the Burst Spread parameter in RAN

- The Burst End Time parameter in RAN is out of scope for Rel-17 IIoT WI.

- No specific enhancements in support of Survival Time in UCE will be studied in R17, but we should aim for solutions for Survival time that also work in UCE

- When Survival Time information is provided in TSC AI, RAN action (gNB and/or UE) can utilize it to improve the associated link reliability so that the survival time requirement is met

- Study fast mechanisms for survival time handling and the need

Following a subsequent offline discussion (captured in R2-2106558) during RAN2#114-e, the following additional agreements were made:

Agreements:

1	RAN2 takes the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms)

2	Survival Time triggered proactively based on Sequence Number is deprioritized

3	UE-based reactive solution based on RLC-NACK is not pursued

4	RAN2 will work/study UE-based reactive solutions to address survival time on top of gNB implementation.   RAN2 assumes that gNB implementation solutions on their own are not sufficient.

It was additionally agreed to hold a post-meeting discussion as captured in the following description, to progress the discussion further, in light of the agreements made:

- [Post114-e][511][URLLC/IIoT] QoS Solutions (Samsung)

Scope:  Identification on UE based solutions, technical discussion on solutions, and aim to down-select

Intended outcome: CR ready to be endorsed in RAN2115-e

Deadline: Long

As per the agreements above, RAN2 assumes that that gNB implementation solutions on their own are not sufficient. RAN2 additionally agreed to work on UE-based, reactive solutions to address survival time (ST) on top of gNB implementation. This discussion (as captured in the present tdoc) focuses therefore on UE-based reactive solutions (specifically two options already identified in previous discussions).

| Company | Name | email address |
| --- | --- | --- |
| Fujitsu | Ohta Yoshiaki | ohta.yoshiaki@fujitsu.com |
| Nokia | Ping-Heng Wallace Kuo | Ping-Heng.Kuo@nokia.com |
| Lenovo | Joachim Löhr | jlohr@lenovo.com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| Qualcomm | Sherif ElAzzouni | selazzou@qti.qualcomm.com |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |
| OPPO | Zhe Fu | fuzhe@OPPO.com |
| Sony | Yassin Awad | Yassin.Awad@sony.com |
| III | Grace Liu | graceliu@iii.org.tw |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| Sequans | Olivier Marco | omarco at sequans.com |
| vivo | Boubacar | kimba@vivo.com |

# Phase-I: Collecting input on outstanding issues

## UE-based reactive solution based on HARQ-NACK triggering

As per previous discussion, in its simplest form this option comprises entering ST mode of operation when the UE experiences N consecutive UL transmission failures for the flow/DRB/LCH configured with ST. Transmission failures are detected by receiving HARQ-NACK. In its simplest form, N = 1 and the ST mode is entered (i.e. ST operation is triggered) upon receiving a HARQ NACK.

Q1-1. Do you believe any additional conditions are needed for this type of ST operation (e.g. counting the number of HARQ-NACKs before entering survival time, …)?

| Company | Answer |
| --- | --- |
| Ericsson | Yes. This triggering condition can be generalized to that UE enters a survival time mode when some existing L1 signals (with some conditions) are received. But, for the ST operation itself, the below aspects are missing: - UE must know what to do when such L1 signals (e.g., HARQ-NACK) are received, e.g., if it is only HARQ-NACK, then network must pre-configure resources to be used during the survival time, since HARQ-NACK itself does not indicate resources. [CATT] Correct, but this is rather part of the “configuration” of survival time. We understand Q1-1 to only focus on the trigger condition, which only needs to be the reception of a L1 signal indicating HARQ NACK for a HARQ process. - A packet can be segmented into multiple TBs. How could the network configure the number N here? [CATT] Per agreement 1, the application message size we focus on is in the 20-50 bytes range. The need for multiple TBs is very unlikely. In this rare event, it can anyways be considered that as soon as one TB fails, ST is triggered. That would be harmless. - Suppose UE is already configured with PDCP duplication for normal operation. If one RLC packet is delivered, then there is no need to enter the survival time. It is not clear how this solution would work in this case, given that the conditions seem to be related to TB delivery only. [CATT] In its simplest form, this solution assumes R15 duplication configuration with two legs, where duplication is not initially activated, but it is activated by the UE upon ST trigger. Then it would be strange indeed that NW activates duplication outside ST, but that wouldn’t hurt either and ST trigger would have no effect. But of course this does not rule out any extension where the DRB is configured with more than two legs and duplication is already activated on a subset of legs. Then, entering ST could e.g. trigger duplication activation on some/all of the remaining legs. - What if N is not equal 1? Is the HARQ NACK counting related with the HARQ process ID or a particular LCH data in any HARQ process? Further, is it expected that gNB configures more resources when the HARQ NACK counting is equal N? [CATT] We don’t see any reason/motivation for supporting N>1. |
| Fujitsu | Yes Some additional conditions have been pointed out in the email discussion of R2-2104897. (1) The UE may receive multiple L1 NACKs (i.e. PDCCH indicating retransmission) in case of PDCP duplication by using multiple legs. In such case, quad counting causes too early transition to ST mode, which is improper. (2) Similarly, if MAC PDU is segmented into multiple TBs, multiple HARQ-NACK of TBs may cause ST mode even if there is still enough time for the MAC PDU to reach survival time. (3) If the retransmission interval or RTT become long depending on the gNB’s retransmission scheduling, it may delay the transition to ST mode. (4) NACK feedback is not available in unlicensed band. |
| Nokia | First of all, there is no “HARQ-NACK” in uplink so the terminology may not be correct. We should use “retransmission grant” instead. We basically agree with the points raised by Ericsson above. |
| Lenovo | We agree with Nokia that the intention is that a retransmission grant, i.e. DCI with non-toggled NDI, indicates that a previous transmission was not acknowledged (NACK). Regarding the additional conditions, we think that for any solution that will be adopted, obviously some details/conditions need to be specified. This is simply due to the fact that currently there is no way to activate PDCP duplication via PHY signalling. Therefore, there are solutions proposed where the activation of PDCP duplication is coupled with some other L1 signalling like HARQ-NACK/retransmission grant or CG activation/deactivation. We think that a simple and low standardization-impact solution can be specified for the “HARQ-NACK” based triggering solution as already discussed at length during the various email discussions. |
| CATT | No. Nothing else is needed (we understand this question to only focus on the trigger condition). In its minimum design, the HARQ-NACK trigger can be a dynamic grant for a retransmission of a MAC PDU multiplexing a logical channel which associated DRB is configured with survivalTimeSupport. Such dynamic grant for retransmission already exists and can be reused as is (e.g. PDCCH to CS-RNTI). RAN2 can of course discuss alternate approaches, if needed, with another L1 signal, with RAN1 consulting. We think the case N=1 is sufficient; there is no need to count for multiple HARQ-NACKs. We also address Ericsson’s concerns above. |
| LG | Yes. 1. If a CG transmission fails for a MAC PDU, the network does not know which RB’s data is included in the MAC PDU. Then, after providing HARQ-NACK, the network has to prepare for duplication reception for all CGs associated with the RBs that can be transmitted on the failed CG. This will cause unnecessary radio resource waste. [ZTE] We think this may not be the issue, both UE and network can know that only the configured CG resources would use PDCP duplication. 2. If the MAC PDU includes data from multiple RBs, the HARQ-NACK will trigger ST for multiple RBs in the UE. However, the network cannot know which RBs data are multiplexed in the MAC PDU, and cannot know which CG resource (associated with each RB) will be used. Thus, the network has to prepare for duplication reception for all CGs associated with the RBs that can be transmitted on the failed CG. This will cause unnecessary radio resource waste. [ZTE] We also think this may not be the issue. RBs multiplexing has no impact on determine the packet reception status. 3. If a PDCP SDU (and corresponding RLC SDU) is segmented into multiple RLC PDUs, the transmission failure of a MAC PDU including first RLC SDU segment will trigger PDCP duplication. However, as the RLC SDU is already segmented, the next RLC SDU segment will not be duplicated. The problem is that the network does not know UE’s segmentation and the network should prepare for duplication reception from the next transmission. This will cause unnecessary radio resource waste. [ZTE] Generally we think it’s no need to consider such segmentation. But if it’s considered, we tend to agree this issue exists for the HARQ-NACK based option. But such issue doesn’t exist for Tx-side timer based option as the AN-PDB based timer is in PDCP layer, it can be easy to coordinate with PDCP duplication and handle the multiple RLC PDUs case. 4. HARQ-NACK is provided for a current MAC PDU (packet A), and the UE will trigger PDCP duplication only after HARQ-NACK is received. If the next packet (packet B) arrives before receiving HARQ-NACK, the UE may already process the PDCP SDU (packet B) and delivered to RLC. Then, even if the PDCP duplication is activated, it is only applied to second next PDCP SDU (packet C). If the transmission of next packet (packet B) fails, it already violates ST requirements. In addition, same problem of radio resource waste explained in 3 (i.e. the network does not know from which transmission duplication is applied) should be considered. [ZTE] As transmission delay of HARQ-NACK is related to gNB scheduling and radio link quality, we tend to agree that this issue may happen for HARQ-NACK based option. That means the UE fails to enter ST state timely in such scenario that time interval between packets is too small. But we think such issue doesn’t exist for Tx-side timer option as we assume the configured PDB should match the time interval between packets. |
| Qualcomm | We support the general framework presented in this solution but propose not an additional condition but an alternate condition, An explicit HARQ-NACK feedback is not available in the licensed band. It was mentioned in the last email discussion that “HARQ-NACK” can also mean a DCI retransmission grant with NDI not toggled, which in our view would be the appropriate way of triggering survival mode in licensed band. It is unclear if this understanding carries over in this email discussion. In any case, it should not be assumed that HARQ-NACK is always available (since this issue will have RAN1 implications). To avoid confusion, we prefer modifying the proposal to something like “Survival state/mode is entered when N consecutive retransmissions are triggered at the UE for PDUs carrying survival time communication service”. The exact wording can be worked out, but the point is that the reactive trigger should be the UE’s realization of a failure or multiple failures, not a mandated HARQ-NACK. This behavior would of course be configured by the network via RRC, e.g., when configuring the CG. This gives a simple clear procedure for the UE to follow upon a HARQ failure for a CG carrying a ST flow (or multiple HARQ failures for PDUs carrying ST flow) to enter survival state and activate PDCP duplication for example. As for counting consecutive HARQ failures, we are fine with always setting N=1 as a conservative choice (since this feature is expected to be configured for stringent survival flows only) or configuring N according to the survival time constraint. In case of configurable N failure, then consecutive retransmissions at the UE would need to be counted. |
| ZTE | Yes. We agree with Nokia and Qualcomm that for this HARQ-NACK based option, re-transmission grant (ReTx grant) can be used instead of HARQ-NACK. However, we think using ReTx grant as trigger for entering ST state has the reliability issue. As transmission failure generally occurs when radio condition deteriorates, such ReTx grant may not be successfully sent to UE. If the ReTx grant keeps being lost, the UE cannot enter ST state timely. Moreover, another issue is the uncertainty on the arrival of “HARQ-NACK”. As it’s related to gNB scheduling, it may arrive later than the allowed range of Survival time. For “N”, we also think a single “HARQ-NACK” (no counting) for triggering link reliability would be too aggressive. It may cause that UE enters ST state too early and cause unnecessary increased link reliability for the retransmission of current packet (this may be improper as we assume that survival time is not meant to improve retransmission reliability, but rather subsequent transmission reliability). Therefore, we agree some above comments that counting N failures would be needed. However, as mentioned in previous email discussion, we think it may be improper to just simply count N consecutive retransmissions at the UE, or in other word, it may be not easy for network to configure a suitable N. As the duration between retransmissions may be variable, e.g., depending on the gNB scheduling, such N counting may not be matched with the expected survival time for a packet. For the LG’s comments about multiplexing and segmentation, please see our some understandings above. |
| Huawei, HiSilicon | For a general solution, we shall consider UE-based reactive solution which is triggered by N consecutive HARQ-NACK, or implicitly by DCI retransmission grant, thus we support Qualcomm’s modified proposal. However for the targeted performance requirements of the top 3 rows of Table 5.2-1, we would think N=1 should be configured. |
| OPPO | Yes Regarding L1 signals (e.g., HARQ-NACK), we also think it can be the existing re-scheduling DCI. What we want to mention is that sometimes L1 signals may not be sent to the UE timely and successfully, if DL LBT fails or DL channel quality is poor. Regarding segmentation, we think it may not be an issue, since the maximum message size in the top three rows of Table 5.2-1 from TS 22.104 is 50 bytes. |
| Futurewei | We agree that, in general, the UE-based reactive solution is triggered by N consecutive HARQ failures, which can be indicated by DCI carrying a retransmission grant. |
| Sequans | Yes, agree with Ericsson points. Especially, duplication may (should?) be already configured to reach the required reliability. It was specified in Rel-15 while this concept of ST was not even there. |
| vivo | Yes The value of N is not easy to set considering the following cases: - A MAC PDU may experience retransmission several times. Then UE will receive multiple HARQ NACKs(e.g retransmission grant) of a MAC PDU carrying PDCP data with survival time requirement. - Duplication can be activated for normal operation. Then UE will receive multiple HARQ NACKs of MAC PDUs carrying the same PDCP data which transmitted via multiple legs. If “N” is not set properly, it will make UE enter survival time too early or too late. |

Q1-2. Do you see any need for restrictions to be imposed on the network (e.g. specific carrier spacing, …) for this type of ST operation?

| Company | Answer |
| --- | --- |
| Ericsson | Yes. Even if this is considered as a UE-based reaction solution, the network must pre-allocate resources for UE (e.g., to use for PDCP duplication). Those resources are a waste and cannot be used for other UEs. There were inputs in the last email discussion that this can be solved by gNB implementation to reclaim those resources. But, it was not concluded. Furthermore, as the network vendor, even if those solution may work, it would introduce a lot of and unnecessary restrictions, given there are better alternatives, see Section 2.3. |
| Fujitsu | Yes. (1) PDCP duplication can be activated when the UE enters ST mode. For the UE to immediately activate PDCP duplication, UE autonomous duplication activation (i.e. proactive method) need to be considered, where activation command by MAC CE is omitted. (2) Pre-allocation of PDCP duplication resources can be considered to boost reliability on Uu. |
| Nokia | Yes. Since Rel-16 we have up to 4 RLC entities per DRB, the gNB must pre-configure which set of RLC entities the UE should activate upon survival time state, as it does not make sense for the UE to select the RLC by itself and cause unexpected interferences. Also, we cannot expect the gNB should pre-allocate resources for carriers associated to all 4 RLC entities. |
| Lenovo | gNB needs to pre-allocate some CG resources. However, this would be common with the solution suggested by Ericsson. It’s clear that in order to provide some fast activation/deactivation of PDCP duplication CG resources needs to be already ready/allocated. |
| CATT | Only one restriction: a DRB configured with survival time support should also be configured with duplication. And we agree with Ericsson that the network must pre-allocate resources. But we don't think they are wasted since minimal normative restrictions can be added to prevent UE to use them outside Survival Time, thus allowing gNB to allocate them to another UE. Since gNB knows when Survival Time is triggered in the ST-UE, it can then timely deactivate the twin resources (e.g. Type2-CG) at the other UE. |
| LG | Yes. There are quite many restrictions imposed. Relating to the comments provided in Q1-1, following restrictions should be considered. 1. Each CG should be associated with only one RB. 2. Each MAC PDU should include data from only one RB (i.e. no MAC multiplexing) 3. RLC segmentation should not be allowed. 4. Pre-processing of PDCP SDU should not be allowed. In addition, following restrictions should be considered. 5. Network should provide HARQ-NACK within a very small amount of time (much less than ST). |
| Qualcomm | No. The basic operation does not need network restrictions beyond configuration/implementation. For issues such as carrier spacing restrictions (as suggested by the rapporteur), we tend to think of these as deployment issues that will highly depend on specific use case/implementation of the solutions being discussed, as well as the specifics of those solutions currently being worked out. We also do not agree with “pre-allocation of resources all the time”. As mentioned by CATT, the gNB knows when the UE would be entering Survival Mode (since it has to send a HARQ-NACK/DCI with non-toggled NDI to UE to trigger survival mode), it can also activate a type 2 CG in-time for SN in survival mode for duplication. Once survival mode is exited, the gNB is free to use these resources. That would be “opportunistic/just-in-time allocation for survival mode” rather than “all the time pre-allocation”. |
| ZTE | Yes. For the restrictions mentioned by LG, we agree that restriction 5, e.g., network should provide HARQ-NACK quickly, exists for this HARQ-NACK based option. Moreover, for clarification on CATT response, we think if only PDCP duplication is preconfigured, UE cannot use it immediately after activating PDCP duplication. In other word, after UE activates PDCP duplication, UE needs to be additionally configured with the corresponding uplink resources to send packets. So in order to avoid the delay introduced by configuring CG resources, we think the network needs to pre-allocate PDCP duplication and CG resources at the same time. On the other hand, we think some above mentioned aspects, e.g., pre-allocation of PDCP duplication resources, are the corresponding processes in network for a workable solution, but not restrictions. We agree with CATT and Qualcomm that such pre-allocated resources can be reasonably handled by accurately exiting from the ST state and by gNB implementation, e.g., to prevent UE from using the resources outside Survival Time state and to allow gNB to allocate them to another UE. |
| Huawei, HiSilicon | Agree with Qualcomm, we don’t see obvious restrictions for the network. On the other hand, we are open to discuss the configuration/implementation details as LG proposed to make the ST mechanism work properly. |
| OPPO | Yes - If PDCP duplication is used for ST state, the network needs to pre-config the resources for duplication packet transmission. If the flexibility L1/L2 transmission mechanism is used for ST state, the network at least needs to set the adaptive L1/L2 parameter(s). - It is required that L1 signals is send timely and successfully. |
| Futurewei | Any sensible network deployment would/should ensure that, when serving a QoS flow with ST requirement, the subcarrier spacing configuration and HARQ-NACK timing is set to meet the stringent timing requirement. However, we are not sure if this needs to be “imposed” on the network. |
| Sequans | Yes, agree with Ericsson points. If resources for duplication would be wasted, why just not leave duplication activated. If the gNB has time to use those resources for other UEs, it has also time to allocate the ST UE. |
| vivo | Yes. At least the following restriction should be considered: - NW must pre-allocate resource for UE. For the issue of resource waste, we have the same view with CATT that NW can allocate the resource to other UEs when the NW expects the pre-allocate resource will not be used(i.e. the UE is not in ST mode) . In addition, we wonder if the solution that allocating resource temporarily via type 2 CG activation command can work well. NW does not know which RB’s data is included in the MAC PDU. Then if a CG transmission fails, NW has to provide type 2 CG activation command for all CGs associated with the RBs that can be transmitted on the failed CG. As the current spec only support to activate type 2 CG configuration one by one, we are not sure if the correct type 2 CG can be activated in time. - NW should configured PDCP duplication in advance which will be activated when UE enters Survival time mode. - NW should configured the value of “N” if the value of N is not equal to 1 by default. |

Q1-3. What impact on RAN2 work (e.g. spec impact/workload…) do you foresee for this type of ST operation?

| Company | Answer |
| --- | --- |
| Ericsson | At least the below points must be discussed. - The details on this (non-existent) L1 signal HARQ-NACK. - The details to guarantee (if possible) that the pre-configured resources would not be used, when the survival time is not entered. - HARQ-NACK is per PUSCH transmission and it is not clear how to link this to the flow/DRB/LCH. - What is the value of N, only equal to one or possibly to be configurable by the network? The other important aspect is that the solutions should preferably be applicable beyond the survival time (which is an optional TSCI AI parameter). |
| Fujitsu | There are RAN1 and RAN2 specifications impact. (1) RAN2 needs to look at TS38.321 on how to specify the timer handling, interaction between PHY and MAC, and ASN.1 on timer configuration. (2) RAN1 needs to look at HARQ aspect. |
| Nokia | First of all, related to the triggering signal, we don’t see feasible to introduce a new L1 signal for the purposes of ST triggering, especially considering that RAN1 is not officially involved in this WI objective. Therefore, as also mentioned for Q1-1, we think we need to build on the existing retransmission grant signal and avoid RAN1 impact as much as possible. Besides, we think the points below should be also discussed: - How should the UE identify the corresponding DRB that should enter the survival time state based on HARQ process ID of the retransmission grant? (Similar to Ericsson’s Point 3 above) - How to provide to the UE the new set of radio resources and/or additional RLC entities to be used during ST state - How to ensure UE and gNB have the common understanding what RLC entities will be activated upon survival time state? (assuming Rel-16 PDCP duplication is the baseline) |
| Lenovo | We don’t think that we need to define a new L1 signal (NACK) but can use the existing retransmission grant for this type of solution. The RLC entities which UE should activate for PDCP duplication when entering ST could be preconfigured. |
| CATT | - RRC configuration of: - The DRB supporting Survival Time (e.g. in PDCP_Config I.E.) - The additional resources to be used during Survival Time 2) MAC behavior: - Allowing UE to activate PDCP duplication upon receiving HARQ-NACK (e.g. upon receiving a dynamic grant for a retransmission of a MAC PDU multiplexing a logical channel which associated DRB is configured with survivalTimeSupport) - Preventing the additional resources to be wasted outside Survival Time The simplest solutions should be foreseen to minimize the above specification impacts and associated RAN2 workload. |
| LG | Our comments to Q1-2 can be applied here. 1. Each CG should be associated with only one RB. 2. Each MAC PDU should include data from only one RB (i.e. no MAC multiplexing) 3. RLC segmentation should not be allowed. 4. Pre-processing of PDCP SDU should not be allowed. 5. Network should provide HARQ-NACK within a very small amount of time (much less than ST). In addition, HARQ-NACK design also has the specification impact. |
| Qualcomm | RAN2 work would have to cover the following: - Details of UE behavior in survival mode, e.g., Spec details of UE autonomous PDCP duplication if agreed to by companies. - How the network can configure this behavior at the RRC-level in terms of signaling, configuration, etc. - Standardizing the specific triggers to enter survival time (e.g., N HARQ failures at the UE discussed in Q1-1) and the mechanism to exit survival state (e.g., autonomously vs signaled by the NW). |
| ZTE | Generally agree with more above thoughts on the RAN2 impacts. Based on our understanding, the transmission failures can be detected by receiving DCI retransmission grant from the NW. The details of the L1 signal “HARQ-NACK” doesn’t need much discussion. The following issues still need to be paid attention to: 1) What is the suitable N value configured by the network? A reasonable value is needed to avoid entering the ST state incorrectly. 2) After UE enters the ST state and activates PDCP duplication, we need to consider how to exit ST and deactivate PDCP duplication. Can it be based on the successful re-transmission or the successful transmission of the next packet? And how? |
| Huawei, HiSilicon | We suggest the following aspects to be considered: - Triggering condition to enter ST state, as explained in Q1-1; - Triggering condition to exit ST state, either timer based method or network controlled exiting method; - On UE entering ST state/exiting ST state, we prefer simple/unified solution in R17, e.g. solution based on duplication activation/deactivation. |
| OPPO | In our understanding, the following should be considered, - How to set “N” to enter ST state. - How to pre-configurate resources or adaptive L1/L2 parameter(s) used in ST state. - The detailed UE behavior upon entering ST state. - Whether to introduce an indication to trigger UE-based reactive solution. |
| Futurewei | The following can be considered: - Triggering conditions to enter/exit ST state; - UE behavior when triggering condition is met; - Potential restrictions on the extra CG configured for reliability boosting during ST state. |
| Sequans | Agree with Ericsson points 2/3/4 and Nokia points. |
| vivo | From the view of RAN2, the following issue should be discussed: - The detail on the interaction between MAC layer and PDCP layer, e.g. how to trigger MAC to indicate PDCP layer to activate/deactivate PDCP duplication based on HARQ NACK or retx grant. - The detail on the condition that UE exits the survival time. - UE behavior when UE enters/exits survival time. - RRC configuration for supporting this type of ST operation. |

Q1-4. In your view, how does this type of ST operation meet/fail to meet the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms)?

| Company | Answer |
| --- | --- |
| Ericsson | This solution fails to properly utilize the survival time, i.e., more radio resources are opportunistically and only allocated to the UE when it is in the survival time mode. It is in our understanding that the solutions require gNB to pre-allocate the resources even if UE is not in the survival time, and no conclusion yet if this can be avoided (e.g., resource reclaimed by gNB to use for other UEs). |
| Fujitsu | In case of ST is set to such a short time, this type of ST operation is still supposed to work but not sure if it is proper operation. Entering ST mode is supposed to be unusual situation, where radio conditions may not be so good. In such a situation, relaying fully on HARQ-NACK may not be so good since it may also be suffered from errors. |
| Nokia | It is extremely challenging and restrictive to enable timely HARQ feedback within 0.5ms. In particular, it requires very specific TDD configurations and subcarrier spacing to make sure the retransmission grant can be sent within time interval as short as 0.5ms. We think it imposes too much restrictions to gNB implementation and applicable deployment scenarios. |
| Lenovo | As already mentioned during the previous email discussion, we think that this type of solution can meet the performance requirements. |
| CATT | This solution meets the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms) as long-time quantitatively demonstrated and never-challenged in R2-2100223/R2-2102726/R2-2104900. |
| LG | We believe this solution is not applicable to such a short ST case. |
| Qualcomm | We think that solution coupled with UE autonomous PDCP duplication properly implemented would be capable of meeting the performance requirements of the first three rows of Table 5.2-1. In particular, at the UE, L2 can take a HARQ indication to start duplicating within the next PUSCH for new Tx, which will fall within the performance requirements. |
| ZTE | Generally, we think all the UE-based reactive solutions can meet the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104. But we also agree with Fujitsu that relying fully on “HARQ-NACK” may not be a good choice as it may be suffered from errors, especially in the bad radio conditions. |
| Huawei, HiSilicon | We believe, as commented for Q1-3, a simple/converged solution based on duplication activation/deactivation can meet those performance requirements. |
| OPPO | If L1 signals can be obtained by the UE timely and successfully, this solution can meet the performance requirements. |
| Futurewei | This solution meets the performance requirements. |
| Sequans | Agree with Ericsson. Also we are not sure there was any proper analysis of performance requirements. E.g. what if the HARQ-NACK is lost too? Assuming TDD it would also be possible/likely. |
| vivo | As we mentioned in Q-1, if the value of N is not set properly, it may make UE enters ST too late. From this point of view, we think this solution cannot meet the performance requirements. |

Q1-5. Taking into account your comments above, do you support the UE-based reactive solution based on HARQ-NACK triggering? (Ideally please just answer with a ‘yes’ or a ‘no’, as you will have already made comments on any additional assumptions/constraints.)

| Company | Answer |
| --- | --- |
| Ericsson | No. Detail solutions remain unclear. Also, in general, efficiency/advantage over gNB based solution is not proven. |
| Fujitsu | No. Our position has not been changed since the email discussion in the last meeting. |
| Nokia | No. However, we may accept some HARQ-NACK triggering based solutions, if simple / low standardization-impact solutions are identified. |
| Lenovo | Yes |
| CATT | Yes |
| LG | No. This is not realistic solution, as explained in previous questions. |
| Qualcomm | Yes. Agree with this solution in general with a preference for modifications suggested in Q1-1 regarding the phrasing of trigger condition. |
| ZTE | No. We still prefer the UE-based reactive solution with Tx-side timer trigger. |
| Huawei, HiSilicon | Yes |
| OPPO | No, if L1 signals can not be obtained by the UE timely and successfully. In that case, we prefer UE-based reactive solution with Tx-side timer. |
| III | No strong view. |
| Futurewei | Yes |
| Sequans | No. Agree with Ericsson. |
| vivo | NO. As we observe that this solution rely on the proper setting of N. if we set N=1 by default, it will bring much restriction on NW,e.g. NW cannot configure UE to activation PDCP duplication for normal operation. |

## UE-based reactive solution based on Tx-side timer

As per previous discussion, the triggering timer is (re)started e.g. upon receiving ACK for the previous packet and stopped upon receiving ACK for ongoing transmission. Survival Time is triggered when such timer expires and optionally also upon receiving NACK.

Q2-1. Do you believe any additional conditions are needed for this type of ST operation?

| Company | Answer |
| --- | --- |
| Ericsson | Yes. The followings are not at all clear: the timer starting/stopping condition, and the UE actions when timer expires. For example, in one version, the timer starts when the packet arrives at the UE side, and it has the same duration as the PDB. When timer expires, then the UE enters the survival time mode. Thus, at least the start of the timer is not related with the packet reception status (like HARQ-ACK) and so it should not restart when the HARQ-ACK is received. [ZTE]: As mentioned in R2-2104897, we have the following clarifications: Timer start: We confirm Ericsson’s understanding that the Tx-side Timer is with the value of one AN PDB and started at the reception of the packet in PDCP. Yes, the start of the timer is not related with the packet reception status. Timer expires: If the Tx-side timer expires, UE can determine that the packet is not sent successfully and then enter ST state. Such process doesn’t rely on feedback from network and therefore no delay issue. Timer stop: If the packet is sent successfully before the Tx-side timer expires, UE can stop the Tx-side timer without entering ST state. E.g.: - During running of Tx-side timer, if UE receives initial Tx grant, the UE can assume implicit ACK for previous packet transmission and stop the timer; - If neither initial Tx grant nor ReTx grant is received during the whole Tx-side timer, the UE can also assume implicit ACK for previous packet transmission and stop the timer; - If one or more ReTx grant have been received, an explicit ACK would be needed from gNB to UE if gNB finally successfully receives the packet. Such ACK can be transmitted on PDCCH. As we assume such explicit ACK would be only for a few packets (not each packet), the resource waste would be limited. If no such explicit ACK can be introduced, in a few cases the Tx-side timer would unnecessary expire and trigger UE to enter ST state. But we think this is harmless. The above are mainly focus on how to trigger UE to enter ST state. Moreover, After entering ST state, UE can immediately trigger reliability enhancement of the subsequent link transmission (e.g., activate PDCP duplication) in the most stringent case. UE also can wait for some more time (e.g., according to another Survival Time timer) to activate PDCP duplication a bit late in other less stringent cases. |
| Fujitsu | Yes. The value of the timer length needs to be discussed. [ZTE] For triggering UE to enter into ST state, such Tx-side timer is with the value of one AN PDB. |
| Nokia | Yes. In our understanding, such timer requires fast/reliable ACK/NACK feedback for every UL packet, so it knows when to start or stop the timer. But this is NOT supported by 5G NR. Therefore, introduction of such timer would bring significant impact to specification across different WGs. [ZTE] No. Only upon expiration of Tx-side timer, UE would enter ST state. So for correctly entering ST state (when it’s necessary), it’s no need of fast/reliable ACK/NACK feedback for every UL packet. This is also an advantage of Tx-side timer-based option over the HARQ-NACK based option. As mentioned above, HARQ-NACK based option would rely on re-transmission grant (as implicit HARQ-NACK) to determine the packet transmission failure. But as re-transmission grant generally occurs when radio conditions deteriorate, such trigger would not be so reliable. But yes, for correctly stopping the Tx-side timer, e.g., to avoid entering ST state when it’s unnecessary, we need implicit or explicit HARQ-ACK. But it should be clarified that Tx-side timer doesn’t mandate gNB to always send ACK for each successful transmission. Only if one or more ReTx grant have been received, an explicit ACK would be needed from gNB to UE if gNB finally successfully receives the packet. We assume the overhead caused by this process would be very small. Moreover, if unnecessary entering ST state in a very few cases can be tolerated (we think it’s harmless), it’s also no need to introduce such explicit HARQ-ACK. |
| Lenovo | Yes. We agree with others that some further details need to be clarified regarding the timer starting/stopping condition. [ZTE] Please see our further clarifications above. |
| CATT | Here again, we understand this question to only focus on the trigger condition. And it is our understanding that, if the ST traffic is served by a configured grant, it requires defining some kind of explicit HARQ-ACK signal. [ZTE] Only upon expiration of Tx-side timer, UE would enter ST state. So for normally entering ST state (when it’s necessary), it’s no need of fast/reliable ACK/NACK feedback. This is main difference from HARQ-NACK based option. Moreover, if unnecessary entering ST state in a very few cases can be tolerated, it’s also no need to introduce explicit HARQ-ACK. |
| LG | Yes. - It is not clear whether the ST timer is configured per MAC PDU or PDCP SDU. We assume it is configured per PDCP SDU, but should be confirmed. [ZTE]Yes. 2. It is not clear whether the ACK/NACK is HARQ feedback or PDCP feedback. We assume it is HARQ feedback, but should be confirmed. [ZTE]Yes, the mentioned ACK/NACK is HARQ feedback. With the assumptions above, we have further comments. - All the comments provided in Q1-1 can be applied here. [ZTE] Sorry, we disagree, please see our response in Q1-1. - If a PDCP SDU (and corresponding RLC SDU) is segmented into multiple RLC PDUs, the ST timer shall be stopped only when all RLC SDU segments are ACKed. Thus, the UE should carefully check whether all RLC SDU segments are correctly transmitted before stopping the ST timer. [ZTE] We think such issue doesn’t exist for Tx-side timer based option as the AN-PDB based timer is in PDCP layer, it can be easy to coordinate with PDCP duplication, e.g., easy to know whether all RLC SDU segments are correctly transmitted. - For this ST operation, both ACK and NACK should be defined, but there is no HARQ feedback defined for UL transmission [ZTE] There is no need of HARQ-NACK. And HARQ-ACK is also optionally needed. |
| Qualcomm | Yes - It is unclear to us what ACK/NACK means here. It was agreed last meeting that “UE-based reactive solution based on RLC-NACK is not pursued”, so assuming ACK here means a HARQ-ACK, then this solution assumes the presence of an explicit HARQ-ACK for CG. We note that this condition would have significant RAN1 implications. [ZTE] Yes, ACK here means a HARQ-ACK and introducing such HARQ-ACK has RAN1 impact. But the Tx-side timer option is also workable without such HARQ-ACK. - This type of operation requires the Survival Time for a bearer carrying Time-sensitive traffic to be signaled to the UE for the UE to configure the timer. [ZTE] Yes - It is unclear here the interplay between the timer, ACK and NACK. Assuming the availability of ACK, then is NACK available as well? If NACK is available then it seems that this solution implements the first solution as a branch (to interrupt the timer and trigger survival), but also has another branch that interprets silence of the network (lack of ACK) as a survival mode trigger. In this case one condition is to align the timer and ACK (and possible PDB) to ensure proper entry to survival mode. [ZTE]Please see our above response to Ericsson’s comments. For triggering UE to enter ST state, neither HARQ-NACK nor a branch relying on HARQ-NACK is needed. This is main difference from or advantage over the HARQ-NACK based option. |
| ZTE | In summary, such scheme with UE autonomously entering ST state and triggering link reliability can avoid the following issues: - To avoid erroneous DL command (MAC CE for activating PDCP duplication) in bad radio condition in all the gNB-based schemes. - To avoid erroneous “HARQ-NACK” in bad radio condition in HARQ-NACK based option. Specifically, in this Tx-side timer scheme, only ACK for a few packets would be optionally needed. If it’s available, as we assume at that time the link quality may be still good, the ACK transmission would be no issue and cause little overhead. |
| Huawei, HiSilicon | Agree with those comments questioning on how to determine ACK for the triggering of ST state, we believe it needs further study. |
| OPPO | Generally, we share the similar view as ZTE on how to trigger UE entering ST state, including e.g. timer starts, timer expires and timer stops. Also, this solution can avoid the issue that L1 signals can not be obtained by the UE timely and successfully. |
| Futurewei | It is unclear how to handle of the case where a retransmission grant DCI is sent but not detected, if not receiving a retransmission grant DCI by AN PDB is used as an implicit HARQ-ACK. |
| Sequans | Unclear to us. It seems to complicate even more, especially if we introduce a new ACK. |
| vivo | Yes The granularity and the start/stop condition of TX_side timer is not clear. From our view, the TX_side timer is used to determine whether a PDCP packer is transmitted unsuccessfully. So this timer can be started when the packet arrives at PDCP and be stopped when it is transmitted successfully. If the transmission time of the corresponding packer exceed the maximum AN delay, UE can determine this packet transmission failed and then enter survival time mode. |

Q2-2. Do you see any need for restrictions to be imposed on the network (e.g. sending ACK or NACK for each packet, transmitting a PDCCH every 0.5 millisecond, …) for this type of ST operation?

| Company | Answer |
| --- | --- |
| Ericsson | Yes. - There is a need to send HARQ-ACK for every packet (e.g., 0.5 millisecond) to stop the expiry of the timer, which triggers the survival time operation and needs more resources. [ZTE] No. If HARQ-ACK is available, it’s only needed in the case that one or more ReTx grant have been received. E.g., an explicit ACK would be needed from gNB to UE if gNB finally successfully receives the packet. - Even if this is considered as a UE-based reaction solution, the network must pre-allocate resources for UE (e.g., to use for PDCP duplication). Those resources are a waste and cannot be used for other UEs. There were inputs in the last email discussion that this can be solved by gNB implementation to reclaim those resources. But, it was not concluded. Furthermore, as the network vendor, even if those solution may work, it would introduce a lot of and unnecessary restrictions, given there are better alternatives, see Section 2.3. [ZTE] We assume this is feasible for gNB implementation. |
| Fujitsu | Yes As per the mechanism of timer mentioned above, HARQ-ACK and HARQ-NACK for each packet is needed. [ZTE] No. Only HARQ-ACK is optionally needed for a few packets. It’s also workable without such HARQ-ACK. |
| Nokia | Yes, First of all, a new ‘ACK’ signal per packet would need to be defined – this brings a lot of specification impacts that is extremely difficult (if not impossible) to conclude in Rel-17. Moreover, such new signal would need to be transmitted fast and very reliable to the UE after each successful packet reception (so we can rule out RLC ARQ directly from this point of view). Basically such scheme introduces a lot of control overhead and error- proneness, and may not bring any benefits at all. |
| Lenovo | ACK/NACK feedback is required for every TB. |
| CATT | We agree with Ericsson’s point 1. |
| LG | Similar to Q1-3, following restrictions should be considered. 1. Each CG should be associated with only one RB. 2. Each MAC PDU should include data from only one RB (i.e. no MAC multiplexing) 3. RLC segmentation should not be allowed. 4. Pre-processing of PDCP SDU should not be allowed. 5. Network should provide HARQ-ACK/NACK within a very small amount of time (much less than ST). [ZTE] For comment #5, as clarified above, in this Tx-side timer based option, it’s no need for network to provide HARQ-ACK/NACK within a very small amount of time. Entering ST state doesn’t rely on packet reception status. |
| Qualcomm | Yes - This solution as described here mandates a HARQ-ACK feedback for every packet, i.e., transmitting a PDCCH every Tx period (0.5 ms for the most stringent case). - UE has to implement a new timer and signal that capability to the network. The network is restricted from applying that mechanism unless the UE has explicitly signaled it has a survival timer. [ZTE] Please see above our clarifications. For issue#2, we think no matter which option, configurations from network is needed for UE autonomously PDCP duplication. Such timer can just be part of the configuration. |
| ZTE | Please see above our clarifications. |
| Huawei, HiSilicon | Yes, agree with rapporteur’s comments that ACK or NACK for each packet shall be transmitted. |
| OPPO | The network needs to consider how to pre-configure the resources for duplication packet transmission, or, how to configure the adaptive L1/L2 parameters. |
| Futurewei | It is unclear if the HARQ-ACK is implicit and explicit? If explicitly transmitted, how often? And how to handle the detection errors on explicitly transmitted HARQ-ACK or HARQ-NACK? |
| Sequans | Unclear to us, but if an explicit ACK is needed, it seems an important restriction/change compared to existing behavior. |
| vivo | HARQ-ACK and HARQ-NACK for each packet is needed. As our understanding, retransmission grant with NDI not toggled can be treated as HARQ-NACK. New grant with NDI toggled can be treated as HARQ-ACK. If new grant with NDI toggled is not always available, new ACK singnaling needs to be introduced. |

Q2-3. What impact on RAN2 work (e.g. spec impact/workload…) do you foresee for this type of ST operation?

| Company | Answer |
| --- | --- |
| Ericsson | Similar to the answer to Q1-3. At least the below points must be discussed. - The clear definition of the timer (e.g., the starting point) and how it is supposed to work. - The details on what these (non-existent) L1 signal HARQ-ACK/HARQ-NACK are. - The details to guarantee (if possible) that the pre-configured resources would not be used, when the survival time is not entered. - HARQ-ACK is per PUSCH transmission and it is not clear how to link this to the flow/DRB/LCH. [ZTE] Per our understanding, HARQ-ACK for a MAC PDU link to all the LCHs for the PUSCH transmission. The other important aspect is that the solutions should preferably be applicable beyond the survival time (which is an optional TSCI AI parameter). |
| Fujitsu | There are RAN1 and RAN2 specifications impact. - RAN2 needs to look at TS38.321 on how to specify the timer handling, interaction between PHY and MAC, and ASN.1 on timer configuration. [ZTE] We assume such timer handling can be in PDCP layer and easily coordinate with PDCP duplication feature. (2) RAN1 needs to look at HARQ aspect. |
| Nokia | As mentioned, a new ‘ACK’ signal would need to be defined, which would need to be transmitted fast and very reliable to the UE after each successful packet reception. As mentioned also for Q1-3, we think that RAN1 impact should be avoided as much as possible and thus we dislike this approach. Besides, the following also needs to be discussed: - Starting/Stopping conditions of the timer – can anything other than HARQ feedback be considered ? - If duplication is already activated in the normal state (i.e. at least two RLC entities are activated for a DRB), how the timer starting/stopping conditions can be defined as two or more HARQ feedback on different carriers or even on different MAC entities (DC-based duplication) may be taken into account. [ZTE] We assume this #2 concern applies to both of UE-based options (or even gNB-based option). We agree this scenario needs to be considered. We think in this scenario, it is easier for Tx-side timer based option to check multiple HARQ feedback in combination as the timer is maintained in PDCP layer. However, it may be infeasible for HARQ-NACK based option as different MAC entities check HARQ feedback independently. |
| Lenovo | We see some potential impact to RAN1, e.g. defining some HARQ feedback channel. |
| CATT | 1) RRC configuration: same as Q1-3 2) MAC behavior: - New timer and associated UE behavior - Preventing the additional resources to be wasted outside Survival Time 3) HARQ-ACK signal specification? Similar to Q1-3, the simplest solutions should be foreseen to minimize the above specification impacts and associated RAN2 workload. |
| LG | Our comments to Q2-2 can be applied here. 1. Each CG should be associated with only one RB. 2. Each MAC PDU should include data from only one RB (i.e. no MAC multiplexing) 3. RLC segmentation should not be allowed. 4. Pre-processing of PDCP SDU should not be allowed. 5. Network should provide HARQ-ACK/NACK within a very small amount of time (much less than ST). In addition, following should be considered 6. Configuration of ST timer. Per MAC PDU or per PDCP SDU? 7. Start/stop condition of ST timer. 8. HARQ-ACK/NACK design. |
| Qualcomm | - Details of UE behavior in survival mode, e.g., Spec details of UE autonomous PDCP duplication if agreed to by companies. - How the network can configure this behavior at the RRC-level in terms of signaling, configuration, setting up the timer, etc. - Standardized UE timer to track survival state. - Specific behavior of the timer regarding how/when to enable/disable increase in reliability i.e., starting and stopping conditions, how to deal with NACK, etc. - Introducing HARQ ACK and/or NACK feedback signaling. - RRC level signaling to communicate the survival timer capability at the UE. |
| ZTE | We give some brief responses to above mentioned impacts. As we think there is still ambiguity in companies understanding for this Tx-side timer based option, the specification impacts may need re-consideration. |
| Huawei, HiSilicon | - On triggering condition to enter ST state, as for last question, we believe further study is needed on determining ACK. - On triggering condition to exit ST state, we need to consider how to determine ACK for a packet as well. - We think currently no explicit or implicit signaling could be reused as ACK. The workload seems to be very high if we try to introduce new L1/L2 signaling for this purpose. |
| OPPO | In our understanding, the following should be considered, - Configure/use Tx-side timer in PDCP layer. - How to pre-configurate resources or adaptive L1/L2 parameter(s) used in ST state. - The detailed UE behavior upon entering ST state. - Whether to introduce an indication to trigger UE-based reactive solution. |
| Futurewei | HARQ-ACK or HARQ-NACK signaling. |
| Sequans | Agree with Ericsson/Nokia/Fujitsu points. |
| vivo | From the view of RAN2, the following issue should be discussed: - The detail on the start/stop condition of TX_side timer. - UE behavior when TX_side timer expires. - UE behavior when UE enters/exits survival time mode. - RRC configuration for supporting this type of ST operation. |

Q2-4. In your view, how does this type of ST operation meet/fail to meet the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms)?

| Company | Answer |
| --- | --- |
| Ericsson | The same as the answer to Q1-4, i.e., this solution fails to properly utilize the survival time, i.e., more radio resources are opportunistically and only allocated to the UE when it is in the survival time mode. It is in our understanding that the solutions require gNB to pre-allocate the resources even if UE is not in the survival time, and no conclusion yet if this can be avoided (e.g., resource reclaimed by gNB to use for other UEs). Additionally, it introduces a PDCCH overhead every 0.5 millisecond, as described in Q2-2. |
| Fujitsu | In case of ST is set to such a short time, this type of ST operation is still supposed to work but not sure if it is proper operation. Entering ST mode is supposed to be unusual situation, where radio conditions may not be so good. In such a situation, relaying fully on HARQ-ACK to (re)start Tx-side timer may not be so good since it may also be suffered from errors. |
| Nokia | We can’t envision how this scheme would work without introducing new per packet ACK/NACK signal for UL, which has to be fast enough. |
| Lenovo | In general this type of scheme could also meet the performance requirements. However we see the need of introducing some HARQ feedback 25ignaling, e.g. like DFI. |
| CATT | This solution should meet the requirements of the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms) if the timer is configured appropriately. |
| LG | We believe this solution is not applicable to such a short ST case. |
| Qualcomm | There are some variables regarding the exact behavior of the timer that will determine that, but at face value, we do not expect this option to cover any scenarios not covered by the first HARQ-NACK option. |
| ZTE | Please refer to our clarifications in Q2-1. Generally, we think all the UE-based reactive solutions can meet the performance requirements of the top 3 rows of Table 5.2-1 from TS 22.104. |
| Huawei, HiSilicon | If we use a simple solution, we believe the performance requirements could be met. |
| OPPO | Yes, it can meet the performance requirements. |
| Futurewei | It depends. Need more details on how often the explicit signaling of HARQ-ACK/NACK is transmitted and how detection errors are handled. |
| Sequans | Same comments as 2-1. |
| vivo | From our understanding, this type of scheme could meet the performance requirements even if new HARQ-NACK/ACK signaling is not introduced. |

Q2-5. Taking into account your comments above, do you support the UE-based reactive solution based on Tx-side timer? (Ideally please just answer with a ‘yes’ or a ‘no’, as you will have already made comments on any additional assumptions/constraints.)

| Company | Answer |
| --- | --- |
| Ericsson | No. Detail solutions remain unclear. Also, in general, efficiency/advantage over gNB based implementation is not proven. |
| Fujitsu | No. Our position has not been changed since the email discussion in the last meeting. |
| Nokia | No |
| Lenovo | Our preference would be the “HARQ-NACK” based solution. |
| CATT | No. We prefer the HARQ-NACK based approach as it does not require the permanent HARQ-ACK feedback from gNB. |
| LG | No. This is not realistic solution, as explained in previous questions. |
| Qualcomm | No We do not support introducing a new timer at the UE with no clear benefits over the simpler first option as we mentioned in Q2-1, thus, first proposal is preferable to us. |
| ZTE | Yes. This Tx-side timer based option define a more reasonable trigger for entering ST state. It’s no need of HARQ-NACK and also be acceptable without HARQ-ACK. Moreover, it can overcome the unreliable issue and uncertain issue of arrival of “HARQ-NACK” in the HARQ-NACK based option. |
| Huawei, HiSilicon | No. Our concern is mainly on the signaling aspect and the needed standardization effort, as explained for Q2-3. |
| OPPO | Yes. Tx-side timer-based solution is better since, for this solution, L1 signals issue mentioned in Section 2.1 seems not a bottleneck. |
| III | No. |
| Futurewei | No for now. But open to more study on this approach. |
| Sequans | No. |
| vivo | Yes |

## Other UE-based reactive solutions

Q3-1. Do you support any other UE-based reactive solutions in addition to the two approaches discussed above? Please provide details.

| Company | Answer |
| --- | --- |
| Ericsson | UE-based reactive solution based on “CG type 2 DCI (de)-activation command”. This is in principle similar to the solutions in section 2.1 and with more details on how it is supposed to work. Upon reception of a L1 CG activation/deactivation command, UE does not only activate/de-activate the indicated configured grant but also activates/de-activates PDCP duplication for a RLC entity. The RLC entity, for which to be activated/de-activated by the CG activation/de-activation command, is configured by RRC, e.g., the logical channel config for the RLC entity contains a CG index. If this CG is activated, then this RLC entity for duplication is activated. This solution addresses all Ericsson’s concern for the UE-based reactive solution based on HARQ-NACK triggering: - The DCI command is the existing CG-type 2 (de)-activation command. No need to introduce a new DCI command. - The resources are allocated by CG-type 2 activation and only upon UE entering survival time mode, i.e., there is no need for pre-configuration. - The CG activation command can link (via RRC configuration) to the LCH/RLC entity. - The solution is general and can be utilized in other cases where there is a need for quick PDCP duplication activation by DCI command. Note that, this is a spec enhancement beyond the gNB based solution proposed by Ericsson before in R2-2106413 to address various concerns raised in the email discussion R2-2106558. |
| Fujitsu | It is assumed that the UE enters the ST mode in case of data transmission fails on CG. The retransmission is assumed to be performed by DG, for which PDCCH indicating retransmission is issued to the UE. The PDCCH indicating retransmission could be used for UE-based reactive solution. The most important reaction is PDCP duplication activation to boost the reliability. The PDCP duplication would be activated autonomously without any signaling from gNB including PDCCH command and MAC CE. This is because there could be some radio problems in the ST mode, where the amount of signaling would be reduced. |
| Nokia | If we really have to introduce any UE-based reactive method, in our view it makes more sense to use a mechanism to gauge whether a packet is late or missing on arrival from the upper layer given that the expected arrival time of each packet of deterministic traffics should be known. This allows the 5G RAN to handle situations where a packet is lost or significantly delayed in the upstream. This cannot take air-interface failure into account, but we think it is a good complementary scheme on top of whatever scheme we will introduce for air interface failure. |
| Lenovo | We think that the solution addressed by Ericsson is similar to the “HARQ-NACK” based solution. gNB basically triggers fast PDCP duplication activation by 28ignaling a retransmission grant respectively PDCCH CG activation. Therefore we don’t think that there is a difference in terms of CG resource allocation usage. In both schemes the CG resources which UE is supposed to used when PDCP duplication is enabled are preconfigured and explicitly enabled by gNB by means of L1 signalling, e.g. either retransmission grant or CG activation grant. In the solution proposed by Ericssson, some additional L1 signalling would be required, PDCCH based CG activation/deactivation is sent in addition to retransmission grant. |
| LG | We think if ST is very short, no solution can meet the ST requirements. In this case, the UE should be configured with PDCP duplication from the beginning. The dynamic control of PDCP duplication is achieved by already defined PDCP duplication activation/deactivation MAC CE. |
| Qualcomm | We support the first proposal, but we prefer modifying the proposal to something like ““Survival state/mode is entered when N consecutive retransmissions are triggered at the UE for PDUs carrying survival time communication service”. The exact wording can be worked out, but the point is that the reactive trigger should be the UE’s realization of a PDU failure (or multiple failures), not a mandated HARQ-NACK. This behavior would be configured by the network via RRC. This gives a simple clear procedure for the UE to follow upon 1/N HARQ failure(s) for a CG carrying a ST flow to enter survival state. Note that our proposed wording allows the UE to infer a HARQ failure from a DCI with the NDI not toggled for the HPN carrying a TSC PDU (identified by a specific bearer). Thus, it does not mandate explicit HARQ-NACK feedback, which is a big change to the current standard that can have RAN1 impact and needs new L1 signaling. |
| ZTE | Till now we cannot change our mind and cannot accept the above Ericsson’s proposed solution. We think it has similar issue as HARQ-NACK based option, e.g. the issue of low reliability in bad radio condition. |
| Huawei, HiSilicon | No other solutions. We’d propose to focus on solution(s) that certain understanding/consensus have been built up. |
| OPPO | If the solutions in Section 2.1 and 2.2 are precluded, we can go back to other solution(s). |
| Sequans | We are still not convinced a solution is needed. If the problem is fast activation of PDCP duplication, then let's just define a proper way to do it via DCI, rather than using NACK or reusing an existing DCI. |

## UE-based, non-reactive solutions

Q4-1. RAN agreed to focus on UE-based reactive solutions. Do you support any other UE-based solutions? (As a reminder, solutions based solely on gNB implementation are off the table as per RAN2 assumptions.)

| Company | Answer |
| --- | --- |
| Nokia | We believe proactive method based on PDCP PDU SN is still more appropriate to handle use cases with stringent requirements. It was de-prioritized because companies think it has low spectral efficiency issue. However, we must note it can be implemented in ways where spectral efficiency is not affected, for examples: - We can send all odd-indexed packets to FR1 and all even-indexed packets to FR2, to avoid consecutive error caused by beam blockage in FR2. - We can cyclically change the RLC entities set for duplication, e,g. Packet 1 on RLC1 & RLC2, Packet 2 on RLC3 & RLC4, and so on, to average out the probability of consecutive error All these schemes do not require boosting additional radio resource as compare to normal mode. More importantly, these schemes are also much simpler and less error-prone than e.g. Tx-side timer based solutions discussed above. |
| LG | The HARQ-NACK based solution and Tx-side timer solution cannot meet the tight ST requirement, and causes radio resource waste as explained in previous questions. The Nokia’s proposal is much better than those two solutions. |
| Qualcomm | No. We prefer keeping the agreement “Survival Time triggered proactively based on Sequence Number is deprioritized” and focusing on reactive solutions for now. |
| ZTE | No. Agree with Qualcomm. |
| OPPO | At least till now, we share the similar view as Qualcomm and ZTE. |

## Any additional comments

Q5-1. Do you have any additional comments to make on handling of ST in the RAN not covered by questions above, and/or any additional issues to introduce? (Please do take into account the agreements and assumptions already made by RAN2, and the multiple previous discussions which were held on this topic as referenced in the introductory section.)

| Company | Answer |
| --- | --- |
| Ericsson | It is necessary to discuss the UE actions upon entering survival time mode, e.g., PDCP duplication or other L2 adaptive transmission mechanisms. |
| Fujitsu | Some aspect of the email discussion of R2-2104897 in the email discussion have not been discussed in the last meeting. Those aspect include: (1) Link reliability increase mechanisms a. Duplication activation b. L1/L2 configuration adaptation c. gNB scheduling (2) ST recovery mechanism a. Survival Time timer b. Return to normal state |
| Nokia | If PDCP duplication is to be adopted as the way to boost reliability, this is important to make sure that the gNB knows which RLC entity will be activated by the UE when entering the survival time state. From our point of view, the simplest way is to configure two Duplication States in PDCP via RRC: - Default duplication state – indicates the RLC entities that should be activated by default (This already exists in Rel-16) - Survival time duplication state – indicates the RLC entities that should be activated when entering survival time state. So essentially, the UE switches the duplication state from a default one to a conditional one when survival time is triggered. |
| Lenovo | Agree with Nokia, that RLC entities which are to be activated for duplication when entering ST state could be preconfigured. |
| LG | We think if ST is very short, no solution can meet the ST requirements. In this case, the UE should be configured with PDCP duplication from the beginning. The dynamic control of PDCP duplication is achieved by already defined PDCP duplication activation/deactivation MAC CE. |
| Qualcomm | Agree with Ericsson. We propose discussing PDCP duplication as the standard behavior in survival state, since this option was supported by a lot of companies. We will need to specify what the UE is doing in survival mode so that we can answer Q1-4/Q2-4. |
| ZTE | Agree with some above comments that the following aspects need further discussion: - UE actions upon entering ST state - Recovery/exiting from ST state |
| Huawei, HiSilicon | Agree with the comments that we can discuss the UE actions upon entering survival time mode, as well as we can discuss UE behavior on exiting ST state. |
| OPPO | Maybe the following can be considered, - UE actions upon entering ST state, e.g., PDCP duplication or the flexibility L1/L2 transmission mechanism. - ST timer and return to normal state. |
| Sony | We think RAN2 should consider this point from previous email discussion: (1) Link reliability increase mechanisms L1/L2 configuration adaptation |
| III | Agree with some comments mentioned above. We can discuss both the UE actions upon entering ST mode, such as PDCP duplication or other L1/L2 adaptive transmission mechanisms, and the UE actions upon existing ST mode. |
| Futurewei | Triggering condition and UE behavior on exiting ST state. |

# Phase-II:…

# Conclusions

In the present tdoc…