3GPP RAN WG2 Meeting #114-e	R2-21XXX

eMeeting 19th – 27th MAY 2021

Title:	[Draft] LS on establishment/resume cause value and UAC

Release:	Release 17

Source:	Xiaomi [to be RAN2]

To:	SA2, CT1

CC:

Contact Person:

Name:	Xing Yang

E-mail Address:	Yangxing1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

During RRC establishment or resume, relay UE would indicate the establishment or resume cause for gNB to decide whether accept or reject the request. RAN2 discussed how relay UE sets cause value if the RRC connection establishment/resume is triggered only for relay purpose. Two options were identified as following,

Option 1: define new establishment/resume cause value;

Option 2: reuse existing establishment/resume cause value.

Question 1: Which option does SA2/CT1 prefer?

RAN2 assumes the access attempt from remote UE shall also under UAC control. RAN2 made following agreement regarding remote UE’s UAC,

Remote UE can reuse legacy access control and no need to enhance the access control procedure of Remote UE.

Question 2: Can SA2/CT1 confirm this agreement?

RAN2 has agreed RRC-CONNECTED relay UE would not perform UAC for remote UE’s data. However, when IDLE/INACTIVE Relay UE intends to access network only for the purpose of relaying but not for its own service, 12 companies prefer Relay UE skips UAC, since remote UE has done UAC locally. While 6 companies have different view.

Question 3: When IDLE/INACTIVE Relay UE intends to access network only for the purpose of relaying but not for its own service, whether relay UE can skip UAC from SA2/CT1 point of view?

2. Actions:

To SA2 and CT1

ACTION:	RAN2 kindly asks:

Question 1: Which option does SA2/CT1 prefer?

Question 2: Can SA2/CT1 confirm this agreement?

Question 3: When IDLE/INACTIVE Relay UE intends to access network only for the purpose of relaying but not for its own service, whether relay UE can skip UAC from SA2/CT1 point of view?

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#115-e	 	Aug 16th – 27th, 2021		Online meeting