3GPP RAN WG2 Meeting #114-e	R2-21XXX

eMeeting 19th – 27th MAY 2021

Title:	[Draft] LS on establishment/resume cause value and UAC on SL Relay

Release:	Release 17

Source:	Xiaomi [to be RAN2]

To:	CT1

CC:	SA2

Contact Person:

Name:	Xing Yang

E-mail Address:	Yangxing1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

At RAN2#114e RAN2 discussed SL Relay issues that may have impacts on other working groups.

At RRC establishment or resume, UE indicates the establishment or resume cause for gNB to enable access control . RAN2 discussed how relay UE should set the cause value if the RRC connection establishment/resume is triggered only for relay purpose. The following two options were identified:

Option 1: define a new establishment/resume cause value;

Option 2: reuse existing establishment/resume cause values.

Question 1: Which option does CT1 prefer?

RAN2 assumes the access attempt from remote UE shall also be under the UAC. RAN2 made following agreement regarding remote UE’s UAC:

Remote UE can reuse legacy access control and no need to enhance the access control procedure of Remote UE.

Question 2: Can CT1 confirm this agreement?

RAN2 has agreed RRC-CONNECTED relay UE would not perform UAC for remote UE’s data. However, there was no agreement in RAN2 whether the relay UE can skip UAC procedure when IDLE/INACTIVE Relay UE intends to access network only for the purpose of relaying but not for its own service, 12 companies prefer Relay UE skips UAC, since remote UE has done UAC locally. While 6 companies have different view.

Question 3: When IDLE/INACTIVE Relay UE intends to access network only for the purpose of relaying but not for its own service, whether relay UE can skip UAC from SA2/CT1 point of view?

2. Actions:

To CT1

ACTION:	RAN2 kindly asks CT1 to provide feedbacks to the questions above.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#115-e	 	Aug 16th – 27th, 2021		Online meeting