3GPP TSG-RAN WG2 Meeting #115 electronic	                                           R2-21xxxxx

Online, August 16 – 27, 2021

Agenda item:	8.15.2

Source: 	SHARP

Title: 	Summary of [POST114-e][704][V2X/SL] How to make sure Rel-16 UEs not supporting SL DRX are not involved in SL communication in DRX manner (Sharp)

Document for:	Discussion and Decision

# Introduction

This contribution is a summary of the following email discussion which was triggered at RAN2#114-e.

- [POST114-e][704][V2X/SL] How to make sure Rel-16 UEs not supporting SL DRX are not involved in SL communication in DRX manner (Sharp)

Scope: Discuss possible options (e.g. based on SL UE capability information via PC5-RRC, TX profile information, or resource pool separation, etc.) (including pros, cons and preference) and decide the most agreeable one. Good to have two sub-deadlines. First one is to collect companies’ options, and the second one is for the discussion and decision.

Intended outcome: Discussion summary

Deadline: Long email discussion

This email discussion is organized into two phases.

Phase 1 is intended to identify use cases and corresponding open issues relevant to this email discussion, and collect companies’ views on potential solutions for the identified open issues (if any). The deadline is Friday July 2nd, 09:00 UTC.

Phase 2 is intended to consolidate and further discuss potential solutions (if any), and strive to identify an agreeable way forward. The deadline is Friday August 6th, 09:00 UTC. Companies are encouraged to provide feedback to Phase 2 questions by Wednesday August 4th, 12:00 UTC so that some time is left for discussion of potential proposals.

# Discussion

## Summary of proposals in contributions

Contributions [2], [3], [4] and [5] highlighted the backward compatibility issue of SL DRX in scenarios with a mix of Rel-17 UE(s) and Rel-16 UE(s).

Contribution [2] mentioned the following potential solutions:

- S1: SL DRX is disabled or deactivated by a Rel-17 UE for all “Rel-16 compatible services”.

- S2: SL DRX is dynamically activated/deactivated by a Rel-17 UE. For example, this can be done by a Rel-17 UE upon detection of “presence of Rel-16 UEs which may be acting as a transmitter for a service”, or “traffic pattern of Rel-16 UEs”, or “resource reservation of Rel-16 UEs”.

- S3: In mode 1 resource allocation, gNB allocates resources for a Rel-16 UE according to SL DRX pattern of Rel-17 UEs.

- S4: A Rel-17 UE adjusts SL DRX (pattern) based on the resource reservation or traffic pattern of Rel-16 UEs.

- S5: UE implementation. For example, Rel-17 UEs can disable or deactivate SL DRX at least when they “expect to receive SL broadcast/groupcast messages from Rel-16 UEs in proximity”.

Contribution [3] mentioned the following potential solution:

- S6: For SL groupcast and broadcast, introduce Tx profile for Rel-17, at least to differentiate traffic targeting at DRX-capable (Rel-17-only) and DRX-incapable (Rel-16 or Rel-17) UE.

Contribution [4] mentioned the following potential solution:

- S7: Resource pool separation. For example, SL DRX related RX-side and TX-side operations are only applied in a subset of RX pools and corresponding TX pools, respectively, and those TX pools are not configured for Rel-16 UEs.

Rapporteur would like to encourage companies especially the proponents of the above (and other, if any) potential solutions to comment during Phase 1, in section 2.4 of this document, on how such solutions work for the use cases identified in section 2.2.2 and/or how to resolve the open issues identified therefrom.

## Identification of use cases

### General

For the purpose of this email discussion, it is assumed that the SL DRX feature is enabled in the SL carrier/BWP. In other words, cases in which the SL DRX feature is disabled in the SL carrier/BWP is considered irrelevant.

Unless otherwise stated, “Rel-17 UE” refers to a Rel-17 UE which supports SL DRX, and may or may not be provided with SL DRX (pre-) configuration(s). “Rel-16 UE” refers to any Rel-16 UE (which certainly does not support SL DRX).

It is assumed that from TX UE perspective, “applying a SL DRX configuration” corresponds to performing some SL DRX related TX-side operations (e.g. resource selection taking RX UE’s active time into account). Details of such operations are considered outside the scope of this email discussion.

It is assumed that from RX UE perspective, “applying a SL DRX configuration” corresponds to performing some SL DRX related RX-side operations (e.g. monitoring of SCI in SL DRX active time). Details of such operations are considered outside the scope of this email discussion.

### List of use cases

For a given SL transmission (or each of a number of SL transmissions corresponding to a same Destination Layer-2 ID for broadcast/groupcast or a pair of Source Layer-2 ID and Destination Layer-2 ID for unicast), Rapporteur’s understanding is that there is no backward compatibility issue when both TX UE and RX UE(s) are Rel-16 UEs or when both TX UE and RX UE(s) are Rel-17 UEs. However, the latter may become relevant in case a Rel-17 UE cannot distinguish such cases from some other cases, for example, in case a Rel-17 TX UE cannot tell whether RX UE is a Rel-17 UE or a Rel-16 UE.

For analysis of potential backward compatibility issue for SL DRX, the following use cases are identified by Rapporteur.

Table 1 Use cases

| Case | Cast type | TX UE | RX UE(s) |
| --- | --- | --- | --- |
| 1 | unicast | Rel-16 | Rel-17 |
| 2 | unicast | Rel-17 | Rel-16 |
| 3 | unicast | Rel-17 | Rel-17 |
| 4 | groupcast | Rel-16 | Rel-16 + Rel-17 |
| 5 | groupcast | Rel-16 | Rel-17 |
| 6 | groupcast | Rel-17 | Rel-16 |
| 7 | groupcast | Rel-17 | Rel-16 + Rel-17 |
| 8 | groupcast | Rel-17 | Rel-17 |
| 9 | broadcast | Rel-16 | Rel-16 + Rel-17 |
| 10 | broadcast | Rel-16 | Rel-17 |
| 11 | broadcast | Rel-17 | Rel-16 |
| 12 | broadcast | Rel-17 | Rel-16 + Rel-17 |
| 13 | broadcast | Rel-17 | Rel-17 |

Do you find other use cases not covered by Table 1, or do you have any suggestion on Table 1? If yes, please specify. Otherwise there is no need to answer this question.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO |  | For unicast, we assume the reason for not listing R16 as Tx and R16 as Rx is that there is no much work to do on this case – if that is the reason, we are fine. For groupcast and broadcast, it is not clear to us that why case-5/6/8 (or 10/11/13) is needed even though case-4/7 (or 9/12) is already listed – to us, case-4/7 (or 9/12) has already covered the case that when Tx UE is R16 or R17 UE, it may not know the release of the Rx UEs in the proximity. [Rapporteur reply] The main intention was for completeness. For example, for the purpose of identifying a use case, it may be necessary to differentiate between “all RX UEs are Rel-17 UEs” and “there are a mix of Rel-16 RX UEs and Rel-17 RX UEs”, assuming that in a future discussion, one may need to refer to one of them. For “Rel-16 TX UE vs. Rel-16 RX UE” your understanding of the intention is correct that at least in Rapporteur’s view SL DRX is totally irrelevant for analysis, i.e. there is no chance of referring to any SL DRX related configurations / UE behaviors. |
| Xiaomi |  | If both TX and RX are R17 UEs, there seems to be no backward compatibility issue. In unicast, the sidelink DRX should be negotiated between peer UEs. Sidelink DRX would not be applied if either TX or RX is R16 UE. Therefore, case 1, 2, 3, 8, 13 should be removed. [Rapporteur reply] As explained in the text above Table 1, “Rel-17 TX UE vs. Rel-17 RX UE” may be relevant for discussion of backward compatibility issues when e.g. a Rel-17 RX UE is unaware of whether the TX UE is a Rel-17 UE or not, i.e. from RX UE perspective it is unaware of whether it is falling into Case 5 or Case 8. |
| vivo |  | The Table 1 has covered all the cases but as OPPO mentioned, some of the cases may not need separate analysis. E.g. case 5 may be covered by case 4. But we may not need to spend time on this table to conclude which cases should be included/excluded, but just go to the questions below to see in which scenarios we need solutions. As for e.g. whether a Rel-17 TX UE can tell whether RX UE is a Rel-17 UE or a Rel-16 UE, we understand at least in unicast, the existing Sidelink UE capability transfer procedure can already support exchanging the release number of the peer UE e.g., use one spare value of the accessStratumReleaseSidelink field to indicate rel17. UECapabilityInformationSidelink-IEs-r16 ::= SEQUENCE { accessStratumReleaseSidelink-r16 AccessStratumReleaseSidelink-r16, pdcp-ParametersSidelink-r16 PDCP-ParametersSidelink-r16 OPTIONAL, rlc-ParametersSidelink-r16 RLC-ParametersSidelink-r16 OPTIONAL, supportedBandCombinationListSidelinkNR-r16 BandCombinationListSidelinkNR-r16 OPTIONAL, supportedBandListSidelink-r16 SEQUENCE (SIZE (1..maxBands)) OF BandSidelinkPC5-r16 OPTIONAL, appliedFreqBandListFilter-r16 FreqBandList OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE{} OPTIONAL } AccessStratumReleaseSidelink-r16 ::= ENUMERATED { rel16, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... } |
| Nokia |  | Case 3,8 and 13 seem not be relevant for discussion on backward compatibility issues. |
| Apple |  | If TX UE does not support DRX, then there is no way to use DRX. Those cases 1,4,5,9,10 can be removed. Case 6, 8, 11,13 can also be removed as it is already covered by case 7 and 12, respectively. |
| LG |  | We have same view with Xiaomi. The cases 1, 2, 3, 8, 13 are not needed. |
| ZTE |  | Since the SL DRX is configured by TX UE side for sidelink unicast, there is no big issue for unicast. For groupcast and broadcast, If TX UE active SL DRX but RX UE does not support SL DRX, there is also no big issue. We shall focus on the case of 4,5,9,10. |
| Intel |  | Firstly, we are not sure if listing the use cases here is meant to allow some discussion on down-scoping/eliminating some of the use cases? We think it would be good to first be clear on what scenarios we should be targeting for this exercise. If this is the case, then for unicast, assuming enhancing Rel-16 UE behavior is not in the scope of this exercise, at least use case 3 can be excluded since we assume that whether or not SL DRX is enabled/disabled can be negotiated via PC5 RRC signaling. Moreover, we assume that disabling SL DRX in case TX and RX UE are not able to converge on a specific DRX configuration to be used over this link is not in the scope of this discussion. For groupcast/broadcast, we have a similar understanding as OPPO, i.e. for cases 5 and 6, they should already be covered in the general case of 4 and 7. In our view, if Rel-17 UE is “DRX disabled”, it falls in the category of Rel-16 UE for the purpose of this exercise (as explained by the rapporteur above). Similar reasoning applies for the broadcast case, i.e. cases 10 and 11 can be removed. [Rapporteur reply] The original intention was just for reference in analysis and discussion of open issues/potential solutions. It is fine to express views on which cases are “relevant” or “irrelevant” in this email discussion, although it seems also no harm to keep the table and numbering of use cases “as is” so that people can refer to them wherever necessary. |
| Huawei, HiSilicon |  | For unicast, there are no backward compatibility issues as TX UE will negotiate with RX UE about SL DRX, therefore case 1,2,3 should be removed. For groupcast and broadcast, when TX UE is R17 UE and RX UE is R17, there seems no backward compatibility issue. If there are other issues related to SL DRX, they should be solved in other discussions and we shall focus on the most possible cases for SL DRX backward compatibility issues. Thus case 8, 13 should be removed or deprioritized and we could focus on case 4, 5,6,7,9,10,11,12. |
| CATT |  | We think in fact the scenarios can be classified based on cast type: - For SL unicast, since there is Tx/Rx UE interaction, no compatible issue, hence case 1, 2 and 3 do not need to be considered when discussing the compatibility issue. - For SL groupcast and broadcast, there is no Tx/Rx UE interaction. It is not possible for the Tx UE to identify the Rx UE release without enhancement. Hence, whether it is really need to listed all of Tx and Rx UE release combination? |

Rapporteur summary for Question 1:

First of all it may be worth clarifying that the intention was to identify use cases for “analysis of potential backward compatibility issue for SL DRX”. For example, the “Rel-17 TX UE vs. Rel-17 RX UE” case itself may not cause any backward compatibility problem, but for analysis of Rel-17 RX UE behavior for any backward compatibility issue, one question may arise as to whether the Rel-17 RX UE is aware of whether the TX UE is a Rel-17 UE or a Rel-16 UE. In that sense, it may be necessary to refer to the “Rel-17 TX UE vs. Rel-17 RX UE” case in such analysis, hence the “relevance”. That was also the reason why it was only asked in Question 1 for anything not covered, and it was not asked for anything to remove.

Anyway, below is a summary of companies views on which cases are “irrelevant”.

| Case | Number of companies considering the case “irrelevant” |
| --- | --- |
| 1 | 6 |
| 2 | 5 |
| 3 | 7 |
| 4 | 1 |
| 5 | 3 |
| 6 | 4 |
| 7 | 1 |
| 8 | 7 |
| 9 | 1 |
| 10 | 2 |
| 11 | 3 |
| 12 | 1 |
| 13 | 7 |

For Table 1, as pointed out by vivo, it is not necessary to make any decision for it, as long as the purpose of the table and corresponding use case numbering is clear.

On the other hand, looking through all the comments provided during Phase 1, it seems we can try to make one step forward in terms of narrowing down the scope of this email discussion, i.e. excluding unicast in our next-step discussions.

- Question 1a for Phase 2.

Do you agree to conclude that for SL DRX, there is no backward compatibility issue for unicast?

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Yes | For unicast, we think UEs can use PC5-RRC to understand each other’s SL capability to determine if SL DRX can be used or not. |
| Ericsson (Min) | No | Even for unicast, RAN2 needs to further discuss how to address coexistence issue for the initial control messages (e.g., direct communication request message and accept message which are used to establish the PC5 link) between UEs with different releases. Not applying DRX is one feasible option in this case. But, RAN2 needs to discuss the issue and makes agreements. In addition, it is worth noting that although the issue is also existing for Rel-17 TX UE and Rel-17 RX UE. However, the issue would be different for the case where Rel-16 UE communicates with Rel-17 UE. For the former case, it is feasible to apply a default DRX configuration to Rel-17 UEs to deal with initial control messages. While for the latter, it is infeasible to apply any DRX configuration to a Rel-16 UE. |
| Intel | Yes (see comment) | As mentioned above, we assume that other than the initial DCR message, the UEs can exchange capability information to determine the SL DRX configuration to be used and there should be no backward compatibility issue. So, the only aspect to be addressed is what configuration to apply for the initial communication request message when considering both Rel-16 and Rel-17 UE (and our assumption is that no SL DRX is applicable when transmitting/receiving it) |

## Identification of open issues

This section is intended to identify a list of open issues in terms of backward compatibility when the SL DRX feature is introduced in Rel-17.

### General

As hinted in contributions [2], [3], [4] and [5], backward compatibility issues arise when there is a misalignment on possible SL transmission occasions between TX UE and RX UE(s), e.g. when SL transmission occasions selected by TX UE fall into inactive time of RX UE.

### Rel-17 TX UE

For a Rel-17 TX UE, ideally a SL DRX (pre-) configuration should be applied when (and only when) it is also applied in RX UE(s). If TX UE is capable of identifying whether RX UE(s) is a Rel-16 UE(s), it seems beneficial for TX UE to be also capable of not applying any SL DRX configuration for corresponding SL transmissions, even if TX UE is provided with a SL DRX (pre-) configuration.

- For unicast, this functionality seems desirable and already possible to support according to previous RAN2 agreements, even for cases where backward compatibility is not a concern, e.g. failure in coordination of SL DRX between a Rel-17 TX UE and a Rel-17 RX UE (i.e. Case 3).

- For groupcast and broadcast, the motivation to support this functionality is unclear to Rapporteur, though, especially considering that, even if a SL DRX configuration is applied in TX UE and not applied in RX UE(s), there seems no issue for RX UE(s), since RX UE(s) is always “active” in this case, with no risk of missing any SL transmission from TX UE due to SL DRX. On the other hand, it may be argued that there are some impacts to TX UE, which needs to “unnecessarily” perform SL DRX related TX-side operations (e.g. imposing restrictions in resource selection).

In order to enable not applying of a SL DRX (pre-) configuration for certain SL transmissions, it seems TX UE has to be capable of identifying whether there is any Rel-16 RX UE(s), specifically, whether TX UE can distinguish among

- Case 2 and Case 3 for unicast.

  - This seems to be already possible, e.g. by exchange of Sidelink UE Capability information, or by exchange of “signaling-1 (RX->TX)” or “signaling-2 (RX->TX)” as previously agreed in RAN2.)

- Case 6, Case 7 and Case 8 for groupcast.

  - For groupcast where TX UE is unaware of RX UEs in the group (e.g. “Application Layer connection-less group” as defined in TS 23.287), it seems impossible to support distinguishing among these cases.

  - For groupcast where TX UE is aware of all RX UEs in the group (e.g. “Application Layer managed group” as defined in TS 23.287), it may be possible for a Rel-17 TX UE to exchange capability related information with each RX UE in separate unicast links, and consequently distinguish among Case 6, Case 7 and Case 8, although this does not come with a trivial cost (in terms of both time and frequency resources, and delay), at least when the number of RX UEs in the group is large.

- Case 11, Case 12 and Case 13 for broadcast.

  - It seems impossible to support distinguishing among these cases.

Do you think the following should be added to the list of open issues?

- Issue 1: Whether a Rel-17 TX UE which is (pre-) configured to activate SL DRX can deactivate SL DRX for some of its SL transmissions.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Question rewording suggestion | “(pre-)configured to activate SL DRX” is not clear to us since we are not sure whether there is a need for such “SL-DRX activation configuration”, suggest to reword the Q in a more general way as follows: Issue 1: Whether a Rel-17 TX UE which is (pre-) configured to activate provided with SL DRX configuration can deactivate SL DRX for some of its SL transmissions. (Although “DRX deactivation” is not a rigorous way to describe it, if intentionally it is clear to companies, we are fine to use it, if some clarification as suggested above can be added in the context) [Rapporteur reply] I took your suggestion, combined with rewording of “deactivate” as explained in section Error: Reference source not found. See issue 1b below. |
| Xiaomi | Yes with comments | The question is a bit confusing. SL DRX activation/deactivation is performed by RX UE. I understand the intention of the question is to not consider RX UE’s active time during SL transmission, since we have agreed LCP enhancement to consider RX UE’s active time. If so, I suggest following wording, Issue 1: Whether a Rel-17 TX UE which is provided with SL DRX configuration can deactivate the LCP enhancement of considering RX UE’s active time for some of its SL transmissions. Generally, we think it’s beneficial to simplify TX UE’s behavior in some cases. [Rapporteur reply] The question was just intended to check, at a high level, whether for a Rel-17 TX UE it is possible to perform these SL-DRX-related operations only for some of its SL transmissions (and not perform such operations for the rest of its SL transmissions). We should of course go into further details if this question is confirmed to be investigated. |
| Sharp | Yes with comments | We are fine with OPPO’s suggestion. Regarding Xiaomi’s suggestion, in the same RAN2 agreement that Xiaomi referred to, there is also an FFS, i.e. “FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE)”, so it might be better to use another term to describe TX UE “deactivating SL DRX”. We are open to whichever term is used, as long as it is clear that it is only for the purpose of this email discussion. In fact, in section 2.2.1 of this document, Rapporteur has clarified intention of the term, so if there is a different preference by the group, the change should be first reflected there. [Rapporteur reply] See the proposed rewording in section Error: Reference source not found and reply to OPPO above. |
| Ericsson | comments | The wording is quite confusing. The wording assumes that the UE is already provided with SL DRX. but we should go one step back and study “Whether and how a Rel-17 UE can be configured with SL DRX if it is not compatible wit all of its SL tranmsissions”. Therefore, we have the following rewording suggestion Issue 1: Whether a Rel-17 TX UE can be configured/preconfigured with SL DRX for some of its SL transmissions. [Rapporteur reply] Regarding “configure” vs. “activate”, see the proposed rewording in section Error: Reference source not found. There seems to be two levels of “configuration” here, for example for unicast, “(pre-)configuration” provided by the network, and “configuration” by means of exchanging PC5-RRC signaling between TX UE and RX UE. Assuming you were referring to the former one, fine to confirm with the group whether your suggestion above can be taken for further discussion. See issue 1a below. |
| vivo | Yes | We agree with OPPO’s suggestion as well. For Xiaomi’s suggestion, as in section 2.2.1 we already have the definition (e.g. resource selection taking RX UE’s active time into account), it seems no need to explicitly mention LCP. For Ericsson’s suggestion, we are fine with current definition but in order to solve their concern the definition in 2.2.1 may be changed as follows: It is assumed that from TX UE perspective, “activating/deactivating SL DRX” corresponds to performing some SL DRX related TX-side operations (e.g. resource selection taking RX UE’s active time into account) or not (resource selection DOES NOT taking RX UE’s active time into account, or DOES NOT have any SL DRX configuration). Details of such operations are considered outside the scope of this email discussion. [Rapporteur reply] See replies to OPPO, Xiaomi, Ericsson. Hopefully it is also OK to you. |
| Nokia | comments | Agree with Oppo & Ericsson that the wording of Q2 is not clear to us. Furthermore we think that in the provided rewording proposals the phrase “for some of its SL transmissions” is not accurate and should be refined to “for SL transmissions to non SL-DRX compatible destinations”. |
| NEC | Yes with comments | For the unicast SL communication, when Rx UE activates/deactivates SL DRX, Tx UE should follow Rx UE’s operation to pursue a better QoS or service quality. Therefore, upon receiving notification of Rx UE SL DRX activation/deactivation, it is reasonable for a Rel-17 Tx UE to activate/deactivate the related SL DRX operation. BTW, regarding the wording, “DRX suspend/resume” might be more appropriate than “DRX deactivation/activation” for those SL DRX operations which have been activated once by Tx and Rx UEs. [Rapporteur reply] See the proposed rewording in section 2.2.1. As clarified in the reply to Nokia in section 2.2.1, the original intention of “activating/deactivating” was for applying / not applying SL DRX configurations. It is OK for companies to propose any potential solution involving activating/deactivating/suspending/resuming SL DRX, though, if this is believed beneficial in terms of resolving backward compatibility issues. |
| Apple | See Comment | The wording of Issue 1 is ambiguous. R17 TX UE’s SL DRX configuration is regulating is a DTX configuration. So, the issue is that if R17 TX UE shall be allowed to “always TX” even if SL DRX is configured. |
| LG |  | The wording should be refined. While a Rel-16 UE performs transmission, a Rel-17 UE performs reception, an issue would be raised SL-DRX is operated by Rel-17 RX UE. In unicast, TX UE can be aware that a reception is performed by Rel-17 UE since capability signals during PC5 connection setup. However, GC/BC cases, TX UE cannot be aware of whether the reception is performed by Rel-17 UE or not. |
| Intel | Yes with comment | We understand the intention to say whether a TX UE can modify TX related operation (e.g. resource selection considering RX UE’s active time) for some of its SL transmissions. We think this issue can be considered, but for us, the higher priority is addressing the RX UE side (rather than TX UE side) operation in this case |
| Huawei, HiSilicon | See comments | We agree with OPPO that the question is confusing and should be changed into “Whether a Rel-17 TX UE which is provided with SL DRX configuration can deactivate SL DRX for some of its SL transmissions.” We can then discuss whether it is needed to deactivate SL DRX for R17 TX UE when there is/are R16 RX UE(s), as there seems no problem for RX UE’s reception if the R17 TX UE anyway “uses” the configured SL DRX. It seems only sensible when all RX UEs are R16 UE, and the R17 TX UE determines that the group/broadcast service better to be transmitted via R16 transmission format/procedure (e.g. with a R16-like TX profile) and then deactivates SL DRX. However, when there is at least one RX UE supporting SL DRX, the R17 TX UE can “use” the configured SL DRX, in order to achieve power saving in RX UEs as much as possible. |
| CATT | See comments | Agree with Oppo&Ericsson that the wording of Q2 is not clear. Whether it is only corresponding to groupcast and broadcast since there is no compatibility issue for SL unicast. If it is only corresponding to groupcast and broadcast, we think it is possible that even if there is SL DRX configuration, the Tx UE cannot use SL DRX if there is Rel-16 Rx UE. |

Rapporteur summary for Question 2:

Companies seem to be in general fine with having some discussions along the lines of issue 1, with divergent views expressed on how to reword the description of the issue, though. Rapporteur’s understanding is that this is partly due to the use of “activating/deactivating” starting in section Error: Reference source not found of this document. With the updated wording proposed in section Error: Reference source not found of this document hopefully clarity is now improved (see Question 1b).

The original intention of question 1 was just to check whether it is supported that for a TX UE provided with SL DRX configuration(s), whether it is allowed to apply the TX-side-SL-DRX-related operations only for some of its SL transmissions and not for other SL transmissions. There was a comment that it may be even possible that the “SL DRX (pre-) configurations” are only provided to some SL transmissions and not to other SL transmissions. Rapporteur thinks it can be OK to check whether such a suggested issue can be further discussed (see Question 1a).

- Question 2a for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 1a: Whether/how it is supported that for a Rel-17 TX UE, SL DRX (pre-) configuration(s) is provided only for some of its SL transmissions, and not for the rest of its SL transmissions.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | No | We think Rel-17 UE will always be (pre)configured with SL-DRX configuration for broadcast/groupcast by NW, the UE may not always need activate or follow the SL-DRX configuration though. |
| OPPO | See comment | It is hard to give a response before understanding the scheme where “configuration(s) is provided only for some of its SL transmissions, and not for the rest of its SL transmissions”, or in another words, if it is an unclear Q, we tend to avoid that being included in the open issue list before further clarification. |
| Ericsson | Yes with comments | Agree with the intention of this question. However, it is better to improve the wording. We think Nokia’s suggestion is good. “for SL transmissions to non SL-DRX compatible destinations”. Based on this, the issue is updated as - Issue 1a: Whether/how it is supported that for a Rel-17 TX UE, SL DRX (pre-) configuration(s) is not provided only for some of its SL transmissions to non SL-DRX compatible destinations, and not for the rest of its SL transmissions. |
| Intel | See comment | We agree with OPPO that the wording is a bit confusing. In our understanding, the main question is whether the UE may use the (pre-)configured SL DRX configuration for all destinations or not. |

- Question 2b for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 1b: Whether/how it is supported that for a Rel-17 TX UE and all of its SL transmissions for which SL DRX configurations are provided, SL DRX configurations are only applied to some of such SL transmissions, and not to the rest of such SL transmissions.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Yes | For a TX UE, it may consider to not follow SL-DRX configurations provided if some or all of the RX UE’s are Rel-16 UE(s). |
| OPPO | No | Suggest to focus on the down-selection of Tx-profile and pool scheme instead of going further for an “open issue” list. |
| Ericsson (Min) | No | We don’t think this issue is needed. Issue 1a is sufficient. This would add additional design efforts for RAN2. Whenever a DRX configuration is provided to a UE, this DRX configuration shall be always applied. |
| Intel | No | Based on our comment above, this is essentially the same as issue 1a, so we don’t need it. |

Do you think the following should be added to the list of open issues?

- Issue 2: Impacts to a Rel-17 TX UE which activates SL DRX while RX UE deactivates SL DRX.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Question rewording suggestion | The Q is to ask “Impact”, it is fine but may be too broad, it is suggest to have a more concrete Q as follows (we leave it to rapp to either replace the current Issue-2, or to add one additional Q as follows). How for a Rel-17 Tx UE to decide whether to activate or deactivate SL DRX for a SL transmission? |
| Xiaomi | Yes | We need to investigate the impact. |
| Sharp | Yes | Although in our view the only relevant “impact” is activation/deactivation of SL DRX (as OPPO indicated), we are fine with the current wording, in order to accommodate other potential impacts as the discussion is moved forward. |
| Ericsson | No | See our comments for Q2. If there is compatibility issue for SL DRX, the best option/the only reasonable option is to not configure SL DRX, therefore, the issue/impact can be avoided. In our views, we proposed issue 1 is sufficient. The issue 2 is not needed. [Rapporteur reply] I have a problem understanding your comment here. The configuration from network is semi-static, and for a Rel-17 TX UE, it may or may not know whether the RX UE(s) is a Rel-16 UE or a Rel-17 UE. How do you think the decision “to not configure SL DRX” is based on? By UE or the network? [Ericsson (Min)] in our understanding, for GC/BC, a configuration is likely to be provided per service/traffic type, the network is of course able to know whether the intended receivers of the service/traffic type is Rel-17 UEs, or Rel-16 UEs, or both. |
| vivo | Yes | The main impact is how the TX UE can decide whether to activate or deactivate SL DRX, as OPPO mentioned, and also if the TX decide to go either way, what is the concrete behavior (e.g. we may not simply say ‘TX UE deactivate SL DRX’) |
| Nokia | No | We have same understanding as Ericsson and understand the question in the following way: The (Rel-17) RX-UE is not configured with SL-DRX while the (Rel-17) TX-UE is configured with SL-DRX. As RAN2 has agreed that the RX-UE will report its SL-DRX configuration to the TX-UE there should be no impact. [Rapporteur reply] See reply to Ericsson. Furthermore, as analyzed in the text above Question 2 in this document, the question here is mainly about a Rel-17 TX UE configured with SL DRX vs. Rel-16 RX UE(s), in which case it seems OK for the Rel-17 TX UE to still apply a SL DRX configuration (e.g. applying some restrictions in TX resource selection), although the impacts (pros and cons) needs to be investigated. |
| NEC | Yes | Similar to OPPO and Sharp, we think the impact to a Rel-17 Tx UE is how to align with Rx UE’s DRX activation/deactivation status change. |
| Apple | Yes with comment | There is indeed impact. But we are not sure this is a backward-compatibility issue. |
| ZTE | Yes | We think this impacts shall be investigated especially for sidelink groupcast and broadcast, |
| Intel | Suggest to clarify | We are not sure from the current wording what the main difference is compared to issue 1, i.e. in both cases, there is potential impact to TX UE side’s SL operation because of something that happens at the RX UE side. It is suggested to clarify the wording as suggested by OPPO [Rapporteur reply] The main difference is that issue 1 intends to check whether TX UE is allowed to not apply a SL DRX (pre-) configuration for a SL transmission (the assumed “normal” case is that if something is configured, it should be applied), and here the situation is that if TX UE applies the SL DRX (pre-) configuration (e.g. regardless of whether RX UE(s) is a Rel-16 UE), what would be the impact to the TX UE. |
| Huawei, HiSilicon | See comments | Similar as our comments for Q2, first we need to discuss whether it is needed to deactivate SL DRX for this case, then we can discuss the impact related to the detailed operations. [Rapporteur reply] See reply to Intel. It is assumed that SL DRX is “activated” for the TX UE in this case. |
| CATT | Yes with comments | For SL broadcast/groupcast, for Rel-17 Tx UE configured with SL DRX, if Rx UE does not support SL DRX, it will impact the Rel-17 Tx UE behavior. This should be further studied. |

Rapporteur summary for Question 3:

8 companies are fine with further discussing issue 2, while 2 companies request clarification or see it conditioned on issue 1, and 2 companies do not see a need to discuss it. Rapporteur observed that some views against discussing the issue were somewhat related to the use of “activating/deactivating” starting in section Error: Reference source not found of this document. With the updated wording proposed in section Error: Reference source not found of this document hopefully clarity on this aspect is now improved.

- Question 3a for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 2a: Impacts to a Rel-17 TX UE due to applying a SL DRX configuration for a SL transmission for which a corresponding RX UE (e.g. Rel-16 RX UE) does not apply any SL DRX configuration.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| OPPO | No | Actually we feel this Q does not help much, because DTX at Tx-UE does not cause packet loss at Rx-UE no matter DRX is applied or not (as clarified by rapp above), so maybe no need to keep this Q in phase-2. |
| Ericsson (Min) | No | As we commented, we don’t think this question is necessary. The only reasonable option is to not configure SL DRX for a service/traffic type which targets (e.g., fully or partly) to Rel-16 UEs as potential receivers. |
| Intel | No | It is not clear what (if any) impact to TX UE as a result of “applying the DRX configuration for which the RX UE does not apply DRX configuration is”. |

### Rel-17 RX UE(s)

- For a Rel-17 RX UE(s), a SL DRX (pre-) configuration should be applied when (and only when) it is also applied in TX UE. Otherwise (i.e. the SL DRX (pre-) configuration is applied in RX UE(s) and not applied in TX UE), RX UE would miss SL transmissions from TX UE during SL DRX inactive time.

- For unicast (i.e. Case 1 and Case 3), the functionality of not applying a SL DRX (pre-) configuration for a unicast link seems already possible to support according to previous RAN2 agreements. Rapporteur observes, though, that not applying a SL DRX (pre-) configuration for a unicast link may bring some impacts to a Rel-17 RX UE. For example, as shown in Error: Reference source not found below, assuming UE-A is a Rel-17 RX UE and UE-D is a Rel-16 TX UE, if UE-A does not appls any SL DRX (pre-) configuration for the unicast link from UE-D to UE-A, it has to always keep “active” for reception of SL transmissions from UE-D, and this may effectively eliminate any “inactive” time (and corresponding power saving gains) obtained from activating SL DRX for other SL unicast/groupcast/broadcast receptions in UE-A.

Figure 1 Active time in a RX UE in case no SL DRX (pre-) configuration is applied for one of the unicast links

For groupcast and broadcast, at least for semi-static SL DRX (pre-) configurations, the cases of Rel-16 TX UE (i.e. Case 4 and Case 5 for groupcast, and Case 9 and Case 10 for broadcast) may become an issue for Rel-17 RX UE(s). And even in some groupcast cases SL DRX (pre-) configurations can be not applied upon identification of TX UE being a Rel-16 UE, same impacts as shown in Error: Reference source not found arise.

In summary, the main backward compatibility issues seem to be, for a Rel-17 RX UE,

- B1: If TX UE is a Rel-16 UE, SL transmissions may be performed during RX UE’s inactive time and RX UE may miss such SL transmissions.

- B2: The overall inactive time may vanish if SL DRX is deactivated for SL receptions corresponding to one (or more) unicast link or Destination Layer-2 ID (for groupcast/broadcast) while it is activated for other SL receptions. This can be viewed as a side effect if some certain measures are taken to resolve B1.

For B1, as hinted in [2], if a service can be categorized as being “Rel-16 compatible/incompatible”, then for a Rel-16 compatible service and the Destination Layer-2 ID to which it is mapped, SL DRX will never be (pre-) configured, avoiding mismatch in terms of applying/not applying a SL DRX (pre-) configuration in TX UE and RX UE(s). However, it is unsure yet what the impact would be to the definition of a “service” which is out of RAN2’s scope, and whether it is intended that Rel-16 UEs are always excluded from SL communications for “Rel-16 incompatible” services. Maybe more details can be provided during the discussion.

Do you think the following should be added to the list of open issues?

- Issue 3: Whether a service can be configured as being “Rel-16 compatible/incompatible”.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Question rewording suggestion | This Q worth some further differentiation on cast-types, since clearly, for unicast, it is easier for Tx and Rx UE to know the release of peer UE, so from OPPO perspective, there is less need for it, which however is not feasible for group-/broad-cast, so is the main uncertainty here. The other point is that “Rel-16 compatible/incompatible” is not a sustainable way, since the method should be future proof considering Rel-17/18/19.., i.e., although intentionally the same, some rewording is needed to highlight the sustainability Issue 3: Whether a service can be configured as being “Rel-16 compatible, Rel-17 compatible but is Rel16 incompatible”, for unicast, groupcast and broadcast respectively. |
| Xiaomi | Yes | This may be the key information to resolve this issue, especially for groupcast/broadcast. However, this is out of RAN2 scope. Input from other groups may be helpful. |
| Sharp | See comments | We do not object to discussing this aspect further, although we are not convinced that tying SL DRX as an AS layer feature to an upper layer service is the correct direction to go. In general, all “services” especially those using broadcast and connection-less groupcast should be supported by a Rel-16 UE (in terms of SL communications among Rel-16 Ues and Ues supporting a future release), unless e.g. the QoS requirement associated with a service defined in a future release cannot be satisfied by only utilizing Rel-16 AS features (e.g. data rate requirement can only be supported by multiple SL carriers which is not supported by a Rel-16 UE). Questions for proponents of such a proposal: 1). So all services supported in Rel-16 are “Rel-16 compatible”, for which SL DRX can never be activated? 2). And all “Rel-16 incompatible” services can never be used by a Rel-16 UE? Any concrete example of a “Rel-16 incompatible” service? |
| Ericsson | Yes with comments | We think we shall go even one step further and go in the direction of: “Whether service X can be transmitted using feature Y”. For example: - Service 1 transmitted using Rel-16 features. - Service 2 transmitted using Rel-16 features and can be configured with SL DRX. - etc. |
| vivo | See comments | Even if this issue needs to be studied, SA2 input is necessary. We think this question needs to be first confirmed by SA2. |
| Nokia | Yes with comments | We support to have a discussion on this issue, however as already noted by other companies the wording “Rel-xx compatible” is not accurate. We see some benefits with Ericsson’s proposal to explicitly state the needed features associated to certain (sidelink) service. RAN2 may discuss whether the some entries in sidelink FG (feature group) would be beneficial and future proof. |
| NEC | See comments | For groupcast and broadcast services, it might be worth discussing more. However, considering it is out of RAN2 scope, we are not sure how to move it forward. |
| Apple | Yes |  |
| LG | Yes | We need to check with SA2 whether a service can be configured as being Rel-16 compatible or not. |
| ZTE | See comments | We shall ask SA2 for this issue first. |
| Intel | See comment | While we agree that this is a worthwhile issue to address, which can hopefully make our job a lot easier (i.e. if the upper layer can somehow indicate this information to the AS), but we agree with Xiaomi that we will need input from other WGs on whether this is feasible |
| Huawei, HiSilicon | See comments | It is confusing to say that “for a Rel-16 compatible service and the Destination Layer-2 ID to which it is mapped, SL DRX will never be configured to activate”. We think it is better to formulate as “Issue 3: Whether a service can be delivered in SL DRX compatible/incompatible manner”. If so, we think it would be valuable to discuss this issue. When the RX UE finds all its interested services indicated by upper layer are “SL DRX compatible“, the RX UE can activate SL DRX, otherwise the RX UE needs to deactivate SL DRX. RX UE can also learn from TX UE indication that the interested service will be transmitted adopting SL DRX and enable SL DRX for reception. |
| CATT | See comments | It depends on SA2, we had better send LS to SA2. |

Rapporteur summary for Question 4:

Companies seem to be fine (with or without some rewording) to have some discussions on issue 3. Rapporteur agrees with comments that the wording “Rel-xx compatible” is not perfectly accurate, as a sidelink feature specified in Rel-xx may not only be used by a “Rel-xx UE”, so some rewording to remove “Rel-16” seems desirable. Regarding comments to check with SA2, companies are encouraged to express views during Phase 2 on what particular question(s) to ask SA2, in case an LS is really deemed necessary.

- Question 4a for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 3a: Whether a service is compatible (or incompatible) with an AS-layer sidelink feature / feature group (e.g. SL DRX).

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| OPPO | No | The Q is confusing: in case the Q is to ask whether Tx-profile solution should associate a service to a AS-layer feature/FG, we see it redundant to the Q9, which is already to discuss the shape of the Tx-profile alternative, so may be good to avoid redundancy? |
| Ericsson (Min) | Yes |  |
| Intel | Yes |  |

- Question 4b for Phase 2.

Do you agree that RAN2 should send an LS to SA2 (and/or other WGs) regarding whether a service is compatible (or incompatible) with an AS-layer sidelink feature / feature group (e.g. SL DRX)? If your answer is “yes”, your comments are welcome on what exact questions to ask in the LS.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | See comment | I think it is better for RAN2 to figure out possible solution(s) first before consulting SA2 about how to support the solution. |
| OPPO | No (not before RAN conclude on the solution) | Firstly, we understand the need of LS is a second step after RAN2 conclude on the solution.. which seems not ready yet.. Secondly, in case RAN2 conclude on the solution, e.g., Tx-profile solution is adopted, the detailed aspects have to be clarified, e.g., “service is compatible (or incompatible) with an AS-layer sidelink feature / feature group” it has been discussed in LTE on whether to associate the per-service indication to a release or to a specific feature in a release. The conclusion is the former one since - It is not reasonable for upper layer to be aware of detailed lower layer feature / FG for feature selection; - The per-Feature/FG solution is not scalable since the combination of Feature/FG will grow rapidly, e.g., 5 feature/FG means 32 combination of Tx profiles! So when multiplexed with number of Release, it would be eventually cost huge signaling to indicate the configuration.. so to discuss per-feature/FG Tx-profile is not reasonable to us. After all the conclusion, a LS can be sent to CT1 for confirm/information, but now seems it is still pre-mature to do it.. Please note that the framework of RAT indication and Tx profile has been ready in CT1 spec, as shown in TS 24.587 f) optionally, a list of V2X service identifier to PC5 RAT(s) and Tx profiles mapping rules. Each mapping rule contains one or more V2X service identifiers, PC5 RAT(s) and, if the PC5 RAT(s) include E-UTRA-PC5, Tx profiles corresponding to the E-UTRA-PC5; So far it only includes RAT indication for NR-V2X but not Tx-profile since there is only one release (R16) specified. So one should note that it is not a new thing to CT1 but somehow already been considered since R16 already. |
| Ericsson (Min) | comments | Agree with Apple, it is premature to send LS before RAN2 has made solid outcome on the solutions. |
| Intel | No strong view | We are fine if companies want to send LS |

From a more general perspective, dimensions related to B1 may include: whether a Rel-17 RX UE can be aware of the problematic situation (i.e. TX UE is a Rel-16 UE), and what operation(s) the Rel-17 RX UE may take to resolve any potential problems arising from such situation.

Do you think the following should be added to the list of open issues?

- Issue 4: Whether a Rel-17 RX UE can be aware of TX UE being a Rel-16 UE or not for determining whether to perform SL DRX related operations for corresponding SL receptions.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO |  | Although we do not think this is the right direction to solve this issue, consider the Q5 is to ask to add this issue into open issue list, we are open to it. |
| Xiaomi | Yes | This is another alternative. However, we understand this may be out of RAN2 scope. Input from other groups may be helpful. |
| Sharp | Yes with comments. | We are fine to discuss this aspect further, although we prefer a unified solution where activation/deactivation of SL DRX at RX UE does not rely on RX UE “detecting” whether TX UE is a Rel-16 UE or not, in one way or another. As pointed out by Rapporteur, such a “detection” is not always feasible. |
| Ericsson | Yes with comments | For groupcast and broadcast, the RX UE needs to be aware of the release or features of the TX UE. This can be achieved on using the concept of Tx profile, which is already existing. The RX UE needs to know the TX profile of the service types that it is interested in. For unicast, using the same solution (as for GC/BC) is desirable, but it is ok to have further discussion on whether/how to use PC5-RRC signalling (e.g., capability signalling) on top of the concept of TX profile. [Rapporteur reply] Looking through all the comments related to “TX profile”, it seems obvious that companies do not have a common understanding on what it actually means by “TX profile”. From your description here, it seems “TX profile” is associated to the service type, which is mapped to a Destination Layer-2 ID for BC/GC cases, and some mechanism is used to ensure that a Rel-16 TX UE will never use such a Destination Layer-2 ID. Correct? Anyway, please provide further details regarding “TX profile” during Phase 2 discussions. |
| Vivo | Yes with comments | We are ok to discussed this issue but we understand for connection less based groupcast and broadcast, such UE identification method may not be realistic for all surrounding Ues. For unicast, as we mentioned in Q1, existing PC5-RRC capability exchange can work. |
| Nokia | No with comments | We would like to understand the intention of the question. If the RX-UE is Rel-17 (supports SL-DRX) and the TX-UE is Rel-16 (no SL-DRX support) there is no issue, as long as the TX-UE is capable to understand the reported SL-DRX configuration of the RX-UE and/or correspondingly adapts the sidelink transmission(s) with the sl-drxOnDuration of the RX-UE. HARQ feedback to the TX-UE and even bidirectional communication can be send by the (Rel-17) RX-UE at any time to the TX-UE (not applying SL-DRX). So we fail to see a need, why the RX-UE should be made aware whether the TX-UE itself supports SL-DRX or not. Does the question imply: A Rel-16 TX-UE can not adapt its sidelink transmission to a Rel-17 RX-UE active time, because the TX-UE can not interpret the sidelink assistance information sent by the RX-UE ? [Rapporteur reply] Regarding your question above, at least my understanding is that the answer should be “No, the Rel-16 TX UE cannot adapt its sidelink transmission to a Rel-17 RX UE active time”. A “Rel-16 TX UE” (as explained in section 2.2.1, for the purpose of this email discussion) is not supposed to understand anything introduced in Rel-17. Furthermore, by mentioning “sidelink assistance information”, were you assuming only unicast? As mentioned by other companies, the main concern on backward compatibility occurs in BC/GC cases. |
| NEC | Yes with comments | Agree with Sharp that a unified solution to activate/deactivate Rx UE SL DRX would be preferred. Considering that Tx-UE centric method was agreed as the baseline in RAN2#114-e meeting, we think there is not a big issue at least for unicast. |
| Apple | No | We do not think there is a feasible solution between R16 TX UE and R17 RX UE. [Rapporteur reply] Since “Rel-16 TX UE vs. Rel-17 RX UE” is considered the main problem in terms of backward compatibility, it seems desirable to investigate some potential solutions, in one way or another. It is of course possible that after further investigations the group still concludes that no feasible solution can be found, but I think this should not be the reason for not carrying out the investigation. |
| LG | Yes | In GC/BC cases, RX UE cannot be aware of whether the transmission is performed by Rel-16 UE or not. |
| ZTE | Yes with comments | This is not a issue for sidelink unicast. For sidelink groupcast/broadcast, maybe the RX UE can be aware of TX UE being a Rel-16 UE or not based on TX profile of the service types that it is interested in. However, this depends on SA2. |
| Intel | Yes with comment | We think that a better way to ask this question would be if the Ues can be aware of certain capabilities of the peer UE(s) regarding support of SL DRX operation [Rapporteur reply] Your wording looks fine. But I just realized that essentially it seems not (only) a matter of capability, but more a matter of applying or not applying a SL DRX configuration. |
| Huawei, HiSilicon | See comments | We think it is not preferred to use the condition that TX UE being a R16 UE or not to determine whether to perform SL DRX, as there is case e.g. that a R17 RX UE needs to further check whether a R17 TX UE activates SL DRX or not. We suggest to follow the logic proposed in Q4, a R17 RX UE should check the interested service being “SL DRX compatible/incompatible “or not, or whether the TX UE of the interested broadcast/groupcast services adopts SL DRX for transmission, and determines whether to perform SL DRX related operations for corresponding reception. [Rapporteur reply] Your comment is fully understood. On the other hand, one of the reasons of discussing “open issues” was to take all potential solutions into account. For example, I can also see some negative comments on going along the way of defining “SL DRX compatibility/incompatibility“ for a service, but I tend to not take such comments as a reason to not further discuss such a potential solution. |
| CATT | See comments | It is only valid for sidelink unicast. For SL broadcast and groupcast, the Rx UE can’t know it unless enhancement is introduced to indicate the mapping between service and its target UE release. In our understanding, the answer is clear. Hence, what is the intention to make it to the open issue lists? [Rapporteur reply] See e.g. comment from Ericsson and my reply there, as an example of one of the potential solution in mind regarding the issue here. And some other companies may have other, probably totally different potential solutions in mind. It is well understood that companies have their own preferred “answer” in mind, but in order to move the discussion forward we may need to first focus on the issues. From Rapporteur perspective it is fine for a potential solution to claim that some of the issues are “non-issue”, but first of all we need to build a list of “issues”. |

Rapporteur summary for Question 5:

9 companies are in general fine with further discussing issue 4, while 4 companies are negative about it.

Rapporteur has provided replies to some comments hopefully making the intention of the question clearer. Reading all the comments Rapporteur also realized that it may be better to rephrase the issue to describe awareness of whether TX UE apply or not apply a SL DRX configuration instead.

- Question 5a for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 4a: Whether a Rel-17 RX UE can be aware of TX UE applying or not applying a SL DRX configuration.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | Yes | But I think even if RX UE detects that TX UE is not applying a DRX configuration, it still cannot distinguish whether the TX UE is an R16 UE or jusu an R17 UE not using the DRX configuration. |
| OPPO | No | We do not think it is the right way to continue in phase-2 discussion. |
| Ericsson (Min) | Yes | We think TX profile is one feasible solution to achieve this. The TX profile maps to feature, or feature group (e.g., SL DRX), and/or service types. The TX profile needs to be aware at TX side and RX side in order to make sure that TX UE(s) and RX UE(s) for the same interested services apply the same TX profile. |
| Intel | No (see comment) | We do not think this question really accomplishes much on its own. The real question in our view is whether there is specific RX UE behavior if it can be aware that the TX UE is “applying” a certain SL DRX configuration or not. |

Do you think the following should be added to the list of open issues?

- Issue 5: What operation(s) a Rel-17 RX UE can perform to avoid missing SL transmissions from a Rel-16 UE during SL DRX inactive time.

  - Granularity of such operation(s), e.g. per service, or per QoS profile, or per TX profile, or per unicast link, or per Destination Layer-2 ID, or per resource pool, or per SL BWP, or per SL carrier, etc.

  - Whether such operation(s) can be done semi-statically or dynamically.

  - Whether such operation(s) depends on being aware of TX UE being Rel-16 UE or not.

  - Impacts due to such operation(s), if any.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | Question rewording suggestion | The current Q sounds like a Rel-17 Rx UE should not miss any transmission from R16 Tx UE, yet that finally depends on whether the said transmission is interested from the said Rel-17 Rx UE perspective, so it is suggested to reword the Q as follows Issue 5: What operation(s) a Rel-17 RX UE can perform to avoid missing SL transmissions from a Rel-16 UE during SL DRX inactive time, if it is interested in the SL transmissions. |
| Xiaomi | Yes with comments | RAN2 had agreed UE can skip PSCCH monitoring during inactive time. Therefore ‘during SL DRX inactive time’ should be removed, otherwise DRX definition would be changed. - Issue 5: What operation(s) a Rel-17 RX UE can perform to avoid missing SL transmissions from a Rel-16 UE during SL DRX inactive time. |
| Sharp | Yes with comments | Agree with OPPO and Xiaomi on their observation of potential ambiguity in interpreting the main bullet of the issue. Maybe the following can help: - Issue 5: For What operation(s) a Rel-17 RX UE activating SL DRX for reception of SL transmissions from a TX UE, what operation(s) the RX UE can perform to avoid missing some of the SL transmissions from due to the TX UE being a Rel-16 UE during SL DRX inactive time. |
| Ericsson | No with comments | In our views, the UE shall avoid configuring SL DRX in this case. If so, there will be no SL DRX configuration provided to the UE. [Rapporteur reply] Again, I have a problem understanding the comment here (which is very similar to that for Question 3). See reply to your comment there. Furthermore, even when looking at your comment on Question 5, if you agree that a RX UE may need to be aware of the “release” or “features” supported by TX UE, then it seems natural that RX UE may take some actions depending on the outcome of checking the “release” or “features” supported by TX UE. No? [Ericsson (Min)] for GC/BC, RAN2 has agreed that DRX configuration is provided per PQI. In this case, RX UE will check “release” or “feature” for each interested RX service/traffic type. RX UE will only configure/apply DRX if all services support the concerned release or feature (e.g., SL DRX). |
| vivo | Yes | We understand issue 5 is the main issue that needs to be discussed in order not to make packet loss happen. For the form of the question, no strong view, suggestion from Sharp is OK. |
| Nokia | No | Please see our response for Q5 [Rapporteur reply] See reply to Q5. |
| NEC | Yes with comments | Agree with Xiaomi, “during SL DRX inactive time” should be removed. We are fine with the rapporteur proposal above. |
| Apple | NO | Same as Q5 [Rapporteur reply] See reply to Q5. |
| ZTE | Yes with comments | Agree with the updated proposal of rapporteur. |
| Intel |  | We are fine in general, but it should be noted that without considering the potential solutions in some detail, it is not very clear which operations should be considered by Rel-17 RX UE |
| Huawei, HiSilicon | See comments | If the “operation(s)” (after “Issue 5”) is referred to as “SL DRX deactivation”, then we are fine to discuss the list of aspects as above. [Rapporteur reply] The original intention was to describe it in a general manner such that no potential solution is excluded at this stage. If it is considered non-contentious, it is fine to reword it along the lines of your suggestion. |
| CATT | See comments | We are wondering how to understand the sub-bullets? In our understanding, we should list the possible operations in the sub-bullets firstly. If no operation options, we can’t discuss the granularity or its impacts. [Rapporteur reply] The sub-bullets are related to aspects mentioned in the potential solutions in companies’ contributions, as summarized in section 2.1. |

Rapporteur summary for Question 6:

8 companies are in general fine with further discussing issue 5 (with or without request for rewording), while 3 companies are negative about issue 5, where 2 of the 3 companies pointed to their comments on issue 4. 1 company requests clarifications.

It seems better for companies to first check replies to their comments on Question 5 and whether Question 5a provides better clarity comparing to Question 5. Meanwhile, Rapporteur would like to update the description of the issue hopefully to take all the wording suggestions into account. In Question 6a, Rapporteur also attempts to take into account comments under Question 7, see the first sub-bullet of issue 5a, as the suggested issues there seem closely related to the original intention of Question 6.

- Question 6a for Phase 2.

Do you think the following should be added to the list of open issues?

- Issue 5a: For a Rel-17 RX UE provided with SL DRX (pre-) configuration(s), and for all of the SL transmissions from a Rel-16 TX UE that the Rel-17 RX UE is interested in, what operation(s) the Rel-17 RX UE can perform to avoid missing some of such SL transmissions.

  - How to decide whether to perform such operation(s).

  - Granularity of such operation(s), e.g. per service, or per QoS profile, or per TX profile, or per unicast link, or per Destination Layer-2 ID, or per resource pool, or per SL BWP, or per SL carrier, etc.

  - Whether such operation(s) can be done semi-statically or dynamically.

  - Whether such operation(s) depends on being aware of TX UE being Rel-16 UE or not.

  - Impacts due to such operation(s), if any.

| Company | Response | Comments |
| --- | --- | --- |
| Apple | See comment | We do not support the bullet “Whether such operation(s) depends on being aware of TX UE being Rel-16 UE or not.” Because this is not feasible for SL broadcast/groupcast as R16 UE will not be able to follow any new signaling/mechanism introduced in R17 to disclose the release information, and it adds overhead to force R17 TX UE to disclose its release information with new OTA signal . |
| OPPO | No | Suggest to focus on the down-selection of Tx-profile and pool scheme instead of going further for an “open issue” list. |
| Ericsson (Min) | No | Actually, we are becoming to share the same sympathy as OPPO somewhat. we really don’t understand why the rapporteur is so keen on an issue list. Some basic issues to motivate the solution are sufficient. RAN2 shall not spend time to create complex issues, which would just kill the precious time. |
| Intel | No | We also share the view with OPPO and Ericsson that in phase2, we should be looking at specific solutions identified that seek to address some of these open issues, rather than spend more time discussing which open issues to capture. |

Do you identify any other open issue(s) not covered above? If your answer is yes, please specify what the issue is. Otherwise there is no need to answer this question.

| Company | Response | Comments |
| --- | --- | --- |
| OPPO |  | We understand the following Q is helpful: How for a Rel-17 Rx-UE to decide whether it can activate or should deactivate SL DRX in order to receive an interested SL transmission? [Rapporteur reply] I had thought this one should have been part of Question 6. The intention of Question 6 was to provide some high level description so as to accommodate all potential solutions. For example, I used “operation” rather than activation/deactivation just in case people suggested some other operations. Please check whether you have any comments to Question 6a. |
| Ericsson |  | Agree with OPPO’s suggestion in general, however with wording suggestion How for a Rel-17 Rx-UE to decide whether it can be configured or not configured with SL DRX in order to receive an interested SL transmission? [Rapporteur reply] See reply to OPPO. |
| Huawei, HiSilicon |  | We agree with OPPO, and would prefer slight modifications on the question as follows: How for a Rel-17 Rx-UE to decide whether it can activate or should deactivate SL DRX for receiving an interested SL broadcast/groupcast service? [Rapporteur reply] See reply to OPPO. |

- Question 7a for Phase 2.

Do you identify any other open issue(s) not already covered for Phase 2 discussion above? If your answer is yes, please specify what the issue is. Otherwise there is no need to answer this question.

| Company | Comments |
| --- | --- |
| OPPO | Suggest to focus on the down-selection of Tx-profile and pool scheme instead of going further for an “open issue” list. |
| Ericsson (Min) | We tend to agree with OPPO somewhat. Rapporteur has spent too much efforts to create complex issues, which is unnecessary. |

## Potential solutions

### For Phase 1 discussion

In order to facilitate discussion of potential solutions in Phase 2, it would be beneficial to collect views on potential solutions with regard to the list of use cases / open issues, already in Phase 1.

In case you would like to propose a solution for resolving issues related to this email discussion, please describe the solution with sufficient details (including pros and cons, if possible) below. You are also encouraged to provide comments in case you believe no particular specification work is necessary (e.g. there may be some backward compatibility issues with the introduction of SL DRX but it can be resolved with gNB and/or UE implementations).

| Company | Comments |
| --- | --- |
| OPPO | According to the email discussion scope: Discuss possible options (e.g. based on SL UE capability information via PC5-RRC, TX profile information, or resource pool separation, etc.) (including pros, cons and preference) and decide the most agreeable one. For PC5-RRC, we believe it is a useful tool for unicast. For Tx-profile, as clarified in our paper [3], we believe TX profile is necessary anyway, as the legacy tool we adopted since LTE, in order for - Tx UE to know whether SL DRX (and later, if any additional Rel-17 features introduced that not fully compatible with Rel-16 behavior) should be adopted for a SL transmission; - Rx UE to know whether SL DRX can be adopted for power saving (and later, if any additional Rel-17 features introduced that not fully compatible with Rel-16 behavior) if a SL transmission is to be received. Which is more useful for G/B-cast where there is no PC5-RRC connection for capability transfer between Tx and Rx Ues. For resource pool separation, we believe it is not useful since - It only solves the issues that how for a Rx UE to behave (i.e., activate/deactivate SL-DRX or not) in a pool, but cannot solve the 2 issues above. - For the issue above, the Tx profile can solve it already, and Tx profile solves it better since it allows shared pool as well which is more resource efficient. |
| Xiaomi | Resource pool segmentation alone couldn’t resolve the issue, since TX UE still need more information about whether this SL transmission is addressed to R16 or R17 Ues to select appropriate resource pool. Information about whether R16 UE is involved with this service/group/destination is required to resolve this issue. However, the feasibility of this information may need confirmation from upper layer. Based on this information, UE can identify whether the transmission or reception with a destination address should apply sidelink DRX or not. We understand the TX profile proposed by OPPO may not be future proof. Sidelink DRX may not be mandatory capability for Ues in future releases. For example, if TX profile is R18, it may indicate CA or packet duplication operation, while doesn’t mean sidelink DRX is applied for this transmission. |
| Sharp | Views regarding TX profile: - First of all, maybe it is beneficial to check whether companies’ understanding is aligned on “TX profile”. In our understanding, “TX profile” for LTE V2X was introduced in Rel-15 in order to configure whether a Rel-15 UE uses a new Rel-15 transmission format (for support of 64QAM MCS table) or the transmission format specified in Rel-14. For a Rel-15 TX UE, if “TX profile” is configured as “REL15”, then the SL transmission is incompatible with Rel-14 and cannot be correctly decoded by a Rel-14 RX UE (i.e. SL communication between a Rel-15 UE and a Rel-14 UE is not possible in this case). “TX profile” is semi-statically (pre-) configured and is applied to all SL transmissions of a Rel-15 UE. - In NR V2X, we don’t see any connection of SL DRX to transmission format, and we don’t see a motivation to define a new transmission format due to introduction of SL DRX (or even other Rel-17 features being discussed). In fact, in NR V2X, MCS table is dynamically indicated in 1st stage SCI, and a Rel-16 RX UE can discard PSSCH in case the indicated MCS table is not supported. - Considering the above two points, we encourage proponents of “TX profile” to first clarify what “TX profile” means here (when comparing with “TX profile” in LTE V2X), and whether the intention is to configure it in the same way as in LTE V2X. - Coming back to the backward compatibility issue, we also encourage proponents of “TX profile” to elaborate on the detailed proposal and how it works for the use cases, especially the cases of “Rel-16 TX UE vs. Rel-17 RX Ues” for broadcast (i.e. Case 9/10). In our understanding, in this case, Rel-16 TX UE will never apply any “TX profile” defined in Rel-17, i.e. no impact to Rel-16 TX UE behavior regardless of whether “TX profile” is defined in Rel-17. It is unclear how RX UE differentiate this case from “Rel-17 TX UE vs. Rel-17 RX Ues” and take different actions accordingly, by means of “TX profile” solution. Views regarding pool separation: First of all, regarding OPPO and Xiaomi’s comments, yes, it is also our understanding that simply configuring separate pools (e.g. respectively for Rel-16 Ues and Rel-17 Ues) does not help. That is why we propose to do a little more than that in order to resolve the backward compatibility issue. Please take a look at the figure and explanations of UE operations in our contribution to RAN2#114-e, i.e. Reference [5]. We will also elaborate more on the proposal below. Our detailed proposal can be summarized as follows, - A RX pool can be configured as one of the following to a Rel-17 UE, - “SL DRX enabled”: a Rel-17 RX UE monitors slots in the RX pool only during SL DRX active time (i.e. “normal” SL DRX behavior). - “SL DRX disabled”: a Rel-17 RX UE monitors all slots in the RX pool, even for slots falling into SL DRX inactive time (i.e. Rel-16 RX UE behavior). - A TX pool can be configured as one of the following to a Rel-17 UE, - “SL DRX enabled”: the TX pool is included in the set of TX pools from which a pool is chosen for resource selection, for and only for SL transmissions with SL DRX activated. - “SL DRX disabled”: the TX pool is included in the set of TX pools from which one pool is chosen for resource selection, for and only for SL transmissions with SL DRX deactivated. Example: suppose TX pools #1, #2 are configured as “SL DRX enabled”, and TX pools #3, #4 are configured as “SL DRX disabled”; for SL transmissions with SL DRX activated, a TX pool is chosen from set {TX pool#1, TX pool#2}, and for SL transmissions with SL DRX deactivated, a TX pool is chosen from set {TX pool#3, TX pool#4}. With regard to applicability to use cases in Table 1, - The proposal is independent of cast type. - The proposal is independent of criteria / mechanisms for activating / deactivating SL DRX in a Rel-17 UE. We assume a general case where for either a Rel-17 TX UE or a Rel-17 RX UE, for some of the SL transmissions SL DRX can be activated and for other SL transmissions SL DRX can be deactivated. - For use cases corresponding to a Rel-17 RX UE (i.e. Case #1, #3, #4, #5, #7, #8, #9, #10, #12, and #13), for each RX pool, UE behavior is as explained in the proposal description depending on whether SL DRX is “enabled/disabled” for the RX pool. - For use cases corresponding to a Rel-17 TX UE (Case #2, #3, #6, #7, #8, #11, #12, and #13), for each TX pool, UE behavior is as explained in the proposal description depending on whether SL DRX is “enabled/disabled” for the TX pool. With regard to resolving the “Rel-16 TX UE vs. Rel-17 RX UE(s)” issue B1 as Rapporteur described in section 2.3.3, at TX side, some TX pools (and corresponding RX pools) are configured with “SL DRX enabled”, and the rest TX pools (and corresponding RX pools) are configured with “SL DRX disabled”, and only the latter are configured for Rel-16 Ues. Since Rel-17 RX UE(s) monitors all slots in the pools where Rel-16 TX UE performs SL transmissions, there is no problem for Rel-17 RX UE(s) in terms of missing SL transmission during inactive time. With regard to resolving issue B2 as Rapporteur described in section 2.3.3, at RX side, “always on” reception is performed in RX pools with “SL DRX disabled”, and “discontinuous reception” is performed in RX pools with “SL DRX enabled”. In case e.g. SL DRX is deactivated for one unicast link, the TX side proposal ensures that corresponding SL transmissions will only be performed in RX pools with “SL DRX disabled”, therefore, power saving in terms of “discontinuous reception” is still possible for RX pools with “SL DRX enabled”. |
| Ericsson | To support coexistence between Rel-16 and Rel-17 (and potentially other future releases), the concept of service type and TX profile introduced in LTE can be reused, with appropriate modifications. As in LTE, each service should be given a service type and each service type should be mapped to a TX profile. Regarding the necessary modifications, at least the following need to be discussed: - In LTE, the TX profile was used only by the TX UE. For NR, the TX profile will be used by the TX and the RX (e.g., as described by OPPO above). - How to multiplex packets of service types with different TX profiles. - Whether TX profile identifies releases (like in LTE) or specific features (e.g., SL DRX) Resource pool separation is not a solution for coexistence in our view. Even if it were, the scope of the WID is clear in that a solution for coexistence in the same pool has to be specified. PC5-RRC is only available for unicast. If necessary, it can be used for coexistence in that case, but it is desirable to have a common solution for all cast types. |
| Vivo | - View on SL UE capability information via PC5-RRC: Workable only for unicast and application layer managed groupcast. Solution is still needed for connection less based groupcast and broadcast. - View on TX profile information Generally, it may be better that the activation/deactivation of DRX operation is transparent to upper layers, as this is a more AS-layer feature. And as replied in Question 4, feasibility of this solution needs to be confirmed by SA2. Even we may simply reuse the concept of ‘TX profile’ but how we handle the differences compared to LTE as Sharp mentioned, and how we could define the TX profile (e.g. in per-Release way or per-feature way) should all be discussed further. Above all, we are slightly not in favor of the TX profile-based solution to avoid cross-WG impacts as possible. - View on resource pool separation From AS point of view, we admit that some forms of distinction on e.g. R16 and R17 RX pools may bring benefit on SL DRX operation. The reason is that the typical case for V2X is that any NR SL UE, no matter with R17 or R16 version, may need to receive Rel-16 services, which include the basic safety V2X services. If we don’t make such distinction on RX pools by releases, then it basically means every UE needs to monitor every RX pool at any time, in order to avoid missing R16 services. Then, the gain of DRX goes nowhere. However, we also share the concern that whether this kind of pool differentiation is realistic and whether it would bring specification impacts which may be far more than the TX profile method. The pros /cons and the specification impacts should be carefully evaluated during this discussion. |
| NEC | Agreed with Xiaomi’s view that resource pool segmentation couldn’t resolve the issue. Regarding the Tx profile, it looks like to be a potential solution for groupcast/broadcast, however we are not sure whether it is a backward/forward compatible solution. For unicast, we prefer to further discuss solutions based on the agreed Tx-centric mechanism, i.e., details of “signaling (Rx->Tx)” and “signaling (Rx->Tx)”. |
| Apple | For unicast, the capabality excahgne will allow UE to figure out whether DRX is feasible or not. For groupcast/broadcast, a TX profile or service-based configuraiton can be provided by NW(preconfiguraiton) so that a R17 UE can know whether it can use SL DRX for a certain SL service of interest. Pool segregation is not the right approach and has many drawbacks as other companies mentioned above. |
| LG | We are fine to be use TX profile which is already defined in LTE specification. But we need to revise a wording “TX profile”. In LTE case, the TX profile is configured for a TX UE’ behavior for suitable transmission. In NR Rel-17, TX profile is used for RX UE’s behavior. Moreover, we think that an approach of pool separation is not a good solution. Firstly, usage of pools is decreased when available pools (e.g., Rel-17, Rel-16) are restricted to UE’s supporting release. Also, pool separation has an impact on RAN1 which should be checked with RAN1. |
| ZTE | To ensure that the TX UE will not perform sidelink transmission during the time occasion out of the range of reception time occasion of the RX UE, following two basic alternatives can be considered: Alternative1: RX UE keep aligned with TX UE, if TX UE is a SL DRX incapable UE, RX UE shall not active SL DRX. Alternative2: ensure that the SL DRX incapable UE will always perform sidelink transmission during the time occasion belong to the active time of the RX UE. To support alternative1, the most important issue is to ensure the TX UE can be aware of whether RX UE supports SL DRX. Some company proposed TX profile per service type can be used to judge whether the RX UE may be a R16 UE or it can only be a R17 UE. However, Sidelink DRX may not be mandatory capability for Ues in future releases. A R17 UE may also not support SL DRX, we are not sure whether the TX profile can be used to judge whether the TX UE support SL DRX or not. Thus, we can send a LS to SA2 to check whether RX UE can be aware of TX UE may not support SL DRX per destination id/services type. If the answer is yes, we think this solution is enough. Otherwise, the Alternative2 –differentiate sidelink resource pool for DRX-capable UE and DRX-incapable UE shall be considered. |
| Intel | For the case of unicast, we believe PC5 RRC signaling can be leveraged to exchange SL DRX related information, including whether the Ues support SL DRX operation or operate in legacy Rel-16 mode. So, we do not foresee need of special enhancements for that scenario. The issue arises when the peer UE cannot be made aware of which mode TX/RX UE is operating in, which is relevant for groupcast/broadcast use cases as in section 2.2. For TX profile based approach, we agree with other companies that we can reuse and adapt the LTE based TX profile mechanism, whereby mapping of service type to specific TX profile can be performed by the upper layer, which can indicate it to the AS. It should be noted that TX profile here does not necessarily mean using a different transmission format as in LTE, but rather just a way to differentiate between transmissions for the RX UE. - Rel-16 TX UE shall use the legacy mechanism for packet transmissions without considering SL DRX operation and Rel-17 RX Ues interested in receiving traffic from the corresponding service type shall not use SL DRX when monitoring PSCCH/PSSCH. - Rel-17 TX Ues, depending on whether the indicated service type corresponds to legacy Rel-16 or Rel-17, can use the associated TX profile for transmission. In case of the latter, the TX UE can consider the DRX sleep pattern of the intended RX Ues based on NW configuration (for groupcast/broadcast). - For the RX side, RX Ues can look at the TX profile/format for the incoming traffic and determine if it is intended for them or not based on their interested service type. For the resource pool separation based option, we think allocation of DRX disabled and DRX enabled pools can also work, based on the following principles: - The Rel-16 legacy RX UE can only be configured with the legacy (DRX disabled) resource pool, because it shall not be performing SL DRX and is expected to be able to monitor transmissions from Rel-16 and Rel-17 Ues. - The Rel-17 RX Ues can be configured with both Rel-17 (DRX enabled) and legacy (DRX disabled) pools for reception, but it can only perform SL DRX when monitoring transmissions within the DRX enabled pool. - The Rel-16 TX UE can only transmit in the corresponding legacy (DRX disabled) shared TX pool, without the need for enhancing the resource selection procedure to account for SL DRX at the RX Ues - The Rel-17 UE on the other hand can transmit in both (DRX enabled) and legacy (DRX disabled) pools, with the consideration that it shall take into account the DRX pattern of the Rel-17 RX Ues when transmitting over the DRX enabled resource pool. Note that this does not necessarily imply the network having to always deploy dedicated/separate pools and in general, we believe it is up to the network to choose if/how to configure DRX enabled/disabled pools to the Ues. |
| Huawei, HiSilicon | We notice that seems many companies are in favor of the “TX profile” solution. In our views, we need to first discuss the real meaning of such “TX profile”. As in LTE, the “TX profile” is used to identify a service is a R14 service or a R15 service. For this very issue discussed here, the key point is to let UE know whether the service can be delivered with SL DRX or not. Therefore, we think such “TX profile” shall be described by “SL DRX compatible/incompatible”. Further, both R17 TX UE and R17 RX UE need to know such “TX profile” related to the service, then the RX UE will e.g. deactivate SL DRX for one interested service configured by upper layers as “SL DRX incompatible”. Alternatively, the R17 TX UE can determine to perform SL DRX for a broadcast/groupcast service when there is at least one R17 RX UE or there is at least one RX UE supporting SL DRX. It requires the R17 RX UE to indicate whether it is an R17 RX UE or whether it supports SL DRX. For the R17 RX UE to determine whether it perform SL DRX for the reception of a broadcast/groupcast service, it requires that the R17 TX UE indicates whether it performs SL DRX for the transmission and the R17 RX UE preforms SL DRX for the reception of this service accordingly. |
| CATT | For sidelink unicast, the SL DRX for Tx UE is configured by network, and the network can acquire both the Tx and Rx UEs’ capability, hence no enhancements is needed for compatibility issue, the current mechanism is enough. For sidelink groupcast and broadcast, the Tx profile which used in LTE can be reused here. But the details of the Tx profile should be further discussed, e.g., what should be contained in the Tx profile, since it may not only impact RAN, but also impacts SA2. |

Rapporteur summary for Question 8:

Firstly Rapporteur shares the views from some companies on potential confusion in using the term “TX profile”, especially when referring to the “TX profile” as specified in LTE V2X. On the other hand, considering the fact that it has been widely used by companies in their comments, it seems OK to keep using this term for now, bearing in mind that the term is just used for the purpose of this email discussion. Proponents are also encouraged to propose a better wording during Phase 2 discussions.

A slight majority seems to be in favor of a “TX profile” based approach. Concerns were also raised on the following aspects:

- Use of the term “TX profile”.

- Whether it is future-proof / backward / forward compatible.

- Lack of detailed proposal (partly due to confusion brought by “reuse of TX profile in LTE V2X”).

- Coupling of an AS-layer feature with upper layers.

- Feasibility pending SA2 confirmation.

Regarding pool separation based approach, it seems proponents of “TX profile” based approach are all negative about going along the lines of this direction. Specifically, 6 companies are negative to this approach, 3 companies think it can be considered as an alternative in some conditions, and 1 company are in favor of it.

Regarding PC5-RRC based approach, in general there is no concern on using it to resolve any issue related to unicast.

### For Phase 2 discussion

As can be seen from the comments under Question 8, it seems two approaches / directions were identified in Phase 1. Rapporteur thinks that for Phase 2, we should at least strive for a clear and stable description of each potential solution, avoiding any confusion when people later refer to any of them. In case of different views on some detailed aspects of an approach, Rapporteur believes that at this stage there is no harm in keeping a few alternatives for future down-selection.

Rapporteur tried to come up with a preliminary draft description of each approach, based on understanding of companies’ input provided in Phase 1. Companies’ views are definitely necessary in improving and stabilizing the draft description.

Companies are also encouraged to comment on the pros and cons of each approach.

#### TX profile based approach

Draft description of the TX profile based approach:

- A service or service type can be mapped to a TX profile.

- A TX profile identifies [a Release (and all sidelink features or sidelink feature groups supported therein), or one or more sidelink features, or one or more sidelink feature groups].

- Multiple TX profiles can be defined/configured.

  - For a Rel-17 TX UE or a Rel-17 RX UE, one or more TX profiles can be indicated from the upper layer to the AS layer.

  - FFS how to multiplex packets of services or service types associated with different TX profiles.

- A Rel-16 TX UE or a Rel-16 RX UE shall not be provided a service or service type with an associated TX profile where SL DRX is included. [FFS how this is ensured.]

- A Rel-17 TX UE or a Rel-17 RX UE shall only apply a SL DRX configuration for a service or service type with an associated TX profile where SL DRX is included.

- Question 9 for Phase 2.

Do you have any comments on improving the draft description of the TX profile based approach above?

| Company | Comments |
| --- | --- |
| Apple | As profile can be associated with a release, instead of associating with a particular feature in that release. I think the “TX profile corresponding to SL DRX” in the above description is better to be rephrased to “TX profile corresponding to SL DRX or the release in which SL-DRX feature is supported”. Regarding the cons raised by other companies, we think the name “TX profile” can always be improved, but the gist is to reuse what we have in LTE V2X. The “coupling of AS layer feature with upper layer” is unavoidable, because the V2X service information is from upper layer. |
| OPPO | - In CT1 spec, the “service type” already points to PSID/ITS-AID, so suggest change to - A service or is associated with a service type, which can be mapped to a TX profile. - According to the current CT1 spec, the signaling framework is already per service type instead of per L2 ID (as in LTE). Even if one believes the said association of service type and L2 ID is needed, it should be just of UE internal implementation instead of signaling design, so suggest to avoid that in the formal conclusion - Since a service type is also mapped to a Destination Layer-2 ID, each Destination Layer-2 ID can be associated with a TX profile. - A Rel-16 TX UE [or a Rel-16 RX UE] shall not be provided a service type Layer-2 Destination ID with an associated TX profile corresponding to SL DRX. [FFS how this is ensured.] - A Rel-17 TX UE or a Rel-17 RX UE shall only apply a SL DRX configuration for a service type Layer-2 Destination ID with an associated TX profile corresponding to SL DRX. Furthermore, w.r.t “ A TX profile identifies [a Release, or one or more sidelink features, or one or more sidelink feature groups].”, as replied for Q4b, we believe that the TX profile should associate with the release. |
| Ericsson (Min) | We think TX profile as an unified solution, is applicable for all cast types including GC, BC and UC. Agree with OPPO that - Service or service type can be mapped to a TX profile - L2 ID may be not needed, in order to reuse the existing CTI signaling interface. In addition, mapping rules between service types and L2 IDs will be supported. So UE can deduce mapping relations between TX profiles and L2 IDs based on implementation. - In LTE, the TX profile was used only by the TX UE. For NR, the TX profile shall be used by both the TX UE and the RX UE. therefore, suggest to add the below bullet A TX profile is indicated to both TX UEs and RX UEs. - How to multiplex packets of service types with different TX profiles. Rules are defined for multiplexing services associated with different TX profiles. - We think the current wording is fine. A TX profile identifies [a Release, or one or more sidelink features, or one or more sidelink feature groups]. |
| Intel | We also think that the TX profile should ideally be associated with a Release. Moreover, we also need to capture the UE behavior (for both Rel-17 and Rel-16 UEs) clearly with respect to the TX profile once it is defined. Therefore, it is suggested to at least capture the TX and RX UE side behavior: - For the TX UE, it shall use the associated TX profile information for SL transmission, based on the indicated service/service type. In case the TX profile corresponds to Rel-17, the TX UE shall consider the SL DRX configuration of the intended RX UE(s) when performing SL transmissions. - For the RX side, RX UE can look at the TX profile/format for the incoming traffic and determine if it is intended for them or not based on their interested service type and apply the corresponding SL DRX configuration for the associated service/service type. [Rapporteur reply August 02] The UE behavior was intended to be captured in the last bullet, where it seems what matters most in the context of this email discussion is whether SL DRX is applied or not. In that sense, for TX UE it seems the last bullet is sufficient. Your suggestion on further changes to the last bullet is welcome. For RX UE, are you suggesting “dynamic” checking of incoming packets in order to determine whether to apply a SL DRX configuration? Could you elaborate a bit further on the details? (I had thought the last bullet was sufficient also for the RX UE). |

#### Pool separation based approach

Proposed description of the pool separation based approach:

- For a Rel-17 UE, RX pools can be split into two subsets:

  - RX pool subset #1 with “SL DRX enabled”: a Rel-17 RX UE monitors slots in the RX pool subset only during SL DRX active time (i.e. “normal” SL DRX behavior).

  - RX pool subset #2 with “SL DRX disabled”: a Rel-17 RX UE monitors all slots in the RX pool subset, even for slots falling into SL DRX inactive time (i.e. Rel-16 RX UE behavior).

Note: a Rel-16 UE can be configured with the same set of RX pools as for a Rel-17 UE.

- For a Rel-17 UE, TX pools can be split into two subsets:

  - TX pool subset #1 with “SL DRX enabled”: the TX pool subset used for resource selection for (and only for) SL transmissions for which a SL DRX configuration is applied.

  - TX pool subset #2 with “SL DRX disabled”: the TX pool subset used for resource selection for (and only for) SL transmissions for which no SL DRX configuration is applied.

Note: a Rel-16 UE cannot be configured with any TX pool in subset #1.

- Question 10 for Phase 2.

Do you have any comments on improving the draft description of the pool separation based approach above?

| Company | Comments |
| --- | --- |
| Apple | Our major concern about pool separation approach is that the solution still relies on UE to get some information to determine which pool to use. Literally, this solution itself does not solve the incompatibility issue. |
| OPPO | This description clarifies the behavior when a pool is selected, how should the Tx/Rx UE behave (i.e., apply DTX/DRX or not), but did not mention how for the Tx/Rx UE to select between SL DRX enabled/disabled pools, can it be clarified? Otherwise, the solution did not answer the key question.. [Rapporteur reply August 02] The proponents can clarify further, but at least in my understanding of relating proposals so far, - There is no (dynamic) “selection between SL DRX enabled/disabled pools”. Instead, among the configured TX (or RX) pools for a Rel-17 UE, some of them are configured as “SL DRX enabled”, and the rest are “SL DRX disabled”, and this splits the configured TX (or RX) pools into two (complementary) subsets. - Some restrictions are applied on the use of a pool depending on whether it is SL DRX enabled/disabled. For example, for a SL transmission where SL DRX is applied, a TX pool can only be selected from the TX pools configured as “SL DRX enabled”; and for a SL transmission where SL DRX is not applied, a TX pool can only be selected from the TX pools configured as “SL DRX disabled”. |
| Ericsson (Min) | Resource pool separation is not a solution for coexistence in our view. Even if it were, the scope of the WID is clear in that a solution for coexistence in the same pool has to be specified. |
| Intel | - We think more detail on how Rel-16 and Rel-17 TX and RX UE can be configured with certain resource pools (based on provided SL DRX configuration) should be clarified. |

#### Other approach(es)

Rapporteur thinks that other approach(es) should also be encouraged, as long as the corresponding description can be stabilized during Phase 2.

- Question 11 for Phase 2.

Would you like to propose any approach other than those described in section 2.4.2.1 and section 2.4.2.2?

| Company | Comments |
| --- | --- |
| Ericsson (Min) | In addition to TX profile based approach, RAN2 shall also study how to address the coexistence between UEs for the initial control signaling for unicast (e.g., direct communication request, and accept messages). |

# Summary and Conclusion

[TO BE ADDED]

# Reference

- R2-2106478, “Report from session on LTE V2X and NR SL”, Session chair (Samsung), RAN2#114-e.

- R2-2101323, “Backward Compatibility Issue of SL DRX with Rel.16 Sidelink”, Nokia, Nokia Shanghai Bell, RAN2#113-e.

- R2-2104835, “Discussion on DRX configuration and DRX timers”, OPPO, RAN2#114-e.

- R2-2105906, “Discussion on SL DRX Timers and Others”, Qualcomm Incorporated, RAN2#114-e.

- R2-2105277, “Discussion on co-existence with UEs not supporting SL DRX”, Sharp, RAN2#114-e.