3GPP TSG-RAN WG2 Meeting #116 electronic	R2-210xxxx

Electronic Meeting, Nov 1 – 12, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | Draft | Rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.304 CR to introduce mobility-state-based cell reselection for NR HSDN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CATT, Ericsson, Huawei, ZTE, OPPO, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  |  | Date: |  | 2021-11-1 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | High-speed-railway dedicated network (HSDN), which is designed and optimized for the high-speed-railway scenario, is deployed for both LTE and NR to improve user experience on the high-speed-train. This means HSDN is dedicatedly designed and deployed to serve the high mobility state UEs only. The mobility state based cell reselection was agreed and specified in LTE TEI15. But NR haven’t support that yet. This CR is to introduce the mobility state based cell reselection for NR HSDN, i.e., high mobility state UE should consider HSDN cell as highest priority, while low and medium mobility state UE should consider HSDN cell as lowest priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following are added to support cell reselection for NR HSDN: - Add abbreviation for HSDN - Priority handling for HSDN capable UE - Number of cell reselection counting for HSDN UE in MSE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HSDN feature cannot be supported in NR for idle and inactive mode UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.2.4, 5.2.4.1, 5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 38.331... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

- 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Acceptable Cell: A cell that satisfies certain conditions as specified in 4.5.

Allowed CAG list: A per-PLMN list of CAG Identifiers the UE is allowed to access (see TS 23.501 [10]).

Available PLMN(s): One or more PLMN(s) for which the UE has found at least one cell and read its PLMN identity(ies).

Available SNPN(s): One or more SNPN(s) for which the UE has found at least one cell and read its SNPN identity(ies).

Barred Cell: A cell a UE is not allowed to camp on.

CAG cell: A cell broadcasting at least one Closed Access Group Identifier.

Camped on a cell: UE has completed the cell selection/reselection process and has chosen a cell. The UE monitors system information and (in most cases) paging information.

Camped on any cell: UE is in idle mode and has completed the cell selection/reselection process and has chosen a cell irrespective of PLMN identity.

Closed Access Group Identifier: Identifier of a CAG within a PLMN.

Commercial Mobile Alert System: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers to CMAS capable UEs.

eCall Only Mode: A UE configuration option that allows the UE to register at 5GC and register in IMS to perform only eCall Over IMS, and a non-emergency IMS call for test and/or terminal reconfiguration services.

EHPLMN: Any of the PLMN entries contained in the Equivalent HPLMN list TS 23.122 [9].

Equivalent PLMN list: List of PLMNs considered as equivalent by the UE for cell selection, cell reselection, and handover according to the information provided by the NAS.

Home PLMN: A PLMN where the Mobile Country Code (MCC) and Mobile Network Code (MNC) of the PLMN identity are the same as the MCC and MNC of the IMSI.

HSDN cell: A cell that has higher priority than other cells for cell reselection for HSDN capable UE in a High-mobility state.

Network Identifier: Identifier of an SNPN in combination with a PLMN ID (TS 23.501 [10]).

Non-Public Network: A network deployed for non-public use, as defined in TS 22.261 [12].

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [16], between two or more nearby UEs, using NR technology but not traversing any network node.

Process: A local action in the UE invoked by an RRC procedure or an RRC_IDLE or RRC_INACTIVE state procedure.

Radio Access Technology: Type of technology used for radio access, for instance NR or E-UTRA.

Registration Area: (NAS) registration area is an area in which the UE may roam without a need to perform location registration, which is a NAS procedure.

Registered PLMN: This is the PLMN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Registered SNPN: This is the SNPN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Reserved Cell: A cell on which camping is not allowed, except for particular UEs, if so indicated in the system information.

Selected PLMN: This is the PLMN that has been selected by the NAS, either manually or automatically.

Selected SNPN: This is the SNPN that has been selected by the NAS, either manually or automatically.

Serving cell: The cell on which the UE is camped.

Sidelink: UE to UE interface for V2X sidelink communication defined in TS 23.287[16].

SNPN Access Mode: Mode of operation wherein UE only selects SNPNs (as defined in TS 23.501 [10]).

SNPN identity: An identifier of an SNPN comprising of a PLMN ID and an NID combination.

Strongest cell: The cell on a particular frequency that is considered strongest according to the layer 1 cell search procedure (TS 38.213 [4], TS 38.215 [11]).

Suitable Cell: This is a cell on which a UE may camp. For NR cell, the criteria are defined in clause 4.5, for E-UTRA cell in TS 36.304 [7].

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [17], between nearby UEs, using E-UTRA technology but not traversing any network node.

- 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CMAS	Commercial Mobile Alert System

CN	Core Network

DCI	Downlink Control Information

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

HRNN	Human-Readable Network Name

HSDN	High Speed Dedicated Network

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MCC	Mobile Country Code

MICO	Mobile Initiated Connection Only

NAS	Non-Access Stratum

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

PLMN	Public Land Mobile Network

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SNPN	Stand-alone Non-Public Network

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

V2X	Vehicle to Everything

CHANGE END

NEXT CHANGE

- 5.2.4	Cell Reselection evaluation process

- 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority.

NOTE 1:	The frequency only providing the anchor frequency configuration should not be prioritized for V2X service during cell reselection, as specified in TS 38.331[3].

NOTE 2:	When UE is configured to perform NR sidelink communication or V2X sidelink communication performs cell reselection, it may consider the frequencies providing the intra-carrier and inter-carrier configuration have equal priority in cell reselection.

NOTE 3:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation.

NOTE 4:	The UE is configured to perform V2X sidelink communication or NR sidelink communication, if it has the capability and is authorized for the corresponding sidelink operation.

NOTE 5:	When UE is configured to perform both NR sidelink communication and V2X sidelink communication, but cannot find a frequency which can provide both NR sidelink communication configuration and V2X sidelink communication configuration, UE may consider the frequency providing either NR sidelink communication configuration or V2X sidelink communication configuration to be the highest priority.

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	the UE receives an RRCRelease message with the field cellReselectionPriorities absent; or

-	a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall consider only the white listed cells, if configured, as candidates for cell reselection.

The UE in RRC_IDLE state shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

CHANGE END

NEXT CHANGE

- 5.2.4.3	Mobility states of a UE

- 5.2.4.3.0	Introduction

The UE mobility state is determined if the parameters (TCRmax, NCR_H, NCR_M, TCRmaxHyst and cellEquivalentSize) are broadcasted in system information for the serving cell.

State detection criteria:

Normal-mobility state criteria:

-	If number of cell reselections during time period TCRmax is less than NCR_M.

Medium-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than or equal to NCR_H.

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than NCR_H.

The UE shall not consider consecutive reselections where a cell is reselected again right after one reselection for mobility state detection criteria. If the UE is capable of HSDN and the cellEquivalentSize is configured, the UE counts the number of cell reselections for this cell as cellEquivalentSize configured for this cell.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

-	else if the criteria for Medium-mobility state is detected:

-	enter Medium-mobility state.

-	else if criteria for either Medium- or High-mobility state is not detected during time period TCRmaxHyst:

-	enter Normal-mobility state.

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in clause 5.2.4.3.1.

- 5.2.4.3.1	Scaling rules

UE shall apply the following scaling rules:

-	If neither Medium- nor High-mobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst" to Qhyst if broadcasted in system information;

-	For NR cells, multiply TreselectionNR by the sf-High of "Speed dependent ScalingFactor for TreselectionNR" if broadcasted in system information;

-	For EUTRA cells, multiply TreselectionEUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionEUTRA" if broadcasted in system information.

-	If Medium-mobility state is detected:

-	Add the sf-Medium of "Speed dependent ScalingFactor for Qhyst" to Qhyst if broadcasted in system information;

-	For NR cells, multiply TreselectionNR by the sf-Medium of "Speed dependent ScalingFactor for TreselectionNR" if broadcasted in system information;

-	For EUTRA cells, multiply TreselectionEUTRA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionEUTRA" if broadcasted in system information.

In case scaling is applied to any TreselectionRAT parameter, the UE shall round up the result after all scalings to the nearest second.

CHANGE END