3GPP TSG-RAN WG2 Meeting #116 electronic	R2-21xxxxx

Electronic Meeting, Nov 1 – 12, 2021

Agenda item:	8.21.2.1

Source:	CMCC (Email Rapporteur)

Title:	Report for [Post115-e][090][TEI17] Mobility-state-based cell reselection for NR High Speed railway Dedicated Network (CMCC)

Document for:	Discussion

# Introduction

RAN2#115-e meeting arranged the following email discussion to check the HSDN CRs. Here some questions are listed to check companies’ views.

- [Post115-e][090][TEI17] Mobility-state-based cell reselection for NR High Speed railway Dedicated Network (CMCC)

Scope: Check and progress CRs to agreeable status.

Intended outcome: Agreeable CRs

Deadline: Long

Please add company contact details into the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| CMCC | Ningyu Chen | chenningyu@chinamobile.com |
| Ericsson | Mattias Bergström | Mattias.a.bergstrom@ericsson.com |
| MediaTek | Chun-Fan (Felix) Tsai | Chun-fan.tsai@mediatek.com |
| CATT | Rui Zhou | zhourui@catt.cn |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| Apple | Yuqin Chen | yuqin_chen@apple.com |

# Discussion

## UE Capability

In LTE spec 36.306, the HSDN capability is missing, i.e., there is no HSDN capability for UE either with or without signalling in LTE. But from rapporteur point of view, a capability for HSDN cell reselection without capability signalling seems needed.

Question 1: Companies are invited to share views on whether a capability for NR HSDN is needed?

- Option 1: Capability without RRC signalling

- Option 2: Capability with RRC signalling

- Option 3: No capability

| Company | Which option? | Comments |
| --- | --- | --- |
| CMCC | Option 1 | HSDN feature only impact on SIB reception and cell reselection priority handling in idle/inactive mode. And no impact on connected mode. So HSDN capability without RRC signalling seems ok. |
| Ericsson | Option 1 | Agree with CMCC. |
| MediaTek | Option 1 | As this is idle/inactive mode behavior and all configuration in SIB, it is reasonable to have optional feature without UE capability. |
| CATT | Option 1 | Agree with CMCC |
| Lenovo | Option 1 |  |
| Apple | Option 1 | Agree with CMCC |

If the answer to Question 1 is option 1, companies are invited to check the following wording for the optional HSDN capability without signalling for 38.306CR?

- 5.x	High speed dedicated network features

Definitions for feature

HSDN cell reselection It is optional for UE to support HSDN cell reselection priority handling in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21] and TS 38.331 [9].

Question 2: Is the wording above for HSDN capability agreeable? Comments are welcome

| Companies | Comments |
| --- | --- |
| Ericsson | It is fine. But another possibility is to place this description under "5.4 Other". [CMCC2]: OK to place the description under “Other features” in both 38.306 and 36.306. |
| MediaTek | Yes |
| CATT | Yes |
| Lenovo | Yes |
| Apple | Yes |

If a capability for NR is needed, then next question is whether we need to also add the missing capability back for LTE 36.306, and in which release to add it.

Question 3: Do you agree the LTE HSDN capability is also needed in 36.306? And which release the 36.306 CR should be?

| Company | Yes/No | Which release? And further comments if any. |
| --- | --- | --- |
| CMCC | Yes | If companies agree to add the missing LTE HSDN capability for 36.306, we would prefer to add it from Rel-15, since HSDN was introduced in R15 LTE. In this case, shadow 36.306CRs for R15, R16 and R17 are also needed. [Lenovo2] No R17 CR is needed since R17 36.306 does not exist yet and will be created based on R16 36.306. Therefore, it is sufficient to have only R15 CR (with cat B) and R16 CR (with cat A). |
| Ericsson | No strong view |  |
| MediaTek | Yes | Same view as CMCC |
| CATT | Yes | Agree with CMCC |
| Lenovo | Yes | Missing capability should be added from R15. Some comments to the 36.306 CR: - Cover page: latest spec version is 16.6.0. - We agree with MediaTek’s comment in 2.2 on the table format in 36.306. - Furthermore, the support of the feature in RRC_INACTIVE needs to be further clarified. Reason: RRC_INACTIVE is only applicable in case of EUTRA connected to 5GC. If the HSDN feature should be applicable for EUTRA connected to EPC as well then the description should be updated as shown below in red: It is optional for UE to support HSDN cell reselection priority handling in RRC_IDLE and RRC_INACTIVE (if the UE supports eutra-5GC-r15) as specified in TS 36.304 [14] and TS 36.331 [5]. [CMCC2]: The corrections are implemented in the latest version. |
| Apple | Yes | Agree with CMCC |

  - CRs for 38.331, 38.304 and 36.331

Draft CRs for 38.331 and 38.304 are generally copy/paste from LTE specification. Companies are invited to check on the details.

Draft CRs for 36.331 is also provided to address inter-RAT cell reselection from LTE to NR, as commented by Lenovo in last meeting (details can be found in the Annex).

Comments or corrections can be added directly to the draft CRs.

In case any essential issue is recognized, companies can also raise it here for discussion.

| Company | Comments |
| --- | --- |
| MediaTek | Editorial comment on 36.306 new section – We don’t use table format in 36.306. It could just be a paragraph. [CMCC2]: OK, it is corrected in the latest version. |
| Ericsson | The "hsdn-Indication" in SIB4 states: hsdn-Indication This field indicates whether there are deployed HSDN cells or not on the the DL carrier frequency indicated by dl-CarrierFreq. It seems that this indication is actually not used anywhere in the specification. Also the field only says whether there are HSDN cells on the frequency or not. In my understanding this says whether there are zero or at least one. But that information is perhaps not so interesting to the UE? Instead we assume that the intention behind this indication was (when it was added to LTE) to indicate whether all cells on a frequency are HSDN-cells. So we should perhaps change the field description to: hsdn-Indication This field indicates, if present, that all cells on the DL carrier frequency indicated by dl-CarrierFreq are HSDN cells. [CMCC2]: We agree the HSDN neighbour cell list and hsdn-Indication in SIB4&SIB5 are redundant design. The hsdn-Indication can be removed. |
| CATT | 1.A editorial comment to the “Reason for change” In the coversheet of the CRs, “The mobility state based cell reselection was agreed and specified in LTE TEI15.”=> “The mobility state based cell reselection for HSDN was agreed and specified in LTE TEI15.” 2. In 36.331 CR, “Need OR” is supposed to be used instead of “Need R”. //36331CR for NR HSDN_v00 Rapp CarrierFreqNR-v17xy ::= SEQUENCE { hsdn-NR-Indication-r17 ENUMERATED {true} OPTIONAL, -- Need R nr-FreqNeighHSDN-CellList-r17 NR-FreqNeighHSDN-CellList-r17 OPTIONAL -- Need R [CMCC2]: Both comments are implemented in the latest version. |
| Lenovo | - Cover page: in all CRs (38.331, 38.304, 38.306, 36.331) the spec version needs to be corrected to 16.6.0. [CMCC2]: Corrected in the latest version. - 36.331: we wonder about the need of field hsdn-NR-Indication-r17 since the presence of nr-FreqNeighHSDN-CellList-r17 already indicates the presence of HSDN cells on the concerned carrier frequency. To our understanding the presence of field hsdn-NR-Indication-r17 makes only sense if field nr-FreqNeighHSDN-CellList-r17 is not present. Likewise, if field nr-FreqNeighHSDN-CellList-r17 is present, then field hsdn-NR-Indication-r17 does not need to be present. Therefore, the use of both fields should be clarified. [CMCC2]: We agree the HSDN neighbour cell list and hsdn-Indication in LTE SIB24 and NR SIB4&SIB5 are redundant design. The hsdn-Indication can be removed. - 38.331: - SIB2: we think that the presence of field cellEquivalentSize-r17 should be condition to the presence of speedStateReselectionPars. Therefore, we suggest to define a new condition “HSDN” as shown below: [[ cellEquivalentSize-r17 INTEGER(2..16) OPTIONAL -- Cond HSDN ]] HSDN The field is optionally present, Need R, if speedStateReselectionPars is present; otherwise it is not present. [CMCC2]: Corrected in the latest version. - Same comment on the need of hsdn-Indication-r17 in SIB4 and hsdn-EUTRA-Indication-r17 in SIB5 as for the 36.331 CR. [CMCC2]: Corrected in the latest version. - 38.304: - 5.2.4.1: There is the Note 2 as shown below. So, in case HSDN coverage is provided by LTE and NR, then acc. to the CR the HSDN capable UE in High-mobility state shall always consider the HSDN cells (on both RATs) to be the highest priority. Should we then still follow the Note 2 or deviate from it? NOTE 2: Equal priorities between RATs are not supported. [CMCC2]: No strong view from our side. I think UE can follow the current behaviour for inter-RAT equal priorities. - 5.2.4.1: We think that the handling of HSDN cells by R17 non-HSDN capable UEs should be clarified since those UEs comprehend the HSDN signaling. Should we clarify that those UEs shall not consider any HSDN cells as candidate for cell reselection? [CMCC2]: We would prefer to reuse the LTE HSDN mechanism, that R15/16/17 non-HSDN capable UEs would not differentiate HSDN cells and normal cells and follow the legacy cell reselection behaviour. Because, if R17 non-HSDN capable UE should not consider any HSDN cells for reselection, we worry that is overkill and will cause problem when the non-HSDN capable UE is on the high speed train. The intention for deploying HSDN cell is to allow the UEs moving fast on the train to camp on the HSDN cells, even if the UE don’t support HSDN capability. - 5.2.4.3.0: Currently, the counting of inter-RAT cell reselections for mobility state determination is not specified in 38.304 and we have already different UE implementations in the field. But since we want to support inter-RAT cell reselection for HSDN cells in R17, should we clarify that inter-RAT cell reselections of HSDN cells should be counted for mobility state determination? [CMCC2]: During online discussion, LG commented this can be left to UE implementation. And Nokia also thought inter-RAT reselection should be rare. So I am not sure if we can specify it now. Maybe we can still leave it to UE implementation. |
| Apple | In 36.331 CR, “maxCellMeasIdle-r15” is used for hsdn cell list. We think a new name like “maxCellNR” (value=8) is better since maxCellMeasIdle is for measurement in idle state introduced for euCA. [CMCC2]: The new name “maxCellNR” is applied. And the value=8 is added to chapter 6.4 in 36.331CR. [Lenovo2] The suffix of the new constant should be “-r17” and not “-r15”. |

# Conclusions

TBD

# Annex: Agreements and comments in RAN2#115e

- MobState cell reselection for HSDN

R2-2108501	Mobility-state-based cell reselection to support NR High Speed railway Dedicated Network (HSDN)	CMCC, CATT, Ericsson, Huawei, ZTE, OPPO, vivo	discussion	Rel-17	TEI17

DISCUSSION

-	QC support this. Is there R4 impact? CMCC indicate that same as LTE there is no impact to R4.

-	LG support to resue this solution for NR. Can this feature be used for FR2 cells? CMCC has no strong opinion on the applicability for FR2. Think it is general.

-	Lenovo wonder for Inter-RAT cell reselection should be counted? There was earlier discussed and decided to be left to UE impl. LG think this can be left for UE impl, no need to specify for this.

-	Nokia think this is copy-paste of LTE solution to NR, and for LTE inter_RAT cell reselection is not counted, but this may not be very relevant, IRAT cell reselection should be rare.

-	Nokia have comments on the CRs.

-	Lenovo wonder about the direction of IRAT reselection. CMCC think it may be considered to support both direction but that would require a LTE RRC CR.

-	Apple wonder whether we need a UE capability. CMCC think in LTE there is no signalled capability. NR could use the same way. CMCC think we may need to check whehterh a CR is needed, in any case no signalling is needed.

- Introduce 1 bit indication in SIB1 to indicate the NR HSDN cell, same as LTE.

- HSDN neighbouring cell list with PCI(s) can be provided in SIB for intra-frequency, inter-frequency, and inter-RAT cell reselection, same as LTE.

- the number of equivalent cells can be indicated for MSE in SIB2, same as LTE.

- When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities), same as LTE.

- When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than network configured priorities), same as LTE

- Discuss and check the CRs in a post meeting email discussion, CRs for next meeting