3GPP TSG-RAN WG2#116-e	R2-21xxxxx

Online, November 1 - 12, 2021

Source: 	Huawei

Title: 	[Post115-e][219][R17 DCCA] UE-initiated SCG activation (Huawei)

Agenda Item:	x.x.x.x.

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[Post115-e][219][R17 DCCA] UE-initiated SCG activation  (Huawei)

Scope: Discuss the details of UE-initiated SCG activation and whether we need it. Should clarify technical aspects.

Intended outcome: report

Deadline:  October 21 UTC 0900

Q0: What is your name and email address?

| Company | Name and email address |
| --- | --- |
| OPPO | Shukun Wang (wangshukun@oppo.com) |
| Lenovo, Motorola Mobility | Congchi Zhang (zhangcc16@lenovo.com) |
| Apple | Naveen Palle ( naveen.palle@apple.com) |
| Ericsson | Stefan Wager (Stefan.wager@ericsson.com) |
| MediaTek | Felix Tsai (chun-fan.tsai@mediatek.com) |
| Qualcomm | Punyaslok Purkayastha (punyaslo@qti.qualcomm.com) |
| Futurewei | Jialin Zou (jialinzou88@yahoo.com) |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| Nokia | Jarkko Koskela (Jarkko.t.koskela@nokia.com) |
| CATT | Chandrika Worrall (chandrika@catt.cn) |
| NTT DOCOMO | Riki Okawa (riki.okawa.rp@nttdocomo.com) |
| NEC | Hisashi Futaki (hisashi.futaki @nec.com) |
| vivo | Wenjuan Pu (wenjuan.pu@vivo.com) |
| Spreadtrum | Lifeng Han (Lifeng.Han@unisoc.com) |

# 2	Discussion

## 2.1	UL data arrival

RAN2 agreed to support UE-triggered SCG activation and the following solutions were suggested:

1)	While the SCG is deactivated, all uplink data are only sent to the MCG and the network can use network-triggered SCG activation based on observed traffic or BSR on MCG.

This means that before SCG deactivation or at SCG deactivation, all DRBs with uplink are (re)configured with an MCG RLC bearer, which can be used for uplink.

FFS whether the network needs to explicitly deactivate all uplink transmissions via the SCG at SCG deactivation using existing signalling (e.g. deactivate PDCP duplication, set primary path to MCG) or this is done as a result of generic UP handling at SCG deactivation (like in case of SCG RLF).

2)	In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG and then the network can use network-triggered SCG activation. (FFS indication details).

3)	In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG (or uses SR on PUCCH) and resumes UL data transmission, using the received UL grants (in RAR or PDCCH).

This means that the UE configuration is not modified, and in the case of SR/PDCCH, the previously activated TCI states are used for PDCCH reception.

In order to allow the SN to respond to RA/SR quickly, the SN informs the MN of SCG activation and does not wait for confirmation from the MN to allocate grants (within RAR or DCI) to the UE and to send the UL data on MN-terminated bearers to the MN.

FFS: can the MN/SN modify the UE configuration before uplink data transmission? If yes, with or without data loss.

Solution 3) relies on the possibility for the UE to send RACH to the SCG to activate the SCG:

-	on the UE side, without any information at SCG activation

-	on the network side, without the possibility for the MN or the SN to reject the SCG activation, and without the possibility to do any RRC reconfiguration at SCG activation.

SN modification procedure as agreed by RAN3 for SCG activation/deactivation would need modifications to support this, so it depends on RAN3.

If RAN3 would support this, solutions 1 and 2 could also be optimized. For example:

- 	1a), the MN sends a network-triggered SCG activation indication to the UE, without any RRC reconfiguration, and notifies the SCG activation to the SN. Upon reception of the network-triggered SCG activation indication:

-	If RACH-less SCG activation is possible, the UE starts monitoring SCG PDCCH as soon as it can and the SN can allocate grants to the UE as soon as it receives the MN notification.

-	If RACH-less SCG activation is not possible, the UE will send a RACH to the SCG as soon as it can.

-	1b), the same as 1a) but the UE initiates SCG activation as soon as it has UL data for the DRB, without waiting for the network-triggered SCG activation indication. This behaviour could be configured per DRB (in case the network prefers to use the SCG for any UL data on a DRB).

-	2a), the UE sends an indication to the MN and

-	if RACH-less SCG activation is possible, the UE starts monitoring SCG PDCCH as soon as it can and the SN can allocate grants to the UE as soon as it receives the MN notification.

-	If RACH-less SCG activation is not possible, the UE will send a RACH to the SCG as soon as it can.

Below is a tentative comparison of the solutions, in terms of activation delay, specification impact and issues or limitations.

| Option | 1: Use MCG RLC bearers for uplink and network-triggered activation |  | 2: The UE sends an indication and the network can use network-triggered activation |  |
| --- | --- | --- | --- | --- |
| Delay components | - UE to MCG data/BSR - MN to SN message - SN to MN message - MN to UE SCG activation indication - RACH-less case: UE delay to start monitoring SCG PDCCH with indicated or previously activated TCI states - RACH case: delay to send RACH towards SCG and get initial grant |  | - UE to MCG activation request - MN to SN message - SN to MN message - MN to UE SCG activation indication - RACH-less case: delay to start monitoring SCG PDCCH with indicated or previously activated TCI states - RACH case: delay to send RACH towards SCG and get initial grant |  |
| Specification impact | None besides capturing the restriction. |  | Define the UL data arrival notification towards the MCG. |  |
| Issues / limitations | SCG activation delay is impacted by MN-SN round-trip delay. |  |  |  |
|  | The network must allocate an MCG leg for all DRBs |  | SCG activation delay delays initial UL data |  |
| Option | 1a: the UE transmits UL data via MCG and the MN activates the SCG | 1b: the UE transmits UL data via MCG and activates the SCG | 2a: the UE notifies UL data arrival to the MN and activates the SCG | 3: the UE activates the SCG |
| Delay components | - UE to MCG data/BSR - MN to UE SCG activation indication - RACH-less case: Max (UE delay to start monitoring SCG PDCCH with previously activated TCI states, MN to SN message) - RACH case: delay to send RACH towards SCG and get initial grant | - RACH-less case: Max(UE to MCG data/BSR + MN to SN message, UE delay to start monitoring SCG PDCCH with previously activated TCI states) - RACH case: delay to send RACH towards SCG and get initial grant | - UE to MCG activation request - RACH-less case: Max (UE delay to start monitoring SCG PDCCH with previously activated TCI states, MN to SN message) - RACH case: delay to send RACH towards SCG and get initial grant | Using RACH: delay to send RACH towards SCG and get initial grant Using SR: delay to send SR and start monitoring SCG PDCCH with previously activated TCI states |
| Specification impact | Modify MN-SN behaviour/interactions to support SCG activation without the possibility for the SN or the MN to reject the SCG activation or do any RRC reconfiguration at SCG activation |  |  |  |
|  | Support SCG activation indication without SN-provided information. | Trigger SCG activation upon UL data arrival. |  |  |
|  |  | Optional: whether to trigger activation for UL data is configurable per DRB | Define the UL data arrival notification towards the MCG. | The MN handles UL PDCP PDUs (for MN-terminated DRBs) from the SN even before SCG activation. |
| Issues and limitations |  |  | SCG activation delay also delays initial UL data |  |
|  | RACH-less only works when RACH-less activation without indication of PDCCH TCI state is possible (not feasible without BFD) |  |  |  |
|  | In RACH-less case, SCG activation delay is affected by one-way MN-SN delay, unless UE delay to start SCG PDCCH monitoring is larger. |  |  | RACH-less requires SCG SR resources (may reduce capacity) |
|  | RACH can only use CBRA (risk of collisions, some more delay) or permanently reserved resources for CFRA (may reduce capacity) |  |  |  |
|  | Reconfigurations may be required while the SCG is deactivated in order to be ready for SCG activation (increases signalling overhead and UE power consumption) |  |  |  |

A number of observations can be made:

-	2 could also be used for split bearers (e.g. if the network prefers the UE to use the SCG for UL data).

-	There are similarities between 1b and 3:

-	UL data arrival triggers SCG activation in both cases.

-	SCG activation is the same in RACH-case in 1b and 3 with RACH (except that initial UL data are transmitted via MCG in 1b)

-	on the UE side, SCG activation is similar in RACH-less case in 1b and 3 with SR (except that SR and/or BSR and/or UL data are sent to the MCG in 1b and SR is sent to the SCG in 3).

-	3 could also be used for split bearers, but this would delay SCG

-	1a, 1b, 2a and 3 are only feasible if RAN3 defines new procedures to support SCG activation without the possibility for MN/SN to reject activation or modify the configuration at activation, otherwise only 1 and 2 are feasible.

In addition, if such procedures would be defined, it would be necessary to decide whether they can always be used when the SCG is deactivated or it is a network choice and it is up to the SN and the MN to negotiate this together with the UE configuration.

Therefore, supporting such solutions may not be straightforward.

Important note: fast MCG link recovery via RACH to the SCG is discussed in the next section. It may or may not be supported regardless of which of the solution(s) for UL data is/are supported.

Q1: Do companies agree with the above descriptions, comparisons of candidate solutions and observations for UE-initiated SCG activation for UL data transmission? (please add any solution if needed)

| Company | Comments |
| --- | --- |
| OPPO | It is ok to us. |
| Lenovo, Motorola Mobility | For solution 1a, 1b, 2a, when MN receives the SCG activation indication from UE and sends the message to SN, it is still possible for SN to reject the SCG activation, right? For example, - SN sends a SN modification reject message over Xn to MN - SN does not provide any UL grant or not responding to RACH request from UE For solution 3 UE activates SCG, it is also possible for MN to reject the SCG activation, for example: - UE sends RACH request to SN - SN informs MN using SN modification required message - MN rejects the SN modification required message. - SN does not send RACH Response message to UE Thus, we don’t think the following assumption is really a precondition: - on the network side, without the possibility for the MN or the SN to reject the SCG activation, and without the possibility to do any RRC reconfiguration at SCG activation. For UE behaviour in this case, following legacy, upon not receiving RAR or UL grant, UE might try again sending RACH request or SR to SN, which is non idea. However, we believe such situation can be avoided/resolved by a reasonable NW implementation, e.g., reconfiguring UE as soon as possible. |
| Apple | Agree with the observations. We do not think UE self-activation of SCG along with sending info to the NW is needed (1a, 1b, 2a, 2b), but more details below. We understand that some level of monitoring the PDCCH on SCG is needed for 3. But just solutions 1 and 2 which are NW triggered (not 1a, 1b, 2a, 2b, where UE activates SCG without NW trigger) are good to pursue. |
| Ericsson | We agree with Apple that the combination of UE self-activation and sending info to the network is needed, but to our understanding that concerns only 1b and 2a. If we understand 1a correctly, the UE always waits for the network to activate the SCG? |
| MediaTek | Some clarification is needed. Solution 1 – This is baseline behavior. It should anyway be allowed and whether additional solution(s) are needed could be further discussed. Solution 1a – We understand it is still controlled by NW. But it is unclear what kind of “NW indication” we are talking about. Is it MAC CE? Solution 1b, 2a, 3 – We understand that the network should “enable” this kind of UE behavior if preferred. So, at some level, the network could still “control” the activation by not configuring it. In that case, solution 1 should be used. |
| Qualcomm | Generally agree with the above descriptions. Some comments: 1. Solution 3: We think that the following option should also be allowed. SN waits for confirmation from the MN before allocating grants. Even considering the additional delay involved in this option, the activation delay of Solution 3 may be comparable to (or better than) Solution 2. 2. One aspect that isn’t fully clear in the discussion above is whether the solutions should apply to all or to specific UL DRBs only. We think that: - Solution 1 (Network-triggered SCG activation based on observed traffic or BSR on MCG), Solution 1a, and Solution 1b, should apply in case of UL data arrival on split UL DRBs. - Solution 2 should apply in case of UL data arrival on UL SCG DRBs. - Solution 3 should apply in case of UL data arrival on UL SCG DRBs. |
| Futurewei | In general, we agree with Rapporteur’s analysis on the alternative solutions with comments: - 1b triggers UE access to SCG and initial SCG data transmission to MN before the SN acknowledgement. In case SN rejects the activation, there will be waste efforts. It would a trade-off with delay reduction consider the chance of SN rejection is very small. - Since the initial SCG data is sent to MN in 1b, it will experience MN/SN RTD comparing with the solution 3. - It is unlikely that the network reconfigures the bearer configuration just for SCG activation. There will be different scenarios of MCG/SCG split bearer or SCG only bearer with SN. The different SCG activation alternatives could be best fit in different scenarios. |
| ZTE | Solution 1 is not “UE-triggered SCG activation”, it is just network-triggered SCG activation based on the observed traffic or BSR on MCG (Same view as QC). The UE even does not know it was requesting SCG activation. And Solution 1 disallows network to configure SCG bearer while SCG is deactivated. Solution 2 looks simpler and may be considered as baseline, but the drawback is the long delay (compared to Solution 3), which makes this function less attractive. We don’t think 1a)/1b)/2a) can be compared with Solution 3, the main problem of 1a)/1b)2a) is that it does not allow SN to decide whether to accept or reject the procedure, as MN will first send indication to UE. ( 1a/1b/2a somehow contradict to the assumption in RAN3). For solution 3, even if the SN needs to coordinate with the MN first (before replying UE), the delay could be less than solution 2. We think solution 3 is configured on top of solution 2 (if network pursues low delay in specific case). If solution 3 is not configured/enabled, then solution 2 is taken. |
| Nokia | Regarding option 1 – could it be clarified is the intention that UE is deconfgiured with all the DRBs from SCG when SCG is deactivated? If so then wouldn’t this cause quite an extra configurations and basically also make even discussion on lower layer activation/deactivation command impossible? Regarding option 3: What is the specification impact? This options seems to be supported implicitly already by the specification without any additional work? ¨ Then For option 3 as specification impact one has listed “MN handles UL PDCP PDUS for MN-terminated DRBs) from the SN even before SCG activation.” How does this cause any impacts to spec for option 3? Option 2 on the other hand seems to have greatest impact as it seems to require unprecented coordination between different MAC entities. We think this should be avoided to avoid huge amount of work with possible issues that has not been identified for mixing handling of different MAC entities. |
| CATT | For solution 1, we agree that the network can (re)configure all RLC bearers with UL transmission to MN and make sure that there is no UL SCG RLC bearer upon deactivation SCG. In this case, the MN can perform network-triggered SCG activation based on the observed traffic or BSR on MCG, and the solution 1a can be taken into consideration. But for solution 1b, it is applies only to split bearer, if no split bearer is configured, and if the UE activates SCG upon increase of the data volume, the data will still be transmitted on MCG bearers after SCG activation which is not useful for the load balance. In this case, the MN should make the decision whether to activate SCG and reconfigure the SCG RLC bearers (i.e. solution 1a). Moreover, solution 1b is UE self-activation of SCG outside the NW control which should be avoided. Solution 2 seems to be a way which SCG activation be controlled by the NW fully. The UE can provide the assistance information to the MN to help the MN makes the decision whether to activate the SCG, which is similar as UE triggers SCG deactivation procedure. For solution 3, it may cause the UE to make the decision of SCG activation; meanwhile, we also need to further discuss the interface influence. In addition, we think the SCG activation/deactivation should be fully controlled by the network. The configuration can also be modified at activation which depends on the network. |
| DOCOMO | Trigerings of SCG activation should not be limited to UL traffics. The UE monitors other potential factors e.g. heating and battery, so the UE should be able to decide not to activate SCG even if UL traffics surge. Based on above view, we have some comments: Solution 1 – Same view as MTK. An additional comment is, if the UE prefers to be SCG deactivated for other reasons than traffics, solution 1 may cause endless round-trip between activation and deactivation. This can be avoided by supporting for the UE to notify SCG activation failure to the MN. (1a, 1b: Same view as 1) Solution 2 – Comparing with solution 3, more messages between the UE and the MN needed (in other words, takes more delay), and the network has more control. Solution 2a – We are not sure why the notification from the UE to the MN is needed when the MN cannot reject activation. Solution 3 – We prefer this option than 1a, 1b, 2a, and 2. We think solution 3 should be supported because we have a high motivation to reduce messages. (Details in our comment for Q2) “Issues and limitations” is limited because the UE releases PUCCH resources when the TAT associated with the SCG expires. |
| NEC | Mostly fine as comparison. However, some clarifications or modifications seem necessary as previous/above comments from companies. Especially, Option 1b,2a,3 should not be understood as “self-activation” by the UE. Option 1b,2a are not really “network-triggered activation”. In Option 3, UE can/may send RACH, but it should not mean the SCG is activated by UE itself and the activation by network (e.g. SN) is necessary. Also, regarding the network control aspect raised by MediaTek, we agree. In any option, network should have some level of control (including enabling/disabling a function). |
| vivo | We share similar view with QC on the following: solution 1 should apply in case of UL data arrival on UL split DRBs, and solutions 2/3 should apply in case of UL data arrival on UL SCG DRBs. Moreover, for solution 3, the network should also be allowed to reject the SCG activation requested from the UE via RACH/SR. That is, upon the UE initiating the RACH/SR on deactivated SCG, the UE starts monitoring SCG PDCCH. If UL grants are received, the UE understands the SCG is activated. If the SCG deactivation indication is received, the UE knows the SCG activation request is rejected and the UE stops monitoring the SCG PDCCH. Note that the RAN2 agreement “Indication of SCG deactivation to the UE via the SCG is not supported.” is just for NW-triggered SCG activation case. |
| Spreadtrum | For solution 1, it can apply to UL data arrival on both UL split DRBs and SCG DRBs with pre-configuration by the network before deactivation SCG. |

Q2: Do companies want to support in Rel-17 SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | It is possible to reallocate radio resource and bearer type due to load in SCG side and MCG side. |
| Lenovo, Motorola Mobility | No, and we don’t think this is an issue | We believe reasonable NW implementation shall not reject, but we don’t think it’s needed to put such restriction in the spec. In addition, please see our comment in Q1, we don’t believe “NW cannot reject” is the precondition for solution 1a, 1b, 2a, 3 |
| Apple | We do not oppose. | Do not see why not. The re-configuration can happen later as well. In our understanding, modification of SCG at activation is always possible with NW triggered activation. But for the cases of UE initiated activation, we can agree that for rel-17, modification is not allowed for R-17. |
| Ericsson | No | The network should be in control of the procedure and there may be a need for RRC reconfigurations during SCG activation/deactivation. |
| MediaTek | Prefer no | NW should of course be able to “control” the UE activation. We are not sure if no configuration change does work but we are open for discussion. |
| Qualcomm | Please see comments | As pointed out by the rapporteur, supporting SCG activation without the possibility for SN or MN to reject requires RAN3 approval/procedures, which seems unlikely. We are not supportive of it. Regarding supporting SCG activation without modifying the UE configuration, it seems possible since it has been agreed that configuration can be provided while in SCG deactivated. We are supportive of it. |
| Futurewei |  | Normally, if activation delay is not a big concern, R17 SCG activation should allow MN/SN configuration modification and rejection. It would be more resource efficient. On the other hand, in case of extremely delay sensitive activations which are worth to use more resources, certain solutions could be FFS, maybe in R18. |
| ZTE |  | Since we think solution 3 is used on top of solution 2, whether to enable solution 3 is already within network’s control. To achieve low latency ultimately, network (SN) can ensure the configuration is valid when solution 3 is enabled (which means modification can be avoided). But we also see no problem to allow network to reject or reconfigure UE during UE-initiated SCG activation. It is up to network, unclear the consequence of this question? |
| Nokia |  | Although not sure what is the question? It seems rather irrelevant? Of course NW can always re/deconfigure SCGif it desires to do so? So we are not sure what is the intention of the question. |
| CATT | No | We think the SCG activation/deactivation should be fully controlled by the network, and any configuration can also be modified at activation/deactivation e.g. for load balancing. |
| DOCOMO | Yes | This could be a bottleneck in signal reduction of SCG activation. We assume SCG activation and deactivation will occur frequently in the actual operation especially of FR2, so we have a high motivation to reduce messages. At the same time the network should be able to configure certain UEs (e.g. which unnecessarily activate and deactivate SCG) not to trigger SCG activation. |
| NEC | No | Regardless it is explicit or implicit, the network shall have a control of SCG activation. The UE should not consider the SCG is activated upon sending e.g. RACH. |
| vivo |  | The NW should be allowed to accept or reject the SCG activation request from the UE for all cases, and thus the NW anyway can use MCG to reconfigure SCG if needed before sending the SCG activation result to the UE. |
| Spreadtrum |  | Network can make the decision whether activation or not. The MCG leg is activated and can be used for the UE anyway. |

Q3: If SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation is supported in Rel-17, which solution do companies prefer to support for UL data arrival on SCG bearer? On split bearer? (could be different)

| Company | Solution(s) | Comments |
| --- | --- | --- |
| Apple | We prefer solution 1 (not 1a, 1b), which is also simplest in terms of spec impact. Next preference is solution 2 (not 2a, 2b) with UAI, which is useful for SCG bearer. | With both sol 1 and sol 2, and with the assumption that the UE does NOT activate SCG until the NW triggers it (NW responds to UE with sol 2, NW triggers with Sol 1), many aspects are resolved, and would be simplest for Rel-17. So do not prefer Sol 1a, 2b, 2a. We do not need to address the SR/PUCCH aspects. We can evaluate others in Rel-18 except for MCG recovery. |
| Qualcomm | Please see comments | In this case we support the following solutions as they can have lower activation delays: - For UL data arrival on split UL DRB, Solutions 1a, 1b. - For UL data arrival on UL SCG DRB, Solutions 2a, 3. |
| Futurewei |  | We consider 1b is such a solution maybe still adopted in R17 for the case of SCG split bearer. |
| DOCOMO | 3 | Please see our comment in Q1. |

Q4: If SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation is NOT supported in Rel-17 (i.e. 1a, 1b, 2a and 3 are not possible), which solution do companies want to support for UL data arrival on SCG bearer? On split bearer? (could be different)

| Company | Solution(s) | Comments |
| --- | --- | --- |
| OPPO | 1a | Except SCG activation indication from UE to network, the rest part of procedure is same as DL data arrival case. |
| Lenovo, Motorola Mobility | 3 | Please see our comment in Q1, we don’t believe “NW cannot reject” is the precondition for solution 1a, 1b, 2a, 3. Among those solutions, we think 3 is the simplest and shall be supported as a start point. For 1a, 1b, 2a, some careful analysis about the signalling flow is needed. |
| Apple | Sol 1 and Sol 2 | We anyway do not prefer UE self-activation of SCG after sending something to MCG (1a, 1b, 2a). |
| Ericsson | 1 or 1a | This causes least specification complexity, as it builds on legacy BSR reporting for indicating arrival of UL data and network triggered SCG activation, which is same as for arrival of DL data. |
| MediaTek | 1 and 2 | Solution 1 is baseline for split bearer. Solution 2 could be used if we want to keep SCG bearer in SCG deactivation state. |
| Qualcomm | Please see comments | In this case we support the following solutions: - For UL data arrival on split UL DRB, Solution 1 (Network-triggered SCG activation based on observed traffic or BSR on MCG). We do not think it is necessary and do not support reconfiguration of UL bearers upon deactivation, since network should deactivate the SCG only when SCG resources are not required to handle the traffic load. - For UL data arrival on UL SCG DRB, Solution 2 (UE sends indication to MN, network triggers SCG activation). |
| Futurewei | 1 and 2 | 1 is for MCG or SCG split bearer. 2 is for SCG bearer. There could be existing SCG bearer with a deactivated SCG. We don’t see a need to reconfigure the bearers only for SCG activation. |
| ZTE | 2 and 3 | As commented to previous questions, we don’t think Solution 3 disallows MN/SN to reject or modify the UE configuration. If SCG bearer is configured when SCG is deactivated, we think 2 is enabled by default, if network pursues low latency, network can further enable 3 on top of 2. |
| DOCOMO | 2 | As commented in Q1, trigerings of SCG activation should not be limited to UL traffics. Solution 2 enables the UE to consider other factors than UL trafffics before indicating activation. |
| NEC | 1 and 2 | 1 for split bearer, 2 for SCG bearer No sub-options, i.e. 1a,b,2a |
| vivo | 2/3 for UL data arrival on SCG bearer, 1 for UL data arrival on split bearer |  |
| Spreadtrum | 1 and 2 |  |

## 2.2	MCG failure recovery via the deactivated SCG

Another proposal is specific to MCG failure recovery, using the deactivated SCG to send the MCGFailureInformation message. In this case, communication via the MCG is not possible so the UE needs to initiate transmission towards the SCG, using either random access or SR.

In this scenario, upon transmission of the MCGFailureInformation message using the SCG RLC bearer associated the SRB1 or SRB3, the UE starts a timer and expects a reconfiguration quickly. In this scenario, it may not be so useful to resume operation on SCG RLC bearers associated with DRBs before the reconfiguration, it is sufficient to resume the SCG DRB associated with SRB1 or SRB3.

Supporting UE-initiated activation of the SCG only for SRB1/SRB3 to send one RRC message and receive an MCG RRC reconfiguration, while MCG transmission is interrupted, is a rather different procedure as compared with full SCG activation simultaneously with MCG operation as in option 3) above, so it should be considered separately and compared with RRC re-establishment.

Below is a tentative comparison of the solutions, in terms of delay until resume of MCG operation, specification impact and issues or limitations.

|  | 1) RRC reestablishment | 2) RACH/SR towards the SCG and resume of SRB1/SRB3 |
| --- | --- | --- |
| Delay | - Cell selection - CBRA with RestablishmentRequest - Context fetch - RRC re-establishment - first RRC reconfiguration | - RA/SR with MCGFailureInformation - Handover preparation - RRC reconfiguration |
| Specification impact | None | The procedure should be configured to the UE, need to ensure support by MN and SN (could be similar to support and configuration of MCG recovery in Rel-16). If CFRA is used, preallocation of resources. If SR is used, SR resources need to be allocated. |
| Issues / limitations | - The network must allocate an MCG leg for all DRBs On the optional optimization: - only works when RACH-less activation without indication of PDCCH TCI state is possible | If CFRA is used, this uses permanently allocated RACH resources, which reduces capacity. If SR is used: - need to reserve PUCCH resources - only works when RACH-less activation without indication of PDCCH TCI state is possible |

Q5: Do companies agree with the above descriptions and comparisons of candidate solutions for MCG RLF while the SCG is deactivated? (please add any solution if needed)

| Company | Comments |
| --- | --- |
| OPPO | For solution 2, CB RACH can be used. And in MSG3, the SCG activation indication can be transmitted. Except this, RAN2 should also consider how to handle the case that SCG deactivation command is received while T316 is running. |
| Lenovo, Motorola Mobility | The description looks fine, agree with rapporteur for MCG link recovery only activating SRB1 or SBR3 is enough, but not sure if it is needed to define a “half” activated SCG, or it could be simpler to fully activate the SCG from spec point of view. |
| Apple | Agree, and also we do not see the necessity to have a CFRA for RLF recovery. MCG failure should not be a common case to have NW assign dedicated resources for this. |
| Ericsson | We don’t see how the issues/limitations listed for RRC reestablishment are related to the RRC reestablishment? They seem more related to SCG deactivation as such? Only issue/limitation we see is the longer interruption time compared to recovery via the deactivated SCG. For the recovery via deactivated SCG, a possible issue is the added step of SCG activation than needs to be executed before the MCGFailureInformation message can be transmitted. This extra step may decrease reliability of the MCG recovery procedure. |
| MediaTek | Similar view as Ericsson, it is unclear why “must allocate an MCG leg for all DRBs” is a limitation for re-establishment. The limitation is related to the solutions that discussed in section 2.1. It seems not direct link to solution 1 here. For solution 2, it should be clarified that this works only if SCG could be activated without any RRC configuration change (i.e. it depends on the solution chosen from section 2.1). |
| Qualcomm | Agree with the above descriptions. |
| Futurewei | In general agree with above analysis. |
| ZTE | Similar comment as Ericsson and MTK, why “must allocate an MCG leg for all DRBs” is the limitation for solution 1? Shouldn’t the issue of solution 1 be longer interruption? For 2, we agree it is simpler to always use CBRA. |
| Nokia | Maybe one could also assess what are impacts to specifications. It seems option 2 is already supported by the existing RRC, X2ap and XnAP? Or do we have wrong understanding? We think this is the case for CBRA. |
| CATT | For 2, besides the response with RRC reconfiguration with reconfigWithSync, the NW may response with RRC release message. If the NW response with RRC release message, the delay for resume SRB1 will be longer. And the precondition of using solution 2 is the configuration of split SRB1 or SRB3. So the solution 2 is applicable for a specific case. Further, the delay analysis for solution 1 and solution 2 is not consistent. For solution 1, the delay analysis including resuming the DRB, but for solution 2 it only includes the reception of the response message, in order to make the delay comparison is reasonable, the delay for solution 2 should also include the delay of sync to the target PCell and perform CBRA/CFRA to the target PCell to resume DRB transmission. Agree with Ericsson that the only issue/limitation for solution 1 is not accurate. |
| DOCOMO | For solution 1, we have similar view as Ericsson, the drawback should be the interruption time. Descriptions of solution 2 is ok to us. Simpler way is to always use CBRA. |
| NEC | mostly agree, but have same comments as Ericsson and others on “must allocate an MCG leg for all DRBs”. it is not sure about this. |
| vivo | Agree with Ericsson on that issue/limitation for solution 1 is not accurate. Besides, the drawback of solution 1 also include SCG release, which may cause more signalling overhead for SCG addition and more latency for starting SCG transmission. |
| Spreadtrum | Generally agreed. |

Q6: Which solution do companies prefer for MCG RLC while the SCG is deactivated?

| Company | Solution(s) | Comments |
| --- | --- | --- |
| OPPO | 2 | CB RACH can be used and MCGFailureInformation is included in MSG3. |
| Lenovo, Motorola Mobility | 2 |  |
| Apple | Sol-2 (a simplified version of this) | UE would RACH to provide the MCG Failure RRC message using SRB1/SRB3, without any changes to DRBs. We already agreed that DRBs/PDCP entities associated with these are not suspended. So no special action is needed, just a change in spec to have the UE trigger sending the SRB message, which would trigger a RACH (assuming SR is not considered valid in SCG deactivated state). The additional aspect would be for the UE to monitor the PDCCH (which would be new for the SCG deactivated state). We think not allowing MCG recovery would be not making use of the available SCG. |
| Ericsson | 1 | While we are in general supportive of MCGFailureInformation, we think that if SCG activation is only supported via MCG (solution 1 in Q1), then for simplicity it is better to trigger RRC reestablishment upon MCG failure when SCG is deactivated. Also the extra step of SCG activation before transmission will increase the delay and decrease the reliability of the MCG failure recovery procedure. |
| MediaTek | 1 | We are not sure fast MCG failure recover is within the scope of the WI. But anyway, we don’t not see motivation to have this enhancement. The MCG RLF should be low possibility in normal deployment. MCG RLF happened in SCG deactivation should be a rare case. Simple solution is enough. |
| Qualcomm | Solution 2) | We think Solution 2) based on RRC MCG Failure Information procedure should be faster since the cell selection component of RRC reestablishment could take a significant amount of time, the other delay components of the two solutions being somewhat comparable. Other delay advantages of Solution 2): - In Solution 2), CFRA/SR can be used to initiate SCG activation, which contributes to lower delay. - In Solution 1), UE performs measurements based on configuration provided in the first RRC reconfiguration and network configures UE with DC/CA in a subsequent RRC reconfiguration based on the reported measurement results. In Solution 2), UE can provide available measurement results in the RRC MCGFailureInformation message itself. This may help with quicker DC/CA configuration. |
| Futurewei |  | We acknowledge the potential less delay benefit with solution 2. But we are not sure it could be done in R17. If the UE direct access to SCG “solution 3” is not developed for R17, so be the fast recovery solution 2. It may be FFS in R18. Solution 1 could be the baseline in R17. |
| ZTE | 2 | “MCG fast recovery” and “fast SCG (de)activation” are designed for different purposes, they both have benefit. We do not understand why operator/network has to give up one when the other one is enabled. In our view, MCG failure is not a rare case, there are several reasons that may trigger RLF. (RACH failure, T310 timer out, RLC failure…) These are the main factors that impact network performance. And we think no matter SCG is activated or deactivated, there is no difference in MCG failure rate. The solution 3 discussed in section 2.1 is for UL data transmission, but MCG failure information is transmitted through SRB 3 or the SCG leg of split SRB 1. The concern on Solution 3 in previous questions should not be a problem here, because SN just needs to forward the message to MN and wait for follow-up reconfiguration, there is no complex MN-SN operation. |
| Nokia | 2 | Seems basically already being in the specification without additional work |
| CATT | 1 | There is a risk of activation failure which could increase more delay for solution 2. We think solution 1 (i.e. re-establishment) is the simplest way for this case without spec impact and without restriction for using. |
| DOCOMO | 2 |  |
| NEC | 1 | No need to mix up the issues. In Rel-17, the work should focus on real motivation, i.e. efficient SCG activation and deactivation. |
| vivo | 2 | Although SCG activation failure may occur, the failure is low possibility. Compared to the reduced latency for MCG recovery, such risk is acceptable. |
| Spreadtrum | 1 |  |

# 3	Conclusion