- 3GPP TSG RAN WG2 Meeting #116-e      	                          R2-210xxxx
E-Conference, 1st – 12th Nov. 2021

Agenda item:	8.7.3.1

Source:	Qualcomm Incorporated

Title:	Summary of email discussion#611 - Discovery shared/dedicated pool issue (Qualcomm)

WID/SID:	FS_NR_SL_relay – Release 17

Document for:	Discussion and Decision

# Introduction

This is for below email discussion:

- [Post115-e][611][Relay] Discovery shared/dedicated pool issue (Qualcomm)

Scope: Clarify from the UE perspective the terminology on the network configuration of dedicated discovery pool vs. shared pool for communication and discovery, and determine whether to support option 2 from discussion [AT115-e][617], in which the network configures both shared and dedicated pools.

Intended outcome: Report to next meeting

Deadline:  Long

The discussion is split into two phases:

- Phase 1: Companies are invited to provide views on the questions provided until the deadline: October 14th, 1200 UTC

- Phase 2: Rapporteur submits summary based on provided inputs and comments from companies until October 20th, 1200 UTC.

# Discussion

In existing TS 38.331 [1], TX and RX resource pools can be configured in NR SIB12, dedicated RRC message and pre-configuration, as illustrated in Figure.1, Figure.2 and Figure.3.

Figure.1 Rel-16 resource pool configuration in SIB

Figure.2 Rel-16 resource pool configuration in dedicated RRC message

Figure.3 Rel-16 resource pool configuration in pre-configuration

To align RAN2 understanding, Rapporteur would like to discuss the following 3 possible cases separately based on Rel-16 resource pools configuration framework:

- Only shared pools are configured

- Only dedicated pools are configured (if support)

- Both shared pools and dedicated pools are configured (if support)

## Only shared pools configured

It is obvious that shared pool will reuse Rel-16 resource pool configuration. Because shared pool was agreed as baseline, Rapporteur would like to first discuss the case when only shared is configured.

According to companies’ contributions in RAN2#115-e [1-9], there are two different understandings on shared pool configuration for discovery:

- Alt-1: All the configured TX pools (in SIB/RRC/Pre-config) can be used for discovery and SL communication [3-9]

  - No ASN.1 change compared with Rel-16 resource pool configured

- Alt-2: Extra indication is introduced to indicate whether each configured TX pool (in SIB/RRC/Pre-config) is shared for discovery and SL communication, or for SL communication only [9].

  - Extra indication (e.g. SLcommOnly) is required to be introduced within SL-ResourcePoolConfigm, e.g., the TX pool only for SL communication if SLcommOnly is set:

Q1: If only shared TX pools are configured in SIB/RRC/Pre-config, which understanding on shared pool configuration for discovery do you prefer?

- Alt-1: All the configured TX pools (in SIB/RRC/Pre-config) can be used for discovery and SL communication

- Alt-2: Extra indication is introduced to indicate whether each configured TX pool (in SIB/RRC/Pre-config) can be used for discovery and SL communication, or for SL communication only.

| Company | Alt-1 or Alt-2? | Comments |
| --- | --- | --- |
| OPPO | Alt-1 | The purpose of having shared Tx pools is to simplify network configuration and avoid resource fragmentation. Thus, if still need to have one bit to differentiate whether the Tx pools can be shared, the gain would be offset. |
| Ericsson | Alt-1 | We don’t see the need to have communication only resource pool, which would require extra design efforts. Also, the benefits are unclear. |
| InterDigital | Alt-1 | We see no benefit to Alt-2. |
| Qualcomm | Alt-1 | We have only up to 8 TX pools (i.e., maxNrofTXPool-r16=8) can be configured. There is no need to put further restriction on legacy TX pool in Alt-2 (i.e., whether SL communication only), which may decrease capability of the system. For Alt-1, our understanding is: - For Mode 1, it is up to gNB implementation to decide whether the shared pool is used for discovery or SL communication in one time instance - For Mode 2, UE follows legacy LCP procedure to decide whether the shared pool is used for discovery or SL communication in one time instance |
| Kyocera | Alt 2 | We think there are some benefits to having the indication for NW control for Mode 2, but we think the real benefit comes when both shared pool and dedicated pool are available for discovery, it should be considered if the indication in the shared pool could allow the NW to move part or all the discovery transmissions to the dedicated pool. |
| Intel | Alt-1 | We prefer simple solution for baseline configuration. |
| LG | Alt-1 | We understand shared TX pool means discovery message can be treated as communication message. So, all the pools that is transmitted communicate messages can be also transmitted discovery messages. We don’t need to any restriction to indicate discovery pool for discovery messages as Alt-2. |
| ZTE | Alt1 | Once Tx resource pool is configured without extra indication, it can be regarded as shared resource pool by default. It is not necessary to further indicate whether it is used for discovery and SL communication, or for SL communication only. |
| vivo | Alt-1 | We don’t agree with any communication-only pool till now, nor it is necessary. When dedicated resource pool is not configured, shared pool should be used for both discovery and SL communication. |
| Sharp | Alt-1 | Alt-2 is more complicated, and it seems there be no additional benefit. |
| Spreadtrum | Alt1 | We understand that the shared pool means both discovery and communication messages can be transmitted. |
| Sony | Alt-1 | We don’t see the motivation to introduce an extra indication as in Alt-2 |
| Huawei, HiSilicon | Alt-2 | Similar to the motivation to introduce dedicated pool, we think extra indication to restrict the usage of shared pools for discovery is beneficial for UE power consumption. For instance, in the case there are less Remote UEs in the network, gNB could configure some of the communication RPs as discovery RP, so that the UEs only needs to monitor such RPs during discovery procedure to save power. |
| Apple | Alt-1 | Discovery traffic are upper layer traffic and there is no obvious reasons to limit the usage of (shared) pools for any discovery traffic. In other words, there is no need to reserve dedicated pool for non-discovery traffic. |
| Lenovo&MM | Alt-1 | We don’t see additional benefit for Alt-2. |

## Only dedicated pools configured?

Although RAN2 agreed to support dedicated pool besides shared pool as illustrated below, there is no explicit agreement whether only dedicated pool can be configured in SIB/RRC/Pre-configuration.

Proposal 6 [discussion]: RAN2 agrees dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design).

In RAN2#115-e [1], there were below two understandings for the UEs which are interested in discovery only:

- Understanding 1: these UEs can be configured with only dedicated pool for discovery (i.e., legacy resource pool is NOT configured).

- Understanding 2: these UEs should always be configured with legacy resource pool and optionally configured with dedicated pool for discovery.

Rapporteur’s understanding is:

- Dedicated RRC message: it seems to be valid that only dedicated pools can be configured for the UEs which are only interested in discovery (but without interest in SL communication)

- SIB: it seems there is no clear use scenario where SIB only configures dedicated pools because we need to support Rel-16 legacy V2X UE anyways

- Pre-configuration: it seems to be valid that only dedicated pools are pre-configured for the UEs which are only capable of, or interested in discovery

Because the answers may depend on how dedicated pools are configured, Rapporteur would like to confirm case by case. Please note that this question is NOT intended to discuss whether shared pool and dedicated pool can be configured simultaneously.

Q2: For RRC_IDLE/RRC_INACTIVE UEs which are interested in discovery only, do you agree that they can be configured with only dedicated pools for discovery in SIB?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | The gNB may not be aware of which UEs are only interested in discovery. Also, a UE may move from a cell to another cell. The situation for a cell will change from time to time. there is no point to limit a cell to only signal dedicated resource pool for discovery in the SIB. |
| InterDigital | No | We agree with rapporteur that we need to support legacy V2X UEs |
| Qualcomm | No | Legacy resource pools (i.e., shared pools) are required to be configured in SIB to support Rel-16 legacy V2X UEs. |
| Kyocera | Yes | We think it’s a bit strange to discuss about configuring UEs with SIB, rather we think the UEs would just use what’s available in SIB. Although V2X is legacy, there is no requirement for a gNB to support V2X, so if there are only UEs that are interested in discovery, then there’s no reason for the gNB not to include only dedicated pool for discovery in SIB. However, we think the typical case is for UEs interested in discovery to also need SL communication resource in which case both dedicated pool and shared pool should be available in SIB. |
| Intel | No at least for relay discovery | Assuming relay discovery case, we consider Understanding 2 above and prefer that shared resource pool is always configured and optionally dedicated pool is configured for discovery. We understand the motivation to reduce overhead/power consumption for discovery-only UEs to monitor shared resource pool, however, is the information that these are discovery-only UEs available to the gNB? We think it is up to the gNB to configure dedicated resource pool or not necessarily for discovery-only UEs and it is an optimization but if majority companies prefer to introduce it, we can be flexible. If we are to support non-relay discovery, it is possible to have UEs that support only discovery, in which case, we would like to understand if RAN2 will differentiate dedicated pool for relay vs. non-relay discovery. |
| LG | No | Shared resource pool configuration is required to support legacy Rel-16 V2X UEs. |
| ZTE | No | It is hard for the network to exactly know the existence of RRC_IDLE/INACTIVE UEs and whether some of them only have interest in discovery. In order to support the RRC_IDLE/INACTIVE UEs as many as possible, network should at least configure the shared resource pool and optionally dedicated discovery resource pool. |
| vivo | Yes with comments | We understand there may be remote UEs which are only interested in discovery message and they can monitor the dedicated resource pool only, but as companies pointed out, it is questionable that whether all the UEs under a gNB would only perform discovery operation. If the scenario is valid, then we think the answer for this question could be ‘Yes’. On the other hand, this is actually left to gNB implementation, so the question is indeed asking whether gNB can provide SIBs with only pools which are dedicated for discovery, and it seems OK if gNB wants to do so. |
| Sharp | No | Agree with rapporteur. |
| Spreadtrum | No | The gNB should configured both pools for discovery and communication in SIB for RRC_IDLE/RRC_INACTIVE UEs, since the gNB is not aware whether the UEs are only interested in discovery. |
| Sony | Yes | We think this configuration option should be supported. |
| Huawei, HiSilicon | See comments | From UE perspective, to know the RP used for discovery it is enough that network can provide dedicated RP (in SIB). But considering the purpose of discovery is to do the SL communication for relaying then communication RP configuration is needed as well. So from network perspective, it should provide dedicated RP for discovery and legacy communication RP for SL communication in legacy way. |
| Apple | See comment | I assume both dedicated pool for discovery, and pools for non-discovery are to be configured in SIB in this case. So, not sure what about “dedicated only” mean here? UEs are configured with all pools in SIB, it is up to UE to decide which pool to use. |
| Lenovo&MM | No | Network is not aware if there is RRC_IDLE/RRC_INACTIVE UEs which are interested in discovery only. |

Q3: For RRC_CONNECTED UEs which are interested in discovery only, do you agree that they can be configured with only dedicated pools for discovery in dedicated RRC message?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | In general, we don’t see the need for the dedicated resource pool for discovery, which may decrease resource utilization issue. If there are few Ues interested in discovery only, the dedicated resource pool will be mostly likely to be under utilized. |
| InterDigital | No | This should be a network choice. |
| Qualcomm | Yes | If the concerned UE is only interested in discovery, RRC can only configure dedicated pools for this UE, to save power consumption. |
| Kyocera | Yes | For dedicated RRC signaling, we think it can be up to NW to decide what to configure to the UE, including only discovery pool if that’s all the UE needs. |
| Intel | No | Same comment as above |
| LG | No | We don’t still understand why discovery dedicated pool is required. It will decrease resource utilities. Power saving can be achieved by other operation (e.g., DRX operation) other than defining discovery dedicated pools. |
| ZTE | No | For U2N relay scenario, relay UE/remote UE should be interest in both sidelink discovery and communication. It does not make sense to consider the scenario that relay/remote UE only interested in discovery and only the dedicated discovery resource pool configuration via SIB. With regard to the other ProSe discovery scenario, it has been captured in the latest SL relay WID that “RAN2 prioritizes completion of relay discovery work, and for 5G ProSe Direct Discovery (Objective 7) takes the agreement for relay-based discovery achieved as baseline while enhancements and optimizations beyond basic functionality for the 5G ProSe Direct Discovery scenario are not considered.” Based on this observation, it is suggested not to consider or de-prioritize the potential enhancement specific for non-relay discovery scenario, for example, how to enable the network to know the discovery only and or communication only interest of Ues. |
| Vivo | Yes | Not sure for now that how the gNB can determine a remote/relay UE in RRC_connected is only interested in discovery message. But we are OK to have this option. |
| Sharp | No | But maybe it is possible if used with SIB method together. |
| Spreadtrum | Yes | Left to NW implementation. |
| Sony | Yes |  |
| Huawei, HiSilicon | Yes | If assuming the resource for relay operation would be configured via dedicated RRC configuration, then we think the discovery RP and communication RP will be requested in different time points, i.e. via separate SUI messages. Thus during discovery procedure, the UE may be only configured with discovery dedicated RP configuration but without SL communication configuration. |
| Apple | Yes with comment | We are not sure why a remote UE or relay UE has to be only interested for discovery. But if there is a pool dedicated for discovery purpose, it could be configured to TX UE in a dedicated RRC signaling. |
| Lenovo&MM | No with comments | It is gNB implementation whether to configure dedicated pools for discovery in dedicated RRC message. No specification impact. |

Q4: For OOC UEs which are interested in discovery only, do you agree that they can be pre-configured with only dedicated pools for discovery via pre-configuration?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | In general, we don’t see the need for the dedicated resource pool for discovery, which may decrease resource utilization issue. If there are few Ues interested in discovery only, the dedicated resource pool will be mostly likely to be under utilized. |
| InterDigital | No | This should also be a network choice. |
| Qualcomm | Yes | If the concerned UE is only interested in discovery, pre-configuration can only configure dedicated pools for this UE, to save power consumption. |
| Kyocera | Yes | If the UE is really only interested in discovery, then pre-configuration with dedicated pool should be possible. |
| Intel | No | Same comment as above |
| LG | No | We don’t still understand why discovery dedicated pool is required. It will decrease resource utilities. Power saving can be achieved by other operation (e.g., DRX operation) other than defining discovery dedicated pools. |
| ZTE | No | Similar comments as Q3. We think the UE interested in discovery only does not applicable for U2N scenario. The potential enhancement specific for non-relay discovery scenarios may be de-prioritized. |
| Vivo | Yes |  |
| Sharp | No |  |
| Spreadtrum | Yes |  |
| Sony | Yes |  |
| Huawei, HiSilicon | No | As we commented above, at least for U2N relay operation, the UE will not only interested in discovery. The purpose of discovery is for U2N relay communication, which involves the normal sidelink communication. Based on this, we don’t think it is feasible to pre-configure with only dedicated discovery RP. |
| Apple | Yes with comment | We are not sure why a remote UE or relay UE has to be only interested for discovery. But if there is a pool dedicated for discovery purpose, it could be preconfigured, along with other non-discovery pools. |
| Lenovo&MM | NO | Left for implementation. |

## Both shared and dedicated pools configured?

In offline discussion#617 [1], this topic was controversial if the UE is interested in both discovery and communication. Rapporteur understand that one reason of the chaos is because different aspects were mixed during the discussion of [617]. Thus, Rapporteur would like to first make clear the assumptions and the issue.

- Assumption: focus is the UEs which are interested in both discovery and SL communication.

- Issues:

- Whether both shared and dedicated pools can be configured simultaneously.

  - It is common understanding that the UE can be configured with only shared pools.

  - The controversial part is whether dedicated pools can also be configured besides shared pools. Some companies had concern that power saving gain of dedicated pool can’t be achieved if both are configured.

- Assume both types of pools for discovery can be configured simultaneously, whether shared pool can be used for both discovery and SL communication, or SL communication only.

  - Some companies had understanding if both are configured simultaneously, shared pool can only be used for SL communication.

For above two issues, Rapporteur would like to discuss separately for SIB, RRC and pre-configuration.

### SIB configuration

First, we discuss configuration in SIB for RRC_IDLE/INACTIVE UE. Please note that the serving cell may have sidelink communication only UEs, discovery only UEs, and UEs which are interested in both discovery and SL communication. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.4 Example of both shared and dedicated pools configured in SIB (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pools (i.e., sl-RxPool and sl-TxPoolSelectedNormal):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously.

Q5: For RRC_IDLE/RRC_INACTIVE UEs, which option(s) do you think can be configured for discovery in SIB besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a) | The benefit to introduce dedicated pools for discovery only is to consider the power consumption for those UEs who are only interested in discovery transmission/reception. To ensure the benefit, shared pools should only be used to transmit sidelink data. |
| Ericsson | b) | If dedicated and shared resource pools can be configured together, option b would be better. The other options put too much restriction to the gNB. |
| InterDigital | b) | The purpose of configuring both dedicated and shared pool is to have flexibility on where to transmit the discovery. So option b) is the only one providing this flexibility. |
| Qualcomm | b) | For IDLE/INACTIVE UEs, camping cell is not aware of whether they are discovery only, or SL communication only, or supporting both. Thus, we think it makes sense that NW is allowed to provide both shared pool and dedicated pool simultaneously in SIB for IDLE/INACTIVE UEs. Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we prefer b) because 2 issues of a) are: - The dedicated pool may be configured sparsely. As a consequence, discovery will take longer if only the dedicated pool can be used for discovery. - For mode 2, it may bring extra (and unnecessary) spec impact on LCP restriction, e.g., TX shared pools need to be precluded in LCP procedure (similar to Rel-16 SL HARQ A/N enable) when UE sends discovery message In option b), we prefer it is up to UE implementation in Mode 2 to select shared pool or dedicated pool to transmit discovery message. |
| Kyocera | b) | Although limiting the use of discovery transmissions using only dedicated discovery resource pool or only shared resource pool is a simple for the UE, it limits the flexibility for the NW to configure resources for discovery. With option b), the indication discussed in Q1 (Alt 2) could be useful for the NW to configure some or all the shared pool only for SL communication, which can bridge the gap between option a) and option b). |
| Intel | b) | For relaying, if dedicated pools for discovery only are configured, then the UEs can use it for discovery rather than the shared pool but the gNB has the flexibility to configure both. On the other hand, we need to discuss whether gNB supporting non-relay discovery only is a valid scenario wherein discovery only UEs is a more likely possibility. |
| LG | see comment | We didn’t make any agreement to configure both shared and dedicated resource pool for discovery message. It’s sill FFS points whether to configure simultaneously. So, we need to make agreement at first clearly whether both shared and dedicated resource pool will be configured simultaneously or not. After that, we can discuss this question. |
| ZTE | b) | We think option b) allows more flexibility for the network configuration and sidelink resource usage. To confine the resource pool only for sidelink communication may be detrimental to load balance. |
| vivo | a) | Agree with OPPO’s comments above. Allowance to transmit discovery message in both dedicated resource pool and shared resource pool collides with the motivation to introduce dedicated resource pool for power consumption reduction. Meanwhile, the condition for option-A should be more clear, i.e. a) Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication when dedicated pools for discovery only are configured. |
| Sharp | b) | Option b) is more flexible. |
| Spreadtrum | a) | If both shared pool and discovery pools are allowed to be configured simultaneously, we prefer the shared pool is only used for communication, then the benefit of having dedicated discovery pool can be kept, e.g. power consumption reduction. |
| Sony | b) |  |
| Huawei, HiSilicon | a) | We would like to emphasis the motivation to introduce dedicated resource pool is for UE power consumption. If both shared and dedicated pools are provided in SIB and allowed for discovery, why we need dedicated pools here? And to be clear, we understand the RP in sl-TxPoolSelectedNormal is supposed for SL communication, whether it could be always taken as shared RP for discovery depends on the discussion above. |
| Apple | a) | Not sure why Option b) is needed. If we want to support option b), then UE shall only be configured with shared pool because there is no power saving gain expected for such a configuration. |
| Lenovo&MM | b) | B provides gNB more flexible for pool configuration. |

### Dedicated RRC configuration

Then, we discuss configuration in dedicated RRC message for RRC_CONNECTED UE. Please note that the serving cell should know whether the concerned CONNECTED UE supports only sidelink communication, or discovery only, or both discovery and SL communication. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.5 Example of both shared and dedicated pools configured in RRC (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pool (i.e., sl-RxPool, sl-TxPoolSelectedNormal and sl-TxPoolScheduling):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can NOT be configured simultaneously.

Q6: For RRC_CONNECTED UEs, which option(s) do you think can be configured in dedicated RRC message for discovery besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can NOT be configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a | See comment in Q5 |
| Ericsson | b) | If dedicated and shared resource pools can be configured together, option b would be better. The other options put too much restriction to the gNB. |
| InterDigital | b) | Same as Q5, because there should be no limitations placed on what the network is allowed to configured in dedicated RRC. |
| Qualcomm | b) | For CONNECTED UE, gNB is aware of whether it is discovery only, or SL communication only, or supporting both. So, we agree with some company that it seems no strong motivation, but we also agree with Ericsson and IDT that RAN2 generally leave the option to NW (i.e., not prohibit it in spec and up to NW whether to configure it. Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we also prefer b) because of the 2 issues of a) analyzed in our comments to Q5. Similar to Q5, in this case, it is up to gNB implementation to select dedicated pool or shared for discovery in Mode 1, and it is up to UE implementation in Mode 2. |
| Kyocera | b) | We should allow the gNB with flexibility to configure both dedicated pool and shared pool for discovery. |
| Intel | b) | Same comment as in Q5 |
| LG | See comment | We didn’t make any agreement to configure both shared and dedicated resource pool for discovery message. It’s sill FFS points whether to configure simultaneously. So, we need to make agreement at first clearly whether both shared and dedicated resource pool will be configured simultaneously or not. After that, we can discuss this question. |
| ZTE | b) | Similar to Q5, we think the option b) is better. |
| vivo | a) | See our comments for Q5 |
| Sharp | b) | Same comment as in Q5. |
| Spreadtrum | a) | Same comment as in Q5. |
| Sony | b) |  |
| Huawei, HiSilicon | a) | Same comments in Q5 |
| Apple | a) | Same comment as Q5. |
| Lenovo&MM | b) | See comment as in Q5. |

### Pre-configuration

Finally, we discuss pre-configuration for OOC UE. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.6 Example of both shared and dedicated pools in pre-configuration (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pool (i.e., sl-RxPool and sl-TxPoolSelectedNormal):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously.

Q7: For OOC UEs, which option(s) do you think can be pre-configured for discovery besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be pre-configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be pre-configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be pre-configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a) | See comment in Q5 |
| Ericsson | b) |  |
| InterDigital | b) | Same as Q5, because there should be no limitations placed on what the network is allowed to configured in preconfiguration. |
| Qualcomm | b) | We tend to agree with some company that it seems no strong motivation, but we also agree with Ericsson and IDT that RAN2 generally leave the option to NW (i.e., not prohibit it in spec and up to NW whether to configure it). Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we also prefer b) because of the 2 issues of a) analyzed in our comments to Q5. In option b), we prefer it is up to UE implementation in Mode 2 to select shared pool or dedicated pool to transmit discovery message. |
| Kyocera | b) | Same comment as in Q6 |
| Intel | b) | Same comment as in Q5 |
| LG | See comment | We didn’t make any agreement to configure both shared and dedicated resource pool for discovery message. It’s sill FFS points whether to configure simultaneously. So, we need to make agreement at first clearly whether both shared and dedicated resource pool will be configured simultaneously or not. After that, we can discuss this question. |
| ZTE | b) | Same as Q5 and Q6. |
| vivo | a) | See our comments for Q5 |
| Sharp | b) | Same comment as in Q5. |
| Spreadtrum | a) | Same comment as in Q5. |
| Sony | b) |  |
| Huawei, HiSilicon | a) | Same comments in Q5 |
| Apple |  | Same comment as Q5. |
| Lenovo&MM | b) | See comment as in Q5. |

### Selection between shared pool and dedicated pool (if allowed)

If you pick option b) for Q5 or Q6 or Q7, it means you think:

- Both dedicated pool and shared pool can be configured simultaneously (at least in some cases), and

- Both can be used for discovery.

Thus, a followed question is how to select between the two (i.e., shared pool and dedicated pool) to carry discovery traffic. This question was discussed in RAN2#115-e, and it was agreed that it is up to gNB for mode 1, but 3 options were captured in Chair Notes to down select for mode 2:

Agreements:

FFS if the network can configure shared and dedicated pool simultaneously.

For mode 1, if agreed that both shared and dedicated resource pools can be configured, it is up to gNB which one the UE should use to transmit discovery message.

For mode 2, if agreed that both shared and dedicated resource pools can be configured, downselect from the following options:

- Left to UE implementation

- Dedicated pool should be prioritised

- Shared pool should be prioritised

Q8: If you pick option b) for either Q5 or Q6 or Q7, which option do you prefer on selection between shared pool and dedicated pool to carry discovery traffic in Mode 2?

- Left to UE implementation

- Dedicated pool should be prioritized.

- Shared pool should be prioritized

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| Ericsson | a | It is sufficient to leave to UE implementation. |
| InterDigital | b) or c) with comments | For the network to have the full advantage of configuring both pools, not only should the UE prioritize one pool over another, but there should be conditions under which the UE should be allowed to transmit on the non-prioritized pool (e.g. based on congestion on one pool). Otherwise, the network would need to account for the case (for example) where all UEs choose the dedicated pool, resulting in significant resource fragmentation. |
| Qualcomm | a | Please note that a) is the outcome of legacy LCP procedure specified in 38.321 without further spec impact: - RAN2 has agreed to use the same LCH priority value 1 for sidelink discovery message - According to existing clause 5.22.1.4.1.2 of TS 38.321, the UE selects the destination associated with the highest SL LCH priority for a new transmission. And we have a NOTE: “if multiple Destinations have the logical channels satisfying all conditions above with the same highest priority, which Destination is selected among them is up to UE implementation.” - Then, when UE sends discovery in Mode 2, it should: - First check all logical channels with LCH priority 1 in configured shared pools and dedicated pools. - If both shared pool and dedicated pool have available logical channels with LCH priority 1, UE can select either one based on its implementation according to the NOTE in clause 5.22.1.4.1.2 of TS 38.321. From technique perspective, benefit of a) is that latency of discovery delivery can be reduced because UE is allowed to select a resource pool which comes earlier. Please note that NW is not aware of when discovery delivery is triggered. Thus, if priority is configured like b) or c), UE may be forced to wait another resource pool for discovery, even if one resource pool is already available. |
| Kyocera | b) and c) with comments | We have similar view with Interdigital regarding NW’s control over the selection between shared pool and dedicated pool, but we could also consider other options for controlling the resource pools, such as RSRP-based or discovery Model type-based. |
| Intel | - or b) | From spec. impact point of view, a) seems to be the best option with least impact. However, from resource utilization point of view, if dedicated pool is configured, it makes sense to prioritize its usage; at the same time, we also need to define when the UE is allowed to use shared pool instead of the dedicated pool (overall, causing more spec. impact). |
| LG | b) or c) | Same opinion with InterDigital. |
| ZTE | a) | We think it can be up to UE implementation. For example, the relay/remote UE may prioritize the dedicated discovery resource pool for discovery message transmission. On the other hand, if the dedicated discovery resource pool is congested, it is not necessary to prohibit the relay/remote UE from using other sidelink resource pool for discovery message transmission. |
| Sharp |  | It can be left to UE’s implementation. |
| Sony | a) |  |
| Lenovo&MM | a) |  |

# Conclusion

Based on companies’ inputs, Rapporteur made below proposals:

# References

[1] R2-2108949, [AT115-e][617][Relay] Continuation of discussion on discovery (CATT), CATT

[2] R2-2106994, Leftover Issues for Sidelink Discovery, CATT

[3] R2-2107089, Remaining issues on relay discovery, Qualcomm

[4] R2-2107313, Leftover aspects of Relay discovery, Intel

[5] R2-2108152, Relay Discovery transmission for stage 3, LG

[6] R2-2107212, Discussion on remaining issue of relay discovery, OPPO

[7] R2-2108143	Further discussion on Relay discovery, ZTE, Sanechips

[8] R2-2107468, Left issues for SL discovery, Ericsson

[9] R2-2108626, Remaining issue on relay discovery, Huawei, HiSilicon