3GPP TSG-RAN WG2 Meeting #116 Electronic							 R2-2xxxxx

Online, Nov. 1st – Nov. 12th, 2021

Title:	LS on TRS-based SCell activation details

Response to:	-

Release:	Release 17

Work Item:	LTE_NR_DC_enh2

Source:	RAN2

To:	RAN1, RAN4

Contact person:	Shukun Wang

wangshukun@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

- 1	Overall description

RAN2 discussed the TRS based SCell activation and mad the following agreements.

- 1: For TRS based SCell activation, RAN2 finalizes the MAC CE based SCell activation case first and come back on RRC case if time allows.

- 2: The TRS can be activated for fast SCell activation, only when all following conditions are met:

- (a)	The TRS for SCell activation is configured for this SCell;

- (b)	The SCell is activated from deactivated state by New SCell A/D MAC CE;

- (c)	The BWP indicated by firstActiveDownlinkBWP-Id is not dormant BWP;

- FFS how we handle the case when some Scells use TRS and some don't

- RAN2 will not specify UE behaviour for the case when new MAC CE is used but a)+c) are not fulfilled for the SCell that uses TRS

- 3: One new MAC CE for to trigger both SCell activation and corresponding temporary RS.

- 4: Define 2 eLCIDs for new MAC CEs with “one octet” SCell activation indication and with “four octet” SCell activation indication respectively.

- Wait for RAN1 input on RRC parameters and capabilities

RAN2 also discussed the design of new MAC CE for TRS activation, including the alternative of using per-SCell TRS configuration index  and the alternative of using per-UE TRS trigger state id  and would like to ask RAN1 to provide feedbacks for the following questions to help RAN2 to progress further.

For Alt1 (include per SCell TRS configuration index in MAC CE):

Q1: What is the maximum number of TRS configurations supported per SCell? Is there a difference for FR1 and for FR2?

For Alt2 (include per UE TRS trigger state id in MAC CE)

Q2: What is the maximum number of TRS trigger states (where a "trigger state" indicates a set of TRS used for activation of a set of SCells) supported per UE? Is there a difference for FR1 and for FR2?

- 2	Actions

To RAN1:

- RAN2 respectfully asks RAN1 to take into account the above RAN2 agreements ,provide answers for the questions of Q1 and Q2.

- RAN2 respectfully asks RAN1 to provide a list of RRC parameters .

To RAN4:

RAN2 respectfully asks RAN4 to take into account the above RAN2 agreements in RAN4 work.

- 3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #116bis-e 	Jan. 17 – Jan. 25, 2022

TSG-RAN2 Meeting #117-e 	Feb. 21 – Mar. 03, 2022