3GPP TSG-RAN2 Meeting #116bis-e	R2-22xxxxx

Online, 17 - 25 January, 2022

Agenda Item:	9.2.1

Source:		Huawei, HiSilicon

Title:		Report of [Post-116bis-e][046][IoT-NTN] 36.331

Document for:	Discussion and decision

# Introduction

This is the summary of offline:

- [Post116bis-e][046][IoT NTN] 36331 (Huawei)

Scope: Updated running CR taking into account agreements of R2-116bis-e. Best effort review. Endorsement if possible. Capture TS related Open Issues, not captured elsewhere and suggest how to treat.

Intended outcome: Updated Running CR, reviewed, baseline for next meeting. TS related Open issue with suggestion how to treat.

Deadline: Short.

The document collect the RRC open issues to be closed in RAN2#117-e.

Based on chair’s recommendations:

- Open Issues are defined for aspects that need to be closed, important to make already agreed functionality work in a reasonable way. Not yet agreed optimisations that may not be needed are not be listed as Open Issues.

- For immature areas, e.g. UE capabilities for some WIs, it might not be useful/helpful to attempt define specific open issues. It might be better to address such area by company tdocs under an Agenda item (as usual) instead. Some Rel-17 WIs has major parts that may be too Immature to usefully define specific Open Issues.

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting).

  - Other, e.g. immature area, reference to dependency, unclear status etc.

# Contact Information

| Company | Name | Email |
| --- | --- | --- |

# Discussion

## Open issues captured as Editor’s notes in the running CR

### Discontinuous coverage

Satellite assistance information for discontinuous coverage

Editor’s Note: Agreement: Satellite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information is FFS. FFS whether any applicable agreements made in NR-NTN can be reused. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS. Editor: this agreement is captured here until it is decided in which SIB the assistance information is located.

Editor’s Note: Providing the start-time of (incoming) satellite’s coverage and end time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites. Editor: this agreement is captured here until it is decided in which SIB the incoming satellite assistance information is located

#### Open Issue: Satellite assistance information for discontinuous coverage

Discussion point 3.1.1-1: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

### User plane aspects

User plane timers

In the running CR, there are the following FFS:

Editor’s Note: Agreement: RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

Editor’s Note: Agreement: The PDCP discardTimer should be extended to support eMTC over NTN. Agreement: If PDCP discardTimer is agreed to be extended to support eMTC over NTN, how to extend the timer value can wait for the conclusion for RLC t-reordering timer

Editor’s Note: Agreement: RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN. Agreement: On how to extend RLC t-Reordering in IoT NTN, wait for NR NTN agreements and see if they can be reused.

#### Open Issue: How to extend sr-ProhibitTimer, and PDCP discardTimer, RLC t-Reordering

Rapporteur thinks these are user plane issues but they have no impact on user plane CRs and have impact on RRC signalling.

Discussion point 3.1.2-1: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

TA report configuration

In the running CR, there is an FFS as follows:

TA-ReportConfig-r17 ::=		TypeFFS

#### Open Issue: Configuration of event-triggered TA report

Rapporteur thinks this is user plane issue but has impact on RRC signalling. Rapporteur thinks that NR NTN signalling can be used as a baseline.

Discussion point 3.1.2-2: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

### Control plane aspects

System information acquisition:

In the running CR, there is an FFS on how to trigger re-reading of SIBXX and an FFS if the UE shall reacquire the new SIB when SI update is triggered.

Editor’s Note: Agreement: Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered. Agreement: UE acquires the NTN specific SIB before accessing the cell. Agreement: When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back. Editor: RRC_IDLE has not been discussed yet.

Acquisition of SIBXX for connected mode has been concluded. However, RRC_IDLE has not been discussed.

#### Open Issue: FFS if RRC_IDLE UE is required to read SIBXX and whether some mechanism is needed to trigger the UE to reacquire the NTN specific SIB in RRC_IDLE

Discussion point 3.1.3-1: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

UL synchronisation timer expiry:

In the running CR, there is an FFS anything additional to rereading SIBX upon UL synchronisation timer expiry.

Editor’s Note: Agreement: When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back FFS whether anything additional is needed.

#### Open Issue: FFS if anything additional is needed on expiry of the UL synchronisation timer

Discussion point 3.1.3-2: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

Barring NTN cell access for NTN UE

In the running CR, there is an FFS whether to introduce a new bit for barring NT UE to access a NTN cell.

Editor’s Note: Agreement: We will have the barring bit to prevent terrestrial UEs to use NTN. FFS if we define a new barring bit for NTN UEs barring.

#### Open Issue: FFS if we define a new barring bit for NTN UEs barring

Discussion point 3.1.3-3: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

RAN1 parameter signalling

In the running CR, there is two Editor’s Notes as follows:

Editor’s Note: The signalling range of positionX, positionY, positionZ is larger than the range indicated by RAN1. Need to check the exact signalling.

Editor’s Note: The signalling range of velocityVX, velocityVY, velocityVZ is shorter than the range indicated by RAN1. Check with RAN1 whether to increase the range or adjust the step size.

#### Open Issue: Signalling range of positionX, positionY, positionZ

Rapporteur thinks this is a RAN2 issue, i.e. whether to allow more than needed or restrict. Note the same issue also applies to NR.

Discussion point 3.1.3-4: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

#### Open Issue: Signalling range and step size of velocityVX, velocityVY, velocityVZ

Rapporteur thinks this is a RAN1 issue and that we should wait for an update of the parameters list. Note the same issue also applies to NR.

Discussion point 3.1.3-5: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

## Open issues not captured in the running CR

UE location reporting

It was agreed in RAN2#116bis-e

-	Assume that eMTC can follow whatever is agreed for NR NTN

-	For NB-IoT, assume that the location info need to be protected, also coarse location info, as has been stated by SA3. FFS if location can be reported by NAS, can ask CT1/SA2. Can also ask SA3 to confirm their view on coarse location information. Keep R3/SA2 informed.

#### Open Issue: UE location reporting in eMTC

Rapporteur thinks this is dependent on NR agreements and feedback for other groups.

Discussion point 3.2-1: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

#### Open Issue: UE location reporting in NB-IoT

Rapporteur thinks this is dependent on NR agreements and feedback for other groups.

Discussion point 3.2-2: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

UE capabilities

UE capability are not captured in the running CR

#### Open Issue: UE capability signalling

Rapporteur thinks discussion on UE capability signalling should take place separately.

Discussion point 3.2-3: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

Signalling of SIBXX in dediacted signalling at HO

eMTC UEs are not required to acquire SI in RRC_CONNECTED mode. Instead, in legacy, the required SI (SIB1, SIB2) are provided in the handover command message. For NTN, SIBXX is also required in connected mode.

#### Open Issue: Provision of SIBxx in dedicated signalling at HO

Discussion point 3.2-4: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

Signalling of 2 EARFCN bits in MIB for NB-IoT

RAN1 has sent a LS to RAN4 (R1-2112768) about DL synchronisation enhancements for NB-IoT. In the LS, RAN1 propose to signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used. However, RAN1 has not listed the parameter in the RRC parameters list.

#### Open Issue: Signalling of Part-of ARFCN indication in MIB for NB-IoT

Rapporteur thinks it is safer to introduce the bits in MIB now independently of RAN4 reply.

Discussion point 3.2-5: Do you agree with the open issue and the suggested treatment

| Company | Yes/No | Comments |
| --- | --- | --- |

## Other

Please indicate any other open issues not discussed in the above.

| Company | Comments |
| --- | --- |

# Conclusion

# Reference

- R2-2201451 Running CR - Support of Non-Terrestrial Network in NB-IoT and eMTC, Huawei, RAN2#116bis, January 2022

- R2-2200095 “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN1#107-e, November 2021

- R1-2112975 “Consolidated high layers parameters list for Rel-17 LTE, RAN1#107-e, November 2021