3GPP TSG-RAN WG2#116bis-e	R2-220xxxx

Online, 17 - 25 January 2022

Source: 	Huawei, HiSilicon

Title: 	Open issues for MR DC/CA further enhancements

Agenda Item:	8.2.2.1

Document for:	Information

# 1	Introduction

This document lists:

-	all issues for MR DC/CA further enhancements raised until this meeting;

-	the related proposals submitted to this meeting.

Issues such that, if nothing is done, the feature would still work, are in blue and listed at the end.

# 2	List of issues

## 2.1	Efficient SCell activation

There are the following RAN2 open issues:

1.	finalize RRC and MAC draft CRs

## 2.2	Efficient activation/deactivation on one SCG

There are the following RAN2 open issues:

1.	At deactivation, for primaryPath, ul-SplitThreshold, PDCP duplication:

-	rely on explicit signalling/reconfiguration for uplink data to go to MCG only; or

-	UE autonomous action to change parameters (and possibly restore at SCG activation); or

-	modify PDCP specification to ensure data submission to MCG leg(s) only

2.	MAC actions at SCG deactivation

-	FFS if UE initializes Bj for each logical channel to zero upon SCG deactivation as a part of partial MAC reset. Should consider e.g. what to do with possible Bj increase while SCG is deactivated.

-	FFS if UE discards explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any, upon SCG deactivation as a part of partial MAC reset.

-	FFS if the BWP associated with PSCell is NOT deactivated upon SCG deactivation.

3.	Active BWP while the SCG is deactivated and at SCG activation

-	Do we model a BWP as active while the SCG is deactivated, for the purpose of RRM/RLM/BFD?

-	Does the UE switch to first active BWP upon SCG deactivation?

-	Can BWP be switched while the SCG is deactivated?

-	Can BWP be switched at SCG activation?

4.	TA timer and RLM/BFD while the SCG is deactivated

-	Stop TA timer upon RLF or BF?

-	Stop RLM/BFD after TA timer expiry?

-	Stop RLM/BFD after RLF/BF?

-	After RLF/BF, resume RLM/BFD upon reconfiguration (withsync)?

-	can the UE perform RACH-less activation if the UE is not configured to perform RLM/BFD while the SCG is deactivated?

5.	MCG power limitation and PDCCH blind decoding limitation while the SCG is deactivated

-	any issue to do "as if the SCG would be activated"?

6.	UE indication that the SCG is deactivated

-	Agreed to support but no design discussed

Note: SCG activation/deactivation could work without

7.	RRC stage 3 aspects

-	finalize UE indication of uplink data while the SCG is deactivated

-	clarify handling of tci-Info for RLM/BFD while the SCG is deactivated

-	Need to clarify that rach-ConfigDedicated is not included when the SCG is deactivated (it is need N)

-	Need for any change in ASN.1/procedure

8.	MAC stage 3 aspects

9.	Non-agreed/discussed proposals

-	MAC CE for SCG activation (needs TP if to be supported)

-	MCG failure information while the SCG is deactivated (needs TP if to be supported)

-	Direct SCell activation at SCG activation (impacts mostly RAN4 requirements and UE capabilities)

-	Dedicated RACH resources indicated before SCG activation indication, i.e. when going to the SCG deactivated state or while the SCG is deactivated (needs TP if to be supported)

## 2.3	CPAC

There are the following RAN2 open issues:

1.	Relation with deactivated SCG

-	is CPC triggered even if the SCG is deactivated SCG

-	can the CPC command include deactivated SCG

2.	Coexistence between Rel-17 CPC (i.e. in MN RRC(Connection)Reconfiguration message) and Rel-16 CPC (i.e. in SN RRCReconfiguration message)

-	is it supported?

-	if yes, what happens upon CPC execution for the other type of onditional configurations?

3.	Coexistence between CHO and Rel-16 CPC, between CHO and Rel-17 CPC

-	is it supported?

-	if yes, what happens upon CPC/CHO execution for the other type of conditional configurations?

4.	Maximum number of CPC configurations

-	how to define it: all combined, distinguish by type of configuration, other

-	MN/SN coordination if needed

5.	Unsynchronized update of MCG configuration at CPC execution

-	rely on network implementation?

-	send a message with the old configuration, before sending the complete message with the new configuration?

-	send the complete message with old configuration?

6.	Any change/optimization related to "Full configuration"?

-	Allow the S-SN to tell the T-SN not to use delta configuration?

-	The T SN always uses full configuration

## 2.4	UE capabilities

1.	efficient SCell activation

2.	SCG deactivation

3.	CPAC