3GPP TSG-RAN WG2 Meeting #117 electronic	R2-220xxxx

- E-meeting, 21st February – 3rd March 2022

Title:	[DRAFT] LS to RAN1 on the inter-UE coordination mechanism

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	vivo [To be RAN2]

To:	RAN1

Cc:

Contact person:	Jing Liang

liangjing <at> vivo <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN2 has reached the following agreement on the priority of IUC related MAC CEs in RAN2 #117-e meeting:

Agreement on IUC:

5: The priority order of a MAC CE for UE-B’s explicit request is between SL CSI reporting MAC CE and SL DRX command MAC CE (when priority of IUC REQ MAC CE is fixed as “1”).

6: The priority order of a IUC Information MAC CE is between SL CSI reporting MAC CE and SL DRX command MAC CE (when priority of IUC Information MAC CE is fixed as “1”).

7: Send LS to RAN1 to inform RAN2 understanding on the priority of IUC INFO/IUC REQ MAC CE and RAN2 preference to fix the priority of IUC INFO/IUC REQ MAC CE as “1”.

RAN2 would like to inform RAN1 that RAN2 prefers to fix the priority value of IUC MAC CE and IUC request MAC CE as “1”. This is because if the priority value of IUC MAC CE and IUC request MAC CE is configurable, the priority order in LCP of IUC MAC CE and IUC request MAC CE will depend on the configured priority value, which is not aligned with the legacy manner and makes the MAC specification complicated. When the priority value is fixed as “1”, the priority order in LCP of an IUC request MAC CE and of an IUC MAC CE, are both between SL CSI reporting MAC CE and SL DRX command MAC CE.

Based on that, RAN2 would like to ask RAN1 to confirm whether the priority value indicated by higher layer parameters priorityScheme1CoordInfoExplicit, priorityScheme1Request, and priorityScheme1CoordInfoCondition refers to the priority value of the MAC CE itself which is used for LCP and multiplexing, or refers to the priority value which is used for sensing and/or candidate resource selection. In the former case (which is RAN2 assumption), RAN2 would like to ask RAN1 to remove these RRC parameters, or in the latter case, RAN2 would like to ask RAN1 to update the field description of these parameters if needed.

Besides, RAN2 has below agreements on the IUC mechanism:

- RAN2 introduces a mechanism of timer-based latency bound restriction for transmission of UE-A’s IUC information. - Timer-based latency bound restriction is applied for the explicit request based UE-A’s IUC information transmission. - RAN2 introduces the timer-based latency bound restriction on the transmission of UE-A’s IUC information for both preferred resource set and non-preferred resource set in explicit request-based IUC. - Working assumption: UE-B sets the timer value to UE-A through PC5 RRC signalling - RAN2 supports that UE-A starts the timer for the transmission of UE-A's IUC information in the explicit request-based IUC when receiving an explicit request from UE-B and deciding to trigger IUC information to be transmitted UE-B.

# 2	Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above RAN2 agreements into account and provide feedback if any concerns.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#118-e	9th May – 20th May 2022		Electronic meeting

RAN2#119	22nd August – 26th August 2022		Toulouse, FR