3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Title:	[Draft]LS out on PEI and UE Subgrouping

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	ZTE Corporation [to be RAN2]

To:	RAN3

Cc: 	SA2, CT1

- Contact Person:

- Name:	                Fei Dong

E-mail Address:	 dong.fei@zte.com.cn

- 1. Overall Description:

RAN2 has discussed about ‘last used cell’ issue for PEI capable UE, and the following agreements were made:

- A PEI-capable UE stores its “last used cell” information. FFS on how to capture this in the specifications. - Do not introduce an associated timer for the “last used cell” information stored by UE. - The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. - Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. - RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell.

With above agreements, RAN2 also has triggered a further discussion on the mismatched understanding about the ‘last used cell’ between UE and NW. RAN2 understands that the UE assumes the ‘last used cell’ is the cell if the last connection was released by it, but CN may have no idea about that if CN is not involved.

To solve this issue, RAN2 assumes that the LTE method can be reused, i.e. to introduce ‘no last cell update’ indication in RRCRelease message for NR PEI. RAN2 would like to ask:

Question 1: Whether the mismatched understanding about the ‘last used cell’ between UE and NW still exists in NR, if so, whether the LTE method (i.e. to introduce ‘no last cell update’ indication in RRCRelease Message) can be reused?

In addition, RAN2 also have discussed one problematic scenario where certain gNB(s) within a RNA does not support CN-assigned subgrouping but others do.

In this scenario, assuming that the anchor gNB does not support CN assigned subgrouping, it may not forward any information (e.g. CN assigned subgroup ID) to other gNBs when initiating a RAN paging to a UE. It may result that the UE will try to receive the paging information according to the CN assigned subgroup ID, if any, after cell-reselection. But the current camped cell that supports CN assigned subgrouping will page UE by not using the CN assigned subgroup ID due to the lack of the CN assigned information about that UE. RAN2 would like to ask:

Question 2: Whether this scenario does exist, if so, whether this problematic scenario can be avoided (i.e  NW support of PEI and Subgrouping has to be uniform across the paging area) or resolved by anything else?

- 2. Actions:

- To RAN WG3, SA2/CT1

- RAN2 respectfully ask RAN3, SA2 and CT1 to take the above into account and provide feed back on the Question 1.

- To RAN WG3

- RAN2 respectfully ask RAN3 to take the above into account and provide feed back on the Question 2.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #118-e		16th – 27th May 2022				e-Meeting

TSG RAN WG2 Meeting #119		TBD								TBD