3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Title:	LS out on PEI and UE Subgrouping

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN2

To:	RAN3

Cc: 	SA2, CT1

- Contact Person:

- Name:	                Fei Dong

E-mail Address:	 dong.fei@zte.com.cn

- 1. Overall Description:

RAN2 has discussed about ‘last used cell’ issue for PEI capable UE, and the following agreements were made:

- A PEI-capable UE stores its “last used cell” information. FFS on how to capture this in the specifications. - Do not introduce an associated timer for the “last used cell” information stored by UE. - The “lastUsedCellOnly” indication is a cell-level configuration and there is no per-subgroup indication. - Introduce a one-bit indication of lastUsedCellOnly in PEI-Config. - RAN2 clarifies the meaning of “last used cell only”: When a cell broadcasts “last used cell only”, a UE monitors PEI only if its last connection was released by this cell.

With above agreements, RAN2 also has triggered a further discussion on the mismatched understanding about the ‘last used cell’ between UE and NW. RAN2 understands that the UE assumes the ‘last used cell’ is a cell by which the last connection was released, but CN may not update the ‘last used cell’ to that cell  if CN was not involved in such connection release.

To solve this issue, one potential solution is to reuse LTE method, i.e. to introduce ‘no last cell update’ indication in RRCRelease message for NR PEI. RAN2 would like to ask:

Question 1: Whether the mismatched understanding about the ‘last used cell’ between UE and NW still exists in NR, if so, whether the LTE method (i.e. to introduce ‘no last cell update’ indication in RRCRelease Message) can be reused?

In addition, RAN2 also have discussed one problematic scenario where certain gNB(s) within a RNA does not support CN-assigned subgrouping but others do.

In this scenario, assuming that the anchor gNB does not support CN assigned subgrouping, it may not forward any information (e.g. CN assigned subgroup ID) to other gNBs when initiating a RAN paging to a UE. It may result that the UE will try to receive the paging information according to the CN assigned subgroup ID, if any, after cell-reselection. But the current camped cell that supports CN assigned subgrouping will page UE by not using the CN assigned subgroup ID due to the lack of the CN assigned information about that UE from the anchor gNB. RAN2 would like to ask:

Question 2: Whether this problematic scenario can be avoided or needs to be resolved through signaling?

- 2. Actions:

- To RAN WG3

- RAN2 respectfully ask RAN3 to take the above into account and provide feed back on above questions.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #118-e		16th – 27th May 2022				e-Meeting

TSG RAN WG2 Meeting #119		TBD								TBD