3GPP TSG-RAN WG2 Meeting #117-e	R2-2206528

# E-meeting, 21st February – 3rd March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0849 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IoT NTN idle mode corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To correct IoT NTN UE idle mode behaviour. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures corrections to IoT NTN: - Removing editor’s note on discontinuous coverage. - Adding condition to do discontinuous coverage. - Correcting use of t-Service. - Remove editor’s note related to NTN-only UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN does not function according to the agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.2.4.2, 5.2.4.2a, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

# 4	General description of Idle mode

## 4.1	Overview

The idle mode tasks can be subdivided into four processes:

-	PLMN selection;

-	Cell selection and reselection;

-	Location registration;

-	Support for manual CSG selection.

The relationship between these processes is illustrated in Figure 4.1-1.

Figure 4.1-1: Overall Idle Mode process

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set TS 23.122 [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

For E-UTRA a cell may be associated with more than one CN type (EPC and/or 5GC) and hence the selected cell can be suitable for more than one CN type. The CN type(s) for which the selected cell is suitable are reported to NAS which selects a CN type to be used for camping and for the NAS registration procedure (see below). Note that CN type selection is only applicabe for UE supporting E-UTRA connected to 5GC.

For E-UTRA a cell may be associated with more than one tracking area. The UE reports all the broadcasted tracking area codes in the selected cell to NAS for registration procedure.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN TS 23.122 [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. Similar to cell selection procedure, if the reselected cell is an E-UTRA cell and the UE supports E-UTRA connected to 5GC, the CN type(s) for which the cell is suitable are reported to NAS which selects one of them. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, a RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 22.011 [4] and search for a suitable cell if another PLMN has been selected by NAS.

Search of available CSGs may be triggered by NAS to support manual CSG selection.

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The UE may perform sidelink communication or V2X sidelink communication or sidelink discovery or NR sidelink communication while in-coverage or out-of-coverage for sidelink, as specified in clause 11.

The purpose of camping on a cell in idle mode is fivefold:

a)	It enables the UE to receive system information from the PLMN.

b)	When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the PLMN receives a call for the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNAs (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in this set of tracking areas. The UE will then receive the paging message because it is tuned to the control channel of a cell in one of the registered tracking areas and the UE can respond on that control channel.

d)	It enables the UE to receive ETWS and CMAS notifications.

e)	It enables the UE to receive MBMS services.

If the UE is unable to find a suitable cell to camp on or if the location registration failed (except for LR rejected with cause #12, cause #14, cause #15 or cause #25, see TS 23.122 [5] and TS 24.301 [16]), it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state.

When NAS indicates that PSM starts, the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while the UE is in PSM it is up to UE implementation whether it performs the corresponding action immediately or the latest when PSM ends. When NAS indicates that PSM ends, the UE shall perform all idle mode tasks.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in SystemInformationBlockType32, SystemInformationBlockType31 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage the UE shall perform all idle mode tasks.

End of change

Start of change

#### 5.2.4.2	Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SIntraSearchP, the UE may choose not to perform intra-frequency measurements.

-	Else if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to TS 36.133 [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SnonIntraSearchP, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the exact time to start measurements before t-Service is up to UE implementation. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

#### 5.2.4.2a	Measurement rules for cell re-selection for NB-IoT

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NB-IoT inter-frequencies which are indicated in system information:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP, the UE may choose not to perform inter-frequency measurements.

-	Otherwise, the UE shall perform inter-frequency measurements.

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3-NB, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE shall perform intra-frequency or inter-frequency measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP or Srxlev > SnonIntraSearchP, and the exact time to start measurements before t-Service is up to UE implementation.

End of change

Start of change

### 5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB message) TS 36.331 [3] by means of the following fields:

-	cellBarred (IE type: "barred" or "not barred") 
This field indicates if the cell is barred for connectivity to EPC.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR.
This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1-BR or SIB1-NB.
In case of multiple EPC PLMNs indicated in SIB1/SIB1-BR, this field is common for all EPC PLMNs

NOTE 1:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 36.331 [3].

-	cellBarred-5GC (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC.
This field is ignored if the UE does not support E-UTRA connected to 5GC or if the UE supports network-based CRS interference mitigation and nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR. 
In case of multiple EPC or 5GC PLMNs indicated in SIB1/SIB1-BR, this field is specified per EPC or 5GC PLMN.

-	cellBarred-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to EPC for UEs supporting network-based CRS interference mitigation.
barred means the cell is barred for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, barred means the cell is barred while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored by the UE if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is common for all PLMNs.

-	cellBarred-5GC-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC for UEs supporting network-based CRS interference mitigation.
This field is ignored if the UE does not support E-UTRA connected to 5GC or network-based CRS interference mitigation.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse-CRS (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use for UEs supporting network-based CRS interference mitigation.
reserved means the cell is "reserved" for operator use for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, reserved means the cell is "reserved" for operator use while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is specified per PLMN.

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. This field indicates if the cell is barred for IAB node or the cell does not support IAB node, or both. When this field is absent, the IAB node shall treat this cell as if cell status is barred.

-	cellBarred-NTN (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to EPC via NTN.
This field is ignored if the UE does not support NTN connectivity.

The following description for handling of barred and reserved cells is per CN type. If the UE supports more than one CN type, the UE shall only exclude a cell as candidate for selection/reselection if it is excluded for both CN types.

NOTE 2:	Fields cellBarred-CRS and cellReservedForOperatorUse-CRS are not indicated in SystemInformationBlockType1-NB

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	UEs assigned to Access Class 11 or 15 (or corresponding Access Identity) operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to an Access Class in the range of 0 to 9 (or corresponding Access Identity 0), 12 to 14 (or corresponding Access Identity) or to Access Identity 1, 2 or 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 3:	ACs 11, 15 (or corresponding Access Identity) are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 (or corresponding Access Identity) are only valid for use in the home country TS 22.011 [4].

NOTE 4:	Access Identities 1, 2 are valid in the PLMNs as specified in TS 22.261 [41].

NOTE 5:	Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [41].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall consider other cells for cell selection/reselection according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MasterInformationBlock (or MasterInformationBlock-NB), the SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB), or the SystemInformationBlockType2 (or SystemInformationBlockType2-NB):

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	the UE may select the same cell in normal coverage if the UE was barred in the cell due to being unable to acquire MasterInformationBlock, SystemInformationBlockType1-BR, or SystemInformationBlockType2 in enhanced coverage, but was able to acquire MasterInformationBlock, SystemInformationBlockType1, and SystemInformationBlockType2 in normal coverage, if the selection criteria are fulfilled.

-	the UE may select the same cell in enhanced coverage if the UE was barred in the cell due to being unable to acquire MasterInformationBlock, SystemInformationBlockType1, or SystemInformationBlockType2 in normal coverage, but was able to acquire MasterInformationBlock, SystemInformationBlockType1-BR, and SystemInformationBlockType2, if the selection criteria are fulfilled.

-	else

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT or, if the previous and selected cell are both E-UTRA cells, a change of the CN type.

End of change