3GPP TSG-RAN2 Meeting #118-e								R2-220xxxx

# Electronic, 9th – 20th May, 2022

Source: 		Email discussion Rapporteur (Huawei, HiSilicon)

Title: 	Summary for [Post118-e][604] 38.321 Positioning CR (Huawei)

Agenda item:	6.6.2

Document for: 	Discussion and Decision

General

This document contains the list of comments made during the review of the MAC CR for Positioning

Contacts

| Name | Company | Email address |
| --- | --- | --- |
| Xiaolong Li | Xiaomi | lixiaolong1@xiaomi.com |

Comments

| Companies | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Xiaomi | We are not clear why UE needs to cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE, in our understanding, the UE shall not cancel it. | Suggest to remove the sentence marked with yellow. The MAC entity shall, 1>if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered, and not cancelled: 2> if indication from upper layer has been received that the triggered Positioning Measurement Gap Activation/Deactiation Request MAC CE should be cancelled: 3> cancel the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. 2> if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer's request; 3> cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE. 2> else: 3> trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE. |  |
| Xiaomi | The reference clause should be 5.26.2. | The MAC entity shall, if the TA of the configured Positioning SRS is valid according to clause 5.26.2: - transmit Positioning Periodic SRS or Semi-Persistent SRS defined in TS 38.214 [7]. |  |